| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E025r1 Filename: JVT-E025r1.doc |
| --- | --- |

| Title: | Clean-up and improved design consistency for ABT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mathias Wien Institut und Lehrstuhl für Nachrichtentechnik RWTH Aachen 52056 Aachen, Germany | Tel: Email: | ++49-241-80-27681 wien@ient.rwth-aachen.de |
| Source: | Aachen University |  |  |

_____________________________

# Introduction

In this document some clean-up and improved design consistency issues are discussed and some simplifications are proposed.

The ABT direct and inverse transform and quantization process as they are used in the JM and specified in the FCD is analyzed. The analysis proves the consistency with the 16-bit design criteria as they were specified by the transform complexity AdHoc group [VCEG-O09].

The syntax for ABT intra block in I slices and P and B slices is harmonized and the specification of the intra prediction modes is modified to make directional intra prediction in blocks of size 4x4, 4x8, 8x4, and 8x8 mode consistent.

Teruhiko Suzuki from Sony Corp pointed out an intra performance problem with the current specification of ABT intra prediction. This problem became obvious especially on HD material in smooth regions. In this document, the application of the base line Intra16x16 in addition to the existing ABT intra block modes is proposed. Introducing Intra16x16 in addition to the ABT intra modes solves this issue at a minimum change to the document as well as to the software, while making the design more consistent with the non-ABT case.

The document is organized as follows. In the first part, the dynamic range analysis for the ABT transform and quantization is given. In the second part, the proposal for the modified intra syntax and intra prediction is presented.

# Dynamic range analysis for ABT transform and quantization

In the AdHoc report of the AHG on transform and quantization complexity reduction at the Pattaya meeting, the conditions for compliant transform and quantization processes were defined [VCEG-O09]. The main issues are

- 16-bit memory for input, intermediate and output

- Arithmetic logic unit supporting 32-bit operations

- 16-bit multiplier.

These constraints were defined for 9-bit residuals.

## Assuming a normalized direct transform

The normalized transform matrix T8 as used in the JM approximates the 8x8 DCT matrix. Before looking at T8, we briefly highlight the dynamics of the DCT. The maximum dynamic increase of the transform is given by

where tij is the (DCT) matrix coefficient at position i in row j. For the DCT, the maximum value is reached for j = 0 (DC) and j = 4. The coefficients of the DC base function are constant and have a value of  .

That means, the dynamic range of a 9 bit input signal is expanded by for each 1D transform. Hence, assuming a quantization of step size 1 in the transform domain, the 2D transform expands the dynamic by a factor of 8, i.e. 3 bit. Therewith, assuming a quantization step size of 1 in the transform domain, the dynamic range of the signal at the encoder side as well as the dynamic range of the re-scaled signal at the decoder side is 12 bit.

## Analysis for the JM 8x8 transform and quantization

In the following, the dynamic range of the forward and inverse transform and quantization process as implemented in the JM software is analyzed.

The 8x8 transform matrix used in the JM is given as

### Direct and inverse transform and quantization process

For an input signal X of size 8x8 samples, the transform and quantization process can be written as follows:

Here, '//' denotes scaling by a factor n ( xq = sign(x)[abs(x) + 2n-1]>>n ).  E contains the transformed and quantized coefficients for the 8x8 block X. The quantization factor is denoted as q. TT is the transposed matrix T. Since the encoder is not specified by the standard, we do not focus on realization of the encoder side in 16 bit dynamic. At the decoder side, the following inverse transform and quantization operations are specified:

After inverse scaling with the inverse scaling factor r and the horizontal inverse transform a first bit shift is introduced. J is the reconstructed signal X.

The forward and inverse scaling factors q and r used for quantization in the FCD double every 6 QP. Therefore, the scaling factors can be separated into a mantissa and an appropriate power of 2. The forward and inverse scaling factors are

Here, QP/6 and QP%6 denote the integer division of Qp and the modulo operation by 6, respectively. (For simplicity reasons we use QP = QPFCD –12 here).  In the FCD, the mantissa tables Q and R are given as

Since the exponent does not influence the dynamic range of the transform process after inverse scaling, it is sufficient to consider the quantization / inverse quantization process for exponent 0.

### Quantizer design

In the JM, the following shift factors ni , i = 0,1,2,3 are used

The squared norm of T8 is . The scaling and inverse scaling factors hold following condition:

For k = 0,1,...,5, and with ni , i = 0,1,2,3, and N8 from above, this gives, in a slight abuse of notation

.

From this property of Q and R it can be concluded, that it is sufficient to analyze the dynamic range for only one scaling / inverse scaling factor pair.

### Dynamic Range

In the following, we give the analysis for Q0 = 2506 and R0 = 15.

The forward and inverse transform can be written as follows

where and .

Applying the transform T8 in one direction boosts the dynamic of the transformed signal by a factor of , hence 6.7 bits are added to the dynamic range. Since, and the dynamic is increased by a factor of  1042 in , only a factor of

is induced by the application of both, the horizontal and vertical inverse transform.

The dynamic range of the outcome of the equations from  and  is given in the table below. It is assumed that the input signal X has a dynamic range of 9 bit.

| Output | dynamic of operation | resulting dynamic [bit] |
| --- | --- | --- |
| X | - | 9 |
| A = T8 X | 9 + 6.7 | 15.7 (16) |
| B = A T8' | 15.7 + 6.7 | 22.4 (23) |
| C = B >> n0 | 22.4 –4 | 18.4 (19) |
| D = q C | 18.4 + 11.3 | 29.7 (30) |
| E = D >> n1 | 29.7 – 19 | 10.7 (11) |
| F = r E | 10.7 + 3.9 | 14.6 (15) |
| G = F T8 | 14.6 + 3.7 | 18.3 (19) |
| H = G >> n2 | 18.3 – 7 | 11.3 (12) |
| I = T8' H | 11.3 + 3.7 | 15.0 (15) |
| J = I >> n3 | 15-6 | 9 |

In order to be compliant with the constraints listed above, all multiplications and memory accesses on the decoder side (F to J) need to be in 16-bit.

- F results from a 16-bit multiplication, the result (that might be stored) does not exceed 16-bit.

- G is derived by adding the results of 16-bit multiplies. The summation might exceed 16-bit but the result is shifted down into the 16-bit range in H before storing to memory.

- I and J stay in the 16-bit range. During the calculation of I, the summation might exceed 16-bit but the result that needs to be stored is reduced to fit into 16-bit dynamics.

## Analysis for the JM 8x4 and 4x8 transform and quantization

The analysis can be given for 4x8 and 8x4 blocks as well.

The JM 4x4 transform and inverse transform matrices are given as

The even basis functions increase the dynamic range of the transformed signal by a factor of 4 (2 bit), while the odd basis functions increase the dynamic by a factor of 6 (2.6 bit). Following the argumentation from equation  onward, the inverse transform does not change the dynamic range of the (already transformed) signal for even base functions. It can be seen, that for odd base functions, the dynamic is reduced by a factor of 5/6 (-0.3 bit).

Due to the variable norm, the analysis has to be done separately for even and odd columns or rows of the 8x4 and 4x8 blocks, respectively. The scaling and inverse scaling factors for these blocks are given as

.

The values for the even rows or columns are given in the first column of Q and R and the values for the odd rows or columns are given in the second column of Q and R, respectively. Again, the first quantizer pair is evaluated. Q = 11030 corresponds to a dynamic increase of 13.4 bit while Q = 7220 gives an increase of 12.8 bit. The inverse scaling factors increase the dynamic by factors of 9 and 11, i.e. 3.2 and 3.5 bit.

### Dynamic Range

For the 4x8 transform:

| Output | dynamic of operation |  | resulting dynamic [bit] |  |
| --- | --- | --- | --- | --- |
| ##### X | even row | odd row | even row | odd row |
| A = T8 X | 9 + 6.7 | 9 + 6.7 | 15.7 | 15.7 |
| B = A T4' | 15.7 + 2 | 15.7 + 2.6 | 17.7 | 18.3 |
| C = B >> n0 | 17.7 – 2 | 18.3 – 4 | 15.7 | 14,3 |
| D = q C | 15.7 + 13.4 | 14.3 + 12.8 | 29.1 | 27.1 |
| E = D >> n1 | 29.1 – 19 | 27.1 – 17 | 10.1 | 10.1 |
| F = r E | 10.1 + 3.2 | 10.1 + 3.5 | 13.3 | 13.6 |
| G = F Tinv4 | 13.3 + 0 | 13.6 – 0.3 | 13.3 | 13.3 |
| H = G >> n2 | 13.3 – 2 | 13.3 – 2 | 11.3 | 11.3 |
| I = T8' H | 11.3 + 3.7 | 11.3 + 3.7 | 15.0 | 15.0 |
| J = I >> n3 | 15.0 - 6 | 15.0 - 6 | 9.0 | 9.0 |

For the 8x4 transform:

| Output | dynamic of operation |  | resulting dynamic [bit] |  |
| --- | --- | --- | --- | --- |
| ##### X | even column | odd column | even column | odd column |
| A = T4 X | 9 + 2 | 9 + 2.6 | 11 | 11.6 |
| C = B T8' | 11.0 + 6.7 | 11.6 + 6.7 | 17.6 | 18.3 |
| B = A >> n0 | 17.6 - 2 | 18.3 - 4 | 15.6 | 14.3 |
| D = q C | 15.6 + 13.4 | 14.3 + 12.8 | 29.0 | 27.1 |
| E = D >> n1 | 29.0 – 19.0 | 27.1 – 17.0 | 10.0 | 10.1 |
| F = r E | 10.0 + 3.2 | 10.1 + 3.5 | 13.2 | 13.6 |
| G = F T8 | 13.2 + 3.7 | 13.6 + 3.7 | 16.9 | 17.3 |
| H = G >> n2 | 16.9 - 2 | 17.3 – 2 | 14.9 | 15.3 |
| I = Tinv4' H | 14.9 - 0.3 | 15.3 – 0.3 | 14.6 (15) | 15.0 (15) |
| J = I >> n3 | 14.6 - 6 | 15.0 - 9 | 8.6 | 9.0 |

As can be seen by applying the argumentation from the 8x8 blocks, both 4x8 and 8x4 suffice the 16-bit memory and multiplication constraints.

# Intra16x16 with ABT

The accompanying modified version of JVT-E022d7 contains the changes to the specification to allow for the Intra16x16 mode to be used when ABT is on.

The changes are

- The indication of the ABT intra block types via  ABTIntra mb_types is removed

- mb_type Intra_4x4 indicates usage of ABT intra for I, P, and B slices when MbABTFlag is 1

- Intra_16x16 can be chosen also for I slices

- intra_block_typeABT is signaled for I, P, and B slices to indicated the ABT intra block type

The missing possibility of having a 16x16 based prediction in the FCD ABT intra specification introduced some weird artifacts, especially on intra coded HD material (thanks again to Teruhiko Suzuki who pointed that out). Allowing for Intra_16x16 in addition to intra coding with the ABT intra block types solves this problem and clearly improves the overall RD performance for ABT Intra on this material. An example for the improved subjective performance is given in the figures below. RD results are given at the end of this document.

(a) Original part of the ShuttleStart sequence		(b) Encoded with JM42 ABT on, QP36

(c) Encoded with JM42 ABT off, QP36	      (d) Encoded with JM42 ABT on +Intra16x16, QP36

Figure 1: Intra coding: Detail of the first frame of the ShuttleStart sequence.

# Harmonized intra prediction modes

For NxM blocks, a pre-filtered prediction is employed. In the FCD, the intra prediction for ABT NxM blocks slightly differed from the specification of the intra prediction for 4x4 blocks. This is mainly due to differences in the rounding operation in the prediction step. Here, we propose to modify the ABT intra prediction make the design for all block sizes more consistent. For this purpose, the intra prediction as specified for 4x4 blocks are extended directly for the most prediction modes. The DC prediction remains unchanged from the FCD.

The accompanying modified version of JVT-E022d7] contains the changes to the specification for the proposed intra prediction in sub clause 12.4.1. Besides prediction modes 0 (vertical), 1 (horizontal), and 7 (vertical right), the base line prediction technique is extended for the larger block sizes. For each of these modes, the extended 4x4 prediction results in almost exactly the same rate / distortion performance as the corresponding intra prediction technique specified in the FCD.

For vertical and horizontal prediction on 4x4 blocks no filtering is applied to the prediction samples. It turned out that for 8x8 blocks filtering of the prediction samples improves the prediction performance compared to the non-filtered prediction. Therefore, filtering is applied for vertical and horizontal prediction on 8x8 blocks. Blocks of size 4x8 and 8x4 use non-filtered prediction.

For 4x8, 8x4, and 8x8 blocks, the vertical right prediction as specified for 4x4 blocks does not perform as well as the vertical right prediction as specified in sub clause 12.4.1.8 of the FCD. Here, the rounding is modified from

( ( ti + ti+1<<1 + ti+2 + 2 ) >> 2  +  ( ti+1 + ti+2<<1 + ti+3 + 2 ) >> 2 ) >> 1

to

( ti + 3ti+1 + 3ti+2 + ti+3 + 4 ) >> 3

in order to reduce the number of operations.

# Simulation results

Simulation results are reported for the QCIF/CIF and SD common conditions test sequences as well as for a set of HD 720p and HD 1088i sequences. JM42 was used for the simulations. For all sequences results are given for the original JM42 software and JM42 with ABT+Intra16x16 and the harmonized Intra prediction modes. For the QCIF/CIF sequence a comparison is given also for the harmonized intra prediction modes only.

All sequences were encoded Intra only with CABAC and RD optimization on. The QCIF/CIF sequences were encoded at 10Hz (Container, Foreman, News), 15Hz (Silent, Paris), and 30Hz (Mobile, Tempete). All other sequences were assumed to be coded at 30Hz for rate calculation.

Table 1: HD test sequences

| Sequence | Size | Number of Frames |
| --- | --- | --- |
| BigShips | 1280x720p | 90 |
| City | 1280x720p | 90 |
| Crew | 1280x720p | 90 |
| Raven | 1280x720p | 90 |
| ShuttleStart | 1280x720p | 90 |
| Fireworks | 1920x1088i | 60 |
| Flamingo | 1920x1088i | 60 |
| Kayak | 1920x1088i | 60 |
| Mountain | 1920x1088i | 60 |

Table 2: BDSNR measurement for JM40d ABT vs JM40d harmonized ABT intra prediction, QCIF/CIF Common Conditions

| Common Conditions CIF/QCIF |  |  |
| --- | --- | --- |
|  | Rate | PSNR |
| container | 0.06% | -0.004 dB |
| foreman | -0.09% | 0.006 dB |
| news | -0.03% | 0.002 dB |
| silent | -0.03% | 0.002 dB |
| paris | 0.03% | -0.003 dB |
| mobile | 0.00% | -0.002 dB |
| tempete | -0.02% | 0.001 dB |

Table 3: BDSNR measurement for JM42 ABT vs JM42 ABT+Intra16x16 (harmonized)

QCIF/CIF Common Conditions

| CC | ABT off / ABT |  | ABT off / ABT + Intra16x16 |  |
| --- | --- | --- | --- | --- |
|  | Rate | PSNR | Rate | PSNR |
| Container | 1.00% | -0.067 dB | -2.80% | 0.185 dB |
| Foreman | -5.88% | 0.367 dB | -6.81% | 0.421 dB |
| News | -2.08% | 0.182 dB | -2.05% | 0.179 dB |
| Silent | -2.85% | 0.230 dB | -4.13% | 0.333 dB |
| Paris | -1.50% | 0.113 dB | -2.59% | 0.198 dB |
| Mobile | -5.21% | 0.296 dB | -6.45% | 0.364 dB |
| Tempete | -3.96% | 0.283 dB | -3.64% | 0.260 dB |

Table 4: BDSNR measurement for JM42 ABT vs JM42 ABT +Intra16x16 (harmonized)

SD Common Conditions

| SD | ABT off / ABT |  | ABT off / ABT + Intra16x16 |  |
| --- | --- | --- | --- | --- |
|  | Rate | PSNR | Rate | PSNR |
| Bus | -3.13% | 0.183 dB | -4.61% | 0.270 dB |
| Canoa | -8.27% | 0.349 dB | -9.04% | 0.382 dB |
| F1 Car | -3.76% | 0.209 dB | -5.45% | 0.300 dB |
| Football |  |  | -7.45% | 0.355 dB |
| Rugby |  |  | -7.13% | 0.389 dB |
| Tempete |  |  | -6.63% | 0.424 dB |

Table 5: BDSNR measurement for JM42 ABT vs JM42 ABT +Intra16x16 (harmonized)

HD 720p sequences

| HD 720p | ABT off / ABT |  | ABT off / ABT + Intra16x16 |  |
| --- | --- | --- | --- | --- |
|  | Rate | PSNR | Rate | PSNR |
| Big Ships | -3.07 | 0.189 dB | -3.25 | 0.197 dB |
| Crew | -5.59 | 0.452 dB | -5.54 | 0.446 dB |
| City | -1.25 | 0.063 dB | -4.82 | 0.203 dB |
| Raven | -8.55 | 0.495 dB | -8.98 | 0.516 dB |
| Shuttle Start | -0.56 | 0.025 dB | -4.92 | 0.180 dB |

Table 6: BDSNR measurement for JM42 ABT vs JM42 ABT +Intra16x16 (harmonized)

HD 1088i sequences

| HD 1088i | ABT off / ABT |  | ABT off / ABT + Intra16x16 |  |
| --- | --- | --- | --- | --- |
|  | Rate | PSNR | Rate | PSNR |
| Fireworks | 0.24% | -0.011 dB | -0.42% | 0.041 dB |
| Flamingo | -7.11% | 0.463 dB | -7.18% | 0.469 dB |
| Kayak | -10.03% | 0.766 dB | -9.41% | 0.717 dB |

# Discussion

As can be seen from the plots comparing the RD performance of the harmonized intra prediction modes to the prediction modes as currently specified in the FCD, the harmonized modes perform almost exactly as the original prediction modes.

The subjective quality is largely improved by the combination of ABT + Intra16x16. The RD plots for JM42 reveal an improved RD performance for the proposed combination of up to about 4% delta rate savings. In some rare events, a slight RD performance loss is observed. However, the subjective quality gains remain visible.

## CIF / QCIF common conditions III

There are two sets of RD plots for the QCIF/CIF sequences. First, results are given for JM40d, comparing the original and the harmonized Intra prediction modes. Second, results are given for JM42.

### Current Intra prediction vs harmonized Intra prediction

container qcif JM40d ABT on III

QP     Rate	PSNR

40    69494	28.68

36   108199	31.43

32   165274	34.24

28   244885	37.11

container qcif JM40d ABT on III new ipred

QP     Rate	PSNR

40    69541	28.68

36   108247	31.43

32   165387	34.24

28   244998	37.11

foreman qcif JM40d ABT on III

QP     Rate	PSNR

40    59917	28.77

36    96282	31.15

32   155353	33.83

28   238624	36.75

foreman qcif JM40d ABT on III new ipred

QP     Rate	PSNR

40    59899	28.77

36    96252	31.16

32   155318	33.83

28   238513	36.75

news qcif JM40d ABT on III

QP     Rate	PSNR

40    82790	28.64

36   123648	31.41

32   185598	34.47

28   264426	37.65

news qcif JM40d ABT on III new ipred

QP     Rate	PSNR

40    82728	28.63

36   123584	31.41

32   185574	34.47

28   263948	37.64

silent qcif JM40d ABT on III

QP     Rate	PSNR

40    90247	28.55

36   148133	30.90

32   244553	33.49

28   387056	36.44

silent qcif JM40d ABT on III new ipred

QP     Rate	PSNR

40    90280	28.55

36   148099	30.90

32   244416	33.49

28   386966	36.44

paris cif JM40d ABT on III

QP     Rate	PSNR

40   617018	27.56

36   943632	30.39

32  1418710	33.41

28  2044954	36.59

paris cif JM40d ABT on III new ipred

QP     Rate	PSNR

40   617205	27.57

36   943281	30.38

32  1418899	33.41

28  2043966	36.58

mobile cif JM40d ABT on III

QP     Rate	PSNR

40  2202953	25.46

36  3360291	28.46

32  4985900	31.80

28  6937267	35.37

mobile cif JM40d ABT on III new ipred

QP     Rate	PSNR

40  2203437	25.46

36  3360745	28.45

32  4985600	31.81

28  6937399	35.37

tempete cif JM40d ABT on III

QP     Rate	PSNR

40  1157321	27.00

36  1892495	29.70

32  2982848	32.77

28  4406497	36.11

tempete cif JM40d ABT on III new ipred

QP     Rate	PSNR

40  1157163	27.00

36  1892052	29.70

32  2982330	32.77

28  4406097	36.11

### Original JM42 vs harmonized Intra prediction + Intra16x16

container qcif JM42 ABT0 III

QP     Rate	PSNR

40    70898	28.85

36   107959	31.48

32   163742	34.26

28   243819	37.12

container qcif JM42 ABT2 III

QP     Rate	PSNR

40    70035	28.75

36   108964	31.45

32   165602	34.27

28   244770	37.12

container qcif JM42 ABT2 new III

QP     Rate	PSNR

40    68018	28.91

36   105834	31.53

32   161780	34.35

28   241320	37.18

foreman qcif JM42 ABT0 III

QP     Rate	PSNR

40    66797	28.56

36   103946	31.08

32   161722	33.77

28   244936	36.72

foreman qcif JM42 ABT2 III

QP     Rate	PSNR

40    61710	28.80

36    98114	31.18

32   156745	33.84

28   239281	36.77

foreman qcif JM42 ABT2 new III

QP     Rate	PSNR

40    60338	28.81

36    97134	31.19

32   155670	33.84

28   238725	36.77

news qcif JM42 ABT0 III

QP     Rate	PSNR

40    87166	28.66

36   129410	31.45

32   189718	34.45

28   267054	37.64

news qcif JM42 ABT2 III

QP     Rate	PSNR

40    83597	28.67

36   124717	31.46

32   185833	34.46

28   264498	37.68

news qcif JM42 ABT2 new III

QP     Rate	PSNR

40    83118	28.75

36   124198	31.54

32   184879	34.52

28   262986	37.69

silent qcif JM42 ABT0 III

QP     Rate	PSNR

40    96675	28.39

36   157238	30.88

32   252764	33.41

28   395168	36.35

silent qcif JM42 ABT2 III

QP     Rate	PSNR

40    91503	28.53

36   148426	30.90

32   243861	33.47

28   387418	36.44

silent qcif JM42 ABT2 new III

QP     Rate	PSNR

40    89475	28.59

36   146096	30.89

32   242889	33.50

28   384846	36.44

paris cif JM42 ABT0 III

QP     Rate	PSNR

40   637090	27.56

36   960372	30.40

32  1436833	33.39

28  2065960	36.56

paris cif JM42 ABT2 III

QP     Rate	PSNR

40   625630	27.62

36   949821	30.41

32  1424743	33.44

28  2049191	36.60

paris cif JM42 ABT2 new III

QP     Rate	PSNR

40   617646	27.67

36   941197	30.46

32  1419690	33.46

28  2045799	36.63

mobile cif JM42 ABT0 III

QP     Rate	PSNR

40  2260619	25.45

36  3413847	28.43

32  5039168	31.75

28  7024647	35.38

mobile cif JM42 ABT2 III

QP     Rate	PSNR

40  2210250	25.50

36  3358716	28.48

32  4979559	31.82

28  6934186	35.40

mobile cif JM42 ABT2 new III

QP     Rate	PSNR

40  2211408	25.51

36  3360330	28.48

32  4982277	31.82

28  6936210	35.40

tempete cif JM42 ABT0 III

QP     Rate	PSNR

40  1249320	26.93

36  1996525	29.71

32  3082638	32.79

28  4508983	36.16

tempete cif JM42 ABT2 III

QP     Rate	PSNR

40  1179578	27.02

36  1909922	29.73

32  2997302	32.82

28  4419418	36.17

tempete cif JM42 ABT2 new III

QP     Rate	PSNR

40  1183478	27.04

36  1925728	29.76

32  3014006	32.83

28  4435371	36.18

## SD common conditions III

bus ccir JM42 ABT0 III

QP     Rate	PSNR

40  1921002	27.56

36  3095131	30.00

32  4985218	32.66

28  7800851	35.57

bus ccir JM42 ABT2 III

QP     Rate	PSNR

40  1966197	27.85

36  3190549	30.33

32  5130371	33.02

28  7930555	35.92

bus ccir JM42 ABT2 new III

QP     Rate	PSNR

40  1918266	27.89

36  3148939	30.36

32  5098160	33.04

28  7895018	35.93

canoa ccir JM42 ABT0 III

QP     Rate	PSNR

40  1958668	27.84

36  3366717	30.08

32  5770617	32.27

28 10163103	34.55

canoa ccir JM42 ABT2 III

QP     Rate	PSNR

40  1967375	28.29

36  3373328	30.45

32  5798258	32.62

28 10354372	34.91

canoa ccir JM42 ABT2 new III

QP     Rate	PSNR

40  1942188	28.34

36  3370990	30.50

32  5809931	32.63

28 10379890	34.92

f1 car ccir JM42 ABT0 III

QP     Rate	PSNR

40  1990977	27.61

36  3471060	30.07

32  5891832	32.85

28  9446232	35.92

f1 car ccir JM42 ABT2 III

QP     Rate	PSNR

40  2106565	27.89

36  3681592	30.53

32  6092931	33.33

28  9464036	36.26

f1 car ccir JM42 ABT2 new III

QP     Rate	PSNR

40  2037563	27.94

36  3625064	30.56

32  6048336	33.34

28  9439628	36.27

football ccir JM42 ABT0 III

QP     Rate	PSNR

40  1175802	29.69

36  1955718	31.77

32  3289803	34.06

28  5327785	36.60

football ccir JM42 ABT2 new III

QP     Rate	PSNR

40  1166041	30.01

36  1968034	32.14

32  3315174	34.45

28  5310012	36.98

rugby ccir JM42 ABT0 III

QP     Rate	PSNR

40  2003726	28.30

36  3366848	30.65

32  5591345	33.23

28  8900193	36.08

rugby ccir JM42 ABT2 new III

QP     Rate	PSNR

40  1990008	28.67

36  3381379	31.04

32  5629603	33.65

28  8859887	36.51

tempete ccir JM42 ABT0 III

QP     Rate	PSNR

40  2606163	26.72

36  4173911	29.35

32  6703029	32.26

28 10389313	35.30

tempete ccir JM42 ABT2 new III

QP     Rate	PSNR

40  2609623	27.20

36  4219685	29.86

32  6710007	32.67

28 10271979	35.57

## HD 720p III

BigShips JM42 ABT0 III

QP     Rate	PSNR

32  9661048	34.86

28 16252205	37.32

24 25932776	40.10

20 40785749	43.12

BigShips JM42 ABT2 III

QP     Rate	PSNR

32  9363496	34.90

28 15655083	37.35

24 25099744	40.01

20 39859832	43.23

BigShips JM42 ABT2 new III

QP     Rate	PSNR

32  9261797	34.94

28 15645536	37.37

24 25172411	40.01

20 39982555	43.22

City JM42 ABT0 III

QP     Rate	PSNR

32 13667323	34.14

28 21467405	37.00

24 31721595	40.10

20 45759941	43.37

City JM42 ABT2 III

QP     Rate	PSNR

32 13148283	34.26

28 20649896	37.17

24 30675477	40.24

20 44047432	43.66

City JM42 ABT2 new III

QP     Rate	PSNR

32 13133800	34.28

28 20669091	37.17

24 30696776	40.23

20 44053656	43.66

Crew JM42 ABT0 III

QP     Rate	PSNR

32  4247896	37.66

28  7311627	39.53

24 12805691	41.66

20 22349819	44.13

Crew JM42 ABT2 III

QP     Rate	PSNR

32  4236235	37.68

28  7192019	39.56 no PM

24 12691389	41.62

20 22206027	44.33

Crew JM42 ABT2 new III

QP     Rate	PSNR

32  4018008	37.79

28  6954520	39.61

24 12405435	41.63

20 21865893	44.31

Raven JM42 ABT0 III

QP     Rate	PSNR

32  5761467	37.77

28  9246755	40.21

24 14419611	42.72

20 22468384	45.18

Raven JM42 ABT2 III

QP     Rate	PSNR

32  5678405	38.03

28  8784592	40.45

24 13579264	42.87

20 21295483	45.47

Raven JM42 ABT2 new III

QP     Rate	PSNR

32  5531781	38.06

28  8711037	40.45

24 13595021	42.85

20 21377005	45.46

ShuttleStart JM42 ABT0 III

QP     Rate	PSNR

32  2154320	40.03

28  3787136	41.83

24  6608731	43.81

20 11362616	45.78

ShuttleStart JM42 ABT2 III

QP     Rate	PSNR

32  2212013	39.92

28  3739320	41.83

24  6451648	43.74

20 11286877	45.96

ShuttleStart JM42 ABT2 new III

QP     Rate	PSNR

32  2071827	40.08

28  3627128	41.92

24  6347709	43.77

20 11098691	45.94

## HD 1088i III

fireworks JM42 ABT0 III

QP     Rate	PSNR

32 68748456	31.54

28 102036272	34.52

24 147941816	37.54

20 210184640	41.37

fireworks JM42 ABT2 III

QP     Rate	PSNR

32 69282220	31.62

28 103086556	34.60

24 151924884	37.72

20 213981036	41.68

fireworks JM42 ABT2 new III

QP     Rate	PSNR

32 69058496	31.63

28 102817516	34.62

24 151553872	37.78

20 213273740	41.66

flamingo JM42 ABT0 III

QP     Rate	PSNR

32 22109536	34.34

28 34820756	37.07

24 53656296	39.74

20 82256340	42.60

flamingo JM42 ABT2 III

QP     Rate	PSNR

32 22197284	34.80

28 34395804	37.48

24 52820496	40.09

20 80679988	42.93

flamingo JM42 ABT2 new III

QP     Rate	PSNR

32 22262816	34.83

28 34458232	37.49

24 52844376	40.10

20 80652772	42.94

kayak JM42 ABT0 III

QP     Rate	PSNR

32 28959260	33.53

28 45762152	36.37

24 67656068	39.19

20 99152780	42.27

kayak JM42 ABT2 III

QP     Rate	PSNR

32 28727996	34.10

28 45068448	37.02

24 67139224	39.91

20 97909944	43.10

kayak JM42 ABT2 new III

QP     Rate	PSNR

32 28988532	34.14

28 45791948	37.04

24 67421364	39.92

20 98163176	43.10

mountain JM42 ABT0 III

QP     Rate	PSNR

32 16114992	33.56

24 47804120	38.78

20 77554932	41.82

mountain JM42 ABT2 III

QP     Rate	PSNR

20 77369660	41.75

mountain JM42 ABT2 new III

QP     Rate	PSNR

32 16497616	33.57

28 29337500	35.98

24 48431660	38.70

20 77585952	41.65

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Aachen University |  |  |
| Mailing address | Institut für Nachrichtentechnik RWTH Aachen 53056 Aachen |  |  |
| Country | Germany |  |  |
| Contact person | Mathias Wien |  |  |
| Telephone | ++49-241-80-27681 |  |  |
| Fax | ++49-241-80-22196 |  |  |
| Email | wien@ient.rwth-aachen.de |  |  |
| Place and date of submission | Geneva, October 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Clean-up and improved design consistency for ABT |  |
| Contribution number |  | JVT-E025 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| x | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| x | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  | Robert Bosch GmbH |  |  |
| Mailing address |  | PO Box 106050 70049 Stuttgart |  |  |
| Country |  | Germany |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: