| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E029 Filename: JVT-E029.doc |
| --- | --- |

| Title: | Modification to the Intra Prediction Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shijun Sun Sharp Labs of America 5750 NW Pacific Rim Blvd Camas, WA 98607, USA | Tel: Email: | 360-817-7531 ssun@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

- Introduction

According to FCD section 9.1.5.1, the intra prediction mode number for a 4x4 block is first signaled by a parameter use_most_probable_mode. This parameter is represented by 1 bit codeword and can take values 0 or 1. If use_most_probable_mode is equal to 1 the most probable mode is used. Otherwise an additional parameter remaining_mode_selector, which can take value from 0 to 7 is sent as 3 bit codeword.

For an Intra MB, the intra prediction mode numbers are coded block-by-block following the block order shown in FCD Figure 9-1-b (shown as Figure 1 below).

Figure 1. Order of intra prediction information in the bitstream (same as FCD Figure 9-1-b).

The current block-based arrangement of intra mode coding in the bitstream is not efficient especially for the decoder implementation because the decoder has to decide to take either 1 or 4 bits block-by-block for each of the 16 4x4 blocks. A simpler MB-based coding method is proposed as the following.

- Proposal

For an Intra MB, first, a 16-bit use_most_probable_mode_map is coded with each bit representing the use_most_probable_mode of a corresponding block following the same block order shown in Figure 1. For example, the 5th bit of use_most_probable_mode_map gives the use_most_probable_mode for block “5” in Figure 1. The use_most_probable_mode_map is then followed by the 3-bit remaining_mode_selector for each block, which has zero-value use_most_probable_mode, following the same block order shown in Figure 1.

The proposed method enables easier decoder implementations for bitstream parsing. Since it is simply a reshuffle to the bits of the original method, there is absolutely no impact on the coding efficiency.

- Proposed changes to FCD

3.1 The second paragraph of Section 9.1.5.1 shall be modified as:

To signal prediction modes for a Intra_4x4 MB, first, a 16-bit use_most_probable_mode_map parameter is transmitted with each bit representing the use_most_probable_mode of a corresponding block following the block order shown in Figure 9-1-b. Following the same block order in Figure 9-1-b, if use_most_probable_mode of a block is equal to 1, the most probable mode is used for that block.  Otherwise, an additional remaining_mode_selector parameter, which can take value from 0 to 7 is sent as 3 bit codeword, which is a binary representation of remaining_mode_selector value. And the prediction mode number of the block is calculated as:

  - Caption of Figure 9-1-b shall be modified as: Block order for intra prediction mode coding.

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sharp Labs of America |  |  |
| Mailing address | 5750 NW Pacific Rim Blvd Camas, WA 98607, |  |  |
| Country | USA |  |  |
| Contact person | Shijun Sun |  |  |
| Telephone | 360-817-7531 |  |  |
| Fax | 360-817-8436 |  |  |
| Email | ssun@sharplabs.com |  |  |
| Place and date of submission | Geneva, Oct. 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | # Modification to the Intra Prediction Mode Coding |  |
| Contribution number |  | JVT-E029 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  | Provisional |  |
| Inventor(s)/Assignee(s) |  | Shijun Sun |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  | To support a royalty-free baseline we follow ITU-T 2.2.1 for Baseline Profile decoder applications agreeing to a free license on condition that all other patent holders do the same. |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: