| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E074 Filename: JVT-E074.doc |
| --- | --- |

| Title: | Proposal of HDTV materials for H.264¦MPEG-4 AVC verification test |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki, Kazushi Sato and Yoichi Yagasaki Gate City Osaki East Tower 20F 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032, Japan | Tel: Email: | +81-3-5435-3669 teruhiko@av.crl.sony.co.jp kazushi@av.crl.sony.co.jp yagasaki@av.crl.sony.co.jp |
| Source: | Sony Corp. |  |  |

_____________________________

# Introduction

In Klagenfurt, AHG on coding efficiency analysis and testing of AVC was establish to evaluate the performance of H.264| MPEG-4 AVC. In this contribution, we propose to test HDTV (both 1080i and 720p) materials. Since the standard supports those picture sizes, the test should be performed using those HDTV materials. We can provide some 1080i materials for this test.

In this contribution, preliminary encoding results for those materials are reported. Test materials and conditions should be discussed in Geneva meeting.

# New 1080i Materials

1080i test materials were distributed from Sony for the experiments within AHG on Level Constraints. Since those are good materials to evaluate the performance of codec, we would like to provide them for the experiments for AHG on coding efficiency analysis and testing of AVC.

The materials are 1080@30i captured by CCD camera and stored with NO compression by 1-inch VTR.

The resolution of the materials is very high. We should add those high resolution sequences as well as film scanned materials to cover wider range of materials.

The list of test materials is as follows.

| Name | Number of frames | Description |
| --- | --- | --- |
| Fireworks | 60 | Detail texture, Random motion |
| Kayak | 60 | Detail texture, Fast motion |
| Flamingo | 60 | Detail texture, Random motion |
| Mountain | 60 | Slow panning, Many trees |
| Train | 60 | Panning, Flat shade area |
| School | 60 | Zoom up, Texture, Dhade area |
| Asakusa Shop | 60 | Detail texture, |
| Running | 60 | Panning, Fast motion |

# Preliminary Encoding Results

We preliminary encoded those materials and report the results in this document.

## Test Conditions

Software: JM39a

Entropy coding: CABAC

Fi/Fr: Frame fixed

Search Range +-32

ABT: Off

RDOpt: On

## Results

|  | QP | SNRY (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- | --- |
| Fireworks | 28 | 32.760000 | 34.300000 | 34.120000 | 80754.53 |
| Kayak | 28 |  |  |  |  |
| Flamingo | 28 |  |  |  |  |
| Mountain | 28 | 35.77 | 40.50 | 43.56 | 10404.05 |
| Train | 28 | 39.86 | 41.84 | 45.51 | 2919.66 |
| School | 28 | 39.25 | 44.90 | 46.84 | 2109.51 |
| Asakusa Shop | 28 | 32.19 | 37.24 | 36.86 | 24435.15 |
| Running | 28 | 36.25 | 41.74 | 42.58 | 20417.18 |

Table 1: Results of 1080i materials

# Preliminary Encoding Results

In addition to the 1080i materials, we also tested 720p materials which were distributed by MPEG video Gp. In this section, we report the results.

## Test Conditions

Software: JM41b

Entropy coding: CABAC

Fi/Fr: Field fixed

Search Range +-32

ABT: Off

RDOpt: On

## Results

|  | QP | SNRY (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- | --- |
| BigShips | 28 | 36.46 | 40.66 | 42.32 | 4040.34 |
| City | 28 | 35.79 | 43.15 | 43.62 | 5350.4 |
| Crew | 28 | 38.54 | 44.10 | 43.86 | 3373.14 |
| Cyclists | 28 | 38.72 | 45.09 | 46.18 | 2670.00 |
| Harbour | 28 | 35.52 | 42.22 | 43.90 | 11514.4 |
| Jets | 28 | 39.90 | 45.08 | 46.66 | 1180.12 |
| Night | 28 | 36.55 | 39.69 | 41.28 | 8397.24 |
| Optis | 28 | 36.79 | 45.79 | 46.36 | 4362.42 |
| Raven | 28 | 39.31 | 43.77 | 42.51 | 3029.54 |
| Sailormen | 28 | 35.67 | 38.95 | 40.57 | 7500.74 |
| Sheriff | 28 | 37.16 | 43.80 | 45.04 | 5194.32 |
| Shuttle Start | 28 | 41.19 | 46.78 | 48.05 | 1024.72 |

Table 2: Results of 720p materials

# Conclusions

H.264 | MPEG-4 AVC supports HDTV and the performance of codec should be tested using HDTV as well as lower resolution video. In this contribution, new 1080i materials are proposed for the verification of the standard. The standard should be tested by various type of video. Those materials should be included in the test sequences.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sony Corp. |  |  |
| Mailing address | Gate City Osaki East Tower 20F 1-11-1 Osaki, Shinagawa-ku, Tokyo, 141-0032 |  |  |
| Country | Japan |  |  |
| Contact person | Teruhiko Suzuki |  |  |
| Telephone | +81-3-5435-3669 |  |  |
| Fax | +81-3-5435-3891 |  |  |
| Email | teruhiko@av.crl.sony.co.jp |  |  |
| Place and date of submission | Tokyo, Japan, Oct. 4th, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Proposal of HDTV materials for H.264¦MPEG-4 AVC verification test |  |
| Contribution number |  | E074 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| X | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: