| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, Switzerland, 9-17 October, 2002 | Document: JVT-E083 Filename: JVT-E083.doc |
| --- | --- |

| Title: | Results on MC Memory Analysis Core Experiment |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Antti Hallapuro, Jani Lainema, Marta Karczewicz Nokia Research Center 6000 Connection Drive Irving, TX 75039, USA | Tel: Email: | +1 972 894 4045 antti.Hallapuro@nokia.com |
| Source: | Nokia Inc. |  |  |

_____________________________

# Introduction

This document reports Nokia’s results on the Motion Compensation Memory Analysis core experiment. Results for three cases are given:

Case 1: 4/3-tap filters for bi-predicted MBs (as proposed by Nokia in JVT-D029 [1])

Case 2: Case 1 combined with 2-tap filters for 8x4, 4x8 and 4x4 blocks (as proposed by Mitsubishi in JVT-D110 [2])

Case 3: Case 1 combined with 4-tap filters for 8x4, 4x8 and 4x4 blocks (as proposed by Sony in JVT-D080 [3], except for the 8x8 blocks)

# Method description

Case 1 is based on Nokia’s proposal JVT-D029 [1]. The filter taps are slightly modified to guarantee full 16-bit implementation. The taps used for full-pixel, 1/4-pixel, 1/2-pixel and 3/4-pixel positions, respectively are:

[ 0, 8,  0,  0], [-1, 7,  2, 0], [-1,5,5,-1], [0,  2, 7, -1]

That is, the sub-pixel values for bi-predicted blocks are obtained by filtering with either 4-tap or 3-tap filters in two directions. The worst-case complexity of interpolation occurs when 4x4 prediction mode is chosen and motion vector points to one of the five middle sub-pixel positions. In this case he number of multiplications when using 6-tap filter is (9*4+4*4)*6 = 312. That is, interpolation of one 4x4 block requires 9*4 horizontal filter operations and 4*4 vertical filter operations and each filter operation requires 6 multiplications (and 6 loads and 6 additions). For 4-tap filter, the number of multiplications is reduced to (7*4+4*4)*4 = 176. Thus, the complexity reduction compared to the 6-tap filter is 44%. For 8x8 prediction mode the number of multiplications in the worst case is (13*8+8*8)*6 = 1008 when using 6-tap filter. Replacing this with 4-tap filter will reduce the number of multiplications to (11*8+8*8)*4 = 608. Therefore, reduction in complexity of the filter kernel is 40%.

In some systems memory traffic rather than computational complexity is a critical constraint for the decoder. Typically, in such systems a memory block is loaded from external memory into internal memory of the processor and the memory block is subsequently used for motion compensation. The proposed method reduces such memory traffic by up to 40% and 28% for 4x4 and 8x8 blocks, respectively.

# PSNR results

Test conditions for MC Memory Analysis CE were followed, Inter ABT was used for both HHR sequences. Tables below summarize the Bjontegaard Delta PSNR / Delta Bitrate by the three cases comparing to JM 4.0d. Attached spreadsheets JVT-E083-IPPP.xls and JVT-E083-IBBP.xls contain full RD-curves and exact results for the test.

Case 1: Nokia

|  |  | IBBP |  | IPPP |  |
| --- | --- | --- | --- | --- | --- |
|  | Resolution | PSNR[dB] | BR[%] | PSNR[dB] | BR[%] |
| Foreman | QCIF | -0.01 | 0.26 | 0.00 | 0.00 |
| Container | QCIF | -0.05 | 1.04 | 0.00 | 0.00 |
| Silent | QCIF | -0.01 | 0.25 | 0.00 | 0.00 |
| News | QCIF | -0.02 | 0.29 | 0.00 | 0.00 |
| Paris | CIF | -0.01 | 0.16 | 0.00 | 0.00 |
| Mobile | CIF | -0.07 | 1.55 | 0.00 | 0.00 |
| Tempete | CIF | -0.02 | 0.42 | 0.00 | 0.00 |
| Bus | HHR | 0.01 | -0.13 | 0.00 | 0.00 |
| Canoa | HHR | 0.00 | 0.09 | 0.00 | 0.00 |
| Average |  | -0.02 | 0.44 | 0.00 | 0.00 |

Case 2: Combined Nokia + Mitsubishi

|  |  | IBBP |  | IPPP |  |
| --- | --- | --- | --- | --- | --- |
|  | Resolution | PSNR[dB] | BR[%] | PSNR[dB] | BR[%] |
| Foreman | QCIF | -0.01 | 0.26 | 0.01 | -0.25 |
| Container | QCIF | -0.08 | 1.51 | -0.04 | 0.94 |
| Silent | QCIF | 0.02 | -0.40 | 0.01 | -0.17 |
| News | QCIF | -0.09 | 1.57 | -0.04 | 0.70 |
| Paris | CIF | -0.07 | 1.39 | -0.04 | 0.64 |
| Mobile | CIF | -0.15 | 3.34 | -0.07 | 1.38 |
| Tempete | CIF | -0.09 | 2.24 | -0.04 | 0.97 |
| Bus | HHR | -0.06 | 1.25 | -0.08 | 1.48 |
| Canoa | HHR | 0.02 | -0.63 | 0.04 | -1.19 |
| Average |  | -0.06 | 1.17 | -0.03 | 0.50 |

Case 3: Combined Nokia + Sony

|  |  | IBBP |  | IPPP |  |
| --- | --- | --- | --- | --- | --- |
|  | Resolution | PSNR[dB] | BR[%] | PSNR[dB] | BR[%] |
| Foreman | QCIF | -0.03 | 0.62 | -0.02 | 0.41 |
| Container | QCIF | -0.10 | 1.89 | -0.04 | 0.77 |
| Silent | QCIF | -0.04 | 0.68 | -0.03 | 0.56 |
| News | QCIF | -0.07 | 1.14 | -0.05 | 0.83 |
| Paris | CIF | -0.05 | 0.90 | -0.04 | 0.67 |
| Mobile | CIF | -0.10 | 2.15 | -0.03 | 0.61 |
| Tempete | CIF | -0.04 | 0.89 | -0.02 | 0.47 |
| Bus | HHR | -0.03 | 0.64 | -0.04 | 0.77 |
| Canoa | HHR | -0.01 | 0.28 | -0.01 | 0.28 |
| Average |  | -0.05 | 1.02 | -0.03 | 0.60 |

# Conclusions

The compression efficiency of the three cases studied in this document are very close to that of JM’s while providing significant reductions in worst case complexity and memory bandwidth. We ask the group to consider all of these and adopt the case the group finds the best trade-off between compression efficiency and complexity.

# References

[1] Antti Hallapuro, Jani Lainema, Marta Karczewicz, Nokia Inc., "4-Tap Filter for Bi-predicted Macroblocks", document JVT-D029, Joint Video Team (JVT) Meeting, Klagenfurt, AT, 22-26, July 2002.

[2] Shun-ichi Sekiguchi, Yoshihisa Yamada, Kohtaro Asai, Mitsubishi Electric Corporation, "Block-size Adaptive Motion Compensation for Complexity Reduction ", document JVT-D110, Joint Video Team (JVT) Meeting, Klagenfurt, AT, 22-26, July 2002.

[3] Kazushi Sato, Teruhiko Suzuki, Yoichi Yagasaki, Sony Corp., "Adaptive MC Interpolation for Complexity Reduction", document JVT-D080, Joint Video Team (JVT) Meeting, Klagenfurt, AT, 22-26, July 2002.

[x] Thomas Wiegand, "Joint Committee Draft (CD)", document JVT-C167, Joint Video Team (JVT) Meeting, Fairfax, VA, USA, 6-10, May 2002.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Inc. |  |  |
| Mailing address | 6000 Connection Drive, Irving, TX 75039 USA |  |  |
| Country | USA |  |  |
| Contact person | Marta Karczewicz |  |  |
| Telephone | +1 972 894 4188 |  |  |
| Fax |  |  |  |
| Email | marta.karczewicz@nokia.com |  |  |
| Place and date of submission | Geneva, Switzerland, 10/7/2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Results on MC Memory Analysis Core Experiment |  |
| Contribution number |  | JVT-E083 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: