| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E090 Filename: JVT-E090.doc |
| --- | --- |

| Title: | Editorial corrections on FCD regarding B-picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal (editorial only) |  |  |
| Author(s) or Contact(s): | Yoshihiro Kikuchi Takeshi Chujoh | Tel: Email: | +81 44 549 2288 yoshihiro.kikuchi@toshiba.co.jp |
| Source: | Toshiba |  |  |

_____________________________

#### Summary

This contribution proposes editorial corrections to the JVT FCD related to the B-picture. The items to be corrected are (1) weighted prediction generation procedure for the implicit bi-predictive weighting and (2) motion vector decoding with scaled MV. The proposed texts in this contribution are relative to JVT-E022d7[1]. We request these items are reviewed and reflected on the revised FCD and the AAP document.

# Implicit bi-predictive weighting (10.4.3)

Problem:

The condition to switch between the two weighting coefficients is incorrect.

Solution:

In subclause 10.4.3 (Implicit bi-predictive weighting), the second paragraph, change:

“If the decoding order of the reference picture indicated by ref_idx_l0 is previous to or the same as that indicated by ref_idx_l1, or for skipped macroblocks or direct mode, the prediction signals are generated as follows:”

to

“If the decoding order of the reference picture indicated by ref_idx_l0 is later than that indicated by ref_idx_l1, the prediction signals are generated as follows:”

# Motion vector decoding with scaled MV (10.3.2)

Problem:

The description on the generation procedure of MV2 (the motion vector corresponding to ref_idx_l1) is not clear.

Solution:

In subclause 10.3.2 (Motion vector decoding with scaled MV), the last paragraph, change:

“Then, MV2 is differentially coded with respect to smv.”

to

“Then, MV2 is calculated by adding the differential motion vector mvd_l1 to the smv.”

Also, in Fig 10-2, change “ MV2-smv ” to “ mvd_l1 ”.

##### References

[1] Thomas Wiegand, “Editor's Proposed Draft Text Modifications for Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC), Draft 7”, JVT-E022d7.