| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E098 Filename: JVT-E098.doc |
| --- | --- |

| Title: | Complexity Reduction for ABT Intra Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chong-Soon Lim Block 1022 Tai Seng Avenue #04-3530 Singapore 534415 Teck-Wee Foo Block 1022 Tai Seng Avenue #04-3530 Singapore 534415 Sheng-Mei Shen Block 1022 Tai Seng Avenue #04-3530 Singapore 534415 Shinya Kadono 1006 Kadomo, Kadomo, Osaka 573-8501 Japan | Tel: Email: Tel: Email: Tel: Email: Tel: Email: | (65) 6550-5487 cslim@psl.com.sg (65) 6550-5474 twfoo@psl.com.sg (65) 6550-5466 shen@psl.com.sg +81-6-6900-9689 kadono@drl.mei.co.jp |
| Source: | Panasonic Singapore Laboratories Pte. Ltd. |  |  |

_____________________________

# Summary

The purpose of this proposal is to present a simpler Adaptive Block Intra Prediction for mode 0 and 1.  The current Intra Prediction for ABT mode 0 and 1 requires a 3-tap low-pass filtering to the prediction values.  We feel that this 3-tap low pass filtering is not necessary and it introduces additional complexity.  Therefore we would like to propose to remove the 3-tap low-pass filtering for these modes and use a simple vertical and horizontal intra prediction instead.

# Problem

The current ABT Intra Prediction for mode 0 and 1 are shown below.   These prediction modes require a low-pass filtering on the prediction values, which introduces additional complexity and does not provide much gain in compression.

Mode 0: vertical prediction

tk, k = 0 to (N-1) shall be available. Prediction sample q is denoted as t-1. If q is not in the slice, t0 substitutes q.

| block size |  | samples | predicted by |
| --- | --- | --- | --- |
| 4x8 | k = 0 to 7, i = 0 to 3 | pki | ( ti-1 + ti<<1 + ti+1 + 2) >> 2 |
| 8x4 | k = 0 to 3, i = 0 to 7 |  |  |
| 8x8 | k = 0 to 7, i = 0 to 7 |  |  |

Mode 1: horizontal prediction

lk, k = 0 to (M-1) shall be available. Prediction sample q is denoted as l-1. If q is not in the slice, l0 substitutes q.

| block size |  | samples | predicted by |
| --- | --- | --- | --- |
| 4x8 | k = 0 to 7, i = 0 to 3 | pki | ( li-1 + li<<1 + li+1 + 2 ) >> 2 |
| 8x4 | k = 0 to 3, i = 0 to 7 |  |  |
| 8x8 | k = 0 to 7, i = 0 to 7 |  |  |

# Simple ABT Intra Prediction Modes 0 and 1.

A simpler ABT Intra Prediction Mode 0 and 1 are presented below.  We remove the 3-tap low-pass filtering and use the direct vertical and horizontal intra prediction.

Mode 0: vertical prediction

tk, k = 0 to (N-1) shall be available. Prediction sample q is denoted as t-1. If q is not in the slice, t0 substitutes q.

| block size |  | samples | predicted by |
| --- | --- | --- | --- |
| 4x8 | k = 0 to 7, i = 0 to 3 | pki | ti |
| 8x4 | k = 0 to 3, i = 0 to 7 |  |  |
| 8x8 | k = 0 to 7, i = 0 to 7 |  |  |

Mode 1: horizontal prediction

lk, k = 0 to (M-1) shall be available. Prediction sample q is denoted as l-1. If q is not in the slice, l0 substitutes q.

| block size |  | samples | predicted by |
| --- | --- | --- | --- |
| 4x8 | k = 0 to 7, i = 0 to 3 | pki | li |
| 8x4 | k = 0 to 3, i = 0 to 7 |  |  |
| 8x8 | k = 0 to 7, i = 0 to 7 |  |  |

# Results

Five SD size interlace sequence were tested using jm40d with and without low-pass filter for ABT Intra Prediction mode 0 and 1.  The results are tabulated as below.  All the sequences are coded using Intra Pictures with field coding.

| Sequence Name | DPSNR(Y) | %Bitrate |
| --- | --- | --- |
| Rugby | 0.027 dB | -0.48% |
| Mobile and Calendar | -0.001 dB | 0.00% |
| Harp_Piano | -0.006 dB | 0.09% |
| F1 | 0.129 dB | -2.01% |
| Canoe | 0.013 dB | -0.20% |

# Conclusion

The results do not show a significant gain in introducing a 3-tap low pass filter for the ABT Intra Prediction Modes 0 and 1.  In most cases, there is a gain without the low pass filter.  Therefore we would like the JVT group to consider removing this low-pass filter and use a simpler vertical and horizontal ABT Intra Prediction instead.

# References

[1]	Thomas Wiegand, “Editor's Proposed Draft Text Modifications for Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC), Draft 7”, JVT-E022d7, JVT Meeting, Geneva, Switzerland, October 2002.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Matsushita Electric Industrial Co., Ltd. |  |  |
| Mailing address | 1006 Kadoma, Kadoma, Osaka, 573-8501 |  |  |
| Country | Japan |  |  |
| Contact person | Shinya Kadono |  |  |
| Telephone | +81-6-6900-9689 |  |  |
| Fax | +81-6-6900-9674 |  |  |
| Email | kadono@drl.mei.co.jp |  |  |
| Place and date of submission | 5th Meeting: Geneva, CH, 9-17 Oct., 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Complexity Reduction for ABT Intra Prediction |  |
| Contribution number |  | JVT-E098 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: