| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E118 Filename: JVT-E118.doc |
| --- | --- |

| Title: | Verification of Alternate Scan for Non-ABT Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazushi Sato, Teruhiko Suzuki and Yoichi Yagasaki Gate City Osaki East Tower 20F 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032, Japan | Tel: Fax: Email: | +81-3-5435-3669 +81-3-5435-3891 kazushi@av.crl.sony.co.jp teruhiko@av.crl.sony.co.jp yagasaki@av.crl.sony.co.jp |
| Source: | Sony Corp. |  |  |

_____________________________

1. Introduction

This contribution is a verification of alternate scan for non-ABT coding proposed by JVT-D073.

2. Simulation Condition

The simulation conditions are shown in the table below.

| Codec | JM42 JM42 + alternate scan |
| --- | --- |
| Sequence | “Canoa” (720x576i) “Bus” (720x480i) “Tempete” (720x480i) |
| GOP | IBBPBB… (n=15;m=3) |
| QP | QPI=QPP=24,28,32,36,40; QPB=QPI+2 |
| EntropyCoding | CAVLC |
| Hadamard | On |
| NumberReferenceFrames | 2 |
| SearchRange | 16x16 |
| RestrictedSearchRange | 2 |
| ABT | Off |
| InterlaceCodingOption | Pict-level field/frame adaptive |

Table 2.1

3. Simulation Result

The simulation results are shown in the table blow.

|  | BD-PSNR [dB] | BD-Bitrate [%] |
| --- | --- | --- |
| Canoa | 0.156 | -3.00 |
| Bus | 0.084 | -1.78 |
| Tempete | -0.052 | 1.53 |

Table 3.1

4. Conclusion

In this contribution alternate scan for non-ABT coding proposed by JVT-D073 was verified. The simulation results show that by employing the proposed method coding efficiency gain by up to 3% will be obtained.

References

[1] Editor's Proposed Draft Text Modifications for Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC), Draft 7, T. Wiegand, JVT-E022d7, October, 2002

[2] “Alternate Scan for Interlaced Coding” B. Jeon et al, JVT-D073, July 2002

[3] “New Interlace Coding Tools”, K. Sato, T. Suzuki and Y. Yagasaki, JVT-B068r1, Feb 2002

[4] “Calculation of average PSNR difference between RD-curves”, G. Bjontegaard, VCEG-M33, March 2001

[5] “Recommended Simulation Conditions for H.26L”, G. Bjontegaard, VCEG-M75, May 2001