| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, Switzerland, 9-17 October, 2002 | Document: JVT-E131 Filename: JVT-E131.doc |
| --- | --- |

| Title: | Parameter Set Repetition SEI Messages |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Mobile Software P.O. Box 88 33721 Tampere Finland Ye-Kui Wang TICSP Tampere Univ. of Technology P.O. Box 553 FIN-33101 Tampere Finland | Tel: Email: | +358 7180 73151 miska.hannuksela@nokia.com +358 3 3115 3972 wyk@ieee.org |
| Source: | Nokia Corporation and Tampere University of Technology |  |  |

_____________________________

# Summary

We propose sequence and picture parameter set SEI messages to enable gradual decoder refresh and to improve error resiliency.

# Proposal

When tuning in to a broadcast or multicast video stream, decoders may start decoding and displaying from an IDR picture or they may start decoding and displaying from non-IDR pictures according to a received random access point SEI message. The latter case is referred to as gradual decoder referesh.

A received sequence parameter set comes into effect just before decoding the next IDR picture. Thus, if a sequence parameter set is updated or transmitted again to enable tuning in to a multicast stream, it must be sent before the IDR picture that refers to it. Consequently, if the reception of a stream is started for a non-IDR picture, the decoder cannot be sure if it has received a correct sequence parameter set in order to be able to start decoding from the non-IDR picture.

A solution to the problem is to transmit redundant copies of sequence parameter sets. A sequence parameter RBSP cannot be used to transmit redundant parameter set copies, because the sequence parameter set RBSP comes into effect just before decoding the next IDR picture. Instead, we propose a sequence parameter set repetition SEI message. The SEI message contains a redundant copy of a valid sequence parameter set.

The sequence parameter set repetition SEI message is an independent SEI message. It is not coupled with the random access point SEI message, because it can also be used for error resiliency in general. For example, the decoder can check if the sequence parameter set repetition is equivalent to the corresponding valid sequence parameter set. If these two parameter sets differ from each other, the decoder may infer an error.

As picture parameter sets become into effect picture by picture, the necessary picture parameter sets could be repeated for each gradual random access entry point. Consequently, no picture parameter set repetition SEI message would be needed for gradual decoder refresh or error resiliency. However, it may be beneficial to indicate that a particular picture parameter set NAL unit is a repetition of a valid picture parameter set and not a picture parameter set update. When video is conveyed over an error-free link, the decoder can then simply skip the repeated picture parameter sets. Thus, we propose a picture parameter set repetition SEI message.

Similarly to all other SEI messages, there is no mandatory decoder operation to process the parameter set repetition SEI messages. The decoder is recommended to copy a repeated parameter set immediately to an appropriate storage location mentioned in section 8.2.2 of the FCD.

# Changes relative to JVT-E022D7

## Section 7.3.2.1 (Sequence parameter set RBSP syntax)

All the parameters of the seq_parameter_set_rbsp excluding the rbsp_trailing_bits( ) function should be extracted to a separate syntax element called seq_parameter_set. The syntax for the seq_parameter_set_rbsp includes calls to seq_parameter_set( ) and rbsp_tralinig_bits( ).

## Section 7.3.2.2 (Picture parameter set RBSP syntax)

The syntax for picture parameter set is divided into the pic_parameter_set syntax element and the pic_parameter_set_rbsp syntax element.

## Section 8.2.2 (Parameter set decoding)

The decoder maintains 16 sequence parameter set locations. For each of the 16 possible values of seq_parameter_set_id, the most recent decoded sequence parameter set is copied into the location referenced by seq_parameter_set_id immediately before the decoding of the next IDR picture. A sequence parameter set residing in one of the 16 locations is referred to as a valid sequence parameter set.

The decoder maintains 64 picture parameter set locations. For each of the 64 possible values of pic_parameter_set_id, the most recent picture parameter set is copied into the location referenced by pic_parameter_set_id immediately before the decoding of a slice or DPA belonging to the next coded picture. A picture parameter set residing in one of the 64 locations is referred to as a valid picture parameter set.

## Section D.2.x (Sequence parameter set repetition SEI message syntax)

| seq_parameter_set_repetition( PayloadSize ) { | Category | Descriptor |
| --- | --- | --- |
| seq_parameter_set( ) | 7 |  |
| } |  |  |

## Section D.2.x (Picture parameter set repetition SEI message syntax)

| pic_parameter_set_repetition( PayloadSize ) { | Category | Descriptor |
| --- | --- | --- |
| pic_parameter_set( ) | 7 |  |
| } |  |  |

## Section D.3.x (Sequence parameter set repetition SEI message semantics)

The sequence parameter set repetition SEI message is used to repeat sequence parameter sets. It can be used to improve error resiliency or to allow random access from a non-IDR picture.

seq_parameter_set( ) shall contain a copy of a valid sequence parameter set.

## Section D.3.x (Picture parameter set repetition SEI message semantics)

The picture parameter set repetition SEI message is used to repeat picture parameter sets. It can be used to improve error resiliency or to allow random access from a non-IDR picture.

pic_parameter_set( ) shall contain a copy of a valid picture parameter set.

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Corporation |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  |  |  |
| Contribution number |  | JVT-E131 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| X | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  | The submitter is not aware of having any granted, pending, or planned patents associated with the proposed changes into the draft Recommendation ¦ Standard. |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. X |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: