| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E151 Filename: JVT-E151.doc |
| --- | --- |

| Title: | Parameter Set Issues |  |  |
| --- | --- | --- | --- |
| Status: | Output Document [not yet Approved by JVT] |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Stephan Wenger Teles AG / TU Berlin Sekr. FR 6-3 Franklinstr. 28-29 D-10587 Berlin, Germany | Tel: Email: | +49-172-3000813 stewe@cs.tu-berlin.de |
| Source: | Teles AG |  |  |

_____________________________

# Summary

The issue of redundant parameter sets was discussed in an informal breakout group among some interested parties, including Sony, Teles, TU München, and Nokia.  It was agreed that the easiest solution to the problems discussed in JVT-E131 (and summarized later in this document) is to remove the store/copy operation of parameter sets immediately before their relevant synchronization points.  It was noted that this feature was introduced in Fairfax as a response to the use of only one single parameter set – and it is, after Klagenfurt (where multiple parameter sets were re-introduced), neither necessary nor helpful.

# Problem Statement

Document JVT-E131 by Nokia suggested to add a scheme for the redundant transmission of picture and sequence parameter sets to the draft Recommendation.  The key motivation was in both cases that, as per the current draft, the parameter sets become effective at certain times synchronous with the NALU stream (e.g. a transmitted sequences parameter set becomes effective immediately before an IDR picture).  Such an “active” parameter set is copied to a location that cannot change during the GOP or picture.  This feature prevents the use of already accepted technologies such as “dirty” random access points and others, see JVT-E131.

While scheme suggested in JVT-E131 seem to be a reasonable solution to the problems stated, it also introduces additional codepoints, several new paragraphs to the text, and is not very simple to understand.  A cleaner and simpler design that solves the same problem seems to be desirable.

# Suggested Solution

We believe that the easiest way to address the problem is to remove the double-buffering feature of the current draft.  This can be done by deleting two sentences from the draft.  However, we also propose to add language that forbids the change of an active parameter set (otherwise we run in all kinds of compliance problems), and editorial notes on how to achieve this.

In the draft, the two paragraphs 7.4.2.1 and 8.2.2 are affected and copied below, with the suggested changes highlighted.

# Changes relative to JVT-E022d7

The following paragraphs are ready for being pasted into the draft:

#### 7.4.2.1 Sequence parameter set RBSP semantics

A sequence parameter set is called an active sequence parameter set when an IDR NAL unit refers to it. A sequence parameter set is said to be active if it was referenced by the most recently decoded IDR picture. The parameters of an active sequence parameter set shall be constant for the decoding of all slices belonging to the SOP.

NOTE: The above condition implies that an update of an active sequence parameter set is only possible by decoding a sequence parameter set RBSP immediately before the first slice of an IDR picture.  However, it is possible to decode redundant copies of the active sequence parameter set at any time.

NOTE - The sequence and picture parameter set mechanism decouples the transmission of infrequently changing information from the transmission of coded macroblock data. The sequence parameter set, the picture parameter set, and the slice header contain all the parameters needed to decode the slice data. It is recommended to convey sequence and picture parameter sets out-of-band using a reliable transport mechanism. In that case, an external means of synchronization should ensure that any active picture/sequence parameter is not overridden with differing content.

If an application requires a self-contained bitstream, in-band parameter set information units may be used. In error-prone transmission environments, in-band sequence and picture parameter set information units should be protected in a way that assures their successful reception. For in-band transmission, it is advisable that the sequence parameter set that becomes active at an IDR immediately be sent before that IDR.  It is also advisable to transmit all picture parameter sets required within a SOP immediately before the IDR picture that begins the SOP, or at the boundaries between pictures in the SOP.

The synchronization between in-band and out-of-band transmission of the sequence and picture parameter set information is outside of the scope of this Recommendation | International Standard.

#### 7.4.2.2 Picture parameter set RBSP semantics

A picture parameter set includes the parameters that remain unchanged within a coded picture. Every picture parameter set shall refer to the active sequence parameter set. A decoded picture parameter set is an active picture parameter set when the first slice NAL unit or first DPA NAL unit of a coded picture refers to it. The picture parameters of an active picture parameter set shall be constant for all NAL units belonging to the picture.

### 8.2.2 Parameter set decoding

The decoder maintains a number of sequence parameter set locations as specified in subclause QQQ of Annex A, and a number of picture parameter set locations as specified in subclause QQQ of Annex A.  (Edt. note: the values are 32 and 128 as per agreed Gary contribution).  Decoded sequence parameter sets are stored in the sequence parameter set location indicated by the seq_parameter_set_id immediately, i.e. before the next NAL unit is decoded.  Similarly, decoded picture parameter sets are stored in the picture parameter set location indicated by the picture_parameter_set_id immediately.

Also add to the Abbreviations Section:

SOP: Sequence of Pictures

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Teles AG |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| X | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: