| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 6th Meeting: Awaji, Island, JP, 5-13 December, 2002 | Document: JVT-F025 Filename: JVT-F025.doc |
| --- | --- |

| Title: | 8x8 Block Restriction to Spatial Direct Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexis Michael Tourapis†, Feng Wu, and Shipeng Li 3F Sigma Center, No.49 Zhichun Road, Haidian District, Beijing, 100080, China Lowell Winger‡ Videolocus, Inc. 97 Randall Dr Waterloo, Ontario, CANADA, N2V1C5 | Tel: | +86 (10) 6261-7711 ext 3108† +1 (519) 725 9797‡ |
|  |  | Email: | alexismt@ieee.org† lwinger@videolocus.com‡ |
| Source: | Microsoft Corporation, Videolocus |  |  |

_____________________________

The current FCD specifies (Table A-1) that it is required for Levels 3.1 that Direct and Bi-predictive blocks should have a minimum size of 8x8. Even though for Bi-predictive blocks such a restriction is rather straightforward since such can be controlled through the motion parameters transmitted, it is not as obvious for Direct mode since its mode structure relates to the mode structure of its co-located block in the first list1 reference picture. Even though a method to allow the Temporal Direct Mode to conform to such restriction was proposed in JVT-C115r3 and adopted in the standard, such a method has not yet been adopted for the Spatial Direct Mode thus in a sense disallowing the usage of this mode within such levels.

In this document, we extend the concept of JVT-C115r3 to also apply for the spatial direct mode, thus also allowing Spatial Direct Mode to conform to the requirements of Table A-1. In brief, similar to the temporal direct case, only motion information from the “edge” blocks within a macroblock (Figure 1) are considered during the zero partitioning process for selecting the motion parameters. Such process is done regardless to whether the direct prediction is single list or bi-predictive. We need to point out that an additional benefit of this method is the requirement to store only ¼ of the information for performing zero partitioning. Processing for zero partitioning can also be reduced by an equivalent amount.

Figure 1. Consideration of Motion from the Edge  blocks for zero partitioning of the Spatial Direct Mode

Since we propose using such limitation for both spatial and direct mode we also recommend changing the flag specified in the text from direct_temporal_constrained_flag to direct_prediction_constrained_flag or direct_constrained_flag. The text for this flag should be changed to:

direct_prediction_constrained_flag specifies the constrains applied  in the decoding process to determine the prediction values of direct prediction. Subclause 8.6.1.4.2 describes direct prediction and how the motion parameters are calculated based on the value of direct_prediction_constrained_flag.  If this flag is equal to 1 then 4x4, 4x8, and 8x4 partition sizes are not allowed in direct mode macroblocks.

- Furthermore the text for spatial direct should be changed as follows.

###### 8.6.1.4.2.2 Derivation process for spatial direct luma motion vector and reference picture index prediction mode

Input to this process is/are ??? [Ed.Note: specify input to this process]

Output of this process is/are ??? [Ed.Note: specify output of this process]

When direct_spatial_mv_pred_flag is equal to 1, spatial direct mode is used to derive the luma motion vectors and reference picture indices for skipped macroblocks and macroblocks with mb_type Direct_16x16 or macroblock partitions with sub_mb_type Direct_8x8.

The first step in the Spatial direct luma motion vector and reference picture index prediction mode is the determination of the  reference picture index for each list (list 0 and list 1), RefIdxL0[ i ] and  RefIdxL1[ i ]. The same process is used for macroblocks coded using the Direct_16x16 macroblock type, and sub-macroblocks coded using the Direct_8x8 sub-macroblock type.

The reference picture indices for the neighbouring blocks A, B, C, and D within the current slice for the current luma macroblock E, for both Direct_16x16 macroblock and Direct_8x8 sub-macroblock types, as described in subclause Error: Reference source not found and shown in Error: Reference source not found, shall be used to determine the reference picture index for each list.  As in 8.4.1.1, the reference index of block D is only used if block C is not available in the current slice. The reference picture index for each list (list 0 and list 1) shall be the minimum reference picture index among the reference picture indices used from the same list for the prediction or bi-prediction of the neighbouring blocks.  If no neighbouring blocks are present within the current slice that use prediction from the same list, either for prediction or bi-prediction, the reference index for that list shall be interpreted as not existing.

If the reference index exists for either list, decision of the associated motion vector values for that list for each 4x4 block of the current macroblock depends on the coded parameters of the co-located macroblock in the first picture in list 1, if both macroblocks are in frame or field mode. If direct_prediction_constrained_flag is equal to 0, then the co-located 4x4 block is used, otherwise, if direct_prediction_constrained_flag is set to 1, then the 4x4 corner block within the co-located 8x8 sub-macroblock that has the same decode number as its 8x8 sub-macroblock is used in place of the co-located 4x4 block.

If the current macroblock is coded in field mode and the co-located macroblock in the first list 1 picture in frame mode, only the same parity lines and their associated motion parameters of the co-located macroblock are considered. Otherwise, if current macroblock is in frame mode and the macroblock in the first list 1 picture is in field mode, then the corresponding frame mode picture containing the first list 1 picture is considered, if such exists. If direct_prediction_constrained_flag is set to 1 only information from the 4x4 corner blocks within each 8x8 sub-macroblock that has the same decode number as its 8x8 sub-macroblock is considered.

If the reference index of either list for the current block is zero, the associated list 1 picture is a short-term picture, and if all lines of the co-located 4x4 block exist and were predicted using list 0 prediction, reference picture index 0 and motion vector components in the range of -1 to 1 in sub-pixel units, inclusive, then the motion vector for this 4x4 block is set to (0, 0). Otherwise, the associated candidate motion vector for each list shall be obtained, using the 16x16 block motion vector prediction, as described in subclause Error: Reference source not found by using the reference picture index of each list.

If both candidate reference picture indices exist, then the block is predicted as a bi-prediction block using the corresponding reference picture index and associated motion vector for each list. Otherwise, if a reference picture index exists for only one of the two lists, the block shall be predicted by single-list prediction using the reference picture index and associated motion vector for the existing list. Finally, if neither reference picture index exists, bi-prediction shall be used with reference picture index zero and associated motion vector (0, 0) for both lists.

- Experimental Results

- We have evaluated the performance of the proposed partitioning method for several HD sequences but also on the common conditions used for lower resolution sequences. Our results demonstrate that the loss is essentially negligible at all resolutions. As it can be seen from Table 1, for the common condition sequences at QCIF and CIF resolutions the worse case is Container with 0.24% bitrate increase or equivalently 0.012dB loss. On the average this means a bitrate increase of 0.04% or loss of 0.002DB which is negligible. Furthermore, our results in  also demonstrate that the performance of the proposed scheme on higher resolution (from CCIR-601 SD material to higher resolution HD material) remains also unaffected, justifying further the use of the proposed 8x8 partitioning for all types of material.

- On an other note, our results on interlace sequences and with MB-AFF enabled also tend to suggest that conforming our current consideration of the co-located block to the one currently used for temporal direct  (consideration of 2*y block position for frame to field, and y/2 for field to frame) would have little or no impact in the performance of spatial direct mode. Even though the above text does not contain such an amendment, such could unify and simplify the concepts of the two direct modes. We recommend though that the co-located block is considered at the block level and not the macroblock level since such was never tested and could possibly have a rather negative impact on the performance of both temporal and spatial direct modes.

Table 1. Comparison of proposed 8x8 constrained spatial direct vs. original on Common Conditions

| sequences | frame rate | number of frames | QP | PSNRY (Spat) | Total Bits | PSNRY (Spat8) | Total Bits | Spat ave. change (excel) | Spat ave. dB gain (excel) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Foreman | 10 | 97 | 28 | 35.974 | 644704 | 35.974 | 644672 | 0.01% | -0.0004 |
|  |  |  | 32 | 33.109 | 388750 | 33.108 | 388728 |  |  |
|  |  |  | 36 | 30.388 | 231702 | 30.388 | 231734 |  |  |
|  |  |  | 40 | 27.871 | 140276 | 27.871 | 140276 |  |  |
| Container | 10 | 97 | 28 | 36.220 | 188824 | 36.221 | 189464 | 0.24% | -0.0122 |
|  |  |  | 32 | 33.437 | 102827 | 33.439 | 103286 |  |  |
|  |  |  | 36 | 30.686 | 60484 | 30.686 | 60559 |  |  |
|  |  |  | 40 | 28.142 | 37948 | 28.142 | 37948 |  |  |
| News | 10 | 97 | 28 | 36.895 | 404783 | 36.896 | 404954 | 0.01% | -0.0005 |
|  |  |  | 32 | 33.880 | 248677 | 33.880 | 248666 |  |  |
|  |  |  | 36 | 30.918 | 149230 | 30.918 | 149241 |  |  |
|  |  |  | 40 | 28.178 | 91091 | 28.178 | 91102 |  |  |
| Paris | 15 | 148 | 28 | 35.594 | 2839165 | 35.594 | 2839459 | 0.02% | -0.0008 |
|  |  |  | 32 | 32.489 | 1589251 | 32.489 | 1589711 |  |  |
|  |  |  | 36 | 29.640 | 865255 | 29.641 | 865657 |  |  |
|  |  |  | 40 | 26.966 | 472723 | 26.967 | 472733 |  |  |
| Silence | 15 | 148 | 28 | 35.943 | 518740 | 35.943 | 518495 | 0.02% | -0.0007 |
|  |  |  | 32 | 33.059 | 301423 | 33.060 | 301600 |  |  |
|  |  |  | 36 | 30.510 | 170731 | 30.510 | 170790 |  |  |
|  |  |  | 40 | 27.987 | 97083 | 27.987 | 97083 |  |  |
| Mobile | 30 | 298 | 28 | 33.805 | 11019648 | 33.805 | 11018817 | 0.03% | -0.0012 |
|  |  |  | 32 | 30.485 | 5090807 | 30.485 | 5092035 |  |  |
|  |  |  | 36 | 27.534 | 2431840 | 27.534 | 2433325 |  |  |
|  |  |  | 40 | 24.777 | 1289526 | 24.777 | 1289546 |  |  |
| Tempete | 30 | 256 | 28 | 34.772 | 7843645 | 34.772 | 7842956 | -0.01% | 0.0004 |
|  |  |  | 32 | 31.683 | 3554772 | 31.683 | 3553692 |  |  |
|  |  |  | 36 | 28.970 | 1690601 | 28.971 | 1691145 |  |  |
|  |  |  | 40 | 26.444 | 866965 | 26.444 | 867025 |  |  |
| total average |  |  |  |  |  |  |  | 0.05% | -0.0022 |

Table 2. Comparison of proposed 8x8 constrained spatial direct vs. original (SR=48,JM5)

| sequences | frame rate | number of frames | QP | PSNRY (Spat) | Total Bits | PSNRY (Spat8) | Total Bits | Spat ave. change (excel) | Spat ave. dB gain (excel) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Canoa (480i) | 30 | 60 | 20 | 42.02 | 21977608 | 42.02 | 21976736 | -0.004% | 0.00 |
| F1 (480i) | 30 | 60 | 20 | 42.56 | 14001480 | 42.56 | 14001128 | -0.003% | 0.00 |
| Crew (720P) | 60 | 60 | 20 | 42.70 | 19842088 | 42.71 | 19846360 | 0.022% | 0.01 |
| BigShips (720P) | 60 | 60 | 20 | 41.39 | 20866392 | 41.39 | 20863112 | -0.016% | 0.00 |
| Flamingo (1080i) | 30 | 60 | 20 | 41.93 | 125748432 | 41.93 | 125751520 | 0.002% | 0.00 |
| Kayak (1080i) | 30 | 60 | 20 | 41.67 | 170386408 | 41.67 | 170389112 | 0.002% | 0.00 |
| Mountain (1080i) | 30 | 60 | 20 | 41.62 | 82732720 | 41.62 | 82741488 | 0.011% | 0.00 |
| total average |  |  |  |  |  |  |  | 0.002% | 0.00 |

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Microsoft |  |  |
| Mailing address | Microsoft Research Asia, 3F Sigma, No49 Zhichun Rd, Haidian Beijing 100080, China |  |  |
| Country | China |  |  |
| Contact person | Alexis Michael Tourapis |  |  |
| Telephone | (86-10) 6261-7711 ext 3108 |  |  |
| Fax | (86-10) 8809 7306 |  |  |
| Email | alexismt@ieee.org |  |  |
| Place and date of submission | Awaji, 5-13 December, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | 8x8 Block Restriction to Spatial Direct Mode |  |
| Contribution number |  | JVT-F025 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: