| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 6th Meeting: Awaji, Island, JP, 5-13 December, 2002 | Document: JVT-F076 Filename: JVT-F076.doc |
| --- | --- |

| Title: | More Results for MV Spread Restriction of JVT-E041 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Anthony Joch UB Video Inc. 400-1788 West 5th Ave Vancouver, BC, V6J 1P2, Canada | Tel: Email: | +1 604 737 2424 .33 anthony@ubvideo.com |
| Source: | UB Video Inc. |  |  |

_____________________________

This document provides cross-verification and further results for JVT-E041, “MV Spread in sub-8x8 Partitions”. Source code provided by the proponents was verified and run for the common conditions, as well as the Canoe and Football sequences. For these two interlaced sequences, 2 B-pictures and frame-level AFF were used at QP=20, 24, 28 and 32. Results were generated for the “Hybrid” motion search case with byte limits of 484 and 676 bytes. Results for the unrestricted software (Ref) and the two bytes limited cases (484B and 676B) are presented below.

The results show an average BD bit-rate increase of 0.072% and 0.054% for the 484 and 676 byte limits, respectively, over the test set. This shows that there is little reason to increase the size of the byte restriction beyond 484 bytes.

Further, the effect of the MV spread restriction is even smaller on the two SD sequences than on the common conditions sequences.

Currently, the worst-case permitted at Level 3 and below allows for 16 motion vectors in each macroblock in the Baseline profile, and 32 motion vectors per macroblock in profiles that allow B-slices. With no restriction on the MV spread, this necessitates up to 48 (or 96) DMA requests for the decoding of each macroblock, which is not reasonable on existing and anticipated DSP and media processor architectures. The proposed restriction would reduce these worst-case numbers by a factor of 4 on most architectures.

If some restriction to limits the number of DMA requests at Level 3 and below is not adopted in the standard, individual industry segments will be forced to define their own limits in order to ensure worst-case interoperability. Thus, we strongly support the adoption of the 484-byte limit as proposed in JVT-E041 for Level 3 and below.

We also note that, with the existing limit of 16 motion vectors per 2 consecutive macroblocks at Level 3.1 and higher, the use of sub-macroblock partitions smaller than 8x8 will certainly be reduced, and therefore the effect of also having an MV spread restriction at these levels is anticipated to be minimal. Therefore, it may be reasonable to consider the limit for these levels also. However, decoders for these levels would be more commonly served by dedicated hardware, which does not have a constraint on the number of DMA requests. Also, the existing restriction of 16 motion vectors per 2 consecutive macroblocks already limits the number of DMA requests to 24 per macroblock, in the worst-case.

| Sequence | frames | QP | Ref PSNR | Ref Bitrate | 484B PSNR | 484B Bitrate | BD-Bitrate | BD-PSNR [db] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| container | 100 | 28 | 36.03 | 23.19 | 36.03 | 23.19 | 0.000% | -0.000 |
|  |  | 32 | 33.23 | 12.92 | 33.23 | 12.92 |  |  |
|  |  | 36 | 30.54 | 7.65 | 30.54 | 7.65 |  |  |
|  |  | 40 | 27.89 | 4.75 | 27.89 | 4.75 |  |  |
| foreman | 100 | 28 | 36.04 | 69.52 | 36.05 | 69.65 | 0.001% | -0.000 |
|  |  | 32 | 33.25 | 42.67 | 33.26 | 42.62 |  |  |
|  |  | 36 | 30.61 | 26.46 | 30.60 | 26.47 |  |  |
|  |  | 40 | 28.10 | 16.71 | 28.08 | 16.74 |  |  |
| silent | 150 | 28 | 35.82 | 56.40 | 35.84 | 56.42 | 0.195% | -0.009 |
|  |  | 32 | 32.98 | 34.05 | 32.97 | 34.09 |  |  |
|  |  | 36 | 30.36 | 20.02 | 30.37 | 20.11 |  |  |
|  |  | 40 | 28.03 | 11.71 | 28.01 | 11.68 |  |  |
| news | 100 | 28 | 36.82 | 44.36 | 36.84 | 44.30 | 0.362% | -0.021 |
|  |  | 32 | 33.75 | 27.30 | 33.71 | 27.40 |  |  |
|  |  | 36 | 30.83 | 16.77 | 30.85 | 16.88 |  |  |
|  |  | 40 | 28.14 | 10.41 | 28.16 | 10.36 |  |  |
| paris | 150 | 28 | 35.50 | 311.17 | 35.49 | 310.94 | -0.122% | 0.006 |
|  |  | 32 | 32.42 | 180.38 | 32.43 | 180.28 |  |  |
|  |  | 36 | 29.51 | 100.87 | 29.51 | 100.69 |  |  |
|  |  | 40 | 26.85 | 57.15 | 26.84 | 57.16 |  |  |
| mobile | 300 | 28 | 34.03 | 1490.12 | 34.03 | 1489.41 | 0.066% | -0.003 |
|  |  | 32 | 30.64 | 731.97 | 30.64 | 732.12 |  |  |
|  |  | 36 | 27.58 | 353.20 | 27.58 | 353.46 |  |  |
|  |  | 40 | 24.66 | 189.39 | 24.65 | 189.64 |  |  |
| tempete | 260 | 28 | 35.00 | 1130.14 | 35.00 | 1129.51 | 0.173% | -0.007 |
|  |  | 32 | 31.85 | 546.95 | 31.84 | 546.90 |  |  |
|  |  | 36 | 29.01 | 270.45 | 28.99 | 269.62 |  |  |
|  |  | 40 | 26.33 | 145.92 | 26.33 | 146.18 |  |  |
| canoe | 220 | 20 | 42.04 | 14374.59 | 42.04 | 14363.92 | 0.001 | -0.000 |
|  |  | 24 | 39.03 | 9111.17 | 39.03 | 9108.87 |  |  |
|  |  | 28 | 36.20 | 5799.22 | 36.19 | 5797.42 |  |  |
|  |  | 32 | 33.23 | 3501.42 | 33.23 | 3501.89 |  |  |
| football | 220 | 20 | 41.38 | 10921.25 | 41.38 | 10925.16 | 0.046 | -0.002 |
|  |  | 24 | 38.75 | 6188.33 | 38.76 | 6185.94 |  |  |
|  |  | 28 | 36.30 | 3664.87 | 36.29 | 3668.77 |  |  |
|  |  | 32 | 33.86 | 2158.01 | 33.86 | 2157.74 |  |  |
| Average |  |  |  |  |  |  | 0.072% | -0.003 |

| Sequence | frames | QP | Ref PSNR | Ref Bitrate | 676B PSNR | 676B Bitrate | BD-Bitrate | BD-PSNR [db] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| container | 100 | 28 | 36.03 | 23.19 | 36.03 | 23.19 | 0.000% | 0.000 |
|  |  | 32 | 33.23 | 12.92 | 33.23 | 12.93 |  |  |
|  |  | 36 | 30.54 | 7.65 | 30.54 | 7.65 |  |  |
|  |  | 40 | 27.89 | 4.75 | 27.89 | 4.75 |  |  |
| foreman | 100 | 28 | 36.04 | 69.52 | 36.02 | 69.69 | -0.245% | 0.014 |
|  |  | 32 | 33.25 | 42.67 | 33.27 | 42.56 |  |  |
|  |  | 36 | 30.61 | 26.46 | 30.62 | 26.43 |  |  |
|  |  | 40 | 28.10 | 16.71 | 28.10 | 16.71 |  |  |
| silent | 150 | 28 | 35.82 | 56.40 | 35.84 | 56.49 | 0.297% | -0.017 |
|  |  | 32 | 32.98 | 34.05 | 32.95 | 34.13 |  |  |
|  |  | 36 | 30.36 | 20.02 | 30.36 | 20.02 |  |  |
|  |  | 40 | 28.03 | 11.71 | 28.03 | 11.71 |  |  |
| news | 100 | 28 | 36.82 | 44.36 | 36.83 | 44.50 | 0.362% | -0.016 |
|  |  | 32 | 33.75 | 27.30 | 33.73 | 27.44 |  |  |
|  |  | 36 | 30.83 | 16.77 | 30.84 | 16.80 |  |  |
|  |  | 40 | 28.14 | 10.41 | 28.14 | 10.41 |  |  |
| paris | 150 | 28 | 35.50 | 311.17 | 35.49 | 311.16 | -0.195% | 0.009 |
|  |  | 32 | 32.42 | 180.38 | 32.44 | 180.40 |  |  |
|  |  | 36 | 29.51 | 100.87 | 29.52 | 100.77 |  |  |
|  |  | 40 | 26.85 | 57.15 | 26.87 | 57.06 |  |  |
| mobile | 300 | 28 | 34.03 | 1490.12 | 34.03 | 1491.01 | 0.202% | -0.009 |
|  |  | 32 | 30.64 | 731.97 | 30.64 | 733.23 |  |  |
|  |  | 36 | 27.58 | 353.20 | 27.57 | 353.28 |  |  |
|  |  | 40 | 24.66 | 189.39 | 24.66 | 189.89 |  |  |
| tempete | 260 | 28 | 35.00 | 1130.14 | 35.00 | 1128.70 | 0.116% | -0.005 |
|  |  | 32 | 31.85 | 546.95 | 31.84 | 546.70 |  |  |
|  |  | 36 | 29.01 | 270.45 | 29.01 | 270.85 |  |  |
|  |  | 40 | 26.33 | 145.92 | 26.34 | 145.95 |  |  |
| canoe | 220 | 20 | 42.04 | 14374.59 | 42.04 | 14370.13 | -0.017% | 0.001 |
|  |  | 24 | 39.03 | 9111.17 | 39.04 | 9109.00 |  |  |
|  |  | 28 | 36.20 | 5799.22 | 36.19 | 5798.14 |  |  |
|  |  | 32 | 33.23 | 3501.42 | 33.23 | 3503.98 |  |  |
| football | 220 | 20 | 41.38 | 10921.25 | 41.38 | 10924.57 | 0.019% | -0.001 |
|  |  | 24 | 38.75 | 6188.33 | 38.76 | 6183.14 |  |  |
|  |  | 28 | 36.30 | 3664.87 | 36.29 | 3666.77 |  |  |
|  |  | 32 | 33.86 | 2158.01 | 33.86 | 2161.73 |  |  |
| Average |  |  |  |  |  |  | 0.054% | -0.002 |