| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 6th Meeting: Awaji, Island, JP, 5-13 December, 2002 | Document: JVT-F083L Filename: JVT-F083L.doc |
| --- | --- |

| Title: | Verification to JVT-F077, Weighted Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hideaki Kimata NTT Cyber Space Laboratories 1-1 Hikari-no-oka, Yokosuka JAPAN | Tel: Email: | +81-468-59-3124 kimata.hideaki@lab.ntt.co.jp |
| Source: | NTT Cyber Space Laboratories, NTT Corporation |  |  |

_____________________________

- Introduction

The purpose of this document is to provide verification results of Weighted Prediction (WP) technique for P-slice and B-slice, originally proposed by JVT-F077.

- Experiment

  - Experimental conditions

Experiments were basically conducted with JM50f, JM50f_wp, and JM50g. In JM50f_wp, WP is implemented on JM50f, where JM50f does not include WP. It seems that WP in JM50g is broken, so I also have experimented WP with JM50f_wp. Test sequences are listed in Table 1. Test conditions are as follows. QPs are 28, 32, 36, 40. And the search range of motion estimation is set to –32 to +32. The IP structure (M=1) and IBBP structure (M=3) are experimented. Entropy coding is CABAC. The number of reference picture is 5.

Table 1. Test sequences

| Name | container_news_cross | container_fade_black | container_fade_white | foreman_fade_black | foreman_fade_white |
| --- | --- | --- | --- | --- | --- |
| Content | Linear dissolve from container to news | Linear fade in of black image to container | Linear fade in of white image to container | Linear fade in of black image to foreman | Linear fade in of black image to foreman |

  - Results for M=1

Table 2 shows the BD-bitsaving for M=1, where explicit single WP is used for P-MB in JM50f_wp. We can see greatly significant improvement by explicit single WP.

Table 2: BD-bitsaving with JM50f_wp (M=1)

| WP mode | container_news_cross | container_fade_black | container_fade_white | foreman_fade_black | foreman_fade_white | Average [%] |
| --- | --- | --- | --- | --- | --- | --- |
| Explicit single | 22.02 | 56.22 | 53.19 | 47.97 | 39.76 | 43.83 |

  - Results for M=3

Table 3 shows the BD-bitsaving for M=3, where explicit single WP is used for P-MB, and either explicit bi-predictive or implicit bi-predictive WP is used for B-MB in JM50f_wp. We can see greatly significant improvement by explicit or implicit bi-predictive WP for B-MB in addition to explicit single WP for P-MB. And generally implicit bi-predictive WP is better than explicit bi-predictive WP for B-MB.

Table 2: BD-bitsaving with JM50f_wp (M=3)

| WP mode | container_news_cross | container_fade_black | container_fade_white | foreman_fade_black | foreman_fade_white | Average [%] |
| --- | --- | --- | --- | --- | --- | --- |
| Only Explicit single | 15.9 | 32.98 | 33.08 | 11.76 | 13.27 | 21.40 |
| Explicit single + Explicit Bipredictive | 4.85 | 51.43 | 37.66 | 41.81 | 20.69 | 31.29 |
| Explicit single + Implicit Bipredictive | 21.73 | 55.57 | 49.27 | 36.28 | 26.38 | 37.85 |

We also experimented the same tests with JM50g. But It seems that WP in JM50g is broken. The test results are shown in the associated Excel files just for reference information to JM50g users.

- Conclusion

We experimented WP technique for P-MB and B-MB. We confirmed explicit single WP for P-MB and implicit bi-predictive WP for B-MB improves coding efficiency great significantly.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | NTT |  |  |
| Mailing address | 1-1 Hikari-no-oka |  |  |
| Country | JAPAN |  |  |
| Contact person | Yoshiyuki Yashima |  |  |
| Telephone | +81 468 59 2831 |  |  |
| Fax | +81 468 59 2829 |  |  |
| Email | yashima.yoshiyuki@lab.ntt.co.jp |  |  |
| Place and date of submission | Awaji, Japan, 5-13, December 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Verification to JVT-F077, Weighted Prediction |  |
| Contribution number |  | JVT-F083L |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: