| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 6th Meeting: Awaji, Island, JP, 5-13 December, 2002 | Document: JVT-F094 Filename: JVT-F094-L.doc |
| --- | --- |

| Title: | Explanation of Issues on H.264/AVC Sub-Sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Mobile Software P.O. Box 88 33721 Tampere Finland Dong Tian TICSP Tampere Univ. of Technology P.O. Box 553 FIN-33101 Tampere Finland | Tel: Email: | +358 40 5212845 miska.hannuksela@nokia.com +358 3 3115 3853 tian@cs.tut.fi |
| Source: | Nokia Corporation and Tampere University of Technology |  |  |

This document is a copy of the MPEG document M9218.

INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC 1/SC 29/WG 11/M9218

December 2002

| Title: | Sub-Sequences in AVC File Format |
| --- | --- |
| Group: | MPEG-4 |
| Sub group: | Systems |
| Source: | Miska M. Hannuksela Nokia Mobile Software P.O. Box 88, 33721 Tampere, Finland miska.hannuksela@nokia.com Dong Tian Tampere University of Technology P.O. Box 553, 33101 Tampere, Finland tian@cs.tut.fi |
| Status | Input |

# Summary

The draft text of the AVC file format (MPEG document N5303) lists the following concerns on the sub-sequence feature of the proposed AVC file format:

- Clarifying whether  there is one division into layers and sub-sequences, or multiple;

- What ‘layers’ mean if sub-sequences may be selected from any layer, provided that their dependents have been decoded;

- Given that sub-sequences begin and end and then begin again, what the exact meaning of a ‘dependency’ is (on which instances?);

- What the use of sub-sequence identifiers is;  whether they refer to definitions or instances, and if the latter, how the identifier space is managed in an indefinite duration sequence;

- How the beginning and end of a sub-sequence can be identified;

- Whether dependencies are only direct, or transitive;

- What the meaning of the bit-rate etc. are, in the definition of sub-sequences, with respect to the individual instances of the sub-sequences.

This contribution responds to the concerns.

# Response to Concerns

- Clarifying whether  there is one division into layers and sub-sequences, or multiple;

Response: The concept of sub-sequences allows multiple divisions into layers and sub-sequences. For example, a separate sub-sequence layer for non-reference pictures may be beneficial in order to have a finer grain in the bit-rate and frame rate statistics of sub-sequence layers and sub-sequences.

Figure 1 shows an example of a GOP where pictures are categorized into three sub-sequence layers. Boxes indicate frames, letters in boxes indicate frame types, numbers in boxes indicate frame numbers, dashed ellipses indicate sub-sequences, vertical alignment of boxes indicate the sub-sequence layer which the picture belongs to, arrows indicate prediction dependencies, and display time runs from left to right. A 7.5 Hz, 15 Hz, or 30 Hz picture rate can be achieved by decoding layers 1, 2, or all 3 layers respectively. The frames in the example could have been groupped into two sub-sequence layers so that all non-reference frames would have been in the second sub-sequence layer. In this case, only 7.5 Hz and 30 Hz picture rate could have been signaled.

Figure 1. An example of a 3-layer sub-sequence structure.

- What ‘layers’ mean if sub-sequences may be selected from any layer, provided that their dependents have been decoded;

Response: Sub-sequences can be but do not have to be organized into sub-sequence layers. Layers help to signal long-term characteristics of a coded data stream (see the Layer Description Entry of the file format). Furthermore, layers help to signal sub-sequence dependencies, since a sub-sequence can only depend on sub-sequences that reside in lower sub-sequence layers (see the Sub-Sequence Description Entry of the file format).

- Given that sub-sequences begin and end and then begin again, what the exact meaning of a ‘dependency’ is (on which instances?);

Response: See the response for case iv)

- What the use of sub-sequence identifiers is;  whether they refer to definitions or instances, and if the latter, how the identifier space is managed in an indefinite duration sequence;

Response: One sub-sequence begins and ends only once. The duration of sub-sequence layers is indefinite. Sub-sequence identifiers refer to instances of sub-sequences. Consecutive sub-sequences within a particular sub-sequence layer in decoding order shall have a different sub-sequence identifier from each other.

When indicating dependencies of a sub-sequence (referred herein to as this sub-sequence), we propose to use a signed integer to help to signal the dependent sub-sequence, called dependencyIndicator, in the dependencyInfo structure. If dependencyIndicator is a non-negative interger n, the sub-sequence used as a reference for this sub-sequence is the (n+1)th closest sub-sequence among those whose first picture precedes the first picture of this sub-sequence in decoding order and which resides on the indicated layerNumber and has the indicated subSequenceIdentifier. If dependencyIndicator is a negative integer whose absolute value is n, the sub-sequence used as a reference for this sub-sequence is the nth closest sub-sequence among those whose first picture succeeds the first picture of this sub-sequence in decoding order and which resides on the indicated layerNumber and has the indicated subSequenceIdentifier.

- How the beginning and end of a sub-sequence can be identified;

Response: In the AVC file format the beginning and end of a sub-sequence is identified implicity by the group of samples that belong to a sub-sequence according to a SampleToGroup box. In the AVC standard, the beginning and the end of a sub-sequence can be indicated by setting the first_ref_pic_flag and the last_pic_flag respectively to 1 in the sub-sequence information Supplemental Enhancement Information message.

- Whether dependencies are only direct, or transitive;

Response: Dependencies are only direct.

- What the meaning of the bit-rate etc. are, in the definition of sub-sequences, with respect to the individual instances of the sub-sequences.

Response: See the latest AVC draft standard (JVT document JVT-E146d37) for the semantics.