| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 7th Meeting: Pattaya II, Thailand, 7-14 March, 2003 | Document: JVT-G043 Filename: JVT-G043 |
| --- | --- |

| Title: | Constraints on Buffering Period SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Natan Peterfreund 549 Baltic Way Sunnyvale, CA 94089 USA | Tel: Email: : | +972-4-6230150 natan.peterfreund@harmonicinc.com |
| Source: | Harmonic Inc. |  |  |

_____________________________

Abstract:

In current CAT-LB model the Buffering Period SEI message provides two parameters, namely the initial_cpb_removal_delay which represents the delay between the time of arrival in the cbp of the first bit of coded data associated with the picture associated with the buffering period SEI message and the removal time of that data from the cpb, and the initial_cpb_removal_delay_offset which represents the delay between the eariliest arrival time  and the actual arrival time to the CPB of the first bit of the above picture data. The earliest arrival time of the  picture data is defined to be the removal time minus the sum of initial_cpb_removal_delay and initial_cpb_removal_delay_offset. This document provides the clarity how to compute both parameters in the presence of any buffering period SEI message along the bitstream. It also imposes constraints on the values of the two parameters which result with same HRD fullness behavior independent of the random access point in the bitstream.

Another related contribution is a separate initial_buffering_bit_rate for the initial_cpb_removal_delay period,  which provides the means to adjust buffer level in the presence of switching between sequence parameter sets with different HRD parameters.

The proposal

Consider the timing diagram in figure 1 describing the removal time tr(n) of  picture n, the final arrival time taf(n-1) of picture n-1, and the earliest arrival time Tai,earliest(n) of picture n provided by subtraction of the sum of initial_cpb_removal_delay and initial_cpb_removal_delay_offset from tr(n). Assuming a buffering period SEI message following the last bit of the data associate with picture n-1 and which is associated with the picture data of picture n. In-order to provide seamless random access at this point, the initial_cpb_removal_delay in the BufferingPeriodSEIMessage should follow the constraint

Initial_cpb_removal_delay <= tr(n) – max( taf(n-1), Tai,earliest(n) )

and

initial_cpb_removal_delay + initial_cpb_removal_delay_offset = constant

The second constraint which provides the delay between Tai,earliest and the removal time of the data associated with the picture is already defined in the current version of the standard.

Based on 1. we have, following a buffering period sei message,

tai(n) = tr(n) – initial_cpb_removal_delay

The proposed constraints provides the following:

- The HRD buffer behavior at and following the removal time of a picture data associated with a buffering period SEI message is independent of the random access point along the bitstream.

- A value of the initial_cpb_removal_delay which is lower than the above constraint would result in  zero bit-rate gaps between max(taf(n-1), Tai,earliest(n) ) and tai(n). This could be used for buffer level control. See the example in Figure 1.

tai(n)

Tai,earliest(n)       taf(n-1)			tr(n)

BufferingPeridSEIMessage                               upper bound on initial_cpb_removal_delay

Figure 1: Example of a timing diagram of a bitstream

Second problem we deal with is to provide HRD compliance in the presence of switching between sequence parameter sets. Here we assume the same problem as defined in Figure 1 with a new active sequence parameter set - set for picture n. The switching between different bit-rates and buffer sizes may cause overflow/underflow problems in the vicinity of the switching point as described in Figure 2.

cpb_size 1/bit_rate 1                                                                                           new cpb_size

new bit-rate

tai(n)                            tr(n-1)                tr(n)

BuffeingPerriodSEIMessage + “New” sequence par. set.

Figure 2: Example of HRD behavior in the presence of a new active

sequence parameter set

Assuming the following two options when the new HRD parameters take affect:

- following tai(n)

- following tr(n-1)

Assuming 1. and assuming we lowered the value of the new cpb_size, then during tai(n) and tr(n-1) –  the new cpb_size may be violated.

Assuming 2. and assuming a higher new bit_rate, then cpb_size 1 may be violated during tai(n) and tr(n-1).

Inorder to prevent the above problem we propose the following:

- A separate buffering_bit_rate at the buffering period SEI message to provide the bit-rate for the period between tai(n) and tr(n) (equal to initial_cpb_removal_delay). This feature would apply only at the switching point between two sequence parameter sets.

- The new HRD parameters takes effect following tr(n-1).

Last thing. We suggest to remove the following restrictions:

- allow change of the sum of initial_cpb_removal_delay + initial_cpb_removal_delay_offset  when switching to a new sequence parameter set.

- remove the restriction initial_cpb_removal_delay + initial_cpb_removal_delay_offset < = buffer_size/bit_rate, and set this bound to 10 sec (?)

Proposed Changes

Change 1

Before (C-11) change the following

….then the initial CPB removal delay for the buffering period sei message associated with the data associated with picture n has the following constraint

Initial_cpb_removal_delay <= tr(n) – max( taf(n-1), Tai,earliest(n) )		(C-11)

and

tai(n) = tr(n) – initial_cpb_removal_delay

Change 2

Make the following change to buffering period sei message

buffering_period( payloadSize ) {

seq_parameter_set_id

if( nal_hrd_parameters_present_flag = = 1 ) {

for( k = 0; k <= cpb_cnt_minus1; k++ ) {

initial_cpb_removal_delay[ k ]

initial_cpb_removal_delay_offset[ k ]

} if ( new_active_sequence_parameter_set) for (k=0; k<= cpb_cnt_minus1; k++) buffering_bit_rate_value[k]

}

if( vcl_hrd_parameters_present_flag = = 1 ) {

for( k = 0; k <= cpb_cnt_minus1; k++ ) {

initial_cpb_removal_delay[ k ]

initial_cpb_removal_delay_offset[ k ]

} if ( new_active_sequence_parameter_set) for (k=0; k<= cpb_cnt_minus1; k++) buffering_bit_rate_value[k]

}

}

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Harmonic Inc. |  |  |
| Mailing address | 19 Alon Hatavor St. P.O.Box 3600 Caesarea Industrial Park 38900 |  |  |
| Country | Israel |  |  |
| Contact person | Natan Peterfreund |  |  |
| Telephone | 972-4-6230150 |  |  |
| Fax | 972-4-6230151 |  |  |
| Email | natan.peterfreund@harmonicinc.com |  |  |
| Place and date of submission | Caesarea Israel, 10/04/02 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: