| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 12th Meeting: Redmond, WA, USA 17-23 July, 2004 | Document: JVT-Lxxx Filename: input.doc |
| --- | --- |

| Title: |  |  |  |
| --- | --- | --- | --- |
| Status: | [e.g., Input Document to JVT or Output Document Approved by JVT] |  |  |
| Purpose: | [e.g, Draft, Proposed Draft, Proposal, Information, Liaison, Report] |  |  |
| Author(s) or Contact(s): | Name Address line 1 Address line 2 Address line 3 | Tel: Email: |  |
| Source: | [Name of sourcing organization] |  |  |

_____________________________

Joint meeting planned with MPEG requirements 3:30pm – 4:30pm

VCEG 8:30 – 10:00am Tues.

VCEG & JPEG Tues. pm? [not yet finalized]

VCEG & MPEG Video Wed. pm. [4pm]

VCEG 8:30 – 10:00 Thurs. [not yet finalized]

JVT & Requirements Thurs 12:30-1pm

Technical FRExt proposals open to further consideration on Monday:

- Quantization offsets [All FRExt] – 2nd time, new variants, needs editing work

- CAVLC for 8x8 [All FRExt] – old technology, good editing [Adopted]

- Residual Color Transform [4:4:4] – technology seen several times, somewhat modified, needs editing

- Lossless [4:4:4] – not fully mature [Not adopted (at this meeting)]

- Stereo SEI [orthogonal] – relatively minor, needs some tweaking – one bit remains.

- Alpha [orthogonal] – may be mature [Adopted]

- Deblocking filter display SEI [Adopted]

Tomorrow :10:00 am

#1, [can we get demo room?] #2, #5, #6, #3, #4

Tuesday

Note joint VCEG and JPEG at 3pm today in JVT room.

Wait until an Amendment 2 for 4:4:4 profile?  No.  Agreed.

Opening remarks

ftp site ftp.imtc-files.org directory jvt-experts, sub-directory 2004_07_Redmond.

Accessing the ftp site: Tip: If trouble with anonymous, try user ID "imtctest" with password "password".

IPR declaration need, IPR policy – should review submitted letter list this week

Email lists: jvt-experts and jvt-bitstream.  Can be found near bottom of "activity groups" in the IMTC web site http://www.imtc.org.

Comment: Many ftp problems can be solved by turning on or off "passive mode" on your ftp client.

Another tip: Use a primitive tool such as the DOS command-line ftp tool. (which doesn't support passive mode), rather than something like a web browser.

"Total commander" and ws_ftp are tools reputed to work well.

Doc List

JVT-L001 [Sullivan, Luthra, Wiegand] Report of Redmond meeting

JVT-L002 [Sullivan, Luthra, Wiegand] Report of Munich meeting

JVT-L003 [Sullivan, Luthra, Wiegand] AHG report: Proj mgmt & errata

JVT-L004 [Wiegand, Suehring, Ma, Lim] AHG report: JM & ref soft

JVT-L005 [McMahon, Sullivan, Wiegand, Gordon] AHG report: FRExt

JVT-L006 [Joch, Suehring, Suzuki] AHG report: Bitstream & conf.

JVT-L007 [Suzuki] AHG report: Impl. Guide

JVT-L008 <vacant>

JVT-L009 [Sullivan] Deployment status review

JVT-L010 [Sullivan] Corrigendum input

JVT-L011 [McMahon] Remarks on profiles

JVT-L012 [Wiegand] Integrated FREXT input draft

JVT-L013 [Haskell] Addition of alpha channel to AVC/H.264 FRext

JVT-L014 [Sun] Residual color transform Using YCoCg-R

JVT-L015 [Sun] Quality scalability for FRExt

JVT-L016 [Sun] Verification of alpha channel coding

JVT-L017 [Lee] Lossless coding for FRExt

JVT-L018 [Hannuksela, Wang] Sub-stream profile & level signaling

JVT-L019 [Kerofsky] Simulcast for low-delay random access

JVT-L020 [Suzuki] Editor's prop. draft AVC|H.264 conformance spec

JVT-L021 [Suzuki] Proposals to improve AVC|H.264 Conformance Spec

JVT-L022 [Winger] CAVLC for 8x8 (JVT-E085 in JM_FREXT19a)

JVT-L023 [Karpinnen] SEI for stereo video in progressive sequences

JVT-L024 [Chen, Lu, Kashiwagi, Kadono, Lim] Transform dynamic range

JVT-L025 [Kim] Residual color transform

JVT-L026 [Gsell] Extended bit depth characterization

JVT-L027 [Gish, Kim] Bit depth interoperability

JVT-L028 [Kim] Additional data on residue sampling (JVT-K018)

JVT-L029 [Wedi, Wittman] Inverse quantization offset for FRExt

JVT-L030 [Wedi, Suzuki, Yamada, Imaide, McMahon, de Waele, Jeon,

Gordon, Ito] 4:2:0 / 8-bit profile for FRExt

JVT-L031 [Choi, Lee, Choi, Jeon] Performance of fast mode decision

JVT-L032 [Tourapis, Boyce] Quant offset matrices for FRext

JVT-L033 [Wedi, Kashiwagi] Subj. quality of FRExt for HD movie

JVT-L034 [Karczewicz, Bao] Need for test model enhancements

JVT-L035 [Lu] DPB use for deblocking filter display pref SEI

--------  Late-Registered Contributions -------

JVT-L036-L [Suzuki] Proposal to support CAVLC for 8x8 transform

JVT-L037-L [Karczewicz, Ridge] Extension to AVC-based scalable video

Subjects

Administrative & General

JVT-L001 [Sullivan, Luthra, Wiegand] Report of Redmond meeting

JVT-L002 [Sullivan, Luthra, Wiegand] Report of Munich meeting

JVT-L003 [Sullivan, Luthra, Wiegand] AHG report: Proj mgmt & errata

JVT-L004 [Wiegand, Suehring, Ma, Lim] AHG report: JM & ref soft

Promote FRExt reference software to CD at this meeting (DURING this meeting).

JVT-L005 [McMahon, Sullivan, Wiegand, Gordon] AHG report: FRExt

JVT-L006 [Joch, Suehring, Suzuki] AHG report: Bitstream & conf.

Some bitstreams have uncorrected Annex A violations and will need to be removed from the set.

Progress made on HRD bitstreams.  HRD parameters missing from many streams.  Method proposed to address that issue.

Missing redundant slices and data partitioning.

FRExt – need cases other than 4:2:2/8.

Would like to promote to PDAM at end of this meeting, but need to find volunteers to provide bitstreams.

JVT-L007 [Suzuki] AHG report: Impl. Guide

No activity.  Encourage future contribution, and contribution to the test model document in the future but plan to not have an IG AHG continue.

JVT-L009 [Sullivan] Deployment status review

Not up to date.

ITU-T systems adoption completed

MPEG-2 and MPEG-4 systems & file format adoption completed

Other organizations and app environments

IETF WG last call for RTP payload

HD DVD in DVD Forum

DMB in Rep. of Korea

Broadcast announcement in Japan
		NHK (Japan Broadcasting Corporation),
		TBS (Tokyo Broadcasting System, Inc.)
		NTV (Nippon Television Network Corporation)
		TV Asahi Corporation
		Fuji Television Network, Inc.
		TV Tokyo Corporation

DVB

TS 102 005 IP datacast (Baseline profile; with constraint_set1_flag == 1;
			forwarded to EBU/ETSI JTC broadcast) and

TS 101 154 Broadcast using MPEG-2 Systems (Main profile;
			to finalize in DVB in Sept.)

France Terrestrial Broadcast announcement (not MPEG-2 SD video,
			but H.264/AVC HD)

Blu-Ray Disc Founders (BDF) – see incoming liaison letter, FRExt emphasis
		See MPEG document M11116.
		Beyond MPEG-2, JVT-K047 4:2:0/8 FRExt is their first choice.

US DoD MISB adoption for below-SD video

3GPP – current working assumption as optional codec in Rel 6 (Sept.)

3GPP2 – similar state to 3GPP

ATSC – under consideration as terrestrial simulcast (e.g., 1.5 Mbps SD), and
		with allowances for future approval as main-stream format

Corrigendum

JVT-L010 [Sullivan] Corrigendum input

Finalizing this into a new corrigendum does not seem especially urgent, base on the lack of any apparent reports that would appear to be likely to generate real interop problems.

Conformance

JVT-L020-M [Suzuki] Editor's prop. draft AVC | H.264 conformance spec

JVT-L021-L [Suzuki] Proposals to improve AVC | H.264 Conformance Spec

Annex A violations – revise or remove.

Missing features – RS, DP, MMCO with fields, ref pic list reordering with fields, MMCO with CABAC (probably don’t' need that), SI picture bitstreams provided but not verified (ref software verification should suffice).

Request to verify constraint set flag settings.

HRD – checked all streams and confirmed that all are conforming (including CBR mode) and have tools to generate HRD information.  Can provide HRD information as text.

Try to get (at this meeting) volunteers for missing features and FRExt profiles.

Dolby: 4:2:0/10, 4:4:4/10&12

Copyright: ITU policy implicit (A.1 sec 3.1.5), ISO policy probably also implicit.

Stefan (Sarnoff?) and Container/Coastguard (Hughes Aircraft?)

FRExt

ISO/IEC ballot responses and other ISO/IEC relevant contributions

Assign numbers to the responses.

For FRExt amendment:

Japan

US

Profiles & Levels

JVT-L011 [McMahon] Remarks on profiles

Proposes Level 4.2 max frame size (currently 1920x1080 or 2048x1024 x60p)

 (2048x1080x60)

Proposes Level 4.2 max bit rate (4*50 Mbps)

 155 Mbps for FRExt

If we do that, we should increase level 5 bit rate also, since it is lower than 155.

Or add a Level 4.3?  Seems preferable.

Need to review all bit rates and CPB sizes for FRExt.  Defer.

Current Level 4.2:

should the max frame size have been bigger by 8704/8192 = 6.25%?

and the number of MB/s bigger by the same ratio?

Put into corrigendum WD as an issue under consideration and make a resolution.  Agreed.

We will make this adjustment for all FRExt profiles.  Agreed.

Increase buffer sizes accordingly (maintain same time relationship)

Network/cable primary distribution rates:

8 Mbps

10 Mbps

20 Mbps

41-45 Mbps

55 Mbps

73 Mbps

90 Mbps

Fiber and VTRs/Servers

45 Mbps

155 Mbps

270 Mbps – Raise level 5.1 bit rate to this?

JVT-L030 [Wedi, Suzuki, Yamada, Imaide, McMahon, de Waele, Jeon, Gordon, Ito] 4:2:0 / 8-bit profile for FRExt

MPEG document M11116 from Blu-Ray Disc Founders (BDF):

Beyond MPEG-2, JVT-K047 4:2:0/8 FRExt is their first choice.

USNB comment on 4:2:0/8

Remark: Adding 4:2:0/8 would make 5 FRExt profiles.  Too many.

Remark: How about dropping 4:2:0/10?

Remark: For high quality progressive material, 8 bits is not sufficient

FRExt Profiles

4:2:0/8 – BDF                                   "High" Agreed.

4:2:0/10 – LSDI, surveillance         "High 10" DROP?  DEFER?  No.  Keep.  Agreed.

4:2:2/8 – DROP THIS ONE.  Agreed.

4:2:2/10 – Primary studio profile   "Studio" (166% higher bit rate than 4:2:0/10)
	Agreed.

4:4:4/12 [Amendment 2?]               "4:4:4" Agreed. When?

Editing

JVT-L012 [Wiegand] Integrated FREXT input draft

Output of post-Munich editing process seems good.

Alpha

JVT-L016 [Sun] Verification of alpha channel coding

Verification of alpha proposal.  Perfect match.

JVT-L013 [Haskell] Addition of alpha channel to AVC/H.264 FRext

Proposes to distinguish alpha using NAL unit type.  Optional extra data.  Existing decoders would ignore the new NAL units.  Make alpha very similar to redundant picture (but require all MBs of picture to be sent), no redundant alpha pictures.

Discussion of informative blending description.  Sometimes polarity of "transparent" to "opaque" is reportedly switched.

e.g., provide TransparentValue and TransparentOpaque values, indicate whether a value is skipped and if so provide a SkippedValue.  If value is outside of range, interpret as nearest limit value.

Alpha pictures would always be the reference pictures for the decoding of the alpha (e.g., for direct prediction motion vector inference). Monochrome inferred.

Favorably disposed toward adoption of this.  Need to refine text.

Consider LS to SMPTE to inform them, assuming we do this.

Tuesday additional review:

Discussion of details of proposal.

Remark: Why is it necessary to constrain the use of "extra video" to alpha use only?  For example, why constrain the extra slices to monochrome format only?  Allow other color formats in alpha, send interpretation-specific stuff in an SEI.  Maybe that's getting too esoteric.

Remark: Does this sort of create new profiles?  Response: Any SEI-type thing has a similar effect – decoders choose whether to implement the extra capability or not, without sacrificing the fundamental interoperability of the PCP video.

Adopted.

Residual color transform

JVT-L014 [Sun] Residual color transform Using YCoCg-R

Improvement for 4:4:4 coding using a component transform prior to residual coding (or after when viewed from decoder perspective)

Verification of K018: similar performance improvements reported

Verification of L025: similar performance improvements reported

JVT-L025 [Kim] Residual color transform

Extension of K018

Results for 4:4:4 10 bit video 1920x1088:

-	10 Mbit/s: 0.5 dB gain

-	40 Mbit/s: 0.9 dB gain

-	100 Mbit/s: 1.2 dB gain

Support from the group. Need to see revised Amendment text for potential adoption.

Discussed more on Monday.

Deferred.

YCoCg vs. YCoCg-R: need to see revised Amendment text for potential adoption of YCoCg.

Lossless

JVT-L017 [Lee] Lossless coding for FRExt

Proposes changing semantics of 4x4 and 16x16 intra prediction in luma and 8x8 intra prediction in chroma.

Roughly 15% bit rate savings for intra lossless operation.

When inter is also used, the overall performance is much better, and the savings from this technique is 1-2%.

Verified by Sharp Labs. (slight difference for Foreman, but probably slightly different originals).

Verified by Samsung AIT for lossless.

Also tested with residual color transform.  The residual color transform helps.

Consider pixel-wise prediction issue.  How to do that editorially, and in an implementation pipeline.

Need 8x8 similar version of it as well, presumably.

Need to understand pipeline issue.  Define something like it using DPCM of residual, without change to the intra prediction process.

Revisited Monday.  Pipeline broken by using only five prediction modes – DC, horizontal, vertical, and two diagonals.  Approximately the same performance.

Remark: Would prefer to keep the prediction process unchanged regardless of whether this mode is in use or not, and avoid adding four new transforms.

Not adopted (at this meeting).

Quality scalability

JVT-L015 [Sun] (Prop. 2.2/3.1) Quality scalability for FRExt

Use spatial and SNR scalability concepts for extension of 8-bit 4:2:0 video to K-bit, 4:M:N video.  Define intra DC prediction to refer to prediction from another stream (e.g., and 8-bit stream).  Define relative position of lower-layer data, and type of encoding for it.  Performance reportedly improved relative to single-layer coding for some video.  Somewhat verified by Samsung.

Specify upsampling method.  In contribution, nearest-neighbor was used.

Samsung: Related to similar concept in JVT-L028.

Comment: Simulcast results?

Requires systems-level support to synchronize the two layers.

Specifying a profile would require what combinations of lower and upper-level capabilities would be needed in the profile.

Interesing topic for further study.  Can't adopt now – too difficult.

Sub-streams

JVT-L018 [Hannuksela, Wang] Sub-stream profile & level signaling

Can be used to signal conformance of the set of reference pictures in sequences, or the set of pictures defined as a sub-sequence.

Remark: Persistence across sequence boundaries.  We

Remark: Could be for trick modes.  If we want to do such work, we should try to do so in a more systematic fashion.

Remark: Would like to see more information provided for justification (studies, demos of usefulness in real systems.  Late to consider.

Remark: Kind of like it.  Favorably disposed.

Remark: Yes, but it seems just a quickie paper design at the moment.

Remark: Yes, but it also looks pretty simple and straightforward.

Not adopted.

Favorably disposed to considering such work in some hypothetical future other set of extensions, after more significant study has been conducted.

Low-delay random access simulcast

JVT-L019  [Kerofsky] (Prop. 2.2/3.1) Simulcast for low-delay random access

Demo of low-latency QCIF (intra 15 fps) with high-latency HD simulcast, so user would experience low-latency low quality.

Remark: Consider turning this into a conventional scalable design, so that the low-res video could be used also as a predictor for the high res video.

Remark: Could also be used as picture-in-picture.

Remark: Consider late entrant into a video-conference – similar problem.

Remark: Consider systems impact.  Where to solve this problem.

Remark: Consider relation to RRU concept.

Further study encouraged.  Good direction to explore.

CAVLC

JVT-L022 [Winger] CAVLC for 8x8 (JVT-E085 in JM_FREXT19a)

Geneva Oct 2002 JVT-E085 method of CAVLC ported into current document and software.  There was also JVT-E120 from NTT DoCoMo using the same method and also providing simulation results.

Allow lower-complexity encoding.

Enable future definition of a lower capability profile with CAVLC-only restriction that would produce bitstreams that could be decoded by the profiles we define now.

JVT-L036 [Suzuki] Proposal to support CAVLC for 8x8 transform

Confirmation of JVT-L022 / JVT-E085 / JVT-E120 method.  Exact match.  Works for MBAFF and PAFF as well as ordinary non-PITA cases.

Performance simulation results: There were such results for JVT-E085 / JVT-E120.  Relative to CABAC and relative to 4x4-only.  Perhaps provide some such results this week.

Monday: Limited test results provided on a couple of 1080p and 720p sequences for small and large QP ranges.  Not adapting transform size, but comparing all-8x8 vs. all-4x4.  Small QP on 1080 sequences (16-28), 8x8 better on some material (tested two sequences, got 11% on one, 4% on the other).  Not using quant matrices.  Used the quantization offsets in the reference software.  For 8x8 case, did not use smaller MC block sizes than 8x8.  Includes the (useless) overhead for the transform block size switching.  Seems sufficient to conclude that there is merit in the CAVLC for 8x8.  For 720p, two sequences in small QP range 8x8 was better by a couple of percent, and on a third it was worse by about 5%.  Any penalty for 8x8 would go to zero for using it adaptively, and the savings would only get better.

Adopted.

Stereo Video SEI

JVT-L023 [Karpinnen & Sun] SEI for stereo video in progressive sequences

Proposes extending stereo video SEI to progressive-scan use and something about

Per-frame SEI indication of whether frame is left or right.  Not signal at sequence level only.  Simplifies server detection of view.  Note: Cannot use fields in Baseline profile.

interlaced_frame_flag – semantics confusing.

Distinguish properly between output order and decoding order.

New timing stuff.

Allow same timestamp on different frames in clock timestamp?  This is already allowed, but is used as an indication of progressive refinement.

Horizontal sub-sampling of frames indication bit – unclear what that is needed for.

Remark: Is this stereo video stuff really within the scope of an SEI concept?  The video would look bad if you didn't interpret the SEI message.

Why do fields at all?  Why not just frames – isn't that more like what we're trying to actually indicate is happening in the video (i.e., no need for attempted redefinition of spatial location of fields)?  The idea of fields creates a natural association of fields with each other.  And the interlaced coding tools can be used to efficiently code fields.

Some aspects, as pointed out above, seem "not fully baked", but we could consider some modification of the current draft SEI with the general spirit of this proposal.  Request side activity to finalize exactly what is needed and consider again.

Revisited Monday and refined.  More work needed.  Deferred.

Residue sampling

JVT-L028 [Kim, Birinov, Kim] (Info) Additional data on residue sampling (JVT-K018)

Basic concept H.263 Annex Q applied to chroma only.  Similar concept in prior recent Thomson VCEG contribution.

Choice could be on MB, picture, or sequence basis.  This test used on whole sequence.  Proponent would like to consider doing it on an MB basis.

Used scanned film 4:4:4 sources.

Decoder invert spatial transform, then invert residue sampling, then invert residual color transform.

Can we really consider this to be 4:4:4 coding?  There is no ability to really code full-res for the chroma.

Comparison of YCoCg-R outside of loop to using this and residual color transform at the same time.  Significant gain shown.  How much of the gain is from each? Proponents says he doesn't really know, but that the coding gain appears at lower bit rate than for just doing residual color transform.

Remark: Consider other ways of reducing chroma fidelity, e.g., just increasing chroma step size or just zeroing out some high-frequency chroma coefficients.

Remark: Doesn't this kind of harm the mission of why you would be using 4:4:4 in the first place?  (blending, non-constant luma as result of linear processing on non-linear signals, etc.) Seems a little scary from that perspective.

Information document.  Interesting results.  Further study along these lines could potentially mature into a more effective coding design.

Inverse quantization

JVT-L029 [Wedi, Wittman] (Prop, Demo) Inverse quantization offset for FRExt

JVT-L032 [Tourapis, Boyce] (Prop, Demo) Quant offset matrices for FRext

JVT-L032 is a superset of JVT-L029.

Sent 8 signed offsets at sequence level.

luma 4x4 intra

luma 4x4 inter

luma 8x8 intra

luma 8x8 inter

Cb 4x4 intra

Cb 4x4 inter

Cr 4x4 intra

Cr 4x4 inter

Offsets are scaled by Qmatrix entry values.

JVT-L032, relative to JVT-L029, raises the following questions:

1)  JVT-L032 proposes, rather than 8 offsets as in JVT-L029, 8 matrices of 16/64 offsets.

2)  JVT-L032 proposes applying offset only to level +/-1, or to all levels, or with offset that reduces as levels get larger in magnitude.

3)  Slice level in addition to SPS level and PPS level.

Try to avoid losing some frequencies.

R-D gain shown for JVT-L032 relative to JM.

Comment on JM: Fixed value of f is not optimal.  Particularly change zero region versus other regions.  Current f = 1/6 not so good for high bit rates.  E.g., 0.45 might be good.

Defer.

Deblocking filter display SEI

JVT-L035 [Lu] DPB use for deblocking filter display pref SEI

Concerns the potential need for the decoder to have extra memory capacity to store the decoded unfiltered pictures.

A bit or a number?  Getting too elaborate.  Just one bit.

Output order HRD would need more memory for this than the output timing HRD.

Does this mean two numbers rather than one?  No, define it in terms of the output timing HRD.

Adopted

.

Bit depth issue analysis

JVT-L026 [Gsell] (Info) Extended bit depth characterization

Around 47 dB, 10-bit coding pulls away from 8 bit.  Around 57 dB, 12 bit pull away from 10 bit.

Differences greater than 2-5 dB

Artificial content – more important to support greater bit depth – more difference in coded quality between coding with different bit depths.

JVT-H016 has some older results.

JVT-L027 [Gish, Kim] (Info) Bit depth interoperability

Downward-rightward error propagation.  Upward rounding bias creates drift.  But thinks changing the rounding still would not fix it.  Seems to show that reduced bit-depth decoding is less useful/effective than we hoped.

Dynamic range analysis

JVT-L024 [Chen, Lu, Kashiwagi, Kadono, Lim] Transform dynamic range

Includes Qmatrix in dynamic range analysis for 8x8 case.  Main conclusion is that 16 bit storage with 32 bit intermediate results is sufficient for the 8x8 case.

Add any informative notes?  If QP gets too small, can the quantized coefficient value exceed a 16-bit range?  Is there any effective limit there?  Lowell, Jiuhuai, and Louis to confer on constraints and informative notes.

Quality study

JVT-L033 [Wedi, Kashiwagi] Subj. quality of FRExt for HD movie

IBBP, GOP size 12, open GOP? don't know.  Open GOP may not be in software.

Reports results of subjective tests done by BDF of FRExt 4:2:0/8 1920x1080x24p 3 clips.

Original 4.03, 20 Mbps 3.90, 16Mbps 4.00, 12 Mbps 3.71, 8M 3.65, DVHS MPEG-2@24 3.59.

Error bars not shown.

Nominal 3:1 advantage to MPEG-2.

JM 7.5b (May) – some bugs in B pictures in that version, and syntax for transform8x8_flag somewhat better now.

Test model

JVT-L034 [Karczewicz, Bao] (Info) Need for test model enhancements

Encourages test model enhancement work.  Example provided of need shows analysis of MCTF scalability schemes.  QP effectively decreased on the "low-pass" temporal pictures.  Big variations in QP from picture to picture.  Example, Paris, got 2 dB improvement by periodic reduction of QP.  Another dB can be obtained and variation in PSNR can be reduced by doing rate control on MB basis.

There may be (or may have been) some problems in JM implementation of B pictures for encoder.

Used hierarchical B picture structure with picture-to-picture QP variation like MCTF.  Got very similar PSNR results to MCTF proposals being analyzed in MPEG.

Comment: Time domain has traditionally been neglected in test model work.  QP has been neglected, except for trying to do rate control.  Dynamic GOP size, etc.

Comment: Some of these tricks create annoying subjective artifacts.  Some of those artifacts are reduced by update step of MCTF.  Depends on the sequence, though.

Shows importance of good JM (text and software).

JVT-L031 [Choi, Lee, Choi, Jeon] Performance of fast mode decision

Proponent not present.  Deferred.

Scalability

JVT-L037 [Karczewicz, Ridge] Extension to AVC-based scalable video

Removed from agenda; submitted to VCEG consideration.

Resolutions

Renaming of FRExt and its conformance and its reference software.

Question: Promote FRExt Soft and FRExt Conf. to CD very fast? [18-22 Oct.]

Starting a new corrigendum.

LS to BDF

LS to SMPTE.

LS to DVB? (they requested 4:2:2 in "AVC 072 rev1" response to CfP N5523)

Chang

Base ref soft FPDAM  FDAM (with editing period) part 5 amendment 6 [BUNDLED]

DoC

Base conformance FPDAM  FDAM (with editing period) part 4 amendment 6

DoC

FRExt text FPDAM  FDAM part 10 amendment 1

DoC

New edition

FRExt ref soft WD?  PDAM (negative editing period) part 5 amendment 8

FRExt conformance WD ? PDAM (with editing period) part 4 amendment 9

COR 2  WD ?

Request