| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 12th Meeting: Redmond, Washington USA July 19-23,2004 | Document: JVT-L014 Filename: input.doc |
| --- | --- |

| Title: | Residual Color Transform Using YCoCg-R |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shijun Sun 5750 NW Pacific Rim Blvd Camas, WA 98607, USA | Tel: Email: | 360-817-7531 ssun@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

# Introduction

# This contribution is a verification for the residue color transform proposal presented by Samsung in the previous meetings [1]; in the meanwhile, it proposes a simple modification to further improve the coding performance.

# In earlier proposals [1], Samsung introduced a color conversion step into the coding loop for prediction residuals, and has demonstrated promising results. However, the color conversion process used in [1], i.e., YCoCg [2], is a lossy transform; in other words, the residual color conversion method can not be used for lossless since the inverse YCoCg transform can not reconstruct the original RGB values. So, to achieve lossless coding performance, the residuals color transform will have to be skipped.

In this proposal, we replaced the YCoCg color conversion process with the reversible color conversion YCoCg-R [2]. Therefore, the lossless coding case can also benefit from the residual color conversion technique. In the same time, there is no negative impact on the coding performance for the lossy coding case. Since YCoCg-R color conversion introduces one more bit for each of the Co and Cg component, an important change need be made for the lossy coding to the quantization process of the Co/Cg components; the corresponding chrominance quantization parameter should be increased by “6” to balance this intermediate bit depth extension.

## Experiments

For a group of HD resolution materials, the lossless compression ratio is improved significantly with the in-loop YCoCg-R color conversion. Since the in-loop color conversion is optional, it is always possible to turn it off for some video data, where direct-RGB coding can give better results. The sequences are coded as IPPP.

In the same time, the YCoCg-R conversion does not introduce any negative impact on the coding performance for the lossy coding case. As shown in the RD curves for the Y component, the coding performance difference between in-loop YCoCg and in-loop YCoCg-r is very small for the lossy cases. YCoCg-R tends to be a little better when the bitrate is high (or the quantization parameter is small). And, they are both much better than the direct YCoCg-R coding without any residual color conversion. The sequences are coded as IBBP for the lossy test at QP 18, 24, and 30. The PSNR values were calculated in the YCoCg-R color space, which is a rough approximation of YCbCr we are used to.

## Discussion

The experiments first verified that the residual color conversion (with either YCoCg or YCoCg-R) performs significantly better than the direct YCoCg coding and secondly showed that the residual color conversion with YCoCg-R can be beneficial to the lossless coding mode. (The reversible color transform has been applied by Samsung in their new proposal to be presented in the Redmond meeting.)

It is requested that the JVT adopt the residue color transform with YCoCg-R into the FRExt.

## References

- [1] W.-S. Kim, D. Birinov, and H. M. Kim, “Adaptive Residual Transform and Sampling,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-K018, March 2004.

- [2] H. Malvar and G. Sullivan, “YCoCg-R: a color space with RGB reversibility and low dynamic range,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-I014, Sep. 2003.

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Organization name |  | Sharp Labs of America |  |  |  |  |
| Mailing address |  | 5750 NW Pacific Rim Blvd Camas, WA 98683 |  |  |  |  |
| Country |  | USA |  |  |  |  |
| Contact person |  | Dr. Shijun Sun |  |  |  |  |
| Telephone |  | 360-817-7531 |  |  |  |  |
| Fax |  | 360-817-8436 |  |  |  |  |
| Email |  | ssun@sharplabs.com |  |  |  |  |
| Place and date of submission |  |  |  |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |  |  |  |
| Name (ex: “JVT”) |  |  |  | JVT |  |  |
| Title |  |  |  |  |  |  |
| Contribution number |  |  |  |  |  |  |
| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |  |  |  |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |  |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |  |  |  |
| x | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |  |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |  |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |  |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |  |  |  |
| Patent number(s)/status |  |  | Provisional application |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |  |  |  |
| Relevance to JVT |  |  |  |  |  |  |
| Any other remarks: |  |  |  |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |  |  |  |

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| x | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  | Samsung; Microsoft |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: