| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 12th Meeting: Redmond, USA, 19-23 July, 2004 | Document: JVT-L024 Filename: input.doc |
| --- | --- |

| Title: | Dynamic Range of Outcome of Inverse Transform with Scaling Matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tao Chen Jiuhuai Lu Yoshiichiro Kashiwagi Panasonic Hollywood Laboratory 100 Universal City Plaza Universal City, CA 91608, USA Shinya Kadono Matsushita Electric Industrial Co., Ltd. 1006, Kadoma Kadoma City Osaka, 571-85001, Japan Chong-Soon Lim Panasonic Singapore Labs Block 1022 Tai Seng Avenue, #04-3530 Singapore 534415 | Tel: Email: | +1 818-777-6175 chent@research.panasonic.com jiuhuai@research.panasonic.com kashiwagiy@research.panasonic.com kadono.shinya@jp.panasonic.com cslim@psl.com.sg |
| Source: | Panasonic/ Matsushita Electric Industrial Co., Ltd. |  |  |

_____________________________

# Introduction

This document discusses the dynamic range of outcome of inverse transform with use of scaling matrices [1]. Both 8x8 and 4x4 transform sizes will be considered. The main focus is on the decoding process since the inverse transform and scaling at the decoder is normative, while some results at the encoder side will also be discussed.

- Dynamic range in 8x8 case

In the case of 8x8 transform without scaling matrices, we have the following forward/inverse transform and scaling processes (specifics on transform and scaling are referred to [1,2]).

A=T⋅XB=Y⋅TTC=q⋅BD=C>>(16+n)E=(D⋅r)<<(n−2)F=E⋅TG=TT⋅FH=G>>6

Analysis on the dynamic range in this case can be found in [3]. When an 8x8 scaling matrix is present [4], the decoding part can be rewritten as follows:

E=(D⋅r')>>(6−n)F=E⋅TG=TT⋅FH=G>>6

Here r’=r*w(i,j), where w(i,j) denotes the 8-bit scaling matrix entry specified for position (i,j) and it is in the range of [1, 255]. Note that, in calculating E, the right-shift operation shall become a left-shift when n>6. Single formula is used here and thereafter just for simplicity.

Based on the above formula, the case where all w(i,j)=16 is equivalent to scaling without matrix

. Note that a constraint on the smallest value of scaling matrix entries shall be imposed in the case of small QP. This is to ensure the dynamic range of scaled coefficients be within 16 bits for 8-bit video input. For 10-bit and 12-bit video input, the dynamic range of results shall increase by 2 and 4 bits, respectively.

In the following, we consider the dynamic range (the maximum values of results) of two cases for different coefficient position (i,j) and analysis for other cases can be derived in a similar way. We assume that the input video is 8-bit unsigned. Tables 1~3 show the results for different values of matrix entry w(i,j) where i=[0,4] & j=[0,4]. Table 4 shows the analysis for the case where i=[2,6] & j=[2,6].

Table 1 Dynamic range for the case that i=[0,4] & j=[0,4] with w(i,j)=16

| Output | dynamic of operation | resulting range [bits] |
| --- | --- | --- |
| E = (D  r’) >> (6-n) | 12.7 + 4.3 + 4 –6 | 15 (15) |
| F = E  T | 15 + 3 – 3 | 15 (15) |
| G = TT  F | 15 + 3 – 3 | 15 (15) |
| H = G >> 6 | 15 – 6 | 9 (9) |

Table 2 Dynamic range for the case that i=[0,4] & j=[0,4] with w(i,j)=2

| Output | dynamic of operation | resulting range [bits] |
| --- | --- | --- |
| E = (D  r’) >> (6-n) | 15.7 + 4.3 + 1 –6 | 15 (15) |
| F = E  T | 15 + 3 – 3 | 15 (15) |
| G = TT  F | 15 + 3 – 3 | 15 (15) |
| H = G >> 6 | 15 – 6 | 9 (9) |

Note that in this case, setting w(i,j)=1 can result in 16.7 (17) bits for the scaled coefficients. As a consequence, in the case of small QP values (e.g., less than 6), an encoder shall avoid using small values (e.g., less than 2) for matrix entries.

Table 3 Dynamic range for the case that i=[0,4] & j=[0,4] with w(i,j)=255

| Output | dynamic of operation | resulting range [bits] |
| --- | --- | --- |
| E = (D  r’) >> (6-n) | 8.7 + 4.3 + 8 –6 | 15 (15) |
| F = E  T | 15 + 3 – 3 | 15 (15) |
| G = TT  F | 15 + 3 – 3 | 15 (15) |
| H = G >> 6 | 15 – 6 | 9 (9) |

Table 4 Dynamic range for the case that i=[2,6] & j=[2,6] with w(i,j)=16

| Output | dynamic of operation | resulting range [bits] |
| --- | --- | --- |
| E = (D  r’) >> (6-n) | 12.6 + 5 + 4 –6 | 15.6 (16) |
| F = E  T | 15.6 + 2.7 – 3 | 15.3 (16) |
| G = TT  F | 15.3 + 2.7 – 3 | 15 (15) |
| H = G >> 6 | 15 – 6 | 9 (9) |

Note that, in the above cases, the dynamic range of intermediate results (prior to the right-shift operation) in the calculation of E is up to 22 bits for good and compliant bitstreams. For 12-bit video, the dynamic range of intermediate results of inverse scaling is still within 32 bits for good and compliant bitstreams.

- Dynamic range in 4x4 case

The analysis for 4x4 transform without scaling matrices is referred to [5]. For the first level transform, Table 1 of [5] shows that the dynamic range of the scaled coefficients is 11.7 (12) bits. In the case of using scaling matrices (whose entries take on values in [1,255]), the dynamic range of scaled coefficients will be 15.7 (16) bits and 7.7 (8) bits when w(i,j)=1 and w(i,j)=255, respectively. The inverse scaling with matrix results in the same dynamic range of inverse scaled values as in the case where scaling matrix is not used, which is 15.6 (16) bits as shown in Table 1 of [5]. The dynamic ranges for the results of other operations remain same regardless of the presence of scaling matrix.

As for the second level transform in the case of 16x16 intra mode, the dynamic range of DC components is increased by 1 bit for 2x2 chroma DC transform and 2 bits for 4x4 luma DC transform, respectively. The dynamic ranges of results before scaling and after inverse scaling remain the same regardless of the presence of scaling matrix. For matrix entry w(0,0), a value of 1 results in an increase of 4 bits for dynamic range of scaled coefficients in the second level transform. Therefore, in the case of small QP values, an encoder shall avoid using small values for matrix entry w(0,0). For example, w(0,0) shall be not less than 2 when 5<QP<12 and not less than 4 when QP<6. Such a constraint shall not have impact in practical video compression applications, since for example, w(i,j)=4 with QP<6 actually results in expansion rather than compression of transform coefficients.

- Discussion

Concerning the 16-bit requirement for inverse transform and scaling, the following note shall be taken to avoid the case that scaled coefficient values exceed 16 bits:

“Care should be used in the design of encoders to avoid difficulty with meeting the dynamic range requirements of the decoding process when using scaling matrix, particularly avoiding use of small values for matrix entries.”

References:

- T. Wiegand, “Version 3 of H.264/AVC”, JVT-L012d1.doc, July 17-23, 2004.

- S. Gordon, et al., “Simplified use of 8x8 transforms – proposal”, JVT-J029.doc, December 8-12, 2003.

- T. Chen, et al., “Dynamic Range of Outcome of Simplified 8x8 Transform and Quantization/De-quantization”, JVT-K031.doc, March 15-19, 2004.

- J. Lu, T. Chen, Y. Kashiwagi, and S. Kadono, “Proposal of quantization weighting for H.264/MPEG-4 AVC Professional Profiles,” JVT-K029r1.doc, March 2004.

- L. Kerofsky, “Notes on JVT IDCT”, JVT-C24.doc, May 6-10, 2002.

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person | Tao Chen |  |  |
| Telephone | +1 818-777-6175 |  |  |
| Fax |  |  |  |
| Email | chent@research.panasonic.com |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| x | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: