| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 12th Meeting: Redmond, WA, USA, 17-23 July, 2004 | Document: JVT-L030 Filename: input.doc |
| --- | --- |

| Title: | Definition of a 4:2:0 / 8-bit Profile for H.264/AVC FRExt |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Thomas Wedi Panasonic European Laboratories Monzastr. 4c 63225 Langen, Germany Teruhiko Suzuki Sony Corp. 6-7-35, Kitashinagawa Shinagawa-ku, Tokyo 141-0001, Japan Yoshihisa Yamada Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura City, Kanagawa 247-8501, Japan Takuya Imaide Ubiquitous Platform Systems R&D Laboratory, Hitachi Ltd. 292 Yoshida-cho, Totsuka-ku, Yokohama 244-0817 Japan Tom McMahon Dolby Laboratories 3601 West Alameda Avenue Burbank, CA 91505 USA Stijn de Waele Philips Research Laboratories Prof. Holstlaan 4 5656 AA Eindhoven, The Netherlands Byeong-Moon Jeon LG Electronics Inc. 360-5 Yatap-Dong, Bundang-Gu, Sungnam-Si, Kyunggi-Do, Korea Stephen Gordon Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA Norio Ito Sharp Corp. 1-9-2 Nakase, Mihama-ku Chiba, Chiba 261-8520, Japan | Tel: Email: Tel: Email: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +49 6103 766 1194 wedi@panasonic.de +81 3 5795 7966 teruhiko@av.crl.sony.co.jp yosihisa@isl.melco.co.jp +81-45-866-6456 imaide@msrd.hitachi.co.jp +1-818-823-2855 Tom@Dolby.Com +31 40 27 43321 stijn.de.waele@philips.com +82-31-789-4240 jeonbm@lge.com 978.475.8990 x222 sgordon@broadcom.com +81 43 299 8696 itoh.norio@sharp.co.jp |
| Source: | Panasonic, Sony, Mitsubishi, Hitachi, Dolby, Philips, LG Electronics, Broadcom, Sharp |  |  |

_____________________________

# Background

New coding tools such as an 8x8 transform and quantization matrix have been adopted in H.264/AVC Fidelity Range Extensions (FRExt) Amendment. These tools lead to a significant subjective picture quality improvement and coding efficiency gain, especially for HD content.

Currently, H.264/AVC FRExt defines four profiles: 4:2:2 / 8-bit, 4:2:0 / 10-bit, 4:2:2 / 10-bit, and 4:4:4 / 12-bit. Although all of these profiles implicitly include 4:2:0 8-bit support, there is no profile that only supports 4:2:0 / 8-bit.

Due to its significant coding efficiency gain and subjective picture quality improvement, H.264/AVC FRExt is a strong candidate for various consumer applications, many of which will be based on 4:2:0 / 8-bit image format. However, the use of one of the above mentioned existing profiles for those applications would unnecessarily increase implementation complexity, and therefore cause delay and raise development costs for associated products. In addition, the lack of the conformance in the current FRExt draft for those applications may introduce interoperability problems to the marketplace.

# Proposal

We propose a separate profile in H.264/AVC FRExt that only supports 4:2:0 / 8-bit.

This profile should include all tools from the Main profile in addition to the new FRExt tools such as the 8x8 transform and quantization matrix. Furthermore, we propose to use the same Levels as defined for Main profile.