| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) JVT Meeting: Redmond, WA, USA, 17-23 July, 2004 | Document: JVT-L036 Filename: JVT-L036.doc |
| --- | --- |

| Title: | Proposal to support CAVLC for 8x8 transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki and Yoichi Yagasaki 6-7-35 Kitashinagawa, Shinagawa-ku, Tokyo, 141-0001, Japan | Tel: Email: | +81-3-5795-7966 teruhiko@av.crl.sony.co.jp |
| Source: | Sony Corp. |  |  |

_____________________________

# Introduction

In this contribution, we propose to add CAVLC entropy coding for FRExt 8x8 DCT with the purpose of enabling software decoding which maybe popular in the near future such as the PC environment. CAVLC could be a good solution for real time software decoder.

The method used is exactly the same as what proposed in JVT-E120, which was verified by several companies in Geneva 2002. We incorporate the method into JMFRExt19 software and show the simulation result in the following.

# Simulation

## Encoder Options

Software : JMFRExt19 + JVT-E120

GOP Structure: IBBP (N15M3)

Entropy coding: CAVLC

Fi/Fr: Frame fixed

Search Range: +-32

Ref: 2

RDOpt: On

Transform Type: 8x8 fixed

60 frames for 720p sequences

## Result

The results are as follows.

|  | QP | SNR Y (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- | --- |
| JM19 CAVLC | 40 |  |  |  |  |
|  | 36 |  |  |  |  |
|  | 32 |  |  |  |  |
|  | 28 |  |  |  |  |
| JM19 CABAC | 40 |  |  |  |  |
|  | 32 |  |  |  |  |
|  | 28 |  |  |  |  |
|  | 24 |  |  |  |  |

Table 1: Comparison of CAVLC/CABAC (Crew)

|  | QP | SNR Y (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- | --- |
| JM19 CAVLC | 40 |  |  |  |  |
|  | 36 |  |  |  |  |
|  | 32 |  |  |  |  |
|  | 28 |  |  |  |  |
| JM19 CABAC | 40 |  |  |  |  |
|  | 32 |  |  |  |  |
|  | 28 |  |  |  |  |
|  | 24 |  |  |  |  |

Table 2: Comparison of CAVLC/CABAC (Shuttlestart)

|  | QP | SNR Y (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- | --- |
| JM19 CAVLC | 40 |  |  |  |  |
|  | 36 |  |  |  |  |
|  | 32 |  |  |  |  |
|  | 28 |  |  |  |  |
| JM19 CABAC | 40 |  |  |  |  |
|  | 32 |  |  |  |  |
|  | 28 |  |  |  |  |
|  | 24 |  |  |  |  |

Table 3: Comparison of CAVLC/CABAC (Jets)

|  | QP | SNR Y (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- | --- |
| JM19 CAVLC | 40 |  |  |  |  |
|  | 36 |  |  |  |  |
|  | 32 |  |  |  |  |
|  | 28 |  |  |  |  |
| JM19 CABAC | 40 |  |  |  |  |
|  | 32 |  |  |  |  |
|  | 28 |  |  |  |  |
|  | 24 |  |  |  |  |

Table 4: Comparison of CAVLC/CABAC (City)

# Conclusions

The modification of supporting the above CAVLC for FRExt 8x8 DCT method is very few no matter relating the spec or the true decoder. But as shown in the whole contribution, the performance is acceptable.

As a solution for real time software decoder for HDTV, CAVLC should be supported for 8x8 transform as well as 4x4 transform.

The software used for simulation can be found at JVT-L036soft.zip

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sony Corp. |  |  |
| Mailing address | 6-7-35, Kitashinagawa, Shinagawa-ku, Tokyo, 141-0001, JAPAN |  |  |
| Country | Japan |  |  |
| Contact person | Teruhiko Suzuki |  |  |
| Telephone | +81-3-5795-7966 |  |  |
| Fax | +81-3-5795-8563 |  |  |
| Email | Teruhiko@av.crl.sony.co.jp |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Proposal to support CAVLC for 8x8 transform |  |
| Contribution number |  | JVT-L036 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| X | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: