| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 12th Meeting: Redmond, WA, USA 17-23 July, 2004 | Document: JVT-L060 Filename: input.doc |
| --- | --- |

| Title: | Liaison statement to ITU-T SG 9 (Television and Sound Transmission) on FRExt Amendment to ITU-T Rec. H.264 ¦ ISO/IEC 14496-10 Advanced Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Approved by JVT; 23 July 2004 |  |  |
| Purpose: | Liaison letter |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corporation One Microsoft Way Redmond, WA 98052 USA Thomas Wiegand Heinrich Hertz Institute (FhG), Einsteinufer 37, D-10587 Berlin, Germany Ajay Luthra Motorola Corporation 6420 Sequence Drive San Diego, CA 92121 USA | Tel: Fax: Email: Tel: Fax: Email: Tel: Fax: Email: | +1 (425) 703-5308 +1 (425) 706-7329 garysull@microsoft.com +49 - 30 - 31002 617 +49 - 30 - 392 72 00 wiegand@hhi.de +1 (858) 404 3470 +1 (858) 404 2501 aluthra@motorola.com |
| Source: | JVT |  |  |

_____________________________

The Joint Video Team (JVT) of ITU-T SG16 Question 6 (VCEG) and ISO/IEC JTC 1/SC 29/WG 11 (MPEG) would like to inform you of the completion of our project for the development of extensions to the Advanced Video Coding standard (ITU-T H.264 & ISO/IEC 14496-10).  The development of these extensions, now called the "Fidelity Range Extensions" (FRExt), was completed during the Redmond JVT meeting of 17-23 July 2004.

Four new profiles were added to the standard in this set of extensions, all of which support the full set of capabilities of the existing Main profile, plus an 8x8 transform and encoder-specified quantization weighting matrices.  The new profiles form an "onion-like" nesting of capabilities relative to each other and the existing Main profile.  These four new profiles are summarized in the table below.  Using the new FRExt extensions, the ability for an encoder to optionally send a high-quality compressed alpha channel is also supported.  Additionally, these extensions support analysis/synthesis representation of film grain characteristics.

| Profile Name | Maximum Chroma Format | Maximum Sample Precision |
| --- | --- | --- |
| "High" (HP) | 4:2:0 | 8 bits |
| "High 10" (Hi10P) | 4:2:0 | 10 bits |
| "High 4:2:2" (Hi422) | 4:2:2 | 10 bits |
| "High 4:4:4" (Hi444) | 4:4:4 | 12 bits |