| # Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznan, Poland, 25-29 July, 2005 | Document: JVT-P013 Filename: input.doc |
| --- | --- |

| Title: | Deblocking Filter for I_BL Blocks in Spatial Scalable Video Coding – Response to CE2 Part 2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Source: | Shijun Sun Sharp Labs of America 5750 NW Pacific Rim Blvd. Camas, WA 98607, USA | Tel: Fax: Email: | 1-360-817-7531 1-360-817-8436 ssun@sharplabs.com |
| Source: | Sharp |  |  |

Abstract – This contribution is a response to CE2 part 2.  It demonstrates a simplification of deblocking filter for macroblocks in I_BL mode in spatial scalable video coding.  By treating the I_BL blocks as inter blocks during boundary strength decision, the deblocking filter operations for I_BL blocks are significantly reduced (34% on average) while with acceptable visual quality.

# 1. Introduction

In the current SVC design, all block boundaries related to an I_BL macroblock will have to be filtered, i.e., with Bs = 1 or 4.  The method has two drawbacks when being applied to spatial scalable coding: (1) the prediction from a lower resolution layer can be unnecessarily blurred and therefore (2) the process unnecessarily spends more computational cycles.

At the Busan meeting, contribution JVT-O008 proposed a change to the loop filter during filter strength decision for a block in I_BL mode.  The change is to be applied to spatial scalable coding, i.e., when SpatialScalabilityType is not equal to 0.  The purpose is to reduce the computational complexity and avoid blurring the prediction from the base layer, which has been filtered (following the process defined in S.8.6.3.1 of JSVM2-Annex-S).

In a sense, the I_BL mode could be considered as an Inter mode so the filtering decision should also be consistent with the existing AVC design.  Therefore, for a block in I_BL mode, the Bs of a boundary between the block and a neighboring block shall be derived as follows.

- If the neighboring block is intra-coded but not in I_BL mode, the Bs = 4.

- Otherwise, if any of the adjacent blocks has coefficient, the Bs = 2.

- Otherwise, if the neighboring block is not in I_BL mode based on the same base layer picture, the Bs = 1.

- Otherwise, Bs = 0.

### 2. Proposed modification for the WD-2 De-blocking Filter Process

#### The following description in S.8.7.2.1 of WD-2

- If the samples p0 and q0 are both in macroblocks coded using the I_BL macroblock prediction mode, a value of bS equal to 1 shall be the output.

shall be replaced by

- If SpatialScalabilityType is not equal to 0 and either luma sample p0 or q0 is in macroblocks coded using the I_BL macroblock prediction mode, the variable bS is derived as follows.

- If either luma samples p0 or q0 is in a macroblock coded using an intra prediction mode other than the I_BL mode, a value of bS equal to 4 shall be the output.

- Otherwise, if one of the following conditions is true, a value of bS equal to 2 shall be the output.

- the luma block containing sample p0 or the luma block containing sample q0 contains non-zero transform coefficient levels.

- the syntax element nal_unit_type is equal to 20 and residual_prediction_flag is equal to 1 for the luma block containing sample p0 or the luma block containing sample q0 and the prediction array resPredX as derived in subclause S.8.5.14 contains non-zero samples, with X indicating the applicable component L, Cb, or Cr.

- Otherwise, if one of the following conditions is true, a value of bS equal to 1 shall be the output.

- either luma samples p0 or q0 is in a macroblock coded using an inter prediction mode.

- the luma samples p0 and q0 are in two separate slices with different base_id_plus1.

- Otherwise, a value of bS equal to 0 shall be the output.

- Otherwise, if the samples p0 and q0 are both in macroblocks coded using the I_BL macroblock prediction mode, a value of bS equal to 1 shall be the output.

3. Experiments (preliminary)

We compared the performance of the proposed method to the existing method in the JSVM software.  Experiments are conducted following the Munich test points.

Table 1: Performance of the proposed I_BL filter decision

The performance of the proposed method is summarized in Table 1.  The “filter count” indicates the number of 4x4 block boundaries related to I_BL blocks with Bs > 0.  Depending on the sequences, the “filter count” is reduced by on average 34% using our proposed method.  In the meanwhile, there is no significant impact on subjective and objective (as shown in the separate excel file) image qualities.

4. Conclusion

In summary, the proposed filtering decision method for I_BL blocks is very effective in reducing the number of filtering operations.  Therefore, we propose that JVT adopt the method into the SVC extension.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sharp |  |  |
| Mailing address | 5750 NW Pacific Rim Blvd Camas, WA 98607 USA |  |  |
| Country | USA |  |  |
| Contact person | Shijun Sun |  |  |
| Telephone | 1-360-817-7531 |  |  |
| Fax | 1-360-817-8436 |  |  |
| Email | ssun@sharplabs.com |  |  |
| Place and date of submission | Busan, Korea (April 2005) |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Extended Spatial Scalability with Picture-Level Adaptation |  |
| Contribution number |  | JVT-O008 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  | Provisional filing (Sharp) |  |
| Inventor(s)/Assignee(s) |  | S. Sun/Sharp |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: