| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznan, Poland, 24-29 July, 2005 | Document: JVT-P062 Filename: JVT-P062.doc |
| --- | --- |

| Title: | Indication of non-required pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Nokia Research Center Visiokatu 1 33720 Tampere, Finland Yiliang Bao Nokia Research Center 6000 Connection Dr. MS 3-4-1400 Irving, TX 75039 | Tel: Email: | +358 50 486 7004 ye-kui.wang@nokia.com +1 972 374 1369 yiliang.bao@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Introduction

This contribution proposes a new SEI message for indication of non-required pictures before their decoding, such that the decoder can avoid decoding and buffering of the non-required picture while at the same time low delay is allowed.

# Non-required pictures in scalable video streams

In a scalable video bitstream, for an enhancement layer picture, a picture from whichever lower layer may be selected for inter-layer prediction. Accordingly, if the video stream includes multiple scalable layers, it may include, in intermediate layers, some pictures that are not needed in decoding and playback of an entire higher layer. Such pictures are called as non-required pictures (for decoding and playback of the entire higher layer).

A Typical inter-layer prediction dependency hierarchy is shown in Fig. 1, wherein an arrow indicates that the pointed-to object uses the pointed-from object for inter-layer prediction reference, and the pair of values in the right of each layer in the figure represents the values of (dependency_id and quality_level) as specified in the JSVM 2. There are three scalable layers shown in Fig. 1, base_layer_0, CGS_layer_1, and spatial_layer_2, which have the same frame rate.

The picture in spatial_layer_2 in Fig. 1 may also select to use the picture base_layer_0 for inter-layer prediction, as shown in Fig. 2. Further, it is possible that a picture in spatial_layer_2 selects to use base_layer_0 for inter-layer prediction while at the same temporal location the picture in CGS_layer_1 selects not to have inter-layer prediction at all, as shown in Fig.3.

When FGS layers are involved, the inter-layer prediction for coding mode and motion information may come from a different base layer than the inter-layer prediction for sample residual. For example, in Fig.4, for the spatial_layer_2 picture, the inter-layer prediction for coding mode and motion comes from the CGS_layer_1 picture while the inter-layer prediction for sample residual comes from the FGS_layer_1_1 picture. For another example, in Fig.5, for the spatial_layer_2 picture, the inter-layer prediction for coding mode and motion still comes from the CGS_layer_1 picture while the inter-layer prediction for sample residual comes from the FGS_layer_1_0 picture.

In the examples shown in Figs. 2 and 3, to decode and playback the shown picture in spatial_layer_2, decoding of the shown picture in CGS_layer_1 is not needed. If the shown CGS_layer_1 picture is also not needed in decoding of all the pictures that are in spatial_layer_2 and succeeding the shown spatial_layer_2 picture in decoding order, then decoding and playback of the entire spatial_layer_2 do not need the shown CGS_layer_1 picture. For example, the above condition holds if the shown CGS_layer_1 picture is a non-reference picture. This is also the case for the shown picture in FGS_layer_1_1 in Fig. 5. Such pictures are examples of non-required pictures (for decoding of the entire spatial_layer_2).

The problem is that there lacks a way to indicate such dependency information before decoding of the non-required pictures. Consequently, the decoder has to decode the non-required pictures (this is wasteful in computation load) and buffer the corresponding decoded pictures (this is wasteful in memory consumption).

An alternative way exists only if the non-required picture is a non-reference picture. Such alternative is to wait for the arrival of the picture at that temporal location in the scalable layer desired for playback, and then parse the dependency information to know which lower-layer pictures are non-required pictures. However, this causes increased end-to-end delay that is not friendly to low-delay applications and not acceptable to conversational applications, and the alternative does not work if the non-required picture is a reference picture. If a picture in a lower layer L1 is a reference picture, even it is not required in decoding of a picture in a higher layer L2 at the temporal location T1, it may still have been used for inter prediction of a succeeding layer L1 picture, which may have been used for inter-layer prediction of the layer L2 picture at the temporal location T2. In this case, the layer L1 picture at the temporal location T1 is not a non-required picture of layer L2.

# Proposal

To solve the problem stated above, we proposes a mechanism to indicate the information of non-required pictures before their decoding, such that the decoder can avoid decoding and buffering of the non-required picture while at the same time low delay is allowed. The indication of the information is signaled using a new SEI message called Non-required Picture Information SEI message. The syntax and semantics are given in the following.

## Non-required picture SEI message syntax

| non_required_picture_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| num_info_entries_minus1 | 5 | ue(v) |
| for ( i = 0; i <= num_info_entries_minus1; i++ ) { |  |  |
| entry_dependency_id[ i ] | 5 | u(3) |
| num_non_required_pics_minus1[ i ] | 5 | ue(v) |
| for ( j = 0; j <= num_non_required_pics_minus1[ i ]; j++ ) { |  |  |
| non_required_pic_dependency_id[ i ][ j ] | 5 | u(3) |
| non_required_pic_quality_level[ i ][ j ] | 5 | u(2) |
| } |  |  |
| } |  |  |
| } |  |  |

## Semantics

The information conveyed in this SEI message concerns an access unit. When present, this SEI message shall appear before any coded slice NAL unit or coded slice data partition NAL unit of the corresponding access unit.

num_info_entries_minus1 plus 1 indicates the number of the information entries following this syntax element. The value shall be in the range of 0 to 7, inclusive.

entry_dependency_id[ i ] indicates the dependency_id value of the target picture whose information of non-required pictures is described by the following syntax elements. The instances of entry_dependency_id[ i ] shall appear in the increasing order of their values. The quality_level value of the target picture is always zero. A non-required picture of the target picture is not required in decoding of any other pictures in the coded video sequence and having the same dependency_id value and quality_level value as the target picture.

Note: A picture having quality_level larger than 0 is a FGS picture whose inter-prediction reference source is always fixed. Therefore, a FGS picture’s non-required pictures are the same as the picture having the same dependency_id value as the FGS picture and quality_level equal to 0.

num_non_required_pics_minus1[ i ] plus 1 indicates the number of non-required pictures signaled in the current entry for the target picture having the dependency_id value equal to entry_dependency_id[ i ] and the quality_level value equal to 0. The value shall be in the range of 0 to 30, inclusive.

non_required_pic_dependency_id[ i ][ j ] indicates the dependency_id value of the j-th non-required picture signaled in the current entry for the target picture having the dependency_id value equal to entry_dependency_id[ i ] and the quality_level value equal to 0.

non_required_pic_quality_level[ i ][ j ] indicates the quality_level value of the j-th non-required picture signaled in the current entry for the target picture having the dependency_id value equal to entry_dependency_id[ i ] and the quality_level value equal to 0. In addition, those pictures that have dependency_id equal to non_required_pic_dependency_id[ i ][ j ] and quality_level larger than non_required_pic_quality_level[ i ][ j] are also non-required pictures for the same target picture.

Besides the non-required pictures explicitly signaled in the SEI message, the following rules shall be applied to derive additional non-required pictures:

- If a picture having dependency_id equal to A is not a non-required picture for the picture having dependency_id equal to B wherein B is larger than or equal to A, then all the non-required pictures for the picture having dependency_id equal to A are also non-required pictures for the picture having dependency_id equal to B.

- If the layer desired for playback has dependency_id equal to C that is not equal to any of the signaled entry_dependency_id[ i ] values, the n-th entry that has the largest entry_dependency_id[ i ] smaller than C is searched for. The picture having dependency_id equal to C shall have the same set of non-required pictures as the picture having dependency_id equal to the entry_dependency_id[ i ] of the n-th entry and quality_level equal to 0. If there is no entry that has entry_dependency_id[ i ] smaller than C, then there are no non-required pictures in the associated access unit for the picture having dependency_id equal to C.

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Corporation |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission | Poznan, Poland, 24-29 July, 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | ITU-T Rec. H.264 ¦ ISO/IEC 14496-10 |  |
| Title |  | Indication of non-required pictures |  |
| Contribution number |  | JVT-P062 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: