| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznań, PL, 24-29 July, 2005 | Document: JVT-P074 Filename: JVT-P074.doc |
| --- | --- |

| Title: | CE 10 : Verification Report of Thomson proposal [JVT-P019] |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Seung-Wook Park, Doe Hyun Yoon, Ji-Ho Park, Byeong-Moon Jeon, DM research lab., LG Electronics Inc., 16 Woomyeon-Dong, Seocho-Gu, Seoul 137-724, Korea | Tel: Email: | + 82 - 2 – 526 - 4369 beatitude@lge.com dhyoon@lge.com scottie@lge.com jeonbm@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Overview

The main purpose of this document is to verify the performance of the CE 10[1] related to improving upsampling process applied in Extended Spatial Scalability (ESS) as described in the JSVM2. In this document, we focus on verifying the results of Thomson’s proposal (JVT-O041/O042) of CE 10. The proposal consists in providing a complexity reduction by fully unifying ESS_3_2 with a limited enhancement / base layer ratio of 3/2 and ESS_generic supporting any ratio. The three ratios of 3/4, 2/3 and 3/5 are considered and two solutions are compared for each of those ratios.

- Generic solution: previous ESS solution which was already implemented in JSVM2.0.

- Unified solution (proposal 2 in CE10): new upsampling motion solution which unified ESS_3_2 and ESS_generic

# Verification process

The bitstreams, binary files and reference sequences were provided by Thomson. And these bit-streams were extracted for the test points defined in the CE 10 description. Then each bitstreams were decoded for every target points. Lastly, PSNR values have been calculated and checked with the results provided by Thomson.

# Verification results

  - Experimental settings

CE10 defined new sequences sizes and ratios as well as target bitrates. These conditions are summarized in the table 1.

Table 1  Test sequences and the corresponding bitrates and size for each ratio

| Ratio Low/High ¾ | bitrates | Low - 528*432@30HZ | High - 704*576@30HZ |
| --- | --- | --- | --- |
|  | City | 810 kbps | 1024 kbps |
|  | Crew, Harbour, Soccer | 1190 kbps | 1500 kbps |
| Ratio Low/High 2/3 | bitrates | Low - 448*384@30HZ | High - 672*576@30HZ |
|  | City | 720 kbps | 1000 kbps |
|  | Crew, Harbour, Soccer | 1050 kbps | 1470 kbps |
| Ratio Low/High 3/5 | bitrates | Low - 384*336@30HZ | High - 640*560@30HZ |
|  | City | 610 kbps | 980 kbps |
|  | Crew, Harbour, Soccer | 890 kbps | 1430 kbps |

  - Experimental results

The test results are summarized in the table 2~4 for each ratio. The results of Thomson are in black and cross-check in red.

Table 2 Results of ratio 3/2

| Ratio | Sequence | Config | Layer | Rate% | DataRate | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Thomson results |  |  |  |  |  |  |  |  |
| 3_2 | City | GENERIC | layer1 | 0.53% | 1720 | 36.12 | 45.81 | 47.43 |
| 3_2 | City | GENERIC | layer0 | 0.69% | 720 | 38.04 | 45.37 | 47.01 |
| 3_2 | City | UNIFIED | layer1 | 1.00% | 1720 | 36.06 | 45.80 | 47.43 |
| 3_2 | City | UNIFIED | layer0 | 0.69% | 720 | 38.04 | 45.37 | 47.01 |
| 3_2 | Crew | GENERIC | layer0 | 0.51% | 1050 | 37.02 | 41.83 | 41.16 |
| 3_2 | Crew | GENERIC | layer1 | 1.53% | 2520 | 37.20 | 43.09 | 43.03 |
| 3_2 | Crew | UNIFIED | layer0 | 0.51% | 1050 | 37.02 | 41.83 | 41.16 |
| 3_2 | Crew | UNIFIED | layer1 | 1.81% | 2520 | 37.22 | 43.13 | 43.08 |
| 3_2 | Harbour | GENERIC | layer0 | 0.57% | 1050 | 32.82 | 42.58 | 44.20 |
| 3_2 | Harbour | GENERIC | layer1 | 0.63% | 2520 | 33.58 | 43.79 | 45.51 |
| 3_2 | Harbour | UNIFIED | layer0 | 0.57% | 1050 | 32.82 | 42.58 | 44.20 |
| 3_2 | Harbour | UNIFIED | layer1 | 0.13% | 2520 | 33.53 | 43.79 | 45.52 |
| 3_2 | Soccer | GENERIC | layer0 | 1.27% | 1050 | 37.84 | 44.23 | 46.04 |
| 3_2 | Soccer | GENERIC | layer1 | 1.35% | 2520 | 37.46 | 44.17 | 46.10 |
| 3_2 | Soccer | UNIFIED | layer0 | 1.27% | 1050 | 37.84 | 44.23 | 46.04 |
| 3_2 | Soccer | UNIFIED | layer1 | 1.71% | 2520 | 37.43 | 44.17 | 46.10 |
| Cross-check results |  |  |  |  |  |  |  |  |
| 3_2 | City | GENERIC | layer1 | 0.53% | 1720 | 36.12 | 45.81 | 47.43 |
| 3_2 | City | GENERIC | layer0 | 0.69% | 720 | 38.04 | 45.37 | 47.01 |
| 3_2 | City | UNIFIED | layer1 | 1.00% | 1720 | 36.06 | 45.80 | 47.43 |
| 3_2 | City | UNIFIED | layer0 | 0.69% | 720 | 38.04 | 45.37 | 47.01 |
| 3_2 | Crew | GENERIC | layer0 | 0.51% | 1050 | 37.02 | 41.83 | 41.16 |
| 3_2 | Crew | GENERIC | layer1 | 1.53% | 2520 | 37.20 | 43.09 | 43.03 |
| 3_2 | Crew | UNIFIED | layer0 | 0.51% | 1050 | 37.02 | 41.83 | 41.16 |
| 3_2 | Crew | UNIFIED | layer1 | 1.81% | 2520 | 37.22 | 43.13 | 43.08 |
| 3_2 | Harbour | GENERIC | layer0 | 0.57% | 1050 | 32.82 | 42.58 | 44.20 |
| 3_2 | Harbour | GENERIC | layer1 | 0.63% | 2520 | 33.58 | 43.79 | 45.51 |
| 3_2 | Harbour | UNIFIED | layer0 | 0.57% | 1050 | 32.82 | 42.58 | 44.20 |
| 3_2 | Harbour | UNIFIED | layer1 | 0.13% | 2520 | 33.53 | 43.79 | 45.52 |
| 3_2 | Soccer | GENERIC | layer0 | 1.27% | 1050 | 37.84 | 44.23 | 46.04 |
| 3_2 | Soccer | GENERIC | layer1 | 1.35% | 2520 | 37.46 | 44.17 | 46.10 |
| Ratio | Sequence | Config | Layer | Rate% | DataRate | PSNRY | PSNRU | PSNRV |
| Cross-check results |  |  |  |  |  |  |  |  |
| 3_2 | Soccer | UNIFIED | layer0 | 1.27% | 1050 | 37.84 | 44.23 | 46.04 |
| 3_2 | Soccer | UNIFIED | layer1 | 1.71% | 2520 | 37.43 | 44.17 | 46.10 |

Table 3 Results of ratio 4/3

| Ratio | Sequence | Config | Layer | Rate% | DataRate | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Thomson results |  |  |  |  |  |  |  |  |
| 4_3 | City | GENERIC | layer1 | 0.90% | 1834 | 36.24 | 44.00 | 46.03 |
| 4_3 | City | GENERIC | layer0 | 3.45% | 810 | 38.39 | 44.86 | 46.59 |
| 4_3 | City | UNIFIED | layer1 | 0.98% | 1834 | 36.20 | 44.00 | 46.02 |
| 4_3 | City | UNIFIED | layer0 | 3.52% | 810 | 38.38 | 44.86 | 46.59 |
| 4_3 | Crew | GENERIC | layer0 | 0.81% | 1190 | 37.46 | 41.82 | 41.37 |
| 4_3 | Crew | GENERIC | layer1 | 0.46% | 2690 | 37.19 | 41.47 | 42.18 |
| 4_3 | Crew | UNIFIED | layer0 | 0.81% | 1190 | 37.46 | 41.82 | 41.37 |
| 4_3 | Crew | UNIFIED | layer1 | 0.45% | 2690 | 37.22 | 41.50 | 42.24 |
| 4_3 | Harbour | GENERIC | layer0 | 0.76% | 1190 | 33.05 | 42.20 | 43.89 |
| 4_3 | Harbour | GENERIC | layer1 | 1.49% | 2690 | 33.59 | 42.75 | 44.73 |
| 4_3 | Harbour | UNIFIED | layer0 | 0.77% | 1190 | 33.05 | 42.20 | 43.89 |
| 4_3 | Harbour | UNIFIED | layer1 | 1.56% | 2690 | 33.56 | 42.76 | 44.73 |
| 4_3 | Soccer | GENERIC | layer0 | 0.47% | 1190 | 38.03 | 44.44 | 46.24 |
| 4_3 | Soccer | GENERIC | layer1 | 0.11% | 2690 | 37.50 | 44.20 | 46.12 |
| 4_3 | Soccer | UNIFIED | layer0 | 0.49% | 1190 | 38.03 | 44.44 | 46.24 |
| 4_3 | Soccer | UNIFIED | layer1 | 0.40% | 2690 | 37.49 | 44.22 | 46.14 |
| Cross-check results |  |  |  |  |  |  |  |  |
| 4_3 | City | GENERIC | layer1 | 0.90% | 1834 | 36.24 | 44.00 | 46.03 |
| 4_3 | City | GENERIC | layer0 | 3.45% | 810 | 38.39 | 44.86 | 46.59 |
| 4_3 | City | UNIFIED | layer1 | 0.98% | 1834 | 36.20 | 44.00 | 46.02 |
| 4_3 | City | UNIFIED | layer0 | 3.52% | 810 | 38.38 | 44.86 | 46.59 |
| 4_3 | Crew | GENERIC | layer0 | 0.81% | 1190 | 37.46 | 41.82 | 41.37 |
| 4_3 | Crew | GENERIC | layer1 | 0.46% | 2690 | 37.19 | 41.47 | 42.18 |
| 4_3 | Crew | UNIFIED | layer0 | 0.81% | 1190 | 37.46 | 41.82 | 41.37 |
| 4_3 | Crew | UNIFIED | layer1 | 0.45% | 2690 | 37.22 | 41.50 | 42.24 |
| 4_3 | Harbour | GENERIC | layer0 | 0.76% | 1190 | 33.05 | 42.20 | 43.89 |
| 4_3 | Harbour | GENERIC | layer1 | 1.49% | 2690 | 33.59 | 42.75 | 44.73 |
| 4_3 | Harbour | UNIFIED | layer0 | 0.77% | 1190 | 33.05 | 42.20 | 43.89 |
| 4_3 | Harbour | UNIFIED | layer1 | 1.56% | 2690 | 33.56 | 42.76 | 44.73 |
| 4_3 | Soccer | GENERIC | layer0 | 0.47% | 1190 | 38.03 | 44.44 | 46.24 |
| 4_3 | Soccer | GENERIC | layer1 | 0.11% | 2690 | 37.50 | 44.20 | 46.12 |
| 4_3 | Soccer | UNIFIED | layer0 | 0.49% | 1190 | 38.03 | 44.44 | 46.24 |
| 4_3 | Soccer | UNIFIED | layer1 | 0.40% | 2690 | 37.49 | 44.22 | 46.14 |

Table 4 Results of ratio 5/3

| Ratio | Sequence | Config | Layer | Rate% | DataRate | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Thomson results |  |  |  |  |  |  |  |  |
| 5_3 | City | GENERIC | layer1 | 0.27% | 1590 | 36.13 | 43.97 | 46.06 |
| 5_3 | City | GENERIC | layer0 | 0.66% | 610 | 37.43 | 44.51 | 46.27 |
| 5_3 | City | UNIFIED | layer1 | 0.14% | 1590 | 36.08 | 43.98 | 46.06 |
| 5_3 | City | UNIFIED | layer0 | 0.66% | 610 | 37.43 | 44.51 | 46.27 |
| 5_3 | Crew | GENERIC | layer1 | 1.63% | 2320 | 36.91 | 41.18 | 41.85 |
| 5_3 | Crew | GENERIC | layer0 | 0.92% | 890 | 36.64 | 40.89 | 40.23 |
| 5_3 | Crew | UNIFIED | layer1 | 1.29% | 2320 | 36.93 | 41.20 | 41.89 |
| 5_3 | Crew | UNIFIED | layer0 | 0.87% | 890 | 36.64 | 40.90 | 40.23 |
| 5_3 | Harbour | GENERIC | layer1 | 1.89% | 2320 | 33.41 | 42.64 | 44.61 |
| 5_3 | Harbour | GENERIC | layer0 | 1.22% | 890 | 32.57 | 41.77 | 43.41 |
| 5_3 | Harbour | UNIFIED | layer1 | 1.67% | 2320 | 33.53 | 42.73 | 44.71 |
| 5_3 | Harbour | UNIFIED | layer0 | 1.23% | 890 | 32.57 | 41.77 | 43.41 |
| 5_3 | Soccer | GENERIC | layer1 | 1.50% | 2320 | 37.43 | 44.12 | 46.03 |
| 5_3 | Soccer | GENERIC | layer0 | 0.59% | 890 | 37.42 | 43.81 | 45.64 |
| 5_3 | Soccer | UNIFIED | layer1 | 1.46% | 2320 | 37.41 | 44.12 | 46.03 |
| 5_3 | Soccer | UNIFIED | layer0 | 0.62% | 890 | 37.42 | 43.81 | 45.64 |
| Cross-check results |  |  |  |  |  |  |  |  |
| 5_3 | City | GENERIC | layer1 | 0.27% | 1590 | 36.13 | 43.97 | 46.06 |
| 5_3 | City | GENERIC | layer0 | 0.66% | 610 | 37.43 | 44.51 | 46.27 |
| 5_3 | City | UNIFIED | layer1 | 0.14% | 1590 | 36.08 | 43.98 | 46.06 |
| 5_3 | City | UNIFIED | layer0 | 0.66% | 610 | 37.43 | 44.51 | 46.27 |
| 5_3 | Crew | GENERIC | layer1 | 1.63% | 2320 | 36.91 | 41.18 | 41.85 |
| 5_3 | Crew | GENERIC | layer0 | 0.92% | 890 | 36.64 | 40.89 | 40.23 |
| 5_3 | Crew | UNIFIED | layer1 | 1.29% | 2320 | 36.93 | 41.20 | 41.89 |
| 5_3 | Crew | UNIFIED | layer0 | 0.87% | 890 | 36.64 | 40.90 | 40.23 |
| 5_3 | Harbour | GENERIC | layer1 | 1.89% | 2320 | 33.41 | 42.64 | 44.61 |
| 5_3 | Harbour | GENERIC | layer0 | 1.22% | 890 | 32.57 | 41.77 | 43.41 |
| 5_3 | Harbour | UNIFIED | layer1 | 1.67% | 2320 | 33.53 | 42.73 | 44.71 |
| 5_3 | Harbour | UNIFIED | layer0 | 1.23% | 890 | 32.57 | 41.77 | 43.41 |
| 5_3 | Soccer | GENERIC | layer1 | 1.50% | 2320 | 37.43 | 44.12 | 46.03 |
| 5_3 | Soccer | GENERIC | layer0 | 0.59% | 890 | 37.42 | 43.81 | 45.64 |
| 5_3 | Soccer | UNIFIED | layer1 | 1.46% | 2320 | 37.41 | 44.12 | 46.03 |
| 5_3 | Soccer | UNIFIED | layer0 | 0.62% | 890 | 37.42 | 43.81 | 45.64 |

# Conclusion

From the verification work, we conclude that the provided bitstreams, coder and decoder for CE 10 - proposal 2 are all correct. And the result given by Thomson is successfully cross-checked and it is identical to the result that has been tested by us.

# References

- Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG, “Core Experiments on Extended Spatial Scalability (CE-10)”, JVT-O310_2.