| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznań, PL, 24-29 July, 2005 | Document: JVT-P100 Filename: input.doc |
| --- | --- |

| Title: | High-Level Syntax for SVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peter Amon, Andreas Hutter Siemens AG, Corporate Technology CT IC 2, 81730 Munich, Germany Thomas Rathgen Ilmenau Technical University Faculty for Electrical Engineering P.O. Box 100565 98684 Ilmenau, Germany | Tel: Email: Tel: Email: | +49 (89) 636 54642 p.amon@siemens.com +49 (3677) 691108 thomas.ratghen@tu-ilmenau.de |
| Source: | Siemens AG |  |  |

_____________________________

# Abstract

This document proposes three different tools for the high-level syntax of SVC. The first topic addresses the scalability description in the NAL unit header. First, the current status of the discussion is briefly reviewed, which is a variable length design. Then two alternate solutions are presented, which basically introduces more flexibility in the usage of the NAL unit header. The second tool describes the concept of Super-NAL units. These container elements include all NAL units of one scalability level (the same spatial, temporal, and SNR ID). This design facilitates scalability operations and supports transport. The third tool deals with the design of FGS NAL units to allow medium and fine grain SNR scalability with the high-level syntax.

# NAL Unit Header

## Current status

The approach for the design of the scalability information consists of a variable length solution using a mandatory first byte and an optional second byte.

The semantics of the fields are as follows:

priority_id (6 bits): one-dimensional index to signal the priority of the NAL unit

DS (1 bit): indicates whether the NAL unit contains data of a dead sub-stream

S (1 bit): switch indicating whether the optional second byte is used or not

dependency_id (3 bits): ID for spatial and CGS layers

temporal_level (3 bits): ID for the temporal resolution

quality_level (2 bits): ID for SNR quality refinements (full FGS layers)

In case the optional second byte is used (e.g. S=0), then structure is unchanged

In case the optional second byte is not used (e.g. S=0), then the following structure for the scalability information in the NAL unit header remains.

If the information in the second byte (dependency ID, temporal ID, quality ID) is needed for decoding then it has to be stored at a different position in the bit-stream (e.g. in the slice header). A second disadvantage is the inflexible definition of the priority ID.

## Proposal 1 with fixed length NAL unit header

The following syntax does not have the shortcomings of the current approach. The differences to the current design are not severe. The main difference is the flexible usage of the first field (formerly priority ID). The meaning of this field is toggled by the switch ‘S’. The length the scalability information in the NAL unit header is fixed (2 bytes).

For S=0 the first field is used to indicate the quality fragment level.

For S=1 the first field is used as a layer ID. The layer ID is a one-dimensional index to signal a fixed path through the three scalability axes (dependency ID, temporal level and quality level). Each combination of these axes is assigned a layer ID. In a higher layer at least one of the three scalability axes also has to increase and none of the scalability axes must decrease).

The layer ID can be overwritten in order to allow new layered mappings or to transform the bit-stream from the fully scalable to the layered representation, e.g during playout or by an adaptation engine.

### Syntax

In Table 1 the syntax of a complete NAL unit for this proposal is given.

Table 1    Syntax of NAL unit

| nal_unit( TotalBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_idc (NRI) | u(2) |
| nal_unit_type | u(5) |
| if (nal_unit_type == 20 ¦¦ nal_unit_type == 21) { |  |
| layer_id_used | b(1) |
| if (!layer_id_used) |  |
| quality_fragment_level | b(6) |
| else |  |
| layer_id | b(6) |
| is_dead_sub_stream | b(1) |
| dependency_id | b(3) |
| temporal_level | b(3) |
| quality_level | b(2) |
| } |  |
| NumBytesInRBSP = 0 |  |
| for( i = 1; i < NumBytesInNALunit; i++ ) { |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| } |  |

### Semantics

TotalBytesInNALunit specifies the size of the NAL unit in bytes. TotalBytesInNALunit is provided by external framing.

forbidden_zero_bit shall be equal to 0.

nal_ref_idc specifies the picture priority if used in AVC NAL unit headers, if used in SVC NAL unit headers or used in scalability extension NAL units of type SCAL_EX its value should correspond to the value of nal_ref_idc of corresponding AVC NAL unit of the same temporal scalability level.

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit.

layer_id specifies the layer ID for use in layered scalability mode, if layer_id_used is set.

is_dead_sub_stream specifies that this NAL unit is marked as dead sub-stream. This value could be ignored, if layer_id_used is set.

layer_id_used specifies whether a layer ID is used. If the layered scalability mode is used, then it shall be set. If the fully scalable mode is used it shall not be set.

quality_fragment_level specifies the fragmentation level of a fragmented progressive refinement NAL unit. If fragmentation of progressive refinement NAL unit is not used, then its value shall be set to zero.

temporal_level specifies the temporal scalability level for use in full scalable mode.

dependency_id specifies the spatial scalability level for use in full scalable mode. If coarse grain scalability is used, one ‘spatial’ level could differ from the next ‘spatial’ level only by different quantization settings.

quality_level specifies the SNR scalability level (full FGS levels)

## Proposal 2 with variable length NAL unit header

If the current variable length design is not seen as a problem then also the following structure of the scalability information in the NAL unit header is a good option:

In case the two byte version (e.g. S=0) is used the

For S=1 the second byte is cut:

Syntax and semantics are similar as in proposal 1.

# Super NAL Units (Scalable Extension NAL Units)

To enable efficient access to scalability levels with a large number of NAL units all SVC NAL units belonging to one particular scalability level could be optionally stored in a virtual aggregation NAL unit (scalable extension NAL units of type SCAL_EX).

The structure of a sample with two AVC base layer NAL units and two scalability levels is depicted in Fig. 1. To signal a NAL unit of type SCAL_EX we propose the usage of NAL unit type 30. A scalable extension NAL units of type SCAL_EX is handled as other SVC NAL unit by transmission engines and e.g. by an SVC file format.

Fig. 1    Access unit containing AVC NAL units and aggregated SVC NAL units

## Syntax

Table 2    Syntax of scalable extension NAL unit

| extension_nal_unit( TotalBytesInExtensionNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_idc (NRI) | u(2) |
| reserved_nal_unit_type_scalable_extension /* equal to 30 */ | u(5) |
| layer_id_used | b(1) |
| if (!layer_id_used) |  |
| quality_fragment_level | b(6) |
| else |  |
| layer_id | b(6) |
| is_dead_sub_stream | b(1) |
| temporal_level | b(3) |
| dependency_id | b(3) |
| quality_id | b(2) |
| NumBytesInExtensionNALunit = 0 |  |
| do { |  |
| NumTotalBytesInNALunit | u(v) |
| NumBytesInNALunit = 0 |  |
| for( i = 0; i < NumTotalBytesInNALunit; i++ ) { |  |
| SVC_NALunit[ NumBytesInNALunit++ ] NumBytesInExtensionNALunit++ | b(8) |
| } |  |
| } while (NumBytesInExtensionNALunit < TotalBytesInExtensionNALunit) |  |
| } |  |

## Semantics

TotalBytesInExtensionNALunit specifies the size of the virtual aggregation NAL unit of Type SCAL_EX in bytes. TotalBytesInExtensionNALunit is provided by external framing.

forbidden_zero_bit shall be equal to 0.

nal_ref_idc specifies the picture priority if used in AVC NAL unit headers, if used in SVC NAL unit headers or used in scalability extension NAL units of type SCAL_EX its value should correspond to the value of nal_ref_idc of corresponding AVC NAL unit of the same temporal scalability level.

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit. For scalability extension NAL units of type SCAL_EX its value is constantly set to 30.

layer_id specifies the layer ID for use in layered scalability mode.

is_dead_sub_stream specifies that the scalability level of this NAL unit is marked as dead sub-stream. This value could be ignored, if layer_id_used is set.

layer_id_used specifies whether a layer ID is used. If the layered scalability mode is used, then it shall be set. If the fully scalable mode is used it shall not be set.

quality_fragment_id specifies the fragmentation ID of a fragmented progressive refinement NAL unit. If fragmentation of progressive refinement NAL unit is not used, then its value shall be set to zero.

temporal_level specifies the temporal scalability level for use in full scalable mode.

dependency_id specifies the spatial scalability level for use in full scalable mode. If coarse grain scalability is used, one ‘spatial’ level could differ from the next ‘spatial’ level only by different quantization settings.

quality_level specifies the SNR scalability level (full FGS levels)

NumTotalBytesInNALunit specifies the length of the NAL unit following. The size of NumTotalBytesInNALunit is specified externally (e.g. by a scalability information SEI message).

SVC_NALunit contains a single NAL unit. SVC_NALunit includes the NAL unit header and the encapsulated payload.

# Fragmentation of Progressive Refinement NAL Units

SVC can support one ore more progressive refinement levels (FGS refinements). To enable simple adaptation operations a progressive refinement NAL unit can be split at given rate points. These points could be provided by the encoder or by a bit stream extraction tool with special rate control features. An adaptation operation can be performed by exploiting quality_level and the quality_fragement_level. The fragments of such split progressive refinement NAL unit are stored beginning with the first three bytes of the NAL unit header with ascending quality_fragement_level count.

Fragmentation of progressive refinement NAL units is possible only if a fully scalable bit stream representation is used. If a layered bit stream representation is used fragmented progressive refinement NAL unit could be used by encapsulating the fragments in scalable extension NAL units (SuperNAL units). If the representation of a bit stream is changed by performing a mapping operation then the fragments are reassembled before performing the mapping operation.

Fig. 2    Fragmentation of a progressive refinement NAL unit

For fragmentation the following definitions apply:

- The first part of a fragmented progressive refinement NAL unit (including the NAL unit header) is stored as is.

- Following fragments of that progressive refinement NAL unit are stored by duplicating the first three bytes of the header of the first fragment with ascending count of quality_level.

- A fragmented progressive refinement NAL units needs to be reassembled before it can be decoded.

# Conclusions

We propose to include the three specifications scalability information in the NAL unit header, Super-NAL units and fragmentation of FGS layers in the next version of the Joint Scalable Video Model and later on in the Working Draft.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Siemens AG |  |  |
| Mailing address | CT IC 2 Otto-Hahn-Ring 6 81730 Munich, Germany |  |  |
| Country | Germany |  |  |
| Contact person | Peter Amon |  |  |
| Telephone | +49 (89) 636 54642 |  |  |
| Fax | +49 (89) 636 51115 |  |  |
| Email | p.amon@siemens.com |  |  |
| Place and date of submission | Poznan, Poland, July 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | ITU T Rec. H.264 ¦ ISO/IEC 14496 10 |  |
| Title |  | High-Level Syntax for SVC |  |
| Contribution number |  | JVT-P100 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: