| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznań, PL, 24-29 July, 2005 | Document: JVT-P104 Filename: input.doc |
| --- | --- |

| Title: | CABAC Context Re-modeling in JSVM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ji-Ho Park, Doe Hyun Yoon, Seung-Wook Park, Byeong-Moon Jeon, DM research lab., LG Electronics Inc., 16 Woomyeon-Dong, Seocho-Gu, Seoul 137-724, Korea | Tel: Email: | + 82 - 2 – 526 - 4369 scottie@lge.com dhyoon@lge.com beatitude@lge.com jeonbm@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

In the JSVM 2.0 [1], CABAC [2] is the one of the most efficient tool for improving the coding efficiency. CABAC has been adopted in ISO/IEC 14496-10 [3] and its scheme can be divided into three modules such as, binarizer, context modeler and binary arithmetic coder. In the current JSVM 2.0, one of the most important modules, context modeler uses the same syntax elements of neighboring maroblocks to calculate context index increment. For the single layer predictive coding scheme, using the same syntax of the neighboring macroblocks is reasonable. However, regarding the multi-layer scheme, syntax elements of upper layer can obtain the information of the lower layer. If the macroblock of spatial enhancement layer has the base macroblock, the two macroblocks exists in the same temporal location so their relations are closer than the relation of neighboring macroblocks.

In this contribution, we first give our observation about the current context model for base_mode_flag based on Palma conditions by showing the actual percentages. And we propose newly expanded context models for base_mode_flag including the effect of the macroblock modes of base layer.

# Observations

In the JSVM 2.0, one of the newly added syntax elements at macroblock layer in scalable extension syntax, base_mode_flag uses one of the 3 predefined contexts from 460 to 462. These predefined contexts are decided by the same syntax element value of the above and left macroblocks. The derivation process of the context index increment of base_mode_flag is using following equation.

ctxIdxInc = condTermA + condTermB

By the above equation, we derive the context index increment which is used for the CABAC initialization.

Table.1 Context index table of base_mode_flag

| context index | base_mode_flag of above macroblock | base_mode_flag of left macroblock | MB type of base macroblock |
| --- | --- | --- | --- |
| 460 | off | off | Don’t care |
| 461 | off | on | Don’t care |
|  | on | off |  |
| 462 | on | on | Don’t care |

To investigate the behavior of the current context model for base_mode_flag, we have experimented to check whether the current context model is well-designed or not. Palma configuration is employed for our investigation. (See Fig.1~4)

Fig.1 Probability distribution at each context

Fig.2 Probability distribution when ctxIdxInc = 0

Fig.3 Probability distribution when ctxIdxInc = 1

Fig.4 Probability distribution when ctxIdxInc = 2

Fig.1 gives the whole distribution of three contexts. In Fig.2~4, the upper region of each bar is the percentage of symbol ‘0’ and the lower region of each bar is the percentage of symbol ‘1’. From Fig.2 which means that none of two neighboring macroblocks sets base_mode_flag, we can expect that the value of MPS is “0” and LPS probability is approximately 20~25%. Accordingly from Fig.3 which means that one of two neighboring macroblocks sets the base_mode_flag and Fig.4 which means that both of two neighboring macroblocks set the base_mode_flag, we can expect the value of MPS are “1” and LPS probabilities are 45~55% and 10%~15% respectively.

During our tests, we found out that macroblock in the intra slice sets the base_mode_flag more often than inter slice. It gives us a hint that the macroblock mode of the base layer affects when deciding the base_mode_flag of the current layer. If the base macroblock is coded as an intra mode, the probability of the base_mode_flag to be set increases up to 90% (refer to Fig.6) and in the opposite case that the base macroblock is coded as inter mode, the probability of the base_mode_flag to be set is about 55% (refer to Fig.7). The following figures show that the probability of base_mode_flag depends on MB type of base macroblock.

Fig.5 Probability distribution according to MB type of base macroblock

Fig.6 Probability distribution when MB type of base macroblock is intra

Fig.7 Probability distribution when MB type of base macroblock is inter

To clarify the effect of MB type of base macroblock on the base_mode_flag, we have analyzed the probability distribution depending on MB type of base macroblock and the current context model using Palma test condition.

Fig.8 Probability distribution according to MB type of base macroblock and context model

Fig.9 Probability distribution when base macroblock is intra and ctxIdxInc = 0

Fig.10 Probability distribution when base macroblock is inter and ctxIdxInc = 0

Fig.11 Probability distribution when base macroblock is intra and ctxIdxInc = 1

Fig.12 Probability distribution when base macroblock is inter and ctxIdxInc = 1

Fig.13 Probability distribution when base macroblock is intra and ctxIdxInc = 2

Fig.14 Probability distribution when base macroblock is inter and ctxIdxInc = 2

As shown figures, if MB type of base macroblock is inter, the probability of base_mode_flag is very similar to that of JSVM 2.0. However, even though none of two neighboring marcoblocks sets the base_mode _flag and the MPS value based on the current JSVM 2.0 is ‘0’, if base macroblock is intra, the macroblock has a higher probability to be coded as Intra_Base so that base_mode_flag is set to 1 and thus the MPS value will be ‘1’ (refer to Fig.9). On the other hand, if MB type of base macroblock is inter and none of two neighboring marcoblocks sets the base_mode_flag, the macroblock has a lower probability to set base_mode_flag so that base_mode_flag is 0 and thus the MPS value will be ‘0’ (refer to Fig.10). Therefore, from our results we need to add new context model considering MPS value is ‘1’ when ctxIdxInc is equal to 0 (i.e., none of two neighboring marcoblocks sets the base_mode _flag), in which the current context model provides the MPS value to be ‘0’.

For the case that ctxIdxInc is equal to 1 or 2 (refer to Fig.11 ~ 14), the MPS value is identical to the value expected from the current context model regardless of MB type of base macroblock. However the probability of the MPS value is quite different so that we need to add new context models.

# Technical descriptions

## Proposed context model

From our observation, we propose to expand new context models for base_mode_flag including MB type of base macroblock. The derivation equation of context model increment is like below

CtxIdxInc = condTermA + condTermB + 3 * intra_base_mb( CurrMbAddr )

1, if the corresponding base layer is coded with Intra mode

where intra_base_mb( CurrMbAddr )

0, otherwise

Table.2 Proposed context index table of base_mode_flag

| context index | base_mode_flag of above macroblock | base_mode_flag ofleft macroblock | MB type ofbase macroblock |
| --- | --- | --- | --- |
| 460 | off | off | Inter |
| 461 | off | on | inter |
|  | on | off |  |
| 462 | on | on | inter |
| 463 | off | off | intra |
| 464 | off | on | intra |
|  | on | off |  |
| 465 | on | on | intra |

# Experimental results

This contribution was integrated into JSVM 2.0 software [1] and the modified Munich configuration was used to compare the bitrate of our method with that of the JSVM 2.0. To show the effect of the new context model, we switched “Inter-layer Prediction” of Munich configuration from “always” to “adaptive”. Also, we used the same .mot file to compare the bitrates under the same PSNR.

Table.3 Bitrate comparison

(Unit = Kbps)

| Sequence | Layer | JSVM 2.0 | Proposed | Difference |
| --- | --- | --- | --- | --- |
| Bus | Layer4 Layer3 Layer2 Layer1 Layer0 | 516.008 386.7392 193.728 95.2832 63.3963 | 515.912 386.648 193.7008 95.2848 63.3963 | 0.096 0.0912 0.0272 -0.0016 0 |
| Football | Layer4 Layer3 Layer2 Layer1 Layer0 | 1026.9351 508.968 384.5723 193.8314 128.8634 | 1026.9 508.896 384.4422 193.8978 128.8634 | 0.0351 0.072 0.1301 -0.0664 0 |
| Foreman | Layer4 Layer3 Layer2 Layer1 Layer0 | 258.0304 192.7384 96.096 47.4512 31.7864 | 257.9912 192.7016 96.1008 47.4616 31.7864 | 0.0392 0.0368 -0.0048 -0.0104 0 |
| Moblie | Layer4 Layer3 Layer2 Layer1 Layer0 | 385.8744 256.3696 128.884 64.149 48.024 | 385.76 256.308 128.8704 64.1488 48.024 | 0.1144 0.0616 0.0136 0.0008 0 |
| City | Layer5 Layer4 Layer3 Layer2 Layer1 Layer0 | 2028.4912 1018.0896 515.2776 257.2168 126.7984 64.3912 | 2027.9816 1017.8944 515.2096 257.1928 126.7944 64.3912 | 0.5096 0.1952 0.068 0.024 0.004 0 |
| Crew | Layer5 Layer4 Layer3 Layer2 Layer1 Layer0 | 3057.4136 1542.1392 764.3728 385.0120 190.1136 96.7912 | 3056.6568 1541.5992 764.2264 384.9536 190.1208 96.7912 | 0.7568 0.54 0.1464 0.0584 -0.0072 0 |
| Harbour | Layer5 Layer4 Layer3 Layer2 Layer1 Layer0 | 3067.9944 1544.4704 765.772 385.6752 192.3368 96.796 | 3067.3832 1544.1680 765.6472 385.6464 192.3312 96.796 | 0.6112 0.3024 0.1248 0.0288 0.0056 0 |
| Soccer | Layer5 Layer4 Layer3 Layer2 Layer1 Layer0 | 3088.036 1527.7784 766.3504 380.7304 190.644 95.0616 | 3087.5256 1527.5296 766.2696 380.6944 190.6392 95.0616 | 0.510 0.249 0.081 0.036 0.005 0 |

# Conclusion

In this contribution, we have proposed CABAC context re-modeling in JSVM. Especially we have experimented about base_mode_flag in macroblock layer. Our proposal uses the relation between base_moe_flag of the enhacement layer and MB type of base macroblock. From the experimental results, the maximum gain is 0.13Kbps for Football CIF sequence and 0.7568Kpbs for Crew 4CIF sequence.

Our proposed method is only applied to base_mode_flag, we suggest that further investigation is needed for the CABAC context model for other new add syntax elements.

# References

- JVT-O202, “Joint Scalable Video Model JSVM-2,” Busan, April, 2005

- D. Marpe, H. Schwarz, T. Wiegand, “Context-Based Adaptive Binary Arithmetic Coding in the H.264/AVC Video Compression Standard,” IEEE Trans. Circuits Syst. for Video Tech., vol. 13, No. 7, pp. 620-636, July 2003.

- ISO/IEC JTC1/SC29/WG11, “Text of ISO/IEC 14496-10 FDIS Advanced Video Coding, ” MPEG Document N5555, Pattaya, March 2003

# Appendix: the proposed modification to JSVM annex.S

## Syntax modifications

No syntax modification is required for our contribution

## Semantics modifications

No semantics modification is required for our contribution

## Decoding process

S.9.3.1.1 Initialisation process for context variables

Replace table S-9-1 with the following

S-9-1 Association of ctxIdx and syntax elements for each slice type in the initialization process

|  | Syntax element | Table | Slice type |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PR | EI | EP | EB |
| macroblock_layer_in_scalable_extension( ) | base_mode_flag | Table S-9-2 |  | 460-462 460-465 | 460-462 460-465 | 460-462 460-465 |
|  | base_mode_refinement_flag | Table S-9-2 |  |  | 463 466 | 463 466 |
|  | intra_base_flag | Table S-9-2 |  | 464 467 | 464 467 | 464 467 |
| mb_pred_in_scalable_extension( ) and sub_mb_pred_in_scalable_extension( ) | motion_prediction_flag_l0 | Table S-9-2 |  |  | 465 468 | 465 468 |
|  | motion_prediction_flag_l1 | Table S-9-2 |  |  | 465 468 | 465 468 |
|  | mvd_ref_l0 | Table S-9-2 |  |  | 466 469 | 466 469 |
|  | mvd_ref_l1 | Table S-9-2 |  |  | 466 469 | 466 469 |
| residual_in_scalable_extension( ) | residual_prediction_flag | Table S-9-2 |  |  | 467 470 | 467 470 |
|  | transform_size_8x8_flag | Table 9-16 | 399-401 | 399-401 | 399-401 | 399-401 |
|  | coded_block_flag_luma[] | Table 9-18 | 85-104 | 85-104 | 85-104 | 85-104 |
|  | coded_block_flag_chromaDC | Table 9-18 | 85-104 | 85-104 | 85-104 | 85-104 |
|  | coded_block_flag_chromaAC | Table 9-18 | 85-104 | 85-104 | 85-104 | 85-104 |
| luma_coefficient() and chromaDC_coefficient() and chromaAC_coefficient() | coded_block_pattern_bit[] | Table 9-18 | 73-76 | 73-76 | 73-76 | 73-76 |
|  | mb_qp_delta | Table 9-17 | 60-63 | 60-63 | 60-63 | 60-63 |
|  | last_significant_coeff_flag | Table 9-19 Table 9-22 Table 9-24 Table 9-24 | 166-226 338-398 417-425 451-459 | 166-226 338-398 417-425 451-459 | 166-226 338-398 417-425 451-459 | 166-226 338-398 417-425 451-459 |
|  | significant_coeff_flag | Table 9-20 Table 9-23 Table 9-24 Table 9-24 Table S-9-3 | 105-165 277-337 402-416 436-450 470-481 473-484 | 105-165 277-337 402-416 436-450 | 105-165 277-337 402-416 436-450 | 105-165 277-337 402-416 436-450 |
|  | coeff_abs_level_minus1 | Table 9-21 Table 9-24 | 227-275 426-435 | 227-275 426-435 | 227-275 426-435 | 227-275 426-435 |
| luma_coefficient_ref ( ) and chromaDC_coefficient_ref () and chromaAC_coefficient_ref () | coeff_refinement_flag | Table S-9-2 | 468 471 |  |  |  |
|  | coeff_refinement_direction_flag | Table S-9-2 | 469 472 |  |  |  |

S.9.3.1.1.1.11 Initialisation process for context variables

Output of this process is ctxIdxInc.

When MbaffFrameFlag is equal to 1 and mb_field_decoding_flag has not been decoded (yet) for the current macroblock pair with top macroblock address 2 * ( CurrMbAddr / 2 ), the inference rule for the syntax element mb_field_decoding_flag as specified in subclause S.7.4.4 shall be applied.

The derivation process for neighbouring macroblocks specified in subclause S.6.4.8.1 is invoked and the output is assigned to mbAddrA and mbAddrB.

Let the variable condTermFlagN (with N being either A or B) be derived as follows.

If mbAddrN is not available or base_mode_flag for the macroblock mbAddrN is equal to 1, condTermFlagN is set equal to 0.

Otherwise (mbAddrN is available and base_mode_flag for the macroblock mbAddrN is equal to 0), condTermFlagN is set equal to 1.

The variable ctxIdxInc is derived by

ctxIdxInc = condTermFlagA + condTermFlagB

ctxIdxInc = condTermFlagA + condTermFlagB + intra_base_mb( CurrMbAddr ) * 3

- Note – ctxIdx from 463 to 481 will be shifted by 3.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: