| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 17th Meeting: Nice, FR, 14-21 October, 2005 | Document: JVT-Q079 Filename: input.doc |
| --- | --- |

| Title: | Fast Subpixel Motion Estimation Support for the Enhanced Predictive Zonal Search Scheme |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal, Information |  |  |
| Author(s) or Contact(s): | Alexis Tourapis 7150 Riverwood Dr. Columbia, MD 1046 Pankaj Topiwala 7150 Riverwood Dr. Columbia, MD 1046 | Tel: Email: Tel: Email: | 408-228-7983 alexis@fastvdo.com 410-309-6066 pankaj@fastvdo.com |
| Source: | FastVDO |  |  |

_____________________________

Abstract

Motion Estimation is in general one of the most computationally complex processes within a video encoding system. This is especially true within the H.264 reference software considering that motion estimation may need to be performed using multiple references or block sizes. With the extension and usage however of the H.264 standard into higher resolutions and formats, encoding speed has become even more vital. In [1] and [2] we presented an extension of our predictive based fast motion estimation algorithms named as the Enhanced Predictive Zonal Search schemes, which were later adopted and implemented into the reference software. Although the EPZS scheme can easily be extended to simultaneously consider fractional samples, our original implementation only considered integer samples. Nevertheless, considering that subpixel motion estimation becomes the biggest bottleneck after the introduction of this scheme, but also since the two other fast motion estimation schemes available in the reference software [3][4][5] already consider fractional fast motion estimation, it was requested that such is also provided. Therefore, in this document we also provide such an algorithm which we hope would assist further in promoting the H.264 standard and improve the usage of the JM reference software. Note, that we may consider at a later stage, and depending on the author’s schedule, whether a full subpixel implementation of EPZS may be worthwhile it to be provided.

- The proposed scheme

We have designed and implemented a rather simple scheme for supporting fractional fast motion estimation in the JM reference software. This scheme is somewhat reminiscent of the schemes also proposed in [3][4][5] and (mostly) [6]. In fact, one may claim that all these schemes are rather similar in general and have relatively minor differences.

More specifically, our fast fractional motion estimation scheme, similar to the schemes presented in [6], assumes that fractional positions aligned on the vertical or horizontal axis (i.e. diamond positions 2 in Figure 1) have higher probability to be the minimum locations. Decisions are also made based on the relationship of the best and second best location but also the minimum distortion as computed at a given point. Our algorithm is in fact as follows:

Assume that current best MV is equal to MV = {mvx ,mvy} and predictor mv is PMV = {pmvx ,pmvy}

Step 1: 	Set diamond patterns DPhalf = {(0,0) (-2,0), (0,2), (2,0), (0,-2)} and 
	DPqpel = {(0,0) (-1,0), (0,1), (1,0), (0,-1)}. Set DP = DPhalf. Set resolution to ½.

Step 2: 	Examine all positions DPi (i=0..4) around the current best location. 
	Store both current best and second best locations (best_pos and second_pos respectively) 
	and their respective distortion values (min_sad and second_sad).

Step 3: 	If (best_pos == 0), (MV == PMV), and (min_sad < T), where T is a threshold, terminate search.

Step 4: 	If (MV == PMV), goto step 6.

Step 5: 	Depending on the location of best_pos and second_pos determine which additional points to examine at current resolution as follows:

- If (best_pos == 0) || (second_pos == 0), examine the two additional points of the same resolution adjacent to the non zero location (Figure 1b). Update appropriately best_pos.

- Otherwise, if (best_pos ^ second_pos), i.e. positions are neighboring, further examine the additional remaining position between the two samples ((Figure 1a). Update appropriately best_pos.

- Otherwise, if best_pos and second are on opposite directions, examine all remaining points in current resolution. Update appropriately best_pos.

Step 6:  Update best MV as MV + DPbest_pos

Step 7: If (resolution == ¼) terminate. Otherwise set DP = DPqpel. Set resolution to ¼.  Goto Step 1.

| (a) | (b) | (c) |
| --- | --- | --- |

Figure 1: Examples of subpixel refinement using the proposed scheme. In case (a) best and second best are adjacent non zero positions, and therefore a single position is additionally tested for refinement. In case (b), the best or second best is the center location, and in this case 2 additional points may be tested. Finally in case (c) best and second best are on opposite locations which suggests that information is not sufficient and all remaining subpixel positions for the current fractional refinement should be tested.

- Simulation Results

To evaluate the proposed scheme we have performed simulations on several CIF and QCIF resolution sequences. We have also examined various variants of the EPZS scheme, i.e. with only one refinement, simpler refinement patters, reduced predictors, and the proposed subpel ME enabled or disabled. Our full simulations can be seen in the associated excel spreadsheet while such were compared to the Simplified Hexagonal search available also in the JM.

We immediately observe from these simulation results, that obviously the simplified Hexagonal search, considering that it also already provides for a fractional scheme, is somewhat faster than the “most” complex mode of EPZS as such is implemented in the software (note that one can provide other variants of EPZS by just defining additional patterns or different conditions). However, the difference is speed is much less noticeable when we compare the overall improvement in speed. Furthermore this improvement also comes at a certain loss in performance which can be up to 2% in terms of bitrate increase. Note also that it is the authors opinion that at this point, the use of Fast Mode Decision is actually more critical in achieving further reduced complexity than Motion Estimation is. Note also that the JM version provided has been further optimized by reducing certain overheads associated with the software.

Performance of the EPZS scheme seems to improve in terms of speed when the fractional fast scheme is introduced. Based on our results, we seem to observe an improvement of around 15% in terms of Motion Estimation speed up. However the impact in terms of quality seems to be still negligible.

Note that due to lack of time, the author has implemented the “position” adaptation of the subpixel ME using conditionals. However it would be better if a table lookup approach was used instead, and the author may try to provide such an implementation depending his time. Furthermore, the same scheme can also be used for biprediction, although no such test was used in this experiment, and code may also be provided.

As we have suggested, EPZS is highly configurable. We have also examined the behavior of the scheme when we adjust some of the EPZS parameters such as the number of refinements, and the predictors used. We basically observe from the simulation results in the associated excel sheet that performance in that both speed and performance become very similar between the different schemes integrated in the software.

Based on our results, we suggest retaining all three Fast Motion Estimation schemes in the software but also adopting the above simple scheme in the JM reference software. We believe that such non-normative concepts are really valuable to the research community since they could assist in the understanding of the JM reference software but also of video compression in general. Hopefully such would also assist in the further deployment and adoption of the H.264 in all applications related to video/image compression.

- References

- H.Y. Cheong, A.M. Tourapis, and P. Topiwala, “Fast Motion Estimation within the JVT codec,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, document JVT-E023, Oct'02.

- A.M. Tourapis, H.Y. Cheong, and P. Topiwala, “Fast ME in the JM reference software,” ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, document JVT-P026, Jul'05.

- Text Description of Joint Model Reference Encoding Methods and Decoding Concealment Methods ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, document JVT-O079, Apr'05.

- Z. Chen, P. Zhou, and Y. He, “Fast Integer and Fractional Pel Motion Estimation for JVT,” JVT-F017r.doc, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG. 6th Meeting, Awaji, Island, Japan, Dec. 5-13, 2002.

- X. Yi, J. Zhang, N. Ling, and W. Shang, “Improved and simplified fast motion estimation for JM”, ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, document JVT-P021, Jul'05.

- P.Yin, H.Y Cheong, A.M. Tourapis, and J. Boyce, “Fast mode decision and motion estimation for JVT/H.264”, in Proceedings of the 2003 IEEE International Conference on Image Processing (ICIP 2003), Volume 3,  pp853-6, Sep’03

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | FastVDO LLC |  |  |
| Mailing address | 7150 Riverwood Dr. Columbia, MD 1046 |  |  |
| Country | USA |  |  |
| Contact person | Alexis Michael Tourapis |  |  |
| Telephone | 408-228-7983 |  |  |
| Fax |  |  |  |
| Email | alexis@fastvdo.com |  |  |
| Place and date of submission | Nice, FR, 14-21 October, 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Fast Subpixel Motion Estimation Support for the Enhanced Predictive Zonal Search Scheme |  |
| Contribution number |  | Q079 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: