| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 17th Meeting: Nice, FR, 14-21 October, 2005 | Document: JVT-Q087 Filename: input.doc |
| --- | --- |

| Title: | Conditions for SVC bit error resilience testing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Florelle Pauchet, Christine Guillemot IRISA-INRIA Campus de Beaulieu 35042 Rennes cedex, France Michel Kerdranvat Thomson R&D 1, avenue de Belle Fontaine CS 17616 35576 Cesson-Sévigné Cedex France Stéphane Pateux, Pierre Siohan France Télécom 4 Rue du Clos Courtel 35510 Cesson-Sévigné, France | Tel: Email: | +33 (0) 2 99 84 71 71 fpauchet@irisa.fr +33 (0)2 99 27 33 95 Michel.Kerdranvat@thomson.net +33 (0) 2 99 12 44 77 stephane.pateux@francetelecom.com |
| Source: | IRISA-INRIA |  |  |

_____________________________

### Abstract

This document proposes common conditions for SVC error resilience testing taking into account bit errors.

# Introduction

So far error resilience has been designed mainly for packet-loss environments. However, emergence of wireless error-prone links network requires new modes of resilience to bit errors. New standards include the possibility to pass erroneous SDUs to the upper layers. The motivation is to save bandwidth by avoiding discarding entire packets if they are only damaged by some bit errors. This implies that application layers should handle corrupted data. In that context we propose error resilience tests taking into account bit errors.

# Need for bit error resilience

Variable Length Codes achieve good compression efficiency but suffer from a high sensitivity to noise: a single error in the bitstream can lead to a complete desynchronisation of the decoder. As a result, all the bitstream from the part where the error occurs until the next position where the decoder is able to resynchronize is lost.

Compression systems have been designed with the assumption that the quality of service was guaranteed by the lower layers of the communication systems or storage media. For instance, for transmission applications, lower layers of the OSI model were supposed to have error detection mechanisms. Those error detection mechanisms lead to discard each corrupted packets. So, a few hundred bytes will be discarded even if there is a single bit error in the whole payload. Packets drop impacts on the bandwidth, which is critical for wireless and mobile applications.

These observations have led some normalization groups to work on new modes that will not discard systematically damaged packets but will forward them to the upper layers (so, to the decoder). This relies obviously on the assumption that the decoder is able to cope with errors.

In particular, 3GPP [5] and Bluetooth norms include transparent mode for the Medium Access Control (MAC) layer that allows to pass erroneous SDUs to the application layer. This evolution of the lower layer specifications of communication systems can be found also for the transport layer. The Internet Engineering Task Force (IETF) has elevated UDP lite (lightweight User Datagram Protocol) [2] to the RFC status. UDP lite allows corrupted packet from the MAC layer to reach applications level. This is done by having the Cyclic Redundancy Check (CRC) error checking covering the header only. The checksum minimally covers the UDP lite header and can extend to any offset within the UDP lite packet. Similar evolutions have been made for the specifications of DCCP (Datagram Congestion Control Protocol): appendix B deals with Partial Check summing Design Motivation.

Last, storage of digital media such as DVD or CD raises the same error resiliency problem. Degradation may render the disc unreadable. The life expectancy of DVD or CD record is linked to the error resilience tool used. Record systems usually use error correction codes; they consist in adding some redundancy to the signal which impacts on the storage capacity. In compact disc system error correction is provided by Cross-Interleaved Reed Solomon Codes (CIRC) that can reach 25% of redundancy. The upper limit of error rate indicated in DVD specifications is 1e-2 before correction so as to obtain a rate of 1e-20 after correction. New trend is to accept a higher error rate in order to increase the storage capacity. However it leads actually to decrease the signal quality. Digital media could highly benefit from new robust decoding solutions that could reduce the need of error correction codes and so, increase the storage capacity while limiting the effect of errors on quality.

# Tests conditions

## Error

The tests are based on the assumption that erroneous SDUs are not discarded but forwarded to the application layer.

Proposals should precisely describe the following points:

- the overhead in the bitstream (if any) introduced by the error resiliency method

- encoding configuration and options

- compatibility with the standard

- syntax elements impacted by the error resilience method

- the impact at coder/decoder part

- complexity of the proposed method

- channel/network simulation environment. The simulator provides a corrupted bitstream. Experiments are performed for two kinds of error conditions: random bit errors and bit error patterns. We propose to use the following parameters:

- Random bit errors BER 10e-4

- Random bit errors BER 10e-3

- 3GPP bit error patterns [see Table 1] detailed in [1]

- 802.11b bit error patterns [see Table 2] detailed in [4]

| Name | Bit rate | BER |
| --- | --- | --- |
| 18681.3 | 64 kbps | 9.3e-3 |
| 18681.4 | 64 kbps | 2.9e-3 |
| Wcdma_64kb_3kph_5e-04.bin | 64 kbps | 5.1e-4 |
| Wcdma_64kb_50kph_2e-04.bin | 64 kbps | 1.7e-4 |
| Wcdma_128kb_3kph_5e-04.bin | 128 kbps | 5.0e-4 |
| Wcdma_128kb_50kph_2e-04.bin | 128 kbps | 2.0e-4 |

Table 1: 3GPP bit error patterns

| Name | Bit rate |
| --- | --- |
| bit_trace2mbps_1.bin | 2 mbps |
| bit_trace2mbps_2.bin | 2 mbps |

Table 2: 802.11b error pattern

## Test sequences

The numbers of trials should be sufficient to avoid influence of the position of errors. 10 runs over the whole sequence should be generated: for random errors, using 10 different seeds for the random number generators; for bit error patterns, using 10 different starting positions of the pattern.

The table below defines the sequences and birates to be used in simulations:

| Sequence | Size | Frame rate | Bit rate |
| --- | --- | --- | --- |
| Foreman | QCIF | 7.5 fps | 64 kbps |
| Mobile | QCIF | 7.5 fps | 64 kbps |
| Soccer | QCIF | 15 fps | 128 kbps |
| Bus | QCIF | 15 fps | 128 kbps |
| City | 4CIF | 60 fps | 2048 kbps |

Table 3: Sequences

## Evaluation criteria

The PSNR value is computed between each frame of the coded sequence and the corresponding reconstructed frame. The PSNR is averaged over the sequence. Then, average, maximal and minimal PSNR over the trials are required.

For subjective quality measurement, some tests sequences should be provided.

# References

[1] ITU-T VCEG-N-80 “Common test conditions for RTP/IP over 3GPP/3GPP2”

[2] RFC 3828 The UDP-Lite Protocol, L-A. Larzon, M. Degermark, S. Pink, L-E. Jonsson, G. Fairhurst , 2004

[3] JVT-P206 “Test conditions for error resiliency proposals”

[4] S. Khayam, S. Karande, H. Radha, and D. Loguinov, "Performance Analysis and Modeling of Errors and Losses over 802.11b LANs for High-Bitrate Real-time Multimedia," Signal Processing: Image Communication, vol. 18, no.7, pp. 575-595, August 2003.

[5] 3GPP TS 23.107, "QoS concept and Architecture", V6.2.0, 12/2004

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | INRIA |  |  |
| Mailing address | Campus Universitaire de Beaulieu 35 042 Rennes Cedex |  |  |
| Country | FRANCE |  |  |
| Contact person | Christine Guillemot |  |  |
| Telephone | 33 2 99 84 74 29 |  |  |
| Fax | 33 2 99 84 71 71 |  |  |
| Email | cguillem@irisa.fr |  |  |
| Place and date of submission | 17th Meeting: Nice, France, October 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT/SVC |  |
| Title |  | Conditions for SVC bit error resilience testing |  |
| Contribution number |  | JVT-Q087 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: