| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 17th Meeting: Nice, FR, 14-21 October, 2005 | Document: JVT-Q209 Filename: JVT-Q209.doc |
| --- | --- |

| Title: | Draft Text of H.264/AVC Amendment 2 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document Approved by JVT |  |  |
| Purpose: | Draft (Joint Working Draft of Amendment 2) |  |  |
| Author(s) or Contact(s): | Haoping Yu Corporate Research, Thomson Inc 101 West 103rd Street Indianapolis, IN 46290 USA | Tel: Email: | 1 317-587-3085 haoping.yu@thomson.net |
| Source: | JVT |  |  |

CONTENTS

Introduction	2

1)	New subclause 6.4.3.1 “Inverse 4x4 Cb or Cr block scanning process”	2

2)	New subclause 6.4.4.1 “Inverse 8x8 Cb or Cr block scanning process”	2

3)	Subclause 6.4.8 “Derivation processes for neighbouring macroblocks, blocks, and partitions”	3

4)	New subclause 6.4.8.2.1 “Derivation process for neighbouring 8x8 Cb or Cr block for 4:4:4 chroma sampling”	3

5)	New subclause 6.4.8.4.1 “Derivation process for neighbouring 4x4 Cb or Cr block for 4:4:4 chroma sampling”	4

6)	Subclause 7.3.2.1 “Sequence parameter set RBSP syntax”	4

7)	Subclause 7.3.5.1 “Macroblock prediction syntax”	5

8)	Subclause 7.3.5.3 “Residual data syntax”	5

9)	Subclause 7.4.2.2 “Sequence parameter set RBSP semantics”	8

10)	Subclause 7.4.5 “Macroblock layer semantics”	8

11)	Subclause 7.4.5.1 “Macroblock prediction semantics”	9

12)	Subclause 7.4.5.3 “Residual data semantics”	9

13)	Subclause 8.3 “Intra prediction process”	11

16)	Subclause 8.4 “Inter prediction process”	12

17)	Subclause 8.4.1 “Derivation process for motion vector components and reference indices”	13

19)	Subclause 8.4.2.2 “Fractional sample interpolation process”	14

21)	Subclause 8.5 “Transform coefficient decoding process and picture construction process prior to deblocking filter process”	15

22)	Subclause 8.5.1 “Specification of transform decoding process for 4x4 luma residual blocks”	15

23)	Subclause 8.5.2 “Specification of transform decoding process for luma samples of Intra_16x16 macroblock prediction mode”	16

24)	Subclause 8.5.3.1 “Specification of transform decoding process for 8x8 luma residual blocks”	16

25)	Subclause 8.5.4 “Specification of transform decoding process for chroma samples”	16

27)	Subclause 8.5.6 “Inverse scanning process for 8x8 luma transform coefficients”	17

28)	Subclause 8.5.9 “Scaling and transformation process for chroma DC transform coefficients”	17

30)	Subclause 8.5.12 “Picture construction process prior to deblocking filter process”	18

31)	Subclause 8.5.13 “Residual colour transform process”	18

32)	New subclause 8.5.13 “Lossless intra residual DPCM process”	18

33)	New subclause 8.5.13.1 “Specification of vertical lossless intra residual DPCM”	19

34)	Subclause 8.7 “Deblocking filter process”	20

35)	Subclause 8.7.2.3 “Filtering process for edges with bS less than 4”	21

37)	Subclause 9.2 “CAVLC parsing process for transform coefficient levels”	23

38)	Subclause 9.2.1 “Parsing process for total number of transform coefficient levels and trailing ones”	24

39)	Subclause 9.3.1.1 “Initialisation process for context variables”	24

40)	Subclause 9.3.2 “Binarization process”	35

41)	Subclause 9.3.3.1 “Derivation process for ctxIdx”	37

42)	Subclause 9.3.3.1.1.9 “Derivation process of ctxIdxInc for the syntax element coded_block_flag”	37

43)	Subclause 9.3.3.1.3 “Assignment process of ctxIdxInc for syntax elements significant_coeff_flag, last_significant_coeff_flag, and coeff_abs_level_minus1”	37

44)	Subclause A.2.7 “High 4:4:4 profiles”	37

45)	New subclause A.2.8 “Advanced 4:4:4 profile”	37

46)	Subclause A.3 “Levels”	37

# Introduction

This document is intended to serve as the basis for a new Advanced 4:4:4 profiles amendment to ITU-T Rec. H.264 & ISO/IEC 14496-10.

The current design of the Advanced 4:4:4 profiles, as drafted herein, uses essentially the same decoding process for chroma samples as what is used for luma samples in the same macroblock when the chroma format is 4:4:4, and includes modified intra decoding for use when the qpprime_y_zero_transform_bypass_flag syntax element is equal to 1 and QP'Y is equal to 0. In addition, there have been requests of creating an Intra-only 4:4:4 profile and extending the bit depth to 14 bits in the new 4:4:4 profiles. To keep the changes to the present standard to minimum, a common spatial prediction mode is used in this document for all three input channels in the Intra coded blocks.

# New subclause 6.4.3.1 “Inverse 4x4 Cb or Cr block scanning process”

Insert a new subclause 6.4.3.1 as follows.

6.4.3.1	Inverse 4x4 Cb or Cr block scanning process

This process is used when 4:4:4 chroma sampling is used in bitstreams.

Input to this process is the index of a 4x4 Cb block cb4x4BlkIdx or the index of a 4x4 Cr block cb4x4BlkIdx.

Output of this process is the location (x, y) of the upper-left sample for the 4x4 Cb with index cb4x4BlkIdx or for the Cr block with index cr4x4BlkIdx relative to the upper-left Cb or Cr sample of the macroblock.

The inverse 4x4 scanning process for Cb or Cr component is identical to the scan process shown in Figure 6-10 for luma.

The inverse 4x4 Cb block scanning process is specified by

x = InverseRasterScan( cb4x4BlkIdx / 4, 8, 8, 16, 0 ) + InverseRasterScan( cb4x4BlkIdx % 4, 4, 4, 8, 0 ) 	(6-1a)

y = InverseRasterScan( cb4x4BlkIdx / 4, 8, 8, 16, 1 ) + InverseRasterScan( cb4x4BlkIdx % 4, 4, 4, 8, 1 ) 	(6-2a)

The inverse 4x4 Cr block scanning process is specified by

x = InverseRasterScan( cr4x4BlkIdx / 4, 8, 8, 16, 0 ) + InverseRasterScan( cr4x4BlkIdx % 4, 4, 4, 8, 0 ) 	(6-3b)

y = InverseRasterScan( cr4x4BlkIdx / 4, 8, 8, 16, 1 ) + InverseRasterScan( cr4x4BlkIdx % 4, 4, 4, 8, 1 ) 	(6-4b)

# New subclause 6.4.4.1 “Inverse 8x8 Cb or Cr block scanning process”

Insert a new subclause 6.4.4.1 as follows.

6.4.4.1	Inverse 8x8 Cb or Cr block scanning process

This process is used when 4:4:4 chroma sampling is used in bitstreams.

Input to this process is the index of an 8x8 Cb block cb8x8BlkIdx, or the index of an 8x8 Cr block cr8x8BlkIdx.

Output of this process is the location (x, y) of the upper-left sample for the 8x8 Cb block with index cb8x8BlkIdx or for the 8x8 Cr block with index cr8x8BlkIdx relative to the upper-left Cb or Cr sample of the macroblock.

The inverse 8x8 scanning process for Cb or Cr component is identical to the scan process shown in Figure 6-11 for luma.

The inverse 8x8 Cb block scanning process is specified by

x = InverseRasterScan( cb8x8BlkIdx, 8, 8, 16, 0 )		(6-5a)

y = InverseRasterScan( cb8x8BlkIdx, 8, 8, 16, 1 )		(6-6a)

The inverse 8x8 Cr block scanning process is specified by

x = InverseRasterScan( cr8x8BlkIdx, 8, 8, 16, 0 )		(6-7b)

y = InverseRasterScan( cr8x8BlkIdx, 8, 8, 16, 1 )		(6-8b)

# Subclause 6.4.8 “Derivation processes for neighbouring macroblocks, blocks, and partitions”

Insert the following sentence after the paragraph starting with “Subcluase 6.4.8.2 specifies”.

Subclause 4).1 specifies the derivation process for neighbouring 8x8 Cb or Cr blocks for 4:4:4 chroma sampling.

Insert the following sentence after the paragraph starting with “Subcluase 6.4.8.4 specifies”.

Subclause 6.4.8.4.1 specifies the derivation process for neighbouring 4x4 Cb or Cr blocks for 4:4:4 chroma sampling.

Replace the paragraph starting with “Table 6-2 specifies” with the following.

Table 6-2 specifies the values for the difference of luma location ( xD, yD ) for the input and the replacement for N in mbAddrN, mbPartIdxN, subMbPartIdxN, luma8x8BlkIdxN, cb8x8BlkIdxN, cr8x8BlkIdxN, luma4x4BlkIdxN, cb4x4BlkIdxN, cr4x4BlkIdxN, and chroma4x4BlkIdxN for the output. These input and output assignments are used in subclauses Error: Reference source not found to Error: Reference source not found. The variable predPartWidth is specified when Error: Reference source not found is referred to.

# New subclause 6.4.8.2.1 “Derivation process for neighbouring 8x8 Cb or Cr block for 4:4:4 chroma sampling”

Insert a new subclause 6.4.8.2.1 as follows.

6.4.8.2.1		Derivation process for neighbouring 8x8 Cb or Cr block for 4:4:4 chroma sampling

This process is used when 4:4:4 chroma sampling is used in bitstreams.

The derivation process for neighbouring 8x8 Cb or Cr block is identical to the process described in subclause 6.4.8.2, substituting luma with Cb or Cr, and substituting luma8x8BlkIdx with cb8x8BlkIdx or cr8x8BlkIdx.

# New subclause 6.4.8.4.1 “Derivation process for neighbouring 4x4 Cb or Cr block for 4:4:4 chroma sampling”

Insert a new subclause 6.4.8.4.1 as follows.

6.4.8.4.1		Derivation process for neighbouring 4x4 Cb or Cr block for 4:4:4 chroma sampling

This process is used when 4:4:4 chroma sampling is used in bitstreams.

The derivation process for neighbouring 4x4 Cb or Cr block is identical to the process described in subclause 6.4.8.3, substituting luma with Cb or Cr, and substituting luma4x4BlkIdx with cb4x4BlkIdx or cr4x4BlkIdx.

# Subclause 7.3.2.1 “Sequence parameter set RBSP syntax”

Insert “profile_idc = = 166” and “profile_idc = = 188” to Sequence Parameter Set and delete the syntax element “residual_colour_transform_flag” as follows.

[Ed. Note: The JVT has identified the use (or the lack of use or the particular design for use) of a residual colour transform (RCT) in the Advanced 4:4:4 profile as a topic for further study at this time.  Thus all aspects of this draft relating to that feature should be considered only as reference material for use in such further study.]

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| profile_idc | 0 | u(8) |
| constraint_set0_flag | 0 | u(1) |
| constraint_set1_flag | 0 | u(1) |
| constraint_set2_flag | 0 | u(1) |
| constraint_set3_flag | 0 | u(1) |
| reserved_zero_4bits /* equal to 0 */ | 0 | u(4) |
| level_idc | 0 | u(8) |
| seq_parameter_set_id | 0 | ue(v) |
| if( profile_idc = = 100 ¦ ¦ profile_idc = = 110 ¦ ¦ profile_idc = = 122 ¦ ¦ profile_idc = = 166 ¦ ¦ profile_idc = = 188) { |  |  |
| chroma_format_idc | 0 | ue(v) |
| bit_depth_luma_minus8 | 0 | ue(v) |

# Subclause 7.3.5.1 “Macroblock prediction syntax”

| mb_pred( mb_type ) { | C | Descriptor |
| --- | --- | --- |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_8x8 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ) |  |  |
| for( luma4x4BlkIdx=0; luma4x4BlkIdx<16; luma4x4BlkIdx++ ) { |  |  |
| prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] | 2 | u(1) ¦ ae(v) |
| if( !prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] ) |  |  |
| rem_intra4x4_pred_mode[ luma4x4BlkIdx ] | 2 | u(3) ¦ ae(v) |
| } |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_8x8 ) |  |  |
| for( luma8x8BlkIdx=0; luma8x8BlkIdx<4; luma8x8BlkIdx++ ) { |  |  |
| prev_intra8x8_pred_mode_flag[ luma8x8BlkIdx ] | 2 | u(1) ¦ ae(v) |
| if( !prev_intra8x8_pred_mode_flag[ luma8x8BlkIdx ] ) |  |  |
| rem_intra8x8_pred_mode[ luma8x8BlkIdx ] | 2 | u(3) ¦ ae(v) |
| } |  |  |
| if( chroma_format_idc = = 1 ¦ ¦ chroma_format_idc = =2 ) |  |  |
| intra_chroma_pred_mode | 2 | u(v) ¦ ae(v) |
| } else if( MbPartPredMode( mb_type, 0 ) != Direct ) { |  |  |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( ( num_ref_idx_l0_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag ) && MbPartPredMode( mb_type, mbPartIdx ) != Pred_L1 ) |  |  |
| ref_idx_l0[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( ( num_ref_idx_l1_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag ) && MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 ) |  |  |
| ref_idx_l1[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode ( mb_type, mbPartIdx ) != Pred_L1 ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l0[ mbPartIdx ][ 0 ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l1[ mbPartIdx ][ 0 ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| } |  |  |
| } |  |  |

# Subclause 7.3.5.3 “Residual data syntax”

Replace the content of subclause 7.3.5.3 with the following.

| Residual( ) { | C | Descriptor |
| --- | --- | --- |
| if( !entropy_coding_mode_flag ) |  |  |
| residual_block = residual_block_cavlc |  |  |
| else |  |  |
| residual_block = residual_block_cabac |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) |  |  |
| residual_block( Intra16x16DCLevel, 16 ) | 3 |  |
| for( i8x8 = 0; i8x8 < 4; i8x8++ ) /* each luma 8x8 block */ |  |  |
| if( !transform_size_8x8_flag ¦ ¦ !entropy_coding_mode_flag ) |  |  |
| for( i4x4 = 0; i4x4 < 4; i4x4++ ) { /* each 4x4 sub-block of block */ |  |  |
| if( CodedBlockPatternLuma & ( 1 << i8x8 ) ) |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) |  |  |
| residual_block( Intra16x16ACLevel[ i8x8 * 4 + i4x4 ], 15 ) | 3 |  |
| else |  |  |
| residual_block( LumaLevel[ i8x8 * 4 + i4x4 ], 16 ) | 3 ¦ 4 |  |
| else if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) |  |  |
| for( i = 0; i < 15; i++ ) |  |  |
| Intra16x16ACLevel[ i8x8 * 4 + i4x4 ][ i ] = 0 |  |  |
| else |  |  |
| for( i = 0; i < 16; i++ ) |  |  |
| LumaLevel[ i8x8 * 4 + i4x4 ][ i ] = 0 |  |  |
| if( !entropy_coding_mode_flag && transform_size_8x8_flag ) |  |  |
| for( i = 0; i < 16; i++ ) |  |  |
| LumaLevel8x8[ i8x8 ][ 4 * i + i4x4 ] = LumaLevel[ i8x8 * 4 + i4x4 ][ i ] |  |  |
| } |  |  |
| else if( CodedBlockPatternLuma & ( 1 << i8x8 ) ) |  |  |
| residual_block( LumaLevel8x8[ i8x8 ], 64 ) | 3 ¦ 4 |  |
| else |  |  |
| for( i = 0; i < 64; i++ ) |  |  |
| LumaLevel8x8[ i8x8 ][ i ] = 0 |  |  |
| if( chroma_format_idc == 1 ¦ ¦ chroma_format_idc = =2 ) { |  |  |
| NumC8x8 = 4 / ( SubWidthC * SubHeightC ) |  |  |
| for( iCbCr = 0; iCbCr < 2; iCbCr++ ) |  |  |
| if( CodedBlockPatternChroma & 3 ) /* chroma DC residual present*/ |  |  |
| residual_block( ChromaDCLevel[ iCbCr ], 4 * NumC8x8 ) | 3 ¦ 4 |  |
| else |  |  |
| for( i = 0; i < 4 * NumC8x8; i++ ) |  |  |
| ChromaDCLevel[ iCbCr ][ i ] = 0 |  |  |
| for( iCbCr = 0; iCbCr < 2; iCbCr++ ) |  |  |
| for( i8x8 = 0; i8x8 < NumC8x8; i8x8++ ) |  |  |
| for( i4x4 = 0; i4x4 < 4; i4x4++ ) |  |  |
| if( CodedBlockPatternChroma & 2 ) /* chroma AC residual present */ |  |  |
| residual_block( ChromaACLevel[ iCbCr ][ i8x8*4+i4x4 ],15) | 3 ¦ 4 |  |
| else |  |  |
| for( i = 0; i < 15; i++ ) |  |  |
| ChromaACLevel[ iCbCr ][ i8x8*4+i4x4 ][ i ] = 0 |  |  |
| } else if (chroma_format_idc = = 3) { |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) /*Cb for 4:4:4*/ |  |  |
| residual_block( CbIntra16x16DCLevel, 16 ) | 3 |  |
| for( i8x8 = 0; i8x8 < 4; i8x8++ ) /* each Cb 8x8 block */ |  |  |
| if( !transform_size_8x8_flag ¦ ¦ !entropy_coding_mode_flag ) |  |  |
| for( i4x4 = 0; i4x4 < 4; i4x4++ ) { /* each 4x4 sub-block of block */ |  |  |
| if( CodedBlockPatternLuma & ( 1 << i8x8 ) ) |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) |  |  |
| residual_block(CbIntra16x16ACLevel[i8x8*4+ i4x4],15) | 3 |  |
| else |  |  |
| residual_block( CbLevel[ i8x8 * 4 + i4x4 ], 16) | 3 ¦ 4 |  |
| else if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) |  |  |
| for( i = 0; i < 15; i++ ) |  |  |
| CbIntra16x16ACLevel[ i8x8 * 4 + i4x4 ][ i ] = 0 |  |  |
| else |  |  |
| for( i = 0; i < 16; i++ ) |  |  |
| CbLevel[ i8x8 * 4 + i4x4 ][ i ] = 0 |  |  |
| if( !entropy_coding_mode_flag && transform_size_8x8_flag ) |  |  |
| for( i = 0; i < 16; i++ ) |  |  |
| CbLevel8x8[ i8x8 ][ 4 * i + i4x4 ] = CbLevel[ i8x8 * 4 + i4x4 ][ i ] |  |  |
| } |  |  |
| else if( CodedBlockPatternLuma & ( 1 << i8x8 ) ) |  |  |
| residual_block( CbLevel8x8[ i8x8 ], 64 ) | 3 ¦ 4 |  |
| else |  |  |
| for( i = 0; i < 64; i++ ) |  |  |
| CbLevel8x8[ i8x8 ][ i ] = 0 |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) /*Cr for 4:4:4 */ |  |  |
| residual_block( CrIntra16x16DCLevel, 16 ) | 3 |  |
| for( i8x8 = 0; i8x8 < 4; i8x8++ ) /* each Cr 8x8 block */ |  |  |
| if( !transform_size_8x8_flag ¦ ¦ !entropy_coding_mode_flag ) |  |  |
| for( i4x4 = 0; i4x4 < 4; i4x4++ ) { /* each 4x4 sub-block of block */ |  |  |
| if( CodedBlockPatternLuma & ( 1 << i8x8 ) ) |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) |  |  |
| residual_block(CrIntra16x16ACLevel[i8x8*4+i4x4],15) | 3 |  |
| else |  |  |
| residual_block( CrLevel[ i8x8 * 4 + i4x4 ], 16 ) | 3 ¦ 4 |  |
| else if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) |  |  |
| for( i = 0; i < 15; i++ ) |  |  |
| CrIntra16x16ACLevel[ i8x8 * 4 + i4x4 ][ i ] = 0 |  |  |
| else |  |  |
| for( i = 0; i < 16; i++ ) |  |  |
| CrLevel[ i8x8 * 4 + i4x4 ][ i ] = 0 |  |  |
| if( !entropy_coding_mode_flag && transform_size_8x8_flag ) |  |  |
| for( i = 0; i < 16; i++ ) |  |  |
| CrLevel8x8[ i8x8 ][ 4 * i + i4x4 ] = CrLevel[ i8x8 * 4 + i4x4 ][ i ] |  |  |
| } |  |  |
| else if( CodedBlockPatternLuma & ( 1 << i8x8 ) ) |  |  |
| residual_block( CrLevel8x8[ i8x8 ], 64 ) | 3 ¦ 4 |  |
| else |  |  |
| for( i = 0; i < 64; i++ ) |  |  |
| CrLevel8x8[ i8x8 ][ i ] = 0 |  |  |
| } |  |  |

# Subclause 7.4.2.2 “Sequence parameter set RBSP semantics”

Delete the paragraph starting with “residual_colour_transform_flag”.

[Ed. Note: The JVT has identified the use (or the lack of use or the particular design for use) of a residual colour transform (RCT) in the Advanced 4:4:4 profile as a topic for further study at this time.  Thus all aspects of this draft relating to that feature should be considered only as reference material for use in such further study.]

Replace the paragraph staring with “bit_depth_luma_minus8” with the following.

bit_depth_luma_minus8 specifies the bit depth of the samples of the luma array and the value of the luma quantisation parameter range offset QpBdOffsetY, as specified by

BitDepthY = 8 + bit_depth_luma_minus8		(7-1)

QpBdOffsetY = 6 * bit_depth_luma_minus8		(7-2)

When bit_depth_luma_minus8 is not present, it shall be inferred to be equal to 0.  bit_depth_luma_minus8 shall be in the range of 0 to 6, inclusive.

Replace the paragraph staring with “bit_depth_chroma_minus8” with the following.

bit_depth_chroma_minus8 specifies the bit depth of the samples of the chroma arrays and the value of the chroma quantisation parameter range offset QpBdOffsetC, as specified by

BitDepthC = 8 + bit_depth_chroma_minus8		(7-3)

QpBdOffsetC = 6 * ( bit_depth_chroma_minus8  )		(7-4)

When bit_depth_chroma_minus8 is not present, it shall be inferred to be equal to 0.  bit_depth_chroma_minus8 shall be in the range of 0 to 6, inclusive.

# Subclause 7.4.5 “Macroblock layer semantics”

Insert the following paragraph at the end of the paragraph starting with “To each Intra_16x16”.

When chroma_format_idc is equal to 3, CodedBlockPatternChroma shall be equal to 0. In addition, CodedBlockPatternLuma equal to 0 specifies that all AC transform coefficient levels in the luma, Cb, and Cr components of the macroblock are equal to 0.  CodedBlockPatternLuma equal to 15 specifies that at least one AC transform coefficient levels in the luma , Cb, and Cr components of the macroblock is non-zero, requiring scanning of AC transform coefficient levels for all 48 of the 4x4 blocks in the three 16x16 blocks.

Insert the following paragraph before the paragraph starting with “mb_qb_delta”.

When chroma_format_idc is equal to 3 and coded_block_pattern is present, CodedBlockPatternChroma shall be set to 0. In addition, CodedBlockPatternLuma specifies, for each of the twelve 8x8 luma, Cb, and Cr blocks of the macroblock, one of the following cases.

-	All transform coefficient levels of the twelve 4x4 luma blocks in the 8x8 luma, 8x8 Cb and 8x8 Cr blocks are equal to zero

-	One or more transform coefficient levels of one or more of the 4x4 luma blocks in the 8x8 luma, 8x8 Cb, and 8x8 Cr blocks shall be non-zero valued.

# Subclause 7.4.5.1 “Macroblock prediction semantics”

Replace the paragraph starting with “prev_intra4x4_pred_mode_flag” with the following.

prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] and rem_intra4x4_pred_mode[ luma4x4BlkIdx ] specify the Intra_4x4 prediction of the 4x4 luma block with index luma4x4BlkIdx = 0..15. When chroma_format_idc is equal to 3, prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] and rem_intra4x4_pred_mode[ luma4x4BlkIdx ] also specify the Intra_4x4 prediction of the 4x4 Cb block and 4x4 Cr block with luma4x4BlkIdx equal to index cb4x4BlkIdx = 0..15 or cr4x4BlkIdx = 0..15.

Replace the paragraph starting with “prev_intra8x8_pred_mode_flag” with the following.

prev_intra8x8_pred_mode_flag[ luma8x8BlkIdx ] and rem_intra8x8_pred_mode[ luma8x8BlkIdx ] specify the Intra_8x8 prediction of the 8x8 luma block with index luma8x8BlkIdx = 0..3. When chroma_format_idc is equal to 3, prev_intra8x8_pred_mode_flag[ luma8x8BlkIdx ] and rem_intra8x8_pred_mode[ luma8x8BlkIdx ] also specify the Intra_8x8 prediction of the 8x8 Cb block and 8x8 Cr block with index luma8x8BlkIdx equal to cb8x8BlkIdx = 0..3 or cr8x8BlkIdx = 0..3.

# Subclause 7.4.5.3 “Residual data semantics”

Replace the content of subclause 7.4.5.3 with the following.

The syntax structure residual_block( ), which is used for parsing the transform coefficient levels, is assigned as follows.

- If entropy_coding_mode_flag is equal to 0, residual_block is set equal to residual_block_cavlc, which is used for parsing the syntax elements for transform coefficient levels.

- Otherwise (entropy_coding_mode_flag is equal to 1), residual_block is set equal to residual_block_cabac, which is used for parsing the syntax elements for transform coefficient levels.

Depending on mb_type, luma, chroma, and chroma format, the syntax structure residual_block( coeffLevel, maxNumCoeff ) is used with the arguments coeffLevel, which is a list containing the maxNumCoeff transform coefficient levels that are parsed in residual_block( ) and maxNumCoeff as follows.

- When chroma_format_idc is not equal to 3, the following applies.

- Depending on MbPartPredMode( mb_type, 0 ), the following applies.

- If MbPartPredMode( mb_type, 0 ) is equal to Intra_16x16, the transform coefficient levels are parsed into the list Intra16x16DCLevel and into the 16 lists Intra16x16ACLevel[ i ]. Intra16x16DCLevel contains the 16 transform coefficient levels of the DC transform coefficient levels for each 4x4 luma block. For each of the 16 4x4 luma blocks indexed by I = 0..15, the 15 AC transform coefficients levels of the i-th block are parsed into the i-th list Intra16x16ACLevel[ i ].

- Otherwise (MbPartPredMode( mb_type, 0 ) is not equal to Intra_16x16), the following applies.

- If transform_size_8x8_flag is equal to 0, for each of the 16 4x4 luma blocks indexed by i = 0..15, the 16 transform coefficient levels of the i-th block are parsed into the i-th list LumaLevel[ i ].

- Otherwise (transform_size_8x8_flag is equal to 1), for each of the 4 8x8 luma blocks indexed by i8x8 = 0..3, the following applies.

- If entropy_coding_mode_flag is equal to 0, first for each of the 4 4x4 luma blocks indexed by i4x4 = 0..3, the 16 transform coefficient levels of the i4x4-th block are parsed into the (i8x8 * 4 + i4x4)-th list LumaLevel[ i8x8 * 4 + i4x4 ]. Then, the 64 transform coefficient levels of the i8x8-th 8x8 luma block which are indexed by 4 * i + i4x4, where i = 0..15 and i4x4 = 0..3, are derived as LumaLevel8x8[ i8x8 ][ 4 * i + i4x4 ] = LumaLevel[ i8x8 * 4 + i4x4 ][ i ].

NOTE – The 4x4 luma blocks with luma4x4BlkIdx = i8x8 * 4 + i4x4 containing every fourth transform coefficient level of the corresponding i8x8-th 8x8 luma block with offset i4x4 are assumed to represent spatial locations given by the inverse 4x4 luma block scanning process in subclause 1).

- Otherwise (entropy_coding_mode_flag is equal to 1), the 64 transform coefficient levels of the i8x8-th block are parsed into the i8x8-th list LumaLevel8x8[ i8x8 ].

- For each chroma component, indexed by iCbCr = 0..1, the DC transform coefficient levels of the 4 * NumC8x8 4x4 chroma blocks are parsed into the iCbCr-th list ChromaDCLevel[ iCbCr ].

- For each of the 4x4 chroma blocks, indexed by i4x4 = 0..3 and i8x8 = 0..NumC8x8  1, of each chroma component, indexed by iCbCr = 0..1, the 15 AC transform coefficient levels are parsed into the (i8x8*4 + i4x4)-th list of the iCbCr-th chroma component ChromaACLevel[ iCbCr ][ i8x8*4 + i4x4 ].

- Otherwise when chroma_format_idc is equal to 3 (4:4:4), the transform coefficient levels luma are followed by the transform coefficient levels for Cb which are followed by the transform coefficient levels for Cr. The following applies.

- Depending on MbPartPredMode( mb_type, 0 ), the following applies.

- If MbPartPredMode( mb_type, 0 ) is equal to Intra_16x16, the transform coefficient levels are parsed sequentially into the list Intra16x16DCLevel, into the 16 lists Intra16x16ACLevel[ i ], into the list CbIntra16x16DCLevel, into the 16 lists CbIntra16x16ACLevel[ i ], into CrIntra16x16DCLevel, and into the 16 lists CrIntra16x16ACLevel[ i ]. Intra16x16DCLevel contains the 16 transform coefficient levels of the DC transform coefficient levels for each 4x4 luma block. For each of the 16 4x4 luma blocks indexed by i = 0..15, the 15 AC transform coefficients levels of the i-th block are parsed into the i-th list Intra16x16ACLevel[ i ]. CbIntra16x16DCLevel contains the 16 transform coefficient levels of the DC transform coefficient levels for each 4x4 Cb block. For each of the 16 4x4 Cb blocks indexed by i = 0..15, the 15 AC transform coefficients levels of the i-th block are parsed into the i-th list CbIntra16x16ACLevel[ i ]. CrIntra16x16DCLevel contains the 16 transform coefficient levels of the DC transform coefficient levels for each 4x4 Cr block. For each of the 16 4x4 Cr blocks indexed by I = 0..15, the 15 AC transform coefficients levels of the i-th block are parsed into the i-th list CrIntra16x16ACLevel[ i ].

- Otherwise (MbPartPredMode( mb_type, 0 ) is not equal to Intra_16x16), the following applies.

- If transform_size_8x8_flag is equal to 0, the transform coefficient levels are parsed sequentially into the 16 lists LumaLevel[ i ], into the 16 lists CbLevel[ i ], and into the 16 lists CrLevel[ i ]. For each of the 16 4x4 luma blocks indexed by I = 0..15, the 16 transform coefficient levels of the i-th block are parsed into the i-th list LumaLevel[ i ]. For each of the 16 4x4 Cb blocks indexed by i = 0..15, the 16 transform coefficient levels of the i-th block are parsed into the i-th list CbLevel[ i ]. For each of the 16 4x4 Cr blocks indexed by i = 0..15, the 16 transform coefficient levels of the i-th block are parsed into the i-th list CrLevel[ i ].

- Otherwise (transform_size_8x8_flag is equal to 1), the following applies.

- For each of the 4 8x8 luma blocks indexed by i8x8 = 0..3, the following applies.

- If entropy_coding_mode_flag is equal to 0, first for each of the 4 4x4 luma blocks indexed by i4x4 = 0..3, the 16 transform coefficient levels of the i4x4-th block are parsed into the (i8x8 * 4 + i4x4)-th list LumaLevel[ i8x8 * 4 + i4x4 ]. Then, the 64 transform coefficient levels of the i8x8-th 8x8 luma block which are indexed by 4 * i + i4x4, where i = 0..15 and i4x4 = 0..3, are derived as LumaLevel8x8[ i8x8 ][ 4 * i + i4x4 ] = LumaLevel[ i8x8 * 4 + i4x4 ][ i ].

NOTE – The 4x4 luma blocks with luma4x4BlkIdx = i8x8 * 4 + i4x4 containing every fourth transform coefficient level of the corresponding i8x8-th 8x8 luma block with offset i4x4 are assumed to represent spatial locations given by the inverse 4x4 luma block scanning process in subclause 6.4.3.

- Otherwise (entropy_coding_mode_flag is equal to 1), the 64 transform coefficient levels of the i8x8-th block are parsed into the i8x8-th list LumaLevel8x8[ i8x8 ].

- Following the luma transform coefficient levels, for each of the 4 8x8 Cb blocks indexed by i8x8 = 0..3, the following applies.

- If entropy_coding_mode_flag is equal to 0, first for each of the 4 4x4 Cb blocks indexed by i4x4 = 0..3, the 16 transform coefficient levels of the i4x4-th block are parsed into the (i8x8 * 4 + i4x4)-th list CbLevel[ i8x8 * 4 + i4x4 ]. Then, the 64 transform coefficient levels of the i8x8-th 8x8 Cb block which are indexed by 4 * i + i4x4, where i = 0..15 and i4x4 = 0..3, are derived as CbLevel8x8[ i8x8 ][ 4 * i+ i4x4 ] = CbLevel[ i8x8 * 4 + i4x4 ][ i ].

NOTE – The 4x4 Cb blocks with cb4x4BlkIdx = i8x8 * 4 + i4x4 containing every fourth transform coefficient level of the corresponding i8x8-th 8x8 Cb block with offset i4x4 are assumed to represent spatial locations given by the inverse 4x4 Cb block scanning process in subclause 1).1.

- Otherwise (entropy_coding_mode_flag is equal to 1), the 64 transform coefficient levels of the i8x8-th block are parsed into the i8x8-th list CbLevel8x8[ i8x8 ].

- Following the Cb transform coefficient levels, for each of the 4 8x8 Cr blocks indexed by i8x8 = 0..3, the following applies.

- If entropy_coding_mode_flag is equal to 0, first for each of the 4 4x4 Cr blocks indexed by i4x4 = 0..3, the 16 transform coefficient levels of the i4x4-th block are parsed into the (i8x8 * 4 + i4x4)-th list CrLevel[ i8x8 * 4 + i4x4 ]. Then, the 64 transform coefficient levels of the i8x8-th 8x8 Cr block which are indexed by 4 * i + i4x4, where i = 0..15 and i4x4 = 0..3, are derived as CrLevel8x8[ i8x8 ][ 4 * i + i4x4 ] = CrLevel[ i8x8 * 4 + i4x4 ][ i ].

NOTE – The 4x4 Cr blocks with cr4x4BlkIdx = i8x8 * 4 + i4x4 containing every fourth transform coefficient level of the corresponding i8x8-th 8x8 Cr block with offset i4x4 are assumed to represent spatial locations given by the inverse 4x4 Cr block scanning process in subclause 1).1.

- Otherwise (entropy_coding_mode_flag is equal to 1), the 64 transform coefficient levels of the i8x8-th block are parsed into the i8x8-th list CrLevel8x8[ i8x8 ].

# Subclause 8.3 “Intra prediction process”

Replace the paragraph starting with “Outputs of this process are specified as follows” with the following.

Outputs of this process are specified as follows.

- If mb_type is not equal to I_PCM, the following applies.

- When chroma_format_idc is not equal to 3, the Intra prediction samples of components of the macroblock or in case of the Intra_NxN prediction process for luma samples, the outputs are NxN luma sample arrays as part of the 16x16 luma array of prediction samples of the macroblock.

- Otherwise, the Intra prediction samples of components of the macroblock or in case of the Intra_NxN prediction process for luma, Cb, and Cr samples, the outputs are NxN luma sample arrays as part of the 16x16 luma array of prediction samples of the macroblock, NxN Cb sample arrays as part of the 16x16 Cb array of prediction samples of the macroblock, and NxN Cr sample arrays as part of the 16x16 Cr array of prediction samples of the macroblock.

- Otherwise (mb_type is equal to I_PCM), constructed macroblock samples prior to the deblocking filter process.

Replace the paragraph starting with “ –  The decoding processes for Intra prediction” with the following.

- The decoding processes for Intra prediction modes for the chroma components are described in subclause Error: Reference source not found. This process shall only be invoked when chroma_format_idc is equal to 1 (4:2:0) or 2 (4:2:2).

Insert a new paragraph with the following content after the paragraph starting with “ –  The decoding processes for Intra prediction”.

- When chroma_format_idc is equal to 3 (4:4:4), the decoding processes for Intra prediction modes for the chroma components are described in subclause Error: Reference source not founda.

- Subclause 8.3.4 “Intra prediction for chroma samples”

In subclause 8.3.4, make the following changes.

Replace the title of the subclause with the following.

8.3.4   Intra prediction for chroma samples with chroma_format_idc equal to 1 or 2

Replace the first paragraph the following.

This process is invoked for I and SI macroblock types. It specifies how the Intra prediction chroma samples for the current macroblock are derived when chroma_format_idc is equal to 1 or 2.

- New Subclause 8.3.4a “Intra prediction for chroma samples with chroma_format_idc equal to 3”

Insert a new subclause 8.3.4a as follows.

8.3.4a	Intra prediction for chroma samples with chroma_format_idc equal to 3

This process is invoked for I and SI macroblock types. It specifies how the Intra prediction chroma samples for the current macroblock are derived when chroma_format_idc is equal to 3.

Inputs to this process are constructed samples prior to the deblocking filter process from neighbouring Cb and Cr blocks and for Intra_NxN (where NxN is equal to 4x4 or 8x8) prediction mode, the associated values of IntraNxNPredMode from neighbouring macroblocks.

Outputs of this process are the Intra prediction samples of the Cb and Cr components of the macroblock or in case of the Intra_NxN prediction process, the outputs are NxN Cb sample arrays as part of the 16x16 Cb array of prediction samples of the macroblock, and NxN Cb sample arrays as part of the 16x16 Cb array of prediction samples of the macroblock.

Each Cb, Cr, and luma block with the same block index of the macroblock shall use the same prediction mode.  The prediction mode is applied to each of the Cb and Cr blocks separately. The process specified in this subclause is invoked for each Cb and Cr block.

Depending on the macroblock prediction mode, the following applies.

- If the macroblock prediction mode is equal to Intra_4x4, the following applies.

- The same process described in subclause 8.3.1 shall also be applied to Cb or Cr samples, substituting luma with Cb or Cr, substituting luma4x4BlkIdx with cb4x4BlkIdx or cr4x4BlkIdx, and substituting pred4x4L  with pred4x4Cb or pred4x4Cr.

- The output variable Intra4x4PredMode[luma4x4BlkIdx] from the process described in subclause 8.3.1.1 shall also be used for the 4x4 Cb or 4x4 Cr blocks with index luma4x4BlkIdx equal to index cb4x4BlkIdx or cr4x4BlkIdx.

- The process to derive prediction Cb or Cr samples shall be identical to the process described in subclause 8.3.1.2 and its subsequent subclauses when substituting luma with Cb or Cr, and substituting pred4x4L with pred4x4Cb or pred4x4Cr.

- If the macroblock prediction mode is equal to Intra_8x8, the following applies.

- The same process described in subclause 8.3.2 shall also be applied to Cb or Cr samples, substituting luma with Cb or Cr, substituting luma8x8BlkIdx with cb8x8BlkIdx or cr8x8BlkIdx, and substituting pred8x8L with pred8x8Cb or pred8x8Cr.

- The output variable Intra8x8PredMode[luma8x8BlkIdx] from the process described in subclause 8.3.2.1 shall also be used for the 8x8 Cb or 8x8 Cr blocks with index luma8x8BlkIdx equal to index cb8x8BlkIdx or cr8x8BlkIdx.

- The process to derive prediction Cb or Cr samples shall be identical to the process described in subclause 8.3.2.2 and its subsequent subclauses when substituting luma with Cb or Cr, and substituting pred8x8L with pred8x8Cb or pred8x8Cr.

- If the macroblock prediction mode is equal to Intra_16x16, the following applies.

- The same process described in subclause 8.3.3 and in the subsequent subclause 8.3.3.1 to 8.3.3.4 shall also be applied to Cb or Cr samples, substituting luma with Cb or Cr, and substituting predL predCb or predCr.

# Subclause 8.4 “Inter prediction process”

Replace the paragraph starting with “Outputs of this process are Inter prediction” with the following.

Outputs of this process are Inter prediction samples for the current macroblock that are a 16x16 array predL of luma samples and when chroma_format_idc is equal to 1 or 2 (4:2:0 or 4:2:2) two 8x8 arrays predCr and predCb of chroma samples, one for each of the chroma components Cb and Cr. When chroma_format_idc is equal to 3 (4:4:4), the outputs of this process are a 16x16 array predL of luma samples, a 16x16 array predCb of Cb samples, and a 16x16 arry predCr of Cr samples.

Replace the paragraph starting with “The Inter prediction process for a macroblock partition” with the following.

The Inter prediction process for a macroblock partition mbPartIdx and a sub-macroblock partition subMbPartIdx consists of the following ordered steps

- Derivation process for motion vector components and reference indices as specified in subclause 17).

Inputs to this process are

- a macroblock partition mbPartIdx,

- a sub-macroblock partition subMbPartIdx.

Outputs of this process are

- luma motion vectors mvL0 and mvL1 that are also used as Cb or Cr motion vectors when chroma_format_idc is equal to 3, and the chroma motion vectors mvCL0 and mvCL1 when chroma_format_idc is equal to 1 (4:2:0) or 2 (4:2:2)

- reference indices refIdxL0 and refIdxL1

- prediction list utilization flags predFlagL0 and predFlagL1

- the sub-macroblock partition motion vector count subMvCnt.

- The variable MvCnt is incremented by subMvCnt.

- Decoding process for Inter prediction samples as specified in subclause Error: Reference source not found.

Inputs to this process are

- a macroblock partition mbPartIdx,

- a sub-macroblock partition subMbPartIdx.

- variables specifying partition width and height for luma and chroma (if available), partWidth, partHeight, partWidthC (if available), and partHeightC (if available)

- luma motion vectors mvL0 and mvL1 and when chroma_format_idc is equal to 1 (4:2:0) or 2 (4:2:2) the chroma motion vectors mvCL0 and mvCL1

- reference indices refIdxL0 and refIdxL1

- prediction list utilization flags predFlagL0 and predFlagL1

Outputs of this process are

- inter prediction samples (pred); which are a (partWidth)x(partHeight) array predPartL of prediction luma samples and when chroma_format_idc is not equal to 0 (monochrome) two (partWidthC)x(partHeightC) arrays predPartCb, and predPartCr of prediction chroma samples, one for each of the chroma components Cb and Cr.

# Subclause 8.4.1 “Derivation process for motion vector components and reference indices”

Replace the paragraph starting with “Outputs of this process are” with the following.

Outputs of this process are

- luma motion vectors mvL0 and mvL1

- the chroma motion vectors mvCL0 and mvCL1 when chroma_format_idc is equal to 1 or 2

- reference indices refIdxL0 and refIdxL1

- prediction list utilization flags predFlagL0 and predFlagL1

- a sub-partition macroblock motion vector count variable subMvCnt

Insert the following paragraph after the paragraph starting with “Outputs of this process are”.

When chroma_format_idc is equal to 3, the luma motion vectors mvL0 and mvL1 derived by the process described in this subclause and the following subsequent subclauses shall also be used in the process to derive the Inter prediction samples for the Cb and Cr component.

- Subclause 8.4.1.4 “Derivation process for chroma motion vectors”

In subclause 8.4.1.4, make the following changes.

Replace the title of the subclause with the following.

8.4.1.4   Derivation process for chroma motion vectors with chroma_format_idc equal to 1 or 2

Replace the first paragraph with the following.

This process shall only be invoked when chroma_format_idc is equal to 1 (4:2:0) or 2 (4:2:2).

# Subclause 8.4.2.2 “Fractional sample interpolation process”

Replace the sentence “When chroma_format_idc is not equal to 0 (monochrome), the following applies.” with the following.

When chroma_format_idc is equal to 1 (4:2:0) or 2 (4:2:2), the following applies.

Delete the following content.

- Otherwise (chroma_format_idc is equal to 3),

xIntC = ( xAL / SubWidthC ) + ( mvCLX[ 0 ] >> 2 ) + xC	(8-5)
yIntC = ( yAL / SubHeightC ) + ( mvCLX[ 1 ] >> 2 ) + yC	(8-6)

xFracC = ( mvCLX[ 0 ] & 3 ) << 1		(8-7)
yFracC = ( mvCLX[ 1 ] & 3 ) << 1		(8-8)

Add the following content at the end of the subclause.

When chroma_format_idc is to 3 (4:4:4), the following applies.

Let ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in quarter-sample units. These variables are used only inside this subclause for specifying general fractional-sample locations inside the reference sample arrays refPicLXCb, and refPicLXCr.

For each chroma sample location (0 <= xC < partWidthC, 0 <= yC < partHeightC) inside the prediction chroma sample arrays predPartLXCb and predPartLXCr, the corresponding prediction chroma sample values predPartLXCb[ xC, yC ] and predPartLXCr[ xC, yC ] are derived as follows:

- The variables xIntC, yIntC, xFracC, and yFracC are derived by

xIntC = xAC + ( mvLX[ 0 ] >> 2 ) + xC		(8-232)
yIntL = yAC + ( mvLX[ 1 ] >> 2 ) + yC		(8-233)

xFracC = mvLX[ 0 ] & 3		(8-234)
yFracC = mvLX[ 1 ] & 3		(8-235)

- The prediction sample value predPartLXCb[ xC, yC ] is derived by invoking the process specified in subclause Error: Reference source not found with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicLXCb given as input.

- The prediction sample value predPartLXCr[ xC, yC ] is derived by invoking the process specified in subclause Error: Reference source not found with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicLXCr given as input.

- Subclause 8.4.2.2.2 “Chroma sample interpolation process”

Replace the first paragraph with the following.

This process shall only be invoked when chroma_format_idc is equal to 1 (4:2:0) or 2 (4:2:2).

# Subclause 8.5 “Transform coefficient decoding process and picture construction process prior to deblocking filter process”

Replace the content of subclause 8.5 before subclause 8.5.1 with the following

Inputs to this process are Intra16x16DCLevel (if available), Intra16x16ACLevel (if available), CbIntra16x16DCLevel (if available), CbIntra16x16ACLevel (if available), CrIntra16x16DCLevel (if available), CrIntra16x16ACLevel (if available), LumaLevel (if available), LumaLevel8x8 (if available), ChromaDCLevel (if available), ChromaACLevel (if available), CbLevel (if available), CrLevel (if available), CbLevel8x8 (if available), CrLevel8x8 (if available), and available Inter or Intra prediction sample arrays for the current macroblock for the applicable components predL, predCb, predCr, predL1, or predL2

NOTE – When decoding a macroblock in Intra_4x4 (or Intra_8x8) prediction mode, the luma component of the macroblock prediction array may not be complete, since for each 4x4 (or 8x8) luma block, the Intra_4x4 (or Intra_8x8) prediction process for luma samples as specified in subclause Error: Reference source not found (or Error: Reference source not found) and the process specified in this subclause are iterated. For the same reason, the Cb and Cr component of the macroblock prediction array may not be complete when chroma_format_idc is equal to 3.

Outputs of this process are the constructed sample arrays prior to the deblocking filter process for the applicable components S’L, S’Cb, S’Cr.

NOTE – When decoding a macroblock in Intra_4x4 (or Intra_8x8) prediction mode, the luma component of the macroblock constructed sample arrays prior to the deblocking filter process may not be complete, since for each 4x4 (or 8x8) luma block, the Intra_4x4 (or Intra_8x8) prediction process for luma samples as specified in subclause Error: Reference source not found (or Error: Reference source not found) and the process specified in this subclause are iterated. For the same reason, the Cb and Cr component of the macroblock constructed sample arrays prior to the deblocking filter process may not be complete when chroma_format_idc is equal to 3.

This subclause specifies transform coefficient decoding and picture construction prior to the deblocking filter process.

When the current macroblock is coded as P_Skip or B_Skip, all values of LumaLevel, LumaLevel8x8, CbLevel, CbLevel8x8, CrLevel, CrLevel8x8, ChromaDCLevel, ChromaACLevel are set equal to 0 for the current macroblock.

[Ed. Note: The JVT has identified the use (or the lack of use or the particular design for use) of a residual colour transform (RCT) in the Advanced 4:4:4 profile as a topic for further study at this time.  Thus all aspects of this draft relating to that feature should be considered only as reference material for use in such further study.]

# Subclause 8.5.1 “Specification of transform decoding process for 4x4 luma residual blocks”

Delete the paragraph starting with “3. When residule_color_transform_flag” in subclause 8.5.1 “Specification of transform decoding process for 4x4 luma residual blocks”

[Ed. Note: The JVT has identified the use (or the lack of use or the particular design for use) of a residual colour transform (RCT) in the Advanced 4:4:4 profile as a topic for further study at this time.  Thus all aspects of this draft relating to that feature should be considered only as reference material for use in such further study.]

# Subclause 8.5.2 “Specification of transform decoding process for luma samples of Intra_16x16 macroblock prediction mode”

Delete the paragraph starting with “d. When residule_color_transform_flag”.

[Ed. Note: The JVT has identified the use (or the lack of use or the particular design for use) of a residual colour transform (RCT) in the Advanced 4:4:4 profile as a topic for further study at this time.  Thus all aspects of this draft relating to that feature should be considered only as reference material for use in such further study.]

# Subclause 8.5.3.1 “Specification of transform decoding process for 8x8 luma residual blocks”

Delete the paragraph starting with “3. When residule_color_transform_flag”.

[Ed. Note: The JVT has identified the use (or the lack of use or the particular design for use) of a residual colour transform (RCT) in the Advanced 4:4:4 profile as a topic for further study at this time.  Thus all aspects of this draft relating to that feature should be considered only as reference material for use in such further study.]

# Subclause 8.5.4 “Specification of transform decoding process for chroma samples”

In subclause 8.5.4, make the following changes.

Replace the title of the subclause with the following.

8.5.4	Specification of transform decoding process for chroma samples with chroma_format_idc equal to 1 or 2

Replace the first paragraph the following.

This process is invoked for each chroma component Cb and Cr separately when chroma_format_idc is equal to 1 or 2.

Delete the paragraph starting with “- Otherwise (chroma_format_idc is equal to 3),”.

- New subclause 8.5.4a “Specification of transform decoding process for chroma samples with chroma_format_idc equal to 3”

Insert a new subclause 8.5.4a as follows.

8.5.4a		Specification of transform decoding process for chroma samples with chroma_format_idc equal to 3

This process is invoked for each chroma component Cb and Cr separately when chroma_format_idc is equal to 3.

Depending on the macroblock prediction mode, the following applies.

- If the macroblock prediction mode is equal to Intra_4x4, the transform decoding process for 4x4 Cb or 4x4 Cr residual blocks shall be identical to the process described in subclause 8.5.1 when substituting luma with Cb or Cr, substituting predL with predCb or predCr, substituting LumaLevel with CbLevel or CrLevel, and substituting luma4x4BlkIdx with cb4x4BlkIdx or cr4x4BlkIdx.

- If the macroblock prediction mode is equal to Intra_8x8, the transform decoding process for 8x8 Cb or 8x8 Cr residual blocks shall be identical to the process described in subclause 8.5.3 when substituting luma with Cb or Cr, substituting predL with predCb or predCr, substituting LumaLevel8x8 with CbLevel8x8 or CrLevel8x8, and substituting Luma8x8BlkIdx with cb8x8BlkIdx or cr8x8BlkIdx.

- If the macroblock prediction mode is equal to Intra_16x16, the transform decoding process for Cb or Cr residual blocks shall be identical to the process described in subclause 8.5.2 when substituting luma with Cb or Cr, substituting Intra16x16DCLevel with CbIntra16x16DCLevel or CrIntra16x16DCLevel, and substituting predL with predCb or predCr, substituting luma4x4BlkIdx with cb4x4BlkIdx or cr4x4BlkIdx, substituting lumaList with CbList or CrList.

# Subclause 8.5.6 “Inverse scanning process for 8x8 luma transform coefficients”

Replace the title of the subclause with the following.

    - Inverse scanning process for 8x8 transform coefficients

# Subclause 8.5.9 “Scaling and transformation process for chroma DC transform coefficients”

Replace the phragraph starting with “–	Otherwise (chroma_format_idc is equal to 3), the inverse transform for the 4x4 chroma DC transform coefficients is specified as follows.” with the following.

- Otherwise (chroma_format_idc is equal to 3), the inverse transform for the 4x4 chroma DC transform coefficients is specified as follows.

f=111111−1−11−1−111−11−1c00c01c02c03c10c11c12c13c20c21c22c23c30c31c32c33111111−1−11−1−111−11−1	(8-9)

[Ed. Note: The JVT has identified the use (or the lack of use or the particular design for use) of a residual colour transform (RCT) in the Advanced 4:4:4 profile as a topic for further study at this time.  Thus all aspects of this draft relating to that feature should be considered only as reference material for use in such further study.]

- Subclause 8.5.11 “Scaling and transformation process for residual 8x8 luma blocks”

Replace the first paragraph with the following.

Input to this process is a 4x4 array c with elements cij which is either an array relating to a residual block of the luma component or an array relating to a residual block of a chroma component when chroma_format_idc is equal to 3 (4:4:4).

Replace the paragraph starting with “The scaling process for 8x8 block transform coefficient levels cij proceeds as follows” with the following.

- The variable qP is derived as follows.

- If the input array c relates to a luma residual block

- qP = QP'Y 			(8-357a)

- Otherwise, if the input array c relates to a chroma residual

- qP = QP'C 			(8-357b)

The scaling process for 8x8 block transform coefficient levels cij proceeds as follows.

- If qP is greater than or equal to 36, the scaled result shall be derived as

dij=( cij*LevelScale8x8( qP%6, i, j ))<<(qP / 6 −6 ), with i,j=0..7	(8-358)

[Ed. Note: The equation format is incorrect.]

- Otherwise (QP'Y is less than 36), the scaled result shall be derived as

dij=( cij∗LevelScale8x8( qP%6, i, j )+25−qP6)>>( 6 −qP / 6 ), with i,j=0..7	(8-359)

[Ed. Note: The equation format is incorrect.]

# Subclause 8.5.12 “Picture construction process prior to deblocking filter process”

Replace the first paragraph with the following.

Inputs to this process are

- luma4x4BlkIdx or chroma4x4BlkIdx or luma8x8BlkIdx or cb4x4BlkIdx or cr4x4BlkIdx or cb8x8BlkIdx or cr8x8BlkIdx

- a sample array u with elements uij which is either a 4x4 luma block or a 4x4 chroma block or an 8x8 luma block or an 8x8 chroma block when chroma_format_idc is equal to 3 (4:4:4).

When u is a chroma block, for each sample uij of the chroma block, the following applies.

- The subscript C in the variable S’C is replaced with Cb for the Cb chroma component and with Cr for the Cr chroma component.

- Depending on the size of the block u, the following applies.

- If u is an 4x4 luma block, depending on the variable chroma_format_idc, the position of the upper-left sample of a 4x4 chroma block inside the macroblock is derived as follows.

- If chroma_format_idc is equal to 1 (4:2:0) or 2 (4:2:2), the following applies.

xO = InverseRasterScan( chroma4x4BlkIdx, 4, 4, 8, 0 ) 	(8-10)

yO = InverseRasterScan( chroma4x4BlkIdx, 4, 4, 8, 1 ) 	(8-11)

- If chroma_format_idc is equal to 3 (4:4:4), the position of the upper-left sample of the 4x4 Cb block with index cb4x4BlkIdx or the 4x4 Cr block with index cr4x4BlkIdx inside the macroblock is derived by invoking the inverse 4x4 Cb or Cr block scanning process in subclause 1).1 with cb4x4BlkIdx or cr4x4BlkIdx as the input and the output being assigned to ( xO, yO ), and the variable nE is set equal to 4.

- Otherwise, u is an 8x8 Cb or Cr block when chroma_format_idc is equal to 3. The position of the upper-left sample of the 8x8 Cb block with index cb8x8BlkIdx or the Cr block with index cr8x8BlkIdx inside the macroblock is derived by invoking the inverse 8x8 Cb or Cr block scanning process in subclause Error: Reference source not found.1 with cb8x8BlkIdx or cr8x8BlkIdx as the input and the output being assigned to ( xO, yO ), and the variable nE is set equal to 8.

# Subclause 8.5.13 “Residual colour transform process”

Delete the subclause.

[Ed. Note: The JVT has identified the use (or the lack of use or the particular design for use) of a residual colour transform (RCT) in the Advanced 4:4:4 profile as a topic for further study at this time.  Thus all aspects of this draft relating to that feature should be considered only as reference material for use in such further study.]

# New subclause 8.5.13 “Lossless intra residual DPCM process”

[Ed. Note: This subclause is provided by Yung-Lyul Lee.]

Insert a new subclause8.5.13 as follows.

8.5.13	Lossless intra residual DPCM process

This process is invoked when qpprime_y_zero_transform_bypass_flag is equal to 1, QP'Y is equal to 0, macroblock prediction mode is equal to intra mode (Intra_4x4, Intra_8x8 or Intra_16x16) and the prediction mode is equal to the vertical or horizontal mode. The process for the Cb and Cr components is applied in the same way as the luma (L or Y) component.

Input to this process is a NxN(4x4 or 8x8 or 16x16) array with elements r’i,j which is either an array relating to a residual DPCM block of a luma component or an array relating to a residual DPCM blocks of the Cb and Cr components.

Output of this process is a NxN array with elements ri,j that performed the lossless intra inverse residual DPCM. Also input array and output array are the same size.

ri,j is the intra horizontal and vertical prediction residual at position (i,j).

# New subclause 8.5.13.1 “Specification of vertical lossless intra residual DPCM”

[Ed. Note: This subclause is provided by Yung-Lyul Lee.]

Insert a new subclause 8.5.13.1 as follows.

8.5.13.1 Specification of vertical lossless intra residual DPCM

This lossless intra residual DPCM shall be used when the prediction mode is equal to the vertical prediction mode after subclause 8.3.1.2.1, 8.3.2.2.2 or 8.3.3.1.

Let the r’i,j be a NxN array that performed the lossless intra residual DPCM. The lossless intra residual DPCM to obtain r’i,j is performed as follows.

r’0,j = r0,j with j = 0..ijMax                                                          (8-421zza)

r’i,j = ri,j – ri-1,j with i = 1..ijMax, j = 0..ijMax                                           (8-421zzb)

-  If macroblock prediction mode is equal to Intra_4x4, the ijMax is set equal to 3.

-  If macroblock prediction mode is equal to Intra_8x8, the ijMax is set equal to 7.

-  If macroblock prediction mode is equal to Intra_16x16, the ijMax is set equal to 15.

The lossless intra inverse residual DPCM process is performed as follows.

rij =  with i,j = 0..ijMax                                                     (8-421zzd)

The subclase 8.3.1.2.1, 8.3.2.2.2 or 8.3.3.1 is performed after the lossless intra inverse residual DPCM.

8.5.13.1.2 Specification of horizontal lossless intra residual DPCM

This lossless intra residual transform shall be used when the prediction mode is equal to the horizontal prediction mode after subclause 8.3.1.2.2, 8.3.2.2.3 or 8.3.3.2.

Let the r’i,j be a NxN array that performed the lossless intra residual DPCM. The lossless intra residual DPCM to obtain r’i,j is performed as follows.

r’i,0, = ri,0 with i = 0..ijMax                                                          (8-421zze)

r’i,j = ri,j – ri,j-1 with i = 0..ijMax, j = 1..ijMax                                            (8-421zzf)

The lossless intra inverse residual DPCM process is performed as follows.

rij =  with i,j = 0..ijMax                                                       (8-421zzh)

The subclase 8.3.1.2.2, 8.3.2.2.3 or 8.3.3.2 is performed after the lossless intra inverse residual DPCM.

# Subclause 8.7 “Deblocking filter process”

Replace the first sentence with the following.

A conditional filtering shall be applied to all NxN (where N = 4 or N = 8 for luma and for chroma when chroma_format_idc is equal to 3 (4:4:4), and N = 4 for chroma when chroma_format_idc is equal to 1 (4:2:0) or 2 (4:2:2)) block edges of a picture, except edges at the boundary of the picture and any edges for which the deblocking filter process is disabled by disable_deblocking_filter_idc, as specified below.

Replace the sentence “-  Otherwise, if chroma_format_idc is equal to 3 (4:4:4 format), both types, the solid bold and dashed bold chroma edges are filtered.” With the following.

- Otherwise, if chroma_format_idc is equal to 3 (4:4:4 format), depending on the transform_size_8x8_flag, the following applies.

- If transform_size_8x8_flag is equal to 0, both types, the solid bold and dashed bold chroma edges are filtered.

- Otherwise (transform_size_8x8_flag is equal to 1), only the solid bold chroma edges are filtered.

Replace the paragraph starting with “When filterInternalEdgesFlag is equal to 1, the filtering of the internal vertical chroma edge is specified as follows” with the following.

–	When filterInternalEdgesFlag is equal to 1, the filtering of the internal vertical chroma edge is specified as follows.

–	The process specified in subclause Error: Reference source not found is invoked with mbAddr, chromaEdgeFlag = 1, iCbCr, verticalEdgeFlag = 1, fieldModeFilteringFlag = fieldModeMbFlag, and (xEk, yEk) = (4, k) with k = 0..MbHeightC – 1 as input and S’C with C being replaced by Cb for iCbCr = 0 and C being replaced by Cr for iCbCr = 1 as output.

–	When chroma_format_idc is equal to 3, the process specified in subclause Error: Reference source not found is invoked with mbAddr, chromaEdgeFlag = 1, iCbCr, verticalEdgeFlag = 1, fieldModeFilteringFlag = fieldModeMbFlag, and (xEk, yEk) = (8, k) with k = 0..MbHeightC – 1 as input and S’C with C being replaced by Cb for iCbCr = 0 and C being replaced by Cr for iCbCr = 1 as output.

–	When chroma_format_idc is equal to 3 and when transform_size_8x8_flag is equal to 0, the process specified in subclause Error: Reference source not found is invoked with mbAddr, chromaEdgeFlag = 1, iCbCr, verticalEdgeFlag = 1, fieldModeFilteringFlag = fieldModeMbFlag, and (xEk, yEk) = (12, k) with k = 0..MbHeightC – 1 as input and S’C with C being replaced by Cb for iCbCr = 0 and C being replaced by Cr for iCbCr = 1 as output.

Replace the paragraph starting with “filterInternalEdgesFlag is equal to 1, the filtering of the internal horizontal chroma edge is specified as follows” with the following.

–	When filterInternalEdgesFlag is equal to 1, the filtering of the internal horizontal chroma edge is specified as follows.

–	The process specified in subclause Error: Reference source not found is invoked with mbAddr, chromaEdgeFlag = 1, iCbCr, verticalEdgeFlag = 0, fieldModeFilteringFlag = fieldModeMbFlag, and (xEk, yEk) = (k, 4) with k = 0..MbWidthC – 1 as input and S’C with C being replaced by Cb for iCbCr = 0 and C being replaced by Cr for iCbCr = 1 as output.

–	When chroma_format_idc is not equal to 1, the process specified in subclause Error: Reference source not found is invoked with mbAddr, chromaEdgeFlag = 1, iCbCr, verticalEdgeFlag = 0, fieldModeFilteringFlag = fieldModeMbFlag, and (xEk, yEk) = (k, 8) with k = 0..MbWidthC – 1 as input and S’C with C being replaced by Cb for iCbCr = 0 and C being replaced by Cr for iCbCr = 1 as output.

–	When chroma_format_idc is equal to 2, the process specified in subclause Error: Reference source not found is invoked with mbAddr, chromaEdgeFlag = 1, iCbCr, verticalEdgeFlag = 0, fieldModeFilteringFlag = fieldModeMbFlag, and (xEk, yEk) = (k, 12) with k = 0..MbWidthC – 1 as input and S’C with C being replaced by Cb for iCbCr = 0 and C being replaced by Cr for iCbCr = 1 as output.

–	When chroma_format_idc is equal to 3 and when transform_size_8x8_flag is equal to 0, the process specified in subclause Error: Reference source not found is invoked with mbAddr, chromaEdgeFlag = 1, iCbCr, chroma_format_idc, verticalEdgeFlag = 0, fieldModeFilteringFlag = fieldModeMbFlag, and (xEk, yEk) = (k, 12) with k = 0..MbWidthC – 1 as input and S’C with C being replaced by Cb for iCbCr = 0 and C being replaced by Cr for iCbCr = 1 as output.

Insert a new paragraph with the following content at the end of the subclause.

When qpprime_y_zero_transform_bypass_flag is equal to 1 and QP’Y is equal to 0, this deblocking filter process is not invoked for lossless coding.

# Subclause 8.7.2.3 “Filtering process for edges with bS less than 4”

Replace the paragraph starting with “The filtered result samples p'0 and q'0 are derived by” with the following

The filtered result samples p'0 and q'0 are derived by

 = Clip3( –tC, tC, ( ( ( ( q0 – p0 ) << 2 ) + ( p1 – q1 ) + 4 ) >> 3 ) )	(8-12)

p'0 = Clip1( p0 +  )		(8-13)

q'0 = Clip1( q0 –  )		(8-14)

where the threshold tC is determined as follows.

-	If chromaEdgeFlag is equal to 0,

tC = tC0 + ( ( ap <  ) ? 1 : 0 ) + ( ( aq <  ) ? 1 : 0 ) 		(8-15)

-	Otherwise (chromaEdgeFlag is equal to 1), the following applies.

- When chroma_idc is equal to 3 (4:4:4),

tC = tC0 + ( ( ap <  ) ? 1 : 0 ) + ( ( aq <  ) ? 1 : 0 ) 		(8-162a)

- Otherwise

tC = tC0 + 1 			(8-17)

Replace the paragraph starting with “The filtered result sample p'1” with the following.

The filtered result sample p'1 is derived as follows

-	If chromaEdgeFlag is equal to 0 and ap is less than ,

p'1 = p1 + Clip3( –tC0, tC0, ( p2 + ( ( p0 + q0 + 1 ) >> 1 ) – ( p1 << 1 ) )  >>  1 )	(8-18)

-	If chromaEdgeFlag is equal to 1, chroma_format_idc is equal to 3 (4:4:4), and ap is less than ,

p'1 = p1 + Clip3( –tC0, tC0, ( p2 + ( ( p0 + q0 + 1 ) >> 1 ) – ( p1 << 1 ) )  >>  1 )	(8-19a)

-	Otherwise (chromaEdgeFlag is equal to 1 and chroma_format_idc is not equal to 3, or ap is greater than or equal to ),

p'1 = p1			(8-20)

Replace the paragraph starting with “The filtered result sample” with the following.

The filtered result sample q'1 is derived as follows

-	If chromaEdgeFlag is equal to 0 and aq is less than ,

q'1 = q1 + Clip3( –tC0, tC0, ( q2 + ( ( p0 + q0 + 1 ) >> 1 ) – ( q1 << 1 ) )  >>  1 )	(8-21)

-	If chromaEdgeFlag is equal to 1, chroma_format_idc is equal to 3 (4:4:4), and aq is less than ,

q'1 = q1 + Clip3( –tC0, tC0, ( q2 + ( ( p0 + q0 + 1 ) >> 1 ) – ( q1 << 1 ) )  >>  1 )	(8-22a)

-	Otherwise (chromaEdgeFlag is equal to 1 and chroma_format_idc is not equal to 3, or aq is greater than or equal to ),

q'1 = q1			(8-23)

- Subclause 8.7.2.4 “Filtering process for edges for bS equal to 4”

Replace the paragraph starting with “The filtered result samples p'i (i = 0..2) are derived as follows” with the following.

The filtered result samples p'i (i = 0..2) are derived as follows.

-	If chromaEdgeFlag is equal to 0 and the following condition holds,

ap <   &&  Abs( p0 – q0 ) < ( (  >> 2 ) + 2 )		(8-24)

then the variables p'0, p'1, and p'2 are derived by

p'0 = ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3		(8-25)

p'1 = ( p2 + p1 + p0 + q0 + 2 ) >> 2		(8-26)

p'2 = ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3		(8-27)

-	If chromaEdgeFlag is equal to 1, chroma_format_idc is equal to 3 (4:4:4), and the condition in Equal 8-484 holds, the variables p'0, p'1, and p'2 are derived by

p'0 = ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3		(8-28a)

p'1 = ( p2 + p1 + p0 + q0 + 2 ) >> 2		(8-29a)

p'2 = ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3		(8-30a)

-	Otherwise (chromaEdgeFlag is equal to 1 and chroma_format_idc is not equal to 3, or the condition in Equation 8-24 does not hold), the variables p'0, p'1, and p'2 are derived by

p'0 = ( 2*p1 + p0 + q1 + 2 ) >> 2		(8-31)

p'1 = p1			(8-32)

p'2 = p2			(8-33)

Replace the paragraph starting with “The filtered result samples q'i (i = 0..2) are derived as follows” with the following.

The filtered result samples q'i (i = 0..2) are derived as follows.

-	If chromaEdgeFlag is equal to 0 and the following condition holds,

aq <   &&  Abs( p0 – q0 ) < ( (  >> 2 ) + 2 )		(8-34)

then the variables q'0, q'1, and q'2 are derived by

q'0 = ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3		(8-35)

q'1 = ( p0 + q0 + q1 + q2 + 2 ) >> 2		(8-36)

q'2 = ( 2*q3 + 3*q2 + q1 + q0 + p0 + 4 ) >> 3		(8-37)

-	If chromaEdgeFlag is equal to 1, chroma_format_idc is equal to 3 (4:4:4), and the condition in Equation 8-491 holds, the variables q'0, q'1, and q'2 are derived by

q'0 = ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3		(8-38a)

q'1 = ( p0 + q0 + q1 + q2 + 2 ) >> 2		(8-39a)

q'2 = ( 2*q3 + 3*q2 + q1 + q0 + p0 + 4 ) >> 3		(8-40a)

-	Otherwise (chromaEdgeFlag is equal to 1 and chroma_format_idc is not equal to 3, or the condition in Equation 8-34 does not hold), the variables q'0, q'1, and q'2 are derived by

q'0 = ( 2*q1 + q0 + p1 + 2 ) >> 2		(8-41)

q'1 = q1			(8-42)

q'2 = q2			(8-43)

# Subclause 9.2 “CAVLC parsing process for transform coefficient levels”

Replace the paragraph starting with “Inputs to this process” with the following.

Inputs to this process are bits from slice data, a maximum number of non-zero transform coefficient levels maxNumCoeff, the luma block index luma4x4BlkIdx or the chroma block index chroma4x4BlkIdx or index cb4x4BlkIdx or index cr4x4BlkIdx of the current block of transform coefficient levels.

Replace the paragraph starting with “Outputs to this process” with the following.

Output of this process is the list coeffLevel containing transform coefficient levels of the luma block with block index luma4x4BlkIdx or the chroma block with block index chroma4x4BlkIdx or index cb4x4BlkIdx or index cr4x4BlkIdx.

# Subclause 9.2.1 “Parsing process for total number of transform coefficient levels and trailing ones”

Replace the paragraph starting with “ inputs to this process” with the following.

Inputs to this process are bits from slice data, a maximum number of non-zero transform coefficient levels maxNumCoeff, the luma block index luma4x4BlkIdx or the chroma block index chroma4x4BlkIdx or index cb4x4BlkIdx or index cr4x4BlkIdx of the current block of transform.

Delete the paragraph “Otherwise (chroma_format_idc is equal to 3), nC is set equal to 0.”

Add two new paragraphs after the paragraph starting with “When the CAVLC parsing process is invoked for Intra16x16DCLevel” with the following

When the CAVLC parsing process is invoked for CbIntra16x16DCLevel, cb4x4BlkIdx is set equal to 0.

When the CAVLC parsing process is invoked for CrIntra16x16DCLevel, cr4x4BlkIdx is set equal to 0.

Add two new paragraphs after the paragraph starting with “If the CAVLC parsing process is invoked for Intra16x16DCLevel” with the following

If the CAVLC parsing process is invoked for CbIntra16x16DCLevel, CbIntra16x16ACLevel, or CbLevel, the process specified in subclause 6.4.8.4.1 is invoked with Cb4x4BlkIdx as the input, and the output is assigned to mbAddrA, mbAddrB, cb4x4BlkIdxA, and cb4x4BlkIdxB. The 4x4 Cb block specified by mbAddrA\cb4x4BlkIdxA is assigned to blkA, and the 4x4 Cb block specified by mbAddrB\cb4x4BlkIdxB is assigned to blkB.

If the CAVLC parsing process is invoked for CrIntra16x16DCLevel, CrIntra16x16ACLevel, or CrLevel, the process specified in subclause 6.4.8.4.1 is invoked with cr4x4BlkIdx as the input, and the output is assigned to mbAddrA, mbAddrB, cr4x4BlkIdxA, and cr4x4BlkIdxB. The 4x4 Cr block specified by mbAddrA\cr4x4BlkIdxA is assigned to blkA, and the 4x4 Cr block specified by mbAddrB\cr4x4BlkIdxB is assigned to blkB.

Replace the paragraph starting with “NOTE - When parsing for Intra16x16DCLevel,” with the following

NOTE - When parsing for Intra16x16DCLevel, CbIntra16x16DCLevel, or CrIntra16x16DCLevel, the values nA and nB are based on the number of non-zero transform coefficient levels in adjacent 4x4 blocks and not on the number of non-zero DC transform coefficient levels in adjacent 16x16 blocks.

# Subclause 9.3.1.1 “Initialisation process for context variables”

Replace Table 9-11 with the following.

Table 9-1 – Association of ctxIdx and syntax elements for each slice type in the initialisation process

|  | Syntax element | Table | Slice type |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SI | I | P, SP | B |
| slice_data( ) | mb_skip_flag | Error: Reference source not found Error: Reference source not found |  |  | 11-13 | 24-26 |
|  | mb_field_decoding_flag | Error: Reference source not found | 70-72 | 70-72 | 70-72 | 70-72 |
| macroblock_layer( ) | mb_type | Error: Reference source not found Error: Reference source not found Error: Reference source not found | 0-10 | 3-10 | 14-20 | 27-35 |
|  | transform_size_8x8_flag | Error: Reference source not found | na | 399-401 | 399-401 | 399-401 |
|  | coded_block_pattern (luma) | Error: Reference source not found | 73-76 | 73-76 | 73-76 | 73-76 |
|  | coded_block_pattern (chroma) | Error: Reference source not found | 77-84 | 77-84 | 77-84 | 77-84 |
|  | mb_qp_delta | Error: Reference source not found | 60-63 | 60-63 | 60-63 | 60-63 |
| mb_pred( ) | prev_intra4x4_pred_mode_flag | Error: Reference source not found | 68 | 68 | 68 | 68 |
|  | rem_intra4x4_pred_mode | Error: Reference source not found | 69 | 69 | 69 | 69 |
|  | prev_intra8x8_pred_mode_flag | Error: Reference source not found | na | 68 | 68 | 68 |
|  | rem_intra8x8_pred_mode | Error: Reference source not found | na | 69 | 69 | 69 |
|  | intra_chroma_pred_mode | Error: Reference source not found | 64-67 | 64-67 | 64-67 | 64-67 |
| mb_pred( ) and sub_mb_pred( ) | ref_idx_l0 | Error: Reference source not found |  |  | 54-59 | 54-59 |
|  | ref_idx_l1 | Error: Reference source not found |  |  |  | 54-59 |
|  | mvd_l0[ ][ ][ 0 ] | Error: Reference source not found |  |  | 40-46 | 40-46 |
|  | mvd_l1[ ][ ][ 0 ] | Error: Reference source not found |  |  |  | 40-46 |
|  | mvd_l0[ ][ ][ 1 ] | Error: Reference source not found |  |  | 47-53 | 47-53 |
|  | mvd_l1[ ][ ][ 1 ] | Error: Reference source not found |  |  |  | 47-53 |
| sub_mb_pred( ) | sub_mb_type | Error: Reference source not found Error: Reference source not found |  |  | 21-23 | 36-39 |
| residual_block_cabac( ) | coded_block_flag | Error: Reference source not found Table 9-18-1 | 85-104 460-483 | 85-104 460-483 | 85-104 460-483 | 85-104 460-483 |
|  | significant_coeff_flag[ ] | Table 9-3 Table 9-6 Error: Reference source not found Error: Reference source not found Table 9-19-1 Table 9-22-1 Table 9-24-1 Table 9-24-2 | 105-165 277-337 484-571 776-863 660-689 718-747 | 105-165 277-337 402-416 436-450 484-571 776-863 660-689 718-747 | 105-165 277-337 402-416 436-450 484-571 776-863 660-689 718-747 | 105-165 277-337 402-416 436-450 484-571 776-863 660-689 718-747 |
|  | last_significant_coeff_flag[ ] | Error: Reference source not found Table 9-7 Error: Reference source not found Error: Reference source not found Table 9-20-1 Table 9-23-1 Table 9-24-1 Table 9-24-2 | 166-226 338-398 572-659 864-951 690-707 748-765 | 166-226 338-398 417-425 451-459 572-659 864-951 690-707 748-765 | 166-226 338-398 417-425 451-459 572-659 864-951 690-707 748-765 | 166-226 338-398 417-425 451-459 572-659 864-951 690-707 748-765 |
|  | coeff_abs_level_minus1[ ] | Table 9-5 Error: Reference source not found Table 9-21-1 Table 9-24-1 | 227-275 | 227-275 426-435 952-1011 708-717 | 227-275 426-435 952-1011 708-717 | 227-275 426-435 952-1011 708-717 |

Insert a new Table 9-18-1 as following.

Table 9-2-1 – Values of variables m and n for ctxIdx from 460 to 483

| ctxIdx | I and SI slices |  | Value of cabac_init_idc |  |  |  |  |  | ctxIdx | I and SI slices |  | Value of cabac_init_idc |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 |  | 1 |  | 2 |  |  |  |  | 0 |  | 1 |  | 2 |  |
|  | m | n | m | n | m | n | m | n |  | m | n | m | n | m | n | m | n |
| 460 | -17 | 123 | -7 | 92 | 0 | 80 | 11 | 80 | 472 | -17 | 123 | -7 | 92 | 0 | 80 | 11 | 80 |
| 461 | -12 | 115 | -5 | 89 | -5 | 89 | 5 | 76 | 473 | -12 | 115 | -5 | 89 | -5 | 89 | 5 | 76 |
| 462 | -16 | 122 | -7 | 96 | -7 | 94 | 2 | 84 | 474 | -16 | 122 | -7 | 96 | -7 | 94 | 2 | 84 |
| 463 | -11 | 115 | -13 | 108 | -4 | 92 | 5 | 78 | 475 | -11 | 115 | -13 | 108 | -4 | 92 | 5 | 78 |
| 464 | -12 | 63 | -3 | 46 | 0 | 39 | -6 | 55 | 476 | -12 | 63 | -3 | 46 | 0 | 39 | -6 | 55 |
| 465 | -2 | 68 | -1 | 65 | 0 | 65 | 4 | 61 | 477 | -2 | 68 | -1 | 65 | 0 | 65 | 4 | 61 |
| 466 | -15 | 84 | -1 | 57 | -15 | 84 | -14 | 83 | 478 | -15 | 84 | -1 | 57 | -15 | 84 | -14 | 83 |
| 467 | -13 | 104 | -9 | 93 | -35 | 127 | -37 | 127 | 479 | -13 | 104 | -9 | 93 | -35 | 127 | -37 | 127 |
| 468 | -3 | 70 | -3 | 74 | -2 | 73 | -5 | 79 | 480 | -3 | 70 | -3 | 74 | -2 | 73 | -5 | 79 |
| 469 | -8 | 93 | -9 | 92 | -12 | 104 | -11 | 104 | 481 | -8 | 93 | -9 | 92 | -12 | 104 | -11 | 104 |
| 470 | -10 | 90 | -8 | 87 | -9 | 91 | -11 | 91 | 482 | -10 | 90 | -8 | 87 | -9 | 91 | -11 | 91 |
| 471 | -30 | 127 | -23 | 126 | -31 | 127 | -30 | 127 | 483 | -30 | 127 | -23 | 126 | -31 | 127 | -30 | 127 |

Insert a new Table 9-19-1 as following.

Table 9-3-1 – Values of variables m and n for ctxIdx from 484 to 571

| ctxIdx | I and SI slices |  | Value of cabac_init_idc |  |  |  |  |  | ctxIdx | I and SI slices |  | Value of cabac_init_idc |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 |  | 1 |  | 2 |  |  |  |  | 0 |  | 1 |  | 2 |  |
|  | m | n | m | n | m | n | m | n |  | m | n | m | n | m | n | m | n |
| 484 | -7 | 93 | -2 | 85 | -13 | 103 | -4 | 86 | 528 | -7 | 93 | -2 | 85 | -13 | 103 | -4 | 86 |
| 485 | -11 | 87 | -6 | 78 | -13 | 91 | -12 | 88 | 529 | -11 | 87 | -6 | 78 | -13 | 91 | -12 | 88 |
| 486 | -3 | 77 | -1 | 75 | -9 | 89 | -5 | 82 | 530 | -3 | 77 | -1 | 75 | -9 | 89 | -5 | 82 |
| 487 | -5 | 71 | -7 | 77 | -14 | 92 | -3 | 72 | 531 | -5 | 71 | -7 | 77 | -14 | 92 | -3 | 72 |
| 488 | -4 | 63 | 2 | 54 | -8 | 76 | -4 | 67 | 532 | -4 | 63 | 2 | 54 | -8 | 76 | -4 | 67 |
| 489 | -4 | 68 | 5 | 50 | -12 | 87 | -8 | 72 | 533 | -4 | 68 | 5 | 50 | -12 | 87 | -8 | 72 |
| 490 | -12 | 84 | -3 | 68 | -23 | 110 | -16 | 89 | 534 | -12 | 84 | -3 | 68 | -23 | 110 | -16 | 89 |
| 491 | -7 | 62 | 1 | 50 | -24 | 105 | -9 | 69 | 535 | -7 | 62 | 1 | 50 | -24 | 105 | -9 | 69 |
| 492 | -7 | 65 | 6 | 42 | -10 | 78 | -1 | 59 | 536 | -7 | 65 | 6 | 42 | -10 | 78 | -1 | 59 |
| 493 | 8 | 61 | -4 | 81 | -20 | 112 | 5 | 66 | 537 | 8 | 61 | -4 | 81 | -20 | 112 | 5 | 66 |
| 494 | 5 | 56 | 1 | 63 | -17 | 99 | 4 | 57 | 538 | 5 | 56 | 1 | 63 | -17 | 99 | 4 | 57 |
| 495 | -2 | 66 | -4 | 70 | -78 | 127 | -4 | 71 | 539 | -2 | 66 | -4 | 70 | -78 | 127 | -4 | 71 |
| 496 | 1 | 64 | 0 | 67 | -70 | 127 | -2 | 71 | 540 | 1 | 64 | 0 | 67 | -70 | 127 | -2 | 71 |
| 497 | 0 | 61 | 2 | 57 | -50 | 127 | 2 | 58 | 641 | 0 | 61 | 2 | 57 | -50 | 127 | 2 | 58 |
| 498 | -2 | 78 | -2 | 76 | -46 | 127 | -1 | 74 | 542 | -2 | 78 | -2 | 76 | -46 | 127 | -1 | 74 |
| 499 | 1 | 50 | 11 | 35 | -4 | 66 | -4 | 44 | 543 | 1 | 50 | 11 | 35 | -4 | 66 | -4 | 44 |
| 500 | 7 | 52 | 4 | 64 | -5 | 78 | -1 | 69 | 544 | 7 | 52 | 4 | 64 | -5 | 78 | -1 | 69 |
| 501 | 10 | 35 | 1 | 61 | -4 | 71 | 0 | 62 | 545 | 10 | 35 | 1 | 61 | -4 | 71 | 0 | 62 |
| 502 | 0 | 44 | 11 | 35 | -8 | 72 | -7 | 51 | 546 | 0 | 44 | 11 | 35 | -8 | 72 | -7 | 51 |
| 503 | 11 | 38 | 18 | 25 | 2 | 59 | -4 | 47 | 547 | 11 | 38 | 18 | 25 | 2 | 59 | -4 | 47 |
| 504 | 1 | 45 | 12 | 24 | -1 | 55 | -6 | 42 | 548 | 1 | 45 | 12 | 24 | -1 | 55 | -6 | 42 |
| 505 | 0 | 46 | 13 | 29 | -7 | 70 | -3 | 41 | 549 | 0 | 46 | 13 | 29 | -7 | 70 | -3 | 41 |
| 506 | 5 | 44 | 13 | 36 | -6 | 75 | -6 | 53 | 550 | 5 | 44 | 13 | 36 | -6 | 75 | -6 | 53 |
| 507 | 31 | 17 | -10 | 93 | -8 | 89 | 8 | 76 | 551 | 31 | 17 | -10 | 93 | -8 | 89 | 8 | 76 |
| 508 | 1 | 51 | -7 | 73 | -34 | 119 | -9 | 78 | 552 | 1 | 51 | -7 | 73 | -34 | 119 | -9 | 78 |
| 509 | 7 | 50 | -2 | 73 | -3 | 75 | -11 | 83 | 553 | 7 | 50 | -2 | 73 | -3 | 75 | -11 | 83 |
| 510 | 28 | 19 | 13 | 46 | 32 | 20 | 9 | 52 | 554 | 28 | 19 | 13 | 46 | 32 | 20 | 9 | 52 |
| 511 | 16 | 33 | 9 | 49 | 30 | 22 | 0 | 67 | 555 | 16 | 33 | 9 | 49 | 30 | 22 | 0 | 67 |
| 512 | 14 | 62 | -7 | 100 | -44 | 127 | -5 | 90 | 556 | 14 | 62 | -7 | 100 | -44 | 127 | -5 | 90 |
| 513 | -13 | 108 | 9 | 53 | 0 | 54 | 1 | 67 | 557 | -13 | 108 | 9 | 53 | 0 | 54 | 1 | 67 |
| 514 | -15 | 100 | 2 | 53 | -5 | 61 | -15 | 72 | 558 | -15 | 100 | 2 | 53 | -5 | 61 | -15 | 72 |
| 515 | -13 | 101 | 5 | 53 | 0 | 58 | -5 | 75 | 559 | -13 | 101 | 5 | 53 | 0 | 58 | -5 | 75 |
| 516 | -13 | 91 | -2 | 61 | -1 | 60 | -8 | 80 | 560 | -13 | 91 | -2 | 61 | -1 | 60 | -8 | 80 |
| 517 | -12 | 94 | 0 | 56 | -3 | 61 | -21 | 83 | 561 | -12 | 94 | 0 | 56 | -3 | 61 | -21 | 83 |
| 518 | -10 | 88 | 0 | 56 | -8 | 67 | -21 | 64 | 562 | -10 | 88 | 0 | 56 | -8 | 67 | -21 | 64 |
| 519 | -16 | 84 | -13 | 63 | -25 | 84 | -13 | 31 | 563 | -16 | 84 | -13 | 63 | -25 | 84 | -13 | 31 |
| 520 | -10 | 86 | -5 | 60 | -14 | 74 | -25 | 64 | 564 | -10 | 86 | -5 | 60 | -14 | 74 | -25 | 64 |
| 521 | -7 | 83 | -1 | 62 | -5 | 65 | -29 | 94 | 565 | -7 | 83 | -1 | 62 | -5 | 65 | -29 | 94 |
| 522 | -13 | 87 | 4 | 57 | 5 | 52 | 9 | 75 | 566 | -13 | 87 | 4 | 57 | 5 | 52 | 9 | 75 |
| 523 | -19 | 94 | -6 | 69 | 2 | 57 | 17 | 63 | 567 | -19 | 94 | -6 | 69 | 2 | 57 | 17 | 63 |
| 524 | 1 | 70 | 4 | 57 | 0 | 61 | -8 | 74 | 568 | 1 | 70 | 4 | 57 | 0 | 61 | -8 | 74 |
| 525 | 0 | 72 | 14 | 39 | -9 | 69 | -5 | 35 | 569 | 0 | 72 | 14 | 39 | -9 | 69 | -5 | 35 |
| 526 | -5 | 74 | 4 | 51 | -11 | 70 | -2 | 27 | 570 | -5 | 74 | 4 | 51 | -11 | 70 | -2 | 27 |
| 527 | 18 | 59 | 13 | 68 | 18 | 55 | 13 | 91 | 571 | 18 | 59 | 13 | 68 | 18 | 55 | 13 | 91 |

Insert a new Table 9-20-1 as following.

Table 9-4-1 Values of variables m and n for ctxIdx from 572 to 659

| ctxIdx | I and SI slices |  | Value of cabac_init_idc |  |  |  |  |  | ctxIdx | I and SI slices |  | Value of cabac_init_idc |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 |  | 1 |  | 2 |  |  |  |  | 0 |  | 1 |  | 2 |  |
|  | m | n | m | n | m | n | m | n |  | m | n | m | n | m | n | m | n |
| 572 | 24 | 0 | 11 | 28 | 4 | 45 | 4 | 39 | 616 | 24 | 0 | 11 | 28 | 4 | 45 | 4 | 39 |
| 573 | 15 | 9 | 2 | 40 | 10 | 28 | 0 | 42 | 617 | 15 | 9 | 2 | 40 | 10 | 28 | 0 | 42 |
| 574 | 8 | 25 | 3 | 44 | 10 | 31 | 7 | 34 | 618 | 8 | 25 | 3 | 44 | 10 | 31 | 7 | 34 |
| 575 | 13 | 18 | 0 | 49 | 33 | -11 | 11 | 29 | 619 | 13 | 18 | 0 | 49 | 33 | -11 | 11 | 29 |
| 576 | 15 | 9 | 0 | 46 | 52 | -43 | 8 | 31 | 620 | 15 | 9 | 0 | 46 | 52 | -43 | 8 | 31 |
| 577 | 13 | 19 | 2 | 44 | 18 | 15 | 6 | 37 | 621 | 13 | 19 | 2 | 44 | 18 | 15 | 6 | 37 |
| 578 | 10 | 37 | 2 | 51 | 28 | 0 | 7 | 42 | 622 | 10 | 37 | 2 | 51 | 28 | 0 | 7 | 42 |
| 579 | 12 | 18 | 0 | 47 | 35 | -22 | 3 | 40 | 623 | 12 | 18 | 0 | 47 | 35 | -22 | 3 | 40 |
| 580 | 6 | 29 | 4 | 39 | 38 | -25 | 8 | 33 | 624 | 6 | 29 | 4 | 39 | 38 | -25 | 8 | 33 |
| 581 | 20 | 33 | 2 | 62 | 34 | 0 | 13 | 43 | 625 | 20 | 33 | 2 | 62 | 34 | 0 | 13 | 43 |
| 582 | 15 | 30 | 6 | 46 | 39 | -18 | 13 | 36 | 626 | 15 | 30 | 6 | 46 | 39 | -18 | 13 | 36 |
| 583 | 4 | 45 | 0 | 54 | 32 | -12 | 4 | 47 | 627 | 4 | 45 | 0 | 54 | 32 | -12 | 4 | 47 |
| 584 | 1 | 58 | 3 | 54 | 102 | -94 | 3 | 55 | 628 | 1 | 58 | 3 | 54 | 102 | -94 | 3 | 55 |
| 585 | 0 | 62 | 2 | 58 | 0 | 0 | 2 | 58 | 629 | 0 | 62 | 2 | 58 | 0 | 0 | 2 | 58 |
| 586 | 7 | 61 | 4 | 63 | 56 | -15 | 6 | 60 | 630 | 7 | 61 | 4 | 63 | 56 | -15 | 6 | 60 |
| 587 | 12 | 38 | 6 | 51 | 33 | -4 | 8 | 44 | 631 | 12 | 38 | 6 | 51 | 33 | -4 | 8 | 44 |
| 588 | 11 | 45 | 6 | 57 | 29 | 10 | 11 | 44 | 632 | 11 | 45 | 6 | 57 | 29 | 10 | 11 | 44 |
| 589 | 15 | 39 | 7 | 53 | 37 | -5 | 14 | 42 | 633 | 15 | 39 | 7 | 53 | 37 | -5 | 14 | 42 |
| 590 | 11 | 42 | 6 | 52 | 51 | -29 | 7 | 48 | 634 | 11 | 42 | 6 | 52 | 51 | -29 | 7 | 48 |
| 591 | 13 | 44 | 6 | 55 | 39 | -9 | 4 | 56 | 635 | 13 | 44 | 6 | 55 | 39 | -9 | 4 | 56 |
| 592 | 16 | 45 | 11 | 45 | 52 | -34 | 4 | 52 | 636 | 16 | 45 | 11 | 45 | 52 | -34 | 4 | 52 |
| 593 | 12 | 41 | 14 | 36 | 69 | -58 | 13 | 37 | 637 | 12 | 41 | 14 | 36 | 69 | -58 | 13 | 37 |
| 594 | 10 | 49 | 8 | 53 | 67 | -63 | 9 | 49 | 638 | 10 | 49 | 8 | 53 | 67 | -63 | 9 | 49 |
| 595 | 30 | 34 | -1 | 82 | 44 | -5 | 19 | 58 | 639 | 30 | 34 | -1 | 82 | 44 | -5 | 19 | 58 |
| 596 | 18 | 42 | 7 | 55 | 32 | 7 | 10 | 48 | 640 | 18 | 42 | 7 | 55 | 32 | 7 | 10 | 48 |
| 597 | 10 | 55 | -3 | 78 | 55 | -29 | 12 | 45 | 641 | 10 | 55 | -3 | 78 | 55 | -29 | 12 | 45 |
| 598 | 17 | 51 | 15 | 46 | 32 | 1 | 0 | 69 | 642 | 17 | 51 | 15 | 46 | 32 | 1 | 0 | 69 |
| 599 | 17 | 46 | 22 | 31 | 0 | 0 | 20 | 33 | 643 | 17 | 46 | 22 | 31 | 0 | 0 | 20 | 33 |
| 600 | 0 | 89 | -1 | 84 | 27 | 36 | 8 | 63 | 644 | 0 | 89 | -1 | 84 | 27 | 36 | 8 | 63 |
| 601 | 26 | -19 | 25 | 7 | 33 | -25 | 35 | -18 | 645 | 26 | -19 | 25 | 7 | 33 | -25 | 35 | -18 |
| 602 | 22 | -17 | 30 | -7 | 34 | -30 | 33 | -25 | 646 | 22 | -17 | 30 | -7 | 34 | -30 | 33 | -25 |
| 603 | 26 | -17 | 28 | 3 | 36 | -28 | 28 | -3 | 647 | 26 | -17 | 28 | 3 | 36 | -28 | 28 | -3 |
| 604 | 30 | -25 | 28 | 4 | 38 | -28 | 24 | 10 | 648 | 30 | -25 | 28 | 4 | 38 | -28 | 24 | 10 |
| 605 | 28 | -20 | 32 | 0 | 38 | -27 | 27 | 0 | 649 | 28 | -20 | 32 | 0 | 38 | -27 | 27 | 0 |
| 606 | 33 | -23 | 34 | -1 | 34 | -18 | 34 | -14 | 650 | 33 | -23 | 34 | -1 | 34 | -18 | 34 | -14 |
| 607 | 37 | -27 | 30 | 6 | 35 | -16 | 52 | -44 | 651 | 37 | -27 | 30 | 6 | 35 | -16 | 52 | -44 |
| 608 | 33 | -23 | 30 | 6 | 34 | -14 | 39 | -24 | 652 | 33 | -23 | 30 | 6 | 34 | -14 | 39 | -24 |
| 609 | 40 | -28 | 32 | 9 | 32 | -8 | 19 | 17 | 653 | 40 | -28 | 32 | 9 | 32 | -8 | 19 | 17 |
| 610 | 38 | -17 | 31 | 19 | 37 | -6 | 31 | 25 | 654 | 38 | -17 | 31 | 19 | 37 | -6 | 31 | 25 |
| 611 | 33 | -11 | 26 | 27 | 35 | 0 | 36 | 29 | 655 | 33 | -11 | 26 | 27 | 35 | 0 | 36 | 29 |
| 612 | 40 | -15 | 26 | 30 | 30 | 10 | 24 | 33 | 656 | 40 | -15 | 26 | 30 | 30 | 10 | 24 | 33 |
| 613 | 41 | -6 | 37 | 20 | 28 | 18 | 34 | 15 | 657 | 41 | -6 | 37 | 20 | 28 | 18 | 34 | 15 |
| 614 | 38 | 1 | 28 | 34 | 26 | 25 | 30 | 20 | 658 | 38 | 1 | 28 | 34 | 26 | 25 | 30 | 20 |
| 615 | 41 | 17 | 17 | 70 | 29 | 41 | 22 | 73 | 659 | 41 | 17 | 17 | 70 | 29 | 41 | 22 | 73 |

Insert a new Table 9-21-1 as follows.

Table 9-5-1 – Values of variables m and n for ctxIdx from 952 to 1011

| ctxIdx | I and SI slices |  | Value of cabac_init_idc |  |  |  |  |  | ctxIdx | I and SI slices |  | Value of cabac_init_idc |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 |  | 1 |  | 2 |  |  |  |  | 0 |  | 1 |  | 2 |  |
|  | m | n | m | n | m | n | m | n |  | m | n | m | n | m | n | m | n |
| 952 | -3 | 71 | -6 | 76 | -23 | 112 | -24 | 115 | 982 | -3 | 71 | -6 | 76 | -23 | 112 | -24 | 115 |
| 953 | -6 | 42 | -2 | 44 | -15 | 71 | -22 | 82 | 983 | -6 | 42 | -2 | 44 | -15 | 71 | -22 | 82 |
| 954 | -5 | 50 | 0 | 45 | -7 | 61 | -9 | 62 | 984 | -5 | 50 | 0 | 45 | -7 | 61 | -9 | 62 |
| 955 | -3 | 54 | 0 | 52 | 0 | 53 | 0 | 53 | 985 | -3 | 54 | 0 | 52 | 0 | 53 | 0 | 53 |
| 956 | -2 | 62 | -3 | 64 | -5 | 66 | 0 | 59 | 986 | -2 | 62 | -3 | 64 | -5 | 66 | 0 | 59 |
| 957 | 0 | 58 | -2 | 59 | -11 | 77 | -14 | 85 | 987 | 0 | 58 | -2 | 59 | -11 | 77 | -14 | 85 |
| 958 | 1 | 63 | -4 | 70 | -9 | 80 | -13 | 89 | 988 | 1 | 63 | -4 | 70 | -9 | 80 | -13 | 89 |
| 959 | -2 | 72 | -4 | 75 | -9 | 84 | -13 | 94 | 989 | -2 | 72 | -4 | 75 | -9 | 84 | -13 | 94 |
| 960 | -1 | 74 | -8 | 82 | -10 | 87 | -11 | 92 | 990 | -1 | 74 | -8 | 82 | -10 | 87 | -11 | 92 |
| 961 | -9 | 91 | -17 | 102 | -34 | 127 | -29 | 127 | 991 | -9 | 91 | -17 | 102 | -34 | 127 | -29 | 127 |
| 962 | -5 | 67 | -9 | 77 | -21 | 101 | -21 | 100 | 992 | -5 | 67 | -9 | 77 | -21 | 101 | -21 | 100 |
| 963 | -5 | 27 | 3 | 24 | -3 | 39 | -14 | 57 | 993 | -5 | 27 | 3 | 24 | -3 | 39 | -14 | 57 |
| 964 | -3 | 39 | 0 | 42 | -5 | 53 | -12 | 67 | 994 | -3 | 39 | 0 | 42 | -5 | 53 | -12 | 67 |
| 965 | -2 | 44 | 0 | 48 | -7 | 61 | -11 | 71 | 995 | -2 | 44 | 0 | 48 | -7 | 61 | -11 | 71 |
| 966 | 0 | 46 | 0 | 55 | -11 | 75 | -10 | 77 | 996 | 0 | 46 | 0 | 55 | -11 | 75 | -10 | 77 |
| 967 | -16 | 64 | -6 | 59 | -15 | 77 | -21 | 85 | 997 | -16 | 64 | -6 | 59 | -15 | 77 | -21 | 85 |
| 968 | -8 | 68 | -7 | 71 | -17 | 91 | -16 | 88 | 998 | -8 | 68 | -7 | 71 | -17 | 91 | -16 | 88 |
| 969 | -10 | 78 | -12 | 83 | -25 | 107 | -23 | 104 | 999 | -10 | 78 | -12 | 83 | -25 | 107 | -23 | 104 |
| 970 | -6 | 77 | -11 | 87 | -25 | 111 | -15 | 98 | 1000 | -6 | 77 | -11 | 87 | -25 | 111 | -15 | 98 |
| 971 | -10 | 86 | -30 | 119 | -28 | 122 | -37 | 127 | 1001 | -10 | 86 | -30 | 119 | -28 | 122 | -37 | 127 |
| 972 | -12 | 92 | 1 | 58 | -11 | 76 | -10 | 82 | 1002 | -12 | 92 | 1 | 58 | -11 | 76 | -10 | 82 |
| 973 | -15 | 55 | -3 | 29 | -10 | 44 | -8 | 48 | 1003 | -15 | 55 | -3 | 29 | -10 | 44 | -8 | 48 |
| 974 | -10 | 60 | -1 | 36 | -10 | 52 | -8 | 61 | 1004 | -10 | 60 | -1 | 36 | -10 | 52 | -8 | 61 |
| 975 | -6 | 62 | 1 | 38 | -10 | 57 | -8 | 66 | 1005 | -6 | 62 | 1 | 38 | -10 | 57 | -8 | 66 |
| 976 | -4 | 65 | 2 | 43 | -9 | 58 | -7 | 70 | 1006 | -4 | 65 | 2 | 43 | -9 | 58 | -7 | 70 |
| 977 | -12 | 73 | -6 | 55 | -16 | 72 | -14 | 75 | 1007 | -12 | 73 | -6 | 55 | -16 | 72 | -14 | 75 |
| 978 | -8 | 76 | 0 | 58 | -7 | 69 | -10 | 79 | 1008 | -8 | 76 | 0 | 58 | -7 | 69 | -10 | 79 |
| 979 | -7 | 80 | 0 | 64 | -4 | 69 | -9 | 83 | 1009 | -7 | 80 | 0 | 64 | -4 | 69 | -9 | 83 |
| 980 | -9 | 88 | -3 | 74 | -5 | 74 | -12 | 92 | 1010 | -9 | 88 | -3 | 74 | -5 | 74 | -12 | 92 |
| 981 | -17 | 110 | -10 | 90 | -9 | 86 | -18 | 108 | 1011 | -17 | 110 | -10 | 90 | -9 | 86 | -18 | 108 |

Insert a new Table 9-22-1 as follows.

Table 9-6-1 – Values of variables m and n for ctxIdx from 776 to 863

| ctxIdx | I and SI slices |  | Value of cabac_init_idc |  |  |  |  |  | ctxIdx | I and SI slices |  | Value of cabac_init_idc |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 |  | 1 |  | 2 |  |  |  |  | 0 |  | 1 |  | 2 |  |
|  | m | n | m | n | m | n | m | n |  | m | n | m | n | m | n | m | n |
| 776 | -6 | 93 | -13 | 106 | -21 | 126 | -22 | 127 | 820 | -6 | 93 | -13 | 106 | -21 | 126 | -22 | 127 |
| 777 | -6 | 84 | -16 | 106 | -23 | 124 | -25 | 127 | 821 | -6 | 84 | -16 | 106 | -23 | 124 | -25 | 127 |
| 778 | -8 | 79 | -10 | 87 | -20 | 110 | -25 | 120 | 822 | -8 | 79 | -10 | 87 | -20 | 110 | -25 | 120 |
| 779 | 0 | 66 | -21 | 114 | -26 | 126 | -27 | 127 | 823 | 0 | 66 | -21 | 114 | -26 | 126 | -27 | 127 |
| 780 | -1 | 71 | -18 | 110 | -25 | 124 | -19 | 114 | 824 | -1 | 71 | -18 | 110 | -25 | 124 | -19 | 114 |
| 781 | 0 | 62 | -14 | 98 | -17 | 105 | -23 | 117 | 825 | 0 | 62 | -14 | 98 | -17 | 105 | -23 | 117 |
| 782 | -2 | 60 | -22 | 110 | -27 | 121 | -25 | 118 | 826 | -2 | 60 | -22 | 110 | -27 | 121 | -25 | 118 |
| 783 | -2 | 59 | -21 | 106 | -27 | 117 | -26 | 117 | 827 | -2 | 59 | -21 | 106 | -27 | 117 | -26 | 117 |
| 784 | -5 | 75 | -18 | 103 | -17 | 102 | -24 | 113 | 828 | -5 | 75 | -18 | 103 | -17 | 102 | -24 | 113 |
| 785 | -3 | 62 | -21 | 107 | -26 | 117 | -28 | 118 | 829 | -3 | 62 | -21 | 107 | -26 | 117 | -28 | 118 |
| 786 | -4 | 58 | -23 | 108 | -27 | 116 | -31 | 120 | 830 | -4 | 58 | -23 | 108 | -27 | 116 | -31 | 120 |
| 787 | -9 | 66 | -26 | 112 | -33 | 122 | -37 | 124 | 831 | -9 | 66 | -26 | 112 | -33 | 122 | -37 | 124 |
| 788 | -1 | 79 | -10 | 96 | -10 | 95 | -10 | 94 | 832 | -1 | 79 | -10 | 96 | -10 | 95 | -10 | 94 |
| 789 | 0 | 71 | -12 | 95 | -14 | 100 | -15 | 102 | 833 | 0 | 71 | -12 | 95 | -14 | 100 | -15 | 102 |
| 790 | 3 | 68 | -5 | 91 | -8 | 95 | -10 | 99 | 834 | 3 | 68 | -5 | 91 | -8 | 95 | -10 | 99 |
| 791 | 10 | 44 | -9 | 93 | -17 | 111 | -13 | 106 | 835 | 10 | 44 | -9 | 93 | -17 | 111 | -13 | 106 |
| 792 | -7 | 62 | -22 | 94 | -28 | 114 | -50 | 127 | 836 | -7 | 62 | -22 | 94 | -28 | 114 | -50 | 127 |
| 793 | 15 | 36 | -5 | 86 | -6 | 89 | -5 | 92 | 837 | 15 | 36 | -5 | 86 | -6 | 89 | -5 | 92 |
| 794 | 14 | 40 | 9 | 67 | -2 | 80 | 17 | 57 | 838 | 14 | 40 | 9 | 67 | -2 | 80 | 17 | 57 |
| 795 | 16 | 27 | -4 | 80 | -4 | 82 | -5 | 86 | 839 | 16 | 27 | -4 | 80 | -4 | 82 | -5 | 86 |
| 796 | 12 | 29 | -10 | 85 | -9 | 85 | -13 | 94 | 840 | 12 | 29 | -10 | 85 | -9 | 85 | -13 | 94 |
| 797 | 1 | 44 | -1 | 70 | -8 | 81 | -12 | 91 | 841 | 1 | 44 | -1 | 70 | -8 | 81 | -12 | 91 |
| 798 | 20 | 36 | 7 | 60 | -1 | 72 | -2 | 77 | 842 | 20 | 36 | 7 | 60 | -1 | 72 | -2 | 77 |
| 799 | 18 | 32 | 9 | 58 | 5 | 64 | 0 | 71 | 843 | 18 | 32 | 9 | 58 | 5 | 64 | 0 | 71 |
| 800 | 5 | 42 | 5 | 61 | 1 | 67 | -1 | 73 | 844 | 5 | 42 | 5 | 61 | 1 | 67 | -1 | 73 |
| 801 | 1 | 48 | 12 | 50 | 9 | 56 | 4 | 64 | 845 | 1 | 48 | 12 | 50 | 9 | 56 | 4 | 64 |
| 802 | 10 | 62 | 15 | 50 | 0 | 69 | -7 | 81 | 846 | 10 | 62 | 15 | 50 | 0 | 69 | -7 | 81 |
| 803 | 17 | 46 | 18 | 49 | 1 | 69 | 5 | 64 | 847 | 17 | 46 | 18 | 49 | 1 | 69 | 5 | 64 |
| 804 | 9 | 64 | 17 | 54 | 7 | 69 | 15 | 57 | 848 | 9 | 64 | 17 | 54 | 7 | 69 | 15 | 57 |
| 805 | -12 | 104 | 10 | 41 | -7 | 69 | 1 | 67 | 849 | -12 | 104 | 10 | 41 | -7 | 69 | 1 | 67 |
| 806 | -11 | 97 | 7 | 46 | -6 | 67 | 0 | 68 | 850 | -11 | 97 | 7 | 46 | -6 | 67 | 0 | 68 |
| 807 | -16 | 96 | -1 | 51 | -16 | 77 | -10 | 67 | 851 | -16 | 96 | -1 | 51 | -16 | 77 | -10 | 67 |
| 808 | -7 | 88 | 7 | 49 | -2 | 64 | 1 | 68 | 852 | -7 | 88 | 7 | 49 | -2 | 64 | 1 | 68 |
| 809 | -8 | 85 | 8 | 52 | 2 | 61 | 0 | 77 | 853 | -8 | 85 | 8 | 52 | 2 | 61 | 0 | 77 |
| 810 | -7 | 85 | 9 | 41 | -6 | 67 | 2 | 64 | 854 | -7 | 85 | 9 | 41 | -6 | 67 | 2 | 64 |
| 811 | -9 | 85 | 6 | 47 | -3 | 64 | 0 | 68 | 855 | -9 | 85 | 6 | 47 | -3 | 64 | 0 | 68 |
| 812 | -13 | 88 | 2 | 55 | 2 | 57 | -5 | 78 | 856 | -13 | 88 | 2 | 55 | 2 | 57 | -5 | 78 |
| 813 | 4 | 66 | 13 | 41 | -3 | 65 | 7 | 55 | 857 | 4 | 66 | 13 | 41 | -3 | 65 | 7 | 55 |
| 814 | -3 | 77 | 10 | 44 | -3 | 66 | 5 | 59 | 858 | -3 | 77 | 10 | 44 | -3 | 66 | 5 | 59 |
| 815 | -3 | 76 | 6 | 50 | 0 | 62 | 2 | 65 | 859 | -3 | 76 | 6 | 50 | 0 | 62 | 2 | 65 |
| 816 | -6 | 76 | 5 | 53 | 9 | 51 | 14 | 54 | 860 | -6 | 76 | 5 | 53 | 9 | 51 | 14 | 54 |
| 817 | 10 | 58 | 13 | 49 | -1 | 66 | 15 | 44 | 861 | 10 | 58 | 13 | 49 | -1 | 66 | 15 | 44 |
| 818 | -1 | 76 | 4 | 63 | -2 | 71 | 5 | 60 | 862 | -1 | 76 | 4 | 63 | -2 | 71 | 5 | 60 |
| 819 | -1 | 83 | 6 | 64 | -2 | 75 | 2 | 70 | 863 | -1 | 83 | 6 | 64 | -2 | 75 | 2 | 70 |

Insert a new Table 9-23-1 as follows.

Table 9-7 – Values of variables m and n for ctxIdx from 864 to 951

| ctxIdx | I and SI slices |  | Value of cabac_init_idc |  |  |  |  |  | ctxIdx | I and SI slices |  | Value of cabac_init_idc |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 |  | 1 |  | 2 |  |  |  |  | 0 |  | 1 |  | 2 |  |
|  | m | n | m | n | m | n | m | n |  | m | n | m | n | m | n | m | n |
| 864 | 15 | 6 | 14 | 11 | 19 | -6 | 17 | -13 | 908 | 15 | 6 | 14 | 11 | 19 | -6 | 17 | -13 |
| 865 | 6 | 19 | 11 | 14 | 18 | -6 | 16 | -9 | 909 | 6 | 19 | 11 | 14 | 18 | -6 | 16 | -9 |
| 866 | 7 | 16 | 9 | 11 | 14 | 0 | 17 | -12 | 910 | 7 | 16 | 9 | 11 | 14 | 0 | 17 | -12 |
| 867 | 12 | 14 | 18 | 11 | 26 | -12 | 27 | -21 | 911 | 12 | 14 | 18 | 11 | 26 | -12 | 27 | -21 |
| 868 | 18 | 13 | 21 | 9 | 31 | -16 | 37 | -30 | 912 | 18 | 13 | 21 | 9 | 31 | -16 | 37 | -30 |
| 869 | 13 | 11 | 23 | -2 | 33 | -25 | 41 | -40 | 913 | 13 | 11 | 23 | -2 | 33 | -25 | 41 | -40 |
| 870 | 13 | 15 | 32 | -15 | 33 | -22 | 42 | -41 | 914 | 13 | 15 | 32 | -15 | 33 | -22 | 42 | -41 |
| 871 | 15 | 16 | 32 | -15 | 37 | -28 | 48 | -47 | 915 | 15 | 16 | 32 | -15 | 37 | -28 | 48 | -47 |
| 872 | 12 | 23 | 34 | -21 | 39 | -30 | 39 | -32 | 916 | 12 | 23 | 34 | -21 | 39 | -30 | 39 | -32 |
| 873 | 13 | 23 | 39 | -23 | 42 | -30 | 46 | -40 | 917 | 13 | 23 | 39 | -23 | 42 | -30 | 46 | -40 |
| 874 | 15 | 20 | 42 | -33 | 47 | -42 | 52 | -51 | 918 | 15 | 20 | 42 | -33 | 47 | -42 | 52 | -51 |
| 875 | 14 | 26 | 41 | -31 | 45 | -36 | 46 | -41 | 919 | 14 | 26 | 41 | -31 | 45 | -36 | 46 | -41 |
| 876 | 14 | 44 | 46 | -28 | 49 | -34 | 52 | -39 | 920 | 14 | 44 | 46 | -28 | 49 | -34 | 52 | -39 |
| 877 | 17 | 40 | 38 | -12 | 41 | -17 | 43 | -19 | 921 | 17 | 40 | 38 | -12 | 41 | -17 | 43 | -19 |
| 878 | 17 | 47 | 21 | 29 | 32 | 9 | 32 | 11 | 922 | 17 | 47 | 21 | 29 | 32 | 9 | 32 | 11 |
| 879 | 24 | 17 | 45 | -24 | 69 | -71 | 61 | -55 | 923 | 24 | 17 | 45 | -24 | 69 | -71 | 61 | -55 |
| 880 | 21 | 21 | 53 | -45 | 63 | -63 | 56 | -46 | 924 | 21 | 21 | 53 | -45 | 63 | -63 | 56 | -46 |
| 881 | 25 | 22 | 48 | -26 | 66 | -64 | 62 | -50 | 925 | 25 | 22 | 48 | -26 | 66 | -64 | 62 | -50 |
| 882 | 31 | 27 | 65 | -43 | 77 | -74 | 81 | -67 | 926 | 31 | 27 | 65 | -43 | 77 | -74 | 81 | -67 |
| 883 | 22 | 29 | 43 | -19 | 54 | -39 | 45 | -20 | 927 | 22 | 29 | 43 | -19 | 54 | -39 | 45 | -20 |
| 884 | 19 | 35 | 39 | -10 | 52 | -35 | 35 | -2 | 928 | 19 | 35 | 39 | -10 | 52 | -35 | 35 | -2 |
| 885 | 14 | 50 | 30 | 9 | 41 | -10 | 28 | 15 | 929 | 14 | 50 | 30 | 9 | 41 | -10 | 28 | 15 |
| 886 | 10 | 57 | 18 | 26 | 36 | 0 | 34 | 1 | 930 | 10 | 57 | 18 | 26 | 36 | 0 | 34 | 1 |
| 887 | 7 | 63 | 20 | 27 | 40 | -1 | 39 | 1 | 931 | 7 | 63 | 20 | 27 | 40 | -1 | 39 | 1 |
| 888 | -2 | 77 | 0 | 57 | 30 | 14 | 30 | 17 | 932 | -2 | 77 | 0 | 57 | 30 | 14 | 30 | 17 |
| 889 | -4 | 82 | -14 | 82 | 28 | 26 | 20 | 38 | 933 | -4 | 82 | -14 | 82 | 28 | 26 | 20 | 38 |
| 890 | -3 | 94 | -5 | 75 | 23 | 37 | 18 | 45 | 934 | -3 | 94 | -5 | 75 | 23 | 37 | 18 | 45 |
| 891 | 9 | 69 | -19 | 97 | 12 | 55 | 15 | 54 | 935 | 9 | 69 | -19 | 97 | 12 | 55 | 15 | 54 |
| 892 | -12 | 109 | -35 | 125 | 11 | 65 | 0 | 79 | 936 | -12 | 109 | -35 | 125 | 11 | 65 | 0 | 79 |
| 893 | 36 | -35 | 27 | 0 | 37 | -33 | 36 | -16 | 937 | 36 | -35 | 27 | 0 | 37 | -33 | 36 | -16 |
| 894 | 36 | -34 | 28 | 0 | 39 | -36 | 37 | -14 | 938 | 36 | -34 | 28 | 0 | 39 | -36 | 37 | -14 |
| 895 | 32 | -26 | 31 | -4 | 40 | -37 | 37 | -17 | 939 | 32 | -26 | 31 | -4 | 40 | -37 | 37 | -17 |
| 896 | 37 | -30 | 27 | 6 | 38 | -30 | 32 | 1 | 940 | 37 | -30 | 27 | 6 | 38 | -30 | 32 | 1 |
| 897 | 44 | -32 | 34 | 8 | 46 | -33 | 34 | 15 | 941 | 44 | -32 | 34 | 8 | 46 | -33 | 34 | 15 |
| 898 | 34 | -18 | 30 | 10 | 42 | -30 | 29 | 15 | 942 | 34 | -18 | 30 | 10 | 42 | -30 | 29 | 15 |
| 899 | 34 | -15 | 24 | 22 | 40 | -24 | 24 | 25 | 943 | 34 | -15 | 24 | 22 | 40 | -24 | 24 | 25 |
| 900 | 40 | -15 | 33 | 19 | 49 | -29 | 34 | 22 | 944 | 40 | -15 | 33 | 19 | 49 | -29 | 34 | 22 |
| 901 | 33 | -7 | 22 | 32 | 38 | -12 | 31 | 16 | 945 | 33 | -7 | 22 | 32 | 38 | -12 | 31 | 16 |
| 902 | 35 | -5 | 26 | 31 | 40 | -10 | 35 | 18 | 946 | 35 | -5 | 26 | 31 | 40 | -10 | 35 | 18 |
| 903 | 33 | 0 | 21 | 41 | 38 | -3 | 31 | 28 | 947 | 33 | 0 | 21 | 41 | 38 | -3 | 31 | 28 |
| 904 | 38 | 2 | 26 | 44 | 46 | -5 | 33 | 41 | 948 | 38 | 2 | 26 | 44 | 46 | -5 | 33 | 41 |
| 905 | 33 | 13 | 23 | 47 | 31 | 20 | 36 | 28 | 949 | 33 | 13 | 23 | 47 | 31 | 20 | 36 | 28 |
| 906 | 23 | 35 | 16 | 65 | 29 | 30 | 27 | 47 | 950 | 23 | 35 | 16 | 65 | 29 | 30 | 27 | 47 |
| 907 | 13 | 58 | 14 | 71 | 25 | 44 | 21 | 62 | 951 | 13 | 58 | 14 | 71 | 25 | 44 | 21 | 62 |

Insert a new Table 9-24-1 as follows.

Table 9-8 -1– Values of variables m and n for ctxIdx from 660 to 717

| ctxIdx | I slices |  | Value of cabac_init_idc |  |  |  |  |  | ctxIdx | I slices |  | Value of cabac_init_idc |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 |  | 1 |  | 2 |  |  |  |  | 0 |  | 1 |  | 2 |  |
|  | m | n | m | n | m | n | m | n |  | m | n | m | n | m | n | m | n |
| 660 | -17 | 120 | -4 | 79 | -5 | 85 | -3 | 78 | 689 | 2 | 59 | 2 | 59 | 2 | 58 | -11 | 68 |
| 661 | -20 | 112 | -7 | 71 | -6 | 81 | -8 | 74 | 690 | 23 | -13 | 9 | -2 | 17 | -10 | 9 | -2 |
| 662 | -18 | 114 | -5 | 69 | -10 | 77 | -9 | 72 | 691 | 26 | -13 | 26 | -9 | 32 | -13 | 30 | -10 |
| 663 | -11 | 85 | -9 | 70 | -7 | 81 | -10 | 72 | 692 | 40 | -15 | 33 | -9 | 42 | -9 | 31 | -4 |
| 664 | -15 | 92 | -8 | 66 | -17 | 80 | -18 | 75 | 693 | 49 | -14 | 39 | -7 | 49 | -5 | 33 | -1 |
| 665 | -14 | 89 | -10 | 68 | -18 | 73 | -12 | 71 | 694 | 44 | 3 | 41 | -2 | 53 | 0 | 33 | 7 |
| 666 | -26 | 71 | -19 | 73 | -4 | 74 | -11 | 63 | 695 | 45 | 6 | 45 | 3 | 64 | 3 | 31 | 12 |
| 667 | -15 | 81 | -12 | 69 | -10 | 83 | -5 | 70 | 696 | 44 | 34 | 49 | 9 | 68 | 10 | 37 | 23 |
| 668 | -14 | 80 | -16 | 70 | -9 | 71 | -17 | 75 | 697 | 33 | 54 | 45 | 27 | 66 | 27 | 31 | 38 |
| 669 | 0 | 68 | -15 | 67 | -9 | 67 | -14 | 72 | 698 | 19 | 82 | 36 | 59 | 47 | 57 | 20 | 64 |
| 670 | -14 | 70 | -20 | 62 | -1 | 61 | -16 | 67 | 699 | 21 | -10 | 21 | -13 | 17 | -10 | 9 | -2 |
| 671 | -24 | 56 | -19 | 70 | -8 | 66 | -8 | 53 | 700 | 24 | -11 | 33 | -14 | 32 | -13 | 30 | -10 |
| 672 | -23 | 68 | -16 | 66 | -14 | 66 | -14 | 59 | 701 | 28 | -8 | 39 | -7 | 42 | -9 | 31 | -4 |
| 673 | -24 | 50 | -22 | 65 | 0 | 59 | -9 | 52 | 702 | 28 | -1 | 46 | -2 | 49 | -5 | 33 | -1 |
| 674 | -11 | 74 | -20 | 63 | 2 | 59 | -11 | 68 | 703 | 29 | 3 | 51 | 2 | 53 | 0 | 33 | 7 |
| 675 | -14 | 106 | -5 | 85 | -3 | 81 | -3 | 78 | 704 | 29 | 9 | 60 | 6 | 64 | 3 | 31 | 12 |
| 676 | -13 | 97 | -6 | 81 | -3 | 76 | -8 | 74 | 705 | 35 | 20 | 61 | 17 | 68 | 10 | 37 | 23 |
| 677 | -15 | 90 | -10 | 77 | -7 | 72 | -9 | 72 | 706 | 29 | 36 | 55 | 34 | 66 | 27 | 31 | 38 |
| 678 | -12 | 90 | -7 | 81 | -6 | 78 | -10 | 72 | 707 | 14 | 67 | 42 | 62 | 47 | 57 | 20 | 64 |
| 679 | -18 | 88 | -17 | 80 | -12 | 72 | -18 | 75 | 708 | -3 | 75 | -6 | 66 | -5 | 71 | -9 | 71 |
| 680 | -10 | 73 | -18 | 73 | -14 | 68 | -12 | 71 | 709 | -1 | 23 | -7 | 35 | 0 | 24 | -7 | 37 |
| 681 | -9 | 79 | -4 | 74 | -3 | 70 | -11 | 63 | 710 | 1 | 34 | -7 | 42 | -1 | 36 | -8 | 44 |
| 682 | -14 | 86 | -10 | 83 | -6 | 76 | -5 | 70 | 711 | 1 | 43 | -8 | 45 | -2 | 42 | -11 | 49 |
| 683 | -10 | 73 | -9 | 71 | -5 | 66 | -17 | 75 | 712 | 0 | 54 | -5 | 48 | -2 | 52 | -10 | 56 |
| 684 | -10 | 70 | -9 | 67 | -5 | 62 | -14 | 72 | 713 | -2 | 55 | -12 | 56 | -9 | 57 | -12 | 59 |
| 685 | -10 | 69 | -1 | 61 | 0 | 57 | -16 | 67 | 714 | 0 | 61 | -6 | 60 | -6 | 63 | -8 | 63 |
| 686 | -5 | 66 | -8 | 66 | -4 | 61 | -8 | 53 | 715 | 1 | 64 | -5 | 62 | -4 | 65 | -9 | 67 |
| 687 | -9 | 64 | -14 | 66 | -9 | 60 | -14 | 59 | 716 | 0 | 68 | -8 | 66 | -4 | 67 | -6 | 68 |
| 688 | -5 | 58 | 0 | 59 | 1 | 54 | -9 | 52 | 717 | -9 | 92 | -8 | 76 | -7 | 82 | -10 | 79 |

Insert a new Table 9-24-2 as following.

Table 9-9 -2– Values of variables m and n for ctxIdx from 718 to 775

| ctxIdx | I slices |  | Value of cabac_init_idc |  |  |  |  |  | ctxIdx | I slices |  | Value of cabac_init_idc |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 |  | 1 |  | 2 |  |  |  |  | 0 |  | 1 |  | 2 |  |
|  | m | n | m | n | m | n | m | n |  | m | n | m | n | m | n | m | n |
| 718 | -17 | 120 | -4 | 79 | -5 | 85 | -3 | 78 | 747 | 2 | 59 | 2 | 59 | 2 | 58 | -11 | 68 |
| 719 | -20 | 112 | -7 | 71 | -6 | 81 | -8 | 74 | 748 | 23 | -13 | 9 | -2 | 17 | -10 | 9 | -2 |
| 720 | -18 | 114 | -5 | 69 | -10 | 77 | -9 | 72 | 749 | 26 | -13 | 26 | -9 | 32 | -13 | 30 | -10 |
| 721 | -11 | 85 | -9 | 70 | -7 | 81 | -10 | 72 | 750 | 40 | -15 | 33 | -9 | 42 | -9 | 31 | -4 |
| 722 | -15 | 92 | -8 | 66 | -17 | 80 | -18 | 75 | 751 | 49 | -14 | 39 | -7 | 49 | -5 | 33 | -1 |
| 723 | -14 | 89 | -10 | 68 | -18 | 73 | -12 | 71 | 752 | 44 | 3 | 41 | -2 | 53 | 0 | 33 | 7 |
| 724 | -26 | 71 | -19 | 73 | -4 | 74 | -11 | 63 | 753 | 45 | 6 | 45 | 3 | 64 | 3 | 31 | 12 |
| 725 | -15 | 81 | -12 | 69 | -10 | 83 | -5 | 70 | 754 | 44 | 34 | 49 | 9 | 68 | 10 | 37 | 23 |
| 726 | -14 | 80 | -16 | 70 | -9 | 71 | -17 | 75 | 755 | 33 | 54 | 45 | 27 | 66 | 27 | 31 | 38 |
| 727 | 0 | 68 | -15 | 67 | -9 | 67 | -14 | 72 | 756 | 19 | 82 | 36 | 59 | 47 | 57 | 20 | 64 |
| 728 | -14 | 70 | -20 | 62 | -1 | 61 | -16 | 67 | 757 | 21 | -10 | 21 | -13 | 17 | -10 | 9 | -2 |
| 729 | -24 | 56 | -19 | 70 | -8 | 66 | -8 | 53 | 758 | 24 | -11 | 33 | -14 | 32 | -13 | 30 | -10 |
| 730 | -23 | 68 | -16 | 66 | -14 | 66 | -14 | 59 | 759 | 28 | -8 | 39 | -7 | 42 | -9 | 31 | -4 |
| 731 | -24 | 50 | -22 | 65 | 0 | 59 | -9 | 52 | 760 | 28 | -1 | 46 | -2 | 49 | -5 | 33 | -1 |
| 732 | -11 | 74 | -20 | 63 | 2 | 59 | -11 | 68 | 761 | 29 | 3 | 51 | 2 | 53 | 0 | 33 | 7 |
| 733 | -14 | 106 | -5 | 85 | -3 | 81 | -3 | 78 | 762 | 29 | 9 | 60 | 6 | 64 | 3 | 31 | 12 |
| 734 | -13 | 97 | -6 | 81 | -3 | 76 | -8 | 74 | 763 | 35 | 20 | 61 | 17 | 68 | 10 | 37 | 23 |
| 735 | -15 | 90 | -10 | 77 | -7 | 72 | -9 | 72 | 764 | 29 | 36 | 55 | 34 | 66 | 27 | 31 | 38 |
| 736 | -12 | 90 | -7 | 81 | -6 | 78 | -10 | 72 | 765 | 14 | 67 | 42 | 62 | 47 | 57 | 20 | 64 |
| 737 | -18 | 88 | -17 | 80 | -12 | 72 | -18 | 75 | 766 | -3 | 75 | -6 | 66 | -5 | 71 | -9 | 71 |
| 738 | -10 | 73 | -18 | 73 | -14 | 68 | -12 | 71 | 767 | -1 | 23 | -7 | 35 | 0 | 24 | -7 | 37 |
| 739 | -9 | 79 | -4 | 74 | -3 | 70 | -11 | 63 | 768 | 1 | 34 | -7 | 42 | -1 | 36 | -8 | 44 |
| 740 | -14 | 86 | -10 | 83 | -6 | 76 | -5 | 70 | 769 | 1 | 43 | -8 | 45 | -2 | 42 | -11 | 49 |
| 741 | -10 | 73 | -9 | 71 | -5 | 66 | -17 | 75 | 770 | 0 | 54 | -5 | 48 | -2 | 52 | -10 | 56 |
| 742 | -10 | 70 | -9 | 67 | -5 | 62 | -14 | 72 | 771 | -2 | 55 | -12 | 56 | -9 | 57 | -12 | 59 |
| 743 | -10 | 69 | -1 | 61 | 0 | 57 | -16 | 67 | 772 | 0 | 61 | -6 | 60 | -6 | 63 | -8 | 63 |
| 744 | -5 | 66 | -8 | 66 | -4 | 61 | -8 | 53 | 773 | 1 | 64 | -5 | 62 | -4 | 65 | -9 | 67 |
| 745 | -9 | 64 | -14 | 66 | -9 | 60 | -14 | 59 | 774 | 0 | 68 | -8 | 66 | -4 | 67 | -6 | 68 |
| 746 | -5 | 58 | 0 | 59 | 1 | 54 | -9 | 52 | 775 | -9 | 92 | -8 | 76 | -7 | 82 | -10 | 79 |

# Subclause 9.3.2 “Binarization process”

Replace Table 9-25 with the following.

Table 9-10 – Syntax elements and associated types of binarization, maxBinIdxCtx, and ctxIdxOffset

| Syntax element | Type of binarization | maxBinIdxCtx | ctxIdxOffset |
| --- | --- | --- | --- |
| mb_type (SI slices only) | prefix and suffix as specified in subclause Error: Reference source not found | prefix: 0 suffix: 6 | prefix: 0 suffix: 3 |
| mb_type (I slices only) | as specified in subclause Error: Reference source not found | 6 | 3 |
| mb_skip_flag (P, SP slices only) | FL, cMax=1 | 0 | 11 |
| mb_type (P, SP slices only) | prefix and suffix as specified in subclause Error: Reference source not found | prefix: 2 suffix: 5 | prefix: 14 suffix: 17 |
| sub_mb_type (P, SP slices only) | as specified in subclause Error: Reference source not found | 2 | 21 |
| mb_skip_flag (B slices only) | FL, cMax=1 | 0 | 24 |
| mb_type (B slices only) | prefix and suffix as specified in subclause Error: Reference source not found | prefix: 3 suffix: 5 | prefix: 27 suffix: 32 |
| sub_mb_type (B slices only) | as specified in subclause Error: Reference source not found | 3 | 36 |
| mvd_l0[ ][ ][ 0 ], mvd_l1[ ][ ][ 0 ] | prefix and suffix as given by UEG3 with signedValFlag=1, uCoff=9 | prefix: 4 suffix: na | prefix: 40 suffix: na (uses DecodeBypass) |
| mvd_l0[ ][ ][ 1 ], mvd_l1[ ][ ][ 1 ] |  | prefix: 4 suffix: na | prefix: 47 suffix: na (uses DecodeBypass) |
| ref_idx_l0, ref_idx_l1 | U | 2 | 54 |
| mb_qp_delta | as specified in subclause Error: Reference source not found | 2 | 60 |
| intra_chroma_pred_mode | TU, cMax=3 | 1 | 64 |
| prev_intra4x4_pred_mode_flag, prev_intra8x8_pred_mode_flag | FL, cMax=1 | 0 | 68 |
| rem_intra4x4_pred_mode, rem_intra8x8_pred_mode | FL, cMax=7 | 0 | 69 |
| mb_field_decoding_flag | FL, cMax=1 | 0 | 70 |
| coded_block_pattern | prefix and suffix as specified in subclause Error: Reference source not found | prefix: 3 suffix: 1 | prefix: 73 suffix: 77 |
| coded_block_flag (ctxBlockCat  5) | FL, cMax=1 | 0 | 85 |
| significant_coeff_flag (frame coded blocks with ctxBlockCat < 5) | FL, cMax=1 | 0 | 105 |
| last_significant_coeff_flag (frame coded blocks with ctxBlockCat < 5) | FL, cMax=1 | 0 | 166 |
| coeff_abs_level_minus1 (blocks with ctxBlockCat < 5) | prefix and suffix as given by UEG0 with signedValFlag=0, uCoff=14 | prefix: 1 suffix: na | prefix: 227 suffix: na, (uses DecodeBypass) |
| coeff_sign_flag | FL, cMax=1 | 0 | na, (uses DecodeBypass) |
| end_of_slice_flag | FL, cMax=1 | 0 | 276 |
| significant_coeff_flag (field coded blocks with ctxBlockCat < 5) | FL, cMax=1 | 0 | 277 |
| last_significant_coeff_flag (field coded blocks with ctxBlockCat < 5) | FL, cMax=1 | 0 | 338 |
| transform_size_8x8_flag | FL, cMax=1 | 0 | 399 |
| significant_coeff_flag (frame coded blocks with ctxBlockCat = = 5) | FL, cMax=1 | 0 | 402 |
| last_significant_coeff_flag (frame coded blocks with ctxBlockCat = = 5) | FL, cMax=1 | 0 | 417 |
| coeff_abs_level_minus1 (blocks with ctxBlockCat = = 5) | prefix and suffix as given by UEG0 with signedValFlag=0, uCoff=14 | prefix: 1 suffix: na | prefix: 426 suffix: na, (uses DecodeBypass) |
| significant_coeff_flag (field coded blocks with ctxBlockCat = = 5) | FL, cMax=1 | 0 | 436 |
| last_significant_coeff_flag (field coded blocks with ctxBlockCat = = 5) | FL, cMax=1 | 0 | 451 |
| coded_block_flag (5 < ctxBlockCat  9) | FL, cMax=1 | 0 | 460 |
| coded_block_flag (9 < ctxBlockCat  13) | FL, cMax=1 | 0 | 472 |
| significant_coeff_flag (5<frame coded blocks with ctxBlockCat <9 ) | FL, cMax=1 | 0 | 484 |
| significant_coeff_flag (9<frame coded blocks with ctxBlockCat <13 ) | FL, cMax=1 | 0 | 528 |
| last_significant_coeff_flag (5<frame coded blocks with ctxBlockCat < 9) | FL, cMax=1 | 0 | 572 |
| last_significant_coeff_flag (9<frame coded blocks with ctxBlockCat < 13) | FL, cMax=1 | 0 | 616 |
| coeff_abs_level_minus1 (5<blocks with ctxBlockCat < 9) | prefix and suffix as given by UEG0 with signedValFlag=0, uCoff=14 | prefix: 1 suffix: na | prefix: 951 suffix: na, (uses DecodeBypass) |
| coeff_abs_level_minus1 (9<blocks with ctxBlockCat < 13) | prefix and suffix as given by UEG0 with signedValFlag=0, uCoff=14 | prefix: 1 suffix: na | prefix: 982 suffix: na, (uses DecodeBypass) |
| significant_coeff_flag (5<field coded blocks with ctxBlockCat < 9) | FL, cMax=1 | 0 | 776 |
| significant_coeff_flag (9<field coded blocks with ctxBlockCat < 13) | FL, cMax=1 | 0 | 820 |
| last_significant_coeff_flag (5<field coded blocks with ctxBlockCat < 9) | FL, cMax=1 | 0 | 864 |
| last_significant_coeff_flag (5<field coded blocks with ctxBlockCat < 13) | FL, cMax=1 | 0 | 908 |
| significant_coeff_flag (frame coded blocks with ctxBlockCat = = 9) | FL, cMax=1 | 0 | 660 |
| significant_coeff_flag (frame coded blocks with ctxBlockCat = = 13) | FL, cMax=1 | 0 | 718 |
| last_significant_coeff_flag (frame coded blocks with ctxBlockCat = = 9) | FL, cMax=1 | 0 | 690 |
| last_significant_coeff_flag (frame coded blocks with ctxBlockCat = = 5) | FL, cMax=1 | 0 | 748 |
| coeff_abs_level_minus1 (blocks with ctxBlockCat = = 9) | prefix and suffix as given by UEG0 with signedValFlag=0, uCoff=14 | prefix: 1 suffix: na | prefix: 708 suffix: na, (uses DecodeBypass) |
| coeff_abs_level_minus1 (blocks with ctxBlockCat = = 13) | prefix and suffix as given by UEG0 with signedValFlag=0, uCoff=14 | prefix: 1 suffix: na | prefix: 766 suffix: na, (uses DecodeBypass) |
| significant_coeff_flag (field coded blocks with ctxBlockCat = = 9) | FL, cMax=1 | 0 | 675 |
| significant_coeff_flag (field coded blocks with ctxBlockCat = = 13) | FL, cMax=1 | 0 | 733 |
| last_significant_coeff_flag (field coded blocks with ctxBlockCat = = 9) | FL, cMax=1 | 0 | 699 |
| last_significant_coeff_flag (field coded blocks with ctxBlockCat = = 13) | FL, cMax=1 | 0 | 757 |

# Subclause 9.3.3.1 “Derivation process for ctxIdx”

Replace Table 9-31 with the following.

Table 9-11 – Assignment of ctxIdxBlockCatOffset to ctxBlockCat for syntax elements coded_block_flag, significant_coeff_flag, last_significant_coeff_flag, and coeff_abs_level_minus1

| Syntax element | ctxBlockCat (as specified in Table 9-12) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| coded_block_flag | 0 | 4 | 8 | 12 | 16 | na | 0 | 4 | 8 | na | 0 | 4 | 8 | na |
| significant_coeff_flag | 0 | 15 | 29 | 44 | 47 | 0 | 0 | 15 | 29 | 0 | 0 | 15 | 29 | 0 |
| last_significant_coeff_flag | 0 | 15 | 29 | 44 | 47 | 0 | 0 | 15 | 29 | 0 | 0 | 15 | 29 | 0 |
| coeff_abs_level_minus1 | 0 | 10 | 20 | 30 | 39 | 0 | 0 | 10 | 20 | 0 | 0 | 10 | 20 | 0 |

# Subclause 9.3.3.1.1.9 “Derivation process of ctxIdxInc for the syntax element coded_block_flag”

Replace the first paragraph with the following.

Input to this process is ctxBlockCat and additional input is specified as follows.

- If ctxBlockCat is equal to 0, 6, and 10, no additional input

- Otherwise, if ctxBlockCat is equal to 1 or 2, luma4x4BlkIdx

- Otherwise, if ctxBlockCat is equal to 3, the chroma component index iCbCr

- Otherwise (ctxBlockCat is equal to 4), chroma4x4BlkIdx and the chroma component index compIdx

- Otherwise, if ctxBlockCat is equal to 8 or 9, cb4x4BlkIdx

- Otherwise, if ctxBlockCat is equal to 11 or 12, cr4x4BlkIdx

Replace the paragraph starting with “Let the variable transBlockN (with N being either A or B)” with the following.

Let the variable transBlockN (with N being either A or B) be derived as follows.

- If ctxBlockCat is equal to 0, 6, or 10, the following applies.

- The derivation process for neighbouring macroblocks specified in subclause Error: Reference source not found is invoked and the output is assigned to mbAddrN (with N being either A or B).

- The variable transBlockN is derived as follows.

- If mbAddrN is available and the macroblock mbAddrN is coded in Intra_16x16 prediction mode, the following applies.

- When ctxBlockCat is equal to 0, the luma DC block of macroblock mbAddrN is assigned to transBlockN

- When ctxBlockCat is equal to 6, the Cb DC block of macroblock mbAddrN is assigned to transBlockN

- When ctxBlockCat is equal to 10, the Cr DC block of macroblock mbAddrN is assigned to transBlockN

- Otherwise, transBlockN is marked as not available.

Insert the following paragraphs before the paragraph starting with “Let the variable condTermFlagN (with N being either A or B) be derived as follows”.

- Otherwise, if ctxBlockCat is equal to 7 or 8 (chroma_format_idc is equal to 3 (4:4:4)), the following applies.

- The derivation process for neighbouring 4x4 Cb blocks specified in subclause 6.4.8.4.1 is invoked with cb4x4BlkIdx as input and the output is assigned to mbAddrN, cb4x4BlkIdxN (with N being either A or B).

- The variable transBlockN is derived as follows.

- If mbAddrN is available, the macroblock mbAddrN is not skipped, mb_type for the macroblock mbAddrN is not equal to I_PCM, ( ( CodedBlockPatternLuma >> ( cb4x4BlkIdxN >>2 ) ) & 1 ) is not equal to 0 for the macroblock mbAddrN, and transform_size_8x8_flag is equal to 0 for the macroblock mbAddrN, the 4x4 Cb block with index cb4x4BlkIdxN of macroblock mbAddrN is assigned to transBlockN.

- Otherwise, if mbAddrN is available, the macroblock mbAddrN is not skipped, ( ( CodedBlockPatternLuma >> ( cb4x4BlkIdxN >>2 ) ) & 1 ) is not equal to 0 for the macroblock mbAddrN, and transform_size_8x8_flag is equal to 1 for the macroblock mbAddrN, the 8x8 Cb block with index ( cb4x4BlkIdxN >> 2 ) of macroblock mbAddrN is assigned to transBlockN.

- Otherwise, transBlockN is marked as not available.

- Otherwise, if ctxBlockCat is equal to 11 or 12 (chroma_format_idc is equal to 3 (4:4:4)), the following applies.

- The derivation process for neighbouring 4x4 Cr blocks specified in subclause 6.4.8.4.1 is invoked with cb4x4BlkIdx as input and the output is assigned to mbAddrN, cb4x4BlkIdxN (with N being either A or B).

- The variable transBlockN is derived as follows.

- If mbAddrN is available, the macroblock mbAddrN is not skipped, mb_type for the macroblock mbAddrN is not equal to I_PCM, ( ( CodedBlockPatternLuma >> ( cb4x4BlkIdxN >>2 ) ) & 1 ) is not equal to 0 for the macroblock mbAddrN, and transform_size_8x8_flag is equal to 0 for the macroblock mbAddrN, the 4x4 Cb block with index cb4x4BlkIdxN of macroblock mbAddrN is assigned to transBlockN.

- Otherwise, if mbAddrN is available, the macroblock mbAddrN is not skipped, ( ( CodedBlockPatternLuma >> ( cb4x4BlkIdxN >>2 ) ) & 1 ) is not equal to 0 for the macroblock mbAddrN, and transform_size_8x8_flag is equal to 1 for the macroblock mbAddrN, the 8x8 Cb block with index ( cb4x4BlkIdxN >> 2 ) of macroblock mbAddrN is assigned to transBlockN.

- Otherwise, transBlockN is marked as not available.

# Subclause 9.3.3.1.3 “Assignment process of ctxIdxInc for syntax elements significant_coeff_flag, last_significant_coeff_flag, and coeff_abs_level_minus1”

Replace Table 9-33 with the following

Table 9-12 – Specification of ctxBlockCat for the different blocks

| Block description | maxNumCoeff | ctxBlockCat |
| --- | --- | --- |
| block of luma DC transform coefficient levels (i.e., list Intra16x16DCLevel as described in subclause Error: Reference source not found) | 16 | 0 |
| block of luma AC transform coefficient levels (i.e., list Intra16x16ACLevel[ i ] as described in subclause Error: Reference source not found) | 15 | 1 |
| block of 16 luma transform coefficient levels (i.e., list LumaLevel[ i ] as described in subclause Error: Reference source not found) | 16 | 2 |
| block of chroma DC transform coefficient levels when chroma_format_idc is equal to 1 or 2 | 4 * NumC8x8 | 3 |
| block of chroma AC transform coefficient levels when chroma_format_idc is equal to 1 or 2 | 15 | 4 |
| block of 64 luma transform coefficient levels (i.e., list LumaLevel8x8[ i ] as described in subclause Error: Reference source not found) | 64 | 5 |
| block of Cb DC transform coefficient levels when chroma_format_idc is equal to 3 (i.e., list CbIntra16x16DCLevel as described in subclause Error: Reference source not found) | 16 | 6 |
| block of Cb AC transform coefficient levels when chroma_fomrat_idc is equal to 3 (i.e., list CbIntra16x16ACLevel[ i ] as described in subclause Error: Reference source not found) | 15 | 7 |
| block of 16 Cb transform coefficient levels when chroma_format_idc is equal to 3 (i.e., list CbLevel[ i ] as described in subclause Error: Reference source not found) | 16 | 8 |
| block of 64 Cb transform coefficient levels when chroma_format_idc is equal to 3 (i.e., list CbLevel8x8[ i ] as described in subclause Error: Reference source not found) | 64 | 9 |
| block of Cr DC transform coefficient levels when chroma_format_idc is equal to 3 (i.e., list CrIntra16x16DCLevel as described in subclause Error: Reference source not found) | 16 | 10 |
| block of Cr AC transform coefficient levels when chroma_fomrat_idc is equal to 3 (i.e., list CrIntra16x16ACLevel[ i ] as described in subclause Error: Reference source not found) | 15 | 11 |
| block of 16 Cr transform coefficient levels when chroma_format_idc is equal to 3 (i.e., list CrLevel[ i ] as described in subclause Error: Reference source not found) | 16 | 12 |
| block of 64 Cr transform coefficient levels when chroma_fomrat_idc is equal to 3 (i.e., list CrLevel8x8[ i ] as described in subclause Error: Reference source not found) | 64 | 13 |

Replace the paragraph starting with “For the syntax elements significant_coeff_flag and” with the following.

For the syntax elements significant_coeff_flag and last_significant_coeff_flag in 8x8 luma, Cb, or Cr blocks with ctxBlockCat  = = 5, 9, or 13, Error: Reference source not found contains the specification of ctxIdxInc for the given values of levelListIdx, where levelListIdx ranges from 0 to 62, inclusive.

Change the title of Table 9-34 to “Mapping of scanning position to ctxIdxInc for ctxBlockCat  = =  5, 9, or 13”.

# Subclause A.2.7 “High 4:4:4 profiles”

Make the following changes to subclause A.2.7.

Delete subclause A.2.7 “High 4:4:4 profile”.

Add a new subclause A.2.7 “Advanced 4:4:4 Intra-only profile” with the following.

A.2.7 	Advanced 4:4:4 Intra-only profile

Bitstreams conforming to the Advanced 4:4:4 Intra-only profile shall obey the following constraints:

–	Only I slice types may be present.

–	NAL unit streams shall not contain nal_unit_type values in the range of 2 to 4, inclusive.

–	Arbitrary slice order is not allowed.

–	Picture parameter sets shall have num_slice_groups_minus1 equal to 0 only.

–	Picture parameter sets shall have redundant_pic_cnt_present_flag equal to 0 only.

–	Sequence parameter sets shall have bit_depth_luma_minus8 in the range of 0 to 6 inclusive.

–	Sequence parameter sets shall have bit_depth_chroma_minus8 in the range of 0 to 6 inclusive.

–	The level constraints specified for the Advanced 4:4:4 Intra-only profile in subclause Error: Reference source not found shall be fulfilled.

Conformance of a bitstream to the Advanced 4:4:4 Intra-only profile is specified by profile_idc being equal to 166. Decoders conforming to the Advanced 4:4:4 Intra-only profile at a specific level shall be capable of decoding all bitstreams in which in which level_idc and constraint_set3_flag represent a level less than or equal to the specified level and either or both of the following conditions are true:

- profile_idc is equal to 166, or

- constraint_set1_flag is equal to 1.

# New subclause A.2.8 “Advanced 4:4:4 profile”

Add a new subclause A.2.8 “Advanced 4:4:4 profile” with the following.

A.2.8	 Advanced 4:4:4 profile

Bitstreams conforming to the Advanced 4:4:4 profile shall obey the following constraints:

–	Only I, P, B slice types may be present.

–	NAL unit streams shall not contain nal_unit_type values in the range of 2 to 4, inclusive.

–	Arbitrary slice order is not allowed.

–	Picture parameter sets shall have num_slice_groups_minus1 equal to 0 only.

–	Picture parameter sets shall have redundant_pic_cnt_present_flag equal to 0 only.

–	Sequence parameter sets shall have bit_depth_luma_minus8 in the range of 0 to 6 inclusive.

–	Sequence parameter sets shall have bit_depth_chroma_minus8 in the range of 0 to 6 inclusive.

–	The level constraints specified for the Advanced 4:4:4 profile in subclause Error: Reference source not found shall be fulfilled.

Conformance of a bitstream to the Advanced 4:4:4 profile is specified by profile_idc being equal to 188. Decoders conforming to the Advanced 4:4:4 profile at a specific level shall be capable of decoding all bitstreams in which in which level_idc and constraint_set3_flag represent a level less than or equal to the specified level and either or both of the following conditions are true:

- profile_idc is equal to 166, or 188, or

- constraint_set1_flag is equal to 1.

# Subclause A.3 “Levels”

Replace the text “,or High 4:4:4 profiles” with “, Advanced 4:4:4 Intra-only, or Advanced 4:4:4 profiles”.

Replaced the text “, and High 4:4:4 profiles” with “, Advanced 4:4:4 Intra-only, and Advanced 4:4:4 profiles”.