| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 19th Meeting: Geneva, Switzerland, 31 March - 7 April, 2006 | Document: JVT-Rxxx Filename: input.doc |
| --- | --- |

| Title: | Meeting Report, Draft 0 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan Thomas Wiegand Jens-Rainer Ohm | Tel: Email: Tel: Email: Tel: Email: | +1 (425) 703-5308 garysull@microsoft.com +49 30 31002 617 wiegand@hhi.de ohm@ient.rwth-aachen.de |
| Source: | JVT |  |  |

_____________________________

# Abstract

The Joint Video Team (JVT) of ITU-T Q.6/16 and ISO/IEC JTC 1/SC 29/WG 11 held its 19th meeting during 31 March - 7 April, 2006  in Geneva, Switzerland. The JVT meeting was held under the chairmanship of Dr. Gary Sullivan (Microsoft/USA) and Dr. Jens-Rainer Ohm (RWTH Aachen/Germany), and under the associate chairmanship of Dr. Thomas Wiegand (Fraunhofer HHI/Germany). Our other associate chairman, Dr. Ajay Luthra (Motorola/USA), was unable to attend this meeting. The JVT meetings opened at approximately 14:00 on Friday 31 March 2006 and closed at ??:?? on Friday 7 April 2006.  Approximately ??? people attended the JVT meetings (as recorded on a sign-in sheet passed at the meeting) and approximately ?? input documents were discussed.  The meetings took place in a co-located fashion with a meeting of ITU-T Study Group 16.  The subject matter of these activities consisted of work on video coding.

# Contents

1.	Abstract	1

2.	Contents	2

3.	Documents of the JVT meeting	2

3.1.	Input documents	2

3.2.	Major output documents	4

3.3.	JVT internal output documents	5

3.4.	SVC core experiment output documents	6

3.5.	4:4:4 core experiment output documents	6

4.	JVT Administrative topics	6

4.1.	Administrative documents	6

4.2.	IPR policy reminder	6

4.3.	Late documents	7

5.	JVT SVC normative modifications	7

5.1.	CE 1 & related docs: CAVLC	7

5.2.	CE 2 & related docs: Inter-layer motion prediction	11

5.3.	CE 3 & related docs: Improved quantization	12

5.4.	CE 4 & related docs: Interlaced Coding	12

5.5.	Resampling	13

5.6.	Lossless Coding	14

5.7.	SNR scalability	15

5.8.	Error Resiliency	15

5.9.	Signaling of inter-layer prediction	15

5.10.	High-level syntax	16

5.11.	Discardable layers and adaptations	18

5.12.	Non-VLC FGS Issues	18

6.	JVT SVC non-normative modifications	19

6.1.	Encoder optimization	19

6.2.	Software	19

7.	JVT 4:4:4 coding normative modifications	19

7.1.	CE 5 & related docs: 4:4:4 coding	19

7.2.	Other subjects	20

8.	JVT 4:4:4 coding non-normative modifications	21

9.	JVT Remaining normative coding modifications	21

10.	JVT remaining non-normative and SEI/VUI aspects	21

10.1.	SEI	21

10.2.	Encoder optimization techniques	22

10.3.	Improved Error resilience	22

10.4.	Reference Software	22

10.5.	Joint Model (JM) text update	22

11.	Withdrawn JVT documents	22

12.	Updating of JVT group documents	22

13.	JVT internal operating rules	22

14.	List of Adoptions	23

14.1.	Normative SVC adoptions	23

14.2.	Non-Normative SVC adoptions	23

14.3.	Non-Normative non-SVC adoptions	23

15.	List of AHGs established	23

16.	Resolutions of the meeting	24

17.	SW integration plan	24

18.	Attendance	24

# Documents of the JVT meeting

## Input documents

JVT-S000 List of documents of Geneva meeting

JVT-S001 [G.J. Sullivan, J.-R. Ohm, A. Luthra, T. Wiegand] AHG Report: Proj mgmt and errata

JVT-S002 [T. Wiegand, K. Suehring, A. Tourapis, K.P. Lim] AHG Report: JM text and ref soft

JVT-S003 [T. Suzuki] AHG Report: Bitstreams & conformance

JVT-S004 [L. Bivolarski, J. Vieron, H. Schwarz, M. Wien] AHG Report: JSVM s/W and new func. integ.

JVT-S005 [J. Reichel, H. Schwarz, M. Wien] AHG Report: JSVM & JD text

JVT-S006 [S. Sun, A. Segall, J. Reichel] AHG Report: Spatial scalability resampling

JVT-S007 [Y.-K. Wang, S. Pateux, M.-H. Lee] AHG Report: High-level syntax, err resil

JVT-S008 [M. Wien, H. Schwarz] AHG Report: Coding eff & JSVM perf test cond

JVT-S009 [T. Suzuki] AHG Report: Study of 4:4:4 functionality

JVT-S010 [J. Vieron, H. Schwarz] AHG Report: SVC interlaced coding

JVT-S011 [N. Cammas, Y. Bao] AHG Report: Improved FGS coding

JVT-S012 [M. Mathew, J. Li, H. Schwarz] AHG Report: Discardable and multi-layer R-D

JVT-S013 [G.J. Sullivan, J.-R. Ohm, A. Luthra, T. Wiegand] Report of previous (Bangkok) meeting (JVT-R200)

JVT-S014 [S. Sekiguchi, Y. Isu, Y. Yamada, K. Asai, T. Murakami] CE5: Results of Core Experiment on 4:4:4 Coding

JVT-S015 [D.Y. Suh, G.H. Park, J. Oh, M.W. Park] Extension of Redundant Pics for Lost Key Pics

JVT-S016 [S. Sun] Upsampling filter design with cubic splines

JVT-S017 [J. Vieron, E. Francois, V. Bottreau] CE4: Report on interlaced SVC

JVT-S018 [J. Vieron, E. Francois, V. Bottreau, H. Schwarz, T. Hinz, S. Sun] CE4: Inter-layer pred for interlace/prog SVC

JVT-S019 [F. Le Leannec] Switching betw. FGS layers with reduced drift

JVT-S020 [T. Tran, L. Liu, P. Topiwala] High-perf low-complexity SVC up/down sampling

JVT-S021 [J. Jia, H.K. Kim, H.C. Choi, J.G. Kim] SP picture for SVC

JVT-S022 [S. Lin, L. Xiong, P. Zeng, J. Zhou] Complexity-reduced adaptive VLC in FGS

JVT-S023 [Q. Xie, L. Xiong] A layer prediction for interlace

JVT-S024 [Y.L. Lee] Lossless coding for SVC

JVT-S025 [S. Rane, P. Baccichet and B. Girod] Lossy Error Prot. based on Red. Slices & FMO

JVT-S026 [K. Lee] CE2: Inter layer motion pred of temporal SVC

JVT-S027 [K. Lee] CE4: Progressive to interlace motion pred

JVT-S028 [J. Zhang, X. Yi, N. Ling, W. Shang] Context Adaptive Lagrange Multiplier (CALM) for Motion Estimation in JM

JVT-S029 [W.J. Han] CE1: Grouped & run-length VLC of respred & cbp

JVT-S030 [S. Wittman, T. Wedi] Post-filter SEI for 4:4:4 coding

JVT-S031 [M. Shima] Modified VLC for refinement pass

JVT-S032 [W.J. Han] Encoding condition of residual pred flag

JVT-S033 [W.J. Han] CE4: Resid & texture pred for interlaced CGS

JVT-S034 [B.K. Lee] Complexity reduction and improved FGS

JVT-S035 [Y.-K. Wang, M.M. Hannuksela, X. Wang] Reference picture marking for key pictures

JVT-S036 [Y.-K. Wang, A. Eleftheriadis, T.C. Thang] Bug fixes and enh scalability info SEI

JVT-S037 [S. Liu, H. Li, Y.-K. Wang, S. Wenger, M.M. Hannuksela] Parameter set CRC SEI message

JVT-S038 [M.M. Hannuksela, Y.-K. Wang] On high-level syntax

JVT-S039 [Y. Guo, H. Liu, H. Li, Y.-K. Wang, M.M. Hannuksela] On discardable lower layer adaptations

JVT-S040 [Y.-K. Wang, M.M. Hannuksela] Some constraints to be considered

JVT-S041 [Q. Shen, H. Li, Y.-K. Wang, M.M. Hannuksela] Ref pic mgm for temporal scalability

JVT-S042 [H. Liu, H. Li, Y.-K. Wang] Verif Thomson proposal JVT-S049 on RPLR

JVT-S043 [M. Mathew, K. Lee, W.J. Han] Multi layer quality layers

JVT-S044 [I. Amonou, N. Cammas, S. Kervadec, S. Pateux] Enh SNR SVC for layered CGS w/ quality layers

JVT-S045 [I. Amonou, N. Cammas, S. Kervadec, S. Pateux] CE3: Improved quantization

JVT-S046 [P. Baccichet and A. Chimienti] Err resil by redundant descriptions

JVT-S047 [B. Haskell] Imp backward compatibility for SVC

JVT-S048 [S. Takamura, Y. Bandoh, K. Kamikura, Y. Yashima] Perf Eval Lossless Video Coding Extension

JVT-S049 [Y. Chen, E. Frangois] Spatial enh layer ref pic list construction

JVT-S050 [withdrawn] withdrawn

JVT-S051 [J.-Z. Xu] Frame-based selected inter-layer pred

JVT-S052 [J.-Z. Xu] Verif Samsung JVT-S043 Multi quality layers

JVT-S053 [T. Kimoto] Verif Samsung JVT-S026 mot pred temporal SVC

JVT-S054 [M.H. Lee, H.W. Sun, D. Ichimura, Y. Honda, S.M. Shen] ROI slice SEI message

JVT-S055 [C. Zhang, L. Yu, L. Xiong] Simplif. Upsampling of IntraBL mode

JVT-S056 [L. Zhu, Y. Chen] Rate control for JSVM

JVT-S057 [J.H. Park, S.-W. Park, B.-M. Jeon] CE1: Modif VLC of 8x8 trans for FGS layer

JVT-S058 [J.H. Park, S.-W. Park, B.-M. Jeon] VLC selector for 4x4 transform blocks

JVT-S059 [J.H. Park, S.-W. Park, B.-M. Jeon] CE1: Verif Samsung JVT-S029 VLC

JVT-S060 [J.H. Park, S.-W. Park, B.-M. Jeon] Verif Qualcomm JVT-Sqqq

JVT-S061 [C. Chen, Y. He] Verif JVT-S022 adaptive VLC in FGS

JVT-S062 [R. Lange, M. Domanski] Vector median MV pred for SVC

JVT-S063 [R. Lange, M. Domanski] Inter-layer MV pred in JSVM  case study

JVT-S064 [X. Ji] Improved FGS for interlaced SVC

JVT-S065 [Y. Bao, Y. Ye] Further results on Generalized FGS

JVT-S066 [Y. Ye, Y. Bao] Improvements to FGS layer VLC

JVT-S067 [G. Sullivan] Position calculation for SVC upsampling

JVT-S068 [H. Schwarz, J. Reichel, M. Wien, D. Marpe, T. Wiegand] Skip mode for SVC slice data syntax

JVT-S069 [H. Schwarz, D. Marpe, T. Wiegand] Indep parsing of spatial and CGS layers

JVT-S070 [H. Schwarz, M. Wien, J. Vieron] Proposal for a JSVM software manual

JVT-S071 [K. Zhang] Cross-verification of JVT-S055 upsampling

JVT-S072 [withdrawn] withdrawn

JVT-S073 [D. Marpe, H. Kirchhoffer, T. Wiegand] CE5: Color transform issues and results

JVT-S074 [H. Kirchhoffer, D. Marpe, T. Wiegand] Verif of JVT-Sqqq on RCP

JVT-S075 [M. Wien] CE5: Verif of JVT-Sqqq HHI 4:4:4 proposal

JVT-S076 [M. Wien] CE1: Verif of JVT-S022 adapt VLC in FGS

JVT-S077 [J. Ridge, X. Wang, A. Hallapuro, M. Karczewicz, I. Amonou, N. Cammas, S. Kervadec, S. Pateux] Simplification and unification of FGS

JVT-S078 [B. Haskell] Interop for interlace in AVC/H.264

JVT-S079 [A.M. Tourapis, K. Suehring, G.J. Sullivan] Updates for reference software manual

JVT-S080 [A. Eleftheriadis] Management of SVC info using control ops

JVT-S081 [H. Yu] CE5: Test report

JVT-S082 [A. Luthra, S. Narasimhan, B. Eifrig] Progressive/Interlace display inference

JVT-S083 [S. Narasimhan, B. Eifrig, A. Luthra] no_output_of_prior_pics_flag & splicing

JVT-S084 [W.-S. Kim, D. Birinov, D. Cho, H. M. Kim] CE5: Report of CE on 4:4:4 Coding

JVT-S085 [W.-S. Kim, D. Birinov, D. Cho, H. M. Kim] SEI for Indep Color Component Coding

JVT-S086 [A. Segall] Filter design for dyadic resampling

JVT-S087 [A. Segall, L. Kerofsky and S. Lei] Tone mapping SEI

JVT-S088 [J. Ridge] CE4: Verif Samsung proposal JVT-Sqqq

JVT-S089 [J. Ridge, X. Wang, M. Karczewicz] An 8x8 VLC for FGS

JVT-S090 [J. Ridge, X. Wang, M. Karczewicz] VLC coding of residual prediction flag

JVT-S091 [Y. Ye] CE1: Verif LG JVT-S057 VLC of 8x8 trans

JVT-S092 [S. Kamp, M. Wien] Local adaptation of AR-FGS leak factor

JVT-S093 [M. Shima] Verif JVT-S058 VLC for 4x4 blocks

---------- LATE REGISTRATIONS ---------

JVT-S094  [SMPTE]    Intra-only

JVT-S095  [Wenger & Schierl] SVC RTP payload packetization format status

JVT-S096  [qq] Cross-check of JVT-S051

## Major output documents

(Dates listed are planned dates of availability.)

##### JVT-S200 Meeting report of the 19th JVT meeting [??/??/??]

##### JVT-S201 Joint Draft 6: Scalable Video Coding [??/??/??]

##### JVT-S202 Joint Scalable Video Model (JSVM) 6 [??/??/??]

##### JVT-S203 JSVM 6 Software [??/??/??]

##### JVT-S204 Joint Draft 3: 4:4:4 coding [??/??/??]

##### JVT-S205 Joint 4:4:4 Video Model (JFVM) 3 [??/??/??]

##### JVT-S206 JFVM 3 Software [??/??/??]

## JVT internal output documents

## SVC core experiment output documents

##### JVT-S301 [???] CE1: ??? [based on ???] (Participants:) [??/??/??]

## 4:4:4 core experiment output documents

##### JVT-S305 [???] CE5: ??? [based on ???] (Participants:) [??/??/??]

# JVT Administrative topics

## Administrative documents

##### JVT-S000 List of documents of Geneva meeting

##### JVT-S001 [G.J. Sullivan, J.-R. Ohm, A. Luthra, T. Wiegand] AHG Report: Proj mgmt and errata

##### JVT-S002 [T. Wiegand, K. Suehring, A. Tourapis, K.P. Lim] AHG Report: JM text and ref soft

##### JVT-S003 [T. Suzuki] AHG Report: Bitstreams & conformance

##### JVT-S004 [L. Bivolarski, J. Vieron, H. Schwarz, M. Wien] AHG Report: JSVM s/W and new func. integ.

##### JVT-S005 [J. Reichel, H. Schwarz, M. Wien] AHG Report: JSVM & JD text

##### JVT-S006 [S. Sun, A. Segall, J. Reichel] AHG Report: Spatial scalability resampling

##### JVT-S007 [Y.-K. Wang, S. Pateux, M.-H. Lee] AHG Report: High-level syntax, err resil

##### JVT-S008 [M. Wien, H. Schwarz] AHG Report: Coding eff & JSVM perf test cond

##### JVT-S009 [T. Suzuki] AHG Report: Study of 4:4:4 functionality

##### JVT-S010 [J. Vieron, H. Schwarz] AHG Report: SVC interlaced coding

##### JVT-S011 [N. Cammas, Y. Bao] AHG Report: Improved FGS coding

##### JVT-S012 [M. Mathew, J. Li, H. Schwarz] AHG Report: Discardable and multi-layer R-D

Some lower layer residuals are not used for inter-lazer prediction and can thus be discarded.  JVT-R050 and <jvtßr064 proposed. Related contibutions to this meeting#

S069 indep lazer parsing ß helps create discardable lazers

S043 discardable <fgs lazers#

S051 frameßbased extension of temporal level concept of <<JVTßR064.

S039 proposal to separate data used for interßlazer prediction from data not used as such

##### JVT-S013 [G.J. Sullivan, J.-R. Ohm, A. Luthra, T. Wiegand] Report of previous (Bangkok) meeting (JVT-R200)

## IPR policy reminder

Participants were reminded of the IPR policies established by the parent organizations of the JVT and were referred to the parent body web sites for further information.

Participants were particularly reminded of the need to supply a completed JVT IPR status reporting form in all technical proposals for normative standardization.

## Late documents

No objections were voiced to the consideration of the late documents.  Documents listed in this report in italics and with a "-L" suffix to their document numbers were classified as late. Those documents marked with [Not available at first meeting day] will only be considered as information documents at the end of the meeting if time permits.

JVT-S015 [D.Y. Suh, G.H. Park, J. Oh, M.W. Park] Extension of Redundant Pics for Lost Key Pics

JVT-S020 [T. Tran, L. Liu, P. Topiwala] High-perf low-complexity SVC up/down sampling

JVT-S033 [W.J. Han] CE4: Resid & texture pred for interlaced CGS

JVT-S041 [Q. Shen, H. Li, Y.-K. Wang, M.M. Hannuksela] Ref pic mgm for temporal scalability

JVT-S042 [H. Liu, H. Li, Y.-K. Wang] Verif Thomson proposal JVT-S049 on RPLR

JVT-S052 [J.-Z. Xu] Verif Samsung JVT-S043 Multi quality layers

JVT-S056 [L. Zhu, Y. Chen] Rate control for JSVM

JVT-S059 [J.H. Park, S.-W. Park, B.-M. Jeon] CE1: Verif Samsung JVT-S029 VLC

JVT-S065 [Y. Bao, Y. Ye] Further results on Generalized FGS

JVT-S068 [H. Schwarz, J. Reichel, M. Wien, D. Marpe, T. Wiegand] Skip mode for SVC slice data syntax

JVT-S069 [H. Schwarz, D. Marpe, T. Wiegand] Indep parsing of spatial and CGS layers

JVT-S070 [H. Schwarz, M. Wien, J. Vieron] Proposal for a JSVM software manual

JVT-S071 [K. Zhang] Cross-verification of JVT-S055 upsampling

JVT-S073 [D. Marpe, H. Kirchhoffer, T. Wiegand] CE5: Color transform issues and results

JVT-S074 [H. Kirchhoffer, D. Marpe, T. Wiegand] Verif of JVT-Sqqq on RCP

JVT-S076 [M. Wien] CE1: Verif of JVT-S022 adapt VLC in FGS

JVT-S079 [A.M. Tourapis, K. Suehring, G.J. Sullivan] Updates for reference software manual

JVT-S080 [A. Eleftheriadis] Management of SVC info using control ops

JVT-S081 [H. Yu] CE5: Test report

JVT-S085 [W.-S. Kim, D. Birinov, D. Cho, H. M. Kim] SEI for Indep Color Component Coding

JVT-S086 [A. Segall] Filter design for dyadic resampling

JVT-S088 [J. Ridge] CE4: Verif Samsung proposal JVT-Sqqq

JVT-S089 [J. Ridge, X. Wang, M. Karczewicz] An 8x8 VLC for FGS

JVT-S091 [Y. Ye] CE1: Verif LG JVT-S057 VLC of 8x8 trans

JVT-S093 [M. Shima] Verif JVT-S058 VLC for 4x4 blocks

JVT-S094  [SMPTE] Intra-only

JVT-S095  [Wenger & Schierl] SVC RTP payload packetization format status

JVT-S096  [qq] Cross-check of JVT-S051

# JVT SVC normative modifications

## CE 1 & related docs: CAVLC

##### JVT-S029 [W.J. Han] CE1: Grouped & run-length VLC of respred & cbp

JVT has recently integrated the CAVLC coding scheme to SVC. This contribution shows that in hierarchical B and multi-layer structure, direct VLC coding scheme encodes certain syntax elements such as residual_prediction_flag and cbp very inefficiently compared to CABAC since every symbol of macroblock is encoded separately whereas the symbols have very consistent correlation as temporal levels become higher. In this contribution, max-length limited run-length or simple skip coding techniques are applied to encode a group of cbp and residual_prediction_flag and it is shown that the average bit-rate reduction per layer can be easily up to 8% at lowest bit-rate points of 4CIF sequences in spatial scalability test configuration without changing the fundamental structure of the decoder.

Presented.

Notes 1:

Inefficiency of CBP esp. at high QP.

Concept – Set up run-length coding of CBP across different non-skipped macroblocks.  Establish a maximum run value and assign a short codeword to that value.  The run-length is sent with the first macroblock in each run.  Requires look-ahead across future macroblocks before sending bits for first macroblock in each run.

Alternative is to code a skip flag / skip run.

Also discusses coding of residual prediction flag.

Advocates being able to switch between different methods.

Software provided with contribution, and have had proposals at previous meetings.

Notes 2:

Analyse in detail encoding of syntax elements at MB level, with emphasis on CBP and residual pred. flag. Of particular importance are cases with long runs of zero symbols (e.g. CBP). Apply runlength coding of zero CBP values. “Normal” Skipped MB method is not considered (currently no implementation of skip flag in SVC EL), he “skip mode” mentioned in the proposal is another variant of the runlength method. Run is sent at first MB. Similar gain could potentially be achieved by implementation of the “normal” skipped MB flag in EL as proposed in S-058, which however would not need t change the current  AVC design.First step should be implementation of skipped MB flag, and then potential further improvements (e.g. allowing a skipped MB with inter-layer prediction) should be shown in ongoing CE.

Conclusion: Implement AVC-based skipped MBs in SVC EL, investigate further improvements in continued CE

##### JVT-S059 [J.H. Park, S.-W. Park, B.-M. Jeon] CE1: Verif Samsung JVT-S029 VLC

Presented.

Verified JVT-S029 using some older video sequences.  Used software provided by Samsung.  Results seem consistent with those reported in JVT-S029.

##### JVT-S068 [H. Schwarz, J. Reichel, M. Wien, D. Marpe, T. Wiegand] Skip mode for SVC slice data syntax

In order to unify the slice data syntax structures for AVC and SVC NAL units, it is proposed to add the syntax elements mb_skip_run / mb_skip_flag, and thus the Skip mode, to SVC NAL units. The coding efficiency was analyzed for spatial scalability using a low-delay and a high-delay configuration and 6 test sequences. Bit-rate savings between -2% (rate increase) and 30% have been measured. For VLC an average bit-rate saving of about 6%, and for CABAC an average bit-rate saving of about 2% is reported.

Presented.

There seems to have been no particular reason why we have not supported MB skipping in SVC as it is done in AVC base layer.

Significant gains shown.  Up to 30% gain.  Average 6% for VLC and 2% for CABAC.

Showed results with container (largest gain), mobile, paris, city, football, soccer. Relationship with JVT-S029, but could be implemented without changing existing AVC concepts.

Seems to be a bug fix, completely consistent with the existing base layer design.

Remark: Should we use inter-layer prediction always for skipped macroblocks?

Ans: We can consider that question later – for now this method seems most well aligned to slice data syntax of base layer – can possibly further improve later by considering such modifications.

Adopted.

Plan to continue CE investigation of other alternatives & variants.

##### JVT-S069 [H. Schwarz, D. Marpe, T. Wiegand] Indep parsing of spatial and CGS layers

In the current JD/JSVM, the slice data syntax is conditioned on partially decoded base layer data. Before parsing an CGS or spatial enhancement layer, all lower layer slices in an access unit need to be parsed and the resampling process for motion data has to be carried out for all lower resolution layers. It is proposed to modify the SVC slice data syntax and the parsing process in a way that enables independent parsing for CGS and spatial layers. The advantages are mainly seen in a reduction of the required decoder complexity. With that modification it is also possible to omit the transmission of slices that only contain macroblocks that are not used for inter-layer prediction. Thus, a more efficient representation of enhancement layer bit-streams could be obtained when the base layer data are partitioned into used and unused macroblocks using slice groups. The coding efficiency loss that results from the proposed modifications has been analyzed for a wide range of configurations for CGS and spatial scalability. The average measured rate increase is less than 1 %.

Presented.

Proposes simplification of syntax (intra base & base layer refinement flags)

Seems to decouple and simplify the design overall.  Only for CGS and spatial scalability.

Create AHG on complexity reduction. Study in AHG on complexity reduction.

##### JVT-S090 [J. Ridge, X. Wang, M. Karczewicz] VLC coding of residual prediction flag

[Withdrawn]

##### JVT-S057 [J.H. Park, S.-W. Park, B.-M. Jeon] CE1: Modif VLC of 8x8 trans for FGS layer

Presented.

In CAVLC FGS, 4x4 and 8x8 blocks share the same VLC code table whose suffix length is 2 bits. However, since long codewords are assigned to large symbols in 8x8 block, the code table does not provide good performance when the run value in the significance pass is big. In JVT-R051 new VLCs assigning the shorter codeword to the large symbol was proposed. The key idea of JVT-R051 is to simply increase the remainder suffix bit length for start-step-stop code instead of 2 bits, so that the length of the codeword for large symbol is reduced. Unfortunately, JVT-R051 could not provide decoder results due to the software problems. In addition to decoder results of JVT-R051 for the 3bits suffix, this contribution proposes the newly designed VLC code table for 8x8 block which matches the probability distribution of symbols and enables simple encoding and decoding operations. With the proposed VLC, average 5.64% bit saving is obtained at test conditions specified in JVT-R301.

Reports a bad fit to pdf for current VLC.

Results with 3 FGS layers: 4.4% QCIF, 6.0% CIF, 7% 4CIF.

Bit rate saving is reported for the case of 3 FGS layers in total, i.e. for the highest rate. Proponent says that the gain may be less for less FGS layers. Perform offline discussion with proponents of JVT-S089 which seems to be similar but less diverging from current design. Revisit later.

Create AHG on CAVLC. Discuss this in AHG on CAVLC.

##### JVT-S091 [Y. Ye] CE1: Verif LG JVT-S057 VLC of 8x8 trans

Presented.

Different results on Soccer sequence – better than what is reported on JVT-S057 in that case.  Otherwise the same results.

##### JVT-S089 [J. Ridge, X. Wang, M. Karczewicz] An 8x8 VLC for FGS

Presented.

Current JSVM uses flags to code significance values.  Suggests to use the de-interleaving concept already found in AVC for FRExt CAVLC of 8x8 blocks.  Try to retain consistency with prior design.  Test results incomplete – but with 2 FGS layers, roughly 5% improvement reported.

Offline discuss – revisit later. Discuss this in AHG on CAVLC.

##### JVT-S058 [J.H. Park, S.-W. Park, B.-M. Jeon] VLC selector for 4x4 transform blocks

In the significance pass for 4x4 transform blocks, run value is encoded by modified start-step-stop VLC codes which are characterized by a cutoff parameter, ‘m’. In JVT-Q040, it is claimed that the optimal value of ‘m’ depends on both the starting position of the cycle (called ScanIndex) and the position of the last-coded coefficient in the base layer (called BaseLastIndex). However, it has been observed that the BaseLastIndex has no great influence on overall coding efficiency under the constraints that VLC selector, which is 2D array specifying ‘m’ value at some combinations of ScanIndex and BaseLastIndex, has to satisfy monotonic increase and ‘m’ value in VLC selector is limited by 4. From the observation, this contribution introduces the modified VLC selector without BaseLastIndex factor to improve coding performance.

Presented.

Proposes to eliminate BaseLastIndex dependency.  Also proposes to eliminate constraints of monotonic increase and limit of 4.

Claim is mainly simplification with minimum change in PSNR results.

Remark: Related to JVT-S066, which reportedly has better results.

##### JVT-S093 [M. Shima] Verif JVT-S058 VLC for 4x4 blocks

Used software provided by LG.  Got successful verification.

##### JVT-S066 [Y. Ye, Y. Bao] Improvements to FGS layer VLC

This contribution proposes several modifications to the current VLC coder for FGS layer coding to improve coding efficiency.  It is observed that the coding performance gap between the current VLC FGS coder and CABAC-based FGS coder is significantly larger than that between the base layer CAVLC coder and base layer CABAC coder.  With the proposed algorithms, the bit rate of FGS layer can be reduced by as much as 15.45% for the SNR test conditions specified in JVT-Q205, with considerable reduction in complexity at the same time.

Proposes:

- Modified significance coding

- Modified refinement coding

- Modified luma & chroma CBP coding (exp-Golomb runs of CBP of same value)

Try to better adapt to statistics of significance pass (beyond the capabilities already in S3 code) by adding a codebook with more flat behaviour. For refinement pass, 3 symbols are allowed instead of separate magnitude & sign (claim of reduced complexity), group each 3 refinement coefficients together (making 27 symbols). Also modify CBP coding (if base layer is CBP=1, it is inherited to enhancement layer (claim of reduced complexity). Improves current CAVLC performance by 5-6 % (for the enhancement layer).

Remark: How do we really estimate complexity and performance impact of these things?

Report of breakout group: long discussions and excellent collaboration

Significance Coding in 4x4 block

- Scheme in JSVM4.x

  - Combine EOB and runs into same symbol set and signal the index of EOB within the symbol set for each cycle in slice header

  - Encode the combined symbol set using S3 codes, with cut-off threshold depending on the coding cycle and the index of the last significant coefficient in the base layer

- Related proposals

  - JVT-S058 removed the dependency of cut-off threshold on base layer to simplify the design

    - The dependency is weak

  - JVT-S066 replaced some S3 codebooks with codebooks of more different statistical properties

    - The original S3 codes do not capture the actual distribution of Run/EOB

- Recommendation of the group

  - Remove base layer dependency from JVT-S066, so only 1-D array is needed to signal selection of VLC codebooks as proposed in JVT-S058

- Simulation results already been obtained. Compared with JVT-S066

  - Improved for QCIF sequences due to reduction of signaling overhead

  - Almost the same for CIF and 4CIF sequences

Adopted

Significance coding in 8x8 block

- Scheme in JSVM4.x

  - Encodes the run/EOB in unary codes

- Related proposals

  - JVT-S057 proposed a new codebook for coding run/EOB to better match the actual distribution

  - JVT-S089 proposed to use similar strategy in AVC/high profile

    - De-interleaving 64 coefficients in zigzag order into 4 groups of 16 coefficients

    - Code each group using the 4x4 VLC coding scheme

- Recommendation

  - Addressed in FGS AHG

Consider significance coding of 8x8 transform coefficients in PR slices in AHG on CAVLC

Refinement coding

- Scheme in JSVM4.x

  - Process refinement coefficients in blocks

  - Encode magnitude and sign in separate groups

  - Flushing at the end of the block à results in ungrouped symbols

  - Adaptive estimation of probability

- Related proposals

  - JVT-S031

    - Reduce the amount of ungrouped symbols

    - Adaptive estimation of probability

  - JVT-S066

    - Define ternary symbol to include both magnitude and sign information to simplify symbol grouping

    - Eliminate the ungrouped symbols

- Both solutions improve coding performance, JVT-S066 performs better for the first and second FGS layer, while JVT-S031 is better on the third layer

- Recommendation of the group

  - Incorporate probability estimation into JVT-S066 to improve coding efficiency at high FGS layers

- Second ternary table already added to JVT-S066 software

Adopted

MB CBP coding

- Scheme in JSVM4.x

  - Encodes Luma CBP using adaptive VLC and Chroma CBP in flags

- Related proposals

  - JVT-022 proposed to improve Luma CBP coding

    - Flip enhancement layer Luma CBP bit if base layer CBP bit is 1

    - Code the processed MB Luma CBP using one of five VLC tables based on number of nonzero CBP bits in the base layer

    - Decrease complexity with no change in coding performance

  - JVT-S066

    - Encode runs of Luma CBP and Chroma CBP using exp-Golomb codes

    - Skip Luma CBP bit if its base layer bit is 1

    - Low complexity with no statistical estimation and VLC tables

    - Improves the coding efficiency

- Recommendation of the group

  - Adopt the new Luma and Chroma coding scheme proposed in JVT-S066

Adopted

Unified FGS coding order

- JSVM4.x has two different coding orders

  - Original cycle-based coding order

  - Subband coding order proposed in R077

    - The coefficients are decoded subband by subband

      - Next run will not be decoded for a block if scanning has not reached end of the previous run

    - A separate loop to code the refinement coefficients for those already significant in the base layer

- Related proposal, JVT-S077

  - Always use the subband scanning order

    - Remove the separate refinement coding loop

  - Use vector coding mode originally proposed in JVT-R069

    - Decode at least certain amount of coefficients (zero/nonzero) from a block before moving to the next block

    - Signal the vector length in the PPS

- Some discussions within the group

Adopted.

##### JVT-S031 [M. Shima] Modified VLC for refinement pass

This contribution presents the modification to improve the compression efficiency of refinement pass VLC coding by conditionally expanding the object of grouping operation to reduce the number of ungrouped symbols.  The simulation results of this contribution are reported to show up to 1.29% bit-saving (about 0.9% bit-saving on average) using the SNR test condition.

Presented.

Re-allocate into cat 5.12 (non-VLC FGS issue).

##### JVT-S022 [S. Lin, L. Xiong, P. Zeng, J. Zhou] Complexity-reduced adaptive VLC in FGS

JVT-Q040 introduced a VLC technique called adaptive VLC to code the symbol CBP (Coded Block Pattern). Dependent on the information from base layer, this contribution presents the modification attempts to reduce the complexity for adaptive VLC by reducing the complexity of the selecting process, and omitting the updating and scaling processes which are the key component of adaptive VLC at the reported cost of 0.008% bit rate increase on average. Sometimes a little bitrate saving is reported.

No condition operations (instead of 32), no comp (inst.18), 66 instead of  9 bit operations, 16 instead of 4 arithmetic. Further consideration to be done in breakout group.

##### JVT-S076 [M. Wien] CE1: Verif of JVT-S022 adapt VLC in FGS

Cross-check OK, checked that changes reported in document match the software; cannot comment the details about precise amount of complexity reduction.

Re-allocate to cat. 5.12 (non-VLC FGS issue).

##### JVT-S077 [J. Ridge, X. Wang, A. Hallapuro, M. Karczewicz, I. Amonou, N. Cammas, S. Kervadec, S. Pateux] Simplification and unification of FGS

It is proposed to modify the FGS coding order so that only the lowest scan position is coded in each cycle, as is done in low-delay FGS (AR-FGS) proposed in JVT-R077.  This can lead to greatly reduced implementation complexity, while unifying the FGS design.  The approach is similar to one proposed in JVT-O055 and also incorporates JVT-R069 to yield further complexity reduction on specific platforms.  Results indicate that, in addition to simplifying the design, a slight coding efficiency gain is possible.

Unification of FGS and AR-FGS: Remove the mechanism from AR-FGS that refinement values from a discrete base layer are coded prior to other values. Introduce “subband oriented” processing instead of “block based” processing which means that coefficients are only loaded once into memory (instead once per cycle). Vector mode of JVT-R069 is introduced. Significance and refinement passes in AR-FGS are fully interleaved.  Results: For FGS, typically loss of 0.03 dB at start of FGS layer, gain of 0.1 dB at higher end. For AR-FGS, <0.1 dB loss at start, no loss at higher end. Mobile shows more dramatic deviations, but this is probably due to different allocation between luma and chroma). Memory accesses reduced by approx. 4-8, further discuss in breakout group.

Re-allocate to 5.12

CE 2 & related docs: Inter-layer motion prediction

##### JVT-S026 [K. Lee] CE2: Inter layer motion pred of temporal SVC

[AAA]

Reports on concept of JVT-R037, which includes the “motion reversing” method in the previous concept. Complexity is estimated as similar to TDM. In worst case, aprox. 202,000 additional memory (16 bit) required per picture. TDM was implemented now, giving approx. 0.48 %  in average and max. 1.5 % bit rate savings over not having it (new SW selects SDM or TDM). Proposed technique gives 5.6 % maximum and 1.6 % average over this configuration. It would be necessary to double the mv memory (list 0 and list 1 required) for each reference picture, because the MV selected by the proposal could be different from TDM, which cannot be foreseen by the decoder.

Not adopted. Close CE.

##### JVT-S053 [T. Kimoto] Verif Samsung JVT-S026 mot pred temporal SVC

The purpose of this report is to verify JVT-S026. It has already been verified that the proponents’ results can be reproduced without any problem. In this report, implementation inspection was carried out.

Results verified.

Re-allocated to section 5.5

## CE 3 & related docs: Improved quantization

##### JVT-S045 [I. Amonou, N. Cammas, S. Kervadec, S. Pateux] CE3: Improved quantization

This document presents results obtained in the Core Experiment 3 on quantization. The proposed technique is based upon embedded refinement technique and adaptive offset of the reconstruction value. Following Bangkok presentation, this technique has been integrated in latest JSVM software (tag 4.2) in order to assess the gain obtained. Evaluations lead on JSVM 4.2 have shown that, when using closed loop encoding, there is no gain by using the proposed embedded quantization technique. There is rather a PSNR degradation of about 0.1 to 0.3 dB. We thus recommend to stay with JSVM implementation of progressive quantization and to close studies on this subject.

Close this CE.

## CE 4 & related docs: Interlaced Coding

##### JVT-S017 [J. Vieron, E. Francois, V. Bottreau] CE4: Report on interlaced SVC

Software presently validated for release, most probably finalized within one week.

##### JVT-S018 [J. Vieron, E. Francois, V. Bottreau, H. Schwarz, T. Hinz, S. Sun] CE4: Inter-layer pred for interlace/prog SVC

This contribution proposes to extend inter-layer prediction processes in order to support any combinations of interlace/progressive scalability. It is a merging of proposals JVT-R014, JVT-R019, JVT-R030 and JVT-R063 made to the 18th JVT meeting. The problem of inter-layer motion prediction addressed here is considered at a macroblock level, and all possible macroblock modes as defined in the base standard are taken into account. The current proposal avoids in-depth modifications, by introducing an intermediate level, called Virtual Base Layer. This intermediate layer avoids or at least significantly limits modifications to the JD5 motion and texture upsampling processes.

Report gains of up to roughly 1.3 dB over current SVC, but results are available only for a relatively small number of sequences currently.

Remark: For continuation of CE and possible further improvements of interlaced tools testing conditions need improvement and clear definition until end of the week.

Software to be used as basis for ongoing CE4.

Adopt interlaced tools as proposed in JVT-S018 into JSVM. Discuss about SW integration schedule later.

##### JVT-S027 [K. Lee] CE4: Progressive to interlace motion pred

This proposal introduces a solution to free the restriction of inter layer motion prediction in JVT-R063 with progressive to interlace configuration. Performance improvement with 176x128 resolution and IP chain structure (key picture only) is reported to be up to 5.4% bit saving (average 2.8%). Performance improvement with CIF resolution and IP chain structure (key picture only) is reported to be up to 5.3% bit saving (average 2.3%). Performance improvement with 176x128 resolution and hierarchical B structure is reported to be up to 3.4% bit saving (average 1.9%). Performance improvement with CIF resolution and hierarchical B structure is reported to be up to 3.2% bit saving (average 2.0%).

Is modification of JVT-S018. Results only given for sub-QCIF (which is unrealistic) and only 2 layers. Technique may not work with more than 2 layers. No gain for SD sequences. No evidence for any decision at this time.

##### JVT-S023 [Q. Xie, L. Xiong] A layer prediction for interlace

In interlace coding for SVC, when base layer is interlaced, enhancement layer is progressive, and this two layers have the same frame rate, splitting the interlaced frame to fields and using the fields to predict corresponding enhancement layer frame, in this way interlayer prediction can be more effective in interlace.

SDI -> SDP average gain is 7.6 %.  SDI -> 720P no gain. With the current proposal, inter-layer prediction would be performed from pictures outside the same access unit which is not foreseen in the current architecture. Breaking this concept would not be justified by the gain reported. Results were made based on SW version from 2 weeks ago that seemed to be buggy; newest version of JVT-S018 shows already improved PSNR. Further study encouraged in ongoing CE

##### JVT-S033 [W.J. Han] CE4: Resid & texture pred for interlaced CGS

Withdrawn.

##### JVT-S088 [J. Ridge] CE4: Verif Samsung proposal JVT-Sqqq

The results presented in JVT-S027, and attached as an excel file, have been independently verified.

Confirmed.

JVT-S064 [X. Ji] Improved FGS for interlaced SVC

In our previous proposal JVT-R083, the technique of open-loop motion compensation is used between the top and bottom fields which belong to the same frame picture to efficiently exploit the temporal redundancy for interlaced sequences. In this proposal, we extend this basic idea to low-delay applications. The basic idea is that the motion-compensated prediction signals for P-slice of bottom fields are formed from partially reconstructed enhancement layer representation or the weighted combination reference between the base layer and the highest quality representation. Different from AR_FGS coding method, only one motion compensation loop is required for bottom fields’ base layer and FGS enhancement layer decoding, while the top fields are coded with AR_FGS to effectively reduce drifting error propagation. Compared with the implementation of AR_FGS [1] in interlaced SVC software, the new approach is able to significantly reduce complexity and meanwhile, maintain comparable compression performance.

Propose to use AR-FGS for top fields, and open-loop prediction for bottom field (can be done with current SVC). As an extension, weighted prediction with alpha <1 could be used for bottom field, no results on that. Information on encoder optimization welcome (in case that low-delay interlaced is needed), no need for any decision on other issue.

## Resampling

Adhoc report given by S. Sun. No visual tests were performed. In particular for chroma upsampling, no difference was visible, or the only difference visible was due to difference in luma (confirmed by Tobias). Different MB modes may be selected, but not possible to say which is better.

##### JVT-S055 [C. Zhang, L. Yu, L. Xiong] Simplif. Upsampling of IntraBL mode

This proposal continues to propose using 2-tap bilinear filters for the up-sampling process of chroma texture of IntraBL mode, which is also described in JVT-Q078 and JVT-R065. New results based on JSVM 4_2 are presented.

Tested both dyadic and both dyadic cases, IDR pictures. Maximum PSNR loss 0.02 dB.

Due to different chroma upsampling filters, different MB modes may be selected. Worst case for Mobile, where luma is worse by 0.1 dB

Perform additional test with sequences Mobile and Paris, where mode decision is only based on luma. Necessary to judge in particular at low rates. Test is necessary to exclude that chroma artifacts are introduced due to the new filters. Negotiate with Tobias and re-visit later in the week

In general  many people in the group seem to agree that bilinear filters for chroma are reasonable.

##### JVT-S071 [K. Zhang] Cross-verification of JVT-S055 upsampling

The purpose of this report is to verify the proposal JVT-S055 Simplification of Up-sampling process. As a verification task, coding performance check was carried out.

Results were verified.

##### JVT-S067 [G. Sullivan] Position calculation for SVC upsampling

The upsampling operation currently found in the JSVM for extended spatial scalability (ESS) uses a particular method of calculating the position and phase information when upsampling the low-resolution layer.  That method relies on the use of an approximate division operator.  The division operator in the JSVM was recently modified in response to JVT-R018 to fix some problems in it.  However, it still has relatively-high computational complexity.  This contribution proposes an alternative method of computing the position and phase information, refining upon a method previously presented in JVT-R067, and reportedly resulting in much lower computational requirements than the JSVM without any significant loss of accuracy.  In nearly all cases, the position and phase calculated by the two methods would reportedly be the same.  However, the contribution asserts that the proposed method can reduce computational requirements significantly – e.g., by reducing nominal dynamic range requirements substantially (by tens of bits).  The necessary supported relationships between luma and chroma positions are also discussed and adjustments are made in the proposed method to account for these relationships.  Software accompanies the contribution for illustration and cross-check purposes.

The least complex version could be multiplier-free design for 16 bit arithmetics, which would however show rounding errors as compared to current JSVM. 32 bit version would give “virtually” the same result as current JSVM in most cases (no deviations found so far), but could theoretically be different in some up-sampling ratios. For the 16-bit implementation and largest picture width (8192 in EL) the accuracy would be approx. 1/8 pixel at the edges. S. Sun, J. Xu and G.S. should report back on the precise savings in computation operation and maximum possible errors.

JSVM: 1 mul 16x16 with 32 bit result, 1 mul 32x32 with 32 bit result, 2 adds 32+32 with 32 bit result, 1 shift 32 -> 16. New method (16 bit) would need 3 adds 16+16 with 16 bit result, 2 shifts 16 -> 16.

Needs to be performed once per column and once per row.

Adopt.

##### JVT-S020 [T. Tran, L. Liu, P. Topiwala] High-perf low-complexity SVC up/down sampling

Presented.

Remark: Proposed upsampler looks like Catmull-Rom / Lanczos-2.

Remark: When QP is smaller in enhancement layer, the base layer is starved of bits and the enhancement layer will essentially act like a single-layer codec.  Under such conditions, the down/upsampling method doesn't much matter.

Remark: Should use Bangkok-output test conditions for resampling evaluation.

Further study in AHG

##### JVT-S086 [A. Segall] Filter design for dyadic resampling

Observation that coding efficiency in IDR could be improved by 2-3% by selecting the correct upsampler with the current downsampler. For lower QP, upsampler should have better highpass properties. The selection of the best upsampling filter is not independent on the target of EL QP.

(Thomas shows additional figures) Typically, 50% of base layer rate can be transferred to the EL (i.e. is usable for prediction). In particular problem arises that the prediction from base layer is less efficient in cases of sequences with aliasing (e.g. Mobile). Selecting other upsampling filters may be beneficial in such cases.

Further study in AHG.

##### JVT-S016 [S. Sun] Upsampling filter design with cubic splines

In SVC, the texture signal of a base layer is upsampled using a set of 6-tap filters before it is used as prediction signal for the enhancement layer.  The 6-tap filters are derived from the Lanczos-3 function and defined in a pre-fixed filter table.  This contribution introduces a matrix-based representation of a set of 6-tap filters, which showed very similar frequency response to that of Lanczos3 filter.  In addition, this contribution also introduces a matrix-based representation of a new set of 4-tap filters, which has wider pass-band than the popular Catmull-Rom filter.

Could lead to a simplified design, as preliminary results indicate that the 4-tap filters are not giving worse results.

Further study in AHG.

JVT-S095  [Wenger & Schierl] SVC RTP payload packetization format status

Information document, was heard.

## Lossless Coding

##### JVT-S024 [Y.L. Lee] Lossless coding for SVC

The JSVM supports various scalable functionalities such as temporal, spatial, and SNR scalabilities. However, JSVM does not support lossless coding at this moment. The lossless coding, which was already adopted in Advanced 4:4:4 profile is proposed for SVC. The lossless coding in JSVM4 codec was implemented to verify the performance of the proposed lossless coding. It is reported that the lossless coding in SVC gives better coding efficiency compared with lossy coding of when QP=0 in SVC. The lossless coding which performs Intra residual DPCM reduces the bit-rate approximately 15% and 8% on average in Base Layer and Base+Enhancement Layer with Intra coding. As a reference, the results were cross-verified by Sharp Lab. using YUV 4:2:0 data format at the Redmond meeting and were cross-verified by Samsung AIT using RGB 4:4:4 data format at the Redmond meeting.

##### JVT-S048 [S. Takamura, Y. Bandoh, K. Kamikura, Y. Yashima] Perf Eval Lossless Video Coding Extension

In this contribution a complexity evaluation and compression comparison with FRExt lossless coding is presented.  According to the reported results, the lossless enhancement coding complexity is about 10-20% of base layer coding in inter-prediction coding.  Inter compression performance is comparable with FRExt non-scalable coding (about 2.5 points increase).  Intra coding performance is comparable with Motion JPEG2000 (about 4.6 points increase).

## Error Resiliency

##### JVT-S015 [D.Y. Suh, G.H. Park, J. Oh, M.W. Park] Extension of Redundant Pics for Lost Key Pics

This contribution proposes a method to recover lost key pictures in the client. The method is an extension of the concept of redundant coded picture. While the previous redundant picture is used to protect only a key picture, this contribution enables to recover one lost picture out of multiple key pictures by using the same amount of redundancy. The redundant coded key picture is generated by performing XOR operation on key pictures of interests. The proposed method is reported to show very good performance under the packet loss condition recommended by JVT.

Presented.

Considers impact of loss of pictures that affect multiple pictures.  Suggests using FEC (actually XOR) across multiple coded pictures, e.g., to protect most important QCIF pictures in a 2-layer scheme with QCIF and CIF layers.

Used common loss conditions - 4000 frames.

Remark: Can this functionality be provided by the systems level?  Proponent: The expertise about exactly what to do may not be there at the systems level.

Remark: Suggesting FEC across PCPs or RCPs?  Proponent: Either one.

Remark: How does decoder know which packets were the input to the FEC?  Proponent: Haven't worked that out yet completely.

Differences from JVT-S025?

1) Applied to PCP, versus JVT-S025 applying to RCP.

2) XOR assumed, JVT-S025 also considers more fancy FEC

For further study, categorized together with JVT-S025.

Related also to JVT-S046, but that is about redundant slices in the video sense, not the channel coding sense.

Software needs some slice functionality and error concealment functionality in order to test this.  Proponent volunteers to work toward adding such functionality into reference software.

##### JVT-S021 [J. Jia, H.K. Kim, H.C. Choi, J.G. Kim] SP picture for SVC

This proposal describes SP-Picture in SVC. SP-Picture in SVC base layer is implemented in this contribution. SP-Picture in SVC enhancement layer is also discussed with the investigation of algorithm and syntax modifications. Primitive results illustrating the performance comparison of SP-Picture to I-Picture in SVC base layer are given in this contribution. For low motion test sequences, around 1dB enhancement on average PSNR are reported to be obtained by using SP pictures compared with using I pictures based on the same bit rate. For high motion test sequences, similar performance of using SP-Picture and I-Picture is reported from the experimental results.

Presented.

(not really especially an error resilience contribution, although that is one use - may recategorize as appropriate)

Example: Fast-forward from Base layer to enhancement layer.

Example: Switching up from Base layer to enhancement layer.

Several other examples.

Put SP pictures at key picture positions in Base layer.

Syntax impact looks straightforward - similar to SP in base layer.

Is the SP concept more effective for SVC than it was for the base layer?  There is still some interest in this concept/functionality - it does provide a streaming switching functionality.

Would it be useful to implement this in the enhancement layer (not done in the contribution)?  Note that using SP in an enhancement layer may not provide as much gain relative to using an EI picture as it did for providing stream switching capability in the base layer.

Seems more useful in some test sequences than others.

Further study encouraged - need to provide results on enhancement layer.  Set up core experiment?  Yes. CE on switching.

##### JVT-S037 [S. Liu, H. Li, Y.-K. Wang, S. Wenger, M.M. Hannuksela] Parameter set CRC SEI message

This contribution proposes an SEI message for AVC/SVC to enable the detection of possible errors in parameter sets, which are crucial to the decoding process. The SEI message contains cyclic redundancy check (CRC) codes for separate or all the parameter sets. The main reason behind the proposal is that detection of whether any error exists in the sequence or picture parameter sets is difficult. Once an incorrect parameter set, be it a sequence or picture parameter set, is used in decoding, the decoder would crash, report irrelevant errors, or produce corrupted video quality. The problem may happen when in-band parameter set transmission (e.g. for adding new parameter sets or updating existing ones) is used. With the proposed SEI message, the decoder can easily detect possible errors in the parameter sets, and take proper actions, e.g. send a feedback message to the encoder.

Presented.

To enable decoder to detect when it has the wrong parameter set information.

How about just repeating the parameter set?  Doesn't seem especially encouraging.  Repeating the parameter set may not take many more bits, and would provide the correct data rather than just an indication of loss.

## Signaling of inter-layer prediction

##### JVT-S032 [W.J. Han] Encoding condition of residual pred flag

Residual prediction is a kind of inter-layer prediction technique between inter-type macroblocks. However, the current syntax allows the residual prediction mode can be used even when the corresponding base-layer MB is a intra-type whereas the decoding process does not describe this situation. Although it does not result in significant performance impacts, this unclear behavior should be fixed due to the potential problem in the decoder-side. Most natural "bug-fix" solution for this issue is to add several new conditions to prohibit the residual prediction when the corresponding base-layer MB is an intra-type.

## High-level syntax

##### JVT-S040 [Y.-K. Wang, M.M. Hannuksela] Some constraints to be considered

This document presents some constraints, mainly on high-level syntax, to be discussed and considered for SVC.

-	VCL NAL units of a picture shall either be only of the AVC VCL NAL unit types (1~5) or be only of SVC NAL unit types (20~21) - adopted

- A key picture shall have temporal_level equal to 0 - not adopted

- An AU with temporal_level being equal to X shall not depend on a AU with a temporal_level Y with Y>X adopted

- In an access unit, if the primary coded picture in one layer is a key picture, the primary coded picture in any other layer shall also be a key picture. - not adopted, re-think

- The temporal_level value of all the pictures in an access unit shall be the same. - adopted

- The temporal_level value of an IDR access unit shall have temporal_level equal to 0. adopted

  - For an enhancement layer picture, if there is no lower layer picture in the same access unit, the enhancement layer picture shall not be an IDR picture. adopted

  - NALU 1-4: ref only NALUs with did==0 for inter prediction in other AUs

  - NALU 5: no ref to other NALUs

  - NALU 20: ref to other NALUs with same did for inter prediction in AUs, ref to NALUs with smaller dids within same AU

  - NALU 21: ref to NALUs with smaller dids within same AU

  - I: bidp1==0, all NALUs, bidp1>0, I -> EI, NALU 20, 21

  - P/B: bidp1==0, NALUs 1-4, 20, bidp1>0, P -> EP, B -> EB

  - PR: quality_level>0, NALU 20, 21

  - EI, EP, EB: quality_level==0, NALU 20, 21

  - dOiDX = dependency_id*4+quality_level

- Remove EI, EP, EB slice_type names

- If an access unit delimiter NAL unit is present, the primary_pic_type shall only be applied to the AVC-compatible base layer picture, if present. In case there is no AVC-compatible base layer picture present in the access unit, the semantics of primary_pic_type is unspecified. revisit

- Layer switching, IDR, EIDR revisit

##### JVT-S038 [M.M. Hannuksela, Y.-K. Wang] On high-level syntax

Two shortcomings in the SVC design are recognized: First, the design decision to indicate key pictures with nal_ref_idc equal to 3 is criticized, as it causes a conflict with nal_ref_idc usage practices of existing H.264/MPEG4-AVC applications. Second, H.264/MPEG4-AVC base layer could be used more efficiently if it were possible to signal temporal_level and simple_priority_id for H.264/MPEG4-AVC base layer pictures. A primary coded picture description NAL unit is proposed. It precedes the VCL NAL units of the primary coded picture in decoding order and includes temporal_level, simple_priority_id, and key_pic_flag for the primary coded picture. As the NAL unit type for the primary coded picture description NAL unit was previously reserved, existing H.264/MPEG4-AVC decoders are required to skip it. Furthermore, it is proposed to include key_pic_flag in the slice header of enhancement coded pictures.

##### JVT-S047 [B. Haskell] Imp backward compatibility for SVC

It is proposed that SVC be able to incorporate multiple H.264/MPEG4-AVC layers in such a way that current gateways and/or receivers can easily route appropriate H.264/MPEG4-AVC NAL Units to non-scalable receivers.

##### JVT-S036 [Y.-K. Wang, A. Eleftheriadis, T.C. Thang] Bug fixes and enh scalability info SEI

This document presents some bug fixes and enhancements to the syntax and semantics of the scalability information SEI message.

##### JVT-S080 [A. Eleftheriadis] Management of SVC info using control ops

This document proposes a new design for the Scalability SEI (SSEI) message. It first discusses use scenarios for the SSEI, demonstrating that the current design represents a significant overhead in terms of bits and also results in considerable complexity for intermediate media gateways. It also identifies overall structural problems in the encoding of scalability information in both the Sequence Parameter Set (SPS) and the SSEI. It then proposes a new structure for managing scalability information that augments the current SSEI message with control messages that indicate changes in the layer composition of the video stream (add/replace/delete). The proposed technique results in both reduction of bitrate as well as reduction of processing complexity.

##### JVT-S035 [Y.-K. Wang, M.M. Hannuksela, X. Wang] Reference picture marking for key pictures

This contribution proposes a separated marking process for the base representations and the enhanced representations of key pictures or access units. If sliding window reference picture marking is used, then a base representation has a higher priority than the corresponding enhanced presentation to be marked as “unused for reference”. If adaptive reference picture marking using memory management control operation (MMCO) commands is used, marking of a base representation as “unused for reference” is controlled by MMCO commands signaled in the slice headers of FGS slices in key access units, while marking of “unused for reference” for other reference pictures is controlled by MMCO commands signaled in the slice headers of the non-FGS slices. The purpose of the proposal is to save DPB memory usage. The saved memory can be used to reduce the memory requirements for SVC decoders (in different levels to be defined similarly to Annex A of H.264/AVC). The proposal allows that base representations are decoded from AVC compatible coded pictures.

Consensus on separate signaling of key_pic_flag, temporal_level, and reference picture marking for the base picture.

Open question: Should we use a nal_unit_type that (a) precedes or (b) succeeds the primary coded picture (c) or both)?

a) There can be only one, if lost together with the first NALU of the pcp, it is lost for all of them

b) There can be one NALU attached at the end to each VCL NALU of the pcp, (potentially would utilize a nal_unit_type from a set of 2 remaining nal_unit_type)

c) a) + within the FGS layer repeat the information

Revisit.

##### JVT-S041 [Q. Shen, H. Li, Y.-K. Wang, M.M. Hannuksela] Ref pic mgm for temporal scalability

This is an information document. It presents the results of the continuation study of reference picture management for temporal scalability, which was proposed in JVT-R055. The basic idea of JVT-R055 was to apply the sliding window decoded reference picture marking operation per each temporal level independently in order to reduce memory requirements for the decoded picture buffer. One counter argument to the proposal in JVT-R055 from the Bangkok meeting was that the same memory saving for reference picture storage could be achieved by marking all the reference pictures as “used for long-term reference”. However, in that case, the decoding processes of both temporal and spatial direct modes are affected. The study here mainly focuses on how coding efficiency is affected if all the reference pictures as “used for long-term reference”. The results show that coding efficiency has not been affected much.

JVT-S049 [Y. Chen, E. Frangois] Spatial enh layer ref pic list construction

Further results for JVT-R028 are given. JVT-R028 provided a new decoding process for reference picture lists construction for spatial enhancement layer. There is no further modification for the syntax and semantics. Cross Verification is done by Nokia.

Bit-rate savings up to 0.7%. Average: 0.2-0.3%.

##### JVT-S042 [H. Liu, H. Li, Y.-K. Wang] Verif Thomson proposal JVT-S049 on RPLR

Verified a sub-set of the results.

Removal of flag for weighted prediction as well. There shouldn't be multiple derivations of weighted prediction parameters and RLPR per slice.

Revisit.

##### JVT-S054 [M.H. Lee, H.W. Sun, D. Ichimura, Y. Honda, S.M. Shen] ROI slice SEI message

A mechanism for signaling of interactive ROI (IROI) information, targeting for easy and convenient extraction of IROI scalable video streams, is proposed in this contribution. The signaling mechanism is enabled via the proposed ROI slice SEI message, which consists of the ROI slice division information and ROI slice pointer information. The solution was previously proposed in JVT-R024, and further studied in AHG on high-level syntax. Modification was made to enable independent signaling at sequence-level and picture-level. The ROI slice division information is a sequence-level signaling while the ROI slice pointer information is a picture-level signaling. A showcase to demonstrate the usefulness of the ROI slice SEI message is also described in this contribution. The ROI slice SEI message has already been implemented in the software of contribution JVT-R023 to assist the ROI extraction process.

Adopted.

##### Discardable layers and adaptations

Also see notes re AHG report

##### JVT-S039 [Y. Guo, H. Liu, H. Li, Y.-K. Wang, M.M. Hannuksela] On discardable lower layer adaptations

This contribution proposes to use slice groups and data partitioning, to separate data used and unused for inter-layer prediction in different NAL units. Consequently, applications can omit transmission of data unused for inter-layer prediction for all layers below the operation point of receivers.

Proposes discardable adaptations through data partitioning using slice groups. Needs more study.

##### JVT-S043 [M. Mathew, K. Lee, W.J. Han] Multi layer quality layers

This proposal looks at discardable lower layer adaptations for improved multi layer performance from the extractor’s point of view. Quality Layers provide a way to truncate an SVC bitstream in an RD optimal fashion. However the current method of priority ID assignment is not multi layer optimal. It is proposed that a multi layer optimal quality layer assignment and a joint truncation at the extractor can improve the truncation performance of the top spatial layer at the cost of decreased quality of the embedded spatial layers. With the proposed method, up to 0.48 dB PSNR gain is observed at the top layer in the combined configuration.

It is possible to do a top layer only extraction even when the priority ID assignment is multi layer optimal. Thus the extractor can choose between a joint truncation or a top layer truncation based on the requirements of the client. This contribution then proposes changes required in the decoding process, for enabling the discarding of lower spatial layer’s FGS NALs that are used for inter layer prediction.

Presented.

Current design uses 'top-to-bottom' truncation rule

proposal suggests some discarding rule that differs from this by having a priority id rank that differs from the layering structure.

Adopted.

##### JVT-S052 [J.-Z. Xu] Verif Samsung JVT-S043 Multi quality layers

The purpose of this report is to verify the proposal JVT-S043 Multi Layer Quality Layers. As a verification task, coding performance check was carried out.

##### JVT-S051 [J.-Z. Xu] Frame-based selected inter-layer pred

Level-based Selective Inter-layer Prediction scheme was shown in JVT-R064 to improve coding performance when lower resolution is not required, while the performance loss is marginal when lower resolution is required. This document presents a frame-based scheme to further improve the performance. Frame-based selective inter-layer prediction can be modeled as a knapsack problem and can be solved using dynamic programming. Improved results are shown and corresponding syntax/semantics changes are discussed.

Signal "not used for inter-layer prediction" in the bitstream. Adopted contingent on overall solution.

Adopt Knappsack optimization into software.

Continue AHG.

## Non-VLC FGS Issues

##### JVT-S034 [B.K. Lee] Complexity reduction and improved FGS

In FGS encoding, JSVM5 use accumulated coefficients to separate significant and refinement pass. The probability of zero coefficients in significant and refinement pass is not optimal above second FGS layer. In this contribution, re-defined FGS coefficients partitioning is applied to encode FGS coefficients and it is reported that PSNR improvement can be up to 0.36 dB at highest bit-rate points of CIF sequences in three layer FGS test condition.

Focused on CABAC.

Complexity reduction questioned

Proponent is asked to make software available. Potentially adoption at this meeting. Revisit.

##### JVT-S019 [F. Le Leannec] Switching betw. FGS layers with reduced drift

[AAA]

Deals with tradeoff between coding efficiency and drift control. Switching FGS signal enables to systematically switch between the base layer to the enhancement layer for recovery from drift. In particular useful when the rate is increased at the client. Encoded res(n) data are rec(enh, n)-rec(base, n-1)-residual(base,n). Additional cost for switching data, would be transmitted in new slice type. Reduces the delay until full quality is reached from roughly 20-25 frames down to about 5-6 frames (dependent on amount of switching data that is transmitted), In principle extends the concept of SP slices into FGS (no enthusiastic support for the concept in the group).

Revisit later. Add to CE on switching.

##### JVT-S092 [S. Kamp, M. Wien] Local adaptation of AR-FGS leak factor

This contribution proposes a modification to FGS coding with adaptive reference (AR-FGS) for P-pictures as presented in JVT-Q039. Based on this method, the alpha leaky factor used for FGS prediction signal calculation in case of zero-coefficient base layer blocks is further adapted depending on the macroblock partition type of the current block. This enables an improved locally adaptive selection between prediction quality and drift reduction and provides a performance gain for closed-loop single FGS layer coding of up to 0.15 dB using the testing conditions in JVT-P307.

Create AHG on AR-FGS optimization.

# JVT SVC non-normative modifications

## Encoder optimization

##### JVT-S044 [I. Amonou, N. Cammas, S. Kervadec, S. Pateux] Enh SNR SVC for layered CGS w/ quality layers

In this proposal, a technique to allow for Medium Grain Scalability in layered (CGS) scenario is presented. It is first proposed to code SNR refinements using a CGS coding technique with NAL units having a quality level greater than 0, instead of using classically progressive refinement slices. Then the concept of quality layers is extended to the CGS coding case: quality layers are assigned to these NAL units. This technique first allows to increase the number of rate points that can be reached with a CGS-like bitstream (an MGS granularity of 10% can be achieved), and second it allows an optimal reconstruction of these rate points.

##### JVT-S056 [L. Zhu, Y. Chen] Rate control for JSVM

## Software

##### JVT-S070 [H. Schwarz, M. Wien, J. Vieron] Proposal for a JSVM software manual

In order to improve the support for using the JSVM software for both JVT experts as well as other interested parties it is proposed that the JVT maintains a JSVM software manual in addition to the JSVM software and the JD/JSVM text description. It is further requested to set up a rule that each party that modifies that JSVM software is also responsible for updating the JSVM software manual. A proposed draft for the JSVM software manual, which corresponds to the JSVM software version 4.9, is provided in this contribution.

# JVT 4:4:4 coding normative modifications

## CE 5 & related docs: 4:4:4 coding

Adhoc report: Miscommunication happened about usage of test sequences. But sufficient overlap to make comparison. Critical issue: Subjective tests.  Identified candidate test sites. Not realistic in near future because number of people available might be too small, and not clear whether differences would be visible. Therefore rely on objective criteria.

##### JVT-S014 [S. Sekiguchi, Y. Isu, Y. Yamada, K. Asai, T. Murakami] CE5: Results of Core Experiment on 4:4:4 Coding

[AA] This contribution provides experimental results for CE5 (4:4:4 Coding) and proposed modifications to the latest 4:4:4 PDAM method based on the results. A verification on the performance of two variations of color-channel independent coding tool (slice-level and MB-adaptive) using common CE5 software has been conducted. Compared with the current PDAM method, it has been observed that the independent coding tool achieves more than 8% bitrate reduction at higher bitrates. Also, benefits of the independent coding tool from the viewpoint of practical implementation and future use of 4:4:4 profile are pointed out.

Claim is made that slice adaptive version gives improvement in coding efficiency at higher bit rates (average of roughly 2.6% reduction with at the same time roughly 0.15 dB PSNR improvement), but less at lower bit rates. As this would not necessarily justifies definition of a separate profile, but main question would be if it is needed by industry due to implementation reasons. Could be simply realized by encoding 3 different pictures monochrome (simple to implement; mux at picture level, i.e. 3 pictures in one AU). Macroblock adaptive method would give similar compression advantage both at higher and lower bit rates, but seems not useful from viewpoint of implementation (needs additional syntax elements to signal separate or joint MB modes).

In any case, definition of a new profile would be necessary. Concern is raised whether a separate profile for the independent color approach would be useful.

##### JVT-S073 [D. Marpe, H. Kirchhoffer, T. Wiegand] CE5: Color transform issues and results

JVT-S075 [M. Wien] CE5: Verif of JVT-Sqqq HHI 4:4:4 proposal

##### JVT-S084 [W.-S. Kim, D. Birinov, D. Cho, H. M. Kim] CE5: Report of CE on 4:4:4 Coding

In this document, the experimental results of CE5: 4:4:4 coding is presented and the performances of 4:4:4 coding proposals are analyzed.

Presents results on the method of residual component prediction (predicting delta-r and delta-b from delta-g), with spatial filtering of the prediction signal. Show gain, and also compare with different other proposals. Found that method of JVT-S073 has better performance with lower complexity, and recommend to adopt that one.

##### JVT-S074 [H. Kirchhoffer, D. Marpe, T. Wiegand] Verif of JVT-Sqqq on RCP

## Other subjects

##### JVT-S030 [S. Wittman, T. Wedi] Post-filter SEI for 4:4:4 coding

In this contribution a post-filter SEI message for 4:4:4 coding is proposed. The SEI message contains filter coefficients of an FIR post-filter that are estimated on encoder side. Thus, the un-coded original signal can be used to optimize the post-filter. In the experimental results the calculation of the filter coefficients is based on a Wiener filter approach. Therefore, the post-filter is not a low-pass filter designed to reduce blocking artifacts, it is rather a filter to remove quantization noise and thus to reconstruct the un-coded signal as much as possible. At a picture quality of 45 dB gains up to 0.5 dB are reported to be achieved, which corresponds to bit-rate savings of 9%.

Presents various results (de-blocking filter on/off, Wiener post filter on/off). Typically, results with post-filter are better even if AVC deblocking is off. Filter is non-separable 7x7, optimized separately on a frame-by-frame basis for each color component. Optimization takes into account original and respective decoded result (such that different filters are computed depending on de-blocking filter on and off). Question was raised whether it would be appropriate to transmit the filter parameters differentially with reference to some default. Another question is raised whether instead of Wiener filtering it would be appropriate to allow other methods of post processing.

Looks very promising, further study with different configurations (size of filter etc.).

##### JVT-S085 [W.-S. Kim, D. Birinov, D. Cho, H. M. Kim] SEI for Indep Color Component Coding

In this document, independent color component coding method is described, where each color component is separately coded using luma coding method in a monochrome chroma format. Since this method results in an independency among color components, it makes it easy to apply a parallel encoding of each color component. Without any normative change, an additional SEI message is proposed to support this method.

Supports concept similar to S-014 discusses two different concepts: Multiplex at slice level or multiplex at picture level, could be realized without normative changes by defining an SEI message.

Would require definition of a new profile as well as any other proposal, but the solution seems to be less general here.

##### JVT-S087 [A. Segall, L. Kerofsky and S. Lei] Tone mapping SEI

The additional bit-depth enabled within the 4:4:4 coding effort allows for smaller quantization intervals and increased PSNR values.  While this fidelity will benefit many applications, it is not the only use for higher bit-depth coding.  Capturing an expanded dynamic range is also a reasonable use for the higher bit-depths.  To better support the display of higher bit-depth content, an SEI message is proposed here to describe a suggestive tone mapping curve.  This tone mapping curve maps higher bit-depth decoded pictures from the bit-stream to a lower bit-depth display.

Purpose: High bit-depth content on low bit-depth display. For example if the extended bit-depth is mainly used to expand the range of illumination, clipping would be more appropriate than cutting off LSBs. Variants proposed are clipping, sigmoid, lookup-table and piecewise-linear.

Questions raised: Relationships with camera characteristics and display characteristics other than bit depth (also ambient/lighting conditions)?

In principle looks like a good idea to be adopted, but may need some more careful thinking to make it generic enough and not close the door for possible extensions. For example the 3 bits to signal the number of models may be insufficient. Consider for inclusion into JD (?) this time or next time.

# JVT 4:4:4 coding non-normative modifications

##### JVT-S081 [H. Yu] CE5: Test report

This document reports the problems that we found when running the latest version of the CE5 reference software.  The decoded pictures did not match the reconstructed pictures for a number of test conditions.

# JVT Remaining normative coding modifications

##### JVT-S078 [B. Haskell] Interop for interlace in AVC/H.264

Two items for the AVC/H.264 errata relating to interoperability of interlaced coded video are reported.

##### JVT-S082 [A. Luthra, S. Narasimhan, B. Eifrig] Progressive/Interlace display inference

Some (minor but important) inconsistent interpretation of syntax are reported. It would be a good idea to develop a more homogeneous and broader common understanding of some of our syntax elements. This issue is centered around hinting a receiver for the display purpose whether a video frame is scanned Progressively or Interlaced, and if it is Interlaced scanned then in what order fields should be displayed.

##### JVT-S083 [S. Narasimhan, B. Eifrig, A. Luthra] no_output_of_prior_pics_flag & splicing

An issue that has surfaced in the use of AVC standard for applications that require ‘seamless splicing’ such as ad-insertion is addressed.

# JVT remaining non-normative and SEI/VUI aspects

## Encoder optimization techniques

##### JVT-S028 [J. Zhang, X. Yi, N. Ling, W. Shang] Context Adaptive Lagrange Multiplier (CALM) for Motion Estimation in JM

Motion estimation and motion compensation play a critical role in hybrid video coding. In the current JM software, the Lagrange multiplier for motion estimation is determined by a function of quantization parameter of the DCT coefficients. However many experiments have shown that the cost function based on the current Lagrange multiplier may not always yield optimal rate-distortion performance. It is often computationally costly to try different Lagrange multipliers to achieve optimal coding efficiency. In this proposal a method — Context Adaptive Lagrange Multiplier (CALM) for rate-constrained motion estimation is presented. Instead of calculating Lagrange multiplier based only on quantization parameter the proposed method additionally adjusts Lagrange multiplier for each block based on its context, the Lagrange cost of its neighboring blocks or the upper layer blocks. Simulation results demonstrate that the new algorithm can achieve Y-PSNR gain of up to 1.0 dB calculated by BD method with negligible extra computational cost.

## Improved Error resilience

##### JVT-S025 [S. Rane, P. Baccichet and B. Girod] Lossy Error Prot. based on Red. Slices & FMO

This proposal describes a scheme for Systematic Lossy Error Protection (SLEP) of an H.264/MPEG4-AVC compressed bit stream based on redundant slices and flexible macroblock ordering (FMO). At the encoder, parity symbols generated from coarsely quantized redundant slices are transmitted at a low bit rate, as error protection information. The redundant slices themselves are not transmitted. At the decoder, these parity symbols enable recovery of the redundant slices that are displayed in lieu of the lost portions of the primary coded picture (PCP). By using coarse quantization in the redundant slices, SLEP allows the video quality to degrade gracefully with worsening channel conditions, thus mitigating the “cliff” effect encountered in traditional FEC schemes. This document specifies the syntax and non-normative codec operations required to implement the SLEP scheme.

Channel coding of concealment side information combined with simultaneous processing at encoder and decoder.

Comparison to uncoded redundant slices?

Overhead due to slice introduction relative to 1 slice per picture?

SEI message provides helper information.

Clarification on system interrelation.

Results for CABAC? Loss due to multiple slices. Results requested. Set-up core experiment.

##### JVT-S046 [P. Baccichet and A. Chimienti] Err resil by redundant descriptions

This document proposes and evaluates the usage of coarsely quantized redundant slices to improve the error resilience of an H.264/MPEG4-AVC bit stream. A simple region-of-interest (ROI) determination scheme is also reported to improve performance for static sequences. Simulation results are provided for both static and dynamic sequences at CIF resolution over a simulated channel affected by random losses.

## Reference Software

##### JVT-S079 [A.M. Tourapis, K. Suehring, G.J. Sullivan] Updates for reference software manual

## Joint Model (JM) text update

# Withdrawn JVT documents

JVT-S050, JVT-S065

# Updating of JVT group documents

# JVT internal operating rules

The following clarifications/adjustments of JVT operating rules have been approved.

All submissions must be made in JVT-P105.zip format with the word docs, excel sheets and other information being in the zip container. A revision of the doc shall be made by adding the revised doc to the zip container. (The document must also contain an abstract and be accompanied with an e-mail notification containing title, authors and abstract (identical to the one in the doc) which is no longer than 200 words and is written in 3rd person in a manner that does not express endorsement of the content of the document.)

Independent verification (necessary for adoption of a proposal) is provided either through

- independent implementation by 1 or more company different than the proponent based on the textual description (after adoption, both decoder source code versions must be made publicly available and one encoder version)

- providing source code to all CE participants prior to the meeting (CEs can only be joined at the meeting, when the CE is created. CEs are created at each meeting and last until the next meeting.)

For every SEI message and every syntax element that are currently in the SVC draft, a showcase has to be provided in order to retain it in the JSVM/WD. If such a showcase is not provided at the next meeting for an SEI message or parts of it, the SEI message or the respective parts will be removed from the JSVM/WD. The source code and executables for the showcase must be made available.

A first CE description must be available at the last day of the meeting. Changes of the CE description are only allowed until 1 month prior to the next meeting. These changes must be of evolutionary characteristic relative to the input documents on which the CE is based and must be agreed by those who contributed the respective input document(s) or be added as an option.

# List of Adoptions

Person listed in bracket is responsible for provisioning of text and software integration.

## Normative SVC adoptions

## Non-Normative SVC adoptions

## Non-Normative non-SVC adoptions

# List of AHGs established

##### AHG on Project management and errata [Chairs: G. J. Sullivan, J.-R. Ohm, A. Luthra, T. Wiegand]

Copy mandate for AHG from previous description.

##### AHG on JM text and reference software [Chairs: T. Wiegand, K. Suehring, A. Tourapis, K. P. Lim]

Copy mandate for AHG from previous description.

##### AHG on Bitstreams & conformance [Chairs: T. Suzuki]

Copy mandate for AHG from previous description.

##### AHG on JSVM S/W and new functionality integration [Chair: L. Bivolarski]

Copy mandate for AHG from previous description.

##### AHG on JSVM & JD text [Chairs: J. Reichel, H. Schwarz, M. Wien]

Copy mandate for AHG from previous description.

##### AHG on Spatial scalability resampling [Chair: S. Sun, A. Segall, J. Reichel]

Copy mandate for AHG from previous description

- Subjective tests

##### AHG on High-level syntax, error/loss resilience test conditions & applications [Chair: Y.-K. Wang, S. Pateux, M.-H. Lee]

Copy mandate for AHG from previous description. ROI SEI message, Scalability info SEI, other subjects: see meeting report.

##### AHG on Coding eff & JSVM coding efficiency testing conditions [Chairs: M. Wien, H. Schwarz]

Copy mandate for AHG from previous description.

##### AHG on Study of 4:4:4 functionality [Chair: T. Suzuki]

Copy mandate for AHG from previous description

- subjective tests

##### AHG on SVC interlaced coding [Chair: J. Vieron, H. Schwarz]

Copy mandate for AHG from previous description.

##### AHG on improved FGS coding [Chair: N. Cammas, Y. Bao]

Study complexity of FGS coding, suggest speed-up methds, study generalized FGS and alternatives to the concept.

##### AHG on discardable lower layer adaptations and multi-layer RD optimization [Chairs: M. Mathew, J. Li, H. Schwarz]

Study modified encoding method and extractor behaivior.

# Resolutions of the meeting

[splice in from separate document]

# SW integration plan

# Attendance