| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 |  |  |  | Document: JCT3V-A_Notes_d4 |
| --- | --- | --- | --- | --- |
| Title: | Meeting report of the first meeting of the Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V), Stockholm, SE, 16-20 July 2012 |  |  |  |
| Status: | Report Document from Chairs of JCT-3V |  |  |  |
| Purpose: | Report |  |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communications Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |  |
| Source: | Chairs |  |  |  |

_____________________________

- Summary

The Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its first meeting during 16-20 July 2012 at the City Conference Centre (CCC), a.k.a. Norra Latin, in Stockholm, SE. The JCT-3V meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.13 of this document.

The JCT-VC meeting sessions began at approximately 1515 hours on Monday 16 July 2012. Meeting sessions were held on all days until the meeting was closed at approximately XXXX hours on Friday 20 July. Approximately XXX people attended the JCT-3V meeting, and approximately 160 input documents were discussed. The meeting took place in a co-located fashion with a meeting of ISO/IEC WG 11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-3V meeting activities consisted of work on 3D extensions of the Advanced Video Coding (AVC) and the High Efficiency Video Coding (HEVC) standards.

The primary goals of the meeting were to start the collaborative work on the topics of 3D video coding extensions of the aforementioned standards, review the work that had previously been done within the AHG on 3DV coding of MPEG, in particular review the results from the interim Core Experiments (CE); review technical input documents; produce the specification Draft of the MVC extension by depth maps (which was issued as an ISO/IEC Draft amendment document); establish the 3rd test model of the AVC 3D extension framework (3D-AVC), draft specification and associated software; establish the 1st version of draft text of the Multiview HEVC extension (MV-HEVC); establish the 1st test model of the HEVC 3D extension framework (3D-HEVC) and associated software; and plan a new set of Core Experiments (CEs) for further investigation of proposed technology.

The JCT-3V produced X particularly important output documents from the meeting:

...

... . Moreover, plans were established to conduct 6 future CEs in the interim period until the next meeting.

For the organization and planning of its future work, the JCT-3V established 11 "Ad Hoc Groups" (AHGs) to progress the work on particular subject areas. The next four JCT-3V meetings are planned for 13–19 Oct 2012 under WG 11 auspices in Shanghai, CN, 17–23 January 2013 under ITU-T auspices in Geneva, CH, 20-26 April 2013 under WG 11 auspices in Incheon, KR, and 27 July – 02 Aug 2013 under WG 11 auspices in Vienna, AT.

The document distribution site http://phenix.it-sudparis.eu/jct2/  was used for distribution of all documents.

The reflector to be used for discussions by the JCT-3V and all of its AHGs is jct-3v@lists.rwth-aachen.de.

- Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-3V are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its first meeting during 16-20 July 2012 at the City Conference Centre (CCC), a.k.a. Norra Latin in Stockholm, SE. The JCT-3V meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

## Meeting logistics

The JCT-3V meeting sessions began at approximately 1515 hours on Monday 16 July 2012. Meeting sessions were held on all days until the meeting was closed at approximately XX00 hours on Friday 20 July. Approximately XXX people attended the JCT-3V meeting, and approximately 160 input documents were discussed. The meeting took place in a co-located fashion with a meeting of ISO/IEC WG 11 – one of the two parent bodies of the JCT-3V. The subject matter of the JCT-3V meeting activities consisted of work on 3D extensions of the Advanced Video Coding (AVC) and the High Efficiency Video Coding (HEVC) standards.

Information regarding preparation and logistics arrangements for the meeting had been provided via the email reflectors mpeg-ftv@lists.rwth-aachen.de, vceg-experts@yahoogroups.com and jct-vc@lists.rwth-aachen.de.

## Primary goals

The primary goals of the meeting were to start the collaborative work on the topics of 3D video coding extensions of the aforementioned standards, review the work that had previously been done within the AHG on 3DV coding of MPEG, in particular review the results from the interim Core Experiments (CE); review technical input documents; produce the specification Draft of the MVC extension by depth maps (which was issued as an ISO/IEC Draft amendment document); establish the Xth test model of the HEVC 3D extension framework and associated software; establish the Xth test model of the AVC depth extension framework, draft specification and associated software; and plan a new set of Core Experiments (CEs) for further investigation of proposed technology. (update as above)

## Documents and document handling considerations

### General

The documents of the JCT-3V meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct2/ .

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

Document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (rather than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a bug in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding … simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of contexts, number of context-coded bins, etc.) … "Decision (Compl.):"

This meeting report is based primarily on notes taken by the chairs and projected (if possible) for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp (http://wftp3.itu.int/av-arch/jct3v-site/) during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much discussion of the contributions and discussions as is feasible in the interest of aiding study, although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Tuesday, 10 July 2012.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Wednesday 11 July 2012 were considered "officially late".

Most documents in this category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All contribution documents with registration numbers JCT2-A0156 to JCT2-A0XXX were registered after the "officially late" deadline (and therefore were also uploaded late). JCT2-A0160 is an exception, where the number had to be re-assigned after before-deadline registration; however, it was uploaded late. Further, some documents in the "A0156+" range include break-out activity reports that were generated during the meeting and are therefore considered report documents rather than late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with this issue was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following other technical proposal contributions were registered in time but were uploaded late:

- JCT2-A0XXX (a technical proposal) [uploaded XX-XX]

- ...

The following other documents not proposing normative technical content were registered in time but uploaded late:

- JCT2-A0XXX (a contribution ...)

- ...

The following cross-verification reports were uploaded late: JCT2-A0XXX, ... .

The following document registrations were later cancelled or otherwise never provided or never discussed due to lack of availability or registration errors: JCT2-A0XXX, ... .

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were rejected in the document management system, as has been agreed since the third meeting.

The initial uploads of the following contribution documents were rejected as "placeholders" without any significant content and were not corrected until after the upload deadline:

- JCT2-A0XXX (a contribution of ... , corrected ...)

- ...

A few contributions had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in all cases (to the extent of the awareness of the chairs).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Measures to facilitate the consideration of contributions

It was agreed that, due to the continuingly high workload for this meeting, the group would try to rely more extensively on summary CE reports. For other contributions, it was agreed that generally presentations should not exceed 5 minutes to achieve a basic understanding of a proposal – with further review only if requested by the group. For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by the cross-check contributor.

Initial version of meeting notes integrated discussion of MPEG AHG

If issues are requested to be considered again, this shall be granted.

### Outputs of the preceding meeting

(delete section or refer to 3DV AHG work – move text to somewhere else when appropriate e.g. for CE procedures)

The group was asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chair asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

It was remarked that, in regard to software development efforts – for cases where "code cleanup" is a goal as well as integration of some intentional functional modification, it was emphasized that these two efforts should be conducted in separate integrations, so that it is possible to understand what is happening and to inspect the intentional functional modifications.

The need for establishing good communication with the software coordinators was also emphasized.

At previous meetings, it has previously been remarked that in some cases the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Also, there should be time to study combinations of different adopted tools with more detail prior to adoption.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology.

Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

It was noted that sometimes there is a problem of needing to look up other referenced documents, sometimes through multiple levels of linked references, to understand what technology is being discussed in a contribution – and that this often seems to happen with CE documents. It was emphasized that we need to have some reasonably understandable description, within a document, of what it is talking about.

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work.

## Attendance

The list of participants in the JCT-3V meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/ SC 29/ WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on 3D video coding projects guidance

- Consideration of 3D video coding technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-XX and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/ SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

Current situation :

3D-HTM: Same as HM (deviating from HM6)

3D-ATM: Same as JM17 in the parts that originated from AVC, unknown what the status of the newly added parts (enhanced texture coding, depth coding) are. Revisit: Clarify with D. Rusanovskyy.

It was noted that it is our understanding according to the practices of the parent bodies to make reference software available under copyright license header language which is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as e.g. recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/ SC 29/ WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software header is currently used in the process of designing the new HEVC standard and for evaluating proposals for technology to be included in this design. Additionally, after development of the coding technology, the software will be published by ITU-T and ISO/IEC as an example implementation of the 3D video standard(s) and for use as the basis of products to promote adoption of the technology. This is likely to require further communication with and between the parent organizations.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct2/. Furthermore, the site http://ftp3.itu.int/av-arch/jct2-site was used for distribution of the contribution document template and circulation of drafts of this meeting report.

Communication of JCT-3V is performed via the list jct-3v@lists.rwth-aachen.de. (add subscription procedure). For a matter of simplicity, the membership of the former MPEG AHG (mpeg-ftv@lists.rwth-aachen.de) is transferred to the reflector without subscribing again.

It was emphasized that reflector subscriptions and email sent to the reflector must use their real names when subscribing and sending messages and must respond to inquiries regarding their type of interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Some terminology used in this report is explained below (update):

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- AIS: Adaptive intra smoothing.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning.

- AMVP: Adaptive motion vector prediction.

- AMVR: Adaptive motion vector resolution.

- APS: Adaptation parameter set.

- ARC: Adaptive resolution coding.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BoG: Break-out group.

- BR: Bit rate.

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CD: Committee draft – the first formal ballot stage of the approval process in ISO/IEC.

- CE: Core experiment – a coordinated experiment conducted after the 3rd or subsequent JCT-VC meeting and approved to be considered a CE by the group.

- Consent: A step taken in ITU-T to formally consider a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC.

- DF: Deblocking filter.

- DT: Decoding time.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- ET: Encoding time.

- GPB: Generalized P/B – a not-particularly-well-chosen name for B pictures in which the two reference picture lists are identical.

- HE: High efficiency – a set of coding capabilities designed for enhanced compression performance (contrast with LC). Often loosely associated with RA.

- HEVC: High Efficiency Video Coding – the video coding standardization initiative under way in the JCT-VC.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBDI: Internal bit-depth increase – a technique by which lower bit depth (8 bits per sample) source video is encoded using higher bit depth signal processing, ordinarily including higher bit depth reference picture storage (ordinarily 12 bits per sample).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC).

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LC: Low complexity – a set of coding capabilities designed for reduced implementation complexity (contrast with HE). Often loosely associated with LD.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Often loosely associated with LC. Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- MANE: Media-aware network elements.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/ SC 29, one of the two parent bodies of the JCT-VC).

- MV: Motion vector.

- NAL: Network abstraction layer (as in AVC).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUT: NAL unit type (as in AVC).

- OBMC: Overlapped block motion compensation.

- PCP: Parallelization of context processing.

- PIPE: Probability interval partitioning entropy coding (roughly synonymous with V2V for most discussion purposes, although the term PIPE tends to be more closely associated with proposals from Fraunhofer HHI while the term V2V tends to be more closely associated with proposals from RIM).

- POC: Picture order count.

- PPS: Picture parameter set (as in AVC).

- QM: Quantization matrix (as in AVC).

- QP: Quantization parameter (as in AVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD). Often loosely associated with HE.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SDIP: Short-distance intra prediction.

- SEI: Supplemental enhancement information (as in AVC).

- SD: Slice data; alternatively, standard-definition.

- SH: Slice header.

- SPS: Sequence parameter set (as in AVC).

- TE: Tool Experiment – a coordinated experiment conducted between the 1st and 2nd or 2nd and 3rd JCT-VC meeting.

- TM: Test Model – a video coding design containing selected coding tools; as contrasted with the TMuC, see HM.

- TMuC: Test Model under Consideration – a video coding design containing selected proposed coding tools that are under study by the JCT-VC for potential inclusion in the HEVC standard.

- Unit types:

  - CTB: code tree block (synonymous with LCU or TB).

  - CU: coding unit.

  - LCU: (formerly LCTU) largest coding unit (synonymous with CTB or TB).

  - PU: prediction unit, with four shape possibilities.

    - 2Nx2N: having the full width and height of the CU.

    - 2NxN: having two areas that each have the full width and half the height of the CU.

    - Nx2N: having two areas that each have half the width and the full height of the CU.

    - NxN: having four areas that each have half the width and half the height of the CU.

  - TB: tree block (synonymous with LCU – LCU seems preferred).

  - TU: transform unit.

- V2V: variable-length to variable-length prefix coding (roughly synonymous with PIPE for most discussion purposes, although the term PIPE tends to be more closely associated with proposals from Fraunhofer HHI while the term V2V tends to be more closely associated with proposals from RIM).

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- WD: Working draft – the draft HEVC standard corresponding to the HM.

- WG: Working group (usually used in reference to WG 11, a.k.a. MPEG).

## Liaison activity

The JCT-3V did not send or receive formal liaison communications at this meeting.

## Opening remarks

No particular non-routine opening remarks were recorded.

## Tentative List of Output Documents from this meeting

- MVC extension with depth maps draft text (MPEG 14496-10:2012 DAM2) + DoC for MPEG (Breakout: Teruhiko Suzuki – first review Tue afternoon)

- AVC extension with depth test model and draft text

- HEVC 3D extension test model and draft text

- White paper (D. Rusanovskyy w. other volunteers)

- Core Experiments

- Common Test Conditions

- Meeting Report

Thoughts about Nicknaming (revisit):

MVC+D (= 14496-10:2012 Amd.2);

MV-HEVC (=upcoming HEVC PDAM for stereo/multiview);

3D-HTM (test model+draft text of future depth extensions);

3D-ATM (test model)3D-AVC (draft text)

## Scheduling of discussions

AHG reports (Monday afternoon)

Initial review of remaining inputs not yet done in AHG (Mon from 18:30)

Tue 8:00-10:00 Revisits from previous reviews of AHG, starting from the issues relating to MVC depth extension.

Tue 10:00-  continue initial review of docs

HL syntax issues: Joint discussion with JCT-VC experts Tuesday 2:30 in JCT-VC

A-0115 (also submitted as JCT-VC doc and partially discussed there)

A-0021

A-0099 (also submitted as JCT-VC doc but not discussed there)

A-0121 (also submitted as JCT-VC doc and partially discussed there)

A-0117, A-0118

Tue 14:00 Continue revisits (excluding any high-level issues)

Tue 17:00 MVC with depth extension draft and DoC

Wed after MPEG plenary: HEVC standardization and suggested WD text,

Wed afternoon (meet at Musiksalen 15:00): BoG report, Continuation on CE revisits, establish CE plans.

Thu: Start from 9:00

- CE3, CE 8

- Other remaining revisits (HL syntax, non-CE)

- New CEs, AHGs

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows.

- AHG reports (section 2)

- CE1.a: View synthesis and inter-view prediction (Coordinator: D. Tian/Mitsubishi) (section 4.1)

- CE1.h: View synthesis prediction (Coordinator: F. Jäger/RWTH Aachen) (section 4.2)

- CE2: Depth representation and coding (Coordinator: O. Stankiewicz/Poznan Univ.) (section 4.3)

- CE3: Depth map resampling and filtering (Coordinator: D. Rusanovskyy/Nokia) (section 4.4)

- CE5.a: Motion/mode parameter prediction (Coordinator: Y. Chang/MediaTek) (section 4.5)

- CE5.h: Motion/mode parameter prediction (Coordinator: S. Yea/LG) (section 4.6)

- CE6.a: Depth intra prediction (Coordinator: M. Hannuksela/Nokia) (section 4.7)

- CE6.h: Depth intra prediction (Coordinator: P. Merkle/FhG-HHI) (section 4.8)

- CE7: Global depth and view prediction (T. Senoh/NICT) (section 4.9)

- CE8: RD Optimization (Coordinator: S. Yea/LG) (section 4.10)

- 3DV Standards Development (section 5.1)

- Clarifications and Bug Fix Issues (section 5.2)

- Test Model Settings (section 5.3)

- Test Model Coding Performance (section 5.4)

- Source Video Test Material (section 5.5)

- High-Level Syntax (section 5.6)

- Other Technical Contributions (section 5.7)

- Non-normative Contributions (section 5.8)

NOTE – The number of contributions noted in each category, as shown in parenthesis above, may not be 100% precise.

# AHG reports

The activities of two ad hoc groups that had been established by the WG11 parent body are discussed in this section.

JCT2-A0001 Ad hoc on 3D Video Coding [Karsten Müller, Anthony Vetro]

JCT2-A0006 Ad hoc on 3D Video Coding High Level Syntax [Ying Chen, Thomas Rusert, Heiko Schwarz] [miss]

# Project development, status, and guidance

## Communication by parent bodies

A joint meeting of the parent bodies with JCT-3V was held prior to the opening of the meeting, concerning the current and possible future work items of JCT-3V, as well as coordination of stereo and 3D activities between the different working groups.

### Frame Compatible Stereo Formats

MPEG-2 Frame-Compatible Signalling for Quincunx Packing

- adding an additional frame-packing arrangement

Responsible: MPEG Video

Timeline: FDAM 12/10

HEVC Frame-Compatible Signalling

- frame-compatible SEI messages in version 1 (similar to those in AVC)

Responsible: JCT-VC

Timeline: DIS 12/07 FDIS 13/01

### AVC Multiview Extension with Depth

Enables depth based 3D enhancements while maintaining MVC stereo compatibility.  The method is invoking an independent second stream for the representation of a depth map as if it were monochrome video data, as well as high-level syntax signaling of the necessary information to express the interpretation of the depth data and its association with the video data. Macroblock-level changes to the AVC or MVC syntax, semantics and decoding processes are not considered.

Responsible: JCT-3V

Timeline: DAM 12/07  FDAM 13/01

### Multiview Extension of HEVC

A subset of the HEVC 3D video coding framework (HTM) includes a simple multi-view extension MV-HEVC, with a concept similar to AVC’s MVC (backwards compatibility for monoscopic decoding). This should already include simple carriage of depth maps similar to  2.

Technical changes at low level (sub-CTB)? Should be justified by market needs (not necessarily a legacy issue, but issues such as complexity, quick availability should be considered).

Responsible: JCT-3V (requires close coordination with scalable extension developed in JCT-VC but should be finished earlier than that)

Timeline: PDAM 13/01 DAM 13/07 FDAM 14/01

### Scalable Stereo/Multiview

- AVC spatial scalable extension of frame-compatible formats (“MFC”): CfP with responses potentially by January 2013 is currently prepared by MPEG Requirements (Q: Would this include hybrid solutions? Joint Call?)

- For HEVC, scalable enhancements of both frame-compatible formats under 1. and MV-HEVC under 3. would be likewise useful and could be prepared in a clean fashion in coordination with the scalable extension.

Responsible: JCT-3V

Timeline: TBD, for HEVC it depends on the timeline for the scalable HEVC extension. Tentative: PDAM 13/10, DAM 14/04, FDAM  15/01

### Depth-based 3D Video Formats

Possibilities to further increase coding efficiency of video-plus-depth formats are currently investigated both for AVC and HEVC environments:

- Development of more specific depth coding tools

- Combined video/depth coding

- AVC compatible video-plus-depth extension

Only monoscopic AVC compatibility. Current “ATM” outperforms the MVC-compatible approach by approximately 15% bit rate reduction for the entire (video-plus-depth) information. Syntax and decoding process for non-base texture views and depth information changed at the MB level. 25% BR reduction would be desirable.

Responsible: JCT-3V

Timeline (N12434):  PDAM 12/10 DAM 13/01  FDAM 13/07

Note: Timeline too aggressive?

- HEVC for stereo and 3D

Base view fully compatible to HEVC monoscopic, while for the coding of dependent views and depth maps additional tools are applied. Similar expectations w.r.t. bitrate savings as under a.

Responsible: JCT-3V

Timeline:  PDAM 14/01?  DAM 14/07  FDAM 15/01

### Hybrid solutions

In addition to HEVC-only solutions for 3D video coding, also “hybrid” solutions could be considered (e.g. AVC base, HEVC enhancement). TBD in alignment with similar concepts in scalable extensions.

### 3D Video Formats beyond depth

Full 3D scene representation, FTV, combined 3D video & graphics etc.

Timeline: 2015+

## Project planning

# Core experiments

## CE1.a: View synthesis and inter-view prediction

### Summary

- JCT2-A0011/m25805 CE1.a summary report: View synthesis and inter-view prediction [Dong Tian (MERL)]

The tools investigated in this CE use the decoded texture and depth information to compute a prediction signal for a dependent view. The following objectives were established.

- Study block-based view synthesis

- Study benefit of depth up-sampling in combination with VSP

- Inter-view and view synthesis prediction with adaptive luminance compensation

- Evaluate benefit of not transmitting MV info and building context

- Evaluate benefit of sub-MB skip/direct

- Evaluate benefit of reference picture refinement

- Further evaluation of variable precision synthesis

The following CE contributions have been submitted:

JCT2-A0015 (Samsung)

JCT2-A0022 (NTT)

JCT2-A0035 (Samsung)

JCT2-A0055 (Zhejiang Univ)

JCT2-A0103 (Qualcomm)

JCT2-A0107 (Nokia)

JCT2-A0158 (Samsung/NTT)

Several CE related contributions have also been submitted:

JCT2-A0023 (NTT)

JCT2-A0036 (Samsung)

JCT2-A0050 (MediaTek)

JCT2-A0056 (Zhejiang Univ)

A summary of the above contributions relative to each objective of the CE can be found in the report.

### CE Contributions

- JCT2-A0015/m25851 3D-CE1.a results on inter-view coding with adaptive luminance compensation by Samsung [Alexey Fartukov, Igor Kovliga, Mikhail Mishurovskiy, Jaejoon Lee, Konstantin Dubkov (Samsung)]

This contribution proposes an auxiliary mode to account for luminance differences between views, which is enabled with a flag in the sequence level and signalled with a 1-bit flag after mb_type at the macroblock level. It is proposed that this auxiliary mode is applied for view synthesis prediction only, and not for inter-view prediction based on the simulation results observed. The compensation method is applied to texture only and has no impact on depth coding. The proponents report a 1.03% bit rate reduction on the coded data and 0.59% bit rate reduction on the synthesized data with 102.98% encoded time, and 103.43% decoding time.

- JCT2-A0024/m26095 3D-CE1.a: Cross-check report of JCT2-A0015 on inter-view coding with adaptive luminance compensation [Shinya Shimizu (NTT)]

- JCT2-A0022/m26093 CE1.a results on adaptive illumination compensation [Shinya Shimizu, Shiori Sugimoto, Hideaki Kimata (NTT)]

This contribution presents the simulation results of the adaptive illumination compensation technique that was previously proposed in m22616 and is related to JCT-A0015 (Samsung). In the proposed method, the parameters for the illumination compensation are derived locally at the decoder by minimizing the differences between the decoded picture and the prediction picture. Two compensation models, a linear function and a 2D filter with offset, are evaluated in the experiments. For the texture coding, average BD-rate improvements of 0.25% and 0.04% were obtained with the linear function and 2D filter with offset, respectively. The maximum gain was 0.98% for the Kendo sequence with the linear function. The primary differences from JCT2-A0015 are in the signalling, parameter estimation, motion estimation and templates for sub-MBs.

A0015 and A0022 are both using illumination compensation. This could hypothetically also be solved by weighted prediction. Further study of both proposals in CE on illumination compensation (new), where a comparison with WP is included.

Both proposals have similar gain (about 1% BR reduction) but also add complexity (both at encoder and decoder.

- JCT2-A0152/m26172 3D-CE1.a: Cross check on NTT's adaptive illumination compensation (JCT2-A0022) [Alexey Fartukov, Igor Kovliga, Mikhail Mishurovskiy, Jaejoon Lee, Konstantin Dubkov (Samsung)]

- JCT2-A0035/m25879 3D-CE1.a results on dilation based view synthesis prediction with upsampling [Seok Lee Seungsin Lee Ho-Cheon Wey Jaejoon Lee Du Sik Park (Samsung)]

In this contribution, dilation-based view synthesis prediction with upsampling is proposed. The document reports 0.23% bit rate reduction on coded data and 0.26% bit rate reduction on synthesized views with 150% decoding time. Due to the high decoding time of the proposed method, an alternative approach with simpler computations is proposed in JCT2-A0036.

No action.

- JCT2-A0058/m25937 3D-CE1.a cross check report of dilation based view synthesis prediction with upsampling by Samsung (m25879/JCT2-A0035) [Yin Zhao (Zhejiang Univ.)]

- JCT2-A0055/m25932 3D-CE1.a results on flexible rendering precision [Yin Zhao, Lu Yu (Zhejiang Univ.)]

This contribution presents a sequence-level signaling of the rendering precision that is used to generate a view synthesis picture. Three look-up tables are proposed (integer, half-pel and quarter-pel) to enable the rendering. The selection of different precision is made based on a frame-level RD optimization process at the encoder. On average, the proposed method maintains similar RD performance while reducing decoding time by 5%.

Discussion: There was a question on whether this proposed method helps with the worst case decoder. It was noted that quarter-pel accuracy still needs to be supported in the worst case, so it was suggested that the claimed benefit may not be significant.

Discussion: There was also a question on how the precision is determined. It was stated by the proponent that the first AU is coded three times at different precisions. Encoding time was not reported in the contribution, but it can be expected to increase. It was noted that practical encoders would need to take this additional encoding requirement into account. A more practical means to determine the precision that did not incur large processing or latency would be needed.

No action.

- JCT2-A0062/m25985 3D-CE1.a: Cross check on synthesis with fractional disparity of Zhejiang Univ.(JCT2-A0056) [Ilsoon Lim, Jin Young Lee (Samsung)]

- JCT2-A0130/m26118 3D-CE1.a: Cross check of JCT2-A0055 on flexible rendering precision [X. Zhao (Qualcomm)] [miss] [late]

- JCT2-A0103/m26058 3D-CE1.a: Generalized view synthesis prediction (GVSP) mode [X. Zhao, L.Zhang, Y.Chen, M. Karczewicz (Qualcomm)]

This contribution proposes to change the signaling of View Synthesis Prediction (VSP) so that a VSP picture is not added into a reference picture list and the motion vectors to the other reference pictures are not used to predict the motion to a VSP picture. Instead, zero motion is always assumed for a block predicted from a co-located block of a VSP picture and a flag is used to indicate such prediction. This proposal is technically identical to m24940, but integrated to the latest 3D-ATM software. Without complexity increase, the proposed method gives 0.8% gain in a stable CE configuration (i.e., with Asymmetric VSP turned off).

Discussion: There was a remark on the benefits of the different aspects of this proposal, which was also noted in the CE summary report. Specifically, the results do not provide a separate evaluation, e.g., related to the benefit of excluding the VSP picture from the reference picture list. In the absence of such results, no conclusions on this potential benefit could be drawn.

Similar: A0050, A0103. Both have the option of sub-block skipping. Further study in CE on VSP to identify the benefits of the signaling methods that are suggested, and the benefit of sub-block skipping as such.

- JCT2-A0138/m26136 3D-CE1.a: Cross-check of generalized view synthesis prediction (GVSP) from Qualcomm by Nokia [Dmytro Rusanovskyy (Nokia)] [miss] [late]

- JCT2-A0146/m26164 3D-CE1.a: Cross check report of JCT2-A0103 on Generalized view synthesis prediction (GVSP) mode of Qualcomm [G. Bang (ETRI), K.Y. Kim, Y.S. Heo, G.H. Park(KHU), W.S. Cheong, N.H. Hur(ETRI)] [late]

- JCT2-A0107/m26069 3DV-CE1.a: Block-based View Synthesis Prediction for 3DV-ATM [Wenyi Su (USTC), Dmytro Rusanovskyy (Nokia), Miska M. Hannuksela (Nokia)]

The current implementation of View Synthesis Prediction (VSP) in 3DV-ATM utilizes a forward warping to produce a complete reference image (VSP frame). An asserted drawback of such frame-level implementation is its high complexity at the decoder side and its incompatibility with a block-based processing architecture which is traditionally utilized for decoder.

This contribution proposes a block-based VSP implementation (B-VSP) which is based on a backward warping process. With the B-VSP, texture pixels of a dependent view are predicted not from the synthesized VSP-frame, but directly from the texture pixels of the base view. Displacement vectors required for this process are produced from the depth map data of the dependent view. It is asserted that B-VSP reduces the complexity of view synthesis prediction considerably and avoids the need to allocate a frame buffer for the VSP reference frame.

The presented simulation results show that proposed B-VSP scheme achieves a 0.4% bit rate reduction for coded texture views and a 0.7% bit rate reduction for synthesized views. It is asserted that the estimated decoding time for B-VSP is approximately 20% lower on average than the decoding time of the anchor. When considering the execution time of the VSP modules only, B-VSP reportedly provides 70% complexity reduction.

In contrast to JCT2-A0050, a related contribution by MediaTek, a single disparity vector is calculated for each 2x2 block rather than each 4x4 block. In this contribution, it is also proposed to disable the DMC tool and reduce the resolution of D-MVP. It was reported by the proponents that these changes incur very minimal impact on coding performance and some reduction in decoding time.

Discussion: It was unclear where the gains are coming from since block-based backward warping is not expected to improve the coding efficiency. It was asserted by some members that the gains come from bug fixes. A cross-check of intermediate configurations was done during the week to confirm the results.

Discussion: Group members expressed positive support for this general direction, mainly due to the complexity reduction.

A0050 and A0107 are using block-based view synthesis through usage of backward mapping (i.e. using the coordinate system and block grid of the dependent image). Not necessarily a worst-case complexity advantage, but does not require hole-filling. The process becomes closer to the concept of block-based MC (in 0050, simply 4x4 MC blocks were used).

The gain of 0.4% is likely to be due to the not transmitted motion parameters (similar where the gain in A0103 comes from), and perhaps some other disabling. There is also a bug known in VSP motion prediction (skip and direct mode) which was disabled in A0107, but not so in A0050 (0050 only give 0.2% gain).

In general this direction seems useful as a simplification, but more investigation is needed to find the best method. Further study in CE1, but relation with CE5 is also relevant, there the block-based scheme should be included in the software as an option.

Decision(SW): Adopt A0107.

- JCT2-A0019/m26086 3D-CE1.a: Cross check on Nokia Proposal on Block based View Synthesis Prediction JCT2-A0107 [Feng Zou, Dong Tian, Anthony Vetro (MERL)]

- JCT2-A0145/m26163 3D-CE1.a: Cross check report of JCT2-A0107 on Block based View Synthesis Prediction of Nokia [G. Bang (ETRI), K.Y. Kim, Y.S. Heo, G.H. Park(KHU), W.S. Cheong, N.H. Hur(ETRI)]

### Related Contributions

- JCT2-A0036/m25882 3D-CE1.a related results on dilation based view synthesis prediction without upsampling [Seok Lee Jin Young Lee Seungsin Lee Ho-Cheon Wey Jaejoon Lee Du Sik Park (Samsung)]

This contribution presents an improvement to JCT2-A0035. In this proposal, the dilation operation is replaced by a max operation on a small set of neighboring pixel positions. This method is reported to achieve a 0.18% bit rate reduction on the coded data, and a 0.22% bit rate reduction on the synthesized views with 101% decoding time. This method is also proposed in a joint contribution with NTT.

adds complexity versus marginal gain – no action.

- JCT2-A0025/m26096 3D-CE1.a related: Cross-check report of JCT2-A0036 on dilation based view synthesis prediction without upsampling [Shinya Shimizu (NTT)]

- JCT2-A0050/m25921 3D-CE1.a related: Interview skip/direct mode with sub-partition scheme [Chi-Ling Wu, Yu-Lin Chang, Yu-Pao Tsai, Shawmin Lei (MediaTek)]

In this contribution, an inter-view skip mode is reported. In AVC-based 3D video coding, View Synthesis Prediction (VSP) is a tool to use the reference from the synthetic frame to improve coding efficiency. However, it is costly in terms of number of computations and amount of required storage to perform per-pixel warping. An inter-view skip mode is proposed to eliminate full frame view synthesis for reducing the computational complexity, memory bandwidth, and buffer. When turning off VSP and utilizing the proposed inter-view skip mode, the experimental results reportedly show an average of 10% decoding time saving with 0.21% BD-rate reduction for synthesized pictures over the total bit-rate.

In contrast to VSP and VSP skip/direct, the proposed approach avoids full-frame view synthesis and uses block-level motion compensation instead. This method is only applied to the texture of dependent views and determines 4x4 disparity vectors based on depth data from the dependent view.

Discussion: There was a question on the decoding order for texture and depth view components. It was noted by the proponents that that decoding order must be depth-first for the dependent views.

Disposition: CE, see under A0103 and A0107

- JCT2-A0063/m25987 3D-CE1.a: Cross check on interview skip/direct mode with sub-partition scheme of MediaTek (JCT2-A0050) [Ilsoon Lim, Jin Young Lee (Samsung)] [late]

- JCT2-A0056/m25933 3D-CE1.a related: improvement on NTT’s synthesis with fractional disparity [Yin Zhao, Lu Yu (Zhejiang Univ.)]

This contribution proposes a method to derive the sub-pel disparities depending on whether a sample falls along depth edges and within a non-edge area. This method aims to improve the geometric accuracy of warped pixels at depth discontinuities, and may be a potential replacement of the boundary-aware splatting process in the current view synthesis software. It is reported that the method provides a 0.1% bit rate reduction for texture coding with negligible change in decoding time.

Discussion: It was remarked that this method is already supported in the rendering software (as a post-process). However, the benefits of applying this approach within the loop are not clear and there were concerns on adopting this method as an in-loop operation.

No action.

- JCT2-A0023/m26094 CE1.a related results on the modification of in-loop view synthesis [Shinya Shimizu, Shiori Sugimoto, Hideaki Kimata (NTT)]

This contribution presents a simplified warping process for view synthesis prediction. This technique was previously proposed in m25195 and implemented in the view synthesis reference software (VSRS-1D-Fast). In the proposed method, the processing direction of the pixel-wise warping is inverted. Also, the hole-filling and inpainting is simplified. Experimental results on the common test condition show that there is negligible impact on encoding or decoding time, but it is reported that the proposed method can reduce memory requirements since no z-buffering is necessary. When applied for texture coding of dependent views, an average BD-rate improvement of 0.53% was reported since more accurate disparities become available to be utilized. The maximum gain was 1.12% for Undo_Dancer sequence.

Simplification which also improves compression

Decision: Adopt

- JCT2-A0158/m26198 3D-CE1.a results on joint proposal of Samsung and NTT [Seok Lee (Samsung), Shinya Shimizu (NTT), Seungsin Lee (Samsung), Shiori Sugimoto (NTT), Ho-Cheon Wey (Samsung), Hideaki Kimata (NTT), Jin Young Lee (Samsung), Jaejoon Lee (Samsung)] [late]

This contribution reports the results of a combined proposal based on JCT2-A0036 (Samsung) and JCT2-A0023 (NTT). The combined gains are reported to achieve a 0.62% reduction in bit rate reduction on the coded data, and 0.66% bit rate reduction on the synthesized views with no increase in decoding time and a 4.2% increase in encoding time.

No action.

JCT2-A0165 3D-CE1.a: Cross check of JCT2-A0158 results on joint proposal of Samsung and NTT [X. Zhao (Qualcomm)

- JCT2-A0061/m25984 3D-CE1.a: Cross check on sub-pel VSP simplification of NTT [Seok Lee, Jin Young Lee (Samsung)]

- JCT2-A0165/m26285 3D-CE1.a: Cross check of JCT2-A0158 results on joint proposal of Samsung and NTT [X. Zhao (Qualcomm)]

### Conclusions

Adoptions: See above A0023 and A0107

Targets: Coding efficiency improvement + complexity reduction

Continue CE, new CE on illumination comp.

## CE1.h: View synthesis prediction

### Summary

- JCT2-A0009/m25801 3D-CE1.h summary report: View synthesis and inter-view prediction [Fabian Jäger (RWTH Aachen University)]

The summary report notes one CE contribution from NTT/Mitsubishi (JCT2-A0018), which reports improvements in coding efficiency and complexity reductions, as well as one CE related contribution (JCT2-A0086) from LG.

### CE Contributions

- JCT2-A0018/m26085 3D-CE1.h Results on View Synthesis Prediction [Shinya Shimizu, Shiori Sugimoto, Hideaki Kimata (NTT), Dong Tian, Feng Zou, Anthony Vetro (MERL)]

View synthesis prediction (VSP) is a technique to remove inter-view redundancies when coding multiview video information, which uses a synthetic picture as a reference picture to predict the current picture. This contribution first proposes inserting synthetic pictures to reference picture lists, consistent with the scheme in ATM. This document reports an average bitrate saving of -2.6% and -0.4% for the dependent views, with maximum gains of -9.9% and -5.8% for these views. The benefit of VSP for depth coding has also been investigated, but the result is not yet conclusive.

This contribution further proposes a VSP skip mode which enables the skip mode relative to the synthetic reference picture. There are two methods to achieve this: additional block-level signalling as done in ATM, and utilizing the existing merge syntax in HEVC. The first approach has been implemented, but degrades performance. The second approach is expected to have better performance, but the implementation has not been completed yet. In general, further study is required to evaluate the different skip mode designs.

Lastly, when VSP is enabled, this contribution proposes to simplify the rendering process in HTM, which would reduce the decoding time by 4% if VSP is only applied to texture, or 20% if VSP is applied to both texture and depth.

This proposal recommends: 1) adopting VSP by adding the view synthesis picture to the reference picture lists for texture and depth into HTM; 2) adopting the simplified rendering process into HTM; 3) continue to study VSP skip in CE1.h; and 4) explore further simplifications of the rendering process.

Discussion: Remark that we should be cautious in adopting this proposal since the gains are mixed with non-negligible increase in decoder time. However, it was mentioned that block-level synthesis and backward warping approaches explored in ATM could also apply in this context.

Discussion: Remark that stereo compatibility would also be lost if this proposal were adopted. However, this is something that could be considered as part of a profiling discussion.

Clarify: Why is the gain for the second dependent view low (0.4%)?

If the overall bitrate reduction is considered, this goes down to 0.1%

Further study to resolve the issues mentioned in discussion.

- JCT2-A0092/m26024 3D-CE1.h: Cross check on Mitsubishi-NTT joint proposal [Jin Young Lee, Byung Tae Oh, Ilsoon Lim (Samsung)] [late]

### Related Contributions

- JCT2-A0086/m26038 3D-CE1.h related: Illumination Compensation for Dependent Views Coding [Hongbin Liu (LG Electronics)]

This contribution proposes illumination compensation (IC) method to improve coding efficiency of dependent views. The basic idea is that illumination discrepancy between different views should be corrected in inter-view prediction. Linear IC model is employed to compensate illumination discrepancy between different views. Parameters in IC model are estimated for each PU (prediction unit) using available reconstructed neighbouring pixels and therefore are not need to be encoded. Applying IC or not is decided at CU (coding unit) level, and an IC flag is coded for CU in which inter-view prediction is used. A method is also designed to adaptively determine whether there is illumination discrepancy in a picture, and an IC flag is encoded for the first slice of each picture in dependent views.

It reported -0.8%, -1.1% for dependent views with encoder complexity with 128% and encoder complexity with 96%. Crosscheck confirmed results, except for 2 sequences due to decoder crash.

This proposal recommends establishing a new CE for illumination compensation.

Discussion: Weighted prediction should be enabled to produce results for comparison.

Overall gain 0.4%.

Further study: New CE on illumination comp.

- JCT2-A0154/m26185 3D-CE1.h related: Cross-check report of JCT2-A0086 on Illumination Compensation for Dependent Views Coding [Shinya Shimizu (NTT)] [miss] [late]

### Conclusions

Some gain observed for view synthesis (up to 10% for dependent views)

This may however be misleading as in the current test set often the dependent views do not require high portion of data rate (too similar to base view) – look for better test sequences

Continue CE, new CE illumination comp.

(AHG on test material?)

## CE2: Depth representation and coding

### Summary

- JCT2-A0089/m26045 CE2 summary report: depth representation and coding [Olgierd Stankiewicz (Poznan Univ. of Technology)]

The summary report notes that there is one contribution from KWU and ETRI registered as JCT2-A0016, but this method reports a loss. As a result, these organizations propose to discontinue the CE.

Discussion: It was noted that this is the second meeting without substantial contribution on this CE. Although some proponents (namely, Poznan and Samsung) propose to continue the CE, it was believed that there is not sufficient justification to continue the CE and that any new results should be submitted when they become available.

### CE Contributions

- JCT2-A0016/m26082 3DV-CE2.h adaptive quantization for depth map (AQD) [Junghak Nam, Sunmi Yoo, Hyomin Choi, Hyunho Jo, Donggyu Sim(KWU), Gun Bang, Won-Sik Cheong, Namho Hur(ETRI)] [late]

This contribution reported the results of adaptive quantization for depth map (AQD) that was proposed at the previous meeting. Experimental results are reported that achieve bitrate increases of approximately 0.1% and 0.1% for 3-view and 2-view cases in terms of synthesized quality, respectively.

The proponents do not urge the JCT-3V to consider this proposal.

- JCT2-A0120 3D-CE2.h cross check of adaptive quantization for depth map (JCT2-A0016) [Yunseok Song, Yo-Sung Ho (??)] [late]

### Conclusions

Due to the lack of contribution and progress for two consecutive meeting cycles, it is recommended that this CE be discontinued. Further contributions in this area are encouraged.

Discontinue.

## CE3: Depth map resampling and filtering

### Summary

- JCT2-A0106/m26068 3D-CE3 summary report: Depth resampling and filtering [Dmytro Rusanovskyy (Nokia)]

CE3 was established to evaluate depth map resampling and filtering solution for improved coding efficiency and to resolve problems with visual artifacts introduced to synthesized views due to resampling of the depth map data.

Two proposals related to normative tools were submitted:

- JCT2-A0037 presents an adaptive range filter as an in-loop filter to improve coding efficiency. This filter replaces the current deblocking filter of AVC. Reported results are -0.6% bit rate reduction for coded views, 6% bit rate reduction for depth map data and 1.7% bit rate reduction for synthesized views. The estimated decoding time of the proposed method was reported to be in the range of 104% to 110% when the deblocking filter was disabled.

- JCT2-A0108 proposes to enable the existing AVC deblocking filter for depth map coding. Reported results are 0.2% bit rate reduction for coded views, 6.8% bit rate reduction for depth map data and 0.7% bit rate reduction for synthesized views. The estimated decoding time was reported as 104% of the anchor.

It was also reported that a combination of non-normative pre- and post-processing (i.e., depth resampling) provides an average of 5.9% bit rate reduction, while the combination with existing deblocking filter enabled for depth provides an average bit rate reduction of 6.6%. A notable reduction in visual artifacts was shown on sample frames, but visual results for video need to be confirmed with viewing during the week.

### CE Contributions

- JCT2-A0037/m25884 3D-CE3.a results on region based adaptive loop filter [Ilsoon Lim, Ho-Cheon Wey, Jaejoon Lee, Du Sik Park (Samsung)]

This contribution present a new depth map coding scheme to improve coding efficiency. For this purpose, an adaptive range filter that restores the coded depth maps in accordance with the estimated noise variance is proposed. More specifically, each pixel is classified in such a way that the distortion cost is minimized and is restored depending on local noise level. The restored picture is used for the motion-compensated prediction of future pictures with the aim to improve coding efficiency. The document reports results that show a 0.60% bit rate reduction on coded data and a 1.70% bit rate reduction on synthesized views. Additional results were reported that the proposed method provides additive gains when combined with a post-processing filter (e.g., the dilation filter proposed in a related contribution).

Discussion: It was remarked that the existing normative deblocking method provides 0.44% gain on decoded views, while the proposed method provides 0.6% gain with deblocking disabled, so it was therefore considered incremental relative to the capabilities of existing tools.

No action.

- JCT2-A0147/m26165 3D-CE3.a: Cross check report of JCT2-A0037 on region based adaptive loop filter of Samsung [G. Bang (ETRI), K.Y. Kim, Y.S. Heo, G.H. Park(KHU), W.S. Cheong, N.H. Hur(ETRI)] [late]

- JCT2-A0108/m26070 3DV-CE3: Enabling H.264/AVC deblocking for depth map coding in 3DV-ATM [Dmytro Rusanovskyy (Nokia), Payman Aflaki (TUT), Miska M. Hannuksela (Nokia)]

This contribution proposes to enable the existing AVC deblocking filter for coding depth map data. The simulation results presented in this document show that the deblocking results achieve a bit rate reduction of 6.7% for depth map data. It was asserted that the improved quality depth map achieves a 0.3% bit rate reduction on the texture and a 0.7% bit rate reduction on the synthesized views.

Agreement: CTC for ATM should enable de-blocking of depth maps.

Decision(SW): Enable de-blocking filter in depth maps (CTC)

- JCT2-A0038/m25885 3D-CE3.a results on dilation filter for depth post processing [Seok Lee Seungsin Lee Ho-Cheon Wey Jaejoon Lee Du Sik Park (Samsung)]

A depth post-filtering using dilation filter is evaluated in this proposal. It is reported that the proposed post-filter achieves a 4.47% bit rate reduction on the synthesized views with the deblocking filter enabled, and a 4.35% bit rate reduction with the deblocking filter disabled. It was also asserted that the post filtering with dilation filter also improves the visual quality of synthesized views at boundary regions.

Turning conventional de-blocking filter on already provides most of the gain (as per A0108). Defining an entireley new building block for a relatively small gain (0.2% BR reduction) does not seem appropriate. No action to include the filter in the loop.

Decision(SW): Adopt post-processing from A0038 (CTC)

- JCT2-A0136/m26134 3DV-CE3: Cross-check of Samsung proposal for 3DV-ATM Post-Processing by Nokia [Payman Aflaki, Dmytro Rusanovskyy (Nokia)]

- JCT2-A0060/m25967 3D-CE3.a results on depth boundary filtering as post-processing by GIST [Yunseok Song, Cheon Lee, Yo-Sung Ho (GIST)] [late]

The proposed filter aims to refine depth boundary regions to improve the quality of synthesized images. The filter is applied as post-processing of depth upsampling. Due to an unexpected memory error, 1920x1088 sequences were not tested by the proponents. A bit rate reduction of 2.32% and 2.59% on the synthesized views were achieved when the deblocking filter was enabled and disabled for depth, respectively. However, a large increase in decoder time of approximately 350-400% was observed.

Large increase in complexity – no action.

- JCT2-A0066/m25990 3D-CE3.a: Cross check on depth boundary filter of GIST [Ho-Cheon Wey, Seok Lee, Seungsin Lee, Ilsoon Lim (Samsung)] [late]

The cross check report from Samsung indicates a match for SD sequences. In this report, synthesized gains were 3.18% and 3.53% for deblocking enabled/disabled when including HD sequences.

### Related Contributions

- JCT2-A0039/m25887 3D-CE3.a related results on depth hybrid post filter [Seok Lee Seungsin Lee Ho-Cheon Wey Jaejoon Lee Du Sik Park (Samsung)]

A hybrid post-filtering method is proposed that utilizes a dilation filter and a filter based on a local segmentation. The document asserts that coding performance can be improved by identifying the type of depth maps using a proper method and by applying the proposed hybrid filter. The proposed post filter achieves a bit rate reduction of 5.69% on synthesized views with the deblocking filter turned on, and a 5.69% bit rate reduction with the deblocking filter turned off.  However, a notable increase in decoder complexity was observed: 193.7% and 177.6% decoding time for deblocking filter on and deblocking filter off, respectively. It was noted that the proposal has not been cross-checked. Also, the proponents are considering further improvements and report that these may be submitted to the next meeting.

No action.

- JCT2-A0109/m26071 3DV-CE3.a: Non-linear Depth Map Upsampling for 3DV-ATM Coding [Payman Aflaki (TUT), Dmytro Rusanovskyy (Nokia), Miska Hannuksela (Nokia)]

3DV-ATM utilizes coding of depth map data at reduced spatial resolution. After decoding of depth map it is upsampled to its original resolution prior to rendering of virtual views. In the current implementation, a basic bi-linear up-sampling is utilized at the post-processing stage of 3DV-ATM. This contribution proposes a non-linear depth map up-sampling method as a post-processing. Simulation results show that proposed method achieved a 0.96% bit rate reduction on synthesized views when the AVC deblocking filter is disabled for depth. When the AVC deblocking is enabled, the proposed scheme achieves a 1.13% bit rate reduction on synthesized views. In addition to the objective gain, it is asserted by the proponent that proposed method provides a reduction in visual artifacts of synthesized views comparing with the anchor approach (i.e. bi-linear up-sampling).

No action.

- JCT2-A0064/m25988 3D-CE3.a: Cross check on depth post processing of Nokia [Ho-Cheon Wey, Seok Lee, Seungsin Lee, Ilsoon Lim (Samsung)] [late]

- JCT2-A0110/m26072 3DV-CE3: Non-linear Depth Map Downsampling for 3DV-ATM Coding (Pre-processing) [Payman Aflaki (TUT), Dmytro Rusanovskyy (Nokia), Miska M. Hannuksela (Nokia)]

3DV-ATM utilizes coding of depth map data at reduced spatial resolution. After decoding, depth map is upsampled to its original resolution and utilized for rendering of virtual views. The CTC specifies that a low resolution depth map data is produced with the JSVM down-sampling filters. Such a low pass filter can introduce an over-smoothing in the depth map and lead to noticeable visual artifacts in the synthesized views.

This contribution proposes to replace the downsampling method for depth map with a non-linear downsampling. Results show that proposed downsampling method provides a bit rate reduction of 4.5% for synthesized views. When combined with non-anchor depth map up-sampling method, these non-normative tools improve provide a 5.9% bit rate reduction. Additionally, it is asserted that proposed non-normative tool significantly reduce visual artifacts that results from downsampling of the depth map.

No action.

- JCT2-A0065/m25989 3D-CE3.a: Cross check on depth pre/post processing of Nokia [Ho-Cheon Wey, Seok Lee, Seungsin Lee, Ilsoon Lim (Samsung)] [late]

- JCT2-A0155/m26188 Sampling filters for depth map video for 3DV (CE3 related) [W Dai, M Krishnan, P Topiwala (FastVDO)] [late]

The contribution presents advanced filters for depth up- and downsampling. Better performance than existing filters in SVC when applied to depth maps. The results were presented for uncoded and HEVC-coded depth data by comparing depth PSNR.

Very preliminary, presented for information.

JCT2-A0169 3D-CE3.a related: The simplification of the de-blocking scheme for PSIP and DISP [G. Bang (ETRI), K.Y. Kim, Y.S. Heo, G.H. Park (KHU), W.S. Cheong, N.H. Hur (ETRI)] [late] [miss]

This contribution proposes to reduce the simplification of the de-blocking scheme for PSIP and DISP mode. Simulation result achieves a 0.06% bit rate reduction on the coded data, and a 0.03% bit rate reduction on the synthesized views. The maximum gain has almost 1.0% bit rate reduction on the depth coded data for the GT_Fly sequence. And the simulation result achieves almost 7.0% bit rate reduction on the coded data, and a 0.75% bit rate reduction on the synthesized views compared with 3DV-ATM v0.4 (De-blocking filter for depth: OFF). We recommend keeping the de-blocking scheme for depth coding, because it has still a chance to improve a performance for depth coding.

No obvious benefit – small gain when normative de-blocking operation is modified for the depth maps.

    - Conclusions

All proposals in ATM.

Preprocessing filtering (non-linear e.g. morphological) improves PSNR of synthesized views – should the depth maps be modified? Postprocessing helps as well. In-loop filtering was also considered and affects the depth-based prediction (best results with all three steps pre-inloop-post), but seems to have the least effect.

Further review was performed after subjective viewing of

A0109+A0110, A0038, A0060 with de-blocking in loop switched on.

BoG report (D. Rusanovskyy) to be uploaded

(include header)

Viewing was performed with 6 subjects and 3 sequences (18 cases in total)

In general, it is observed that some of the proposals may solve certain visual artefact problems, but introduce new problems.

A0038 operated as post-filtering was found to give benefit such that it was judged better in 11 cases, equal in 5 cases and worse in 2 cases, compared to the anchor (no PP, but de-blocking in the loop). Adopt A0038 post processing (see above), no action on other proposals

discontinue CE, but future proposals in this category are welcome if they show benefit.

## CE5.a: Motion/mode parameter prediction

### Summary

- JCT2-A0043/m25912 3D-CE5.a summary report: motion/mode parameter prediction [Yu-Lin Chang (MediaTek)]

The summary report notes that there was one CE contribution from MediaTek (JCT2-A0045) and four CE related contributions from Samsung (JCT2-A0040/A0041), MediaTek (JCT2-A0046) and Nokia (JCT2-A0111). The main aspects of these contributions are summarized below.

- JCT2-A0045 (MediaTek): This proposal explicitly signals the MVP index to the decoder. The contribution reports a 9.2% BD-Rate reduction for texture and a 7.2% BD-Rate reduction for coded and synthesized views with slight reduction in decoding complexity.

- JCT2-A0040 (Samsung): This contribution proposes to change the coefficient of the depth-to-disparity function to a shifting parameter.

- JCT2-A0041 (Samsung): This contribution proposes to employ the corresponding and reference depth maps with quarter resolution.

- JCT2-A0041 (MediaTek): This contribution proposes a simplification to derive the disparity vector from a maximum depth value of four corner depth samples.

- JCT2-A0111 (Nokia): This contribution proposes to disable DMC for VSP as a simplification to reduce the decoding time.

The summary report recommends adopting the MV competition-based skip/direct mode with explicit signaling of the MVP index as proposed in m24847 with the size of the MVP list fixed to 1. It was noted that this method provides best performance and avoids the complexity and memory access bandwidth issue caused by the depth-based motion vector competition (DMC) in the ATM. The summary report also suggests further investigating other related ideas in a continued CE. There was favorable support from the group on these recommendations and no objections or concerns were raised.

### CE Contributions

- JCT2-A0045/m25916 3D-CE5.a results on motion vector competition-based Skip/Direct mode with explicit signaling [Jian-Liang Lin, Yi-Wen Chen, Yu-Lin Chang, Yu-Pao Tsai, Yu-Wen Huang, Shawmin Lei (MediaTek)]

This contribution reports the results of various tests to confirm the efficiency of the proposed motion prediction tool proposed in m24847 compared to the existing depth-based motion vector competition (DMC) tool in the ATM. The anchor used in the tests turns DMC off for the skip and direct modes, which falls back to the median-based derivation of the motion vector predictor (MVP) as done in AVC.

The results are summarized as follows. In experiments T01 and T02, the MV competition-based skip/direct mode with explicit signaling of the MVP index as proposed in m24847 is tested with the size of the MVP list fixed to 1 and 2, respectively. (It is later said that in T01 no explicit signalling of the MV candidate is performed).  In T01, which fixes the size of the MVP list to 1, 9.2% BD-Rate reduction can be achieved for texture coding and 7.2% BD-Rate reduction can be achieved for synthesized pictures over the total bit-rate (texture + depth). In T02, which fixes the size of the MVP list to 2, BD-Rate reductions of 5.1% and 3.2% are achieved for texture coding and synthesized results, respectively. T03 is conducted to evaluate the performance of DMC for the skip and direct modes. The results show that DMC for the skip and direct modes achieves 2.5% and 2.3% BD-Rate reduction for texture coding and synthesized results, respectively.

Discussion: It was requested that specification text be provided so that all aspects of this proposal are clear. The proponents agreed to provide the text during this week for review.

Review was done by 3D-ATM editor, confirmed that the quality is good enough

Decision: Adopt version T01 to 3D-ATM and 3D-AVC (draft text)

Note: This contribution introduces MV competition like concepts, it becomes a not insignificant change of the AVC base spec. for the coding of the dependent texture view. After all, it might be questionable whether AVC with large amount of changes for dependent view would be desirable compared to hybrid AVC/HEVC solutions.

- JCT2-A0020/m26087 3D-CE1.a: Cross check on MediaTek Proposal on Motion Vector Competition-Based Skip/Direct Mode JCT2-A0045 [Dong Tian, Anthony Vetro (MERL)]

- JCT2-A0078/m26023 3D-CE5.a: Cross check on motion vector competition-based skip/direct mode with explicit signaling (JCT2-A0045) [Jin Young Lee (Samsung)]

- JCT2-A0101/m26056 3D-CE5.a: cross-check result on JCT2-A0045 [Tadashi Uchiumi, Yoshiya Yamamoto, Tomohiro Ikai (Sharp)]

- JCT2-A0129/m26117 3D-CE5.a: Cross check of JCT2-A0045 on motion vector competition-based Skip/Direct mode with explicit signaling [X. Zhao (Qualcomm)] [miss] [late]

- JCT2-A0148/m26166 3D-CE5.a: Cross check report of JCT2-A0045 on motion vector competition-based Skip/Direct mode with explicit signaling of MediaTek [G. Bang (ETRI), K.Y. Kim, Y.S. Heo, G.H. Park(KHU), W.S. Cheong, N.H. Hur(ETRI)]

### Related Contributions

- JCT2-A0040/m25889 3D-CE5.a related results on improved skip and direct motion vector prediction [Jin Young Lee, Jaejoon Lee, Du-Sik Park (Samsung), Tadashi Uchiumi, Yoshiya Yamamoto (Sharp)]

This proposal presents an improved skip and direct motion vector prediction on the top of the current depth-based motion vector prediction and the new competition-based skip and direct motion vector prediction. It is reported that the proposed method reduces the bit rate by 0.2% on coded date with 4% reduction in decoding complexity due to modifying the depth-to-disparity function to a shift operation.

This was considered by the group to be a relatively minor change and there were no objections or concerns raised on adopting this change.

The benefit appears low. No action.

- JCT2-A0051/m25924 Cross-check report for M25889 on CE5.a related: improved skip and direct motion vector prediction [Yi-Wen Chen, Jian-Liang Lin, Yu-Wen Huang (MediaTek)] [miss] [late]

- JCT2-A0028/m26099 3D-CE5.a related: Cross-check report of JCT2-Axxxx on Samsung's proposal1 [Shinya Shimizu (NTT)]

- JCT2-A0041/m25890 3D-CE5.a related results on depth-based motion vector prediction for asymmetric texture and depth resolutions [Jin Young Lee, Jaejoon Lee, Du Sik Park (Samsung)]

This contribution proposes to remove the up-sampling process when the D-MVP process is invoked. The method is also evaluated with respect to the recently proposed competition-based skip and direct motion vector prediction proposed in JCT2-A0045. It is demonstrated that the proposed method reduces the decoding complexity by 11% in the current ATM and by 4% with the method of JCT2-0045, with negligible impact on coding performance.

Discussion: A related contribution (?) suggests a 6% reduction by removing the up-sampling. The proponents noted that the discrepancy was likely due to the difference in computing platforms, but the group thought that this was a larger difference than expected.

Complexity reduction (how large?). Further study

- JCT2-A0052/m25925 Cross-check report for M25890 on CE5.a related: depth-based motion vector prediction for asymmetric texture and depth resolutions [Yi-Wen Chen, Jian-Liang Li, Yu-Wen Huang (MediaTek)] [miss] [late]

- JCT2-A0029/m26100 3D-CE5.a related: Cross-check report of JCT2-Axxxx on Samsung's proposal2 [Shinya Shimizu (NTT)]

- JCT2-A0046/m25917 3D-CE5.a related: Simplification on the disparity vector derivation for AVC-based 3D video coding [Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei (MediaTek)]

In the AVC-based 3D video coding, a direction-separated MVP is utilized for the temporal and inter-view motion vector predictions in the inter mode. When the reference picture is an inter-view prediction picture, the inter-view motion vectors of the adjacent blocks are employed for the inter-view prediction. If an inter-view motion vector is unavailable, it is replaced by a disparity vector derived from a maximum depth value within the depth block associated with current block. This contribution proposes a simplification to derive the disparity vector from a maximum depth value of four corner depth samples instead of all depth samples within the associated depth block. With this simplification, the number of the depth samples to be accessed can be reduced from 256 to 4 and the number of the required comparisons can also be reduced from 255 to 3. The experimental results reportedly show that this proposed simplification does not cause any coding loss.

Discussion: It was remarked that this method could also be applied to HTM.

Complexity reduction. This is not reflected in the computation time. Decision: Adopt to 3D-ATM (not 3D-AVC, as no draft text is available).

- JCT2-A0128/m26116 3D-CE5.a related: Cross check of JCT2-A0046 on simplified derivation of disparity vector [X. Zhao (Qualcomm)] [miss] [late]

- JCT2-A0111/m26073 3DV-CE5.a: Modified DMVP for purposes of View Synthesis Prediction (Bug-fix) [Wenyi Su (USTC), Dmytro Rusanovskyy (Nokia), Miska M. Hannuksela (Nokia)]

This contribution asserts that current ATM design utilizes two contradicting approaches to select VSP_SKIP and VSP_DIRECT modes. The first approach determines whether to select VSP_SKIP or VSP_DIRECT through a Depth-based Motion Competition (DMC). In this scheme a synthesized depth map is utilized for the mode derivation at the decoder side and no signaling is required. The second approach is the explicit signaling of VSP_SKIP and VSP_DIRECT modes with a flag for which the encoder performs mode selection using the Rate-Distortion Optimization. The latter scheme overwrites the decision making of the DMC and uses explicit signalling even if it is not required by the DMC design. To resolve these conflicting approaches it is proposed to disable the DMC for VSP. Simulation results show that proposed scheme provides less than 0.1% bit rate increase on the coded data, but it provides about 5% reduction in decoding time. This proposal is consistent with the information and results provided in JCT2-A0107.

This is no longer relevant due to adoption of A0045.

- JCT2-A0067/m25991 3D-CE5.a: Cross check on DMVP simplification of Nokia [Kwan-Jung Oh, Jin Young Lee (Samsung)]

### Conclusions

(see above)

Note: These approaches give good gain, but require storage of motion and mode parameters, such that likely the complexity is increased. (same applies to CE5.h)

## CE5.h: Motion/mode parameter prediction

Breakout to review and report back (Sehoon Yea) Tue 19-21:00 to be announced by email

### Summary

- JCT2-A0076/m26021 CE5.h Summary Report: Motion/Mode Parameter Prediction [Sehoon Yea (LG Electronics)]

Two options within the HTM reference software for disparity vector generation were made available as per the decisions at the previous meeting: (a) the PDM-based approach; and (b) inter-view motion vector-based approach which is the result of harmonization of m25024, m24989, m24937. The mandates of the CE included investigating the potential benefits of combining global disparity vector with the inter-view motion vector-based approach. In this investigation, the inter-view motion vector-based approach (i.e., option B above) is used as an anchor. The CE also aimed to compare the pros and cons of the two approaches (i.e., options A and B) for generating disparity vector in terms of coding efficiency and complexity.

The summary report notes that there is one CE contribution (JCT2-A0097) and 11 related proposals, which have been grouped as follows:

- PDM-based approach (Option A): A0013, A0031, A0047, A0095

- Interview motion-vector approach (Option B): A0097, A0126

- Modification of Merge/AMVP-list construction: A0014, A0048, A0049, A0096, A0133, A0134

### CE Contributions

- JCT2-A0097/m26052 3D-CE5.h: Disparity vector generation results [L. Zhang, Y. Chen, M. Karczewicz (Qualcomm)]

In the current 3DV-HTM software, disparity vectors can be estimated by two methods, and are used for inter-view motion prediction and inter-view residual prediction. One method is to maintain a depth map for each picture of each view and the depth map is generated from disparity motion vectors and predicted by propagating the depth map to a different time instance with temporal motion vectors and propagating the depth map to a different view using view synthesis. The other method, as proposed in m24937, derives a disparity vector from spatial and temporal neighboring blocks, once a disparity motion vector in any of the blocks is identified, the disparity vector is derived. The coding efficiency of the two methods is analyzed. It is reported that deriving the disparity vector from neighboring blocks will significantly reduce the complexity with negligible impact on compression efficiency, where the complexity analysis includes required memory and implementation impact on hardware design. The proponents recommend adopting this proposal.

Discussion: The group appeared to express favorable support to adopt this proposal.

Decision: Adopt in combination with A0126

- JCT2-A0156/m26196 3D-CE5.h: Cross check report on Disparity vector generation results of Qualcomm [Jaewon Sung, Sehoon Yea (LG Electronics)] [late]

### Related Contributions

- JCT2-A0013/m25842 3D-CE5.h related: Simplification of depth-based inter-view prediction [Tadashi Uchiumi, Tomohiro Ikai, Yoshiya Yamamoto (Sharp)]

This contribution proposes a technique is for the PDM-based approach with disparity vector generation. The proposal decreases number of samples needed for deciding the disparity vector from a PDM block. Specifically, four corner samples in a PDM block are used regardless of the PU size. The simulation result reports that there is no coding loss on average.

- JCT2-A0080/m26026 3D-CE5.h: Cross check on simplification of depth-based inter-view prediction of Sharp [Jin Young Lee (Samsung)]

- JCT2-A0014/m25843 3D-CE5.h related: Simplification of AMVP [Yoshiya Yamamoto, Tomohiro Ikai, Tadashi Uchiumi (Sharp)]

This contribution presents a simplified motion vector candidate construction for AMVP. The proposed method adds a zero vector to the candidate list when first predictor (derived by inter-view motion prediction) is not available and changes the pruning combination to avoid the unnecessary candidate motion vector derivation process. It is reported that average BD-rate of the proposal is not changed from that of the anchor. This method decouples the inter-view candidate derivation and normal AMVP candidate derivation.

Discussion: It was asked whether other candidates were tested. The proponents mentioned that other candidates were tested, but the zero-vector candidate worked best.

- JCT2-A0143/m26151 3D-CE5.h related: Cross-check report of JCT2-A0014 on simplification of AMVP [Y. Takahashi, S. Hattori, T. Suzuki (Sony)] [late]

- JCT2-A0031/m25894 3D-CE5.h related results on improved inter-view residual and motion parameter prediction [Jin Young Lee, Byeongdoo Choi, Jaehyun Kim, Jaejoon Lee, Jeonghoon Park, Du Sik Park (Samsung)]

This proposal introduces an improved residual prediction based on the estimated depth image that uses the maximum disparity instead of the middle sample of an associated block. The proposed method achieves a bit reduction of 0.1% with no change in encoding and decoding time.

- JCT2-A0141/m26139 3D-CE5.h related: Cross-check report of JCT2-A0031 on improved residual prediction [Shinya Shimizu (NTT)] [miss] [late]

- JCT2-A0047/m25918 3D-CE5.h related: Simplification on disparity vector derivation for HEVC-based 3D video coding [Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei (MediaTek)]

In the HEVC-based 3D video coding, a disparity vector derived from the depth values is used to locate a corresponding block for the inter-view motion prediction in the merge, skip and inter modes, where the disparity vector is derived from a maximum depth value within the depth block associated with the current PU. In this contribution, a simplification is proposed to derive the disparity vector from a maximum depth value of four corner depth samples instead of all depth samples within the associated depth block. With this simplification, the number of the depth samples to be accessed can be significantly reduced from 256 to 4 and the number of the required comparisons can also be reduced from 255 to 3 when a 64x64 PU is coded. The experimental results also reportedly show that this proposed simplification can even achieve slight coding gain (0.2% BD-rate reduction for view 1). It was noted that the proposed method is similar to JCT2-A0013 (proposed by Sharp).

- JCT2-A0160/m26184 3D-CE5.h related: Cross Check of Disparity Vector Derivation Simplification by MediaTek (JCT2-A0047) [J. Jung, K. Viswanathan, (Orange Labs)] [miss] [late]

- JCT2-A0048/m25919 3D-CE5.h related: Pruning process for inter-view candidate [Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei (MediaTek)]

In the HEVC-based 3D video coding, a number of parallelizable motion information comparisons between the spatial merging candidates are performed for the redundancy removal in the candidate list of the merge and skip modes. This contribution proposes to make one additional parallelizable motion information comparison between the inter-view candidate and the first spatial candidate in the pruning process. This contribution also proposes to remove the check on the number of available candidates before the pruning process in the inter mode for simplification. The experimental results reportedly show BD-rate savings of 0.2% and 0.4% are achieved for texture view 1 and view 2, respectively, with this simple candidate comparison.

Discussion: Proposed simplification appears to be beneficial, and enables a small gain.

Discussion: Were comparisons made to JCT2-A0096? Both include the inter-view candidate in the pruning process. But, JCT2-A0096 requires five additional comparisons, while the proposed approach requires only one between inter-view and spatial candidate (A1). It is claimed that the processing latency is not increased and parallel processing capability is not affected with the proposed method.

- JCT2-A0079/m26025 3D-CE5.h: Cross check on pruning process for inter-view candidate of MediaTek (JCT2-A0048) [Jin Young Lee (Samsung)] [miss] [late]

- JCT2-A0049/m25920 3D-CE5.h related: Inter-view motion prediction for HEVC-based 3D video coding [Jicheng An, Yi-Wen Chen, Jian-Liang Lin, Yu-Wen Huang, Shawmin Lei (MediaTek)]

In this contribution, two methods are proposed to simplify and to improve the inter-view motion prediction in the merge, skip and inter modes in the HEVC-based 3D video coding. First, a constraint is applied to the inter-view motion prediction which only refers to one inter-view picture to derive the inter-view motion parameters. Second, it is proposed to improve the derivation of the inter-view merging candidate in the merge and skip modes. Following the concept for deriving the spatial merging candidate, the same motion parameters (prediction direction, reference pictures, and motion vectors) as those of the corresponding block in the inter-view picture are directly used as an inter-view merging candidate. When the two proposed methods are combined, 1.1% and 1.4% BD-Rate gains can be reportedly achieved for texture view 1 and view 2, respectively, and 0.3% total BD-Rate gain is reportedly achieved for coded and synthesized views. Moreover, the proposed constraint can provide a systematical way to manage the reconstructed motion data by synchronizing the reconstructed motion data buffer with the decoded picture buffer.

Discussion: It was remarked by the group that the first part of the proposal can be considered a bug fix (?), similar to JCT2-A0095.

Decision: Adopt second part of the proposal to 3D-HTM

- JCT2-A0139/m26137 3D-CE5.h related: Cross check of Inter-view motion prediction of MediaTek [L. Zhang (Qualcomm)] [late]

- JCT2-A0095/m26050 3D-CE5.h related: Bug fix for independent estimated depth maps of non-base views in CTC [L. Zhang, Y. Chen, M. Karczewicz (Qualcomm)]

In the current HTM, the estimated disparity map of the second non-base view depends on the estimated disparity map of the first non-base view. However, it was asserted that during the previous meeting there was agreement that the two non-base views, each with the base view corresponds to a stereo operation point, shall be independently decodable in the Common Test Condition (CTC), mainly due to the fact that no inter-view texture prediction between the non-base views is used. Therefore, the second non-base view depends on the first non-base view for decoding, which is considered a bug of the current HTM software by the proponents. In this contribution, this bug is fixed in a way that a first depth map frame associated with each frame in base view is maintained for the first non-base view while the second depth map frame associated with each frame in base view is maintained for the second non-base view and there is no interaction between them. Compression efficiency loss of 0.1% is reported.

Discussion: There was a question on whether the requirement for the two non-base views to have stereo compatibility is too strict as a requirement for the specification. It was clarified that this proposal applies to the reference software only and is done to satisfy the conditions of the CTC.

- JCT2-A0132/m26122 3D-CE5.h related: Cross check of independent estimated depth maps of Qualcomm [Heiko Schwarz, Gerhard Tech (Fraunhofer HHI)]

- JCT2-A0142/m26150 3D-CE5.h related: Cross-check report of JCT2-A0095 on bug fix for estimated depth maps [Y. Takahashi, S. Hattori, T. Suzuki (Sony)] [late]

- JCT2-A0096/m26051 3D-CE5.h related: Improved merge mode for inter-view predicted motion [L. Zhang, Y. Chen, M. Karczewicz (Qualcomm)]

When inter-view motion prediction is enabled, the current HTM design of the merging candidate list includes an inter-view candidate from a dependent view. However, an inter-view candidate might be identical to existing spatial merging candidates in the merging candidate list. It is proposed to remove duplicated motion vector candidates with one additional step of pruning. Compared to the current HTM design, the proposed method achieves compression efficiency gain of 0.2%, 0.1% and 0.1% for coded views, synthesized views, coded and synthesized views, respectively, in terms of BD-Rate.

This contribution is related to JCT2-A0048; see notes on comparison between the two approaches under JCT2-A0048.

- JCT2-A0157/m26197 3D-CE5.h related: Cross check report on Improved merge mode for inter-view predicted motion of Qualcomm [Jaewon Sung, Sehoon Yea (LG Electronics)] [late]

- JCT2-A0126/m26079 3D-CE5.h: Simplification of disparity vector derivation for HEVC-based 3D video coding [Jaewon Sung, Moonmo Koo, Sehoon Yea (LG Electronics)]

For inter-view motion parameter prediction and inter-view residual prediction, a disparity vector should be determined to specify a corresponding location in an inter-view reference picture. During the last meeting, a disparity vector derivation algorithm, which searches DCP blocks in spatial and temporally neighboring blocks, was included in the HTM software. This contribution proposes an efficient disparity vector derivation algorithm that reduces the number of accesses to temporal reference pictures, and extends the source of disparity information by including DV-MCP blocks, which used a disparity vector for its inter-view motion parameter prediction. Experimental result show that the proposed algorithm increased BD-Rate by 0.1% for texture views and no coding performance change for synthesized views. It is claimed that a key benefit of the proposed algorithm is that it reduces the number of accesses to temporal reference pictures from 18 positions to 2 positions, which are limited to only the co-located picture.

Decision: Adopt in combination with A0097

- JCT2-A0137/m26135 3D-CE5.h related: Cross check of JCT2-A126 of LG [L. Zhang (Qualcomm)] [late]

This cross-check reports a 0.1% bit rate increase in synthesized view, which is a slight mismatch with the results reported in JCT2-A0126.

- JCT2-A0131/m26120 CE5.h related: Information on the impact of the merge candidate list pruning process on side views encoding [Thomas Guionnet, Laurent Guillo, Christine Guillemot (INRIA)]

This is an informative contribution on the merge candidate list pruning process. In HEVC, the construction of the candidate vector list is normatively specified. A simplification of the candidate list construction process has been proposed in JCTVC-G593. The impact of this simplification on the HEVC coding performance is negligible while decreasing the complexity. It has been adopted and is implemented in the HM 6.1, hence also in the current HTM software. It is reported in this document that in the context of 3D video coding, the simplification of the candidate list construction has a non-negligible impact on the encoding performance of the side views (0.4% loss on average). The base view is unaffected, thus confirming that the simplification has a negligible impact on the HEVC coding performance.

- JCT2-A0133/m26123 CE5.h related: Reducing the coding cost of merge index by dynamic merge candidate list re-ordering [Thomas Guionnet, Laurent Guillo, Christine Guillemot (INRIA)]

HEVC implements a candidate vector list for merge and skip modes. When merge or skip modes are selected, a merge index is written in the bitstream. This index is first binarized using a unary code, then CABAC encoded. A CABAC context is dedicated to the first bin of the unary coded index while the remaining bins are considered as equiprobable. This strategy is efficient as long as the candidate list is constructed such as being ordered by decreasing index occurrence probability. In the context of 3D video encoding, an inter-view motion vector predictor is added at the first position of the candidate list according to the current HTM. It is reported in this document that the inter-view motion vector predictor is not always the most probable candidate since it generally depends on the video sequence characteristics. Therefore, a dynamic candidate vector list ordering is proposed. Coding gains of 0.1% on average are observed on side views and a maximum gain of 0.6% is obtained for the GTFly sequence view 2. It is proposed that this method be further studied in a CE.

- JCT2-A0134/m26124 CE5.h related: Merge candidate list extension for disparity compensated prediction [Thomas Guionnet, Laurent Guillo, Christine Guillemot (INRIA)]

The construction of the merge candidate list has been extensively studied in the JCT-VC group (e.g., see JCTVC-G039). It has been shown in JCTVC-I0293 that it is possible to improve the HEVC coding performance by adding copies in the merge list corresponding to the first candidate shifted by an arbitrary offset. This document proposes this approach for disparity compensation, whereby two new candidates are appended to the list. A gain of 0.4% is obtained on average for the dependent views with minimal impact on complexity. It is proposed that this method be further studied in a CE.

Discussion: There was a question on the status of JCTVC-I0293; it was mentioned that this proposal was not adopted in JCT-VC but that such techniques seem promising in the context of 3D video.

### Conclusions

Was discussed in BoG

JCT2-A0171 JCT-3V Break-Out Report : CE5.h Motion/mode Parameter Prediction [Sehoon Yea]

A BoG meeting on CE5.h was held Tuesday, July 18, 2012 from 19:00 through 21:00. Approximately 15 delegates were present. A summary of discussions and recommendations from the BoG is provided in this document.

The input contributions in CE5.h at this meeting can be categorized as follows:

  - Modification of Merge/AMVP-list construction: A0014, A0048, A0049, A0096, A0133, A0134

- A0049 (MediaTek) : achieves highest significant coding improvement among all the relevant proposals, simplification of the derivation of interview merge candidate while achieving 0.4% for the coded & synthesized cases.

- Other proposals regarding on Merge/AMVP are to be further studied in the CE.

- One opinion suggesting to separate the candidate-list construction for Merge and AMVP and continue the CE

- The BoG recommends to adopt the motion skip/merge candidate derivation in A0049.

  - DMDV (Depth Map Disparity Vector): A0013, A0031, A0047

- It was pointed out and agreed that the approaches in A0013 and A0047 are identical.

- No action recommended for the proposals in this category

  - NBDV (Neighboring Blocks Disparity Vector)

: A0097, A0126

- There seemed to be a general agreement among participants of the BoG that NBDV could provide substantial complexity reduction without coding loss.

- The BoG recommends adoption of NBDV into the 3D-HTM: Use NBDV (A0097) in CTC, A0126 proposes to further simplify the NBDV. The proponents will provide the corresponding text for the “normative specification annex” of the 3D-HTM.

Decision: Adopt. Remove DMDV from the 3D-HTM, keep as an option in the software (off by default).

Note: If used in future experiments, experiments using DMDV software should disable prediction between two dependent views, as e.g., proposed in A0049 and A0095.

## CE6.a: Depth intra prediction

### Summary

JCT2-A0162/m26268 CE6.a Summary Report: Depth Intra Prediction [M. M. Hannuksela (Nokia)]

The software integration of coding tools into the latest version of the ATM software was not finalized and hence no simulation results were obtained. There was a desire by the proponent to continue the CE.

### CE Contributions

### Conclusions

Discontinue CE

## CE6.h: Depth intra prediction

Breakout (Philipp Merkle) to report back Tue 19-21 Room 361

### Summary

- JCT2-A0104/m26061 3D-CE6.h summary report: Depth intra prediction [Philipp Merkle (Fraunhofer HHI)]

The goal of this core experiment is to test tools for depth intra prediction which are applied to HEVC compatible 3D video coding. The following summarizes key highlights and similarities of the proposal.

- JCT2-A0070 (LG) and JCT2-A0032 (Samsung) propose a depth intra coding method based on continuous contour partition of a block.

- JCT2-A0068 (LG) proposes a region-based intra prediction.

- JCT2-A0098 (Qualcomm) and JCT2-A0105 (HHI) propose methods for reducing complexity of the Wedgelet search used in DMM modes 1 and 3.

- JCT2-A0010 (Aachen) and JCT2-A0087 (LG) propose methods for depth intra coding without transmitting the residuum.

- JCT2-A0010 (Aachen) proposes a number of depth intra prediction modes together with depth look-up table based residual value coding. This approach enhances the deltaDC method used in the existing DMM.

- JCT2-A0088 (LG) proposes a skip mode for depth intra coding.

### CE Contributions

- JCT2-A0068/m26010 3D-CE6.h: Simple region-based intra prediction for depth-map [Jin Heo, EunYong Son, Sehoon Yea (LG Electronics)]

This contribution presents a region-based intra prediction method for depth map coding. The proposed method separates a block into two partitions using a straight line without any extra signalling bits by replacing the conventional intra prediction. The method is considered as a replacement for DMM modes 1 and 2. The proposal provides approximately 4% and 2.5% encoding and decoding complexity reduction with 0.1% BD-rate loss. It is asserted that the proposed method has two advantages: prediction mode reuse and less complexity compared to the existing DMM.

Discussion: It was suggested that some harmonization among the different proposals could be considered to achieve the complexity reductions and avoid the coding loss.

- JCT2-A0114/m26081 3D-CE6.h: Cross check on region-based intra prediction of LG [Hunn Rhee, Philipp Merkle (Fraunhofer HHI)]

- JCT2-A0070/m26012 3D-CE6.h: Region boundary chain coding for depth-map [Jin Heo, Eunyong Son, Sehoon Yea (LG Electronics)]

This contribution represents a region boundary chain coding method for the depth-map. The proposed method introduces an edge chain that separates the block into two regions and codes the chain explicitly. It is reported that the proposed method achieves a reduction of 0.2% in terms of BD-Rate.

Discussion: It was asked whether Intra only results are available. Proponents stated that the results could be made available during the week.

Discussion: It was asked where the gains come from relative to the existing DMM. It was stated by the proponents that the shape is determined directly from the depth signal and independent from the texture.

Additional mode for encoding the depth map contour in DMM (so far, arbitrary contours can only be predicted from the texture)

Gain is not very high, whereas it adds some additional decoder complexity (however no additional dependency of depth and texture)

Decision: Adopt to 3D-HTM (“normative” description currently incomplete)

General note (not about this proposal): More detailed investigation how much benefit we get from the low-level dependent coding of T+D may be worthwhile w.r.t potential complexity impact.

- JCT2-A0081/m26027 3D-CE6.h: Cross check on LG proposal [Byung Tae Oh (Samsung)] [miss] [late]

### Related Contributions

- JCT2-A0010/m25802 3D-CE6.h related: Model-based Intra Coding for Depth Maps using a Depth Lookup Table [Fabian Jäger (RWTH Aachen University)]

In this contribution a very similar prediction scheme as DMM is used to approximate the depth map’s sharp edges. With DMM the resulting residual is transformed and quantized as with conventional intra prediction modes. At this stage the proposed Depth Mode-based Coding (DMC) does not use transform and quantization to code the resulting residual signal. The DMC prediction stage always results in two to three depth segments per CTB and for each of these segments a single residual DC depth value is coded. By skipping the transform step and coding the residual based on the pixel-domain information, ringing artifacts are eliminated for DMC-coded blocks. Moreover, the number of bits to signal the residual values for each segment can be further reduced by integrating a Depth Lookup Table (DLT), which depth values maps to valid depth values of the original depth map. The DLT is constructed based on an initial analysis of the input depth map and is then coded in the SPS. For sequences with strongly quantized depth maps DLT yields additional gain on top of the gain coming from DMC alone.

The proposed method shows 0.76% coding gain with 110% encoder and 116% decoder complexity for the random access configuration and 1.46% coding gain with 109% encoder and 75% decoder complexity for the all-intra configuration.

The proposal suggests further study of DMC and DLT within a corresponding core experiments.

Discussion: combination of different rate points makes it different to compare the results. Also the similarity between proposed DMC and current DMM technology was mentioned, including comparable complexity.

- JCT2-A0030/m25891 3D-CE6: crosscheck of JCT2-A0010, "Model-based Intra Coding for Depth Maps using a Depth Lookup Table" [Imed Bouazizi, Giovanni Cordara, Lukasz Kondrad (Telecom Italia)] [miss] [late]

- JCT2-A0032/m25895 3D-CE6.h related results on depth intra prediction [Byung Tae Oh, Jaejoon Lee, Du Sik Park (Samsung)]

A depth intra prediction method based on plane segmentation is proposed to improve the coding efficiency of depth images. This method is similar to that adopted for ATM. The proposed mode replaces the existing DMM mode 2. The results report a 0.4% bit rate reduction with similar complexity as the anchor. Additionally, an alternative configuration in which the proposed method is tested together with DMM modes 1, 3 and 4 are also presented, but results for this configuration have not been cross-checked.

- JCT2-A0085/m26036 3D-CE6.h: Cross check on plane segmentation based intra prediction (PSIP) of Samsung [Jin Heo, Eunyong Son, Sehoon Yea (LG Electronics)]

- JCT2-A0087/m26039 3D-CE6.h related: Rate Distortion Optimized Selection between Non-Zero Residual and All-Zero Residual in Intra Coding of Depth [Hongbin Liu (LG Electronics)]

This contribution proposes to perform rate distortion optimized (RDO) selection between non-zero residual and all-zero residual in intra mode coding, and proposed method is only applied to inter-picture of depth. The proposed method is expected to save bits spent on residual and is claimed to have a minor impact on the quality of synthesized views. This method is employed at the encoder only and does not change any syntax. It is reported that proposed method can achieve 0.1% bit rate reduction on synthesized views as well as coded and synthesized views.

Discussion: Favorable remark on this approach and support to adopt as a non-normative contribution in the reference software.

Decision(SW): Adopt

- JCT2-A0127/m26110 Cross-check of JCT2-A0087 3D-CE6.h related: Rate Distortion Optimized Selection between Non-Zero Residual and All-Zero Residual in Intra Coding of Depth [Liang Zhao, Jicheng An, Shawmin Lei (MediaTek)] [late]

- JCT2-A0088/m26040 3D-CE6.h Intra Skip mode for depth map I-slices [Eunyong Son, Jin Heo, Hongbin Liu, Sehoon Yea (LG Electronics)]

This contribution presents a technique for intra skip mode for depth map I-slices. The proposed intra skip mode does not transmit residual data and only sends the intra prediction modes for depth map coding. The proposed method does not achieve any gains for the 3-view case, but achieves a 0.1% bit rate reduction for the 2-view case.

No support by other experts. No action.

- JCT2-A0082/m26029 3D-CE6.h: Cross check on depth intra skip of LG [Jin Young Lee, Kwan-Jung Oh (Samsung)]

- JCT2-A0098/m26053 3D-CE6.h related: Depth Modeling Mode (DMM) 3 simplification for HTM [X. Zhao, Y.Chen, L.Zhang, M. Karczewicz (Qualcomm)]

This contribution proposes a method to simplify DMM mode 3 in HTM, in which the Wedgelet pattern is not signalled, but derived by an exhaustive search from a large number of candidates at the decoder. In this proposal, it is firstly proposed that DMM mode 3 is disabled when the top-left 4×4 block in the Co-located Texture Luma Block (CTLB) is not coded as Intra. Furthermore, it is proposed that the Intra mode within the CTLB is used to derive the starting point for Wedgelet pattern searching with a fixed search range. It is asserted that the proposal significantly reduces the number of Wedgelet candidates to be searched at the decoder for DMM mode 3, without changing the coding efficiency of the current DMM. When DMM mode 3 is invoked, it was claimed that approximately 90% of the search patterns could be eliminated with the proposed approach.

Discussion: It would be interesting to also report Intra only results and check the impact on the proposed simplification.

- JCT2-A0094/m26049 Crosscheck report for JCT2-0098 3D-CE6.h related: Depth Modeling Mode (DMM) 3 simplification of Qualcomm [H. Liu (LG Electronics), J. Jia (LG Electronics)]

- JCT2-A0105/m26062 3D-CE6.h related: Fast Wedgelet search [Philipp Merkle, Hunn Rhee, Karsten Müller, Thomas Wiegand (Fraunhofer HHI)]

This contribution proposes a fast search method for the Wedgelet-based depth coding modes. It introduces a two-step search strategy as an alternative to the full search currently used in HTM 3.1. It is reported that this method reduces the encoder complexity to 95.7%; for the intra-only configuration, the encoder/decoder complexity reduces to 46.5%/78.3%. For instance, when DMM modes 1 and 3 are invoked, it was claimed that approximately 75% of the search patterns could be eliminated with the proposed approach for 32x32. For both configurations the BD-rate increases negligibly by less than 0.1%.

- JCT2-A0084/m26035 3D-CE6.h: Cross check on fast wedgelet search of HHI [Jin Heo, Eunyong Son, Sehoon Yea (LG Electronics)]

### Conclusions

Several proposals improve PSNR of synthesized views. Subjective tests whether the improved depth intra coding produces visual artifacts?

Review was performed in BoG

JCT2-A0173 BoG Report on HEVC-based depth intra prediction (CE6.h) [Philipp Merkle]

This BoG was formed to consider potential adoption and discuss future CE plans for CE6.h on depth intra prediction.

The BoG meeting took place on July 17, 2012 from 7pm to 8:30pm with about 8 participants including representatives for all proposals. The following recommendations were made:

A0068: Further study in CE as joint proposal with HHI, harmonizing the method described in JCT2-A0068 with DMM mode 2 methods.

A0070: consider adoption of JCT2-A0070.

A0010: Further study in CE as the following two proposals

- Depth lookup table (DLT) method described in JCT2-A0010 as alternative to current deltaDC method for DMM prediction modes.

- DLT method together DMC prediction modes according to JCT2-A0010.

A0032: Further study PSIP method described in JCT2-A0032 in CE as alternative to current DMM 2 prediction method, identifying R-D performance and complexity impact.

A0087 (non-normative): consider adoption of JCT2-A0087.

A0088: consider adoption, if the syntax changes are small and well understood, having no impact on other things.

A0098 and A0105: Further study in CE as combined proposal together with JCT2-A0105.

## CE7: Global depth and view prediction (TBR)

### Summary

- JCT2-A0071/m26013 3DV CE7 summary on Global Depth and View Prediction [Takanori Senoh (NICT)]

The goal of this CE is to investigate Global Depth and View Prediction (GDVP) based on a concept of Epipolar Plane Image (EPI) of depth and view, which is applicable to both AVC and HEVC-based codecs. The summary report indicates that one CE contribution has been submitted (JCT2-A0069) and summarizes the main result. A number of objective and subjective results are presented. The summary report recommends that a test model based on CE7 be established.

Discussion: It was remarked that the framework is quite different from that of other CEs and the numerical results reported cannot be easily compared or interpreted.

Discussion: There was a question on what would be standardized. The codec that is utilized in this framework is the existing AVC or HEVC-based codec, but the input format and pre/post-processing operations are different.

### CE Contributions

- JCT2-A0069/m26011 3DV CE7.h results on Global Depth and View Prediction by NICT [T. Senoh, K. Yamamoto, R. Oi, Y. Ichihashi (NICT)]

This contribution reviewed the details of the pre-processing operations to compute the global view and global depth and obtain the residual data, as well as the post-processing operations that are required to synthesize views from the reconstructed data. It was noted that the results are available for viewing and that anchors at comparable bit rates are also available.

Discussion: It was suggested that others not directly involved in this work are encouraged to cross-check the results.

- JCT2-A0059/m25945 Cross Check of 3D-CE7.h Results on Global Depth and View Prediction of NICT by NISRI [Kazuyoshi Suzuki, Masayuki Tanimoto (NISRI)]

JCT2-A0168 Additional Cross Check of 3D-CE7.h Results on Global Depth and View Prediction of NICT by Poznan [Krzysztof Wegner, Olgierd Stankiewicz (Poznan Univ)] [late]

### Related Contributions

- JCT2-A0072/m26014 Proposal on Simple Test Model for 3DV Coding based on CE7 [T. Senoh, K. Yamamoto, R. Oi, Y. Ichihashi (NICT), M. Tanimoto, K. Suzuki (NISRI)]

The proposed test model includes (a) global view that is compatible with 2D codec, (b) global depth that is formed based on the depth maps from individual viewpoints, and (c) residual data, which is necessary for view synthesis. It is suggested that all components of the data format be coded in an access unit of AVC or HEVC, but these details have not been specified or implemented.

Discussion: It was suggested that perhaps this coding format could specified as an extension of MPEG-C Part 3 in which the residual information is added to the existing 2D+Depth format. Some additional signaling would need to be specified and informative annexes could provide descriptions of the pre-processing and post-processing operations.

### Conclusions

Viewing of CE7 results was performed Tuesday evening.

JCT2-A0172 3DV CE7h Subjective Test Results on Global Depth and View Prediction [Takanori Senoh, M. Tanimoto]

3DV CE7h subjective test results are summarized. 3DV-CE7h on Global Depth and View Prediction were subjectively evaluated by 10 subjects. The results show almost the same quality on average comparing to the HTMv.3.1 Anchor. In detail, six synthesized sequences (S1, S2, S3, S4, S5, S6) showed almost same quality at the same bitrates. One sequence (S8) showed slightly worse quality at all bitrate but will be improved. Together with this results, since CE7 is simple and fast, it is recommended to develop Simple 3DV coding.

Test sequences of 069 were tested

What is transmitted:

- Base view with full resolution

- Global depth in terms of camera parameters

- Residuals of dependent view(s) combined into one stream (H/V subsampled)

Does not require any normative specification in HTM, but can likewise be used with ATM (or any other codec)

- Some commonality with VS pred., with specific constraint that the depth map is predicted from camera parameters, and that residuals are encoded by reduced resolution.

Q: Is there need to specify the reconstruction of the dependent views (according to proponent, this does not need to be normative)?

Further study, potential to express such formats via SEI message? Or MAF?

Discontinue CE.

Establish AHG on study of alternative 3D formats.

Would this require normative pre- and post-processing to recover the views?

## CE8: RD Optimization

### Summary

- JCT2-A0077/m26022 CE8 Summary Report: RD-Optimization [Sehoon Yea (LG Electronics)]

CE8 was established to investigate quality fluctuations at various intermediate view positions and using different rendering algorithms when the view synthesis optimization (VSO) is enabled. Several proposals were also integrated into the reference software for evaluation with the aim to ultimately harmonize them. The summary report notes that five proposals have been submitted:

- JCT2-A0033 (Samsung/HHI)

- JCT2-A0057 (Zhejiang)

- JCT2-A0083 (PKU/LG)

- JCT2-A0093 (Samsung/HHI/LG/PKU)

- JCT2-A0119 (LG/Yonsei)

### CE Contributions

- JCT2-A0033/m25896 3D-CE8.h results on view synthesis optimization [Byung Tae Oh, Jaejoon Lee, Du Sik Park (Samsung), Gerhard Tech, Karsten Müller, Thomas Wiegand (Fraunhofer HHI)]

Modifications of VSO as reported in m24830 and m24865 are proposed to reduce encoder runtime and increase coding gains. Specifically, the following modifications are suggested: (a) run-time optimization of VSO algorithm implementation, (b) increase of number of views regarded for error computation by a factor of three, and (c) model-based view synthesis distortion estimation, i.e., the synthesis view distortion is computed without view rendering. It is reported that this configuration shows a 2.7% bit rate reduction on synthesized views with 90% encoding time.

Additionally an alternative VSO configuration using further modification to decrease runtime is presented. Specifically, the following changes were added: (d) distortion computation for luminance components only, and (e) no rendering model update for already coded blocks, i.e., the synthesized view distortion change is computed assuming uncoded depth data in all blocks apart from the depth block to be tested. It is reported that this configuration shows a 1.3% bit rate reduction on synthesized views with 73.6% decoding time. It was noted that results for this configuration have not been cross-checked.

The recommendation from the proponents is to adopt the first configuration since it is believed to offer a more favorable tradeoff in terms of coding efficiency and complexity reduction.

Decision(SW): Adopt (a)-(c) (CTC)

JCT2-A0057/m25935 3D-CE8.h results on JRDO [Li Wang, Lu Yu (Zhejiang Univ.)] [late]

This document evaluates a proposed Joint RDO (JRDO) scheme that has been implemented in HTM 3.1 and has been evaluated with two view synthesis tools (VSRS-1D-Fast and VSRS 3.5). This contribution proposes to harmonize the proposed JRDO and existing VSO scheme. The harmonized scheme reportedly achieves better coding efficiency than JRDO and VSO using both the VSRS and VSRS-1D-Fast rendering software modules. Further study of JRDO and the harmonized approach is suggested.

Discussion: It was asked whether complexity has been measured. The proponents noted the run times are not currently available, but will be provided in the future.

Further study

- JCT2-A0083/m26034 CE8.h: Results of Simplification of View Synthesis Optimization by Detection of Zero Distortion Change in Synthesized View [S. Wang (Peking Univ.), S. Ma (Peking Univ.), H. Liu (LG Electronics), J. Jia (LG Electronics)]

This contribution proposes a simplification of view synthesis optimization (VSO) method by detection of zero distortion change in synthesized view. In the proposed scheme, three aspects are taken into account: smoothness of texture, information of occlusion and mapping from depth to disparity. It reports average of 84% encoding time with 0.2 % BD-rate increase on average.

Discussion: What are the reasons for BD rate increase?

No action

- JCT2-A0135/m26125 3D-CE8.h: Cross check on simplification of view synthesis optimization of PKU and LG [L. Zhang (Qualcomm)] [late]

JCT2-A0123 3DV-CE8.h: Cross-check of LG’s & Peking University’s proposal on View Synthesis Optimization for 3DV-HTM by Nokia [Michal Joachimiak, Dmytro Rusanovskyy (Nokia)] [late]

- JCT2-A0093/m26048 3D-CE8.h results on view synthesis optimization by Samsung, HHI and LG-PKU [H. Liu (LG Electronics)]

The proposal reports improvements on top of A0033. It reports 2.6% and 2.2% bit rate reduction for synthesized and coded synthesized respectively with 77% encoding time.

Decision(SW): Adopt (CTC)

JCT2-A0167 3D-CE8.h: cross check report of view synthesis optimization by Samsumg, HHI and LG-PKU (JCT2-A0093) [Yichen Zhang, Lu Yu (Zhejiang University)] [late]

### Related Contributions

- JCT2-A0119/m26106 CE8.h Depth distortion metric with a weighted depth fidelity term [Jiwook Jung, Sehoon Yea (LG), Seungchul Ryu, Kwanghoon Sohn (Yonsei Univ.)]

Under the Common Test Conditions (CTC) of 3D Video Coding, the quality of synthesized views at specified viewing positions has been used as a sole measure of coded depth fidelity, with which RD-mode decision was performed at the encoder when coding depth. In the current reference encoder implementation, a technique called VSO is being used to synthesize the intermediate views and calculate the distortion of coded depth indirectly in terms of the quality of those synthesized views.

In m25010, it was pointed out that use of such an indirect distortion metric for depth coding may introduce undesirable quality fluctuation in the synthesized views along intermediate viewing positions as well as noticeable artifacts in synthesized views when alternative view-synthesis algorithms, such as VSRS, are used at the receiver. It was asserted that the primary reason for such problems is that the fidelity of coded depth was not sufficiently high, especially around object boundaries where spurious depth values could often be created in the coded depth due to the inherent nature of the current depth coding metric.

This contribution proposes to add a depth-fidelity term in the distortion metric currently used for RD-optimization at the encoder for depth coding. Comparisons between the cases with and without the proposed depth-fidelity term are provided to highlight how the proposed change can mitigate the aforementioned problems. Using VSRS-1D-Fast, it is reported that an average bit rate increase on synthesized views is 1.6%. Using VSRS3.5, an average bit rate increase on synthesized views is 0.9%. A maximum bit rate reduction of 8.8% and 3.7% was reported for the Undo_Dancer and GT_Fly sequences, respectively. Subjective results were shown for sample frames that indicated cleaner depth maps and improved subjective quality improvements in the rendered views when VSRS software was used for rendering.

The recommendation from the proponent is to adopt the proposed depth fidelity term into the HTM reference software as an encoder only option.

Discussion: Favorable remark to support adding this option.

Discussion: It was asked whether there are examples available for other sequences. The proponent noted that complete results could be provided for viewing if there was interest within the group.

Decision(SW): Adopt (as non-default option, not CTC)

### Conclusions

Breakout group (G. Tech) to come with a suggestion how to modify VSO.

JCT2-A0176 BoG Report on HEVC-based RD Optimization (CE8.h) [Gerhard Tech]

The BOG suggests

- to adopt proposals A0033 (a)-(c) and A0093 in HTM software and for CTC.

- to adopt proposals A0119 to HTM software for further studies.

- to further study harmonization of A0057 with adopted changes.

Several participants recommended to use modifications proposed in A0119 already now for some CEs as an additional option.  Some participants recommended using different renderers for evaluation of CE8.

Continue CE8; note: in principle, this could also be studied in an AHG in future, e.g. in relation with other quality metrics.

# Non-CE Technical Contributions

## 3DV Standard Development

### AVC related

JCT2-A0074/m26018 3DV-ATM HP: Proposed HRD text for ISO/IEC 14496-10:2012/PDAM 2 MVC extensions for inclusion of depth maps [S. Hattori, T. Suzuki (Sony), Y. Chen (Qualcomm), A. Vetro (MERL)]

(initial discussion in HL syntax AHG)

This document proposes text on HRD for ISO/IEC 14496-10:2012/PDAM 2 MVC extensions for inclusion of depth maps [MPEG N12731]. The proposal extends current Annex C in AVC for Annex I in MVC extensions for inclusion of depth maps [MPEG N12731] with similar concept of extending process applied for MVC.

The proposed HRD in Annex J follows the following design principles:

- When the decoding process specified in Annex I is applied, the process specified for Annex H in Annex C is invoked for particular texture view and depth view with view order index VOIdx.

- During the invocation of the process for a particular texture view, only the texture view components of the particular view are considered and during the invocation of the process for a particular depth view, only the depth view components of the particular view are considered.

- When the coded video sequence conforms to a profile specified in Annex I and the decoding process specified in Annex I is used, the view components of all the target output views of a picture are output at the same time instant and in increasing order of the view order index VOIdx.

- Inside each view component of the current primary coded picture, depth view component if present, is processed after the texture view component within the same view component.

- The same setting of OutputFlag applies to the texture view component and depth view component wit same view order index VOIdx within the same view component.

This proposal in principle applies the same process as in annex H to texture and depth (same view_id) independently in Annex I.

This proposal does not consider the problem when an MVC decoder gets a 3DV bitstream, which is addressed in JCTVC-A0002. Proposal is claimed to be independent of JCTVC-A0002.

Straightforward extension of HRD to “MV+D” case. Proposal was reviewed in AHG on Sunday. In general, the opinion is that this is straightforward, specifying the processes for the decoding and output of texture and depth data, common CPB and DPB management, imposing some constraints e.g. simultaneous output of texture and depth.

Decision: Adopt

JCT2-A0075/m26019 3DV-ATM HP Technical Input : A study on depth range based weighted prediction tool [S. Hattori, T. Suzuki (Sony)]

(initial discussion in HL syntax AHG)

This document reports a comparison between depth range based weighted prediction (DBWP) and explicit weighted prediction (EWP) in AVC. In this document, we compared coding efficiency of both methods with 3D-ATM software. The experimental result shows that coding efficiency of each method is almost the same.

Explicit weighted prediction (as in the AVC) and DBWP (as in the current ATM software) are compared.

When explicit weighted prediction is used, the calculation of the weights can be done only at the encoder as it is now done at the decoder.  It is noted that such calculations of the weights are not needed if explicit weights are signalled.

When explicit weighted prediction is used, depth range parameters in the SPS and depth range update NAL unit are not needed, although the current software implementation still have them present in the bitstream.

Coding loss for GT_fly sequence is about 0.77% for depth only. There is no loss for the other sequences since the depth range of the other sequences keeps unchanged. Results based on synthesized views are reported as 0.08% for GT_fly.

It is commented that the PSNR might be kept unchanged in the proposal compared to the anchor, but this might require some further confirmation from the proponent.

Overhead of explicit weighted prediction might be higher for the case of multiple slices

Gain only for one sequence – is there sufficient justification for introducing a modification?

It is argued that for other sequences with larger depth variation (znear/zfar) this would be different.

Some existing hardware implementations may implement the signalling of weighted prediction hardwired.

The current DBWP may not work for other kinds of depth representation.

In the MV+D standard, normative description of the depth map value interpretation (i.e. mapping the binary values to actual depth data) is not desirable, as it may be application dependent (e.g. view synthesis). It should be expressed only by SEI message. Therefore, the meaning of the depth map e.g. znear/zfar should not have influence on the decoding process.

Decision:

- Remove DBWP from draft for MV+D, use explicit WP.

- Remove znear/zfar from SPS, and discard depth range update NAL unit type.

Note: This would be different in the context of the standardization of “AVC plus depth” extension, as there the depth map is used in texture decoding process, and it is likely that changes at the sub-slice leval are done anyway.

Replace DBWP by explicit WP in common test conditions for MVC&AVC depth extensions.

- JCT2-A0144/m26157 3DV technical input: JMVC software integration of coding interlace texture with interlace depth [Ching-Chieh Lin, Fang-Chu Chen, Wen-Hao Chung (Industrial Technology Research Institute)] [late]

The proposal presents a modified JMVC SW to include interlaced depth coding. Here, a AU for Interlaced texture plus interlaced depth is suggested. Currently, only progressive depth data is permitted in the simple MVC extension.

At the encoder, an InterlacedDepth flag is added to the configuration file. At the decoder, the PicBufferListSize is increased to accommodate interlaced depth decoding. No syntax changes are required and therefore, the contribution only refers to software modification

The proposal recommends adopting the modifications to JMVC.

Discussion: Texture and depth need to be temporally aligned. Proposal is usefully for further study.

Agreement: The current constraint of only progressive depth maps should be given up. Following combinations should be allowed:

- Progressive texture plus progressive depth at same frame rate

- Interlaced texture plus progressive depth (Q: Which combinations of texture/depth frame/field rates are currently allowed? Texture field rate = 1x|2x Depth frame rate? )

- Interlaced texture plus interlaced depth at same field rate.

Provide text for MV+D draft. Done.

JCT2-A0170 Proposed modifications to MVC+depth WD to include interlaced depth [Fang-Chu Chen, Ching-Chieh Lin (??)] [late]

Decision(SW): Adopt for JMVC

- JCT2-A0164/m26282 3DV: Cross check report of ITRI’s JMVC software integration of coding interlaced texture with interlaced depth (JCT2-A0144) [Chao-Hsiung Hung, Hsueh-Ming Hang (NCTU)] [late]

Draft MV+D and ISO/IEC DoC were reviewed Tue afternoon. Editors will make the draft available as input doc for further review by other experts.

JCT2-A0174 Draft Text of ISO/IEC 14496-10:2012/DAM 2 MVC extensions for inclusion of depth maps [T. Suzuki (Sony), M. Hannuksela (Nokia), Y. Chen (Qualcomm), S. Hattori (Sony)]

Tentative editing period for DAM: Jul 30.

### HEVC related

- JCT2-A0012/m25840 Tentative working draft text for the HEVC compatible 3D video coding extension [Gerhard Tech, Philipp Merkle, Martin Winken, Heiko Schwarz, Karsten Mueller, Thomas Wiegand (Fraunhofer HHI)]

According to the MPEG document N12434 the MPEG 3D video group expected the availability of a working draft for HEVC compatible 3D video coding extension early in 2012. The attached document is kindly offered to the Joint Collaborative Team on 3D Video Coding Extension Development as starting point for discussion and text specification.

The document is based on the document "High efficiency video coding (HEVC) text specification draft 7" (JCTVC-I1003_d1). In Annex F, it contains a preliminary and incomplete specification of most of the 3DV coding tools currently adopted in the HTM version 3.1.

It includes preliminary description of motion vector inheritance, inter-view residual prediction, inter-view motion vector prediction, depth modelling modes, inter-view prediction and camera parameters.

The contribution proposes to use the attached document as a starting point for the working draft for HEVC-3D extension standardisation according to document N12434.

Decisions:

Develop a draft text of “MV-HEVC” (extension of HEVC to multi-view without low-level changes, without depth)

Editors: G. Tech, K. Wegner, Y. Chen.

All other parts of A0012 to be included in the 3D-HTM description, as annex for precise description of the “normative” elements.

## Clarification and Bug Fix Issues

### AVC related

### HEVC related

## Test model settings and common test conditions

### General

- JCT2-A0073/m26016 Proposal of Depth-aware 3DV Test Condition and Evaluation [Takanori Senoh, Kenji Yamamoto, Ryutaro Oi, Yasuyuki Ichihashi (NICT), Masayuki Tanimoto, Kazuyoshi Suzuki (NISRI)]

This contribution requests a depth-aware test condition and evaluation method for 3DV that provides depth maps of higher coding quality, especially for HEVC-based model. It is asserted that the currently decoded depth map quality, even at the highest bit rate, appear unacceptable for emerging 3D applications. It was suggested that to increase the longevity of our standards, the codec should be specified with greater consideration for depth map quality. To achieve this goal, it is proposed that the Common Test Conditions use the same QP values for both texture and depth map, and that the BD-Rate for depth maps also be evaluated in the evaluation reports. The contribution also suggests that down-scaling the depth maps with finer quantization provides a better trade-off in terms of depth map quality and bit rate, compared to coding at a coarse quantization at the full resolution.

Discussion: It was asked whether the quantization used by the codec is the main reason for degradation, or the source or optimizations prior to encoding lead to greater errors. It was remarked that main reason for degradation is the view synthesis optimization.

Discussion: It was noted that it has been a challenge to implement full-resolution texture coding with reduced-resolution depth in the HTM software. NICT mentioned that they have a software implementation that achieves this and could make it available to group members upon request. However, the gains of reduced-resolution depth map coding in HTM may not be as large as that in ATM. The benefits would need to be confirmed before this capability could be enabled.

Discussion: It was asked whether there is a need for a lossless or near-lossless mode for depth. It was remarked the current coding tools may be sufficient.

### AVC related

### HEVC related

- JCT2-A0090/m26046 Impact of View Synthesis Optimization (VSO) on depth quality [Olgierd Stankiewicz, Krzysztof Wegner, Marek Domanski (Poznan Univ. of Technology)]

This contribution considers the impact of VSO on depth quality. It is noted that VSO allows the encoder to discard depth information which appears to be irrelevant for the view synthesis process for the purpose of rate reduction. However, it is asserted that high-fidelity depth maps are required for a number of applications. Moreover, future encoders may not implement VSO or may have VSO implemented differently, and thus the current CTC may not resemble practical solutions. It is reported that VSO achieves approximately 12% bit rate reduction on synthesized views, but there is approximately 7dB drop in BD-PSNR for the depth data. It is suggested that the loss of depth information is not acceptable for some applications. Examples were shown that illustrate the serious degradation in the depth maps with VSO enabled. Several options were suggested: (a) disable VSO in some CEs related to depth coding only; (b) disable VSO in CTC to allow good evaluation of depth coding tools; or (c) use two variants: VSO enabled and disabled, to allow a wider perspective and is analogous to RDQ in HEVC.

Discussion: It was remarked by several members of the group that alternative metrics to improve the depth fidelity can be considered as part of the optimization process. It was suggested that this may be a better path than disabling the tool so that the gains can be maintained and the depth quality can also be preserved.

Discussion: It was also remarked that the proposed solution to evaluate all proposals with VSO enabled and disabled would require twice the number of encoding runs, which would be an additional burden for proponents.

Discussion: The group seemed more inclined to allow for VSO to be disabled in select CEs, in particular for those CEs that are evaluating the coding efficiency for depth maps.

Discussion: It was the consensus of the group to pursue improvements to VSO (e.g., as proposed in CE8) and allow CEs to also evaluate proposals with VSO disabled if there may be benefits in the evaluation of the tools being tested.

- JCT2-A0091/m26047 Independent intra-period coding in 3D-HTM [Jakub Siast, Olgierd Stankiewicz, Krzysztof Wegner, Marek Domanski (Poznan Univ. of Technology)]

This contribution proposed an HTM encoder software modification that aims to enhance the experimentation process in terms of parallel intra-periods and fast debugging functionality. Specifically, it is proposed to parallelize the encoding of the multiview video sequence by splitting the sequence into multiple runs, each with a different IntraPeriod. Each IntraPeriod would be encoded independently (in parallel) and then the results would be merged into a single concatenated bitstream. The resultant bitstream should be binary identical to the one produced with a single-pass of the encoder (without parallelization). The contribution also suggests an approach for verification of the deterministic operation of the encoder, and allowing stand-alone encoding/decoding of a selected IntraPeriod for debugging for easier tracking of errors.

Discussion: It was asked whether the proposed change impact the encoder only. The proponents confirmed that the resulting bitstream would be identical.

Discussion: The main change that is needed to the software is to ensure that CABAC is properly initialized at random access points in the dependent views so that each IntraPeriod can be independently encoded.

Discussion: It was remarked that some cross-checking of the software and reproduction of the results with parallel encoding would be desirable before agreeing to adopt the proposed changes.

- JCT2-A0113/m26075 On coding configurations for 3DV-HTM development [Dmytro Rusanovskyy (Nokia)] [late]

This contribution proposes a change to the HTM coding configuration that enables the generation of a bitstream with only high-level syntax changes to HEVC (i.e., slice-level and above). The contribution also proposes to update the Common Test Conditions document to allow comprehensive evaluation of tools targeting such configuration.

The contribution also provides simulation results with such a coding configuration, indicating about 8% bit rate increase on average for coded and synthesized views when compared to the 3DV-HTM according to the Common Test Conditions.

Discussion: The remarks from the group indicated that it is not necessary to impose such a configuration in the CTC. It would only be needed for proposed tools that target improvements to the simple multiview extension of HEVC.

## Test model coding performance

### General

- JCT2-A0150/m25818 3DV: Quality assessment of stereo pairs formed from two synthesized views [Philippe Hanhart (EPFL), Touradj Ebrahimi (EPFL), on behalf of Qualinet]

In this contribution, the objective quality assessment of stereo pairs formed from two synthesized views is investigated. State-of-the-art 2D quality metrics are benchmarked using as ground truth a set of subjective data, which has been collected during the formal evaluation test campaign of the proposals on 3D Video Compression Technology. It is reported that some of the considered metrics, such as VIFp, VQM, MS-SSIM, and SSIM, are more correlated with perceived quality than PSNR for the scenario under analysis.

For synthesized views in 3-view scenario, same metrics are useful that were shown before to be useful for the 2-view scenarios.

Do we currently have software to compute such metrics?

AHG on objective quality metrics (Touradj, Anthony)

### AVC related

### HEVC related

## Source video test material

## High-level syntax related contributions

### MVC related

#### Sequence parameter set

- JCT2-A0100/m26055 3D-HLS: Signalling number of to-be-decoded texture/depth views for MVC based 3DV [Y. Chen, Y.-K. Wang (Qualcomm)]

In MVC, for each operation point, the number of views to be decoded is signalled in the MVC SPS extension (by the syntax element applicable_op_num_views_minus1[ i ][ j ]). In MVC based 3DV, the semantics of this syntax element becomes ambiguous and insufficient, since the current PDAM allows a depth view to have a different view dependency than that of the texture view. It is proposed that applicable_op_num_views_minus1[ i ][ j ] is used to signal the number of texture views to be decoded while the number of depth views to be decoded for each operation point is further signalled by another syntax element in the subset SPS.

The proposal is to indicate the number of texture and depth view in the SPS MVC extension for 3DV operation points.

The issue with current MVC based 3DV spec.: the same syntax as SPS MVC extension for 3DV which is not clear if the target views are for texture or depth or texture and depth.

The number of depth views is additionally signalled in the 3DV SPS extension, as a bug fix since the number of depth views to be decoded is not clear in the current MVC extension.

3DV SPS extension will include target output view for texture and depth. The new proposed syntax will only be used to indicate operation points containing depth views.

Discussion: there was positive support for the proposal.

Decision: Adopt.

Note: The current spec imposes that in case of video plus depth three different SPS must be sent:

- One for base view (backward compatibility with AVC)

- One for multi-view texture (backward compatibility with MVC)

- One for texture plus depth

#### HRD and bitstream extraction

JCT2-A0002/m25751 3DV-ATM HP high-level syntax: HRD parameters for texture sub-bitstreams [M. M. Hannuksela (Nokia)]

It is asserted that the semantics of the MVC video usability information extension and the MVC scalable nesting SEI message used to carry picture timing and buffering period SEI messages rely on the sub-bitstream extraction process specified in subclause H.8.5.3, which treats NAL units of nal_unit_type equal to 21 as non-VCL NAL units and does not perform temporal_id and view_id based extraction for them. Hence, it is asserted that no proper HRD parameters can be conveyed for sub-bitstreams consisting of texture views only and therefore the contribution proposes a mechanism to convey HRD parameters of VUI as well as picture timing and buffering period SEI messages for the sub-bitstream that consists only of texture views while the depth views are removed by the sub-bitstream extraction process of subclause I.8.5.3. It is proposed that the texture sub-bitstream HRD parameters are conveyed in a second instance of mvc_vui_parameters_extension( ) in a 3DVC sequence parameter set and picture timing and buffering period SEI messages are conveyed in a specific 3DVC texture sub-bitstream HRD nesting SEI message. If a texture sub-bitstream is extracted using the sub-bitstream extraction process, these nested HRD parameters and SEI messages replace the respective MVC ones, which, as stated above, assume the presence of NAL units of nal_unit_type 21 as non-VCL NAL units.

In addition, many related smaller changes are proposed including the following:

- Annex C (Hypothetical Reference Decoder) did not refer to Annex I or subclauses of Annex I. Proper references are to Annex I are now added.

- A clarification of the definition of term view, which proposed to be defined as: unless explicitly limited to either texture view or depth view, a texture view and a depth view of the same value of view_id.

- The use of SEI messages with and without the various nesting SEI message is clarified in subclause I.13.2.

- sei_op_view_applicability_flag[ i ] is removed from 3DV scalable nesting SEI message, because the 3DV operation points always include both texture and depth as specified in I.8.

A clarification of the definition of term base view, and an introduction of two new related terms base texture view and base depth view.

MVC extractor doesn’t extract NAL unit type 21 out because the MVC bit-stream extraction process doesn’t extract out non-VCL NAL units. Such non-VCL NAL units include depth slices which have NAL unit type 21.

Syntax elements as part of HRD contained in the SPS 3DVC extension are not accurate.

It is mentioned by the proponent that 3DVC bitstream extractor could be used to extract texture sub-bitsteams.

Two solutions complementary to each other are proposed:

Proposal part 1: Two sets of HRD parameters in the 3DVC SPS extension

- One set is for the texture ONLY

- The other set is for each 3DV operation point (containing both texture and depth).

Proposal part 2: A new type of the nesting SEI message for the carriage of picture timing and buffering period for texture only sub-bitstreams is proposed.

It was commented that the solutions seem to solve valid known problems, due to the lack of forward-compatibility of the AVC/MVC design.

In the current design, HRD parameters are commonly used for video and depth. Therefore, the MVC texture-only decoder operates on assumption and constraints of the entire bitstream, even though it only decodes a part of it (texture). The proposal is to use different HRD parameter sets for the MVC decoder (backward compatible) and for the MVC plus depth decoder.

Q: Would the design support mono view plus depth? In principle yes (num views = 1), but there would be some redundancy in HL syntax (i.e. the SPS and HRD for the MVC stage are sent though not needed). That may be acceptable.

Decision: Adopt proposal part 1.About part 2: Agreed that this is needed, but rather than defining a new SEI message it would be more consistent to extend the scalability nesting SEI message (Miska provides text update).

#### Slice header

JCT2-A0112/m26074 Calculation process for parameters of Depth Range-based Weighted Prediction with fixed-point/integer operations Dmytro Rusanovskyy (Nokia)] [late]

Parameters of Depth Range-based Weighted Prediction (DRWP) in 3DV-ATM are currently implemented with floating point operations. This contribution proposes an integer (fixed-point) implementation for this process. Presented in this document results show that proposed implementation can be deployed with insignificant penalty on the coding gain: 0.03% of dBR on total bitrate reduction for GT_Fly test sequence.

This proposal contains two aspects:

The first aspect evaluates the DRWP. 5.6% loss for depth views only was reported when DRWP is disabled, for the GT_fly sequence, which is the only sequence that this technique applies to. The loss leads to 0.2% loss for synthesized views was reported when DRWP is disabled (for GT_fly sequence).

The second aspect simplifies the DRWP with integer based operations. The integer based DRWP introduces only negligible loss.

In the integer based DRWP, all operations, including divisions, can be done with 32-bit accuracy.

Floating point calculations are only done once per slice. However, it is commented that this is the only place in the 3DV spec. that uses the floating point calculations.

It was mentioned that JNB had a comment which suggests changing the floating point to fixed-point calculation during DRWP.

It was commented that this proposal is related to JCT2-A0075.

Due to the removal of DBWP from the MV+D DAM, this is only relevant for the 3D-AVC draft. Appears to be a desirable simplification.

Decision: Adopt to 3D-AVC draft, text will be provided after the meeting.

#### Depth range signaling (TBR)

Note: Fin NB requests that the quantization of DM be specified (would be done as SEI message out of the reasons noted under A0075)

JCT2-A0007/m25780 3D-AVC HLS: Depth Range Parameter Set for MVC Compatible 3D Video Coding [M. Li, P. Wu (ZTE)]

In this document, the concept and operations of parameter set are proposed to be applied to coding depth range parameters for MVC compatible 3D video coding. Therefore, depth range parameters can be effectively shared among different access units by slices’ referring to depth range parameter set (DRPS), which saves bits from repeatedly generating depth range update RBSPs for different access units using the same depth range parameters in decoding.

It was commented that this proposal is related to JCT2-A0075.

Duplications of depth range parameters might be needed when they are identical. It is mentioned that prediction scheme is allowed in the current design, but there might still be slight overhead for NAL unit header and several syntax elements indicating the prediction.

The prediction scheme might not be robust, since the current design enables prediction between access units.

It is proposed that the DRPS can be designed similar to APS (in HEVC).

- A parameter set id is added.

- A reference id of the DRPS is signalled to replace frame_num, so prediction of the DRPSes is still enabled.

- The id equal to zero is reserved for the depth range in SPS. It is commented that the id of the DRPS can be signalled directly in the slice header.

A comment was given that this proposal is giving improvements in terms of error resiliency although detailed design needs to be carefully studied.

Not needed for MV+D draft.

Current approach is to transmit the depth range parameters (and some other parameters necessary for VSP) are in a separate NAL unit type, which has some error resilience problem. By specifying it as new parameter set type, this issue would be resolved.

These data could change frequently (once per picture could happen if the camera is moving or in case of synthetic sequences (GT Fly))

Decision: Adopt for 3D-AVC draft as it solves the current problem, but seek for further improvements

For example, further modifications are necessary w.r.t. activation process, which does not appear necessary for both texture and depth views (no modifications to this during editing period, if not suggested by an input brought to the current meeting).

General question to be further investigated: Is it worthwhile to define a new type of parameter set and new NAL type for that single purpose? What are benefits compared to other solutions, e.g. sending depth range data in case of changes in the slice header (with flag that signals its presence)? It is said that this may have disadvantages in case of data losses.

JCT2-A0053/m25930 3DV-ATM HP/EHP HLS：Camera Parameter Set (CPS) for camera parameter signaling [P. Lu, Y. Zhao, L. Yu (Zhejiang University)]

This proposal modifies the mechanism of camera parameter signaling both in MVC based 3DV and AVC based 3DV. The main characteristics of the proposed method can be summarized as follows:

	This proposal applies the concept of parameter set on camera parameters. Camera parameters are included in Sequence Parameter Set (SPS) 3DVC extension and Camera Parameter Set (CPS).

	Slice can refer to CPS by cam_parameter_set_id in slice header.

	Each CPS contains the camera parameters of one or more access unit.

	The temporal prediction among camera parameters is only carried out within CPS.

	CPS may use the camera parameters in SPS 3DVC extension as references to do temporal prediction among camera parameters.

It is claimed by the proponent that this proposal is similar to JCT2-A0007. It was commented that the proposal might be a bit over optimized for bit-rate saving. It aims to resolve the same problem of error resilience for the signaling of some camera parameters, especially depth range.

Not needed for MV+D draft.

First part: Information about camera parameters is identical in spirit to the already adopted  DRPS of A0007, but the suggested text is slightly different.

Second part targets for data reduction in camera parameters. Overall bitrate saving? Further study about the potential benefit.

Dependent encoding of parameter sets is usually not desirable, decoding should be as simple as possible.

No action currently.

JCT2-A0166/m26293. 3DV-ATM High-Level Syntax: Loss Detection of Camera Parameter Sets [M. M. Hannuksela (Nokia)] late

Contributions JCT2-A0007, JCT2-A0008, and JCT2-A0053 propose the use of camera parameter sets to convey information for the closest and farthest depth as well as parameters related to view synthesis prediction. While parameter sets enable loss-resilient transmission of parameters, for example through retransmission possibility, it is asserted in this contribution that the mentioned proposals do not propose means to detect a loss of a camera parameter set. More specifically, if the camera_param_set_id value of the lost camera parameter set NAL unit has been used previously by another camera parameter set NAL unit, it is claimed that in such a case the decoder would continue decoding using the previously received camera parameter set NAL unit, which would be likely to result into clearly visible artifacts. It is asserted that this problem is being discussed also in JCT-VC related to adaptation parameter sets, which may be frequently updated similarly to camera parameter sets. This contribution proposes a parameter set loss detection approach based on constraints on camera_param_set_id value assignment.This proposal is technically identical to JCTVC-0072. It was commented that proposed functionality might be achievable by encoder only method.

Not needed for MV+D draft

Not urgent to take action at this point. Further study.

Version 2 of A0166 contains a suggested simplification of A0007. This was reviewed on Thu afternoon, and support was expressed that it is a desirable simplification.

Decision: Adopt from the green highlighted parts in JCT3V-A0166r1.doc the strikethrough sections for 3D-AVC draft

JCT2-A0140/m26138 3D-HLS: On Depth Definition [Shinya Shimizu, Shiori Sugimoto, Hideaki Kimata (NTT)]

This contribution proposes a modification on the depth acquisition information SEI message and a definition of depth representation type in order to support many kinds of multiview video plus depth map other than those used in the development. Regarding the depth acquisition information SEI message, it is suggested to support convergence camera setting. The proposed depth representation type includes three kinds of representation: disparity, an inverse of Z value, and Z value. Since there are some coding tools which requires understanding the definition of depth, this contribution proposes to define depth representation type as a part of the sequence parameter set extension. The extension of the depth-range-based weighted prediction, which is one of the coding tool effected by the depth representation type, is also discussed.

It is proposed that many types of depth representations may exist in the marketplace, so the MVC based 3DV standard may support more types such that the current DRWP can be applicable to other depth representations.

It is proposed new types of depth representations are defined in SPS.

- Disparity value

- Z value (instead of 1/Z)

A comment was given that if the group wants to seriously consider new types, it would be good to have test sequences belonging to such types.

It was commented that discussion is needed for the definition of depth.

First item: Add rotation parameter to the depth acquisition information SEI message. Decision: Adopt

(Note: In MVC, the camera parameter SEI message contains rotation)

Second item: Depth representation type SEI message, three different types: Z value, disparity, inverse Z value.

In the last meeting, M25017 proposed a nonlinear depth representation SEI message which was adopted but never integrated into the PDAM text (defines a mapping table for the 8 bit representation into the actual depth).

Decision: Adopt the depth representation SEI message as proposed in A0140, add type 4 to subsume the nonlinear depth representation originally proposed in M25017 (was also requested to be included by the JNB). Proponents of A0140 and M25017 to develop the integrated text and provide it to DAM editor.

Note: This SEI message would also be useful in context of HEVC extension with depth map.

Note 2: For these particular SEI messages the benefit is obvious and showcase not needed.

### HEVC related

#### NAL unit header

JCT2-A0115/m26102 3D-HLS: On NAL Unit Header and Video Parameter Set Design for HEVC 3DV [B. Choi, J. Kim, J. Park (Samsung)]

NAL unit header and VPS design are proposed for HEVC 3D/scalable extension with consideration of hybrid 3D/scalable CODEC design with non-HEVC video coding standards.

It was noticed that this proposal is almost the same as the JCTVC-J0430.

The reserved_one_5_bits in the NAL unit header of VPS is re-defined to signal the scalability_extension_type.

Type 1 is proposed for 3DV, 1-bit depth_flag and 4-bit view order index.

The motivation of signaling the scalability type only in the VPS NAL unit header was claimed as the reserved_one_5_bits is less useful in the VPS.

This proposal needs to change the base specification.

It was commented that the advantage of having view order index and depth_flag in the NAL unit header versus the generic layer_id is not clear in 3DV, but might be desirable in a harmonized design for 3DV and SVC.

One potential advantage of this proposal is that bitstream extraction can be done in a more flexible way, e.g., extracting MVC operation points by checking only the depth_flag.

The same proposal as in JCTVC-0432 is proposed for the support of multi-standard.

It was noticed that the software package supporting multi-standard stereo video hasn’t been made available yet.

A comment was made that it might be desirable for an AVC NAL decoder to filter out the HEVC NAL units by just checking the NAL unit header.

Encapsulation of heterogeneous view streams (e.g., base AVC, non-base: HEVC) sounds like something that can be taken care of by the system layer.

TBR in joint meeting JCT-VC/JCT-3V (Fri 8:00?) – BoG to make suggestions. Revisit

#### Video parameter set and sequence parameter set

JCT2-A0021/m26089 3D-HEVC HLS: Inter-layer SPS Prediction [Thomas Rusert (Ericsson)]

This contribution addresses a redundancy problem that is asserted to be present when separate SPSs are used across layers in layered HEVC extensions. It is reported that re-use of SPSs across layers is not an option if certain SPS parameters are layer-specific, an example being profile_idc and level_idc, thus with the current SPS design a separate SPS has to be used if layer-specific parameters are present. Since many SPS syntax elements may have identical values across layers, those values are duplicated if separate SPSs are sent for different layers.

It is proposed for the layered HEVC extensions to introduce SPS prediction to reduce the redundancy. This implies extending the definition of SPS in the extension specifications. In the proposed SPS prediction scheme, SPS parameters are inherited at the time of SPS activation. The reference SPS for prediction is determined as the active SPS of the reference layer, which is signaled in the referencing SPS. The parameters to be inherited are indicated by flags.

reserved_zero_6bits is to be used as layer_id.

SPS is associated only to layers with the same layer_id, this may lead to cleaner bitstream extraction. So a very flexible SPS design (which doesn’t have to be aligned with the current HEVC SPS design) is possible, meaning it doesn’t have to be compatible to the SPS design in the HEVC base specification.

SPS prediction follows the same dependency as the inter-view dependency used for e.g., texture prediction.

It is mandated to have two separate SPS instances for two layers.

All syntax elements in the SPS are grouped and can be predicted. Whether a group of parameters is to be predicted or not is indicated by a flag in the sequence parameter set (present in enhancement view only).

A comment was made that this design requires activation of the SPS, however in SVC, the IDR pictures are not aligned. De-referencing of SPSs depending on the active SPS for each layer might introduce some complication during random access.

De-referencing was said to be only needed during the activation time. This might have some issue, e.g., the encoder has to be carefully designed such that even when some layers are not present, the prediction can still get the correct reference SPS. This design might complicate the decoder, which needs to track the active sequence parameter sets.

A copy of de-referenced SPS and a NAL unit of each current SPS are needed for each layer.

TBR in joint meeting JCT-VC/JCT-3V (Fri 8:00?) – BoG to make suggestions. Revisit

JCT2-A0026/m26097 3D-HLS: On Slice Header and Parameter Set [B. Choi, J. Kim, J. Park (Samsung)]

Slice header prediction and parameter set sharing are proposed between texture and depth map inside a view component. [The abstract is too short.]

Slice header prediction is proposed between texture and depth.

It was proposed that in some cases, the slice partition for texture and depth picture can be the same, meaning the same slice address for each co-located (overlapped in terms of the covered LCU locations) slice. In this case, the slice address can also be predicted.

Sharing of the parameter set e.g., PPS is proposed.

It was mentioned that SPS sharing is not possible due to the nature of the parameters in SPS. For example syntax elements such as HRD parameters are not the same for texture and depth. Based on that, it is not possible to share the PPS because PPS having the same id must refer to the same SPS.

TBR in joint meeting JCT-VC/JCT-3V (Fri 8:00?) – BoG to make suggestions. Revisit

JCT2-A0099/m26054 3D-HLS: Video parameter set for 3D-HEVC [Y.Chen, Y.-K. Wang (Qualcomm)]

In JCTVC-J0114, VPS syntax design with corresponding changes in SPS (including VUI), is proposed to enable the use of VPS in session negotiation as well as to reduce the number of bits needed for the representation of SPSs. The motivation of JCT2-A0099/JCTVC-J0124 is reportedly to enable more convenient session negotiation for various operation points in the HEVC 3DV extension and better bit efficiency for the coding of parameter sets. A VPS extension design for 3D-HEVC is proposed based on JCTVC-J0114.

One part of the proposal is that dependency between layers/views is signalled in SPS. It was commented, that each layer/view therefore has to use a separate SPS. However, it was asserted by the proponent that since the SPS is small, this design choice causes only a moderate amount of repeated data between SPSes. It was commented that an alternative could be to signal the dependency data in VPS.

It was asked why HRD is located in VPS (instead of SPS). The proponent responded that HRD typically applies over many layers/views, hence suits better VPS (as VPS may be thought to be the place to store cross-layer/view information). It was also commented that fewer SPSes could be needed if HRD parameters are stored in VPS. It was also commented that the relationship between operation points and their HRD parameters can be reportedly mapped more easily with each other if they are stored in VPS.

It was commented that VPS may contain data for layers/views that have been extracted out and are no longer in the bitstream – hence, there could be some redundant information present in VPS.

It was asked what happens if a middlebox reduces the number of layers/views in the middle of a coded video sequence. It was responded that layers/views not present SEI message would be used in such a case and VPS would be left intact.

TBR in joint meeting JCT-VC/JCT-3V (Fri 8:00?) – BoG to make suggestions. Revisit

JCT2-A0121/m26214 3D-HLS: Design of the Video Parameter Set for 3D-HEVC [Robert Skupin, Valeri George, Thomas Schierl (Fraunhofer HHI)] [late]

Identical with JCTVC-JXXXX

TBR in joint meeting JCT-VC/JCT-3V (Fri 8:00?) – BoG to make suggestions. Revisit

#### Random access

JCT2-A0117/m26105 3D-HLS: On Random Access Pictures [B. Choi, J. Kim, J. Park (Samsung)]

TBR

TBR in joint meeting JCT-VC/JCT-3V (Fri 8:00?) – BoG to make suggestions. Revisit

JCT2-A0118/m26107 3D-HLS: On View Layer Switching [B. Choi, J. Kim, J. Park (Samsung)]

TBR

TBR in joint meeting JCT-VC/JCT-3V (Fri 8:00?) – BoG to make suggestions. Revisit

### AVC based 3DV

JCT2-A0102/m26057 3D-HLS: On reference picture list construction for AVC based 3DV [Y.-K. Wang, Y. Chen (Qualcomm)]

This document discusses some problems associated with the reference picture list construction (RPLC) process in the current AVC based 3DV specification. Two alternative solutions are proposed to solve the problem, one for the case wherein each texture view component can use at most one view synthetic prediction (VSP) reference component, and the other for the case wherein each texture view component can use more than VSP reference component. In both solutions, whether VSP reference is used is signalled in the slice header. It is suggested that the group make a firm decision on whether to allow for more than one VSP reference components for one texture view component, and choose one of the options accordingly.

This document is related to the specification of the reference picture list construction (RPLC) process in the current AVC-based 3DV specification. It is suggested that there be explicit signalling to indicate that VSP reference picture be added to the initial reference picture list. Two alternative solutions are proposed, one for the case wherein each texture view component can use at most one view synthetic prediction (VSP) reference component, and the other for the case wherein each texture view component can use more than VSP reference component. In both cases, a slice header flag is proposed to signal that a VSP reference is possible.

In the case of one VSP reference, the corresponding reference picture index is also signalled in the slice header when the slice header flag is set. In the case of more than one VSP reference, the maximum reference picture index is signalled in the slice header when the slice header flag is set. In either case, the RPLM process would operate without any change.

Discussion: It was suggested in this contribution that the group make a firm decision on whether to allow for more than one VSP reference components for one texture view component, and choose one of the options accordingly. It was remarked that there is currently no constraint on this; it was desired by the group that the decoding process allow for multiple VSP reference pictures and that any limitations be imposed as part of the profile/level definitions.

Discussion: The main point of this proposal is to consider whether to directly enable the VSP reference picture to be part of the initial RPL. The current text assumes

that the RPLC derives a VspRefExists variable after RPLM, i.e., first initialize the RPL according to MVC, and then modify the list according to 3DV which may add the VSP reference during the modification process. Three options seem possible:

- Always put VSP reference pictures in initial list (as probably done in the current software)

- Explicitly signal that VSP reference pictures should be added to the initial list (as proposed)

- Use modification process to insert VSP reference picture in the list (current text)

Decision(Ed.): This was considered by the group to be an editorial issue, and the means to construct and modify the reference picture list should be specified by the editors. It was considered desirable to maintain consistency with the MVC process and improve readability of the current text.

JCT2-A0008/m25781 3D-AVC HLS: Depth Range Parameter Set for AVC Compatible 3D Video Coding [M. Li, P. Wu (ZTE)]

It is technical the same as JCT2-A0007, but proposed to the AVC based 3DV track.

See JCT2-A0007 for detail.

### SEI messages

JCT2-A0034/m25897 3DV SEI messages on depth perception [Kwan-Jung Oh Jaejoon Lee Du Sik Park (Samsung)]

In this proposal, a 3DV depth perception information SEI message is proposed for both 3DV-ATM and 3DV-HTM. The SEI message includes the following: 1) a picture/frame level SEI message for baseline distance; 2) a picture/frame level SEI message for zero parallax.

Baseline ratio and Zero parallax are proposed in a new 3DV depth perception SEI message, potentially for each access unit.

Baseline ratio specifies the ratio (e.g., 0.8, or 1.1) to the IPD. IPD was not defined in this document but it was commented that IPD achieves the similar functionality as that of the reference baseline distance defined in the current 3DVC reference displays information SEI message we adopted during the last meeting (M25256 with the condition that a show case is to be provided during this meeting).

It was commented that zero parallax can be converted to shift value defined in the current 3DVC reference displays information SEI. So the need of zero parallax on top of shift value is not clear.

It was understood that the proposed 3DV depth perception SEI could be made as an extension to 3DVC reference displays information SEI message if the group agrees that frame level adaptation of depth perception is needed.

The benefit of this proposal is to be reviewed in a show case, which may require some short viewing sessions.

No experts (except for proponents) were available to confirm that this has been demonstrated as useful. No action.

JCT2-A0153/m26178 3D-AVC HLS: a show case of m25256 (view synthesis adaptation to display parameters) [Yin Zhao, Lu Yu (Zhejiang Univ.)]

This document describes the design of a show case for demonstrating the benefits from the SEI messages proposed in m25256.

This input document provides a show case of the 3DVC reference displays information SEI message. The show case is to be arranged jointly with the show case as described in JCT2-A0034.

After subj. viewing, several experts who participated confirmed that this functionality is useful.

JCT2-A0122/m26225 Maximum depth discontinuity signaling for view synthesis range determination [Andrey Norkin, Martin Pettersson, Thomas Rusert (Ericsson)] late

This document presents a proposal to signal maximum discontinuity value for the depth map. In the proposed mechanism, the encoder signals a maximum depth discontinuity value that can be used to determine a maximum disocclusion width when performing view extrapolation. Using the maximum depth discontinuity value, the view renderer can determine a proper view synthesis range depending on its "hole filling" capabilities. The proposal relates in particular to the MVC track since the Working Draft for this track also exists. It is however applicable to all depth-based 3DV tracks. The proposal contains explanations how the maximum depth discontinuity may be used determining the disocclusion width and the proposed syntax to be included in the depth acquisition SEI message.

Comment: MVC already specifies max disparity in an access unit. Is this not sufficient?

This SEI is mainly intended for single view.

For a single time instance, this parameter could be directly known from the depth map; the SEI message would rather be suitable for applications where the max discontinuity is known in advance for the entire sequence.

No support by other experts. No action

JCT2-A0163/m26275 Show-case and syntax for SEI message on reference display information signaling [A. Norkin, L. Girdzijauskas (Ericsson), Y. Zhao, Y.Luo (Zhejiang University] late

This contribution contains a description of a demo and an a semantics for SEI on reference display information parameters which was conditionally adopted at Geneva meeting 2012 subject to a show-case and syntax and semantics clean-up at the following Stockholm meeting.

The show case is to be arranged jointly with the show case as described in JCT2-A0034.

After subj. viewing, several experts who participated confirmed that this functionality is useful.

Decision: Adopt for MVC+D draft (was previously proposed in M25256, and improved text is proposed in A0163)

## Other technical proposal contributions

- JCT2-A0003/m25753 3DV: Test data and view synthesis software for warp coding and IDW-based view synthesis experiments [Nikolce Stefanoski, Aljoscha Smolic (Disney Research)]

In this contribution, image warp data and binary view synthesis software are provided; the data is provided for the 2-view case only at this stage. The view synthesis software generates new views using multiview video plus warp (MVW) data as input. It generates new views by image-domain-warping-based rendering (IDWR). The IDWR software can be used for interpolation and extrapolation of views. The warp data and rendering software are provided to evaluate the benefits of warp coding in the scope of the 3D video coding extension development and establish suitable test conditions. The proponents mentioned that viewing results of uncompressed results are available at this meeting.

Discussion: There was a question on whether there was any difference in the new results relative to earlier viewings. The main difference is that the warp data is extracted at the encoder rather than at the decoder.

Discussion: Consider establishing an exploration experiment on coding and synthesis based on the warp data representation. It was remarked that the similarities to the existing depth/disparity format would also need to be studied.

Informal Viewing was performed on Wednesday (N. Stefanoski).

- JCT2-A0004/m25754 3DV: Results on coding of warps using HEVC [Nikolce Stefanoski, Can Bal, Aljoscha Smolic (Disney Research)]

This contribution proposes to encode warps in order to use them at the decoder side for view synthesis with Image-Domain-Warping-based Rendering (IDWR). In particular, a warp coding method is proposed, which first quantizes the warps to obtain a YUV representation and then applies HEVC. It is noted that there is no dependency between texture and warp data. The warp data and IDWR software provided in JCT2-A0003 are used in the experiments. Warp coding results are reported for all 2-view input test sequences. It is reported that warp data can be coded at bit rates of 35kbit/s or below without introducing visible artifacts in comparison to a synthesis with original warp data. The proponents recommended establishing a CE on Warp Coding.

Discussion: As noted in the notes of JCT2-A0003, it was considered to establish an exploration experiment on coding and synthesis based on the warp data representation, where similarities to the existing depth/disparity format should also be studied.

Discussion: The group considered the type of experiments that may be conducted. One level of comparison would be comparisons of the warp framework to the existing depth framework, but a more detailed level of comparisons and analysis would be more desirable. The group was encouraged to consider drafting an experimentation plan as a break-out group activity.

JCT2-A0175 BoG Report: Preparation of Text on an Exploration Experiment on Warp Coding and Synthesis [Nikolce Stefanoski]

No recording of the precise outcome of the viewing session exists, but experts who participated express opinion in the JCT-3V plenary the warping method avoids artifacts in some cases where the current video-plus-depth based method has some. On the other hand, other artifacts appear due to local warping. Several expressed that this method is interesting and should be further investigated (AHG on alternative 3D formats, put reference to JCT2-A0175 into AHG mandates).

One idea would be that an SEI message could be used to express that a depth map is to be interpreted as a warping grid.

Another aspect could be to further investigate whether local warping in existing depth maps (as a kind of post processing) would hypothetically be able to avoid some of the artifacts of current view synthesis.

- JCT2-A0042/m25892 3D-AVC results on improved inside view motion prediction [Kwan-Jung Oh Jaejoon Lee Du Sik Park (Samsung)]

Modifications to the existing IVMP tool are proposed to improve coding efficiency. In this proposal, the IVMP availability conditions and motion prediction method are changed. Other parts including syntax are unchanged. Coding results for these modifications suggest that this method achieves a 0.2% bit rate reduction without any impact on complexity. It was noted by the proponent that this improvement is approximately half of the gain that the existing IVMP provides.

Discussion: There is some level of interest within the group to study this proposal further. This technique does not fit within the scope of any existing CEs, so we may consider establishing new CE for further evaluation.

One expert who has studied the proposal mentions that several additional condition checks are necessary at the decoder, therefore the complexity is increased.

Exploits additional dependencies between texture and depth MV, but according to the opinion of other experts (including cross-checker) this is marginal compared to already existing exist.

Further study (CE5 or new CE on inter-component?).

- JCT2-A0027/m26098 Cross-check report of JCT2-A0042 on improved inside view motion prediction [Shinya Shimizu (NTT)]

- JCT2-A0044/m25914 Depth Quadtree Prediction for HTM [J. Jung, K. Viswanathan (Orange Labs)]

This document presents a modification of the depth quadtree decoding process. For a given CTU, the quadtree of the depth is linked to the collocated CTU quadtree in the texture, in a way that a given CU of the depth cannot be split more than its collocated CU in the texture. The quadtree of the depth is predictively encoded relatively to the quadtree of the texture. A 62% runtime is reported compared to the reference, while bit rate reductions of 0.8% and 1.1% are observed for synthesized only, and coded and synthesized cases, respectively.

Discussion: Can the QT limitation be imposed as an encoder only option? Is there any normative change to the specification? It was agreed that it would be possible to impose the limitation in a non-normative way. However, with only this limitation, the gain drops from 0.8% to 0.1% bit rate reduction on synthesized views, and from 1.1% to 0.6% on coded and synthesized views.

Discussion: Is there a parsing dependency? The proponents clarified that the proposal assumes that the texture has already been parsed and is available.

Further study: AHG on studying dependent coding of texture and depth components (J. Jung).

CE on inter-component prediction

Decision(SW): Adopt the non-normative part (encoder decision) (non CTC)

- JCT2-A0159/m26199 Cross-check report for JCT2-A044: Depth Quadtree Prediction for HTM [Jicheng An, Liang Zhao, Shawmin Lei (MediaTek)] [miss] [late]

  - Non-normative contributions

- JCT2-A0151/m25966 3D-HEVC - Rate control for 3D multi-view video coding [W. Lim, H. Choi, J. Nam, (KWU), D. Sim, Ivan V. Bajić (SFU)] [late]

This contribution presents a rate control scheme for multi-view video coding based on the unified rate-quantization (URQ) model for HEVC described in JCTVC-H0213 [1] and JCTVC-I0094 [2]. The proposed rate control scheme is implemented on 3D-HTM3.1. The proposed rate-control scheme includes all features described in JCTVC-H0213 and JCTVC-I0094, which means that it includes GOP, frame, and unit (LCU) level rate control based on the quadratic pixel-based unified rate-quantization model. In addition, this contribution describes initial QP setting for the first frame of extended view, as well as depth map-based inter-view MAD (Mean Absolute Difference) prediction. In the case of extended view, the first frame doesn’t have temporal reference frame but has an inter-view reference frame in the base view, so it is possible to remove the view-axis spatial redundancy utilizing a P-frame coding scheme. Hence, the initial QP for the first frame of the extended view needs to be different from the QP value calculated by using the conventional pixel-based URQ model. For the MAD prediction for extended view, depth map based inter-view MAD prediction from the previously coded and neighbouring view is applied. To evaluate the proposed multi-view rate control scheme, 3D-HTM3.1, which uses hierarchical QP setting, is used for the constant bit rate (CBR) case.

By using this rate control for 3D multi-view video coding, the average bitrate accuracy was over 99%, with about 2.1dB average PSNR degradation.

Proponents will further investigate.

## Withdrawn and missing contributions

JCT2-A0005 [Withdrawn]

JCT2-A0017 [Withdrawn]

JCT2-A0054 [Withdrawn]

- JCT2-A0116 [Withdrawn]

- JCT2-A0124 [Withdrawn]

- JCT2-A0125 [Withdrawn]

- JCT2-A0149 [Withdrawn]

# Plenary Discussions and BoG Reports

## Project development

## BoGs

# Project planning

## General issues for CEs

A preliminary CE description is to be approved at the meeting at which the CE plan is established.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.A.a, CEX.H.b (where A stands for AVC and H for HEVC related 3D technology), etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using same software codebase, which should be based on either the ATM or HTM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for experiments were described in the output document JCT2-A1100.

A deadline of three weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-3V reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the XXX software basis. Exceptions must be justified, discussed on the general JCT-3V reflector, and recorded in the abstract of the summary report.

Final CEs shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

Experiment descriptions should be written in a way such that it is understood as a JCT-3V output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-3V document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

Non-final CE plan documents were reviewed and given tentative approval during the meeting (in some cases with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE is described in an associated output document JCT2-A11xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-3V is not obliged to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-3V members can participate in a CE

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

## Common Conditions for 3D Video Coding Experiments

Preferred Common Conditions for experiment testing that are intended to be appropriate for both CEs and other experiments were selected by the group and described in output document JCT2-A1100.

## Software development

...

Version(s) XX ... should be available within XX weeks after the meeting, and will be used for CEs. Version XY  is planned to be available 3 weeks later.

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-3V reflector (jct-3v@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-3V project management (AHG1) (jct-3v@lists.rwth-aachen.de) - Coordinate overall JCT-3V interim efforts. - Report on project status to JCT-3V reflector. - Provide report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| MVC+D / 3D-AVC Draft and Test Model editing (AHG2) | M. Hannuksela, S. Hattori (co-chairs), Y. Chen, T. Suzuki, J.-R. Ohm, G. Sullivan (vice chairs) | N |
| MV-HEVC / 3D-HEVC Test Model editing (AHG3) | G. Tech, K. Wegner (co-chairs), Y. Chen, T. Suzuki, J.-R. Ohm, G. Sullivan (vice chairs) | N |
| 3D-AVC Software Integration (AHG4) | D. Rusanovskyy (chair), D. Tian, ... | N |
| 3D-HEVC Software Integration (AHG5) | G. Tech (chair), ... | N |
| 3D Coding Tool Testing (AHG6) (jct-3v@lists.rwth-aachen.de) - Discuss and finalize the text for Common Test Conditions and CE work plans - Report on status of core experiments. - Study possible changes to test conditions. - Prepare viewing for 2nd JCT-3V meeting | K. Müller, A. Vetro | N |
| 3D High level syntax (AHG7) | Y. Chen, T. Rusert (co-chairs) | N |
| Study on alternative 3D Formats (AHG8) | T. Senoh, N. Stefanoski, M. Tanimoto (co-chairs) | N |
| JCT-3V 3D Quality Assessment (AHG9) (jct-3v@lists.rwth-aachen.de) - Study objective quality assessment metrics that would provide an accurate evaluation of synthesized views - Collect software that could be used for reporting on the evaluation of coding and rendering tools - Recommend improvements to evaluation methodology | T. Ebrahimi, A.Vetro, V. Baroncini (co-chairs) | N |
| Inter component dependencies (AHG10) | J. Jung, ... | N |
| 3D Test material (AHG11) | T. Wiegand, S. Yea (co-chairs) | N |

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate those responsible for document production.

JCT2-A1000 Meeting Report of 1st JCT-3V Meeting [J.-R. Ohm, G. J. Sullivan]

JCT2-A1001 MVC Extension for Inclusion of Depth Maps Draft Text 4 [T. Suzuki, M. Hannuksela, Y. Chen, S. Hattori] (WG11 14496-10 DAM2)

JCT2-A1002 3D-AVC Draft Text 3 [M. Hannuksela, Y. Chen, ??]

JCT2-A1003 3D-AVC Test Model 3 [M. Hannuksela, Y. Chen, ??]

JCT2-A1004 MV-HEVC Draft Text 1 [G. Tech, K. Wegner, Y. Chen, ??]

JCT2-A1005 3D-HEVC Test Model 1 [G. Tech, K. Wegner, Y. Chen, ??]

JCT2-A1100 Common Test Conditions of 3DV Core Experiments [D. Rusanovskyy, K. Müller, A. Vetro]

JCT2-A1101 CE1: View synthesis prediction [Coordinators: S. Shimizu, F. Jäger]

JCT2-A1102 CE2: Illumination compensation [Coordinator: J. Lee]

JCT2-A1103 CE3: Inter component prediction [Coordinator: J. Jung]

JCT2-A1104 CE4: RD optimization [Coordinator: S. Yea]

JCT2-A1105 CE5: Motion/mode parameter prediction [Coordinators: Y. Chang, S. Yea]

JCT2-A1106 CE6: Depth intra coding [Coordinator: P. Merkle]

# Future meeting plans, expressions of thanks, and closing of the meeting

tbd

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Monday or Tuesday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Wednesday or Thursday prior to such meetings and closing it on the last day of the WG 11 meeting).

Some specific future meeting plans were established as follows:

- 13–19 October 2012 under WG 11 auspices in Shanghai, CN.

- 17–23 January 2013 under ITU-T auspices in Geneva, CH.

- 20-26 April 2013 under WG 11 auspices in Incheon, KR.

- 27 July–02 August 2013 under WG 11 auspices in Vienna, AT.

XXX was thanked for its excellent hosting of the 1st meeting of the JCT-3V and for providing the viewing equipment used at the meeting.

The JCT-3V meeting was closed at approximately XXXX hours on Friday 20 July 2012.

# Annex A to JCT-XX report:
List of documents

# Annex B to JCT-XX report:
List of meeting participants

The participants of the sixth meeting of the JCT-XX, according to a sign-in sheet passed around during the meeting (approximately XXX in total), were as follows: