| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0049 |
| --- | --- |

| Title: | 3D-CE5.h related: Inter-view motion prediction for HEVC-based 3D video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jicheng An, Yi-Wen Chen, Jian-Liang Lin, Yu-Wen Huang, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 { jicheng.an, yiwen.chen, jl.lin, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

In this contribution, two methods are proposed to simplify and to improve the inter-view motion prediction in the merge, skip and inter modes in the HEVC-based 3D video coding. First, a constraint is applied to the inter-view motion prediction which only refers to one inter-view picture to derive the inter-view motion parameters. Second, we propose to improve the derivation of the inter-view merging candidate in the merge and skip modes. Following the concept for deriving the spatial merging candidate, the same motion parameters (prediction direction, reference pictures, and motion vectors) as those of the corresponding block in the inter-view picture are directly used as an inter-view merging candidate. When the two proposed methods are combined, 1.1% and 1.4% BD-Rate gains can be reportedly achieved for texture view 1 and view 2, respectively, and 0.3% total BD-Rate gain is reportedly achieved for coded and synthesized views. Moreover, the proposed constraint can provide a systematical way to manage the reconstructed motion data by synchronizing the reconstructed motion data buffer with the decoded picture buffer.

Introduction

In the HEVC-based 3D video coding, HTM 3.1 [1], the inter-view motion prediction is employed to derive the inter-view motion vector predictor (MVP) candidate for the inter mode (AMVP) and the inter-view merging candidate for the merge and skip modes. Both the inter-view MVP candidate and inter-view merging candidate are derived from the corresponding blocks, which are located by the disparity vector (DV) derived from the depth map associated with current PU, in the inter-view pictures as shown in Figure 1.

The derivation of the inter-view motion prediction needs to access the motion data of the corresponding blocks in all the previous coded inter-view pictures even if the inter-view pictures are not in the reference picture list of current picture. For example, as shown in Figure 1, assumed that the view coding order is V0 (base view), V1 and V2, and the picture in V2 is being coded, the motion data of all the previous coded inter-view pictures (V0 and V1) are needed to be stored even though the V1 is not in the reference picture list.

Figure 1 Inter-view motion vector prediction [Update Figure]

For the merge and skip modes in HTM 3.1, the derivations of spatial and temporal merging candidates follow the same derivation algorithms in HEVC. Basically, the same motion parameters (prediction direction, reference indices, and motion vectors) as those of a neighboring block are used for a spatial merging candidate. However, the algorithm for deriving the inter-view merging candidate is more complex in HTM 3.1. The inter-view merging candidate for the merge and skip modes is derived as follows:

Algorithm 1: Inter-view merging candidate derivation

- Given temporal reference picture with the smallest reference index in list 0 as the target reference picture, derive the MV by Algorithm 2; - Given temporal reference picture with the smallest reference index in list 1 as the target reference picture, derive the MV by Algorithm 2; - If no MV can be derived in step 1 and step 2, go to step 4; Else, go to step 6; - Check other reference pictures in list 0 with reference index i from 0 to k0. 4.1 Given reference picture with index i as the target reference picture, derive the MV by Algorithm 2. 4.2 If MV can be derived in step 4.1, go to step 5; - Check other reference pictures in list 1 with reference index i from 0 to k1. 5.1 Given reference picture with index i as the target reference picture, derive the MV by Algorithm 2. 5.2 If MV can be derived in step 5.1, go to step 6; - Done.

Algorithm 2: The MV derivation given a target reference picture

- If the target reference picture is a temporal reference picture, from view 0 to the previous coded view, the first MV of the corresponding block pointing to an inter-view picture associated with the target reference picture is used as the inter-view merging candidate - If the target reference picture is an inter-view reference picture, the disparity derived from depth map associated with current PU is used as the inter-view merging candidate.

In this contribution, a constraint on the reference inter-view picture and a simple and efficient inter-view merging candidate derivation are proposed to simplify and to improve the inter-view motion prediction. The first one is to apply the constraint that only the inter-view picture in the reference picture list of the current picture with the smallest view identifier (viewId) can be used in the inter-view motion prediction for the inter, merge and skip modes. The second one is to propose a simpler and efficient derivation for the inter-view merging candidate for the merge and skip modes.

Proposed Methods

Constraint on inter-view motion prediction

In this proposal, we propose to apply a constraint for inter-view motion prediction for the inter, merge and skip modes to provide better management of decoded motion data. The constraint is described below:

- Only the motion data of the inter-view picture which is in the reference picture lists (List 0 or List 1) of current picture with the smallest viewId can be used in inter-view motion prediction

With this constraint, only the motion data of one inter-view picture will be accessed for the current picture in the inter-view motion prediction. Moreover, the motion data of the inter-view pictures which are not in the decoded picture buffer are not required to be stored any more.

Improvement of the derivation of inter-view merging candidate

In this contribution, we propose to simplify and to improve the derivation of the inter-view merging candidate for the merge and skip modes. The proposed derivation algorithm for the inter-view merging candidate is shown in Figure 2. Basically, the proposed method for deriving the inter-view merging candidate follows the same concept employed for deriving the spatial merging candidate. The same motion parameters (prediction direction, reference pictures, and motion vectors) as those of the corresponding block are used as an inter-view merging candidate. Combined with the constraint in method 1, only the inter-view picture with the smallest viewId in the reference picture list is used to derive the inter-view merging candidate. The proposed inter-view merging candidate derivation algorithm is shown as follows:

Proposed inter-view merging candidate derivation

- The same motion parameters (prediction direction, reference pictures, and motion vectors) as those of the corresponding block in temporal direction are used as the inter-view merging candidate - If the motion parameters are not referable (i.e., the corresponding block is intra block, or the reference picture is not in the reference list of current picture) the disparity vector derived from the depth map associated with current PU is used as the inter-view merging candidate.

Figure 2 Proposed derivation of inter-view merging candidate for merge and skip modes

Experiments

The proposed methods are integrated into HTM 3.1 [1]. Three tests are conducted under common test  conditions [2]. Test1 is the experiment for evaluating the proposed constraint for the inter-view motion prediction. Test2 is the experiment for evaluating the simplified and improved derivation of the inter-view merging candidate. Test3 combines Test1 and Test2, i.e., the simplified and improved derivation of the inter-view merging candidate for the merge and skip modes and the constraint for the derivation of inter-view MVP candidate for the inter mode. The coding results for three tests compared to the anchor (HTM-3.1) are summarized by Table 1.

In Test 1, since the proposed constraint only affects the coding of view2, a 0.3% BD-Rate saving can be observed in coding view 2. In Test 2, BD-Rate savings 1.1% and 1.4% are achieved for view 1 and view 2 respectively, and it leads to a 0.3% total BD-Rate saving for coded and synthesized views. In Test 3, the similar gain is achieved compared to Test 2 while the inter-view MVP candidate derivation for the inter mode is also applied with the proposed constraint on the reference inter-view picture. The detailed experimental results of each test are given in Table 2, Table 3 and Table 4, respectively.

Table 1 Summarized results (BD-Rate savings) of three tests

|  | View 1 | View 2 | Coded texture views over the bitrates of texture | Coded and Synthesized pictures over the total bitrates (texture + depth) | Encoding time | Decoding time |
| --- | --- | --- | --- | --- | --- | --- |
| Test 1 | 0.0% | -0.3% | -0.1% | 0.0% | 99.8% | 99.1% |
| Test 2 | -1.1% | -1.4% | -0.5% | -0.3% | 98.1% | 99.7% |
| Test 3 | -1.1% | -1.4% | -0.5% | -0.3% | 99.4% | 100.7% |

Table 2 Results of test 1

|  | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.3% | -0.1% | 0.0% | 0.0% | 99.6% | 99.5% | 99.3% |
| Kendo | 0.0% | -0.4% | -0.1% | 0.0% | 0.0% | 97.1% | 98.6% | 100.6% |
| Newspapercc | 0.0% | -0.5% | -0.1% | -0.2% | -0.2% | 100.8% | 98.1% | 100.3% |
| GhostTownFly | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% | 102.1% | 99.7% | 100.7% |
| PoznanHall2 | 0.0% | -0.3% | -0.1% | 0.0% | -0.1% | 99.6% | 100.3% | 100.1% |
| PoznanStreet | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 99.6% | 99.1% | 99.6% |
| UndoDancer | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 100.2% | 98.5% | 102.1% |
| 1024x768 | 0.0% | -0.4% | -0.1% | -0.1% | -0.1% | 99.1% | 98.7% | 100.1% |
| 1920x1088 | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 100.4% | 99.4% | 100.6% |
| average | 0.0% | -0.3% | -0.1% | 0.0% | 0.0% | 99.8% | 99.1% | 100.4% |

Table 3 Results of test 2

|  | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -1.8% | -2.4% | -0.8% | -0.6% | -0.7% | 94.3% | 98.1% | 97.9% |
| Kendo | -1.4% | -2.1% | -0.7% | -0.5% | -0.6% | 97.2% | 99.2% | 98.3% |
| Newspapercc | -1.5% | -1.6% | -0.6% | -0.4% | -0.5% | 99.1% | 98.8% | 99.3% |
| GhostTownFly | -0.3% | -0.5% | -0.1% | -0.1% | -0.1% | 99.1% | 100.4% | 99.6% |
| PoznanHall2 | -0.4% | -0.8% | -0.2% | -0.1% | -0.1% | 99.0% | 100.7% | 99.8% |
| PoznanStreet | -0.9% | -1.1% | -0.3% | -0.2% | -0.2% | 99.0% | 100.4% | 99.7% |
| UndoDancer | -1.2% | -1.7% | -0.5% | -0.3% | -0.4% | 99.1% | 100.3% | 99.6% |
| 1024x768 | -1.6% | -2.0% | -0.7% | -0.5% | -0.6% | 96.8% | 98.7% | 98.5% |
| 1920x1088 | -0.7% | -1.0% | -0.3% | -0.2% | -0.2% | 99.1% | 100.4% | 99.7% |
| average | -1.1% | -1.4% | -0.5% | -0.3% | -0.4% | 98.1% | 99.7% | 99.2% |

Table 4 Results of test 3

|  | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -1.8% | -2.4% | -0.8% | -0.6% | -0.7% | 98.6% | 97.7% | 98.0% |
| Kendo | -1.4% | -2.1% | -0.7% | -0.5% | -0.6% | 101.4% | 98.2% | 98.4% |
| Newspapercc | -1.5% | -1.6% | -0.6% | -0.2% | -0.3% | 101.2% | 98.6% | 99.3% |
| GhostTownFly | -0.3% | -0.4% | -0.1% | -0.1% | -0.1% | 98.8% | 102.7% | 101.9% |
| PoznanHall2 | -0.4% | -0.7% | -0.2% | -0.2% | -0.2% | 98.6% | 102.0% | 100.1% |
| PoznanStreet | -0.9% | -1.1% | -0.3% | -0.3% | -0.3% | 98.7% | 103.1% | 101.7% |
| UndoDancer | -1.2% | -1.7% | -0.5% | -0.3% | -0.3% | 98.8% | 102.9% | 101.9% |
| 1024x768 | -1.6% | -2.0% | -0.7% | -0.5% | -0.5% | 100.4% | 98.2% | 98.5% |
| 1920x1088 | -0.7% | -1.0% | -0.3% | -0.2% | -0.2% | 98.7% | 102.7% | 101.4% |
| average | -1.1% | -1.4% | -0.5% | -0.3% | -0.3% | 99.4% | 100.7% | 100.2% |

Conclusion

A constraint on the reference inter-view picture and a simple and efficient inter-view merging candidate derivation are proposed to provide better decoded motion data management and also to provide better coding efficiency with reduced complexity. The proposed constraint can provide a systematical way to manage the decoded motion data with 0.3% BD-Rate saving in view 2. The proposed inter-view merging candidate derivation significantly reduces the complexity of the inter-view merging candidate and shows  BD-Rate savings 1.1% and 1.4% for view 1 and view 2, respectively, and 0.3% total BD-Rate saving for coded and synthesized views.

Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

[1] HTM-3.1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-3.1/

[2] Heiko Schwarz, and Dmytro Rusanovskyy, “Common Test Conditions for 3DV experimentation”, ISO/IEC JTC1/SC29/WG11 MPEG2011/N12745, May 2012, Geneva, Switzerland.