| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B1104 |
| --- | --- |

| Title: | Description of Core Experiment 4 (CE4): Residual Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Li Zhang | Tel: Email: | +1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document defines Core Experiment (CE) 4 on residual prediction to be performed for the 3rd JCT-3V meeting.

# Introduction

The goal of this CE is to investigate the methods for residual prediction proposed at the 2nd JCT-3V meeting. Tools under test will be evaluated according to their impact on compression efficiency and implementation complexity.

# Participants

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| Qualcomm | Li Zhang | lizhang@qualcomm.com | P/C |
| MediaTek | Jicheng An Jian-Liang Lin | jicheng.an@mediatek.com, jl.lin@mediatek.com | P/C |
| NTT | Shinya Shimizu | shimizu.shinya@lab.ntt.co.jp | C |
| Intel | Yi-Jen Chiu Zhipin Deng Wenhao Zhang | yi-jen.chiu@intel.com zhipin.deng@intel.com wenhao.zhang@intel.com | - |

(P=proponent, C=crosss checker)

# Tools under Test

## Advanced residual prediction for multiview coding [JCT3V-B0051]

Inter-view residual prediction is enabled in the current HTM design to code the residue of dependent texture views more efficiently. In this proposal, an advanced residual prediction (ARP) is proposed to further improve the coding efficiency of inter-view residual prediction. In ARP, to ensure high correlation between residues of two views, motion of the current block of picture in current view is applied to the corresponding block in a reference view picture to generate residual in the base view to be used for inter-view residual prediction. Moreover, an adaptive weighting factor is applied to the residue signal so that the prediction error is further reduced. Meanwhile, the parsing issue of current inter-view residual prediction is solved.

## Removal of the parsing dependency of inter-view residual prediction [JCT3V-B0093]

In this contribution, two methods are proposed to improve the inter-view residual prediction in HTM. Firstly, an approach to solve the parsing problem related to the inter-view residual prediction is proposed. Secondly, the interpolation in inter-view residual prediction is further removed to reduce the complexity, and the disparity vector is directly rounding to a nearest integer pixel.

# Mandates

Mandates for the CE are as follows:

- To investigate ways to further improve the inter-view residual prediction scheme as proposed in JCT3V-B0051, the SW implementation of which will be provided in the 3DV-HTM.

- To find out optimal ways for solving the parsing problem of inter-view residual prediction as proposed in JCT3V-B0093 and JCT3V-B0051, the SW implementation of which will be provided in the 3DV-HTM.

# Software, Configuration and Evaluation

## Software

Experiments in CE4 will use the 3D-HTM v5.0 software that is recommended in JCT3V-B1100. Proponents are requested to provide software that can be compiled under Windows and Linux platforms.

## Test Sequences, Bit Rates and Coding Conditions

The CE will use the test sequences, configuration and conditions that are recommended in JCT3V-B1100. Moreover, proponents and cross checkers are required to provide simulation results for the Random access configuration as specified in JCT3V-B1100.

## Evaluation of CE Results

The performance measurements are evaluated by switching on and off individual tools to identify their relative performance. The following measurements are considered to be used in this core experiment.

- Coding Performance Measurements: Measure impact on bitrate/PSNR. PSNR shall be calculated for the decoded texture views, relative to original texture views and for the synthesized views relative to uncompressed synthesized views. Use 4-point BD-PSNR and BD-Rate according to common conditions. The anchors will be generated according to common test conditions.

- Complexity measurements: Measure impact on encoding and decoding runtimes for all test cases relative to the unmodified test model software.

# Timelines

2013/01/07	Make source code, simulation results and draft text available for all proponents and cross-checkers.

2013/01/??	Register documents for the JCT-3V 3rd meeting

2013/01/??	Upload contributions to JCT-3V 3rd meeting