| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0057 |
| --- | --- |

| Title: | AHG7: Bitstream extraction for MVC+D |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In MVC+D, texture views and depth views can have different view dependencies, and a target output view may contain only texture or only depth. In the current MVC+D bitstream extraction process, both the texture view and the depth view, when present, of a dependent view are included in the sub-bitstream when a target output view depends on either the texture view or the depth view, even when one of the two is needed neither for decoding nor for output. It is proposed that the bitstream extraction process be modified such that unnecessary depth views or texture views are not included in the output sub-bitstream.

# Introduction

A corrigendum to the MVC sub-bitstream extraction process was proposed in ITU-T document VCEG-AT11 and MPEG document m26708. The change has been agreed. With the change, each view included in the output sub-bitstream is required to be complete, i.e. in each access unit, each view has a view component.

In MVC+D, the extraction process is inherited from MVC. The list of views to be decoded is defined as a union of the to-b-decoded texture views and the to-be-decoded depth views, which are derived separately based on the view dependency of texture views and the view dependency of depth views. Once a texture or depth view component has a view_id belonging to the list of views to be decoded, it will be included in the output sub-bitstream. Therefore, an unnecessary texture or depth view may be present in the output sub-bitstream.

# Proposal

It is proposed that unnecessary texture or depth views of the dependent views are not included in the sub-bitstream, as the presence of them is simply not needed and transmission of them simply wastes transmission bandwidth.

The detailed proposed changes are provided below. Section 2.1 provide the changes based on the current MVC+D draft (wherein the agreed MVC corrigendum has not yet been integrated), just for people who are more familiar with the current MVC+D draft to easily see the changes. Section 2.2.1 provides the changes based on the current MVC+D draft with integration of the MVC corrigendum. For convenience, the MVC bitstream extraction process with the corrigendum integrated is copied in Section 2.2.2.

## Proposed text change based on current MVC+D draft

#### I.8.5	Specification of bitstream subsets

The specifications of subclause H.8.5 apply.

#### I.8.5.1	Derivation process for required anchor view components

The specification of subclause H.8.5.1 apply with substituting "view component" with either "depth view component" or "texture view component" and "view" with either "depth view" or "texture view" and substituting “required for anchor” with “required for anchor depth”.

The specification of subclause H.8.5.1 apply with substituting "view component" with "texture view component" and "view" with "texture view" and substituting “required for anchor” with “required for anchor texture”.

#### I.8.5.2	Derivation process for required non-anchor view components

The specification of subclause H.8.5.2 apply with substituting "view component" with either "depth view component" or "texture view component" and "view" with either "depth view" or "texture view",  and substituting “required for anchor” with “required for anchor depth”.

The specification of subclause H.8.5.2 apply with substituting "view component" with "depth view component" with either "depth view", and substituting “required for anchor” with “required for anchor texture”.

#### I.8.5.3	Sub-bitstream extraction process

It is requirement of bitstream conformance that any sub-bitstream that is the output of the process specified in this subclause with depthPresentFlagTarget equal to 0 or 1, pIdTarget equal to any value in the range of 0 to 63, inclusive, tIdTarget equal to any value in the range of 0 to 7, inclusive, viewIdTargetList consisting of any one or more values of viewIdTarget identifying the views in the bitstream, shall be conforming to this Recommendation | International Standard.

NOTE 1 – A conforming bitstream contains one or more coded slice NAL units with priority_id equal to 0 and temporal_id equal to 0.

NOTE 2 – It is possible that not all operation points of sub-bitstreams resulting from the sub-bitstream extraction process have an applicable level_idc or level_idc[ i ]. In this case, each coded video sequence in a sub-bitstream must still conform to one or more of the profiles specified in Annex A, Annex H and Annex I, but may not satisfy the level constraints specified in subclauses A.3, H.10.2 and I.10.2, respectively.

Inputs to this process are:

–	a variable depthPresentFlagTarget (when present),

–	a variable pIdTarget (when present),

–	a variable tIdTarget (when present),

–	a list viewIdTargetList consisting of one or more values of viewIdTarget (when present).

Outputs of this process are a sub-bitstream and a list of VOIdx values VOIdxList.

When depthPresentFlagTarget is not present as input to this subclause, depthPresentFlagTarget is inferred to be equal to 0.

When pIdTarget is not present as input to this subclause, pIdTarget is inferred to be equal to 63.

When tIdTarget is not present as input to this subclause, tIdTarget is inferred to be equal to 7.

When viewIdTargetList is not present as input to this subclause, there shall be one value of viewIdTarget inferred in viewIdTargetList and the value of viewIdTarget is inferred to be equal to view_id of the base view.

The sub-bitstream is derived by applying the following operations in sequential order:

- Let VOIdxList be empty and minVOIdx be the VOIdx value of the base view.

- For each value of viewIdTarget included in viewIdTargetList, invoke the process specified in subclause H.8.5.1 for texture views with the viewIdTarget as input.

- If depthPresentFlagTarget is equal to 1, for each value of viewIdTarget included in viewIdTargetList, invoke the process specified in subclause H.8.5.1 for depth views with the viewIdTarget as input.

- For each value of viewIdTarget included in viewIdTargetList, invoke the process specified in subclause H.8.5.2 for texture views with the value of viewIdTarget as input.

- If depthPresentFlagTarget is equal to 1, for each value of viewIdTarget included in viewIdTargetList, invoke the process specified in subclause H.8.5.2 for depth views with the viewIdTarget as input.

- Mark all VCL NAL units and filler data NAL units for which any of the following conditions are true as "to be removed from the bitstream":

–	priority_id is greater than pIdTarget,

–	temporal_id is greater than tIdTarget,

–	anchor_pic_flag is equal to 1, nal_unit_type is not 21 and view_id is not marked as "required for anchor texture",

–	anchor_pic_flag is equal to 0, nal_unit_type is not 21 and view_id is not marked as "required for non-anchor texture",

–	anchor_pic_flag is equal to 1, nal_unit_type is 21 and view_id is not marked as "required for anchor depth",

–	anchor_pic_flag is equal to 0, nal_unit_type is 21  and view_id is not marked as "required for non-anchor depth",

–	nal_ref_idc is equal to 0 and inter_view_flag is equal to 0 and view_id is not equal to any value in the list viewIdTargetList,

–	nal_unit_type is equal to 21 and depthPresentFlagTarget is equal to 0.

- Remove all access units for which all VCL NAL units are marked as "to be removed from the bitstream".

- Remove all VCL NAL units and filler data NAL units that are marked as "to be removed from the bitstream".

- When VOIdxList contains only one value of VOIdx that is equal to minVOIdx, remove the following NAL units:

–	all NAL units with nal_unit_type equal to 14 or 15,

–	all NAL units with nal_unit_type equal to 6 in which the first SEI message has payloadType in the range of 36 to 44, inclusive.

NOTE 3 – When VOIdxList contains only one value of VOIdx equal to minVOIdx, the sub-bitstream contains only the base view or only a temporal subset of the base view.

- Remove all NAL units with nal_unit_type equal to 6 in which the first SEI message has payloadType equal to 0 or 1, or the first SEI message has payloadType equal to equal to 37 (MVC scalable nesting SEI message) and operation_point_flag in the first SEI message is equal to 1.

NOTE – The buffering period SEI and picture timing SEI messages, when not nested or nested in the MVC scalable nesting SEI message, apply for a sub-bitstream obtained with the sub-bitstream extraction process of subclause H.8.5.3, which does not process NAL units of nal_unit_type equal to 21.

- When depthPresentFlagTarget is equal to 0, the following applies in sequential order.

–	Replace all NAL units with nal_unit_type equal to 6 in which payloadType indicates a 3DVC texture scalable nesting SEI message with sei_op_texture_only_flag equal to 0 with a MVC scalable nesting SEI message with the same values of num_view_components_op_minus1, sei_op_view_id[ i ] and sei_op_temporal_id and the same nested SEI messges.

–	Remove all NAL units with nal_unit_type equal to 6 in which payloadType indicates a 3DVC texture scalable nesting SEI message.

–	The following applies for each active texture 3DVC sequence parameter set RBSP.

–	Replace mvc_vui_parameters_extension( ) syntax structure in an active texture 3DVC sequence parameter set RBSPs with the mvc_vui_parameters_extension( ) syntax structure of the 3DVC texture sub-bitstream VUI parameters extension, if both mvc_vui_parameters_extension( ) syntax structures apply to the same views.

–	Otherwise, remove mvc_vui_parameters_extension( ) syntax structure in an active texture 3DVC sequence parameter set RBSP.

–	When depthPresentFlagTarget is equal to 0, remove all NAL units with nal_unit_type equal to 6 in which the first SEI message has payloadType in the range of 48 to 52, inclusive.

- Let maxTId be the maximum temporal_id of all the remaining VCL NAL units. Remove all NAL units with nal_unit_type equal to 6 that only contain SEI messages that are part of an MVC scalable nesting SEI message or 3DVC scalable nesting SEI message with any of the following properties:

–	operation_point_flag is equal to 0 and all_view_components_in_au_flag is equal to 0 and none of sei_view_id[ i ] for all i in the range of 0 to num_view_components_minus1, inclusive, corresponds to a VOIdx value included in VOIdxList,

–	operation_point_flag is equal to 1 and either sei_op_temporal_id is greater than maxTId or the list of sei_op_view_id[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, is not a subset of viewIdTargetList (i.e., it is not true that sei_op_view_id[ i ] for any i in the range of 0 to num_view_components_op_minus1, inclusive, is equal to a value in viewIdTargetList).

- Remove each view scalability information SEI message and each operation point not present SEI message, when present.

- When VOIdxList does not contain a value of VOIdx equal to minVOIdx, the view with VOIdx equal to the minimum VOIdx value included in VOIdxList is converted to the base view of the extracted sub-bitstream. An informative procedure that outlines key processing steps to create a base view is described in subclause I.8.5.6.

NOTE 4 – When VOIdxList does not contain a value of VOIdx equal to minVOIdx, the resulting sub-bitstream according to the operation steps 1-9 above does not contain a base view that conforms to one or more profiles specified in Annex A. In this case, by this operation step, the remaining view with the new minimum VOIdx value is converted to be the new base view that conforms to one or more profiles specified in Annex A and Annex H.

## Proposed text change based on agreed MVC text

### MVC+D text

#### I.8.5	Specification of bitstream subsets

The specifications of subclause H.8.5 apply.

#### I.8.5.1	Derivation process for required anchor view components

The specification of subclause H.8.5.1 apply with substituting "view component" with either "depth view component" or "texture view component" and "view" with either "depth view" or "texture view.

#### I.8.5.2	Derivation process for required non-anchor view components

The specification of subclause H.8.5.2 apply with substituting "view component" with either "depth view component" or "texture view component" and "view" with either "depth view.

#### I.8.5.3	Sub-bitstream extraction process

It is requirement of bitstream conformance that any sub-bitstream that is the output of the process specified in this subclause with depthPresentFlagTarget equal to 0 or 1, pIdTarget equal to any value in the range of 0 to 63, inclusive, tIdTarget equal to any value in the range of 0 to 7, inclusive, viewIdTargetList consisting of any one or more values of viewIdTarget identifying the views in the bitstream, shall be conforming to this Recommendation | International Standard.

NOTE 1 – A conforming bitstream contains one or more coded slice NAL units with priority_id equal to 0 and temporal_id equal to 0.

NOTE 2 – It is possible that not all operation points of sub-bitstreams resulting from the sub-bitstream extraction process have an applicable level_idc or level_idc[ i ]. In this case, each coded video sequence in a sub-bitstream must still conform to one or more of the profiles specified in Annex A, Annex H and Annex I, but may not satisfy the level constraints specified in subclauses A.3, H.10.2 and I.10.2, respectively.

Inputs to this process are:

–	a variable depthPresentFlagTarget (when present),

–	a variable pIdTarget (when present),

–	a variable tIdTarget (when present),

–	a list viewIdTargetList consisting of one or more values of viewIdTarget (when present).

Outputs of this process are a sub-bitstream and a list of VOIdx values VOIdxList.

When depthPresentFlagTarget is not present as input to this subclause, depthPresentFlagTarget is inferred to be equal to 0.

When pIdTarget is not present as input to this subclause, pIdTarget is inferred to be equal to 63.

When tIdTarget is not present as input to this subclause, tIdTarget is inferred to be equal to 7.

When viewIdTargetList is not present as input to this subclause, there shall be one value of viewIdTarget inferred in viewIdTargetList and the value of viewIdTarget is inferred to be equal to view_id of the base view.

Set viewIdTargetListDepth to be equal to viewIdTargetList.

The sub-bitstream is derived by applying the following operations in sequential order:

- Let VOIdxList be empty and minVOIdx be the VOIdx value of the base view.

- For each value of viewIdTarget included in viewIdTargetList, invoke the process specified in subclause H.8.5.1 with the value of viewIdTarget as input.

- If depthPresentFlagTarget is equal to 1, for each value of viewIdTarget included in viewIdTargetListDepth, invoke the process specified in subclause H.8.5.1 for depth views with the viewIdTargetListDepth as input.

- For each value of viewIdTarget included in viewIdTargetList, invoke the process specified in subclause H.8.5.2 with the value of viewIdTarget as input.

- If depthPresentFlagTarget is equal to 1, for each value of viewIdTarget included in viewIdTargetListDepth, invoke the process specified in subclause H.8.5.2 for depth views with the viewIdTargetListDepth as input.

- Mark all VCL NAL units and filler data NAL units for which any of the following conditions are true as "to be removed from the bitstream":

–	priority_id is greater than pIdTarget,

–	temporal_id is greater than tIdTarget,

–	nal_unit_type is not 21 and view_id is not in the viewIdTargetList,

–	nal_unit_type is 21 and view_id is not in the viewIdTargetListDepth.

- Remove all access units for which all VCL NAL units are marked as "to be removed from the bitstream".

- Remove all VCL NAL units and filler data NAL units that are marked as "to be removed from the bitstream".

- When VOIdxList contains only one value of VOIdx that is equal to minVOIdx, remove the following NAL units:

–	all NAL units with nal_unit_type equal to 14 or 15,

–	all NAL units with nal_unit_type equal to 6 in which the first SEI message has payloadType in the range of 36 to 44, inclusive.

NOTE 3 – When VOIdxList contains only one value of VOIdx equal to minVOIdx, the sub-bitstream contains only the base view or only a temporal subset of the base view.

- Remove all NAL units with nal_unit_type equal to 6 in which the first SEI message has payloadType equal to 0 or 1, or the first SEI message has payloadType equal to equal to 37 (MVC scalable nesting SEI message) and operation_point_flag in the first SEI message is equal to 1.

NOTE – The buffering period SEI and picture timing SEI messages, when not nested or nested in the MVC scalable nesting SEI message, apply for a sub-bitstream obtained with the sub-bitstream extraction process of subclause H.8.5.3, which does not process NAL units of nal_unit_type equal to 21.

- When depthPresentFlagTarget is equal to 0, the following applies in sequential order.

–	Replace all NAL units with nal_unit_type equal to 6 in which payloadType indicates a 3DVC texture scalable nesting SEI message with sei_op_texture_only_flag equal to 0 with a MVC scalable nesting SEI message with the same values of num_view_components_op_minus1, sei_op_view_id[ i ] and sei_op_temporal_id and the same nested SEI messges.

–	Remove all NAL units with nal_unit_type equal to 6 in which payloadType indicates a 3DVC texture scalable nesting SEI message.

–	The following applies for each active texture 3DVC sequence parameter set RBSP.

–	Replace mvc_vui_parameters_extension( ) syntax structure in an active texture 3DVC sequence parameter set RBSPs with the mvc_vui_parameters_extension( ) syntax structure of the 3DVC texture sub-bitstream VUI parameters extension, if both mvc_vui_parameters_extension( ) syntax structures apply to the same views.

–	Otherwise, remove mvc_vui_parameters_extension( ) syntax structure in an active texture 3DVC sequence parameter set RBSP.

–	When depthPresentFlagTarget is equal to 0, remove all NAL units with nal_unit_type equal to 6 in which the first SEI message has payloadType in the range of 48 to 52, inclusive.

- Let maxTId be the maximum temporal_id of all the remaining VCL NAL units. Remove all NAL units with nal_unit_type equal to 6 that only contain SEI messages that are part of an MVC scalable nesting SEI message or 3DVC scalable nesting SEI message with any of the following properties:

–	operation_point_flag is equal to 0 and all_view_components_in_au_flag is equal to 0 and none of sei_view_id[ i ] for all i in the range of 0 to num_view_components_minus1, inclusive, corresponds to a VOIdx value included in VOIdxList,

–	operation_point_flag is equal to 1 and either sei_op_temporal_id is greater than maxTId or the list of sei_op_view_id[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, is not a subset of viewIdTargetList (i.e., it is not true that sei_op_view_id[ i ] for any i in the range of 0 to num_view_components_op_minus1, inclusive, is equal to a value in viewIdTargetList).

- Remove each view scalability information SEI message and each operation point not present SEI message, when present.

- When VOIdxList does not contain a value of VOIdx equal to minVOIdx, the view with VOIdx equal to the minimum VOIdx value included in VOIdxList is converted to the base view of the extracted sub-bitstream. An informative procedure that outlines key processing steps to create a base view is described in subclause I.8.5.6.

NOTE 4 – When VOIdxList does not contain a value of VOIdx equal to minVOIdx, the resulting sub-bitstream according to the operation steps 1-9 above does not contain a base view that conforms to one or more profiles specified in Annex A. In this case, by this operation step, the remaining view with the new minimum VOIdx value is converted to be the new base view that conforms to one or more profiles specified in Annex A and Annex H.

### MVC text (information)

#### H.8.5.1	Derivation process for required anchor view components

This process is recursively invoked to derive the set of required anchor view components for a specified view. Input to this process is a variable viewId, representing a view with view_id equal to viewId, with its corresponding view order index denoted by vOIdx.

Outputs of this process are a possibly updated VOIdxList, and additional invocations of the derivation process based on the inter-view dependency for anchor view components in the view with view_id equal to viewId as specified in the sequence parameter set MVC extension.

The following ordered steps are specified:

- Add vOIdx to VOIdxList if the same value is not already included in VOIdxList.

- Depending on num_anchor_refs_l0[ vOIdx ] and num_anchor_refs_l1[ vOIdx ], the following applies.

–	If both num_anchor_refs_l0[ vOIdx ] and num_anchor_refs_l1[ vOIdx ] are equal to 0, terminate this process.

–	Otherwise (num_anchor_refs_l0[ vOIdx ] or num_anchor_refs_l1[ vOIdx ] is not equal to 0), the following ordered steps are specified:

- When num_anchor_refs_l0[ vOIdx ] is not equal to 0, invoke the process specified in subclause H.8.5.1 for each viewId equal to anchor_ref_l0[ vOIdx ][ i ] for all i in the range of 0 to num_anchor_refs_l0[ vOIdx ] − 1, inclusive, in ascending order of i.

- When num_anchor_refs_l1[ vOIdx ] is not equal to 0, invoke the process specified in subclause H.8.5.1 for each viewId equal to anchor_ref_l1[ vOIdx ][ i ] for all i in the range of 0 to num_anchor_refs_l1[ vOIdx ] − 1, inclusive, in ascending order of i.

#### H.8.5.2	Derivation process for required non-anchor view components

This process is recursively invoked to derive the set of required non-anchor view components for a specified view.

Input to this process is a variable viewId, representing a view with view_id equal to viewId, with its corresponding view order index denoted by vOIdx.

Outputs of this process are a possibly updated VOIdxList and additional invocations of the derivation process based on the inter-view dependency for non-anchor view components in the view with view_id equal to viewId as specified in the sequence parameter set MVC extension.

The following ordered steps are specified:

- Add vOIdx to VOIdxList if the same value is not already included in VOIdxList.

- Depending on num_non_anchor_refs_l0[ vOIdx ] and num_non_anchor_refs_l1[ vOIdx ], the following applies.

–	If both num_non_anchor_refs_l0[ vOIdx ] and num_non_anchor_refs_l1[ vOIdx ] are equal to 0, terminate this process.

–	Otherwise (num_non_anchor_refs_l0[ vOIdx ] or num_non_anchor_l1[ vOIdx ] is not equal to 0), the following ordered steps are specified:

- When num_non_anchor_refs_l0[ vOIdx ] is not equal to 0, invoke the process specified in subclause H.8.5.2 for each viewId equal to non_anchor_ref_l0[ vOIdx ][ i ] for all i in the range of 0 to num_non_anchor_l0[ vOIdx ] − 1, inclusive, in ascending order of i.

- When num_non_anchor_refs_l1[ vOIdx ] is not equal to 0, invoke the process specified in subclause H.8.5.2 for each viewId equal to non_anchor_ref_l1[ vOIdx ][ i ] for all i in the range of 0 to num_non_anchor_l1[ vOIdx ] − 1, inclusive, in ascending order of i.

#### H.8.5.3 Sub-bitstream extraction process

It is requirement of bitstream conformance that any sub-bitstream that is the output of the process specified in this subclause with pIdTarget equal to any value in the range of 0 to 63, inclusive, tIdTarget equal to any value in the range of 0 to 7, inclusive, viewIdTargetList consisting of any one or more values of viewIdTarget identifying the views in the bitstream, shall be conforming to this Recommendation | International Standard.

NOTE 1 – A conforming bitstream contains one or more coded slice NAL units with priority_id equal to 0 and temporal_id equal to 0.

NOTE 2 – It is possible that not all operation points of sub-bitstreams resulting from the sub-bitstream extraction process have an applicable level_idc or level_idc[ i ]. In this case, each coded video sequence in a sub-bitstream must still conform to one or more of the profiles specified in Annex A and Annex H, but may not satisfy the level constraints specified in subclauses A.3 and H.10.2, respectively.

Inputs to this process are:

–	a variable pIdTarget (when present),

–	a variable tIdTarget (when present),

–	a list viewIdTargetList consisting of one or more values of viewIdTarget (when present).

Outputs of this process are a sub-bitstream and a list of VOIdx values VOIdxList.

When pIdTarget is not present as input to this subclause, pIdTarget is inferred to be equal to 63.

When tIdTarget is not present as input to this subclause, tIdTarget is inferred to be equal to 7.

When viewIdTargetList is not present as input to this subclause, there shall be one value of viewIdTarget inferred in viewIdTargetList and the value of viewIdTarget is inferred to be equal to view_id of the base view.

The sub-bitstream is derived by applying the following operations in sequential order:

- Let VOIdxList be empty and minVOIdx be the VOIdx value of the base view.

- For each value of viewIdTarget included in viewIdTargetList, invoke the process specified in subclause H.8.5.1 with the value of viewIdTarget as input.

- For each value of viewIdTarget included in viewIdTargetList, invoke the process specified in subclause H.8.5.2 with the value of viewIdTarget as input.

- Mark all VCL NAL units and filler data NAL units for which any of the following conditions are true as "to be removed from the bitstream":

–	priority_id is greater than pIdTarget,

–	temporal_id is greater than tIdTarget,

–	view_id is not in the viewIdTargetList

- Remove all access units for which all VCL NAL units are marked as "to be removed from the bitstream".

- Remove all VCL NAL units and filler data NAL units that are marked as "to be removed from the bitstream".

- When VOIdxList contains only one value of VOIdx that is equal to minVOIdx, remove the following NAL units:

–	all NAL units with nal_unit_type equal to 14 or 15,

–	all NAL units with nal_unit_type equal to 6 in which the first SEI message has payloadType in the range of 36 to 44, inclusive.

NOTE 3 – When VOIdxList contains only one value of VOIdx equal to minVOIdx, the sub-bitstream contains only the base view or only a temporal subset of the base view.

- Let maxTId be the maximum temporal_id of all the remaining VCL NAL units. Remove all NAL units with nal_unit_type equal to 6 that only contain SEI messages that are part of an MVC scalable nesting SEI message with any of the following properties:

–	operation_point_flag is equal to 0 and all_view_components_in_au_flag is equal to 0 and none of sei_view_id[ i ] for all i in the range of 0 to num_view_components_minus1, inclusive, corresponds to a VOIdx value included in VOIdxList,

–	operation_point_flag is equal to 1 and either sei_op_temporal_id is greater than maxTId or the list of sei_op_view_id[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, is not a subset of viewIdTargetList (i.e., it is not true that sei_op_view_id[ i ] for any i in the range of 0 to num_view_components_op_minus1, inclusive, is equal to a value in viewIdTargetList).

- Remove each view scalability information SEI message and each operation point not present SEI message, when present.

- When VOIdxList does not contain a value of VOIdx equal to minVOIdx, the view with VOIdx equal to the minimum VOIdx value included in VOIdxList is converted to the base view of the extracted sub-bitstream. An informative procedure that outlines key processing steps to create a base view is described in subclause H.8.5.5.

NOTE 4 – When VOIdxList does not contain a value of VOIdx equal to minVOIdx, the resulting sub-bitstream according to the operation steps 1-9 above does not contain a base view that conforms to one or more profiles specified in Annex A. In this case, by this operation step, the remaining view with the new minimum VOIdx value is converted to be the new base view that conforms to one or more profiles specified in Annex A.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).