| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20-26 Apr. 2013 |  |  |  | Document: JCT3V-D_Notes_d1 |
| --- | --- | --- | --- | --- |
| Title: | Meeting report of the forth meeting of the Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V), Incheon, KR, 20-26 APR. 2013 |  |  |  |
| Status: | Report Document from Chairs of JCT-3V |  |  |  |
| Purpose: | Report |  |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communications Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 +49 241 80 27671 ohm@ient.rwth-aachen.de |  |
| Source: | Chairs |  |  |  |

_____________________________

- Summary

The Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its forth meeting during 20-26 Apr. 2013 at the Songdo Conversia, Incheon, KR.. The JCT-3V meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany) and Dr Gary Sullivan (Microsoft/USA). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.13 of this document.

The meeting was mainly held in a “single track” fashion, with few breakout activities (as documented in this report) held in parallel. Several plenary sessions were chaired by Dr Anthony Vetro, particularly in cases when neither of the two chairs was able to attend due to commitments in other groups or joint meetings. It was usually avoided to make decisions which might cause objections in such cases.

The JCT-3V meeting sessions began at approximately 0900 hours on Saturday 20 Apr 2013. Meeting sessions were held on all days until the meeting was closed at approximately XXXX hours on Friday 26 Apr. Approximately XXX people attended the JCT-3V meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JCT-3V. The subject matter of the JCT-3V meeting activities consisted of work on 3D extensions of the Advanced Video Coding (AVC) and the High Efficiency Video Coding (HEVC) standards.

The primary goals of the meeting were to review the work that was performed in the interim period since the first JCT-3V meeting in producing

- the 6th version of specification Draft of the MVC extension by depth maps (which had been issued as an ISO/IEC FDAM document and been submitted for ITU-T consent);

- the 5th version of specification Draft of the AVC 3D extension framework (3D-AVC, which had been issued as an ISO/IEC Study of PDAM document);

- the 5th test model of the AVC 3D extension framework (3D-AVC), and associated software;

- the 3rd version of draft text of the Multiview HEVC extension (MV-HEVC), which had been issued as ISO/IEC 23008-2/PDAM2;

- the 3rd test model of the HEVC 3D extension framework (3D-HEVC) and associated software;

- the 2nd Draft of MVC plus depth conformance specification, which had been issued as ISO/IEC 14496-4:2004/PDAM41.

- the 2nd Draft of Multi-resolution Frame Compatible Stereo, which had been issued as ISO/IEC 14496-10:2012/PDAM5.

Furthermore, the JCT-3V reviewed the results from the interim Core Experiments (CE); reviewed technical input documents; produced updated versions of the draft texts, framework descriptions and software implementations of the items above; and planned a new set of Core Experiments (CEs) for further investigation of proposed technology.

The JCT-3V produced 8 particularly important output documents from the meeting (update):

- Draft Text 6 of 3D-AVC (ISO/IEC 14496-10:2012/DAM 2 – for ballot)

- Test Model 6 of 3D-AVC, and associated software

- Draft Text 4 of MV-HEVC (Study of ISO/IEC 23008-2:200X/PDAM 2– not for ballot)

- Test Model 4 of 3D-HEVC (includes MV-HEVC as subset), and associated software

- Draft 3 of MVC plus Depth Conformance (ISO/IEC 14496-4:2004/DAM 41 – for ballot)

- Draft 3 of Multi-resolution Frame Compatible Stereo (ISO/IEC 14496-10:2012/DAM 5 – for ballot)

- Common Test Conditions of 3DV Core Experiments

Moreover, plans were established to conduct XX future CEs in the interim period until the next meeting.

It was emphasized that short editing periods would apply to have the DAM ballot results available by the time of the October meeting.

For the organization and planning of its future work, the JCT-3V established XX "Ad Hoc Groups" (AHGs) to progress the work on particular subject areas. The next five JCT-3V meetings are planned for 27 July – 02 Aug 2013 under WG 11 auspices in Vienna, AT, 26 Oct– 01 Nov. 2013 under ITU-T auspices in Geneva, CH, 11–17 Jan. 2014 under WG 11 auspices in San Jose, US, 29 March – 4 April 2014 under WG 11 auspices in Valencia, ES, and 19–25 June 2014 in Geneva, CH.

The document distribution site http://phenix.it-sudparis.eu/jct3v/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-3V and all of its AHGs is jct-3v@lists.rwth-aachen.de.

- Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-3V are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its forth meeting during 20-26 Apr. 2013 at the ITU-T premises in Geneva, CH. The JCT-3V meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany) and Dr Gary Sullivan (Microsoft/USA). A couple of sessions were also chaired by Dr Anthony Vetro (MERL/USA).

## Meeting logistics

The JCT-3V meeting sessions began at approximately 0900 hours on Saturday 20 Apr 2013. Meeting sessions were held on all days until the meeting was closed at approximately XXXX hours on Friday 26 Apr. Approximately XXX people attended the JCT-3V meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-3V meeting activities consisted of work on 3D extensions of the Advanced Video Coding (AVC) and the High Efficiency Video Coding (HEVC) standards.

Information regarding preparation and logistics arrangements for the meeting had been provided via the email reflector jct-3v@lists.rwth-aachen.de.

## Primary goals

The primary goals of the meeting were to review the work that was performed in the interim period since the first JCT-3V meeting in producing

- Draft Text 6 of 3D-AVC (ISO/IEC 14496-10:2012/DAM2)

- Test Model 6 of 3D-AVC, and associated software

- Draft Text 4 of MV-HEVC (Study of ISO/IEC 23008-2:200X/PDAM2)

- Test Model 4 of 3D-HEVC (includes MV-HEVC as subset), and associated software

- Draft 3 of MVC plus Depth Conformance (ISO/IEC 14496-4:2004/DAM41)

- Draft 3 of Multi-resolution Frame Compatible Stereo (ISO/IEC 14496-10:2012/DAM5)

- Common Test Conditions of 3DV Core Experiments

Furthermore, the JCT-3V reviewed the results from the interim Core Experiments (CE); reviewed technical input documents; produced updated versions of the draft texts, framework descriptions and software implementations of the items above; and planned a new set of Core Experiments (CEs) for further investigation of proposed technology.

## Documents and document handling considerations

### General

The documents of the JCT-3V meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct3v/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

Document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (rather than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a bug in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding … simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of contexts, number of context-coded bins, etc.) … "Decision (Compl.):"

This meeting report is based primarily on notes taken by the chairs and projected (if possible) for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp (http://wftp3.itu.int/av-arch/jct3v-site/) during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much discussion of the contributions and discussions as is feasible in the interest of aiding study, although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Saturday, 13 Apr 2013.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Sunday 14 Apr 2013 were considered "officially late".

Most documents in this category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All contribution documents with registration numbers JCT3V-D0249 to JCT3V-D0XXX were registered after the "officially late" deadline (and therefore were also uploaded late). Some documents in the "D0249+" range include break-out activity reports that were generated during the meeting and are therefore considered report documents rather than late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with this issue was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical proposal contributions were classified as late either due to late upload or late registration (update):

- JCT3V-DXXXX (a technical proposal on ...) [uploaded 04-XX]

- ...

The following other documents not proposing normative technical content were classified as late:

- JCT3V-DXXXX (a proposal on ...)

- ...

The following cross-verification reports were uploaded late: JCT3V-DXXXX, ....

The following document registrations were later cancelled or otherwise never provided or never discussed due to lack of availability or registration errors: JCT3V-DXXXX, ....

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were rejected in the document management system, as has been agreed since the third meeting.

(the following case did not happen in the current meeting)

The initial uploads of the following contribution documents were rejected as "placeholders" without any significant content and were not corrected until after the upload deadline:

- JCT3V-D0XXX (a contribution of ... , corrected ...)

- ...

JCT3V-DXXXX and ... had some problems relating to IPR declarations in the initial uploaded versions (missing declarations of contributing companies). These issues were corrected by later uploaded versions in all cases (to the extent of the awareness of the chairs). In case of JCTVC-DXXXX and ..., the first complete versions appeared only on 1-14, such that this contribution was also flagged as “late”.

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Measures to facilitate the consideration of contributions

For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by the cross-check contributor.

### Outputs of the preceding meeting

The report documents of the previous meeting, particularly the meeting report (JCT3V-C1000), the MVC extension for inclusion of depth maps draft text 6 (JCT3V-BC1001), the 3D-AVC draft text 5 (JCT3V-C1002) and test model 5 (JCT3V-C1003), the multi-view MV-HEVC draft text 3 (JCT3V-C1004), the 3D-HEVC test model 3 (JCT3V-C1005), the draft 2 of MVC plus depth conformance spec (JCT3V-C1006), and the draft 2 of Multi-resolution Frame Compatible Stereo (JCT3V-C1006), which had been produced in the interim period, were approved. The ATM and HTM reference software packages produced by AHG4 and AHG5 on software development, and the software technical evaluations were also approved.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

## Attendance

The list of participants in the JCT-3V meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/ SC 29/ WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on 3D video coding projects guidance

- Consideration of 3D video coding technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-3V and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-3V as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jct3v-site (JCT-3V contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-3v/index.html (JCT-3V general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/ SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that it is our understanding according to the practices of the parent bodies to make reference software available under copyright license header language which is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as e.g. recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/ SC 29/ WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software header is currently used in the process of designing the new HEVC standard and for evaluating proposals for technology to be included in this design. Additionally, after development of the coding technology, the software will be published by ITU-T and ISO/IEC as an example implementation of the 3D video standard(s) and for use as the basis of products to promote adoption of the technology. This is likely to require further communication with and between the parent organizations.

Reportedly, both ATM (AVC related) and HTM (HEVC related) software packages that are used in JCT-3V follow these principles. The view synthesis software used for non-normative post processing is included in the HTM package and also has the BSD header.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct3v/. Furthermore, the site http://ftp3.itu.int/av-arch/jct3v-site was used for distribution of the contribution document template and circulation of drafts of this meeting report.

Communication of JCT-3V is performed via the list jct-3v@lists.rwth-aachen.de (to subscribe or unsubscribe, go to http://mailman.rwth-aachen.de/mailman/listinfo/jct-3v).

It was emphasized that reflector subscriptions and email sent to the reflector must use their real names when subscribing and sending messages and must respond to inquiries regarding their type of interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Some terminology used in this report is explained below:

- AHG: Ad hoc group.

- AMVP: Advanced motion vector prediction.

- ATM: AVC based 3D test model

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BoG: Break-out group.

- BR: Bit rate.

- B-VSP: Backward view synthesis prediction.

- CABAC: Context-adaptive binary arithmetic coding.

- CD: Committee draft – the first formal ballot stage of the approval process in ISO/IEC.

- CE: Core experiment – a coordinated experiment conducted between two subsequent JCT-3V meetings and approved to be considered a CE by the group.

- Consent: A step taken in ITU-T to formally consider a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions.

- DC: Disparity compensation

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC.

- DF: Deblocking filter.

- DMC: Depth based motion competition.

- DMM: Depth modeling mode.

- DRPS: Depth-range parameter set.

- DRWP: Depth-range based weighted prediction.

- DT: Decoding time.

- DV: Disparity vector

- ET: Encoding time.

- HEVC: High Efficiency Video Coding – the video coding standardization initiative under way in the JCT-VC.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- HTM: HEVC based 3D test model

- IC: Illumination compensation

- IVMP: Inside-view motion prediction (which means motion for depth component is inherited from texture component motion) Note: “IV” and other acronyms should be used consistently

- IVRC: Inter-view residual prediction.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/ SC 29, one of the two parent bodies of the JCT-3V).

- MPI: Motion parameter inheritance.

- MV: Motion vector.

- NAL: Network abstraction layer (HEVC/AVC).

- NBDV: Neighbored block disparity vector (used to derive unavailable depth data from reference view’s depth map) and DoNBDV = depth oriented NBDV

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NUT: NAL unit type (HEVC/AVC).

- PDM: Predicted Depth Map

- POC: Picture order count.

- PPS: Picture parameter set (HEVC/AVC).

- QP: Quantization parameter (as in AVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD). Often loosely associated with HE.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- RQT: Residual quadtree.

- SAO: Sample-adaptive offset.

- SEI: Supplemental enhancement information (as in AVC).

- SD: Slice data; alternatively, standard-definition.

- SH: Slice header.

- SPS: Sequence parameter set (HEVC/AVC).

- Unit types:

  - CTB: code tree block (synonymous with LCU or TB).

  - CU: coding unit.

  - LCU: (formerly LCTU) largest coding unit (synonymous with CTB or TB).

  - PU: prediction unit, with four shape possibilities.

    - 2Nx2N: having the full width and height of the CU.

    - 2NxN: having two areas that each have the full width and half the height of the CU.

    - Nx2N: having two areas that each have half the width and the full height of the CU.

    - NxN: having four areas that each have half the width and half the height of the CU.

  - TB: tree block (synonymous with LCU – LCU seems preferred).

  - TU: transform unit.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-3V).

- VPS: Video parameter set.

- VS: View synthesis.

- VSO: View synthesis optimization (RDO tool for depth maps).

- VSP: View synthesis prediction.

- WD: Working draft – the draft HEVC standard corresponding to the HM.

- WG: Working group (usually used in reference to WG 11, a.k.a. MPEG).

## Liaison activity

The JCT-3V did not send or receive formal liaison communications at this meeting.

## Opening remarks

Approval and pre-publication of MVCD in ITU-T on 2013-04-13. The text is currently pre-published as an integrated complete version of Rec. ITU-T H.264 (not a list of differences in amendment form).

MVCD 14496-10/PDAM 3 ballot closed in ISO/IEC on 2013-03-04, with summary of voting in WG 11 M28437.

MFC 14496-10/PDAM 5 ballot closed in ISO/IEC on 2013-04-14, with summary of voting in WG 11 M29164.

An increase in number of document contributions was noted.

Late document situation has not really improved.

Urgent need for coordination with JCT-VC on HL syntax 3D/SHVC to be started in joint meeting Sat afternoon

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows. update numbers

- AHG reports (section 2) (14)

- Project development and status (section 3) (7)

- CE1. View synthesis prediction and related (section 4.1) (35)

- CE2: Disparity vector derivation and related (section 4.2) (37)

- CE3: Inter-component prediction and related (section 4.3) (5)

- CE4: Residual prediction and related (section 4.4) (6)

- CE5: Inter-view / motion prediction and related (section 4.5) (28)

- CE6: Depth intra coding and related (section 4.6) (59)

- CE7: Skip mode coding (section 4.7) (1)

- 3DV Standards Development (section 5.1) (8)

- High-Level Syntax (section 5.2) (40)

- Other technical contributions on coding layer (section 5.3) (14)

- Conformance testing (section ) (0)

- Software (section 5.5) (2)

- Source Video Test Material (section 5.6) (1)

- Alternative Depth Formats (section 5.7) (5)

- Non-normative Contributions (section 5.8) (1)

- Complexity assessment (section 5.9) (2)

- Quality assessment (section 5.10) (1)

- Withdrawn, unclear allocation (section 5.11) (9)

NOTE – The number of contributions noted in each category, as shown in parenthesis above, may not be 100% precise.

# AHG reports

The activities of ad hoc groups that had been established at the prior meeting are discussed in this section.

JCT3V-D0001 JCT-3V AHG Report: JCT-3V project management [J.-R. Ohm, G.J. Sullivan]

The work of the JCT-3V overall has proceeded well in the interim period. A large amount of discussion was carried out on the group email reflector. All documents from the preceding meeting had been made available at the document site (http://phenix.it-sudparis.eu/jct3v/) or the ITU-based JCT-3V site (http://wftp3.itu.int/av-arch/jct3v-site/2012_01_B_Shanghai/), particularly including the following:

- The meeting report (JCT3V-B1000)

- the 5th version of specification Draft of the MVC extension by depth maps (which had been issued as an ISO/IEC Study of DAM document) (JCT3V-B1001)

- the 4th version of specification Draft of the AVC 3D extension framework (3D-AVC, which had been issued as an ISO/IEC Proposed Draft Amendment document) (JCT3V-B1002)

- the 4th test model of the AVC 3D extension framework (3D-AVC) (JCT3V-B1003)

- the 2nd version of draft text of the Multiview HEVC extension (MV-HEVC) (JCT3V-B1004)

- the 2nd test model of the HEVC 3D extension framework (3D-HEVC) (JCT3V-B1005)

- The work plan on 3D standards development (JCT3V-B1006)

- The document on NAL unit header and parameter set designs for HEVC extensions (JCT3V-B1007)

- the Draft of MVC plus depth conformance specification (JCT3V-B1008)

- Common Test Conditions of 3DV Core Experiments (JCT3V-B1100)

- Finalized core experiment descriptions (JCT3V-B1101 through JCT3V-B1107)

NOTE – The CE description documents had been revised somewhat after their intended finalization.

The 14 ad hoc groups and core experiments had made progress, and various reports from those activities had been submitted.

The software for ATM versions 6 and 6.1 and HTM versions 5 and 5.1 had been prepared and released approximately as scheduled.

Preparations have been made to install "bug tracking" systems for software and text specifications. Such a system is expected to be extremely helpful to the software coordinators and text editors in JCT-3V and is planned to be taken into full operation after the 3rd meeting. The bug tracker reports will be automatically forwarded to the group email reflector, where the issues could be further discussed.

More than 200 input contributions to the current meeting had been registered. A significant number of late-registered and late-uploaded contributions were noted. It must however be noted that the situation has improved since the last meeting, as the official registration and upload deadline was set earlier.

A preliminary basis for the document subject allocation and meeting notes had been circulated to the participants.

Meeting report C1000 was approved tentatively subject to no objections raised within 48 hrs.

JCT3V-D0002 JCT-3V AHG report: MVC+D / 3D-AVC Draft and Test Model editing (AHG2) [M. M. Hannuksela, S. Hattori, Y. Chen, T. Suzuki, J.-R. Ohm, G. J. Sullivan]

The editing team consisted of T. Suzuki, M. M. Hannuksela, Y. Chen, S. Hattori, and G. Sullivan.

For MVC Extension for Inclusion of Depth Maps Draft Text 6, the editors released the following aligned documents (roughly within the planned schedule):

- JCT3V-C1001: MVC Extension for Inclusion of Depth Maps Draft Text 6

- MPEG N13327: Text of ISO/IEC 14496-10:2012/FDAM2 MVC extension for inclusion of depth maps

- Text integrated to the ITU-T H.264 version 8

For the 3D-AVC Draft Text 5, the editing team consisted of M. M. Hannuksela, Y. Chen, T. Suzuki, J.-R. Ohm, and G. J. Sullivan. The editors released the following aligned documents (in the planned schedule):

- JCT3V-C1002 3D-AVC draft text 5

- MPEG N13332: Study Text of ISO/IEC 14496-10:2012/PDAM3 AVC compatible video-plus-depth extension

For the 3D-AVC Test Model 5 Description, the editing team consisted of D. Rusanovskyy, F. C. Chen, L. Zhang, and T. Suzuki. The editors released the following aligned documents:

- JCT3V-C1003: 3D-AVC Test Model 5

- MPEG N13342: 3D-AVC Test Model 5

For gathering and addressing comments, the following comments were received for 3D-AVC:

- Mikhail Mishourovsky reported (13 Feb 2013) to the editors that ALC-related text was not completely correctly integrated. The editors plan to address this in the next release of 3D-AVC text.

- 3D-AVC was subject to balloting in MPEG. Some ballot comments were received and are to be resolved in the JCT3V-D meeting.

Regarding coordination with the 3D-AVC Software Integration AhG, there were reportedly no known issues of text and software alignment.

The bug tracking system has been prepared by Fraunhofer HHI but not taken into use yet.

The recommendations by JCT-3V AHG2 were to:

- Approve JCT3V-C1001, JCT3V-C1002 and JCT3V-C1003 as JCT-3V output documents.

- Encourage inputs to get the Test Model document up-to-date.

- Continue to improve the overall editorial quality of 3D-AVC text and the Test Model.

JCT3V-D0003 JCT-3V AHG report: MV-HEVC / 3D-HEVC Test Model editing (AHG3) [G. Tech, K. Wegner, J. Boyce, Y. Chen, T. Suzuki, S. Yea, J.-R. Ohm, G. Sullivan]

The third MV-HEVC working draft and the third 3D-HEVC Test Model were developed from the second MV-HEVC working draft and the second 3D-HEVC Test Model following the decisions taken at the 3rd JCT-3V meeting in Geneva (17-23 January 2013).

Two editorial teams were formed to work on the two documents that were to be produced:

For JCT3V-C1004 MV-HEVC Working Draft 3, the editing team consisted of Gerhard Tech, Miska Hannuksela, Ying Chen, Jill Boyce, and Krzysztof Wegner.

For JCT3V-C1005 3D-HEVC Test Model 3, the editing team consisted of Gerhard Tech, Ying Chen, Krzysztof Wegner, and Sehoon Yea.

Editing JCT3V-C1004 was assigned a higher priority than editing JCT3V-C1005.

For the MV-HEVC Working Draft, four versions of JCT3V-C1004 were published by the AHG following the 3rd JCT-3V meeting in Geneva. The first version was released directly after the meeting. The last version was uploaded on 27. March and corresponds to the text submitted to MPEG secretariat for ISO/IEC PDAM ballot. The major changes compared to working draft JCT3V-B1004 were:

- Adoption of the unified design of the multi-view and the scalable extension. (JCT-C0238)

- Incorporation of all adopted proposals.

- Editorial update to HEVC version 1.

Open issues in JCT3V-C1004 that need discussion (details can be found as editor's comments in the draft text):

- Definition of access unit/coded picture: There was a discussion and disagreement between the editors if the current definition of an access unit was adopted or if the definition as in MV-HEVC draft 2 is valid. In current design an access unit consist of a coded picture and the coded picture consist of only one layer. In MV-HEVC draft 2 the coded picture within the access unit consist of different layer components.

- ViewId: It was noticed by an editor that the current ViewId derived from DimensionIds does not support the same features as the ViewId as defined in MV-HEVC draft 2, since it needs to be assigned based on coding order or layer dependencies and cannot be selected without constraints as it was in MV-HEVC draft 2.

- Picture size in stereo main profile: An editor noticed that in the stereo main profile the size of the picture of the second view is not restricted to the size of the picture of the first view. It was not clear if this is common sense and only a defect or if this needs further input/discussion. (this aspect was shortly discussed when the report was presented, and it was pointed out that mixed resolution coding in combination with inter-view dependent coding would require components for inter-layer processing that are not defined and would break the “pure” high-layer syntax approach)

- Byte alignment in profile_tier_level syntax structure: A byte alignment was adopted with JCTVC-L0363, however the adoption of JCTVC-L0180 requires a reintroduction of the profilePresentFlag in the profile_tier_level structure. Hence, both proposals are not fully compatible. An intermediate fix has been added that might require further discussion.

For the 3D-HEVC Test Model, two versions of JCT3V-C1005 have been published by the editing AHG following the 3rd JCT-3V meeting in Geneva.

All adoptions of the last meeting have been incorporated. Moreover existing text has been revised and improved and missing text from previous meeting has been added. As decided the document was split into two parts. One part containing the test model text description. The other part contains the draft specification. Changes of JCT3V-C1005 relative to JCT3V-B1005 were listed in the report.

In terms of general issues:

- A formal decision is missing to derive the DepthFlag (and ViewId) in the sense of MV-HEVC draft 3 form the VPS. However, such a decision would cause parsing dependencies for SPS with camera parameters as adopted last meeting.

- Regarding inter-view residual/motion prediction: With the adoption of tools related to inter-view motion prediction (JCT3V-A0126, JCT3V-A0049, JCT3V-A0097) the issue arose that inter-view motion prediction is reasonably specified for CTC only.

Editorial issues were identified as follows:

- Update of low-level specification to HEVC 1 is required.

- Mismatches to HTM-6.1 software exist since the 3D-HEVC “normative” Annex H is based on HEVC text specification 8 [3] and HTM-6.1 software is based on HM-6.1.

- The Test Model description includes tools not integrated in software.

Text is missing or not sufficient for the following adoptions, AHG3 kindly asks proponents to provide further input:

- For VSP the test model description is missing. Moreover text has not been provided for several modifications on AMVP and the loop filter.

- For illumination compensation the test model description needs to be extended.

- The specification of a table related to DMMs is missing.

- For Edge Intra the derivation of edge position is ambiguous.

A list of other minor issues is listed in the bug tracking system.

The setup of the bug tracking has been carried out by Karsten Sühring and David Flynn. AHG3 thanked Karsten and David for their support. The bug tracking system has been announced on the reflector can be accessed at:

https://hevc.hhi.fraunhofer.de/trac/3d-hevc/

For MV- and 3D-HEVC editing two tracking components have been created:

- 3D draft issue: for problems in 3D-HEVC Test Model description.

- MV draft issue: for problems in the MV-HEVC Draft

Issues reported by the editors often require additional inputs or clarification by the proponents. So it is recommended that proponents of adopted tools check the tracking system, whether there is an issue with their proposal and suggest to the editors how to fix it.

The recommendations of the MV-HEVC / 3D-HEVC Test Model editing AHG were to:

- Approve the edited JCT3V-C1004 and JCT3V-C1005 documents as JCT-3V outputs.

- Continue to edit both documents to ensure that all agreed elements of 3D- and MV-HEVC are fully described.

- Compare the documents with the HTM-software and resolve any discrepancies that may exist, in collaboration with the Software AHG.

- Continue to improve the overall editorial quality of the MV-HEVC working draft 3D-HEVC test model description.

- Ensure that properly drafted candidate text for both the MV-HEVC working draft and the 3D -HEVC Test Model (if appropriate) is available prior to making any decision to change the MV-HEVC or 3D-HEVC specification.

- Discuss reported open issues.

JCT3V-D0004 AHG4: 3D-AVC Software Integration (AHG4) [D. Rusanovskyy, O.Stankiewicz, D. Tian, J.Y. Lee, J.Lin]

The reference 3DV-ATM software version 7.0 has been made available according to plan within 3 weeks after the 3st JCT-3V meeting. The reference software version 7.1 including bug-fixes and non-CTC changes was released according to the plan. The reference software version 7.2 which includes interlace texture support for MVC+D specification was released shortly before the 4th JCT-3V meeting.

The most recent version of the 3DV-ATM software and related documentation is located in a public domain in the predefined location.

http://mpeg3dv.research.nokia.com/svn/mpeg3dv/tags/3DV-ATMv7.2/

AhG4 has performed an analysis and reconfirmation check for tools adopted during the 3rd JCT-3V meeting. Simulation results produced with 3DV-ATM v7.0 has been compared against the previous version 6.0, and a summary of the results is given in a table in the AHG report.

No changes to CTC configurations were suggested to be currently required.

AhG4 has performed an analysis and reconfirmation check for tools adopted during the 3rd JCT-3V meeting. Simulation results produced with 3DV-ATM v7.0 has been compared against the previous version 6.0, finding delta bit rate reduction of 0.8% total, 0.3% synthesized views.

JCT3V-D0005 JCT-3V AHG report: 3D-HEVC Software Integration (AHG5) [G. Tech, L. Zhang, Y. Chang, K. Wegner]

For tool integration to HTM, development of the software was co-ordinated with the parties needing to integrate changes. Development of HTM-6.0 was conducted in three parallel tracks each performing sequential integration. Software of all three tracks was merged by the software coordinators. HTM-6.1 was developed using a single track.

Prior to each integration source code to be integrated has been reviewed by the software coordinator and suggestions for improvement have been made to the integrators when necessary.

The distribution of the software was announced on the JCT-3V e-mail reflector and the software was made available through the SVN server:

https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/

Anchor bitstreams have been created and uploaded to:

ftp.hhi.fraunhofer.de; login: mpeg3dv_guest; path: /MPEG-3DV/HTM-Anchors/

History of software tags

Multiple versions of the HTM software were produced and announced on the JCT-3V email reflector. The AHG report provided a summary of the integrated tools and the achieved coding gains.

Comparison HTM-6.0 vs. HTM-5.1 (CTC, three view configuration)

|  | video video rate | video total rate | synth total rate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| average | -1,3% | -0,9% | -2,2% | 96,5% | 105,9% | 100,9% |

MV-HEVC software based on HM10 has also been provided.

It would be desirable to coordinate this with the SHM developments, where the newest version is also based on HM 10.

JCT3V-D0006 JCT-3V AHG Report: 3D Coding Tool Testing (AHG6) [Karsten Müller, Anthony Vetro]

The CE work plans were finalized and made available via the JCT-3V web site by January 23, 2013.

Possible changes to the Common Test Conditions from the various CEs were to be discussed further during the meeting.

For the AVC-based CEs, the 3DV-ATM software version 7.0 was used.

For the HEVC-based CEs, the 3DV-HTM software version 6.0 was used.

Other than the software versions to be used for the various CEs, there were no other needs identified during this AHG period to coordinate between any of the core experiments.

A few inputs with coding results for non-CTC configurations have been received and were to be discussed during the meeting.

There were 7 CEs established at the previous meeting to further study various aspects of the AVC and HEVC-based coding designs.

To get a sense of where the main areas of development are focused, the approximate number of CE reports and CE related contributions were noted in the AHG report. The CEs on view synthesis prediction (CE1), disparity vector derivation (CE2), inter-view / motion prediction (CE5) and depth intra coding (CE6) were very active with approximately 29-66 contributions in each of the related areas. CE3 on inter component prediction and CE4 on residual prediction had 5 contributions, while CE7 on skip mode coding seemed to be the least active, with 1 technical input.

The 3D viewing equipment had been prepared and the test room (room 112) had been setup. Viewing was planned to be performed during the week.

CE2 and CE7 reports are missing. For CE7, there is only one input. For CE2, it is still being prepared, the review of the CE should wait until it is available.

JCT3V-D0007 JCT-3V AHG report: 3D high level syntax (AHG7) [M. M. Hannuksela, J. Boyce, Y. Chen, T. Rusert, Y.-K. Wang]

There had not been much discussion related to the mandates of this ad-hoc group over the reflector between the 3rd JCT-3V meeting and the 4th JCT-3V meeting.

There were roughly 42 input contributions that were identified as relating to the mandates of AHG7.

Most of the HEVC high-level syntax contributions apply also to SHVC, and the AHG recommended discussing those contributions in a joint BoG with JCT-VC.

The following contributions seemed to relate only to MV-HEVC and/or 3D-HEVC. The AHG recommended reviewing them in JCT-3V.

- JCT3V-D0196 AHG7: Comments on parameter sets for MV-HEVC [Y. Chen, Y.-K. Wang(Qualcomm)]

- JCT3V-D0199 AHG7: Parallel decoding SEI message for MV-HEVC [Y. Chen, A. K. Ramasubramonian, Y.-K. Wang, S. Seregin, L. Zhang(Qualcomm)]

- JCT3V-D0202 MV-HEVC/3D-HEVC HLS: on ViewId signalling [M. M. Hannuksela (Nokia)]

- JCT3V-D0218 3D reference displays information SEI message for MV-HEVC and 3D-HEVC [A. Norkin (Ericsson)]

The following contributions had some aspects that related to SHVC but are otherwise specific to MV-HEVC or 3D-HEVC. The AHG recommends discussing them first in in a joint BoG with JCT-VC and subsequently in JCT-3V.

- JCT3V-D0051 AHG 13: Restriction of inter-layer prediction on tile boundaries [G. Tech, Y. Sanchez, K. Suehring, R. Skupin, T. Schierl (HHI)]

The AHG report commented that the proposed inter_layer_pred_tile_constraint_flag seems applicable to SHVC too. The specific impact of the flag on 3D-HEVC is recommended for discussion in JCT-3V.

- JCT3V-D0140 3D-HEVC HLS: Camera parameter signaling and Depth reference selection [Y.-L. Chang, Y.-P. Tsai, S. Lei (MediaTek), D. Tian, A. Vetro (MERL)]

The AHG report commented that this relates to general mechanisms of signalling inter-layer prediction type, even though the proposed types of prediction and parameters are JCT-3V-specific.

Documents that relate to both MV and scalable extensions of HEVC to be discussed jointly with JCT-VC (first joint meeting Sat. 1400)

JCT3V-D0008 AHG8 Summary Report on Alternative 3D Formats [M. Tanimoto, T. Senoh]

Two Alternative 3D Formats, Global View and Depth format, and Warp format were agreed to be included in 3D-AVC or 3D-HEVC at the 3rd JCT-3V meeting held at Geneva in January 17–23, 2013 AHG8 has been developing SEI message descriptions to signal these formats, software to be integrated in ATM or HTM according to the above mandates.

A total of five input documents relating to the Alternative 3D Format have been registered. The contents of these documents have been discussed through e-mail during the AHG meeting period with primary focus on the AHG mandates. A brief summary of the progress on each mandate and notes on the input documents that were contributed is given below.

After the review of draft text on the SEI message in JCT3V-C1002, some corrections were proposed and discussed to improve the accuracy of the description. The proposal of correction was summarized in the contribution JCT3V-D0070. AHG8 recommended to adopt this correction in the 3D-AVC Draft Text 6.

Production of a text which describes the usage of depth view information SEI message was proposed and discussed. The results were integrated in the contribution JCT3V-D0071. AHG8 recommended to output this document as an output of the 4th JCT-3V meeting and 104th MPEG meeting.

An experimental result and a request of Global View and Depth format based on the 3D-ATM v7.0 (GVD Type-2) were reported in JCT3V-D0102. In order to reduce the unnecessary coding load of dummy base view or dummy residual view from the GVD encoder, AHG8 requests ATM software to support single-depth or two-depths with no-texture view sequences.

A Global View and Depth software based on the 3D-ATM v7.1 (GVD Type-1) was under development. The releasing URL was planned be announced through the JCT-3V reflector. Since the current 3D-ATM v.7.x was not supporting multi-view plus one depth sequence, current GVD Type-1 software needs a dummy depth. AHG8 requested ATM software to support multi-view plus one depth in order to reduce the coding load of the dummy depth.

Two draft texts for Signalling Alternative 3D Format in 3D-HEVC were proposed.

- JCT3V-D0103 proposes an alternative depth info SEI message for Warp Maps in 3D-HEVC, by specifying a depth_type.

- JCT3V-D0272 proposes an extension of the depth_type to include Global View and Depth format.

AHG8 reported that both depth_types have been evaluated to have a good performance in several CEs and subjective tests and recommended to adopt the extended SEI message description for Warp and Global View and Depth signalling to 3D-HEVC.

The AHG8 on Alternative 3D Format recommended

- To adopt the correction of description for depth view information SEI message proposed in JCT3V-D0070 to 3D-AVC DT6,

- To output the description for Global View and Depth format for depth view information SEI message proposed in JCT3V-D0071as an output of the 4th JCT-3V meeting and 104th MPEG meeting,

- ATM software to support N-view (N=0, 1, 2,…) plus one-depth or two-depths in order to reduce the coding load of Global View and Depth encoder as reported in JCT3V-D0102,

- To adopt the description of depth view information SEI message for Warp and Global View and Depth signaling as proposed in JCT3V-D0103 and D0272 to 3D-HEVC, and

- HTM software to support N-view (N=0, 1, 2,…) plus one-depth or two-depths in order to reduce the coding load of Global View and Depth encoder as well as ATM.

AHG8 requests ATM software to support single-depth or two-depths with no-texture view sequences.

Shortly discussed in opening plenary - Only-depth transmission could in principle be supported by MVC – even though it does not support monochrome, the chroma could be set to zero.

JCT3V-D0009 JCT-3V AHG Report: 3D Quality Assessment (AHG9) [T. Ebrahimi, A. Vetro, V. Baroncini]

The following document related to this AHG mandate had been registered:

- JCT3V-C0032: AHG9: On the evaluation on stereoscopic and multiview autostereoscopic displays [P. Hanhart, T. Ebrahimi (EPFL), on behalf of Qualinet]

During the last meeting, it was agreed to focus our work on verification tests for 3D-AVC using MVC+D as a reference. Since 3D-AVC was expected to reach ISO/IEC DAM stage at the April meeting, it is the appropriate time for the verification tests to proceed.

Several aspects of the test were discussed by the AHG via the reflector and are summarized below:

- Displays: It is considered sufficient to conduct the subjective tests using stereoscopic displays

- Content: It is desired to have at least 4 test sequences per class; training sequences are also needed. We currently have 4 Class A test sequences and 3 Class C test sequences, as well as a couple sequences that were used for training in the CfP evaluation (which can probably be reused for the AVC verification test). There are several options to consider, including:

  - Add additional Class C test sequence (similar viewing as in CfP)

  - Mix Class A and Class C (single viewing session, same viewing distance)

  - Test Class A only (existing sequences and possibly any new sequences)

- Bit rates: We should aim for 4-5 rate points and allow some deviation. Quality range should be sufficiently broad, as done in CfP. There seem to be at least two options to consider:

  - Use the rate points from the CfP, which were designed to have the proper range and should still be valid since MVC+D is the reference

  - Apply fixed QPs to MVC+D (e.g., current CTC) and match the rate for 3D-AVC

Qualinet reported via the reflector that their members are prepared to conduct the subjective evaluation based on the test materials prepared by JCT-3V in the May/June time frame. A possible schedule to consider is outlined below:

- Finalize 3D-ATM v8 (3 weeks after meeting, 17 May 2013)

- Prepare test bitstreams and viewing material with cross-verification (2 weeks, 31 May 2013)

- Transfer results to test site (1 week, 7 June 2013)

- Conduct subjective viewing (5 weeks, 12 July 2013)

- Report results at 5th JCT-3V meeting in Vienna (2 weeks, 27 July 2013)

The AHG did not have any recommendations at this time on changes to the evaluation methodology, but suggested to consider this based on the experiments. However, it is encouraged that CE participants consider using alternative metrics as part of their work.

The AHG on 3D Quality Assessment recommended to:

- Review the input contribution related to 3D quality assessment

- Finalize the conditions and plans for the verification test of 3D-AVC during this meeting

- Re-establish the AHG to prepare test materials for the verification test and conduct further study in this area

Note from opening plenary: For the subjective tests, another option would be to use same QP for MVC+D and 3D-AVC, to find the rate saving.

Subjective viewing should be performed during the meeting, to make sure that the ranges of quality are appropriate

JCT3V-D0010 JCT-3V AHG Report: Complexity assessment (AHG10) [G. G. Lee (NCKU), T. Ikai (Sharp), K. Rapaka (Qualcomm), D. Rusanovskyy (Nokia)]

The AHG, identified and tabulated the coding tools or cases to be studied at different suggested data granularities in a spreadsheet.  The spreadsheet provides a high level overview of the scope of the work in this AHG. This spreadsheet was planned be uploaded after discussions by all co-chairs in this meeting.

JCT3V-D0301 "AHG 10: Overview of Algorithmic Intrinsic Complexity Measures" was identified as providing an informative overview on the complexity assessment of computational models or algorithms.

JCT3V-D0090 "AHG 10: Complexity Assessment on Adaptive Luminance Compensation and Plane Segmentation based Intra Prediction in 3D-AVC" was reported to provide an informative case study for two coding tools in 3D-AVC based on number of operations, data transfer rate, and data storage requirements.

Overview document (D0301). Preliminary version of complexity analysis spreadsheet should be attached.

JCT3V-D0011 JCT-3V AHG Report: 3D Test Material (AHG11) [S. Shimizu, S. Yea]

There were some efforts to bring new 3D test material as an offline work although there was no e-mail exchange on this topic. At this meeting we have one input contribution in this AHG.

Some negotiation and discussion were conducted with NICT, the copyright holder of the potential test sequences presented at the last meeting. NICT kindly offers to relax the condition for the use of their test materials although such relaxed condition can be applied only to 3D CG sequences.

The 3D CG sequences are originally designed for the 200-inch multiview auto-stereoscopic display developed by NICT. It might be necessary to perform subjective viewings in order to assess the suitability for JCT-3V activities.

New test sequences by NICT. CG content, originally produced for larger autostereoscopic display. At least one of them has disparity range that is larger than 255 (8 bit) – when mapped into znear/zfar depth representation, this should not be a problem.

JCT3V-D0012 JCT-3V AHG report: Conformance testing development (AHG12) [T. Suzuki, D. Rusanovskyy]

The ftp site at ITU-T is used to exchange bitstreams. The ftp site for downloading bitstreams is,

http://wftp3.itu.int/av-arch/jct3v-site/bitstream_exchange/

The spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available at this directory. It includes the list of bitstreams, codec features and settings, and status of verification.

The setting of dropbox at jct3v ftp site was reported to have a problem, such that it is not possible to upload any files due to the setting of the permission. It was suggested for this to be fixed as soon as possible.

The activity that should be started after this meeting as the first exercise was reported as follows:

- Update the software first, based on the text (output document at this meeting)

- Start to generate bitstreams (about one month after this meeting)

- Then revise the bitstreams when FDAM based software is produced in Jan 2013.

- All bitstreams must be ready by FDAM of conformance spec (maybe one year later)

The list of the candidate of the conformance bitstream and its volunteers are summarized in a table in the report.

So far, 11 bitstreams had been collected. The generated bitstreams were available at

http://wftp3.itu.int/av-arch/jct3v-site/bitstream_exchange/under_test/

ATM software should enable an option to output non-upsampled depth maps for conformance testing.

JCT3V-D0013 JCT-3V AHG Report: Parallel decoding (AHG13) [O. Nakagami, T. Ikai, A. K. Ramasubramonian]

An input contribution (JCT3V-D0056) had been received toward establishing reasonable constraints in terms of image resolution in the context of MV-HEVC stereo profile.

Regarding the relationship of view parallel scheme and other parallel schemes (tiles, wavefront proc. etc.), five related contributions (JCT3V-D0051, JCTVC-M0200, JCTVC-M0202, JCTVC-M0204 and JCTVC-M0334) were noted to have been received.

An input contribution (JCT3V-D0051) proposes tile related constraint by SPS for 3DV including decoding process change in 3D-HEVC.

Two input contributions (JCTVC-M0202 and JCTVC-M0334) proposed tile boundary alignment indication by VUI for 3DV / SHVC.

An input contribution (JCTVC-M0200) proposes to tile / WPP unit indication on inter-layer dependency by VUI for 3DV / SHVC.

An input contribution (JCTVC-M0204) proposes to enable decoding unit (DU) base interleave within an access unit with tile / WPP / slice segment by VPS for 3DV / SHVC.

Regarding the indication methods of parallel decoding in the context of multiview coding, seven related contributions were reported to have been received. Five of them were identified above. Two input contributions (JCT3V-D0064 and JCT3V-D0199) propose to indicate unreferenced region  by SEI for 3DV.

Relevant contribution documents were noted.

JCT3V-D0014 JCT-3V AHG Report: Mixed Resolution Coding (AHG14) [Krzysztof Wegner, Shinya Shimizu]

The following contributions are related to AHG14:

- JCT3V-D0080: Krzysztof Wegner, Olgierd Stankiewicz, Marek Domański, “Comparison of half resolution depth map coding versus full resolution depth map coding in 3D-ATM”

- JCT3V-D0164: Dmytro Rusanovskyy, Payman Aflaki, Miska M. Hannuksela “3D-AVC: Removal of texture to depth resolution ratio restrictions”

- JCT3V-D0215: S. Sugimoto, S. Shimizu, K. Kimata, “Report on Reduced Resolution Depth Map Coding in 3D-HEVC”

- JCT3V-D0216: S. Sugimoto, S. Shimizu, K. Kimata, “Report on Asymmetric Resolution Coding in 3D-HEVC”

Enhancement of 3D-HEVC test model software allowing coding reduced resolution depth map is proposed. The proposed modifications let several coding tools to support variable resolution depth map with minor changes. Even through the experimental result presents slight loss on coding performance (2.5% BD rate on synth views) according to the common test conditions with full resolution depth map, it also presents reduction of coding complexity (26% decoding time reduction).

Moreover, if the renderer supports reduced resolution depth map, the postdecoder upsampling process is not needed. Thus the data volume is vastly reduced to 15/4 of Full HD and the requirement as specified in N11829 is satisfied.

Regarding the desire to identify commonality of reduced resolution depth coding in MVC plus depth, 3D-ATM and 3D-HTM, there had not been any new activity on this mandate since the previous meeting, but further discussion on this mandate was expected during the meeting.

Regarding approaches and design implications to support coding of mixed resolution data (texture and depth), the current 3D-AVC specification imposes restrictions on spatial resolution ratios between coded texture and depth: only two ratios are supported, 1:1 (texture and depth are coded at equal spatial resolution) and 1:0.5 (depth data is presented at half spatial resolution in horizontal and vertical directions). It is argued that these restrictions should be relaxed to make 3D-AVC applicable to a broader range of applications. This contribution asserts that MVD data with an arbitrary depth-to-texture resolution can be coded with 3D-AVC without significant changes to the codec design and proposes changes to the specification text.

Regarding the implications of mixed resolution coding with regard to complexity and memory requirements of 3D-ATM and 3D-HTM, experimental results provided for the meeting reportedly show that coding with half resolution of depth (currently used in CTC) provides better results in comparison to coding with full resolution of depth. It is also more computationally efficient.

One of observations of the document is that comparing schemes of coding with different depth resolution require common reference in order to obtain meaningful results.

Coding with full resolution of depth requires 64-bit version of 3D-ATM to run correctly in case of Full HD sequences.

Some bug with reporting PSNR value of full resolution depth map was observed.

The AHG on Mixed Resolution Coding recommended to:

- Review the related input contributions

- Supports reduced resolution depth map coding in 3D-HEVC.

- Supports arbitrary depth-to-texture resolution in 3D-AVC.

- Bug fix and improve 3D-ATM to properly report full resolution depth maps PSNR values

- Re-establish the AHG for further study and experiments in this area

Discussion in plenary on bug with reporting PSNR value of full resolution depth map: This requires clarification/fixing, though it is not harmful, since the depth PSNR is usually not considered as a quality criterion.

# Project development, status, and guidance

## Communication by parent bodies

## Profile/level definitions

JCT3V-D0201 Multiview-video-plus-depth profile in MV-HEVC specification [M. M. Hannuksela, D. Rusanovskyy (Nokia)]

## Common test conditions

JCT3V-D0058 AHG6: SAO filter in Common Test Conditions [H. Brust, K. Mueller (HHI)]

JCT3V-D0278 Cross check on AHG6: SAO filter in Common Test Conditions (JCT3V-D0058 ) [J. Kang, Y. Chen (Qualcomm)] [late]

## Performance evaluation

JCT3V-D0079 Coding performance comparison between 3DV-ATM 7.0 and JMVM 8.0 [K.Y. Kim, H.S. Hwang, G.H. Park (KHU)]

## Asymmetric coding modes

JCT3V-D0080 Comparison of half resolution depth map coding versus full resolution depth map coding in 3D-ATM [K. Wegner, O. Stankiewicz, M. Domanski (Poznan)]

JCT3V-D0215 3D-HEVC: Report on Reduced Resolution Depth Map Coding in 3D-HEVC [S. Sugimoto, S. Shimizu, K. Kimata (NTT)] [late]

JCT3V-D0216 3D-HEVC: Report on Asymmetric Resolution Coding in 3D-HEVC [S. Sugimoto, S. Shimizu, K. Kimata (NTT)] [late]

# Core experiments

## CE1: View synthesis prediction

### Summary

JCT3V-D0021 3D-CE1 Summary Report on View Synthesis Prediction [S. Shimizu]

### CE contributions

#### AVC

none.

#### HEVC

Test (a): block size for the second disparity compensation

| D number | Min. size | Max. size | How to decide | BD-Rate (Coded PSNR) | BD-Rate (Synthesis PSNR) | Decoder runtime | Crosschecked |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D0089 | 4x8, 8x4 | 8x8 | Gradient of virtual depth map | 0.01% | -0.04% | 101.3% | N |
|  |  |  |  | 0.04% | -0.01% | 100.2% | Y (D0292) |
|  |  |  |  | 0.04% | 0.00% | 101.9% | N |
|  |  |  |  | 0.05% | 0.03% | 99.9% | Y (D0292) |
| D0165 | 1x1 | 1x1 | n/a | -0.22% | -0.21% | 107.6% | Y (D0225) |
|  |  |  |  | -0.20% | -0.19% | 103.2% |  |
| D0179 | 4x4 | N/2 x N/2 | CU size | 0.11% | 0.04% | 99.0% | Y (D0251) |
| D0213 | 4x8, 8x4 | 4x8, 8x4 | Corner samples of virtual depth map | 0.08% | 0.02% | 97.2% | Y (D0219) |

Current HTM VSP:

- at PU level (which can be as large as it could get) VSP is signalled by a merge index (there are 6 candidates in the merge list, where the presence of the VSP candidate index depends on the neighborhood).

– If VSP is signalled,

– derived disparity at PU level is used to locate the corresponding depth values from the base-view depth map

– PU is sub-divided into 4x4 blocks, and the maximum depth value of the corresponding 4x4 depth block is used to get the disparity vector for VSP (if pointing out of reference picture, clipping is performed at pixel level)

JCT3V-D0089 3D-HEVC: Adaptive Virtual Depth Block Partition for View Synthesis Prediction and Complexity Analysis [C.-F. Chen, G. G. Lee, B.-S. Li(NCKU)]

(insert abstract)

Method of D0089 subdivides into 8x8, then an analysis of the corresponding depth block (based on gradient) is made to determine whether it is further subdivided into 8x4/4x8 or left as 8x8. Then, the maximum value of depth is used for compensation in the block

JCT3V-D0292 Cross check of 3D-HEVC: Adaptive Virtual Depth Block Partition for View Synthesis Prediction and Complexity Analysis (JCT3V-D0089) [C.-H. Hung, C.-C. Lin(ITRI)] [late]

JCT3V-D0213 3D-CE1.h: Adaptive block size on VSP [S. Shimizu, S. Sugimoto, H. Kimata (NTT)]

(insert abstract)

Method of D0213 analyses four corner samples of the 8x8 subblock, and then decides whether it is split horizontally or vertically (it is always split) – remaining procedure stays

JCT3V-D0219 CE1.h: Crosscheck on NTT proposal JCT3V-D0213 [D. Tian (MERL)] [late]

(insert abstract)

JCT3V-D0179 CE1.h related: Low-complexity BVSP for 3D-HEVC [L. He, L. Zhang, Y. Chen(Qualcomm)]

Method of D0179 further subdivides the PU where VSP is signaled into N/2xN/2 blocks (e.g. 2NxN is subdivided into 8 blocks – asymmetric partitioning currently not in HTM), then proceeds as usual, minimum VSP block size still 4x4.

JCT3V-D0251 CE1.h related: cross-check of Low-complexity BVSP (JCT3V-D0179) [T. Ikai (Sharp)] [late]

JCT3V-D0165 3D-CE1.h: VSP Complexity Analysis and Constraint VSP (CVSP) [F. Zou, D. Tian, A. Vetro, H. Sun (MERL)]

(insert abstract)

Method of D0165 uses the depth values of the corresponding area in the depth map to apply pixel-based compensation, with some additional constraints.

JCT3V-D0225 3D-CE1.h: VSP Complexity Analysis and Constraint VSP (JCT3V-D0165) [S. Shimizu, S. Sugimoto (NTT)] [late]

JCT3V-D0293 3D-CE1.h related: Cross-Check of the results on the Constraint VSP in JCT3V-D0165 [Y.-W. Chen, J.-L. Lin (MediaTek)] [late]

Revisit test a) after offline analysis by participants: Worst-case number of operations (comparisons, mult/add) and memory accesses per LCU (or per sample).

Test (b): representative depth value derivation for each block

| D number | # of tested depth samples | BD-Rate (Coded PSNR) | BD-Rate (Synthesis PSNR) | Encoder runtime | Decoder runtime | Crosschecked |
| --- | --- | --- | --- | --- | --- | --- |
| Anchor | 16 | 0.00% | 0.00% | 100.0% | 100.0% | n/a |
| D0094 | 5 | 0.01% | -0.04% | 99.6% | 99.6% | Y (D0247) |

JCT3V-D0094 CE1.h The results on unification of simplified depth retrieval process for VSP and DoNBDV [J. W. Jung, S. Yea (LG)]

JCT3V-D0138 is doing a similar approach, where only 4 corner samples are used, but does not report separate results for VSP case, as it is also applying the same reduction the NBDV.

The simplification of D0094 also targets the unification with NBDV; the results indicate that this is clearly beneficial and should be adopted; if the results of D0138 indicate that further reduction to 4 depth samples is possible, D0138 should be adopted.

JCT3V-D0247 CE1.h: Crosscheck on LG proposal JCT3V-D0094 [D. Tian (MERL)] [late]

Test (c): Signalling of VSP

| D number | List with VSP pic. | Motion vector | BD-Rate (Coded PSNR) | BD-Rate (Synthesis PSNR) | Encoder runtime | Decoder runtime | Crosschecked |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor | none | n/a | 0.00% | 0.00% | 100.0% | 100.0% | n/a |
| D0212 | L0/L1 | Not used | 0.29% | 0.24% | 108.9% | 101.6% | Y (D0216) |
|  | L0/L1 | Not coded | 0.24% | 0.21% | 111.7% | 100.4% | Mismatch (D0216) |
|  | L0/L1 | Used | 0.28% | 0.25% | 233.8% | 102.4% | Y (D0216) |
|  | L0 | Not used | 0.03% | -0.01% | 111.1% | 100.2% | N |
|  | L0 | Not coded | 0.04% | 0.03% | 110.4% | 99.7% | N |
|  | L0 | Used | 0.04% | 0.00% | 231.6% | 101.1% | N |

JCT3V-D0212 3D-CE1.h: VSP signalling by reference picture index [S. Shimizu, S. Sugimoto, H. Kimata (NTT)]

(insert abstract)

No need for action, as the current anchor seems to give best results at lowest run time.

JCT3V-D0126 3D-CE1.h related: Cross check of VSP signalling by reference picture index (JCT3V-D0212) [J. Y. Lee, C. Kim (Samsung)] [late]

    - Related contributions

#### AVC

JCT3V-D0162 3D-CE1.a related: Harmonization of inter-view and view synthesis prediction in 3D-AVC [W. Su, D. Rusanovskyy, M. Hannuksela (Nokia)]

This contribution proposes harmonization of view synthesis prediction (VSP) and inter-view prediction (IVP) in 3D-AVC. This contains two parts.

The first proposal is related to the VSP reference picture. In the proposed harmonization, the reference picture index is shared for VSP and IVP. A b_vsp_flag is coded to specify whether the vector is coded or not. When the vector is coded, prediction image is generated by IVP. When the vector is not coded, VSP is performed by deriving disparity vectors from co-located depth map. The proposed flag is conditionally coded, i.e. only when the reference picture is inter-view reference picture. Besides, a slice level syntax is also proposed which enables the configuration where all blocks with inter-view reference pictures are predicted by VSP without b_vsp_flag.

- 0.04% and 0.08% BD-Rate reductions are reported for coded views and synthesis views, respectively. No complexity impact is reported, but it is asserted that the impact is negligible.
There is no performance and complexity assessment on the proposed slice level syntax.

Second proposal is related to the block size on BVSP. 8x8 block size is proposed for VSP_SKIP and VSP_DIRECT. For the other inter modes, 4x4 block size is still used.

- No separate result from the first proposal is available. In combination with the first proposal, 0.07% and 0.04% BD-Rate increases are reported for coded and synthesized views, respectively. However, decoder runtime is decreased about 15% on average (maximum reduction is 25% for GT_Fly sequence).

It was confirmed that the conditional usage of b_vsp flag does not introduce parsing dependency. The name of this syntax element could be improved.

The change to 8x8 block sizes is a clear benefit in terms of memory access complexity.

A slice level flag is also proposed that has the options a) always IVP, b) switchable by b_vsp flag c) always VSP.

Decision: Adopt D0162 except the aspect of slice-level signalling. Instead, a flag for dis/enabling VSP shall be implemented at slice layer.

JCT3V-D0246 CE1.a: Crosscheck on Nokia proposal JCT3V-D0162 [D. Tian (MERL)] [late]

#### HEVC

[Related] Category (i): VSP for depth coding

|  | Purpose | How | BD-Rate (Coded PSNR) | BD-Rate (Synthesis PSNR) | Encoder runtime | Decoder runtime | Crosschecked |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D0068 | Simplification | Limit DV precision | 0.00% | 0.00% | 105.6% | 99.6% | Y (D0???) |
| D0105 | Simplification | Disable | -0.03% | -0.03% | 100.1% | 98.5% | Y (D0125) |
| D0137 | Improvement | New DV derivation | -0.03% | -0.05% | 103.8% | 97.2% | Y (D0228) |
| D0139 | Simplification | Disable | 0.00% | 0.03% | 99.9% | 93.2% | Y (D0207) |

JCT3V-D0068 CE1.h related: Unification of Disparity Vector Accuracy in VSP with Motion Vector/Disparity Vector Accuracy in Temporal/Inter-View Prediction for Depth Coding [H. Liu, J. Jia, T. Kim (??)]

This contribution proposes to unify the DV (disparity vector) accuracy in VSP (view synthesis prediction) with the MV (motion vector)/DV accuracy in temporal/inter-view prediction for depth coding. That is to say, only use integer-pel DV for VSP in depth coding. It is reported that there is no compress performance loss.

Opinion is expressed that this is useful, but as depth map VSP will be disabled, this becomes obsolete.

JCT3V-D0286 CE1.h related: Crosscheck on Unification of Disparity Vector Accuracy in VSP with Motion Vector/Disparity Vector Accuracy in Temporal/Inter-View Prediction for Depth Coding (D0068) [X. Zheng (Hisilicon)] [late]

JCT3V-D0105 3D-CE1.h related: simplifications on view synthesis prediction [Y.-W. Chen, J. An, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek)]

In HTM 6.0, view synthesis prediction (VSP) uses the scheme of depth oriented neighboring block disparity vector (DoNBDV) to derive a depth block to perform the backward warping operation for both texture and depth coding. However, it is observed that VSP does not introduce sufficient coding gain for depth coding in current design. In this contribution, it is proposed to disable VSP for depth coding. Moreover, for each VSP coded CU, the current VSP needs to access depth blocks of the reference view twice and these two accessed depth blocks may be totally un-overlapped. To reduce the memory bandwidth of the reference depth data access, it is further proposed to simply use the neighboring block disparity vector (NBDV) instead of DoNBDV for VSP so that the access time of depth block for each VSP coded CU could be reduced from two to one. The experimental results reportedly show that disabling VSP for depth coding causes no BD-rate change and the proposed VSP using NBDV can even achieve 0.1% BD-rate saving for overall coded and synthesized views compared to HTM-6.0 while the memory bandwidth to access the reference depth data is reduced. The experimental results also show that the combination of the proposed two simplifications brings 0.1% BD-rate saving for overall coded and synthesized views.

Presentation deck to be uploaded.

The first aspect suggests not to use VSP for depth maps – this could be disabled by conf parameter in CTC.

Decision(SW): It is agreed to disable depth map VSP in CTC, but still needs clarification about how to handle MPI in that case – revisit in context JCT3V-D0191 that would resolve this

Revisit: Check whether depth map VSP is specified in WD, if yes, remove.

DoNBDV uses neighbor block DV to fetch a depth block from base view, converts this into disparity and uses this to fetch another depth block that is then used for VSP. The proposal is to use the first depth block directly for VSP. This means that in current VSP it is necessary to fetch two depth blocks, whereas in DoNBDV as used in finding the co-located block in the reference view only one depth block is fetched. This looks like an interesting simplification, but if other changes are made to VSP it will be necessary to investigate in a CE whether it still is beneficial. Revisit: This seems to be a minor change, and might be adopted when supported by other experts after thinking about possible implications.

JCT3V-D0125 3D-CE1.h related: Cross check of VSP simplification (JCT3V-D0105) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-D0297 CE1.h: Crosscheck on MediaTek proposal JCT3V-D0105 [D. Tian (MERL)] [late]

JCT3V-D0137 3D-CE1.h related: Improved DV for BVSP in depth coding [J. Heo, T. Kim, H. Liu, S. Yea (LG)]

The backward-warping view synthesis prediction (BVSP) approach uses neighbouring blocks to derive a depth block to perform the backward warping operation in both texture and depth coding. In order to find the depth block, BVSP uses a disparity vector from neighbouring blocks in texture coding. However, since there is no disparity vector derivation process in depth coding, a zero disparity vector is directly used to derive a depth block. This contribution replaces the zero disparity vector with a refined disparity vector in BVSP approach. Under the common test condition, this method provides 0.1% BD-rate gain with no impact on the encoder/decoder complexity.

Presentation deck to be uploaded.

Actual gain is rounded up. The benefit seems to be too small to be considered, as on the other hand completely disabling depth map VSP is a significant simplification, reportedly without loss.

JCT3V-D0228 3D-CE1.h related: Cross check of Improved DV for BVSP in depth coding (JCT3V-D0137) [S. Shimizu, S. Sugimoto (NTT)] [late]

JCT3V-D0139 3D-CE1.h related: Results on removal BVSP for depth coding [J. Heo, S. Yea (LG)]

In the HTM6.0, a backward-warping view synthesis prediction (BVSP) approach which uses the neighbouring blocks to derive a depth block to perform the backward warping operation was applied to both texture and depth coding. However, the BVSP scheme is not efficient to code the depth data. In addition, although BVSP approach is less complex than forward-warping VSP (FVSP) scheme, it still incurs high complexity increase. Therefore, this contribution proposes the removal of BVSP approach for depth coding. The result reports that the proposal does not impact coding performance with a minor coding complexity reduction.

Presentation deck to be uploaded.

Same as first aspect of D0105 (disabling VSP for depth coding). In case where the texture is coded by VSP, the motion vector is marked as not available, whereas D0105 still uses the MV of texture.

JCT3V-D0207 3D-CE1.h related: Cross check on removal BVSP for depth coding (JCT3V-D0139) [X. Zhao (Qualcomm)] [late]

[Related] Category (ii): Merge candidate list construction

| D number | Default position | Spatial VSP candidate | Multiple VSP candidates | BD-Rate (Coded PSNR) | BD-Rate (Synthesis PSNR) | Encoder runtime | Decoder runtime | Crosschecked |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor | Before temporal | All | No | 0.00% | 0.00% | 100.0% | 100.0% |  |
| D0092 | After DCP | All? | No | 0.00% | 0.00% | 99.9% | 93.7% | Y (D0160) |
|  | Before temporal | All? | Yes | ? | ? | ? | ? | N |
|  | After DCP | All? | Yes | -0.06% | -0.12% | 100.1% | 90.9% | Y (D0160) |
| D0117 | Before temporal | None | No | 0.01% | -0.02% | 99.8% | 98.9% | Y (D0223) |
|  | Before temporal | left, above | No | -0.02% | -0.05% | 99.0% | 98.9% | Y (D0223) |
|  | After temporal | left, above | No | -0.06% | -0.11% | 99.6% | 98.7% | Y (D0223) |

Note: D0117 basically marks spatial neighboring blocks and corresponding texture block coded by VSP unavailable. Exceptions are left and above neighbors in test 2 and 3.

JCT3V-D0092 CE1.h related : BVSP mode inheritance [T. Kim, J. Nam, S. Yea (LGE)]

In the 3rd JCT-3V meeting, a backward view synthesis prediction (BVSP) mode was proposed and adopted into 3D-HEVC. The BVSP mode is currently inserted into the merge candidate list as an additional inter-view prediction. In this contribution, firstly, it is proposed that additional BVSP candidates be allowed to be in the merge-list. To be more specific, it is proposed to reuse DV information from spatially-neighboring BVSP blocks (if available) for producing additional BVSP candidates for the current block (PU). Consequently, the merge candidate list could include multiple BVSP mode candidates with different DVs, potentially leading to a better coding efficiency trade-off. Secondly, it is proposed that the order BVSP is inserted into the merge-list be modified in order to attain better coding efficiency by guaranteeing BVSP shows up in the list at least once. Bit rate gains are respectively 0.3% and 0.1% for the dependent views 1, 2, and the overall bit rate gain is 0.1%.

The simplification is achieved by unconditionally putting a VSP mode as candidate to the end of the merge list (at position 5), regardless if a VSP mode is already found at any other position.

Several experts supported this idea, as it simplifies the merge list construction, and guarantees that always a VSP candidate is in the list (which is likely the reason for the coding gain reported).

JCT3V-D0223 3D-CE1.h related: Cross check of BVSP mode inheritance (JCT3V-D0117) [S. Shimizu, S. Sugimoto (NTT)] [late]

JCT3V-D0160 3D-CE1.h related: Cross check of LG's BVSP mode inheritance (JCT3V-D0092) [I. Lim, H. Wey (??)]

JCT3V-D0117 3D-CE1.h related: Simplification of VSP insertion process [J. Y. Lee, M. W. Park, B. Choi, C. Kim (Samsung)]

View synthesis prediction (VSP) was integrated on the current 3D-HTM (JCT3V-C0152). VSP is used as a new candidate in Skip and Merge modes and inserted right before a temporal candidate. However, if any of the adjacent blocks is coded with VSP, it is inserted as the candidate for the block. In the insertion process for a candidate from a potentially VSP block, the current process investigates all the neighboring blocks whether they are coded with VSP or not. In this contribution, three methods are introduced for simplification of the current VSP insertion process. Especially, the proposed VSP insertion process only based on the left and above blocks, such as the VSP skip flag signalling in 3D-AVC, achieves -0.1% gain in both the decoded and synthesized views as well as the complexity reduction.

Presentation deck to be uploaded.

Three different options are presented

Offline activity: Analysis of the method from D0092 and the three options from D0117, and the current merge list construction in HTM, in terms of worst-case number of comparisons necessary. Revisit.

[Related] Category (iii): Simplification

|  | D number | How | BD-Rate (Coded PSNR) | BD-Rate (Synthesis PSNR) | Encoder runtime | Decoder runtime | Crosschecked |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | D0068 | Limit DV precision for depth coding | 0.00% | 0.00% | 105.6% | 99.6% | Y (D0???) |
| 2 | D0105 | Disable for depth coding | -0.03% | -0.03% | 100.1% | 98.5% | Y (D0125) |
| 3 |  | Use NBDV instead of DoNBDV | 0.01% | -0.08% | 100.3% | 97.8% | Y (D0125) |
| 4 |  | Combination of 2 and 3 | -0.03% | -0.08% | 99.8% | 96.0% | Y (D0125) |
| 5 | D0117 | No VSP check | 0.01% | -0.02% | 99.8% | 98.9% | Y (D0223) |
| 6 |  | VSP check only with left and above | -0.02% | -0.05% | 99.0% | 98.9% | Y (D0223) |
| 7 |  | VSP check only with left and above, and place VSP after temporal candidates | -0.06% | -0.11% | 99.6% | 98.7% | Y (D0223) |
| 8 | D0118 | Enable only for 2Nx2N partition | 0.00% | 0.00% | 99.6% | 100.0% | Y (D0224) |
| 9 | D0119 | Combination of 2 and 8 | -0.04% | -0.02% | 99.6% | 99.2% | Y (D0227) |
| 10 | D0139 | Disable for depth coding | 0.00% | 0.03% | 99.9% | 93.2% | Y (D0207) |
| 11 | D0191 | Remove vsp_idx | -0.11% | -0.07% | 93.3% | 112.1% | Y (D0221) |

Note: Grayed proposals are also related to other categories (light gray: (i) depth map coding, dark gray: (ii) merging candidate list)

JCT3V-D0118 3D-CE1.h related: VSP restriction [J. Y. Lee, C. Kim (Samsung)]

(include abstract)

further evidence required that this is beneficial in terms of simplification that requires nromative change. It could also be implemented as encoder-only.

JCT3V-D0224 3D-CE1.h related: Cross check of VSP restriction (JCT3V-D0118) [S. Shimizu, S. Sugimoto (NTT)] [late]

JCT3V-D0119 3D-CE1.h related: Simplification of VSP [J. Y. Lee, C. Kim (Samsung), J. An, Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek)]

(include abstract)

No need for further consideration per dispositions on D0105 and D0118.

JCT3V-D0227 3D-CE1.h related: Cross check of Simplification of VSP (JCT3V-D0119) [S. Shimizu, S. Sugimoto (NTT)] [late]

JCT3V-D0191 CE1.h related: Clean-ups for BVSP in 3D-HEVC [L. Zhang, Y. Chen, L. He, M. Karczewicz(Qualcomm)]

In the current implementation of backward view synthesis prediction (BVSP) in 3D-HEVC, a BVSP merging candidate is added with its reference index set to -1. In addition, BVSP mode flags need to be maintained in decoded picture buffer (DPB) and utilized for several modules, such as de-blocking filter, temporal merging candidate derivation, motion parameter inheritance for depth coding, depth-oriented neighboring block disparity vector (Do-NBDV). However, majority of these changes are not reflected in the working draft. To reduce the unnecessary complication and complexity introduced by additional checking of BVSP mode flags and additional memory, it is proposed to align the software with the working draft by modifying the reference index of BVSP merging candidate and remove the storage of BVSP mode flags in DPB. In this way, BVSP coded blocks could be considered transparently as normal Inter predicted blocks. The contribution reports an average coding gain of 0.1% for texture views, and 0.1% for synthesized views in terms of bitrate reduction.

As one additional aspect, the current software for VSP modifies the de-blocking filter, which is not reflected in the WD text description. By the proposal, the SW would be aligned.

In general agreed that this is a useful cleanup and alignment between WD text and software (as also confirmed by the cross-checker), further inspection to be done by Dong and Shimizu to confirm – revisit.

JCT3V-D0221 CE1.h related: Cross check of Clean-ups for BVSP in 3D-HEVC (D0191) [G. Tech (HHI)] [late]

[Related] Category (iv): Others

JCT3V-D0156 3D-CE1.h related: A high level syntax suggestion for stereo compatibility in 3D-HEVC [K. Zhang, J. An, J.-L. Lin, S. Lei (MediaTek)]

This contribution presents a high level syntax suggestion for stereo compatibility in 3D-HEVC. In HTM-6.0, depth dependent coding (DDC) methods such as BVSP and DoNBDV are adopted to improve the overall coding efficiency. However, the dependency between texture and depth pictures breaks the stereo compatibility in 3D-HEVC. With the proposed syntax, DDC tools can be switched on or off. An encoder can decide depending on the application scenario.

Presentation deck to be uploaded

  - JCT3V-D0156: HLS to disable depth dependent coding tools

- Purpose: Stereo compatibility

- Proposal: New syntax elements to switch depth dependent coding tools (VSP and DoNBDV) for VPS and SPS extensions: one global flag and two tool-based flags.

This is assessed to be useful, more study is suggested about which tools should be enabled/disabled individually.

Decision: VPS Flag to disable BVSP and DoNBDV

JCT3V-D0166 3D-CE1.h: On reference view selection in NBDV and VSP [F. Zou, D. Tian, A. Vetro, H. Sun (MERL)]

In the current 3D-HEVC working draft, there lacks a way to deal with B view case for NBDV derivation and VSP prediction. With the proposed technique, the reference view used in NBDV function would not be fixed to the base view (with refViewIdx = 0), and thus the derivation of disparity vector from the NBDV function for a B view is more general. On the other hand, the reference view used for VSP prediction is not fixed as the base view (with refViewIdx = 0), but is selectable and the selection of the reference view and prediction direction is determined by the availabilities of interview reference pictures in both RefPicList0 and RefPicList1 when a prediction block is chosen as VSP in a B view picture. The proposed fixes show an average bitrate saving 3.1% on a B view, or 0.3% for video PSNR/video bitrate, 0.2% for video PSNR/total bitrate and 0.1% for synthesis PSNR/total bitrate under the IBP test condition.

Presentation deck to be uploaded

  - JCT3V-D0166: Reference view selection and Bi-VSP

- Purpose: Fix the problem on B-view coding, where there are two reference views.

- Proposal #1: Target reference view for DoNBDV is decided by the view associated with the disparity vector from the neighboring block.

- Proposal #2: Bi-VSP, i.e. weighted average of Uni-VSP. Bi-VSP is applied when there are inter-view reference pictures on each reference picture list.

|  | BD-Rate (Coded PSNR) | BD-Rate (Synthesis PSNR) | Encoder runtime | Decoder runtime | Crosschecked |
| --- | --- | --- | --- | --- | --- |
| Proposal #1 | 0.00% | 0.02% | 100.5% | 102.5% | Y (D0226) |
| Proposal #2 | -0.23% | -0.15% | 101.6% | 101.6% | Y (D0226) |
| #1 and #2 |  |  |  |  |  |

Note: Experiments were conducted under IBP configuration (non-CTC)

One concern is expressed that bi-directional VSP would be too complex

Generally it would be desirable to specify as generic as possible (i.e. not disallowing bi-B-VSP and handle it different from other prediction modes)

Current WD text that comes with the contribution is not mature yet; to be updated and checked by text editor, Y. Chen and Shimuzu – revisit.

JCT3V-D0226 3D-CE1.h: Cross check of reference view selection in NBDV and VSP (JCT3V-D0166) [S. Shimizu, S. Sugimoto (NTT)] [late]

JCT3V-D0168 CE1.h related: Forward Block-based View Synthesis Prediction [Y. Zhang, Y. Zhao, L. Yu (Zhejiang University)]

This contribution describes the forward block-based view synthesis prediction (FVSP) design for 3D-HEVC. FVSP estimates a reference block in the base view by only using the base view depth map. Then pixels in the reference block are warped to a target block in current coding view to be the prediction block of current coding block. The overall bitrate savings on the coded and synthesized views are -0.2% for XGA sequences and -0.7% for HD sequences compared to HTM-6.0 and the decoding time is 98.1% for all sequences on average. The maximum bitrate savings of texture on dependent left and right views are -6.2% and -6.6%, respectively.

Presentation deck not uploaded.

The benefit compared to block-based B-VSP is on average 0.4% for coded and 0.5% for synthesized views; the method suggested here is pixel-based. According to the results of  CE1.h (JCT3V-D0165) could achieve 0.2% gain by pixel-based B-VSP.

Two experts suggest to study the method of JCT3V-D0168 in upcoming CE; this should include a thorough complexity analysis (in terms of number of computation, memory access etc.) compared to current BVSP approach.

  - JCT3V-D0168: Forward block-based VSP

- Forward warping based view synthesis as in the post processing

- Pixels within the reference block are warped

- Reference block is determined using depth map on reference view

- 1x1 warping is performed

|  | BD-Rate (Coded PSNR) | BD-Rate (Synthesis PSNR) | Encoder runtime | Decoder runtime | Crosschecked |
| --- | --- | --- | --- | --- | --- |
| Proposal | -0.44% | -0.50% | 103.8% | 98.1% | N? |

JCT3V-D0208 3D-CE1.h related: Cross check on Samsung's proposal (JCT3V-D0124) [X. Zhao (Qualcomm)] [late]

## CE2: Disparity vector derivation

### Summary

JCT3V-D0022 CE2: Summary Report on Disparity Vector Derivation [Y. Chen, S. Yea]

### CE contributions

#### AVC

JCT3V-D0107 3D-CE2.a: MB-level depth-to-DV conversion in ATM [J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek), J.-Y. Lee, H-C. Wey, C. Kim (Samsung), G. Bang, W.-S. Cheong, N. Hur (ETRI), K.-Y. Kim, Y.- S. Heo, G.- H. Park (KHU)]

JCT3V-D0231 3D-CE2.a: Cross check of joint proposal of Samsung and MediaTek (JCT3V-D0107) [S. Shimizu, S. Sugimoto (NTT)] [late]

JCT3V-D0232 3D-CE2.a: Cross check of MB-level depth-to-DV conversion in ATM (JCT3V-D0107) [S. Shimizu, S. Sugimoto (NTT)] [late]

#### HEVC

JCT3V-D0176 CE2.h: Enhanced disparity vector derivation [J. Kang, Y. Chen, L. Zhang, M. Karczewicz (Qualcomm)]

JCT3V-D0302 Cross check of CE2.h: Enhanced disparity vector derivation (JCT3V-D0176) [O. Nakagami (Sony)] [late]

JCT3V-D0171 Cross-check of Qualcomm CE2 proposal [J. W. Jung, S. Yea (LG)] [late]

JCT3V-D0087 CE2.h: Crosscheck of enhanced disparity vector derivation (JCT3V-D0176) (Hongbin Liu (??))] [late]

### Related contributions

#### AVC

JCT3V-D0108 3D-CE2.a related: Simplified disparity vector derivation in Skip mode and Direct mode [J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCT3V-D0185 CE2.a related: MB-level NBDV for 3D-AVC [Y. Chen, J. Kang, L. Zhang, X. Zhao, Y.-K. Wang, R. Joshi, M. Karczewicz(Qualcomm)]

JCT3V-D0233 3D-CE2.a related: Cross check of Simplified disparity vector derivation in Skip mode and Direct mode (JCT3V-D0108) [S. Shimizu, S. Sugimoto (NTT)] [late]

JCT3V-D0222 CE2.a: Crosscheck on Qualcomm proposal JCT3V-D0185 [D. Tian (MERL)] [late]

JCT3V-D0260 3D-CE2.a related: Crosscheck on MB-level NBDV for 3D-AVC (JCT3V-D0185) [Y.-L. Chang (MediaTek)] [late]

JCT3V-D0186 CE2.a related: Derived disparity vector in 3D-AVC [X. Zhao, Y. Chen, L. Zhang, M. Karczewicz(Qualcomm)]

JCT3V-D0259 3D-AVC: Cross-check result on the package of NBDV proposed by Qualcomm (JCT3V-D0186) [G. Bang (ETRI), K.Y. Kim, Y.S. Heo, G.H. Park (KHU), G.S. Lee, N.H. Hur (ETRI)] [late]

JCT3V-D0187 CE2.a related: Simplifications on the disparity vector derivation in 3D-AVC [V. Thirumalai, L. Zhang, Y. Chen, M. Karczewicz(Qualcomm)]

JCT3V-D0072 3D-CE2.a related: Cross-check result on the package 3 of disparity vector derivation proposed by Qualcomm [G.Bang(ETRI), K.Y. Kim, Y.S. Heo, G.H. Park(KHU), G.S. Lee, N.H. Hur(ETRI)] [late]

#### HEVC

JCT3V-D0095 CE2.h related Improved disparity vector derivation [J. W. Jung, J. Heo, T. Kim, S. Yea (LG)]

JCT3V-D0129 3D-CE2.h related: Cross check of Improved disparity vector derivation (JCT3V-D0095) [M. W. Park, C. Kim (Samsung)] [late]

JCT3V-D0104 3D-CE2.h related: Problem fix and simplification for DV derivation [Y.-W. Chen, N. Zhang, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek)]

JCT3V-D0130 3D-CE2.h related: Cross check of Problem fix and simplification for DV derivation (JCT3V-D0104) [M. W. Park, C. Kim (Samsung)] [late]

JCT3V-D0112 3D-CE2.h related: Default disparity vector derivation [M. W. Park, J. Y. Lee, B. Choi, J. Yoon, H.-C. Wey, Y. Cho, C. Kim (Samsung)]

JCT3V-D0276 CE2.h related: Cross check on Default disparity vector derivation (JCT3V-D0112) [J. Kang, Y. Chen (Qualcomm)] [late]

JCT3V-D0113 3D-CE2.h related: Simplification of disparity vector derivation [M. W. Park, J. Y. Lee, J. Yoon, H.-C. Wey, B. Choi, Y. Cho, C. Kim (Samsung)]

JCT3V-D0230 3D-CE2.h related: Cross check of Simplification of disparity vector derivation (JCT3V-D0113) [S. Shimizu, S. Sugimoto (NTT)] [late]

JCT3V-D0138 3D-CE2.h related: Simplified DV derivation for DoNBDV and BVSP [Y.-L. Chang, Y.-W. Chen, J.-L. Lin, N. Zhang, J. An, Y.-W. Huang, S. Lei (MediaTek)]

JCT3V-D0134 3D-CE2.h related: Cross check of Simplified DV derivation for DoNBDV and BVSP (JCT3V-D0138) [J. Yoon, J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-D0143 3D-CE2.h related: report on coding performance of improved DV searching order [J. Heo, S. Yea (LG)]

JCT3V-D0149 3D-CE2.h related: Inter-view scaling for DV derivation [J. An, K. Zhang, J.-L. Lin, S. Lei (MediaTek)]

JCT3V-D0252 3D-CE2.h related: cross-check of Inter-view scaling for DV derivation (JCT3V-D0149) [T. Ikai (Sharp)] [late]

JCT3V-D0170 CE2.h related: Enhanced Disparity Vector Prediction [Y. Zhang, Y. Zhao, L. Yu (Zhejiang University)]

JCT3V-D0180 CE2.h related: Bug fix on unavailable disparity vector in 3D-HEVC [J. Kang, Y. Chen, M. Karczewicz(Qualcomm)]

JCT3V-D0181 CE2.h related: CU-based Disparity Vector Derivation in 3D-HEVC [J. Kang, Y. Chen, L. Zhang, M. Karczewicz(Qualcomm)]

JCT3V-D0250 CE2.h related: cross-check of CU-based Disparity Vector Derivation (JCT3V-D0181) [T. Ikai (Sharp)] [late]

JCT3V-D0194 CE2.h related: Derived disparity vector for 3D-HEVC [L. Zhang, Y. Chen, M. Karczewicz(Qualcomm)]

JCT3V-D0131 3D-CE2.h related: Cross check of Qualcomm's proposal (JCT3V-Dxxxx) [M. W. Park, C. Kim (Samsung)] [late]

JCT3V-D0132 3D-CE2.h related: Cross check of Qualcomm's proposal (JCT3V-Dxxxx) [M. W. Park, C. Kim (Samsung)] [late]

JCT3V-D0291 CE2.h related: Crosscheck on Enhanced Disparity Vector Prediction [X. Zheng (Hisilicon)] [late]

## CE3: Inter-component prediction

### Summary

JCT3V-D0023 3D-CE3: Summary report of Core Experiment 3 on Inter-Component Prediction [J. Jung, S. Kamp]

### CE contributions

#### AVC

#### HEVC

### Related contributions

#### AVC

#### HEVC

JCT3V-D0120 3D-CE3.h related: MPI restriction [J. Y. Lee, C. Kim (Samsung)]

Motion parameter inheritance (MPI) was integrated on the current 3D-HTM 6.0 (JCT3V-C0137). MPI is used as a new candidate in Skip and Merge modes in a depth component. Part a) This proposal uses MPI only in Skip and Merge modes of a 2Nx2N partition, because a motion vector correlation between texture and depth components is less correlated in partitioned blocks than 2Nx2N block. Part b) Also, the proposed method does not inherit view synthesis prediction (VSP) instead of MPI when a corresponding texture block is coded with VSP. Experimental results demonstrate that the proposed method simplifies the MPI process without the coding loss.

Presentation deck to be uploaded.

About a) Worst-case complexity would be increased, as it is necessary to perform one more condition check for 2Nx2N partition at the decoder, upon which MPI would be invoked.

About b) is obsolete, as it was already decided to disable VSP of depth, and a solution on how to derive the motion vector for depth map will be found (see under CE1hrelated i).

JCT3V-D0267 3D-CE3.h related: Cross-Check of the results on the MPI restriction proposed in JCT3V-D0120 [Y.-W. Chen, J.-L. Lin (MediaTek)] [late]

JCT3V-D0121 3D-CE3.h related: Removal of depth quadtree prediction and simple early depth CU decision for fast encoding [J. Y. Lee, C. Kim (Samsung)]

Depth quadtree prediction was integrated on the current 3D-HTM (JCT3V-B0068). The depth quadtree prediction forces an encoder to limit the depth quadtree at the same level as the texture quadtree, so the encoding time can be significantly reduced by about 75%. Also for the coding improvement, a split flag and a partition size in the depth CU are not transmitted to a decoder, but predicted by referring the coding information of the collocated texture CU. Hence, there exists a parsing dependency between the texture and depth components. To solve this parsing problem while maintaining low encoding complexity, this proposal introduces early depth CU determination instead of the depth quadtree prediction. The proposed method does not split a depth CU further when the best mode of the texture CU is Skip as well as the collocated texture CU is coded with Skip. The result shows that the proposed method maintains the current encoding complexity without the parsing dependency.

Presentation deck to be uploaded.

Two results are presented: a) removing the CU tree dependency of the depth map from the texture CU, which results in 0.3% BR increase both for coded and synthesized views. b) adding an encoder optimization that makes a decision of terminating the depth CU tree earlier depending on the skip mode of texture CU, which shortens the above-mentioned gap to 0.1% and 0.2% BR increase, while still allowing independent parsing. However, this encoder solution seems to have a recently detected bug, according to the report in JCT3V-D0303. Revisit after new results with bug fix are available.

JCT3V-D0303 3D-CE3.h-related: Comments on contribution JCT3V-D0121 on early depth CU decision [J. Jung, E. Mora (Orange Labs)] [late]

(include abstract)

Contributors not present at the meeting, but was inspected in the context of JCT3V-D0121.

JCT3V-D0268 3D-CE3.h related: Cross-Check of the results on the removal of depth quadtree prediction and simple early depth CU decision proposed in JCT3V-D0121 [Y.-W. Chen, J.-L. Lin (MediaTek)] [late]

## CE4: Residual prediction

### Summary

JCT3V-D0024 CE4: Summary Report on Residual Prediction [L. Zhang]

### CE contributions

JCT3V-D0177 CE4: Advanced residual prediction for multiview coding [L. Zhang, Y. Chen, X. Li, M. Karczewicz (Qualcomm)]

JCT3V-D0093 Cross-check of Qualcomm's proposal JCT3V-D0177 [T. Kim, S. Yea (LGE)]

### Related contributions

JCT3V-D0063 3D-CE4.h related: Simplified residual prediction with luma clipping and chroma removal [T. Ikai (Sharp)]

JCT3V-D0265 3D-CE4.h related: Crosscheck of Sharp's JCT3V-D0063 on simplified residual prediction [J. An, J.-L. Lin (MediaTek)] [late]

JCT3V-D0148 3D-CE4.h related: Limited inter-view residual prediction by CBF checking [J. An, K. Zhang, J.-L. Lin, S. Lei (MediaTek)]

JCT3V-D0253 3D-CE4.h related: cross-check of Limited inter-view residual prediction by CBF checking (JCT3V-D0148) [T. Ikai (Sharp)] [late]

## CE5: Inter-view / motion prediction

### Summary

JCT3V-D0025 3D-CE5: Summary report on inter-view/motion prediction [Y.-L. Chang, S. Yea]

### CE contributions

#### AVC

#### HEVC

JCT3V-D0096 CE5.h The results on simplified illumination compensation [J. W. Jung, H. Liu, J. Jia, S. Yea (LG)]

JCT3V-D0258 CE5.h: cross-check of The results on simplified illumination compensation (JCT3V-D0096) [T. Ikai (Sharp)] [late]

JCT3V-D0178 CE5.h: Merge candidates derivation from vector shifting [V. Thirumalai, L. Zhang, Y. Chen, M. Karczewicz(Qualcomm), T. Guionnet, C. Guillemot, L. Guillo(INRIA)]

JCT3V-D0261 3D-CE5.h: Crosscheck on Merge candidates derivation from vector shifting (JCT3V-D0178) [Y.-L. Chang (MediaTek)] [late]

JCT3V-D0248 CE5.h : Inter-view prediction using image deformation characteristics between multi-view images [T. Kim, J. Heo, S. Yea (LG)] [late]

### Related contributions

#### AVC

JCT3V-D0192 CE5.a related: Direct/skip mode simplification for 3D-AVC [V. Thirumalai, L. Zhang, Y. Chen, M. Karczewicz(Qualcomm)]

JCT3V-D0262 3D-CE5.a related: Crosscheck on Direct/skip mode simplification for 3D-AVC (JCT3V-D0192) [Y.-L. Chang (MediaTek)] [late]

#### HEVC

JCT3V-D0060 3D-CE5.h related: Removal of parsing dependency for illumination compensation [T. Ikai (Sharp)]

JCT3V-D0274 CE5.h related: Cross check on Removal of parsing dependency for illumination compensation (JCT3V-D0060) [J. Kang, Y. Chen(Qualcomm)] [late]

JCT3V-D0061 3D-CE5.h related: Illumination compensation regression improvement and simplification [T. Ikai (Sharp)]

JCT3V-D0099 Cross-check of 3D-CE5.h related: Illumination compensation regression improvement and simplification(JCT3V-D0061) [J. W. Jung, S. Yea (LG)]

JCT3V-D0062 3D-CE5.h related: Combination of JCT3V-D0060 and JCT3V-D0061 [T. Ikai (Sharp)]

JCT3V-D0275 CE5.h related: Cross check on Combination of JCT3V-D0060 and JCT3V-D0061 (JCT3V-D0062) [J. Kang, Y. Chen (Qualcomm)] [late]

JCT3V-D0091 CE5.h related : Inter-view SAO process in 3DV coding [T. Kim, J. Heo, S. Yea (LG)]

JCT3V-D0279 CE5.h related: Cross check on Inter-view SAO process in 3DV coding (JCT3V-D0091) [J. Kang, L. Zhang] [late]

JCT3V-D0106 3D-CE5.h related: On complexity reduction of bi-prediction for illumination compensation [Y.-W. Chen, T.-D. Chuang, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek)]

JCT3V-D0300 3D-CE5.h related: Cross check of complexity reduction of bi-prediction for illumination compensation (JCT3V-D0106) [S. Shimizu, S. Sugimoto (NTT)] [late]

JCT3V-D0109 3D-CE5.h related: Additional inter-view merging candidate [J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCT3V-D0256 3D-CE5.h related: cross-check of Additional inter-view merging candidate (JCT3V-D0109) [T. Ikai (Sharp)] [late]

JCT3V-D0114 3D-CE5.h related: Simplification of illumination compensation [M. W. Park, J. Y. Lee, H.-C. Wey, J. Yoon, B. Choi, Y. Cho, C. Kim (Samsung)]

JCT3V-D0146 3D-CE5.h related: Cross check on Simplification of illumination compensation [J. Heo, S. Yea (LG)] [late]

JCT3V-D0122 D-CE5.h related: AMVP candidate list construction [J. Y. Lee, M. W. Park, C. Kim (Samsung)]

JCT3V-D0249 3D-CE5.h related: cross-check of AMVP candidate list construction (JCT3V-D0122) [T. Ikai (Sharp)] [late]

JCT3V-D0135 3D-CE5.h related: Unification of disparity vector rounding [J. Nam, S. Yea (LG)]

JCT3V-D0128 3D-CE5.h related: Cross check of Unification of disparity vector rounding (JCT3V-D0135) [M. W. Park, C. Kim (Samsung)] [late]

JCT3V-D0145 3D-CE5.h related: Unification of the reference view for inter-view motion prediction and inter-view residual prediction [J. An, J.-L. Lin, K. Zhang, S. Lei (MediaTek)]

JCT3V-D0254 3D-CE5.h related: cross-check of Unification of the reference view for inter-view motion prediction and inter-view residual prediction (JCT3V-D0145) [T. Ikai (Sharp)] [late]

JCT3V-D0152 3D-CE5.h related: Removal of parsing dependency for illumination compensation [K. Zhang, Y.-W. Chen, J. An, J.-L. Lin, S. Lei (MediaTek)]

JCT3V-D0255 3D-CE5.h related: cross-check of Removal of parsing dependency for illumination compensation (JCT3V-D0152) [T. Ikai (Sharp)] [late]

JCT3V-D0086 3D-CE5.h related: Crosscheck of removal of parsing dependency for illumination compensation (JCT3V-D0152) [Hongbin Liu (??)] [late]

## CE6: Depth intra coding

### Summary

JCT3V-D0026 CE6 Summary Report: Depth intra coding [P. Merkle]

### CE contributions

JCT3V-D0032 CE6.H: Results on CABAC Context Reduction for Simplified Depth Coding (SDC) [F. Jäger (RWTH Aachen University), J. Heo (LG)]

JCT3V-D0037 CE6.H: Cross check of CABAC Context Reduction for SDC (JCT3V-D0032)  [S. Van Leuven, J. De Cock, G. Van Wallendael, Rik Van de Walle (Ghent University - iMinds)] [late]

JCT3V-D0035 CE6.H: Results on DLT for DMM deltaDC coding [P. Merkle, K. Müller, T. Wiegand (HHI)]

JCT3V-D0203 3D-CE6.h related: Cross check on DLT for DMM deltaDC coding (JCT3V-D0035) [X. Zhao (Qualcomm)] [late]

JCT3V-D0141 3D-CE6.h: Concatenate binarization for residual index coding [J. Heo, S. Yea (LG)]

JCT3V-D0157 3D-CE6.h: Cross check of LG's Concatenate binarization for residual index coding (JCT3V-D0141) [I. Lim, H. Wey (??)]

JCT3V-D0142 3D-CE6.h: Improved DLT coding method [J. Heo, S. Yea (LG)]

JCT3V-D0133 3D-CE6.h related: Cross check of Improved DLT coding method (JCT3V-D0142) [M. W. Park, C. Kim (Samsung)] [late]

JCT3V-D0147 3D-CE6.h: Results on Signalling of Depth Lookup Table [I. Lim, H. Wey, J. Y. Lee, B. Choi, C. Kim]

JCT3V-D0150 3D-CE6.h: Results on Signalling and DPCM coding of Depth Lookup Table [I. Lim, H. Wey, J. Y. Lee, B. Choi, C. Kim, J. Heo, S. Yea]

JCT3V-D0282 3D-CE6.h related: Crosscheck on Signalling and DPCM coding of Depth Lookup Table from SamSung (JCT3V-D0150) [Z. Deng, W. Zhang (??)] [late]

JCT3V-D0304 3D-CE6.h related: Crosscheck results on signalling and DPCM coding of depth lookup table (JCT3V-D0150-v4) [M. Li, P. Wu (ZTE)] [late]

JCT3V-D0151 3D-CE6.h results on an efficient coding method for DLT in 3DVC [K. Zhang, J. An, S. Lei (MediaTek)]

JCT3V-D0269 CE6.H: Cross check of efficient DLT coding in SPS (JCT3V-D0151) [P. Merkle (HHI)] [late]

### Related contributions

#### AVC

JCT3V-D0110 3D-CE6.a related: Sample-based simplified depth coding [J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCT3V-D0257 3D-CE6.h related: cross-check of Sample-based simplified depth coding (JCT3V-D0110) [T. Ikai (Sharp)] [late]

JCT3V-D0281 3D-CE6.h related: Crosscheck on Sample-based SDC from MTK (JCT3V-D0110) [Z. Deng, W. Zhang (??)] [late]

#### HEVC

JCT3V-D0033 CE6.H related: Results on Complexity Reduction for Simplified Depth Coding (SDC) [F. Jäger (RWTH Aachen University)]

JCT3V-D0034 CE6.H related: Results on Fixes for Context Initialization and Unary Coding for SDC [F. Jäger (RWTH Aachen University)]

JCT3V-D0205 3D-CE6.h related: Cross check on fixes for context initialization and unary coding for SDC (JCT3V-D0034) [X. Zhao (Qualcomm)] [late]

JCT3V-D0036 CE6.H related: Results on aligning 3D-HTM software with 3D-HEVC draft specification for Wedgelet pattern construction [P. Merkle, K. Müller, T. Wiegand (HHI)]

JCT3V-D0040 CE6.H related: Cross check of aligning 3D-HTM software with 3D-HEVC draft specification for Wedgelet pattern construction (D0036) [F. Jäger (RWTH Aachen University)]

JCT3V-D0054 CE6.H related: Results on Updating Mechanism for Coding of Depth Lookup Table (Delta-DLT) [F. Jäger (RWTH Aachen University)]

JCT3V-D0055 CE6.H related: Cross Check of Updating Mechanism for Coding of Depth Lookup Table (Delta-DLT) (JCT3V-D0054) [S. Van Leuven, G. Van Wallendael, R. Bailleul, J. De Cock, R. Van de Walle] [late]

JCT3V-D0066 CE6.h related: Removal of DC from SDC Mode [H. Liu, J. Jia, J. Heo, J. Nam (??)]

JCT3V-D0067 CE6.h related: Removal of Overlap between DMM1 and DMM3 [S. Wang, H. Liu, S. Ma, J. Jia (??)]

JCT3V-D0285 CE6.h releated: Crosscheck on Removal of Overlap between DMM1 and DMM3 (D0067) [X. Zheng (Hisilicon)] [late]

JCT3V-D0076 CE6.H related: Results on Fixes for Region Boundary Chain Coding in 3D-HEVC draft specification [P. Lu, L. Yu (??)]

JCT3V-D0077 CE6.H related: Results on Modified Binarization for Region Boundary Chain Coding [P. Lu, L. Yu (??)]

JCT3V-D0123 3D-CE6.h related: DMM simplification and signalling [J. Y. Lee, H.-C. Wey, M. W. Park, C. Kim (Samsung)]

JCT3V-D0271 CE6.H related: Cross check of DMM simplification and signalling (JCT3V-D0123) [P. Merkle (HHI)] [late]

JCT3V-D0124 3D-CE6.h related: SDC with DLT in intra slices [J. Y. Lee, H.-C. Wey, C. Kim (Samsung)]

JCT3V-D0136 3D-CE6.h related: Improvement of SDC coding [J. Nam, H. Liu, S. Yea (LG)]

JCT3V-D0159 3D-CE6.h related: Cross check of LG's proposal (JCT3V-D0136) [I. Lim, H. Wey (??)] [late]

JCT3V-D0153 3D-CE6.h related: A flexible DLT signaling method in 3D-HEVC [K. Zhang, Y.-W. Huang, J. An, J.-L. Lin, S. Lei (MediaTek)]

JCT3V-D0155 3D-CE6.h related: Direct simplified depth coding in 3D-HEVC [K. Zhang, J. An, J.-L. Lin, Y.-W. Chen, S. Lei (MediaTek)]

JCT3V-D0100 Crosscheck of MTK's CE6.h related proposal [J. W. Jung, S. Yea (LG)] [late]

JCT3V-D0163 CE6.h related SDC with WVSO metric [J. W. Jung, S. Yea (LG)]

JCT3V-D0169 CE6.h related: Results on disabling of DMM 4 [Y. Song, Y.-S. Ho (GIST)]

JCT3V-D0172 3D-CE6.h related: Differential coding method for DLT in 3D-HEVC [M. Li, P. Wu, H. Li, G. Shang, Y. Xie (ZTE)]

JCT3V-D0173 CE6.h related: Removal of division operation for DMM and SDC [X. Zheng, Y. Lin, J. Zheng (??)]

JCT3V-D0204 3D-CE6.h related: Cross check on removal of division operation for DMM and SDC (JCT3V-D0173) [X. Zhao (Qualcomm)] [late]

JCT3V-D0174 CE6.h related: Extension of reference samples sub-sampling to all block-sizes in DMM and SDC [X. Zheng, Y. Lin, J. Zheng (Hisilicon)]

JCT3V-D0283 3D-CE6.h related: Crosscheck on Extension of reference samples sub-sampling to all block-sizes in DMM and SDC (JCT3V-D0174) [Z. Deng, W. Zhang (??)] [late]

JCT3V-D0175 3D-CE6.h related: Fast intra prediction mode selection for intra depth map coding [Z. Gu(SCU), J. Zheng(HiSilicon), N. Ling(SCU), P. Zhang(HiSilicon)] [late]

JCT3V-D0284 3D-CE6.h related: Crosscheck on Fast intra prediction mode selection for intra depth map coding (JCT3V-D0175) [Z. Deng, W. Zhang (??)] [late]

JCT3V-D0182 CE6.h related: On Signaling of DLT for depth coding [X. Zhao, L. Zhang, Y. Chen, M. Karczewicz(Qualcomm)]

JCT3V-D0273 CE6.H related: Cross check of Signaling of DLT for depth coding (D0182) [F. Jäger (RWTH Aachen University)] [late]

JCT3V-D0183 CE6.h related: Simplified DC predictor for depth intra modes [X. Zhao, L. Zhang, Y. Chen, M. Karczewicz(Qualcomm)]

JCT3V-D0270 CE6.H related: Cross check of simplified DC predictor for depth intra modes (JCT3V-D0183) [P. Merkle (HHI)] [late]

JCT3V-D0184 CE6.h related: Wedgelet modes unification [X. Zhao, L. Zhang, Y. Chen, M. Karczewicz(Qualcomm)]

JCT3V-D0190 CE6.h related: SDC without per-pixel prediction [Zhipin Deng, Wenhao Zhang, Lidong Xu, Yu Han, Xiaoxia Cai, Yi-jen Chiu, Hong Jiang (Intel)]

JCT3V-D0266 3D-CE6.h related: Crosscheck of Intel's JCT3V-D0190 on simplified SDC mode [J. An, K. Zhang (MediaTek)] [late]

JCT3V-D0193 CE6.h related: Clean up for 64x64 SDC [Zhipin Deng, Wenhao Zhang, Lidong Xu, Yu Han, Xiaoxia Cai, Yi-jen Chiu, Hong Jiang (Intel)]

JCT3V-D0195 CE6.h related: Unification of new intra modes in 3D-HEVC [Q. Yu(PKU), L. Zhang, Y. Chen, M. Karczewicz(Qualcomm), S. Ma(PKU)]

JCT3V-D0154 3D-CE6.h Cross check of Samsung's Proposal on DLT Signalling [J. Heo, S. Yea] [late]

JCT3V-D0263 3D-CE6.h related: Crosscheck on Removal of dc in SDC from LG [K. Zhang (MediaTek)] [late]

JCT3V-D0264 3D-CE6.h related: Crosscheck on Simplified SDC from RWTH [K. Zhang (MediaTek)] [late]

JCT3V-D0288 CE6.h related: Crosscheck on Wedgelet modes unification [X. Zheng (Hisilicon)] [late]

JCT3V-D0289 CE6.h related: Crosscheck on Clean up for 64x64 SDC [X. Zheng (Hisilicon)] [late]

JCT3V-D0290 CE6.h related: Crosscheck on flexible DLT signaling method in 3D-HEVC [X. Zheng (Hisilicon)] [late]

JCT3V-D0299 CE6.h related: Cross check on Unification of new intra modes in 3D-HEVC [J. Heo (LG)] [late]

## CE7: Skip mode coding

### Summary

### CE contributions

### Related contributions

JCT3V-D0115 Skip flag coding in 3D-AVC [J. Y. Lee, M. W. Park, B. Choi, H.-C. Wey, C. Kim, I. Kovliga, A. Fartukov, M. Mishurovskiy (Samsung)]

JCT3V-D0294 3D-CE7.a: Cross-check of skip flag coding in 3D-AVC (JCT3V-D0115) [D. Rusanovskyy (Nokia)] [late]

# Non-CE technical contributions

## 3DV standard development

### MVC plus depth

### 3D-AVC

JCT3V-D0161 3D-AVC: Coding of unpaired MVD data [L. Chen, D. Rusanovskyy, M. Hannuksela (Nokia)]

JCT3V-D0164 3D-AVC: Removal of texture to depth resolution ratio restrictions [D. Rusanovskyy, P. Aflaki, M. Hannuksela (Nokia)]

JCT3V-D0305 Cross-check of JCT3V-D0164: 3D-AVC: Removal of texture to depth resolution ratio restrictions [G. Bang(ETRI), K.Y. Kim, Y.S. Heo, G.H. Park(KHU), G.S. Lee, N.H. Hur(ETRI)] [late]

### MFC

JCT3V-D0085 Remarks on Multi-resolution Frame-Compatible (MFC) extension of AVC [G. J. Sullivan]

JCT3V-D0097 Study of the Carrier Signal in Multi-resolution Frame-Compatible (MFC) extension of AVC [T. Lu, H. Ganapathy, P. Yin, T. Chen, W. Husak (Dolby)]

JCT3V-D0098 Editorial Improvements on PDAM5 study text for MFC (multi-resolution frame compatible stereo coding) [P. Yin, H. Ganapathy, T. Lu, T. Chen, W. Husak (Dolby)]

JCT3V-D0101 Syntax modification for backward compatibility of MFC [G.-Y. Lee, J.-Y. Suh, Y. Jeon, S. Park (??)]

### MV-HEVC

JCT3V-D0056 AHG13: Disparity vector constraint study on 4K stereo sequences [O. Nakagami, T. Suzuki (Sony)]

JCT3V-D0064 AHG13: Parallel decoding SEI message [T. Ikai, Y. Yamamoto (Sharp)]

## High-level syntax

### AVC related

### HEVC related

(to be discussed jointly with JCT-VC)

JCT3V-D0041 MV-HEVC/SHVC HLS: On access unit definition [M. M. Hannuksela (Nokia)]

JCT3V-D0042 MV-HEVC/SHVC HLS: On inter-layer sample and syntax prediction indications [M. M. Hannuksela, K. Ugur (Nokia)]

JCT3V-D0043 MV-HEVC/SHVC HLS: Layer-wise startup of the decoding process [M. M. Hannuksela (Nokia)]

JCT3V-D0044 MV-HEVC/SHVC HLS: Step-wise layer access (STLA) pictures [M. M. Hannuksela (Nokia)]

JCT3V-D0045 MV-HEVC/SHVC HLS: On some fundamental concepts in HEVC 3DV and scalable extensions [Y.-K. Wang, J. Chen, Y. Chen (Qualcomm)]

JCT3V-D0046 MV-HEVC/SHVC HLS: On cross-layer alignments in HEVC 3DV and scalable extensions [Y.-K. Wang, A. K. Ramasubramonian, K. Rapaka, J. Chen, V. Seregin, Y. Chen (Qualcomm)]

JCT3V-D0047 MV-HEVC/SHVC HLS: VPS and SPS designs in HEVC 3DV and scalable extensions [Y.-K. Wang, Y. Chen, A. K. Ramasubramonian (Qualcomm)]

JCT3V-D0048 MV-HEVC/SHVC HLS: Signalling and derivation of inter-layer RPS for HEVC 3DV and scalable extensions [Y.-K. Wang, J. Chen, K. Rapaka, Y. Chen, X. Li, M. Karczewicz (Qualcomm)]

JCT3V-D0049 MV-HEVC/SHVC HLS: On DPB operations in HEVC 3DV and scalable extensions [Y. Chen, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)]

JCT3V-D0050 MV-HEVC/SHVC HLS: Multi-mode bitstream extraction in HEVC 3DV and scalable extensions [Y.-K. Wang, Y. Chen, A. K. Ramasubramonian (Qualcomm)]

JCT3V-D0051 AHG 13: Restriction of inter-layer prediction on tile boundaries [G. Tech, Y. Sanchez, K. Suehring, R. Skupin, T. Schierl (HHI)]

JCT3V-D0052 Extension parameter set for 3D-HEVC and MV-HEVC [B. Choi, Y. Cho, E. Alshina, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCT3V-D0053 Inter-layer reference picture set initialization for MV-HEVC [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCT3V-D0057 AHG7: On initialization process of reference picture lists for HEVC extensions (JCTVC-M0081) [O. Nakagami, T. Suzuki (Sony)]

JCT3V-D0306 Cross-check of JCTVC-D0057: AHG7: On initialization process of reference picture lists for HEVC extensions [?? (??)] [late]

JCT3V-D0059 MV-HEVC/SHVC HLS: High-level syntax modifications for HEVC extensions [J. Boyce, D. Hong, W. Jang (Vidyo)]

JCT3V-D0074 MV-HEVC: On Marking for sub-layer non-reference pictures [X. Yang, P. Yue, Y. Zhang (??)]

JCT3V-D0081 MV-HEVC/SHVC HLS: Comments on SHVC and MV-HEVC [S. Deshpande (Sharp)] [late]

JCT3V-D0082 MV-HEVC/SHVC HLS: On Design for Signaling Inter-layer Prediction [S. Deshpande (Sharp)] [late]

JCT3V-D0083 MV-HEVC/SHVC HLS: On Signaling DPB Parameters and DPB Operation [S. Deshpande (Sharp)] [late]

JCT3V-D0084 MV-HEVC/SHVC HLS: On Output Layer Sets Change Signaling [S. Deshpande (Sharp)] [late]

JCT3V-D0140 3D-HEVC HLS: Camera parameter signaling and Depth reference selection [Y.-L. Chang, Y.-P. Tsai, S. Lei (MediaTek), D. Tian, A. Vetro (MERL)]

JCT3V-D0229 3D-HEVC HLS: Cross check of Camera parameter signaling and Depth reference selection (JCT3V-D0140) [S. Shimizu, S. Sugimoto (NTT)] [miss]

JCT3V-D0196 AHG7: Comments on parameter sets for MV-HEVC [Y. Chen, Y.-K. Wang(Qualcomm)]

JCT3V-D0197 AHG7: Slice header prediction for MV-HEVC [A. K. Ramasubramonian, Y. Chen, Y.-K. Wang(Qualcomm)]

JCT3V-D0198 AHG7: Inter-layer reference pictures in reference picture list initialization [A. K. Ramasubramonian, Y. Chen, Y.-K. Wang, L. Zhang(Qualcomm)]

JCT3V-D0199 AHG7: Parallel decoding SEI message for MV-HEVC [Y. Chen, A. K. Ramasubramonian, Y.-K. Wang, S. Seregin, L. Zhang(Qualcomm)]

JCT3V-D0202 MV-HEVC/3D-HEVC HLS: on ViewId signalling [M. M. Hannuksela (Nokia)]

JCT3V-D0211 MV-HEVC/SHVC HLS: Extending the supported number of layers [K. Suehring, G. Tech, R. Skupin, T. Schierl(HHI)]

JCT3V-D0218 3D reference displays information SEI message for MV-HEVC and 3D-HEVC [A. Norkin (Ericsson)]

JCT3V-D0220 AHG7: MV-HEVC inter-view reference picture list construction based on ViewId [T. Rusert, A. Norkin (Ericsson)]

JCT3V-D0235 Reference picture marking process for MV-HEVC and other HEVC extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCT3V-D0236 Unused reference picture management for MV-HEVC and other HEVC extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCT3V-D0237 On video parameter set extension for MV-HEVC and other HEVC extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCT3V-D0238 Layer identifier extension for MV-HEVC and other HEVC extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCT3V-D0239 On parameter set prediction for MV-HEVC and other HEVC extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCT3V-D0240 Parameter set and other related issues for MV-HEVC and other HEVC extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCT3V-D0241 On random access point for MV-HEVC and other HEVC extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCT3V-D0242 Decoded picture buffer for MV-HEVC and other HEVC extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCT3V-D0243 Temporal layer management for MV-HEVC and other HEVC extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCT3V-D0244 Slice header design for MV-HEVC and other HEVC extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

## Other technical contributions on coding layer

### HEVC related

JCT3V-D0065 On collocated picture and low-delay checking for MV-HEVC [Y. Lin, X. Zheng, X. Chen, J. Zheng (HiSilicon)]

JCT3V-D0088 Crosscheck of on collocated picture and low-delay checking for MV-HEVC (JCT3V-D0065) [Hongbin Liu, Jiwook Jun, Shiqi Wang (??)] [late]

JCT3V-D0069 Non-CE: Simplified Inter Mode Coding of Depth [H. Liu, J. Jia, J. Nam (??)]

JCT3V-D0206 Cross check on simplified inter mode coding of depth (JCT3V-D0069) [X. Zhao (Qualcomm)] [late]

JCT3V-D0209 Inter-view motion field mapping for MV-HEVC [D. Bugdayci, K. Ugur, M. Hannuksela, D. Rusanovskyy (Nokia)]

JCT3V-D0245 MV-HEVC: Crosscheck on Nokia proposal JCT3V-D0209 [D. Tian (MERL)] [late]

### AVC related

JCT3V-D0078 Deblocking for ALC in 3D-AVC [K.Y. Kim, H.S. Hwang, G.H. Park (KHU)]

JCT3V-D0280 Cross check of deblocking for ALC in 3D-AVC (JCT3V-D0078) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-D0298 Crosscheck - Deblocking for ALC in 3D-AVC (JCT3V-D0078) [J. Yoo, W. Lim, H. Jo, D. Sim] [late]

JCT3V-D0116 Simplification of adaptive luminance compensation in 3D-AVC [J. Y. Lee, M. W. Park, C. Kim, I. Kovliga, A. Fartukov, M. Mishurovskiy (Samsung)]

JCT3V-D0277 Cross check on Simplification of adaptive luminance compensation in 3D-AVC (JCT3V-D0116) [late]

JCT3V-D0188 Simplified adaptive luminance compensation in 3D-AVC [J. Kang, Y. Chen, L. Zhang, M. Karczewicz(Qualcomm)]

JCT3V-D0127 Cross check of Simplified adaptive luminance compensation in 3D-AVC (JCT3V-D0188) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-D0189 Bug fix and simplifications for ATM IVMP [X. Zhao, L. Zhang, Y. Chen, M. Karczewicz(Qualcomm)]

JCT3V-D0295 Cross-check on bug fix and simplifications for ATM IVMP, Test 2.1 in JCT3V-D0189 [D. Rusanovskyy (Nokia)] [late]

JCT3V-D0200 Improved inside-view motion prediction for 3D-AVC [L. He, X. Zhao, L. Zhang, Y. Chen(Qualcomm), J. Y. Lee, H.-C. Wey, C. Kim (Samsung)]

JCT3V-D0296 Cross-check of improved inside-view motion prediction for 3D-AVC proposed in (JCT3V-D0200) [D. Rusanovskyy (Nokia)] [late]

## Conformance

## Software

### General

### AVC related

JCT3V-D0073 AHG4: Interlace coding on ATM 7.2 [C.-C. Lin, F.-C. Chen, C.-H. Hung(ITRI)]

### HEVC related

JCT3V-D0167 Multiview and multiview-video-plus-depth support in SHM [A. Hallapuro, Y. Yan, D. Rusanovskyy, M. Hannuksela (Nokia)] [late]

JCT3V-D0287 AHG5 Report on update of HTM: MV-HEVC Software based on HM10.0 [H. Brust, G. Tech, K. Mueller (HHI), Y. Chen, L. Zhang (Qualcomm)]

## Source video test material

JCT3V-D0234 AHG11: NICT-3D test seqeunces [S. Shimizu, S. Sugimoto (NTT)] [late]

## Alternative depth formats

JCT3V-D0070 AHG8 Revision on Depth View Information SEI message in 3D-AVC DT5 [T. Senoh, Y. Ichihashi, H. Sasaki (??)]

JCT3V-D0071 AHG8 Description of Global View and Depth Format for Depth View SEI Message [T. Senoh (??)]

JCT3V-D0102 AHG8 GVD (Global View and Depth) Coding using 3D-ATM 7.0 and Request [M. Tanimoto, K. Suzuki (NISRI), T. Senoh (NICT)]

JCT3V-D0103 AHG8: Draft Text for Signaling Warp Maps as an Alternative 3D Format in 3D-HEVC [N. Stefanoski, A. Smolic (DRZ)]

JCT3V-D0272 AHG8: Draft Text for Signaling Global View and Depth as an Alternative 3D Format in 3D-HEVC [Takanori Senoh, Yasuyuki Ichihashi, Hisayuki Sasaki, Kenji Yamamoto, Masayuki Tanimoto] [late]

## Non-normative contributions

JCT3V-D0111 JCT3V – The rate control schemes for 3D multi-view video coding [W. Lim, H. Jo, J. Yoo, D. Sim (??)]

## Complexity assessment

JCT3V-D0090 AhG 10: Complexity Assessment on Adaptive Luminance Compensation and Plane Segmentation based Intra Prediction in 3D-AVC [C.-F. Chen, G. G. Lee (NCKU)]

JCT3V-D0301 AHG 10 Overview of Algorithmic Intrinsic Complexity Measures [G. G. Lee (NCKU), T. Ikai (Sharp), K. Rapaka (Qualcomm), D. Rusanovskyy (Nokia)] [late]

## Quality assessment

JCT3V-D0217 AHG9: On the evaluation on stereoscopic and multiview autostereoscopic displays [Philippe Hanhart, Touradj Ebrahimi (EPFL), on behalf of Qualinet]

## Withdrawn, missing, unclear allocation

JCT3V-D0031 [Withdrawn]

JCT3V-D0038 [Withdrawn]

JCT3V-D0039 [Withdrawn]

JCT3V-D0075 [Withdrawn]

JCT3V-D0144 [Withdrawn]

JCT3V-D0158 Withdrawn

JCT3V-D0210 [Withdrawn]

JCT3V-D0214 [Withdrawn]

# Plenary Discussions and BoG Reports

## Project development

see under relevant topics.

## BoGs

see under relevant topics.

# Project planning

## General issues for CEs

A preliminary CE description is to be approved at the meeting at which the CE plan is established.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.A.a, CEX.H.b (where A stands for AVC and H for HEVC related 3D technology), etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using same software codebase, which should be based on either the ATM or HTM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for experiments were described in the output document JCT3V-D1100.

A deadline of three weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-3V reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the relevant software basis. Exceptions must be justified, discussed on the general JCT-3V reflector, and recorded in the abstract of the summary report.

Final CEs shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

Experiment descriptions should be written in a way such that it is understood as a JCT-3V output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-3V document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

Non-final CE plan documents were reviewed and given tentative approval during the meeting (in some cases with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE is described in an associated output document JCT3V-D11xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-3V is not obliged to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-3V members can participate in a CE

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

## Common Conditions for 3D Video Coding Experiments

Preferred Common Conditions for experiment testing that are intended to be appropriate for both CEs and other experiments were selected by the group and described in output document JCT3V-B1100.

## Software development (update)

ATM software:

Current availability: (see CTC), to be made publicly available without password protection

Version 7.0 (including only adoptions that affect CTC) should be available within 3 weeks after the meeting (Feb 14). Version 7.1 (including all remaining adoptions) is planned to be available 2 weeks later (Feb 28). All CEs should be based on Version 7.0 unless otherwise stated in the CE workplan. Any versions that provide software fixes or features beyond Version 7.0 may be used in CEs as the discretion of the CE coordinator and with consensus of all CE participants.

HTM software:

Current availability: (see CTC), already available without password protection

Version 6.0 (including only adoptions that affect CTC) should be available within 4 weeks after the meeting (Feb 22). Version 6.1 (including all remaining adoptions) is planned to be available 1 week later (Mar 1). All CEs should be based on Version 6.0 unless otherwise stated in the CE workplan. Any versions that provide software fixes or features beyond Version 6.0 may be used in CEs as the discretion of the CE coordinator and with consensus of all CE participants.

Note: It would be desirable in the future to make software integration of adopted tools more in parallel

It is to the discretion of the software coordinators to set up a time line for the integration and request proponents to finish integration by a given date.

Integration Procedure & Guidelines

Integration is done in a serial way. Each integrator cross-checks the version provided by his predecessor.  The cross check for the last version is carried out by the software coordinators.

Integration Guidelines

When integrating

- software changes should be enclosed by macros switchable by defines including company and proposal number e.g.

#define MYCOMPANYS_DEPTHFILTER_JCT3V_B0555  1

#if MYCOMPANYS_DEPTHFILTER_JCT3V_B0555

// do stuff

#endif

- new tools should be made switchable in the cfg-file if reasonable

- cfg-files should be updated

Delivery of software

Before delivering the software to the next integrator it should be checked if

- the software compiles under windows and linux

- software compiles and delivers same results as previous version when integrated tools are disabled by macro or cfg-settings

- there are encoder-decoder mismatches

- there are memory leaks by measuring maximum memory consumption (or specific tools e.g. valgrind) is

- visual quality is not disturbed

Additional to the software cfg-files that reflect proposed settings and an excel sheet with coding results should be provided. Software and cfg-files should be delivered by checking it in to the corresponding (HTM or ATM) software repositories.

When software is delivered this should be announced to the reflector. Moreover, every further change on the software should be announced. If there is a delay in integration this should be communicated to the reflector.

## Cross check

For cross checking the same steps as described in section  should be carried out as soon as possible, but should not take more than five working days. Moreover it should be checked if integrated tools correspond to the adopted proposal. Results of cross check should be announced to the reflector. The result-sheet should be made available to the group.

## Procedure if cross check fails or planned delivery data cannot be held

If a planned delivery date cannot be held this should be announced to the reflector.

If the crosscheck fails or the previous integrator has not delivered the software within 3 days or result sheet within 5 days after planned delivery date

- the current integrator should integrate in the last cross-checked version

- the previous integrator falls back to the end of the integration plan

## Software repositories

### HTM

HTM software can be checked in

https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/

Therefore for each integrator an own software branch will be created by the software coordinator containing the current anchor version or the version of the previous integrator:

e.g.  branches/0.7-mycompany

The branch of the last integrator will become the new release candidate tag.

e.g. tags/0.8rc1

This tag can be cross-check by the group for. If no problems occur the release candidate will become the new tag after 7-days:

e.g. tags/0.8

If reasonable intermediate release candidate tags can be created by the software coordinator.

### ATM

An official release of the 3DV-ATM software can be check out by mpeg3dv SVN users from the following location:

http://mpeg3dv.research.nokia.com/svn/mpeg3dv/trunk/

Following every MPEG 3DV meeting, software coordinator creates a new branch for the integration of adopted proposals:

http://mpeg3dv.research.nokia.com/svn/mpeg3dv/branches/mpeg10X_post_integration/

Software integrator checks-out the software from integration branch at its turn of integration plan and integrates proposal as it is specified in Sections 3.1-3.2.

Software integrator communicates a new software version over the email list to the cross-checker and to the software coordinator.

The software coordinator checks-in to the integration branch every new software integration with confirmed cross-check, as it is specified in the Section 3.3.

Once the integration plan is completed, the software coordinator cross-check version available in the integration branch and checks-in a new official release of the 3DV-ATM to the http://mpeg3dv.research.nokia.com/svn/mpeg3dv/trunk/.

## List of CEs:(update)

- CE1: View synthesis prediction [Coordinator: S. Shimizu]

- CE2: Disparity vector derivation [Coordinator: Y. Chen, S. Yea]

- CE3: Inter-component prediction [Coordinator: J. Jung, S. Kamp]

- CE4: Residual prediction [Coordinator: L. Zhang]

- CE5: Inter-view/motion prediction [Coordinator: Y. Chang, S. Yea]

- CE6: Depth intra coding [Coordinator: P. Merkle]

- CE7: Skip mode coding [Coordinator: M. Mishurovskiy, D. Rusanovskyy]

# Establishment of ad hoc groups (update)

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-3V reflector (jct-3v@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-3V project management (AHG1) (jct-3v@lists.rwth-aachen.de) - Coordinate overall JCT-3V interim efforts. - Report on project status to JCT-3V reflector. - Provide report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| MVC+D / 3D-AVC Draft and Test Model editing (AHG2) (jct-3v@lists.rwth-aachen.de) - Produce and finalize JCT3V-C1001 MVC Extension for Inclusion of Depth Maps Draft Text 6 (the text for FDAM ballot in MPEG and AAP consent in ITU-T). - Produce and finalize JCT3V-C1002 3D-AVC Draft Text 5. - Produce and finalize JCT3V-C1003 3D-AVC Test Model 5 Description. - Gather and address comments for refinement of these documents, resolve potential AAP comments - Coordinate with the 3D-AVC Software Integration AhG to address issues relating to mismatches between software and text. - Set up a bug tracking system. | M. Hannuksela, S. Hattori (co-chairs), Y. Chen, T. Suzuki, J.-R. Ohm, G. Sullivan (vice chairs) | N |
| MV-HEVC / 3D-HEVC Test Model editing (AHG3) (jct-3v@lists.rwth-aachen.de) - Produce and finalize JCT3V-C1005 3D-HEVC Test Model Description draft 3. - Produce and finalize JCT3V-C1004 MV-HEVC text specification draft 3 (submitted for ISO/IEC PDAM ballot). - Gather and address comments for refinement of these documents. - Coordinate with the 3D-HEVC Software Integration AhG to address issues relating to mismatches between software and text. - Set up a bug tracking system. | G. Tech, K. Wegner (co-chairs), J. Boyce, Y. Chen, M. Hannuksela, T. Suzuki, S. Yea, J.-R. Ohm, G. Sullivan (vice chairs) | N |
| 3D-AVC Software Integration (AHG4) (jct-3v@lists.rwth-aachen.de) - Coordinate development of the 3DV-ATM software and its distribution to JCT-3V members - Produce documentation of software usage for distribution with the software - Prepare and deliver 3DV-ATM v7.0 software version and the reference configuration encodings according to JCT3V-C1100 based on common conditions suitable for use in most core experiments (expected within 3 weeks after the meeting). - Prepare and deliver 3DV-ATM v7.1 software version with remaining non-CTC adoptions (expected within 2 weeks after v7.0). - Prepare and deliver 3DV-ATM v7.2 software version with interlace support (expected to be available and cross-checked 1 week before the next meeting). - Perform analysis and reconfirmation checks of the behavior of technical changes adopted into the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with MVC+D / 3D-AVC Draft and Test Model editing (AHG2) to identify any mismatches between software and text. - Prepare one package of reference software related to JCT3V-C1001. - Set up a bug tracking system. | D. Rusanovskyy (chair), F. C. Chen, J. Y. Lee, J.-L. Lin, O. Stankiewicz, D. Tian (vice chairs) | N |
| 3D-HEVC Software Integration (AHG5) (jct-3v@lists.rwth-aachen.de) - Coordinate development of the HTM software and its distribution to JCT-3V members - Produce documentation of software usage for distribution with the software - Prepare and deliver HTM 6.0 software version and the reference configuration encodings according to JCT3V-C1100 based on common conditions suitable for use in most core experiments (expected within 4 weeks after the meeting). - Prepare and deliver HTM 6.1 software that include additional items not integrated into the 6.0 version (expected within 2 weeks after the 6.0 software release). - Prepare the migration of high-level syntax part corresponding to JCT3V-C1004 into HM10 (basic functionalities for multiview coding). - Perform analysis and reconfirmation checks of the behaviour of technical changes adopted into the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with MV-HEVC Draft and 3D-HEVC Test Model editing AhG to identify any mismatches between software and text. - Set up a bug tracking system. | G. Tech, L. Zhang (co-chairs), Y. Chang, K. Wegner (vice chairs) | N |
| 3D Coding Tool Testing (AHG6) (jct-3v@lists.rwth-aachen.de) - Discuss and finalize CE work plans - Study the Common Test Conditions and suggest possible changes - Coordinate between core experiments when necessary. - Study coding tool performance of non-CTC configurations - Report on status of core experiments. - Prepare viewing for 4th JCT-3V meeting. | K. Müller, A. Vetro (co-chairs) | N |
| 3D High level syntax (AHG7) (jct-3v@lists.rwth-aachen.de) - Study high level syntax design extensions of AVC for 3DV functionalities. - Study the relations of AVC multiview/3D extensions and HEVC multiview/3D extensions on high-level syntax concepts, e.g., related to SEI messages. - Identify commonality of multi-view and scalable high-level extensions of HEVC. - Study NAL unit header, video parameter set, sequence parameter set, picture parameter set, and slice header syntax designs. - Study signalling of inter-layer prediction constraints by taking JCTVC-L0449 as a starting point. - Study methods for reduction of SPS redundancy. - Study SEI messages and VUI syntax designs. - Study the hypothetical reference decoder (HRD) syntax and operations and the related text for bitstream conformance and decoder conformance. - Assist in software development and text drafting for the high-level syntax in the HEVC extensions designs. - Coordinate efforts with JCT-VC AHG 9 | M. M. Hannuksela (Chair), J. Boyce, Y. Chen, T. Rusert, Y.-K. Wang (vice chairs) | N |
| Alternative 3D Formats (AHG8) (jct-3v@lists.rwth-aachen.de) - Produce and finalize depth view information SEI message text in JCT3V-C1002 3D-AVC Draft Text 5. - Produce an informative description to help thorough understanding of the depth view information SEI message. - Prepare and deliver Global View and Depth software based on 3D-ATM v7.x software version and the reference configuration encodings. - Discuss and prepare draft text for a SEI message for 3D-HEVC which signals warping maps as an alternative 3D format. | M. Tanimoto, T. Senoh, N. Stefanoski (co-chairs) | N |
| 3D Video Quality Assessment (AHG9) (jct-3v@lists.rwth-aachen.de) - Study objective quality assessment metrics that would provide an accurate evaluation of synthesized views - Prepare test material with a range of codec configurations for systematic assessment of quality metrics and verification testing - Recommend improvements to evaluation methodology | T. Ebrahimi (chair), A.Vetro, V. Baroncini (vice chairs) | N |
| Complexity assessment (AHG10) (jct-3v@lists.rwth-aachen.de) - Study intrinsic complexity measures or metrics to assess hardware and software implementations of algorithms/computational models based on number of operations, potential for parallelism, data transfer rate, and data storage requirements. - Identify criteria to analyze and determine the implement ability of key modules, video sub-systems, and video systems at different scales for ASIC, FPGA, ASIP, multi-core GPU, GPCPU architectures, etc. - Identify bottlenecks in the current 3D video system designs with regard to implementation complexity based on the metrics defined above. - Develop a recommendation to JCT-3V on formalized complexity assessment in a reporting document. | C. Lee (chair), T. Ikai, K. Rapaka, D. Rusanovskyy | N |
| 3D Test Material (AHG11) (jct-3v@lists.rwth-aachen.de) - Identify deficiencies and limitations of current 3D video test material. - Identify, collect, and make available a variety of additional 3D video sequences. - Study the characteristics of test materials and their impact on coding performance, synthesis quality etc. - Recommend appropriate test materials for use in 3D Video Coding Extension Development. - Assess the suitability of the new test material that was offered to the 3rd JCT-3V meeting. | S. Shimizu, S. Yea (co-chairs) | N |
| Conformance testing development (AHG12) (jct-3v@lists.rwth-aachen.de) - Further discuss and improve the conformance draft related to the MVC plus depth extension of AVC (JCT3V-C1006, issued as ISO/IEC PDAM) - Collect the conformance test streams according to JCT3V-C1006 - Prepare an initial draft document for 3D AVC conformance testing. | T. Suzuki, D. Rusanovskyy (co-chairs) | N |
| Parallel Decoding (AHG13) (jct-3v@lists.rwth-aachen.de) - Study reasonable constraints in terms of image resolution in the context of MV-HEVC stereo profile - Study the relationship of view parallel scheme and other parallel schemes (tiles, wavefront proc. etc.) - Study the indication methods of parallel decoding in the context of multiview coding | O. Nakagami, T. Ikai (co-chairs), A. K. Ramasubramonian | N |
| Mixed Resolution Coding (AHG14) (jct-3v@lists.rwth-aachen.de) - Investigate possible benefits of mixed resolution coding in 3D-HEVC. - Identify commonality of reduced resolution depth coding in MVC plus depth, 3D-ATM and 3D-HTM. - Study approaches and design implications to support coding of mixed resolution data (texture and depth), including asymmetric texture coding. - Study implications of mixed resolution coding with regard to complexity and memory requirements of 3D-ATM and 3D-HTM. | K. Wegner, S. Shimizu (co-chairs) | N |

# Output documents (update)

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate those responsible for document production.

(it was communicated by the chair that only the subsequent lists of adoptions is relevant, and in case of contradiction with any of meeting notes above the latter are considered invalid in first place)

JCT3V-C1000 Meeting Report of 3rd JCT-3V Meeting [J.-R. Ohm, G. J. Sullivan]

(Note: Initial versions of the subsequent draft documents should be uploaded by the end of the meeting, continually updating to be performed until the final the version is released)

JCT3V-C1001 MVC Extension for Inclusion of Depth Maps Draft Text 6 [T. Suzuki, M. Hannuksela, Y. Chen, S. Hattori, G. Sullivan] (ISO/IEC 14496-10 FDAM2, WG 11 N13327) [2013-03-22]

Note: It was mentioned by the chair that further work on aligning reference software and conformance for this amendment will be necessary.

Summary of Adoptions for MVC+D

- ...

JCT3V-C1002 3D-AVC Draft Text 5 [M. Hannuksela, Y. Chen, T. Suzuki] (ISO/IEC 14496-10 Study of PDAM3, WG 11 N13332) [2013-02-06]

Note: It was mentioned by the chair that work on conformance for this new amendment will be necessary, and it should be started ahead of the final approval.

JCT3V-C1003 3D-AVC Test Model 5 [D. Rusanovskyy, F. C. Chen, L. Zhang, T. Suzuki] (WG 11 N13342) [2013-03-22]

Summary of Adoptions for 3D-AVC (relating to C1002 and C1003)

- ...

JCT3V-C1004 MV-HEVC Draft Text 3 [G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce] (ISO/IEC 23008-2 PDAM2, WG 11 N13347) [2013-03-22]

Summary of Adoptions for MV-HEVC

- ...

JCT3V-C1005 3D-HEVC Test Model 3 [G. Tech, K. Wegner, Y. Chen, S. Yea] (WG 11 N13348) [2013-03-22]

Summary of Adoptions for 3D-HEVC

- ...

Summary of SW only adoptions and bug fixes:

- ...

JCT3V-C1006 Draft 2 of MVC plus Depth Conformance [T. Suzuki, D. Rusanovskyy] (ISO/IEC 14496-4 PDAM41, WG 11 N13324) [2013-02-01]

JCT3V-C1007 Draft 2 of Multi-resolution Frame Compatible Stereo (ISO/IEC 14496-10:2012/PDAM5,WG 11 N13331) [2013-02-01]

JCT3V-C1100 Common Test Conditions of 3DV Core Experiments [D. Rusanovskyy, K. Müller, A. Vetro]

JCT3V-C1101 Description of Core Experiment 1 (CE1) on View Synthesis Prediction [S. Shimizu (CE Coordinator)]

JCT3V-C1102 Description of Core Experiment 2 (CE2) on Disparity Vector Derivation [Y. Chen, S. Yea]

JCT3V-C1103 Description of Core Experiment 3 (CE3) on Inter-Component Prediction [J. Jung, S. Kamp]

JCT3V-C1104 Description of Core Experiment 4 (CE4) on Residual Prediction [L. Zhang]

JCT3V-C1105 Description of Core Experiment 5 (CE5) on Inter-view/Motion Prediction [Y. Chang, S. Yea]

JCT3V-C1106 Description of Core Experiment 6 (CE6) on Depth Intra Coding [P. Merkle]

JCT3V-C1107 Description of Core Experiment 7 (CE7) on Skip Mode Coding [M. Mishurovskiy, D. Rusanovskyy]

# Future meeting plans, expressions of thanks, and closing of the meeting

The document upload deadline for the 4th meeting of the JCT-3V will be April 13, 2013, 2359 MET (Geneva/Paris time zone).

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Wednesday or Thursday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Saturday prior to such meetings and closing it on the last day of the WG 11 meeting).

Some specific future meeting plans were established as follows:

- 20–26 April 2013 under WG 11 auspices in Incheon, KR.

- 27 July–02 August 2013 under WG 11 auspices in Vienna, AT.

- 26Oct -1 Nov. 2013 under ITU-T auspices in Geneva, CH.

- 11–17 Jan. 2014 under WG 11 auspices in San Jose, US.

- 29 March–04 April 2014 under WG 11 auspices in Valencia, ES.

ITU was thanked for its excellent hosting of the 3rd meeting of the JCT-3V. EBU was thanked for providing the 3D viewing equipment used at the 3rd JCT-3V meeting.

3D video experts were reminded about a joint Meeting in MPEG (video and 3DG subgroups) on possible relationships between 3D video and augmented reality Thu 24th 9:00-10:00 (3DG room 15 in CICG).

It was also reminded that final output documents (if also registered under a WG11 output doc number) have to be uploaded separately with a WG11 header.  To do this in a timely fashion is particularly important for standards submitted for ballot.

The JCT-3V meeting was closed at approximately 1200 hours on Wednesday 23 Jan 2013.

# Annex A to JCT-3V report:
List of documents

# Annex B to JCT-3V report:
List of meeting participants

The participants of the fourth meeting of the JCT-3V, according to a sign-in sheet passed around during the meeting (approximately 200 in total), were as follows: