| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-Exxxx |
| --- | --- |

| Title: | CE4.h: The results on illumination compensation using offset model |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiwook Jung Hongbin Liu Jie Jia Sehoon Yea | Tel: Email: | +82-2-6912-6477 jiwook.jung@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

- Abstract

In the 3rd and 4th JCT-3V meeting, an offset model for Illumination Compensation(IC) was proposed to simplify the IC process. The offset model requires no multiplication in the parameter estimation/prediction process while retaining comparable coding efficiency with the linear model. In this proposal, results of the simplified IC using the offset model both for texture and depth coding is presented. It has BD-rate change 0.0% compared to current IC model on video only result with 100.3% encoding time, and 97.4% decoding time.

Introduction

In JCV3V-B0045[1], linear model compensation was applied to correct inter-view luminance / chrominance mismatches. When the IC flag on CU was turned on, each pixel on disparity compensated block were compensated the below equations.

Predx,y=α·Refx,y+ β

Where, Predx,y is compensated prediction samples in a block and Refx,y is prediction samples in a block before compensation is applied.

Parameter α and  β can be derived from reconstructed samples around redx,y , Refx,y. Both encoder and decoder should solve the linear least square equations to estimate α and  β. The below equations presents the linear least square solution. To reduce computing complexity, the division was replaced by multiplication, shift, and look-up-table. But, It has still multiplications of arbitrary sample values for computing cross-correlation of samples.

α= I∙i=1IPredNiRefNi- i=1IPredNi·i=1IRefNiI∙i=1IRefNiRefNi-i=1IRefNi2= A1A2

β= i=1IPredNi- αi=1IRefNiI

Where, PredNx,y , RefNx,y are left and top neighbor pixels by Predx,y , Refx,y, respectively.

Especially, it is more important to simplify IC process as IC process has been applied to chroma, depth samples. [2]

Proposed Method

In the 3rd and 4th JCT-3V meeting, an offset model for Illumination Compensation(IC) was proposed to simplify the IC process. [3], [4]

For the complexity reduction of current IC, Only offset model is applied to compensate luminance/chrominance mismatches. IC can be carried out using only additions and shifts without multiplications. The compensation model is just adding an offset as the below equation.

Predx,y=Refx,y+ β

The parameter β can be calculated by the below equation.

β= i=1IPredNi- i=1IRefNiI

There are some coding loss on the average with respect to reduce the estimation order. However, It has the advantage removing multiplications of arbitrary numbers on IC process.

Moreover, the process of motion estimation is calculated by Mean Removed SAD(MR-SAD). The search method is matched with offset model rather than linear model.

Experimental results

The offset model is integrated into HTM-7.0r1. The configuration of common test condition is applied. Table1 and Table2 reportedly shows the experimental results. The simplification method is applied on texture, depth and texture & depth, respectively.

Table1. results on offset model on CTC configuration

Table2. results on offset model on IBP configuration

Although the complexity is reduced, there is coding gain in some cases, especially Poznan_Hall2 video2.

Complexity Analysis

Table 3. Number of operations of HTM70

Table 4. Number of operations of proposed method.

Multiplication is not used for IC. Especially, multiplication between arbitrary samples is difficult to implement. Also, the number of addition & shift can be reduced by 1/3.

The parameter estimation which is normative process can be calculated within 16 bit operation while the conventional linear model use 32 bit operation. Because the correlation between samples increase the operation range as well as the operation complexity.

Conclusion

In this contribution, a method to use simple offset model is proposed to simplify IC process. The current method for IC is relatively complex because the process contains multiplications of arbitrary samples. In this proposal, the process to calculate least square solution is replaced with the calculation of the difference of mean values. The proposed method has the benefits of multiplication-free and matching with RD-metric. The result reportedly shows the simplified method has negligible change on the average coding result.

It is recommended that proposed method is adopted into 3D-HEVC.

Reference

- H. Liu, J. Jung, J. Sung, etc., (LG), "3D-CE2.h : Results of Illumination Compensation for Inter-View Prediction", Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-B0045, Shanghai, China, October, 2012.

- H. Liu, J. Jung, J. Sung, etc., (LG), "CE5.h related : Bug Fix and Extension of Illumination Compensation", Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-C0046, Geneva, Switzerland, January, 2013.

- J. Jung, H. Liu, etc., (LG), "Non-CE Simplification of Illumination Compensation", Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-C0111, Geneva, Switzerland, January, 2013

- J. Jung, H. Liu, etc., (LG), " CE5.h The results on simplified illumination compensation", Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-D0096, Incheon, Korea, April, 2013

Patent rights declaration(s)

LG electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).