| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0059 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0036 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On sharing parameter sets across layers |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Tomoyuki Yamamoto Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 tsukuba.takeshi@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

- Abstract

This contribution presents two proposals regarding sharing parameter sets:

- Restriction that sharing parameter sets (SPS/PPS) shall be used in the condition the parameter sets is direct or indirect reference layer of the layer.

- New direct_dependency_type to show sharing parameter set dependency in VPS.

It is asserted that the proposed changes can benefit on the following issues;

- Avoid the improper removal of sharing parameter set in bitstream extraction.

- Enable parameter sharing in case of pixel, motion or any other dependency is not utilized.

Introduction

Current high level syntax design allows multiple layers to refer to the same SPS or PPS. Omitting representation data in SPS by signalling representation format in VPS also enhances SPS sharing capability. However, there exists an issue regarding sharing parameter sets:

- In sub bitstrem extraction, when NAL unit with the target nuh_layer_id or NAL unit with nuh_layer_id of direct or indirect reference of the the target layer, sharing SPS could be removed. It is because SPS can also be shared in case of no dependency between the layers of shared SPS and the referring layer.

This situation can be illustrated in left part of Figure 1.

  - Layer0 is direct reference layer of Layer2.

  - Parameter sets of Layer1 is shared among Layer1 and Layer2,  but Layer 1 is not direct reference layer of Layer2.

  - When extracting Layer2, parameter sets of Layer1 could be discarded.

Figure 1: Example case that the coded pictures with nuh_layer_id equal to LayerId2 are un-decodable since shared parameter sets with nuh_layer_id equal to LayerId1 are discarded through bitstream extraction process.

This issue can be solved by Proposal 1 (adding a restriction that sharing parameter sets (SPS/PPS) shall be used in the condition the parameter sets is direct or indirect reference layer of the layer).

With the Proposal 1 change, it is not allowed to share the parameter sets if there are no direct dependency in sample dependency or motion dependency. Therefore, we additionally suggest to use Proposal 2, that is, to add new direct_dependency_type to show sharing parameter set dependency in VPS.

Proposal

Proposal 1

The proposal adds the following restriction.

- F.7.4.2.4 Order of VPS, SPS and PPS RBSPs and their activation

It is a requirement of bitstream conformance that, when an SPS with nuh_layer_id equal to nuhLayerIdA is active for a layer with nuh_layer_id equal to nuhLayerIdB, the layer with nuh_layer_id equal to nuhLayerIdA shall be a direct or indirect reference layer of the layer with nuh_layer_id equal to nuhLayerIdB.

It is a requirement of bitstream conformance that, when a PPS with nuh_layer_id equal to nuhLayerIdA is active for a layer with nuh_layer_id equal to nuhLayerIdB, the layer with nuh_layer_id equal to nuhLayerIdA shall be a direct or indirect reference layer of the layer with nuh_layer_id equal to nuhLayerIdB.

Proposal 2

It is proposed to add new direct_dependent_type to indicate Non-VCL dependency which is mainly (currently only) used for parameter set dependency, including some editorial improvements.

If applies proposal2, proposal1 is modified as follows:

It is a requirement of bitstream conformance that, when an SPS with nuh_layer_id equal to nuhLayerIdA is active for a layer with nuh_layer_id equal to nuhLayerIdB, the layer with nuh_layer_id equal to nuhLayerIdA shall be a direct reference layer of the layer with nuh_layer_id equal to nuhLayerIdB and the direct dependency type between the two layers indicates non-VCL dependency.

It is a requirement of bitstream conformance that, when a PPS with nuh_layer_id equal to nuhLayerIdA is active for a layer with nuh_layer_id equal to nuhLayerIdB, the layer with nuh_layer_id equal to nuhLayerIdA shall be a direct reference layer of the layer with nuh_layer_id equal to nuhLayerIdB and the direct dependency type between the two layers indicates non-VCL dependency type.

Changes are highlighted in yellow, and both of removals and typos are stroke through in red.

F.7.4.3.1.1 Video parameter set extension semantics

direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. direct_dependency_flag[ i ][ j ] equal to 1 specifies that the layer with index j may be a direct reference layer for the layer with index i. When direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, it is inferred to be equal to 0.

…snipped…

direct_dependency_type[ i ][ j ] indicates the type of dependency between the layer with nuh_layer_id equal layer_id_in_nuh[ i ] and the layer with nuh_layer_id equal to layer_id_in_nuh[ j ].

direct_dependency_type[ i ][ j ] equal to 0 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for inter-layer sample prediction but not for inter-layer motion prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. 
direct_dependency_type[ i ][ j ] equal to 1 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for inter-layer motion prediction but not for inter-layer sample prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. 
direct_dependency_type[ i ][ j ] equal to 2 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for both inter-layer sample motion prediction and inter-layer motion prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ].
((direct_dependency_type[ i ][ j ] + 1) & 1) equal to 1 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for inter-layer sample prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. 
(((direct_dependency_type[ i ][ j ] + 1) & 2) >> 1) equal to 1 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for inter-layer motion prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ].
(((direct_dependency_type[ i ][ j ] + 1) & 4) >> 2) equal to 1 indicates that any parameter sets having nuh_layer_id with layer_id_in_nuh[ j ] for the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] may be used as the parameter sets for the layer with nuh_layer_id equal to layer_id_in_nuh[ i ].

Conclusion

This contribution proposes two proposals.

- Restrict sharing parameter sets (SPS/PPS) for direct or indirect reference layer.

- Add new direct_dependency_type to show sharing parameter set dependency.

We think the first proposal is necessary for efficient bitstream extraction, and we also think the new direct dependency type is useful to accomplish sharing parameter set in case that any other dependency is not applied. Therefore, it is recommended to adopt both of proposals to the next SHVC and MV-HEVC.

Reference

- J. Chen, et.al, “SHVC Draft 3”, JCTVC-N1008, Vienna, AT, 25 July–2 Aug. 2013.

- G. Tech, et.al, “MV-HEVC Draft Text 5”, JCT3V-E1004, Vienna, AT, 27 July–2 Aug. 2013.

Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).