| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 |  |  |  | Document: JCT3V-I_Notes_d1 |
| --- | --- | --- | --- | --- |
| Title: | Meeting report of the ninth meeting of the Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V), Sapporo, JP, 3 – 9 July 2014 |  |  |  |
| Status: | Report document from chairs of JCT-3V |  |  |  |
| Purpose: | Report |  |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communications Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |  |
| Source: | Chairs |  |  |  |

_____________________________

- Summary

[Ed. Check/fix style setting in Heading 9 style for font size – should be 12.]

The Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its ninth meeting during 3 – 9 July 2014 at the Sapporo Convention Center, Sapporo, JP. The JCT-3V meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany) and Dr Gary Sullivan (Microsoft/USA). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.13 of this document.

The meeting was mainly held in a “single track” fashion, with few breakout activities (as documented in this report) held in parallel. Several plenary sessions were chaired by XXX, particularly in cases when neither of the two chairs was able to attend due to commitments in other groups or joint meetings. It was usually avoided to make decisions which might cause objections in such cases.

The JCT-3V meeting sessions began at approximately 900 hours on Thursday 3 July 2014. Meeting sessions were held on all days until the meeting was closed at approximately XXXX hours on Wednesday 9 July 2014. Approximately XXX people attended the JCT-3V meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JCT-3V. The subject matter of the JCT-3V meeting activities consisted of work on 3D extensions of the Advanced Video Coding (AVC) and the High Efficiency Video Coding (HEVC) standards.

The AHG on 3D High-level Syntax (AHG7) had met the 2 days before the JCT-3V meeting (1-2 July), to discuss HEVC HLS input contributions jointly with JCT-VC experts. The outcome of this joint meeting, which continued after the beginning of the JCT-3V meeting, is documented in the JCT-VC report JCTVC-R1000.

The primary goals of the meeting were to review the work that was performed in the interim period since the seventh JCT-3V meeting in producing

- Draft 3 of 3D-AVC reference software (ISO/IEC 14496-5:2001/DAM 35 – for ballot)

- Draft 4 of MVC+D reference software (bug fixing before progressing to FDAM – not for ballot)

- Draft Text 8 of MV-HEVC (Preliminary Draft of ISO/IEC 23008-2:200X/FDAM 2 – not for ballot)

- Test Model 8 of 3D-HEVC and  MV-HEVC, and associated software

- Draft 4 of 3D-HEVC text specification (ISO/IEC 23008-2:2013/PDAM4 Study – not for ballot)

- Draft 5 of 3D-AVC Conformance (ISO/IEC 14496-4:2004/DAM 43 – for ballot)

- Common Test Conditions of 3DV Core Experiments and CE Descriptions.

Furthermore, the JCT-3V reviewed the results from two interim Core Experiments (CE); reviewed technical input documents; produced updated versions of the draft texts, framework descriptions and software implementations of the items above; and planned a new set of Core Experiments (CEs) for further investigation of proposed technology.

The JCT-3V produced XX particularly important output documents from the meeting (update):

- Draft 4 of 3D-AVC reference software (ISO/IEC 14496-5:2001/DAM 35 Study – not for ballot)

- Draft Text 9 of MV-HEVC (Text of ISO/IEC 23008-2:2013/FDAM 2 – for ballot) (integrate new edition?)

- Test Model 9 of 3D-HEVC and  MV-HEVC, and associated software

- Draft 5 of 3D-HEVC text specification (Preliminary Text of ISO/IEC 23008-2:2013/DAM 4 – not for ballot)

- Draft 6 of 3D-AVC Conformance (Text of ISO/IEC 14496-4:2004/DAM 43 Study – not for ballot)

- Draft 5 of MVC+D Reference Software (ISO/IEC 14496-5:2001/FDAM XX  – for ballot)

- Common Test Conditions of 3DV Core Experiments.

Moreover, plans were established to conduct XX future CEs in the interim period until the next meeting.

For the organization and planning of its future work, the JCT-3V established XX "Ad Hoc Groups" (AHGs) to progress the work on particular subject areas. The next four JCT-3V meetings are planned for 18 – 24 Oct. 2014 under WG 11 auspices in Strasbourg, FR, during 12 – 18 Feb. 2015 under ITU-T auspices in Geneva, CH, during 20 – 26 June 2015 under WG 11 auspices in Warsaw, PL, and during XX – XX Oct. 2015 under ITU-T auspices in XXXX, XX.

The document distribution site http://phenix.it-sudparis.eu/jct3v/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-3V and all of its AHGs is jct-3v@lists.rwth-aachen.de.

- Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-3V are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its ninth meeting during 3 – 9 July 2014 at the Sapporo Convention Center, Sapporo, JP. The JCT-3V meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany) and Dr Gary Sullivan (Microsoft/USA). A couple of sessions were also chaired by XXX (XXX/XXX).

## Meeting logistics

The JCT-3V meeting sessions began at approximately 900 hours on Thursday 3 July 2014. Meeting sessions were held on all days until the meeting was closed at approximately XXXX hours on Wednesday 9 July 2014. Approximately XXX people attended the JCT-3V meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JCT-3V. The subject matter of the JCT-3V meeting activities consisted of work on 3D extensions of the Advanced Video Coding (AVC) and the High Efficiency Video Coding (HEVC) standards.

The AHG on 3D High-level Syntax (AHG6) and XXX (AHG10) had met the 2 days before the JCT-3V meeting (1-2 July), to discuss HEVC HLS input contributions jointly with JCT-VC experts.

Information regarding preparation and logistics arrangements for the meeting had been provided via the email reflector jct-3v@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jct3v-site/2014_07_I_Sapporo/.

## Primary goals

The primary goals of the meeting were to review the work that had been performed in the interim period since the seventh JCT-3V meeting in producing

- Draft 3 of 3D-AVC reference software (ISO/IEC 14496-5:2001/DAM 35 – for ballot)

- Draft 4 of MVC+D reference software (bug fixing before progressing to FDAM – not for ballot)

- Draft Text 8 of MV-HEVC (Preliminary Draft of ISO/IEC 23008-2:200X/FDAM 2 – not for ballot)

- Test Model 8 of 3D-HEVC and  MV-HEVC, and associated software

- Draft 4 of 3D-HEVC text specification (ISO/IEC 23008-2:2013/PDAM4 Study – not for ballot)

- Draft 5 of 3D-AVC Conformance (ISO/IEC 14496-4:2004/DAM 43 – for ballot, not delivered yet)

- Common Test Conditions of 3DV Core Experiments and CE Descriptions.

Furthermore, the JCT-3V reviewed the results from the interim Core Experiments (CE); reviewed technical input documents; produced updated versions of the aforementioned draft texts, framework descriptions, conformance and software implementations of the items above; and planned a new set of Core Experiments (CEs) for further investigation of proposed technology.

## Documents and document handling considerations

### General

The documents of the JCT-3V meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct3v/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

Document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (rather than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a bug in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding … simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of contexts, number of context-coded bins, etc.) … "Decision (Compl.):"

This meeting report is based primarily on notes taken by the chairs and projected (if possible) for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp (http://wftp3.itu.int/av-arch/jct3v-site/) during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much discussion of the contributions and discussions as is feasible in the interest of aiding study, although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Thursday, 26 June 2014. For documents on HEVC high level syntax, if submitted as joint contributions to JCT-VC and JCT-3V, the deadline of Friday, 20 June 2014 had been announced.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Friday 27 June 2014 were considered "officially late". For HLS documents, a similar consideration applies with the date of 21 June, 2359 hours.

Most documents in the "late" category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All regular contribution documents with registration numbers JCT3V-I0150 and higher, as well as common SHVC/MV-HEVC HLS documents with registration numbers JCT3V-I0059 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). Some documents in the "H0150+" range include break-out activity reports that were generated during the meeting and are therefore considered report documents rather than late contributions.

Generally, the late document submission situation had improved relative to previous history.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with this issue was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical proposal contributions were classified as late either due to late upload or late registration:

- JCT3V-I0XXX (a technical proposal on XXX) [uploaded 06-XX]

- …

The following other documents not proposing normative technical content were classified as late (this did not apply in the current meeting):

- JCT3V-I0XXX (an input on …) [uploaded XX-XX]

The following cross-verification reports were classified as late either due to late upload or late registration: JCT3V-I0XXX, ….

The following document registrations were later cancelled or otherwise never provided or never discussed due to lack of availability or registration errors: JCT3V-I0XXX.

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were rejected in the document management system, as has been agreed since the third meeting.

(the following cases did not happen in the current meeting)

The initial uploads of the following contribution documents were rejected as "placeholders" without any significant content and were not corrected until after the upload deadline:

- JCT3V-I0XXX (a contribution of ... , corrected ...)

- ...

A few documents had some problems relating to IPR declarations (missing or excess declarations of contributing companies), inconsistent filenames in the header etc. in the initial uploaded versions. These issues were corrected by later uploaded versions in all cases (to the extent of the awareness of the chairs). In case of JCTVC-I0XXX and ..., the first complete versions appeared only on 06-XX, such that this contribution was also flagged as “late”.

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Measures to facilitate the consideration of contributions

For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by the cross-check contributor.

### Outputs of the preceding meeting

The report documents of the previous meeting, particularly the meeting report (JCT3V-H1000), the 3D-HEVC draft text 4 (JCT3V-H1001), the multi-view MV-HEVC draft text 8 (JCT3V-H1002), the MV-/3D-HEVC test model 8 (JCT3V-H1003), Draft 5 of 3D-AVC Conformance (JCT3V-H1004), Draft 3 of 3D-AVC Reference Software (JCT3V-H1005), and Draft 4 of MVC+D Reference Software (JCT3V-H1006), which had been produced in the interim period, were approved. The ATM and HTM reference software packages produced by AHG3 and AHG4 on software development, and the software technical evaluations were also approved.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

## Attendance

The list of participants in the JCT-3V meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/ SC 29/ WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on 3D video coding projects guidance

- Consideration of 3D video coding technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-3V and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-3V as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jct3v-site (JCT-3V contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-3v/index.html (JCT-3V general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/ SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that it is our understanding according to the practices of the parent bodies to make reference software available under copyright license header language which is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as e.g. recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/ SC 29/ WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software header is currently used in the process of designing the new HEVC standard and for evaluating proposals for technology to be included in this design. Additionally, after development of the coding technology, the software will be published by ITU-T and ISO/IEC as an example implementation of the 3D video standard(s) and for use as the basis of products to promote adoption of the technology. This is likely to require further communication with and between the parent organizations.

The ATM, HTM and MFC software packages that are used in JCT-3V follow these principles. The view synthesis software used for non-normative post processing is included in the HTM package and also has the BSD header.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct3v/. Furthermore, the site http://ftp3.itu.int/av-arch/jct3v-site was used for distribution of the contribution document template and circulation of drafts of this meeting report.

Communication of JCT-3V is performed via the list jct-3v@lists.rwth-aachen.de (to subscribe or unsubscribe, go to http://mailman.rwth-aachen.de/mailman/listinfo/jct-3v).

It was emphasized that reflector subscriptions and email sent to the reflector must use their real names when subscribing and sending messages and must respond to inquiries regarding their type of interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Note: Acronyms should be used consistently. For example, “IV” is sometimes used for “inter-view” and sometimes for “intra-view”.

Some terminology used in this report is explained below:

- AHG: Ad hoc group.

- AMVP: Advanced motion vector prediction.

- ARP: Advanced residual prediction.

- ATM: AVC based 3D test model

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BoG: Break-out group.

- BR: Bit rate.

- B-VSP: Backward view synthesis prediction.

- CABAC: Context-adaptive binary arithmetic coding.

- CD: Committee draft – the first formal ballot stage of the approval process in ISO/IEC.

- CE: Core experiment – a coordinated experiment conducted between two subsequent JCT-3V meetings and approved to be considered a CE by the group.

- Consent: A step taken in ITU-T to formally consider a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CPB: Coded picture buffer.

- CTC: Common test conditions.

- DBBP: Depth based block partitioning.

- DC: Disparity compensation

- DDD: Disparity derived depth (which uses the motion disparity vector to reconstruct a certain block (PU) of the depth map)

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC.

- DF: Deblocking filter.

- DLT: Depth lookup table.

- DMC: Depth based motion competition.

- DMM: Depth modeling mode.

- DPB: Decoded picture buffer.

- DRPS: Depth-range parameter set.

- DRWP: Depth-range based weighted prediction.

- DT: Decoding time.

- DV: Disparity vector

- ET: Encoding time.

- HEVC: High Efficiency Video Coding – the video coding standardization initiative under way in the JCT-VC.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- HRD: Hypothetical reference decoder.

- HTM: HEVC based 3D test model

- IC: Illumination compensation

- IDV: Implicit disparity vector

- IVMP: Inside-view motion prediction (which means motion for depth component is inherited from texture component motion)

- IVRC: Inter-view residual prediction.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/ SC 29, one of the two parent bodies of the JCT-3V).

- MPI: Motion parameter inheritance.

- MV: Motion vector.

- NAL: Network abstraction layer (HEVC/AVC).

- NBDV: Neighbored block disparity vector (used to derive unavailable depth data from reference view’s depth map) and DoNBDV = depth oriented NBDV

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NUT: NAL unit type (HEVC/AVC).

- PDM: Predicted Depth Map

- POC: Picture order count.

- PPS: Picture parameter set (HEVC/AVC).

- QP: Quantization parameter (as in AVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD). Often loosely associated with HE.

- RAP: Random access picture.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- REXT: Range extensions (of HEVC).

- RPS: Reference picture set.

- RQT: Residual quadtree.

- SAO: Sample-adaptive offset.

- SEI: Supplemental enhancement information (as in AVC).

- SD: Slice data; alternatively, standard-definition.

- SDC: Segment-wise DC coding.

- SH: Slice header.

- SHVC: Scalable HEVC.

- SPS: Sequence parameter set (HEVC/AVC).

- TSA: Temporal sublayer access.

- Unit types: Needs change

  - CTB: code tree block (synonymous with LCU or TB).

  - CU: coding unit.

  - LCU: (formerly LCTU) largest coding unit (synonymous with CTB or TB).

  - PU: prediction unit, with four shape possibilities.

    - 2Nx2N: having the full width and height of the CU.

    - 2NxN: having two areas that each have the full width and half the height of the CU.

    - Nx2N: having two areas that each have half the width and the full height of the CU.

    - NxN: having four areas that each have half the width and half the height of the CU.

  - TB: tree block (synonymous with LCU – LCU seems preferred).

  - TU: transform unit.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-3V).

- VPS: Video parameter set.

- VS: View synthesis.

- VSO: View synthesis optimization (RDO tool for depth maps).

- VSP: View synthesis prediction.

- WD: Working draft – the draft HEVC standard corresponding to the HM.

- WG: Working group (usually used in reference to WG 11, a.k.a. MPEG).

## Liaison activity

The JCT-3V did not send or receive formal liaison communications at this meeting.

## Opening remarks

Software and conformance for MVC+D, 3D-AVC and MFC should be progressed to a new edition in the ITU version of the spec.

Karsten Sühring coordinates the software package, Teruhiko Suzuki the conformance.

Documents to be approved by SG16 need to be delivered 07-09 evening at latest

New edition HEVC including MV-HEVC

Planning Conformance/software for MV-HEVC

Planning verification tests for MV-HEVC

PDAM ballot for 3D-HEVC closed, whereas DAM is planned for Oct. – need to consider ballot comments – Gerhard will report on Friday

Software development for 3D AVC: DAM delivered, but Ssoftware stll needs bug fixing, conformance

New amendment for AVC3D-AVC Conformance not yet delivered (w14383, JCT3V-H1004, ISO/IEC 14496-4:2004/DAM43)

MFC conformance (ISO/IEC 14496-4:2004/DAM 42) Summary of Voting in m33645 – no negative votes – only comment was from U.S. with a Yes vote – encouraging unspecified general improvement if possible. We can proceed directly to publication (issuing a DoCR to reply to the comments, saying that no needs for action were identified). (Y. Peng prepares DoC)

MFC software and conformance: Issue DoC (P. Yin prepares), resolution to proceed with publication

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows.

- AHG reports (section ) (10)

- Project development and status (section 3) (7)

- CE1 and related : Residual prediction (section 4.1) (29)

- CE2 and related: Depth intra coding (section 4.3) (17)

- 3DV standards development (incl. software, conformance) (section 5) (6)

- High-Level Syntax (section 6) (42)

- Non-CE technical contributions (section 7) (60)

- Non-normative Contributions (section 8) (0)

- Withdrawn, unclear allocation (section 9) (3)

NOTE – The number of contributions noted in each category, as shown in parenthesis above, may not be 100% precise.

## Scheduling planning

Scheduling: Generally meeting time was scheduled during 0900 – 2000, with coffee and lunch breaks as convenient. Ongoing refinements were announced on the group email reflector as needed.

Some particular scheduling notes are shown below, although not necessarily 100% accurate:

- Thursday, first day

  - 0900-1100: Opening and AHG report review

  - 1130-13:00 and 1400-XX CE review

  - Joint BoG on HLS in parallel

- Friday, second day

  - 900 New AVC amendment

  - DoCs (MV-HEVC, MFC SW&Conf), conformance MV-HEVC

  - Continue reviewing technical inputs MV-HEVC

  - …

- Saturday, third day

  - ..

- Sunday, fourth day

  - 900 Joint meeting on HLS for MV-HEVC/SHVC

  - ..

- Monday, fifth day

  - Morning: Breakouts etc.

  - …

- Tuesday, sixth day

  - …

  - 1400-1800 FTV workshop

- Wednesday, seventh day

  - 1000-1400 final plenary

# AHG reports (10)

The activities of ad hoc groups that had been established at the prior meeting are discussed in this section.

JCT3V-I0001 JCT-3V AHG Report: JCT-3V project management (AHG1) [J.-R. Ohm, G. J. Sullivan]

JCT3V-I0002 JCT-3V AHG Report: MV-HEVC / 3D-HEVC Draft and Test Model editing (AHG2) [G. Tech, K. Wegner, J. Boyce, Y. Chen, T. Suzuki, S. Yea, J.-R. Ohm, G. Sullivan] [miss]

The fourth 3D-HEVC draft, the eighth Test Model of 3D-HEVC and MV-HEVC and the eighth MV-HEVC draft were developed from the third 3D-HEVC draft, the seventh 3D-HEVC Test Model and the seventh MV-HEVC draft, respectively, following the decisions taken at the 8th JCT-3V meeting in Valencia.

Three editorial teams were formed to work on the three documents that were to be produced:

- H1001 3D-HEVC Draft Text 4

- Gerhard Tech, Ying Chen, Krzysztof Wegner, Sehoon Yea

- H1002 MV-HEVC Draft Text 8

- Gerhard Tech, Miska Hannuksela, Ying Chen, Jill Boyce, Krzysztof Wegner

- Ye-Kui Wang, Adarsh Ramasubramonian

- H1003 3D-HEVC Test Model 8

- Ying Chen, Gerhard Tech, Krzysztof Wegner, Sehoon Yea

- Two versions of H1001 have been published by the editing AHG following the 8th JCT-3V meeting in Valencia, the last version has been submitted as study text to the MPEG secretariat. The 3D-HEVC draft text has been updated to be aligned with MV-HEVC draft 8.

- Open issues:

- A list of other minor issues is listed in the bug tracking system.

- Subsequent to the Valencia meeting, proponents of H0094 and H0205 provided revised specification text. However, additional changes have neither mentioned in the original input proposals nor in the meetings notes such that they have not been incorporated into the draft. Further clarification might be required since the changes have been integrated to the software.

One version of H1003 has been published by the editing AHG following the 8th JCT-3V meeting in Valencia. The last version corresponds to the text submitted to MPEG secretariat.

Five versions of H1002 were published by the AHG following the 8th JCT-3V meeting in Valencia. The last version corresponds to the text submitted to MPEG secretariat.

Open issues in H1002:

- Some text bugs reported in the bug tracking system or indicated by editor's comments.

The recommendations of the MV-HEVC / 3D-HEVC Draft and Test Model editing AHG are to:

- Approve the edited documents H1001, H1002 and H1003 as JCT-3V outputs.

- Continue to edit documents to ensure that all agreed elements of 3D- and MV-HEVC are fully described.

- Compare the documents with the HTM-software and resolve any discrepancies that may exist, in collaboration with the Software AHG.

- Continue to improve the overall editorial quality of the documents.

- Ensure that properly drafted candidate text is available prior to making any decision to change the specifications.

- Discuss reported open issues.

- The three documents were approved as requested.

JCT3V-I0003 JCT-3V AHG Report: 3D-AVC Software Integration (AHG3) [D. Rusanovskyy, F. C. Chen, J. Y. Lee, J.-L. Lin, O. Stankiewicz, T. Suzuki, D. Tian] [miss]

No report available during opening plenary.

TBR

JCT3V-I0004 JCT-3V AHG Report: MV-HEVC and 3D-HEVC Software Integration (AHG4) [G. Tech, H. Liu, Y.W. Chen, K. Wegner] [miss]

Development of the software was coordinated with the parties needing to integrate changes.

The distribution of the software was announced on the JCT-3V e-mail reflector and the software was made available through the SVN server:

https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/

Anchor bitstreams have been created and uploaded to:

ftp.hhi.fraunhofer.de; login: mpeg3dv_guest; path: /MPEG-3DV/HTM-Anchors/

One version of the HTM software were produced and announced on the JCT-3V email reflector. The following sections give a brief summary of the integrated tools and achieved coding gains.

Starting point for development of HTM-11.0 was HTM-10.2. Development of HTM-11.0 was conducted in three parallel tracks each performing sequential integration. Development of each branch has been supervised by one software coordinator. Software of all three tracks was merged by the software coordinators.

MV-HEVC: The coding results for MV-HEVC are identical to results obtained from version HTM-10.2.

3D-HEVC: HTM-11.0 vs. HTM-10.2 (CTC, three view configuration)

|  | video video rate | video total rate | synth total rate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0,1% | -0,1% | -0,1% | 104,7% | 130,5% | 113,9% |
| Kendo | -0,1% | 0,1% | 0,1% | 103,4% | 129,9% | 113,7% |
| Newspaper_CC | 0,0% | 0,1% | 0,1% | 104,0% | 108,4% | 113,8% |
| GT_Fly | 0,0% | -0,1% | 0,0% | 100,2% | 105,1% | 100,7% |
| Poznan_Hall2 | -0,2% | -0,2% | -0,3% | 102,4% | 106,5% | 101,5% |
| Poznan_Street | -0,1% | -0,1% | -0,1% | 99,6% | 100,3% | 103,9% |
| Undo_Dancer | 0,0% | -0,1% | 0,0% | 100,2% | 115,0% | 100,6% |
| Shark | 0,0% | -0,1% | -0,1% | 100,2% | 104,0% | 101,2% |
| 1024x768 | -0,1% | 0,0% | 0,1% | 104,1% | 122,9% | 113,8% |
| 1920x1088 | -0,1% | -0,1% | -0,1% | 100,5% | 106,2% | 101,6% |
| average | -0,1% | -0,1% | 0,0% | 101,9% | 112,5% | 106,1% |

Note: Computation time numbers are likely unreliable, since no changes were applied to the renderer.

HTM-11.1 has not been released yet. A preliminary version that will be cross checked and issued during the meeting can be found at:

https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/branches/HTM-11.0-dev0

In this version MV-HEVC 8 HLS has been integrated in a single track. Moreover the software basis was updated to HM-14.0.

Open issues:

- Some minor mismatches related to 3D-HEVC HLS.

- Most of MV-and 3D-HEVC SEI messages, auxiliary pictures, and hybrid scalability are not supported yet.

- Some items related to MV-HEVC HLS (POC alignment, DBP management) are not integrated yet.

- Other minor issues are listed in the bug tracking system.

The recommendations of the 3D-HEVC Software integration group are:

- Develop reference software HTM version 12 based on HTM-11 and improve its quality.

- When a proposal is adopted to discuss how to enable it in the HTM software (e.g. encoder parameter / parameter set flag, or always on).

- Continue to identify bugs and discrepancies with text, and address them.

- Fix open issues.

- Discuss on how to address open issues.

Comments: The bug tracking system reports an encoder crash. This may however be due to compilation with 32 bit compiler version, where the memory allocation is insufficient.

JCT3V-I0005 JCT-3V AHG Report: 3D Coding Tool Testing (AHG5) [K. Müller (Fraunhofer HHI), A. Vetro (MERL), S. Yea (LG Electronics)]

Mandate 1: 	Discuss and finalize CE work plans.

The CE work plans were finalized and made available via the JCT-3V web site by April 4, 2014.

Mandate 2: 	Study the Common Test Conditions and suggest possible changes.

Possible changes to the Common Test Conditions from the CEs should be discussed further during the meeting.

Mandate 3: 	Coordinate between core experiments when necessary.

For the CEs, the 3DV-HTM software version 11.0 was used, as announced over the reflector by April 25, 2014.

Other than this, there were no other needs during this AHG period to coordinate between any of the core experiments.

Mandate 4: 	Study coding tool performance of non-CTC configurations.

The two CEs used the CTC configuration. 
In addition, CE2 used the all-intra configuration.

Furthermore, two AHG5-related input contributions, JCT3V-I0115 on Performance evaluation for 3D-HEVC and its coding tools and JCT3V-I0116 on Coding results for flexible coding order (FCO) have been received.

Mandate 5: 	Report on status of core experiments.

There were 2 CEs established at the previous meeting to further study various aspects of the HEVC-based coding designs.

To get a sense of where the main areas of development are focused, the approximate number of CE reports and CE related contributions are noted below.

CE2 on Depth Intra Coding is most active with 17 contributions. CE1 on Residual Prediction is less active with 9 contributions.

Mandate 6: 	Prepare viewing logistics for 9th JCT-3V meeting.

The 3D viewing equipment has been prepared and the test room 203 has been setup. Viewing will be performed during the week.

JCT3V-I0006 JCT-3V AHG report: 3D High level syntax (AHG6) [J. Boyce, M. M. Hannuksela (co-chairs), Y. Chen, S. Deshpande, J. Samuelsson, G. Tech, Y.-K. Wang, P. Wu (vice-chairs)]

Not reported in opening plenary

TBR – plan for joint meeting Sunday 900 in “big” JCT-VC plenary room

JCT3V-I0007 JCT-3V AHG Report: Complexity Assessment (AHG7) [G. G. Lee, G. Bang, T. Ikai, K. Rapaka]

7 (complextr)Mandate 1: Study intrinsic complexity measures or metrics to assess hardware and software implementations of algorithms/computational models based on number of operations, potential for parallelism, data transfer rate, and data storage requirements. An informative guideline which introduces complexity assessment without biasing towards either software or hardware has been presented in JCT3V-D0301.

- Discussed, identified and tabulated coding tools or cases to be studied at different suggested data granularities in a spreadsheet which provides a high level overview of the scope of the work in this ad hoc group. Furthermore, several examples performed by co-chairs and proponents in previous JCT3V meetings were also included in this spreadsheet which was uploaded together with AHG 7 report.

Mandate 2: Analyze existing designs and produce complexity perspective per tool, from the viewpoint of possible slowdown, bottlenecks or implementation issues.

- According to Mandate 1, complexity of several coding tools are assessed and they also are discussed the bottleneck per tools, such as depth intra coding tools, inter-prediction tools, etc. The analyzed results also included the attached spreadsheet which is illustrated in Mandate 1.

JCT3V-I0069, AHG7: Complexity Assessment on Depth Intra Modes with Simplified Data Flows.

This document provides an informative case study for the coding tools in 3D-HEVC based on number of operations, data transfer rate, and data storage requirements. With respective to JCT3V-H0127, this proposal provides simplified data flow to express the process of depth intra modes from the perspectives of segment-wise coding. The results show that the efficient data flow could clear designers’ doubts with reasonable expressions.

JCT3V-I0111, AHG7: Complexity assessment of inter-prediction tools.

This contribution reports a complexity assessment of inter prediction tools in terms of number of operation and memory bandwidth. This report has changes from the former report of JCT3V-G0208 in the following: (1) include memory bandwidth analysis instead of data transfer rate analysis, (2) include depth access for memory bandwidth calculation, (3) consider memory pattern for memory bandwidth calculation, (4) add DBBP to the target tools.

Assessed tools:

3D-HEVC

- Inter-prediction tools, 9th meeting, JCT3V-I0111, Sharp

- Depth intra modes and its simplified data flows, 8th meeting, JCT3V-H0127, NCKU and Sharp Error: Reference source not found, 9th meeting, JCT3V-I0069, NCKU

- Illumination compensation (IC), 7th meeting, JCT3V-G0114, NCKU Error: Reference source not found; 7th meeting, JCT3V-G0208, Sharp Error: Reference source not found

- Advanced residual prediction (ARP), 7th meeting, JCT3V-G0208, Sharp Error: Reference source not found

- Simplified depth coding, 5th meeting, JCT3V-E0132, NCKU Error: Reference source not found (It was integrated into the segment-wise DC coding in depth intra modes)

- View synthesis prediction (VSP), 4th meeting, JCT3V-D0089, NCKU Error: Reference source not found; 7th meeting, JCT3V-G0208, Sharp Error: Reference source not found

3D-AVC

- Adaptive luminance compensation (ALP), 4th meeting, JCT3V-D0090, NCKU Error: Reference source not found

- Plane Segmentation based Intra Prediction (PSIP), 4th meeting, JCT3V-D0090, NCKU Error: Reference source not found (It was removed from 3D-AVC)

new contributions (I0069, I0078, I0111)

This AHG on Complexity Assessment recommends to:

- review the input contribution during the meeting

- continue with mandates 1 and 2 and move on to mandate 3

JCT3V-I0008 JCT-3V AHG Report: 3D Test Material (AHG8) [S. Shimizu, S. Yea]

8 (test maealThere was no e-mail exchange over the reflector.

There were some offline activities to prepare the 3D test materials that were offered at the 5th and 6th JCT-3V meetings.

Deutsche Telekom has kindly prepared six stereo clips, 1 Kaunatal and 5 MissInternet, for the evaluation on whether they are useful for our activities. These sequences can be used for MV-HEVC verification and conformance. However, it is necessary to estimate depth maps if we want to use them for 3D-HEVC/3D-AVC developments.

Regarding the KBS sequences, some investigations including de-interlacing and camera parameter estimations are ongoing by LGE.

Action item: View the new stereo sequences during the meeting.

The AHG on 3D Test Material recommends:

- To discuss the current status and potential problems on making the test sequence available.

- To discuss and possibly set up a plan to resolve the issues.

JCT3V-I0009 JCT-3V AHG Report: Conformance testing development (AHG9) [T. Suzuki, D. Rusanovskyy, D. Tian, Y. W. Chen]

In Valencia, problems in the reference software and MVC+D conformance bitstreams were reported. The problems are,

- 3D-AVC HLS were included in MVC+D conformance bitstreams

- Bugs related to IBP configuration

In Valencia, it was decided to include the revised bitstreams of MVC+D conformance bitstreams rather than producing COR to fix the problem.

But after the meeting, it took a time to fix the problems in reference software, and the revision of bitstreams was delayed. Therefore, the output document from Valencia meeting, JCT3V-H1004 and w14383, are not approved yet.

Before Sapporo, all 3D-AVC conformance bitstreams were generated and are available at the following site.

http://wftp3.itu.int/av-arch/jct3v-site/bitstream_exchange/under_test/3D-AVC/revised/

3D-AVC are available,  7 bitstreams which require correction in MVC+D are still missing.

Discussion:

Target was to include the corrected and new MVC+D streams into the 3D-AVC DAM. Some of the corrected MVC+D bitstreams are not delivered yet. It was decided to proceed with issuing the DAM (and expect NB comments in case that further corrections are necessary); for the ITU version, bitstreams need to be delivered Wednesday, and if further updates are anticipated during editing period, this should be described.

JCT3V-I0010 JCT-3V AHG report: Layered coding constraint specifications and capability indications (AHG10) [G. J. Sullivan (coordinator)]

Not reported in opening plenary

TBR – plan for joint meeting Sunday 900 in “big” JCT-VC plenary room

# Project development, status, and guidance (7)

## Communication by parent bodies (0)

## 3D video application areas (0)

## Profile/level definitions (1)

JCT3V-I0134 On the Stereo Main profile [M. M. Hannuksela (Nokia)]

## 3D-HEVC / MV-HEVC performance assessment (2)

JCT3V-I0175 3D-HEVC: Simplification of DMM method [H. Sabirin, K. Kawamura, S. Naito (KDDI)] [late]

This contribution presents the effect of using only one mode of DMM in depth coding. Since the first proposal time, the modes of DMM have been simplified into two modes: DMM1 and DMM4 by the last meeting. It is reported from the experiments that only one DMM mode is possible to maintain the coding performance instead of two modes.

It is proposed to remove DMM4. Results are presented that indicate negligible differences (average 0% but sequence dependent) for CTC, and 0.2% average BR increase in AI

JCT3V-I0115 AHG5: Performance evaluation for 3D-HEVC and its coding tools [S. Yoo, S. Yea (LGE)]

JCT3V-I0116 AHG5: Coding results for flexible coding order (FCO) [J. Nam, S. Yea (LGE)]

JCT3V-I0182 AHG5: Crosscheck on Coding results for flexible coding order (JCT3V-I0116) [S. Shimizu (NTT)] [late] [miss]

## Complexity assessment (4)

JCT3V-I0069 AHG7: Complexity Assessment on Depth Intra Modes with Simplified Data Flows [G. G. Lee, C.-F. Chen, C.-H. Huang (NCKU)]

JCT3V-I0078 Memory Complexity for DBBP and VSP [M. W. Park, J. Y. Lee, Y. Cho, C. Kim (Samsung)]

JCT3V-I0160 Cross-check of Memory Complexity for DBBP and VSP (JCT3V-I0078) [T. Ikai (Sharp)] [late]

JCT3V-I0111 AHG7: Complexity assessment of inter-prediction tools [T. Ikai (Sharp)]

## Quality assessment (0)

## Source video test material (0)

# Core experiments

## CE1: Residual prediction (17)

(chaired by JRO, Thu morning)

### Summary (1)

JCT3V-I0011 CE1 Summary Report: Residual Prediction [H. Liu, T. Ikai]

The following table shows the simulation results and complexity comparisons for CE proposals. In this table, memory bandwidth of depth (in DoNBDV) is not considered since both proposals only focus on residual prediction.

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Memory bandwidth compared to HEVC (1x1) | Memory bandwidth compared to HEVC (4x2) | Memory bandwidth compared to HEVC (4x4) | Memory bandwidth compared to HEVC (8x2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ARP in 3D-HEVC |  | CTC |  | 122% | 135% | 128% | 158% |
| JCT3V-I0051 #1 | 0.00% | -0.04% | -0.01% | 101% | 113% | 106% | 132% |
| JCT3V-I0151 #1 + #2 | 0.30% | 0.31% | 0.08% | 94% | 103% | 96% | 119% |
| JCT3V-I0104 | 0.41% | 0.41% | 0.11% | 105% | 95% | 88% | 108% |

It is noted that for both JCT3V-I0051 #2 and JCT3V-I0104, coding loss of chroma components is much higher than luma component. For example, the coding loss of Cb (Cr) PSNR/video bitrate on video 1 is 0.46% (0.42%) and 0.51% (0.68%) for JCT3V-I0051 #2 and JCT3V-I0104 respectively. This however not very indicative, since the rate used for chroma BD rate computation is the total rate, and values like this can even occur with a quite small PSNR drop.

Combination of I0051#1 and I0104 would bring the memory bandwidth of ARP down to 94% (compared to the worst case of current HEVC, which is 8x8 bi-pred.). This would give a drop in luma performance of approx. 0.1% BR increase.

Decision: Adopt JCT3V-I0051#1 and JCT3V-I0104.

Related proposals

Summary of coding performance and complexity

The following table shows the simulation results and complexity comparisons for CE1 related subset A proposals. In this table, memory bandwidth of depth (in DoNBDV) is considered since both proposals try to remove depth access when ARP applies. Here, it is assumed that the whole depth block is fetched in DoNBDV.

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Memory bandwidth compared to HEVC (1x1) | Memory bandwidth compared to HEVC (4x2) | Memory bandwidth compared to HEVC (4x4) | Memory bandwidth compared to HEVC (8x2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ARP + DoNBDV in 3D-HEVC |  | CTC |  | 132% | 148% | 139% | 172% |
| JCT3V-I0052 | -0.08% | -0.09% | -0.02% | 122% | 135% | 128% | 158% |
| JCT3V-I0081 | -0.08% | -0.09% | -0.02% | 122% | 135% | 128% | 158% |

DoNBDV increases the memory bandwidth of worst case in HEVC by approx. 14%. This means that the worst case memory bandwidth is 114% anyway, and disallowing DoNBDV just in case of ARP (which is not the worst case mode any more) does not help.

No action on the proposals I0052 and I0081.

### CE contributions (4)

JCT3V-I0051 CE1: Simplification of advanced residual prediction [H. Liu, Y. Chen (Qualcomm)]

Two aspects are proposed in this contribution:

- For bi-predictively coded PUs, the number of accessed blocks used in ARP is reduced from 4 to 3:

  - When current PU is predicted from one temporal reference picture and one inter-view reference picture, the temporal reference block of base block in inter-view ARP is identified with the coded motion vector.

  - When current PU is predicted from two inter-view reference pictures, the temporal reference block in the ARP target reference picture associated with reference picture list 1 is set to the one associated with reference picture list 0.

- It is proposed to disable bi-predictive 4x4 chroma blocks with ARP mode by applying uni-prediction from reference picture list 0.

This proposal was crosschecked by Sharp, and detail information can be referred to JCT3V-I0155.

A comment was made on 1b that it might not allow independent processing of list 0 and list 1 any more. It is however argued that similar interdependencies exist also in other places.

JCT3V-I0155 CE1: Cross-check of Simplification of advanced residual prediction (JCT3V-I0051) [T. Ikai (Sharp)] [late]

JCT3V-I0104 CE1: Results of ARP simplification [T. Ikai (Sharp)]

It is proposed to disable ARP for 4x4 chroma blocks to reduce ARP’s bandwidth.

This proposal was crosschecked by Qualcomm, and detail information can be referred to JCT3V-I0166.

JCT3V-I0166 CE1: Crosscheck of results of ARP simplification (JCT3V-I0104) [H. Liu (Qualcomm)] [late]

### Related contributions (12)

JCT3V-I0052 CE1 related: Alignment of disparity vector in inter-view motion and residual predictions for ARP [H. Liu, Y. Chen (Qualcomm)]

This contribution proposes to align the disparity vector used in inter-view motion prediction and residual prediction. In this way, DoNBDV is not performed when ARP applies and memory bandwidth can be saved for ARP.

This proposal was crosschecked by Hisilicon and Sharp, and detail information can be referred to JCT3V-I0071 and I0159.

Disposition: See above

JCT3V-I0071 Cross check of alignment of disparity vector in inter-view motion and residual predictions for ARP (JCT3V-I0052) [Y. Zhang, Y.-B. Lin (HiSilicon)] [late]

JCT3V-I0159 CE1 related: Cross-check of Alignment of disparity vector in inter-view [T. Ikai (Sharp)] [late]

JCT3V-I0081 3D-CE1 related: Reduction of Worst Case Memory Bandwidth for ARP [M. W. Park, J. Y. Lee, Y. Cho, C. Kim (Samsung)]

This contribution proposes to use NBDV instead of DoNBDV for the inter-view merge candidates when the ARP is enabled so that depth memory fetch is not needed anymore for ARP in the worst case.

This proposal was crosschecked by Sharp, and detail information can be referred to JCT3V-I0157.

Based on the idea description, memory bandwidth analysis and coding performance results, JCT3V-I0052 and JCT3V-I0081 are identical.

Disposition: See above

JCT3V-I0157 3D-CE1 related: Cross-check of Reduction of Worst Case Memory [T. Ikai (Sharp)] [late]

## CE2: Depth intra coding (23)

JCT3V-I0178 3D-CE1 related: Crosscheck on Diagonal prediction using ARP (JCT3V-I0089) [S. Shimizu (NTT)] [late] [miss]

JCT3V-I0185 Crosscheck on complexity reduction of bi-prediction for advanced residual prediction (JCT3V-I0096) [S. Shimizu (NTT)] [late] [miss]

JCT3V-I0180 AHG4: Crosscheck on editorial improvement for ARP specification text in 3D-HEVC (JCT3V-I0129) [S. Shimizu (NTT)] [late] [miss]

JCT3V-I0181 AHG4: Crosscheck on Semantics clean-up for advanced residual prediction weighting factor (JCT3V-I0055) [S. Shimizu (NTT)] [late] [miss]

## CE2: Depth intra coding (23)

(chaired by XX)

### Summary (1)

JCT3V-I0012 CE2 Summary Report on Depth Intra Coding [Y. Chen, Y. W. Chen] [miss]

Results of the proposals

Although there might be interactions between the three proposals especially on the overlaps of the coding gains. The results of these proposals are separately presented, although each of them has already been investigated in a way that the more closely related tools have been tested.

Results for JCT3V-I0118: CE2: Advanced boundary chain coding for depth

|  |  |  |  | n |  | e |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 118 #1 | 01% | 5% | 28% | 0. |  | % |  |  |
| 18 #2 (w/CA) | 01% | .0% | .2% | % |  | % |  |  |
| 118 #1 |  |  |  |  |  |  |  |  |
| 18 #2 (w/CA) |  |  |  |  |  |  |  |  |
|  | Video 1 | Video 2 | Video PSNR | synth PSNR / total bitrate | Enc. time |  | Dec. time |  |
| CTC |  |  |  |  |  |  |  |  |
| JCT3V-I0118 #1 | -0.11% | -0.05% | 0.14% | -0.28% | 100.8% |  | 98.4% |  |
| JCT3V-I0118 #2 (w/o CABAC) | -0.11% | -0.09% | 0.14% | -0.21% | 100.9% |  | 99.7% |  |
| AI |  |  |  |  |  |  |  |  |
| JCT3V-I0118 #1 | 0.00% | 0.00% | 0.02% | -0.29% | 101.4% |  | 100.4% |  |
| JCT3V-I0118 #2 (w/o CABAC) | 0.00% | 0.00% | 0.04% | -0.22% | 101.8% |  | 97.3% |  |

Note: The proposal shows some loss in case of coded views (0.14% on average). This indicates that the new coding mode effects a higher bit rate for depth, which might give a better tradeoff w.r.t. synthesized PSNR. Another version of boundary chain coding was removed during the 6th JCT-3V meeting, where the results by that time were as follows:

Results on removal of RBC (CTC)

    - difference when old BCC was removed)

    - Report coded er prls.Results for JCT3V-I0095: 3D-CE2: Single depth intra mode for 3D-HEVC

|  | Video 1 | Video 2 | Video PSNR | synth PSNR / total bitrate | Enc. time | Dec. time |
| --- | --- | --- | --- | --- | --- | --- |
| CTC |  |  |  |  |  |  |
| I0095 Test 1 | 0.01% | 0.07% | -0.07% | -0.29% | 101.7% | 98.2% |
| I0095 Test 2 | 0.02% | 0.01% | 0.00% | 0.00% | 104.7% | 98.9% |
| I0095 Test 3 | 0.01% | 0.07% | -0.06% | -0.26% | 100.3% | 97.1% |
| I0095 Test 4 | -0.01% | 0.01% | -0.06% | -0.29% | 105.0% | 98.4% |
| AI |  |  |  |  |  |  |
| I0095 Test 1 | 0.00% | 0.00% | -0.2% | -0.34% | 99.2% | 96.0% |
| I0095 Test 2 | 0.00% | 0.00% |  | -0.01% | 123.8% | 101.7% |
| I0095 Test 3 | 0.00% | 0.00% |  | -0.34% | 98.9% | 96.6% |
| I0095 Test 4 | 0.00% | 0.00% |  | -0.35% | 106.3% | 99.0% |

|  |  |  |  | n | e |
| --- | --- | --- | --- | --- | --- |
| Test 1 | % | % | 29% | 1. | % |
| Test 2 |  |  |  |  |  |
| Test 3 | % | % |  |  |  |
| Test 4 |  |  |  |  |  |
| Test 1 |  |  |  |  |  |
| Test 2 |  |  |  |  |  |
| Test 3 |  |  |  |  |  |
| Test 4 |  |  |  |  |  |

Results for JCT3V-I0082: 3D-CE2: Residual DC quantization in intra SDC

|  | Video PSNR / video bitrate | Video PSNR / total bitrate | Video PSNR | Synth PSNR / total bitrate | Enc. time | Dec. time |
| --- | --- | --- | --- | --- | --- | --- |
| CTC |  |  |  |  |  |  |
| I0082 #1 | 0.00% | 0.01% | 0.01% | 0.04% | 100.0% | 99.9% |
| I0082 #2 | 0.02% | -0.01% | -0.01% | 0.07% | 100.3% | 99.8% |
| I0082 #3 | -0.01% | -0.07% | -0.07% | -0.07% | 100.2% | 99.6% |
| I0082 #4 | 0.01% | -0.07% | -0.07% | -0.06% | 100.2% | 100.0% |
| AI |  |  |  |  |  |  |
| I0082 #1 | 0.00% | 0.01% | 0.01% | 0.13% | 99.6% | 99.7% |
| I0082 #2 | 0.00% | -0.05% | -0.05% | 0.19% | 100.3% | 99.8% |
| I0082 #3 | 0.00% | -0.07% | -0.07% | -0.07% | 100.1% | 99.6% |
| I0082 #4 | 0.00% | -0.11% | -0.11% | -0.08% | 99.7% | 99.5% |

|  |  | total bitrate | PSNR bitrate | n | e |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  | % | % |
| 2 |  |  |  | % | % |
| 3 |  |  |  | % | % |
| 4 |  |  |  | % | % |
| 1 |  |  |  | % | % |
| 2 |  |  |  | % | % |
| 3 |  |  |  | % | % |
| 4 |  |  |  | % | % |

    - CE contributions (8)

JCT3V-I0082 3D-CE2: Residual DC quantization in intra SDC [J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung)]

This contribution presents the results of CE2 in which residual DC quantization is proposed to reduce the magnitude of the residual DC for the general modes in intra SDC, instead of DLT. DLT signals an index instead of the DC value in intra SDC. Hence, to reconstruct the residual DC in a decoder side, DC of the predicted pixels should be first calculated and then re-mapping from the DLT index to the value is performed. In order to simplify these operation, the proposed method quantizes the residual DC with one shift operation to reduce the magnitude of the residual DC, instead of using DLT. The results shows that the proposed method achieves the coding gain of -0.1% and removes the DC calculation and re-mapping processes.

Test#1: DLT is applied to all the sequences by removing a threshold with 128

Test#2: The residual quantization is implemented without a decoder change by using the existing DLT function. When DLT is not constructed by the threshold, it stores every even depth pixel value as an index increases from 0 to 127.

Test#3: The residual DC quantization with default QP equal to 0

Test#4: The residual DC quantization with default QP equal to 1

This proposal is cross checked by JCT3V-I0122 Test#2/#3 (RWTH) and JCT3V-I0135 Test#4 (Sharp).

Test#3 is discussed as a most promising candidate from the results

In the discussion, it is pointed out the change would not be a simplification, since it changes the way of residual quantization for some modes, whereas DMM1 and DMM4 still would use the other method, and both methods need to be described and implemented.

There was also some discussion whether the proposed method restricts the possible accuracy of residual quantization.

Reduction in terms of rate rather low, no consensus that this is beneficial.

JCT3V-I0122 Cross Check of Method 2 and 3 of Residual DC quantization in intra SDC (JCT3V-I0082) [F. Jäger (RWTH Aachen University)]

JCT3V-I0135 3D-CE2: Cross-check of Test 4 of Residual DC quantization in intra SDC (JCT3V-I0082) [T. Tsukuba (Sharp)] [late]

JCT3V-I0095 3D-CE2: Single depth intra mode for 3D-HEVC [Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek)]

This contribution presents the results of CE2 in which the single depth mode is proposed to code the smooth area within a depth map. A CU-level flag is signaled to indicate whether a CU is coded as single depth mode. When a CU is coded as single depth mode, it is reconstructed by filling this CU with one single depth value. Additionally, an index is transmitted to select one out of several available depth sample candidates derived from the spatial neighboring pixels. Compared to the anchor HTM-11.0, the proposed method reportedly shows overall 0.3% BD-rate savings under common test conditions.

This proposal is cross checked by JCT3V-I0168 (Qualcomm).

The tests related to this category are listed as below.

Test 1: Test the coding performance of the proposed single depth mode. The proposed scheme in H0087 has been further simplified by reducing the size of the candidate list from 5 to 2 and replacing the corner spatial candidates with the candidates located at the center of the PU boundary.

Several additional experiments have also been tested to address several mandates of CE2, including the following.

Test 2: Test the coding performance of the current intra SDC scheme with modified encoder design. The encoder is modified by additionally testing the RDO performance for those DC values equal to the sample candidates of single depth mode. For each block coded as intra SDC mode, as shown in figure 1, the values of the neighboring pixels An-1, Bn-1, B0, A0, B-1 are additionally tested as the DC value at the encoder to see whether the coding performance could be further improved.

Figure 1. The spatial sample candidates.

Test 3: Test the coding performance of inserting DDD value as a new candidate for the single depth mode.

Test 4: Test the coding performance of applying residual coding to single depth mode.

Generally, the method is asserted to give a reasonable benefit in compression while being of relative low complexity (less complex than many other modes used in 3D-HEVC)

Decision: Adopt I0095 Test 1 (method with 2 candidates)

It was also pointed out that the benefit of other compression tools (e.g. DDD) might become smaller after this adoption. Generally, contributions showing such cases are welcome.

JCT3V-I0168 CE2: Crosscheck of single depth intra mode for 3D-HEVC (JCT3V-I0095) [H. Liu (Qualcomm)] [late]

JCT3V-I0118 CE2: Advanced boundary chain coding for depth intra mode [J. Seo, M. Koo, J. Nam, S. Yea (LGE)]

This proposal provides the complexity assessment of the advanced boundary chain coding (ABC). The ABC is an improved coding tool based on the region boundary coding which separates a block into two partitions following a contour described by a chain code. The method of complexity assessment is based on JCT3V-H0127 which provides a complexity analysis for depth intra coding modes. This proposal also presents the experimental results of ABC mode with and without CABAC (coded bins). The proposed method with CABAC reportedly shows 0.3% and 0.3% BDR gains in synthesized views under CTC and AI configurations, respectively. The coding gains of ABC without CABAC are 0.2% and 0.2% under CTC and AI configurations, respectively.

This proposal is cross checked by JCT3V-I0140 (MediaTek).

The proposal shows better benefit now than the previous chain coding mode that had been removed during the 6th meeting (where no benefit in terms of synthesis PSNR had been observed).

It is however increasing the complexity, in particular the possible maximum number of context-coded bins per sample might be of concern. If it would increase the worst case CABAC throughput that HEVC currently allows, this would be highly undesirable when comparing against the relative small benefit in terms of compression quality.

Action item: investigate highest possible number of context coded bins for an 8x8 CU. Revisit.

JCT3V-I0140 3D-CE2: Crosscheck on advanced boundary chain coding for depth intra mode (JCT3V-I0118) [Y.-W. Chen (MediaTek)] [late]

JCT3V-I0150 3D-CE2: Cross check of advanced boundary chain coding for depth intra mode (JCT3V-I0118) [J. Y. Lee, C. Kim (Samsung)] [late]

### Related contributions (14)

JCT3V-I0066 CE2 related: Simplification of DMM4 [P. Merkle, D. Marpe (HHI)]

This contribution proposes two modification of the DMM4 depth intra coding mode: simplification of threshold calculation as well as encoder optimization and simplification.

Currently the threshold value for contour segmentation derivation is calculated as the mean value of all depth samples, requiring N²-1 additions and one shift operation for an NxN block. Similar as in proposal JCT3V-H0058, the threshold calculation is simplified by using the mean of the four corner values as the threshold value for contour segmentation, requiring 3 additions and one shift, independent of the block size.

Currently, intra modes in inter slices are only tested under certain conditions in order to limit the encoder complexity. It is proposed that this encoder restriction is removed for DMM4 mode, i.e. always testing it in inter slices. There is also another encoder optimization aspect proposed that avoids a double check.

A similar approach is proposed for DBBP in JCT3V-I0076 (original proposal was JCT3V-H0068).

Decision(SW): Adopt encoder modification proposed in I0066

For threshold derivation simplification, consider a unified solution with DBBP (revisit in context of I0076)

JCT3V-I0068 Cross Check of CE2 related: Simplification of DMM4 (JCT3V-I0066) [F. Jäger (RWTH Aachen University)]

JCT3V-I0110 Lookup table size reduction in DMM1 [T. Tsukuba, T. Ikai (Sharp)]

This contribution proposes a method to reuse wedgelet pattern of 16x16 block for 32x32 block instead of specifying 32x32 wedgelet pattern in order to reduce the storage size for wedgelet pattern in DMM1. Actually, a pattern for a 16x16 block is scaled to form a 32x32 block. Experimental result reportedly shows that 0.06% / 0.03% coding loss for synth PSNR vs. total bitrate in the CTC / all-intra configuration, respectively. It is asserted that this proposed method can achieve storage size reduction by 79.5%.

General consensus that this is a very useful simplification, and the systematic structure of the 32x32 is also well known. Candidate for adoption.

Revisit: Cross-checking party to confirm that the WD text matches with the software implementation.

JCT3V-I0142 Crosscheck on lookup table size reduction in DMM1 (JCT3V-I0110) [Y.-W. Chen (MediaTek)] [late]

JCT3V-I0083 3D-CE2 related: Allowing zero DC candidate in inter SDC [J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung)]

At the 7th meeting, multiple DC candidates were adopted to improve the performance of the inter SDC (G0101). However, the zero DC candidate was not adopted due to the additional branch in merge mode in WD (G0058). To figure out the problem and for the design generality (that covers the zero DC candidate as well), the proposed method allows the zero candidate. It can be represented by replacing or deleting one condition in WD. Results demonstrate that the proposed method does not affect the coding performance a lot.

This proposal suggests two solutions to change the condition of depth_dc_flag, indicating whether the DC values are present or not. One solution leads to slight coding loss and the other doesn’t introduce coding loss.

Presentation deck not uploaded.

Option 1 would remove one condition check at the CU level, but would incur a small loss.

Option 2 changes one condition check by a different one, with no loss (or negligible gain)

No action

JCT3V-I0169 CE2 related: Crosscheck of allowing zero DC candidate in inter SDC (JCT3V-I0083) [H. Liu (Qualcomm)] [late]

JCT3V-I0084 3D-CE2 related: Fast SDC DC offset decision [J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung)]

Multiple DC candidates were adopted to improve the performance of inter and intra SDC. At an encoder, SDC checks five different candidates, which consist of DC, DC-1, DC+1, DC-2, and DC+2, and decides the best one through exhaustive comparison. In order to reduce the encoding complexity, the proposed method introduces a fast SDC DC offset decision method. Results demonstrate that the proposed method save the encoding time without coding loss.

Presentation deck not uploaded.

Decision(SW): Adopt I0084

JCT3V-I0143 3D-CE2 related: Crosscheck on fast SDC dC offset decision (JCT3V-I0084) [Y.-W. Chen (MediaTek)] [late]

JCT3V-I0117 3D-CE2 related: Reduction of the number of intra prediction modes for depth coding [J. Heo, J. Nam, S. Yea (LGE)]

This contribution presents an intra prediction mode for depth map coding. The proposed method reduces the number of intra prediction modes for depth map because the depth map has different characteristics compared to the texture video: 1) little variation within a block 2) simple or smooth region. It means that 35 intra prediction modes used in the conventional intra coding is unnecessarily expensive for depth intra coding. Therefore, this proposal reduced the number of conventional intra prediction modes for depth map coding. The proposed method has minor coding gains under the common test condition (CTC) and all-intra configuration (AI) with negligible encoding/decoding complexity.

Presentation deck not uploaded.

The proposal is to use only 11 intra coding modes instead of 35 (i.e. 3 MPM and 8 remaining instead of 32)

This would be a substantial modification of existing directional intra prediction (including the parsing process) for the purpose of depth map coding, which is not justified by the relative small compression benefit it gives.

JCT3V-I0179 3D-CE2 related: Crosscheck on reduction of the number of intra prediction modes for depth coding (JCT3V-I0117) [S. Shimizu (NTT)] [late] [miss]

JCT3V-I0120 CE2 related: Remove “depth_dc_flag” signalling in DMM cases [X. Chen, X. Zheng, Y. Lin, J. Zheng (HiSilicon)]

In the current 3D-HEVC, SDC DMM and DMM are available in depth intra coding and 4 DMM cases(depth_dc_flag=0 or 1, DMM1 DMM4) are used in DMM process. In this contribution, depth_dc_flag is removed in DMM process. Experimental results reportedly show the proposed simplifications bring no coding efficiency impact.

It is proposed that the depth_dc_flag (indicating whether residual value of segments to be 0 in case of sdc_flag equal to 1 and whether dc offset exists in case of when sdc_flag is equal to 0) is conditionally signalled when sdc_flag (indicating whether SDC is used) is unequal to 0. Experimental results reportedly show the proposed simplifications bring no coding efficiency impact.

Several experts expressed that this is a useful simplification of the syntax

Decision: Adopt I0120

JCT3V-I0171 CE2 related: Crosscheck of remove “depth_dc_flag” signalling in DMM cases (JCT3V-I0120) [H. Liu (Qualcomm)] [late]

JCT3V-I0085 3D-CE2 related: Separate enabling flag for intra coding tools [J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung)]

Intra coding tools such as intra SDC and DMM (DMM1 and DMM4) are enabled and disabled by a single flag ‘vps_depth_modes_flag’, while inter coding tools are individually selectable by their corresponding flags. When intra SDC is on and DMM is off, a DMM flag need to be signalled in each CU, which may be redundant. This contribution proposes to define a separate flag for DMM and intra SDC. In addition, it is suggested to change the names of DMM1 and DMM4 into DMM and IVP (Intra-view prediction), respectively.

Presentation deck not uploaded.

Consensus that separate DMM4 flag is needed

Separate DMM1/intra SDC not needed, as these are always used in combination

Decision:

    - Retain inter_sdc flag as is

    - Introduce new dmm4 flag

    - Change semantics of depth_modes flag such that it controls dmm1 and intra sdc

(naming is to discretion of editors)

JCT3V-I0119 Cross check of separate enabling flag for intra coding tools (JCT3V-I0085) [P. Merkle (HHI)]

JCT3V-I0179 3D-CE2 related: Crosscheck on reduction of the number of intra prediction modes for depth coding (JCT3V-I0117) [S. Shimizu (NTT)] [late] [miss]

JCT3V-I0175 3D-HEVC: Simplification of DMM method [H. Sabirin, K. Kawamura, S. Naito (KDDI)] [late]

This contribution presents the effect of using only one mode of DMM in depth coding. Since the first proposal time, the modes of DMM have been simplified into two modes: DMM1 and DMM4 by the last meeting. It is reported from the experiments that only one DMM mode is possible to maintain the coding performance instead of two modes.

It is proposed to remove DMM4. Results are presented that indicate negligible differences (average 0% but sequence dependent) for CTC, and 0.2% average BR increase in AI

# 3DV standards development (incl. software, conformance) (6)

## AVC Extensions (5)

JCT3V-I0025 AhG4: Software Improvements on 3DVATM [D. Tian, A. Vetro (MERL), S. Shimizu (NTT)]

JCT3V-I0102 3D-AVC defect correction: texture view dependencies missing from SPS [M. M. Hannuksela (Nokia), X. Huang (USTC)]

JCT3V-I0126 Proposed Draft Text of New AVC Amendment: MFC plus Depth, and Texture and Depth View Packing SEI message [P. Yin, T. Lu, H. Ganapathy, T. Chen, W. Husak (Dolby), M. Tanimoto (NISRI), T. Senoh (NICT), Y. Chen (Qualcomm)]

Note: The SEI message code point is already in the spec (probably was forgotten to remove).

JCT3V-I0127 On MFC plus Depth: Reference Software Update [L. Grimaldi, K. Schueuer, P. Yin, T. Lu, H. Ganapathy, T. Chen, W. Husak (Dolby)]

JCT3V-I0063 Software Improvement for Texture and Depth View Packing SEI Message [T. Senoh, K. Wakunami, Y. Ichihashi, H. Sasaki, R. Oi, K. Yamamoto (NICT)]

## MV-HEVC (0)

## 3D-HEVC (1)

JCT3V-I0093 Depth coding compatible with arbitrary bit-depth in 3D-HEVC [K. Zhang, J. An, X. Zhang, H. Huang, J.-L. Lin, S. Lei (MediaTek)]

Open issue from AHG2 report:

- Subsequent to the Valencia meeting, proponents of H0094 and H0205 provided revised specification text. However, additional changes have neither mentioned in the original input proposals nor in the meetings notes such that they have not been incorporated into the draft. Further clarification might be required since the changes have been integrated to the software.

The proponents were requested to submit late input documents about the intended changes. Revisit.

# High-level syntax (42)

## MV-HEVC/3D-HEVC specific (6)

JCT3V-I0090 3D HEVC HLS Cleanup [Y. Cho, M. W. Park, J. Y. Lee, C. Kim (Samsung)]

JCT3V-I0099 On the video parameter set extension 2 in 3D-HEVC [H. Huang, J. An, J.-L. Lin, K. Zhang, X. Zhang, S. Lei (MediaTek)]

JCT3V-I0147 Cross check of On the video parameter set extension 2 in 3D-HEVC (JCT3V-I0099) [M. W. Park, C. Kim (Samsung)] [late]

JCT3V-I0100 3D-HEVC HLS: General comments on 3D-HEVC high level syntax [B. Choi, J.Y. Lee, C. Kim (Samsung)]

JCT3V-I0101 3D-HEVC HLS: Dependency between texture and depthmap [B. Choi, J.Y. Lee, C. Kim (Samsung)]

JCT3V-I0103 MV-HEVC HLS: Clean-up of MV-HEVC SEI messages [M. M. Hannuksela (Nokia)]

## Common MV-HEVC/SHVC (36)

### Summary

JCT3V-I0190 BoG report on HLS [J. Boyce]

### Contributions (36)

JCT3V-I0021 AHG10: Profile indication for additional layer sets [M. M. Hannuksela (Nokia)]

(=JCTVC-R0042)

JCT3V-I0022 SHVC/MV-HEVC level definitions and related decoder capability requirements [A. K. Ramasubramonian, Y.-K. Wang, K. Rapaka, Hendry (Qualcomm)]

(=JCTVC-R0043)

JCT3V-I0023 Use of general_profile_space for auxiliary picture profile indication [J. Boyce (Vidyo)]

(=JCTVC-R0041)

JCT3V-I0024 AHG10: On decoder capability indication [M. M. Hannuksela (Nokia)]

(=JCTVC-R0044)

JCT3V-I0026 AHG10: Comments on JCTVC-R0043/JCT3V-I0022 [M. M. Hannuksela (Nokia)]

(=JCTVC-R0046)

JCT3V-I0027 MV-HEVC/SHVC HLS: VPSs with nuh_layer_id greater than 0 [M. M. Hannuksela (Nokia)]

(=JCTVC-R0049)

JCT3V-I0028 Picture size related constraints and level limits [S. Deshpande (??), Y.-K. Wang, Hendry, A. K. Ramasubramonian (??), M. M. Hannuksela (??)]

(=JCTVC-R0052)

JCT3V-I0029 AHG10 and MV-HEVC/SHVC HLS: On invocation of a correct decoding process for the current picture [M. M. Hannuksela (Nokia)]

(=JCTVC-R0053)

JCT3V-I0030 AHG10: Comments on JCTVC-R0041/JCT3V-I0023 [M. M. Hannuksela (Nokia)]

(=JCTVC-R0054)

JCT3V-I0031 On monochrome coding (concerning multi-layer profiles and auxiliary pictures) [M. M. Hannuksela (Nokia)]

(=JCTVC-R0062)

JCT3V-I0032 MV-HEVC/SHVC HLS: On cross-layer impacts of IRAP and EOS [M. M. Hannuksela (Nokia)]

(=JCTVC-R0071)

JCT3V-I0033 MV-HEVC/SHVC HLS: On hybrid codec scalability [B. Choi, C. Kim (Samsung)] [late]

(=JCTVC-R0124)

JCT3V-I0034 MV-HEVC/SHVC HLS: On layer set and bitstream extraction [B. Choi, C. Kim (Samsung)] [late]

(=JCTVC-R0125)

JCT3V-I0035 MV-HEVC/SHVC HLS: On External Base Layer [S. Deshpande (Sharp)]

(=JCTVC-R0153)

JCT3V-I0036 MV-HEVC/SHVC HLS: On Alternative Output Layer and Default Output Layer Indication [S. Deshpande (Sharp)]

(=JCTVC-R0154)

JCT3V-I0037 MV-HEVC/SHVC HLS: Comments on SHVC and MV-HEVC [S. Deshpande (Sharp)]

(=JCTVC-R0155)

JCT3V-I0038 MV-HEVC/SHVC HLS: Move conformance cropping window parameters to rep_format( ) syntax structure of VPS extension [J. Boyce, D. Hong (Vidyo)]

(=JCTVC-R0156)

JCT3V-I0039 Modifications to Scalable Main profile and general and profile-specific level limits [J. Boyce (Vidyo)]

(=JCTVC-R0158)

JCT3V-I0040 HEVC/MV-HEVC/SHVC HLS: SEI message recommended by MMT for improvement of HEVC packet loss resilience [M. Sychev, S. Ikonin (Huawei)]

(=JCTVC-R0159)

JCT3V-I0041 HEVCv1/MV-HEVC/SHVC HLS: On extensibility of nesting SEI messages [A. K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)]

(=JCTVC-R0221)

JCT3V-I0042 MV-HEVC/SHVC HLS: On POC [Hendry, A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)]

(=JCTVC-R0223)

JCT3V-I0043 MV-HEVC/SHVC HLS: Clarifications to the specification of end of sequence (EOS) NAL unit [A. K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)]

(=JCTVC-R0224)

JCT3V-I0044 MV-HEVC/SHVC HLS: On indication of the use of temporal motion vector prediction [Hendry, A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)]

(=JCTVC-R0226)

JCT3V-I0045 MV-HEVC/SHVC HLS: Miscellaneous cleanups [Y.-K. Wang, Hendry, A. K. Ramasubramonian (Qualcomm)]

(=JCTVC-R0227)

JCT3V-I0046 MV-HEVC/SHVC HLS: Unavailable and useless stuff [Y.-K. Wang, A. K. Ramasubramonian, Hendry (Qualcomm)]

(=JCTVC-R0230)

JCT3V-I0047 MV-HEVC/SHVC: Signalling and use of HRD parameters for bitstream partitions [Y.-K. Wang, Hendry, A. K. Ramasubramonian (Qualcomm), S. Deshpande (Sharp), M. M. Hannuksela (Nokia)] [late]

(=JCTVC-R0231)

JCT3V-I0048 AHG10: Processing of bitstreams without an available base layer [M. M. Hannuksela (Nokia)]

(=JCTVC-R0235)

JCT3V-I0049 MV-HEVC/SHVC HLS: On access unit boundary detection [M. M. Hannuksela (Nokia)]

(=JCTVC-R0236)

JCT3V-I0050 MV-HEVC/SHVC HLS: Editorial cleanups [A. K. Ramasubramonian, Y.-K. Wang, Hendry (Qualcomm)]

(=JCTVC-R0238)

JCT3V-I0059 MV-HEVC/SHVC HLS: On persistence of SEI messages [M. M. Hannuksela (Nokia)] [late]

(=JCTVC-R0255)

JCT3V-I0060 Language indication for Overlay information SEI message [K. Suehring (Fraunhofer HHI)] [late]

(=JCTVC-R0257)

JCT3V-I0062 HEVCv1/MV-HEVC/SHVC HLS: Conformance cleanups [Y.-K. Wang, A. K. Ramasubramonian, Hendry (Qualcomm)] [late]

(=JCTVC-R0272)

JCT3V-I0064 MV-HEVC/SHVC HLS: On active parameter sets SEI message [M. M. Hannuksela (Nokia)] [late]

(=JCTVC-R0274)

JCT3V-I0067 MV-HEVC/SHVC HLS: On sub-bitstream extraction [M. M. Hannuksela (Nokia)] [late]

(=JCTVC-R0276)

JCT3V-I0091 MV-HEVC/SHVC HLS: More miscellaneous cleanups [Hendry, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)] [late]

(=JCTVC-R0279)

JCT3V-I0092 MV-HEVC/SHVC HLS: Bitstream conformance restrictions in subclause C.4 [A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)] [late]

(=JCTVC-R0280)

# Non-CE technical contributions (60)

## ARP

JCT3V-I0089 3D-CE1 related: Diagonal prediction using ARP [J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung)]

Note: This is not CE1 related.

Advanced residual prediction (ARP) is used to efficiently code a residual between original and predicted pixels. Both temporal and inter-view ARP are supported and three weighting factors such as 0, 1, and 0.5 are considered. However, it is not clear why the 0.5 weighting factor is used. This contribution proposes to support diagonal prediction instead of the 0.5 weighting factor. The result illustrates that, even though the proposed method achieves the minor coding gain (about -0.1% in a dependent view), it reduces the memory bandwidth (from 172% to 67% in the 8x2 memory pattern) by removing the additional block accesses when the weighting factor is equal to 0.5.

Presentation deck not uploaded.

Reduction of memory bandwidth refers to average (not worst case).

Worst case memory bandwidth has been resolved per adoptions from CE1

The removal of 0.5 weighting does not give a critical complexity advantage and would give up flexibility that may be desirable in ARP.

No action.

JCT3V-I0178 3D-CE1 related: Crosscheck on Diagonal prediction using ARP (JCT3V-I0089) [S. Shimizu (NTT)] [late] [miss]

JCT3V-I0096 On complexity reduction of bi-prediction for advanced residual prediction [Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek)]

In HTM-11.0, when motion information of list0 is the same as that of list1, i.e., two motion vectors point to a same reference block, the encoder and the decoder can non-normatively change the inter prediction direction from bi-prediction to uni-prediction to avoid the unnecessary computation caused by the redundant motion compensation process. Experimental results show that the decoder could save around 5% decoding time by this non-normative fast bi-prediction. However, this non-normative fast bi-prediction is currently disabled for advanced residual prediction (ARP) in reference software, HTM-11.0 due to the unequal operations between uni-prediction and bi-prediction in ARP. To resolve this issue, it is proposed to align the operations between uni-prediction and bi-prediction for ARP. Experimental results reportedly show that the proposed method causes no coding efficiency loss while the unnecessary computation of bi-prediction can be removed for ARP coded PUs.

Presented results are somewhat confusing (e.g. the reported 5% reduction of decoding time includes the benefit of normal bi-prediction for which HEVC already includes such mechanism)

In the last meeting (JCT3V-H0097), it was analysed that the benefit would only be effective in 1% of all MC blocks with current test set (i.e. where ARP with identical motion realistically appears). There is not doubt that this is realistic.

The new proposal (which give a slight increase in compression) would make the normal process more complicated due to additional clipping operation and block-level check about the reference to use.

Benefit not obvious.

No action.

JCT3V-I0167 Crosscheck of On complexity reduction of bi-prediction for advanced residual prediction (JCT3V-I0096) [H. Liu (Qualcomm)] [late]

JCT3V-I0173 Cross-check of On complexity reduction of bi-prediction for advanced residual prediction (JCT3V-I0096) [T. Ikai (Sharp)] [late]

JCT3V-I0185 Crosscheck on complexity reduction of bi-prediction for advanced residual prediction (JCT3V-I0096) [S. Shimizu (NTT)] [late] [miss]

JCT3V-I0072 Problem fix for MV scaling in inter-view ARP [J. An, K. Zhang, X. Zhang, H. Huang, J.-L. Lin, S. Lei (MediaTek)]

In current inter-view ARP, a problem exists in the scaling process when the MV in the reference view is in inter-view direction (IBP configuration) which results in the software crash. This contribution proposes to fix this problem by setting the derived MV directly as (0, 0) for that situation to avoid the problematic scaling.

Decision(BF/ED/SW): Adopt

Note (to editor): Terms like ‘temporal reference picture’, ‘inter view reference picture’ should be avoided and/or clearly specified.

JCT3V-I0129 AHG4: Editorial improvement for ARP specification text in 3D-HEVC [L. Zhang, H. Liu, Y. Chen (Qualcomm)]

In current 3D-HEVC, the advanced residual prediction (ARP) working draft text is implemented in a way that reuses the sub-clauses of weighted sample prediction process in HEVC version 1 which invokes right shifting operations due to the increased bit-depth in interpolation process. However, the bilinear interpolation process is defined in a way that the internal bit-depth of the prediction signal in interpolation process is kept unchanged during the bilinear filtering process used by ARP process. Therefore, the reuse of the weighted sample prediction process results in wrong prediction signal. It is proposed to fix these unintended bugs in this contribution by modifying the bilinear filtering process.

There is no doubt that the current text description of bilinear interpolation and the subsequent processing for generating the prediction deviates from the software. It is intended:

- describe the interpolation process in a unified style with other HEVC interpolation processes

- have the same number of clipping operation (asserted to be one in the very end) as in the usual HEVC prediction process

- clarify the bit depth of the residual

Offline activity (Y. Chen, J.-L. Lin, Shimizu) to identify the issues and suggest actions both in text and software.

JCT3V-I0180 AHG4: Crosscheck on editorial improvement for ARP specification text in 3D-HEVC (JCT3V-I0129) [S. Shimizu (NTT)] [late] [miss]

JCT3V-I0055 AHG4: Semantics clean-up for advanced residual prediction weighting factor [Y. Chen, L. Zhang (Qualcomm)]

In current 3D-HEVC, a weighting factor index is signaled to indicate whether advanced residual prediction (ARP) is enabled for one coding unit (CU), and if yes, what is the weight applied to the predicted residue. The weighting factor index iv_res_pred_weight_idx however has an issue of having unclear semantics. Even when it is not equal to 0, it might happen that ARP doesn’t apply to the current CU. Such a behavior is considered contradictive to the value of this syntax element of iv_res_pred_weight_idx. In this proposal, it is proposed that iv_res_pred_weight_idx has to be set to 0 (at the encoder side) if eventually no ARP applies to the current CU. From the decoder perspective, a simple constraint is added to clean-up the design and make the semantics of iv_res_pred_weight_idx more reasonable. In addition, the iv_res_pred_weight_idx is currently constrained to be 0 when NBDV doesn’t return a valid disparity vector. However, this constraint shouldn’t apply for inter-view ARP thus the constraint is proposed to be modified. The proposed fixes don’t introduce any coding performance change under common test conditions.

There is no doubt that the current spec. is clear enough such that a decoder when receiving a bitstream would not run into problems. In principle, the proposal would modify the spec such that additional bitstream constraints are imposed. On the other hand, some concern was raised that encoders might be forced to follow these constraints and implement additional checks which could increase encoder complexity. Decoders would not benefit by reduced complexity when these constraints are imposed.

Furthermore, the results show that the coding performance is not unchanged when these constraints are imposed. This seems to indicate that the combination of syntax elements and other states that is intended to be disallowed can happen even under CTC. Therefore, there might even be the possibility that specifically smart encoders utilize this to gain some more benefit.

No action at this point

Note: It should also be checked whether the existing constraint disallowing ARP when NBDV is unavailable is necessary. Y. Chen to report back on this issue. Revisit.

JCT3V-I0181 AHG4: Crosscheck on Semantics clean-up for advanced residual prediction weighting factor (JCT3V-I0055) [S. Shimizu (NTT)] [late] [miss]

## IC (4)

(chaired by XX)

JCT3V-I0080 Parameter Derivation for Illumination Compensation [M. W. Park, J. Y. Lee, Y. Cho, C. Kim (Samsung)]

JCT3V-I0158 Cross-check of Parameter Derivation for Illumination Compensation [T. Ikai (Sharp)] [late]

JCT3V-I0132 Analysis for redundant clippings in illumination compensation [H. Liu, Y. Chen (Qualcomm)]

JCT3V-I0172 Crosscheck of analysis for redundant clippings in illumination compensation (JCT3V-I0132) [S. Yoo, S. Yea (LGE)] [late]

## MV/DV inheritance / coding (21)

(chaired by XX)

JCT3V-I0053 Simplifications for 3D-HEVC merge candidate list generation [H. Liu, Y. Chen (Qualcomm)]

JCT3V-I0139 Crosscheck for JCT3V-I0053 Simplifications for 3D-HEVC merge candidate list generation [J. An (Mediatek)] [late]

JCT3V-I0154 Cross check of simplifications for 3D-HEVC merge candidate list generation (JCT3V-I0053) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-I0073 MV sharing for depth coding in 3D-HEVC [K. Zhang, J. An, X. Zhang, H. Huang, J.-L. Lin, S. Lei (MediaTek)]

JCT3V-I0161 Cross-check of MV sharing for depth coding in 3D-HEVC (JCT3V-I0073) [T. Ikai (Sharp)] [late]

JCT3V-I0074 Depth compensation calibration in 3D-HEVC [K. Zhang, J. An, X. Zhang, H. Huang, J.-L. Lin, S. Lei (MediaTek)]

JCT3V-I0151 Cross check of depth compensation calibration in 3D-HEVC (JCT3V-I0074) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-I0086 Simplification of shift DV candidate [J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung)]

JCT3V-I0113 Crosscheck on simplification of shift DV candidate (JCT3V-I0086) [Y. Zhang (Zhejiang University)]

JCT3V-I0087 Simplification of 3D Merge list construction [J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung), Y. Zhang, L. Yu (Zhejiang University)]

JCT3V-I0156 Cross-check of Simplification of 3D Merge list construction (JCT3V-I0087) [T. Ikai (Sharp)] [late]

JCT3V-I0098 Unification of Merging Candidate List Construction in 3D-HEVC [J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)

JCT3V-I0152 Cross check of unification of the merge candidate sets in 3D-HEVC (JCT3V-I0098) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-I0112 Simplification for VSP-related Merge Candidate List Construction [Y. Zhang, Q. Wang, L. Yu (Zhejiang Univ.)]

JCT3V-I0136 Crosscheck results on simplification for VSP-related merge candidate list construction (JCT3V-I0112) [M. Li, P. Wu (ZTE)]

JCT3V-I0114 Improvement on merge list construction [S. Yoo, J. Nam, S. Yea (LGE)]

JCT3V-I0141 Crosscheck on Improvement on merge list construction (JCT3V-I0114) [Y.-W. Chen (MediaTek)] [late]

JCT3V-I0130 Simplification of VSP candidate derivation [G. Bang (ETRI), M. S. Lee, Y. S. Heo, G. H. Park (KHU), G. S. Lee, N. H. Hur (ETRI)]

JCT3V-I0174 Crosscheck on Simplification of VSP Candidate Derivation (JCT3V-I0130) [Y. Zhang (Zhejiang University)] [late]

JCT3V-I0131 Adaptive candidate selection [G. Bang (ETRI), Y. S. Heo, G. H. Park (KHU), G. S. Lee, N. H. Hur (ETRI)]

JCT3V-I0149 Cross check of Adaptive candidate selection (JCT3V-I0131) [M. W. Park, C. Kim (Samsung)] [late]

## SDC (6)

(chaired by XX)

JCT3V-I0058 Segment-wise Prediction (SP) Merge Candidate [F. Jäger (RWTH Aachen University), J. Konieczny, G. Cordara (Huawei Technologies)]

JCT3V-I0170 Crosscheck of segment-wise Prediction (SP) Merge Candidate (JCT3V-I0058) [H. Liu (Qualcomm)] [late]

JCT3V-I0075 Segmental prediction for Inter-SDC in 3D-HEVC [K. Zhang, J. An, X. Zhang, H. Huang, J.-L. Lin, S. Lei (MediaTek)]

JCT3V-I0133 Cross check of MediaTek's proposal on Segmental prediction for Inter-SDC in 3D-HEVC(JCT3V-I0075) [J. Zheng (HiSilicon)] [late]

  - DBBP (23)

(chaired by XX)

JCT3V-I0054 Improved DBBP in 3D-HEVC [Y. Chen, Q. Yu, H. Liu (Qualcomm), S. Ma (PKU)]

JCT3V-I0061 Cross Check of Improved DBBP in 3D-HEVC (JCT3V-I0054) [F. Jäger (RWTH Aachen University)]

JCT3V-I0056 Cleanup of Depth-based Block Partitioning (DBBP) [F. Jäger (RWTH Aachen University)]

JCT3V-I0065 Cross check of cleanup of DBBP (JCT3V-I0056) [P. Merkle (HHI)]

JCT3V-I0058 Segment-wise Prediction (SP) Merge Candidate [F. Jäger (RWTH Aachen University), J. Konieczny, G. Cordara (Huawei Technologies)]

JCT3V-I0170 Crosscheck of segment-wise Prediction (SP) Merge Candidate (JCT3V-I0058) [H. Liu (Qualcomm)] [late]

JCT3V-I0070 Simplification of partition derivation in DBBP [C. Zhu, Y.-H. Zhang (UESTC), Y.-B. Lin, J.-H. Zheng (HiSilicon), J.-J. Song (UESTC)]

JCT3V-I0148 Cross check of Simplification of partition derivation in DBBP (JCT3V-I0070) [M. W. Park, C. Kim (Samsung)] [late]

JCT3V-I0076 Simplification of Threshold Derivation for DBBP and DMM4 [M. W. Park, J. Y. Lee, Y. Cho, C. Kim (Samsung)]

JCT3V-I0138 Crosscheck for JCT3V-I0076 Simplification of Threshold Derivation for DBBP and DMM4 [H. Huang (MediaTek)] [late]

JCT3V-I0077 Partition Derivation for DBBP [M. W. Park, J. Y. Lee, Y. Cho, C. Kim (Samsung)]

JCT3V-I0183 Crosscheck on Partition Derivation for DBBP (JCT3V-I0077) [S. Shimizu (NTT)] [late] [miss]

JCT3V-I0088 Simplification of filtering in DBBP and its extension to PU boundary [J. Y. Lee, M. Mishurovskiy, M. W. Park, Y. Cho, C. Kim (Samsung)]

JCT3V-I0137 Crosscheck for JCT3V-I0088, Samsung's Simplification of filtering in DBBP and its extension to PU boundary [X. Zhang (MediaTek)] [late]

JCT3V-I0094 DBBP simplification based on sub-block level processing [X. Zhang, K. Zhang, J. An, H. Huang, J.-L. Lin, S. Lei (MediaTek)]

JCT3V-I0153 Cross check of DBBP simplification based on sub-block level processing (JCT3V-I0094) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-I0097 Removal of the decoder-side DBBP partition derivation process [J.-L. Lin, Y.-W. Chen, X. Zhang, K. Zhang, Y.-W. Huang, S. Lei (MediaTek)]

JCT3V-I0186 Crosscheck on removal of the decoder-side DBBP partition derivation process (JCT3V-I0097) [S. Shimizu (NTT)] [late] [miss]

JCT3V-I0107 Partition decision unification for DBBP and VSP [T. Ikai, T. Tsukuba (Sharp)]

JCT3V-I0145 Cross check of Partition decision unification for DBBP and VSP (JCT3V-I0107) [M. W. Park, C. Kim (Samsung)]

JCT3V-I0108 Disparity vector unification for DBBP and VSP [T. Ikai, T. Tsukuba (Sharp)]

JCT3V-I0146 Cross check of Disparity vector unification of DBBP and VSP (JCT3V-I0108) [M. W. Park, C. Kim (Samsung)]

JCT3V-I0109 One dimensional DBBP boundary filtering [T. Ikai, T. Tsukuba (Sharp)]

JCT3V-I0144 Cross check of One dimensional DBBP boundary filtering (JCT3V-I0109) [M. W. Park, C. Kim (Samsung)] [late]

JCT3V-I0189 Cross check for option2 of one dimensional DBBP boundary filtering (JCT3V-I0109) [H. Huang (MediaTek)]

JCT3V-I0124 On DBBP Filtering Simplification [Z. Gu (SCU), J. Zheng (HiSilicon), N. Ling (SCU), P. Zhang (HiSilicon)]

JCT3V-I0165 Crosscheck for JCT3V-I0124, On DBBP Filtering Simplification [X. Zhang, K. Zhang (MediaTek)] [late]

JCT3V-I0125 Bug-fixing of depth-based block partitioning signaling [Y. Lin, J. Zheng (HiSilicon), X. Wang, C. Zhu (UESTC)]

JCT3V-I0163 Cross-check of Bug-fixing of depth-based block partitioning signaling (JCT3V-I0125) [T. Ikai, T. Tsukuba (Sharp)] [late] [miss]

## VSP (3)

(chaired by XX)

JCT3V-I0079 DV Derivation for VSP [M. W. Park, J. Y. Lee, I. K. Choi, Y. Cho, C. Kim (Samsung)]

JCT3V-I0184 Crosscheck on DV Derivation for VSP (JCT3V-I0079) [S. Shimizu (NTT)] [late] [miss]

JCT3V-I0105 Adaptive depth access for VSP enhancement [T. Ikai, T. Tsukuba (Sharp)]

JCT3V-I0187 Crosscheck on Adaptive depth access for VSP enhancement (JCT3V-I0105) [S. Shimizu (NTT)] [late] [miss]

JCT3V-I0106 Further VSP inheritance simplification [T. Ikai, T. Tsukuba (Sharp)]

JCT3V-I0188 Crosscheck on Further VSP inheritance simplification (JCT3V-I0106) [S. Shimizu (NTT)] [late] [miss]

## Other (3)

(chaired by XX)

JCT3V-I0057 Clipping Operation for DLT Indexes [F. Jäger (RWTH Aachen University)]

JCT3V-I0177 Cross-check of Clipping Operation for DLT Indexes (JCT3V-I0057) [J. Konieczny, G. Cordara] [late] [miss]

JCT3V-I0176 3D-HEVC: Quantization of Residual Depth Data [H. Sabirin, K. Kawamura, S. Naito (KDDI)] [late]

# Non-normative contributions (0)

## Encoder optimization (0)

JCT3V-I0123 Fast Intra SDC coding for 3D-HEVC Intra Coding [Z. Gu (SCU), J. Zheng (HiSilicon), N. Ling (SCU), P.Zhang (HiSilicon)]

JCT3V-I0164 Crosscheck for JCT3V-I0123, Fast Intra SDC coding for 3D-HEVC Intra Coding [X. Zhang, K. Zhang (MediaTek)] [late] [miss]

## Rate control (0)

# Withdrawn, unclear allocation (3)

JCT3V-I0121 Withdrawn

JCT3V-I0128 Withdrawn

JCT3V-I0162 Withdrawn

# Plenary Discussions and BoG Reports

## Project development

## BoGs

# Project planning

## General issues for CEs

A preliminary CE description is to be approved at the meeting at which the CE plan is established.

It is possible to define sub-experiments within particular CEs. As a general rule, it was agreed that each CE should be run under the same testing conditions using same software codebase, which should be based on either the ATM or HTM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for experiments were described in the output document JCT3V-F1100.

A deadline of two weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-3V reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the relevant software basis. Exceptions must be justified, discussed on the general JCT-3V reflector, and recorded in the abstract of the summary report.

Final CEs shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

Experiment descriptions should be written in a way such that it is understood as a JCT-3V output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-3V document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

Non-final CE plan documents were reviewed and given tentative approval during the meeting (in some cases with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE is described in an associated output document JCT3V-E110x for CE n, where “n" is the CE number (n = 1, 2, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-3V is not obliged to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-3V members can participate in a CE

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

### Cross check

For cross checking the same steps as described in the section above should be carried out as soon as possible, but should not take more than five working days. Moreover it should be checked if integrated tools correspond to the adopted proposal. Results of cross check should be announced to the reflector. The result-sheet should be made available to the group.

### Procedure if cross check fails or planned delivery data cannot be held

If a planned delivery date cannot be held this should be announced to the reflector.

If the crosscheck fails or the previous integrator has not delivered the software within 3 days or result sheet within 5 days after planned delivery date

- the current integrator should integrate in the last cross-checked version

- the previous integrator falls back to the end of the integration plan

## Common Conditions for 3D Video Coding Experiments

Preferred Common Conditions for experiment testing that are intended to be appropriate for both CEs and other experiments were selected by the group and described in output document JCT3V-F1100.

## Software development (update)

ATM software:

Due to the absence of key developers of this software from the meeting, the remaining bug fixing has to be done and planned within the mandates of the AHG and software coordinator / software specification editing team. It has already been expressed by email that this will be done.

It was later mentioned in the closing plenary that the software may require further improvements, containing too many macros which are difficult to disable. Furthermore, a manual would be desirable.

Since the bug free software is necessary to finalize the conformance of the spec, and a standard without conformance shall not exist, action could be taken in longer term to remove 3D AVC from the AVC spec if this situation is not resolved.

HTM-11.0 Integration plan

Track 1: Merge, RP

Coordinator: Yulin Chang??

Starting day:

| Item | Integrator | Days |
| --- | --- | --- |
| JCT3V-H0077/JCT3V-H0099/JCT3V-H0111/JCT3V-H0133 Sub-PU MPI | Qualcomm?? |  |
| JCT3V-H0137 Control of the availability of advanced inter-view coding predictions (HLS) | Qualcomm |  |
| JCT3V-H0083 CE1 Bug-fix and clean up on merge list construction | ETRI |  |
| JCT3V-H0070 CE1: Results on Adaptive Disabling Inter-view Motion Vector Candidates | Samsung |  |
| JCT3V-H0062 CE1: Simplification of merge candidate construction | Sharp |  |
| JCT3V-H0205 Disallow bi-prediction in case of 4x8 and 8x4 sub PU sizes | ?? |  |
| JCT3V-H0085 AHG5: Complexity reduction of bi-prediction for RP | Mediatek |  |

Track 2: DBBP, DLT, others

Coordinator: Hongbin Liu??

Starting day:

| Item | Integrator | Days |
| --- | --- | --- |
| - JCT3V-H0057 Sub-PU Restriction for DBBP | RWTH |  |
| JCT3V-H0104 Partition boundary filtering in DBBP | Samsung |  |
| JCT3V-H0103 Vertical DV restriction after depth-based refinement | Samsung |  |
| JCT3V-H0105 Performance evaluation on DLT-based residual coding | Samsung |  |
| JCT3V-H0072 Bug-fix of depth-based block partitioning | Mediatek |  |
| - JCT3V-H0094 Improvement on the signaling of DBBP | Mediatek |  |
| JCT3V-H0091 On DLT signaling (HLS) | Mediatek |  |

Track 3: DMM SDC

Coordinator: Gerhard Tech.

Starting day:

| Item | Integrator | Days |
| --- | --- | --- |
| JCT3V-H0086 Low-latency illumination compensation (IC) encoding algorithm (Non CTC) | Mediatek |  |
| JCT3V-H0092 CE3: Results on simplified DMM mode coding | Mediatek |  |
| JCT3V-H0095 On SDC signaling (HLS) | Mediatek |  |
| JCT3V-H0084/JCT3V-H0100/JCT3V-H0113 CE3: depth_dc_flag | ?? |  |
| JCT3V-H0131 CE3: Delta DC coding for SDC and DMM modes | Qualcomm |  |
| JCT3V-H0119/JCT3V-H0135 CE3 Simplification of dim_not_present_flag coding | LGE |  |
| JCT3V-H0108 Clean-up on DMM and SDC DC prediction | HiSilicon |  |

It is to the discretion of the software coordinators to set up a time line for the integration and request proponents to finish integration by a given date.

Integration Procedure & Guidelines

Integration is done in a serial way. Each integrator cross-checks the version provided by his predecessor.  The cross check for the last version is carried out by the software coordinators.

Integration Guidelines

When integrating

- software changes should be enclosed by macros switchable by defines including company and proposal number e.g.

#define MYCOMPANYS_DEPTHFILTER_JCT3V_B0555  1

#if MYCOMPANYS_DEPTHFILTER_JCT3V_B0555

// do stuff

#endif

- new tools should be made switchable in the cfg-file if reasonable

- cfg-files should be updated

Delivery of software

Before delivering the software to the next integrator it should be checked if

- the software compiles under windows and linux

- software compiles and delivers same results as previous version when integrated tools are disabled by macro or cfg-settings

- there are encoder-decoder mismatches

- there are memory leaks by measuring maximum memory consumption (or specific tools e.g. valgrind) is

- visual quality is not disturbed

Additional to the software cfg-files that reflect proposed settings and an excel sheet with coding results should be provided. Software and cfg-files should be delivered by checking it in to the corresponding (HTM or ATM) software repositories.

When software is delivered this should be announced to the reflector. Moreover, every further change on the software should be announced. If there is a delay in integration this should be communicated to the reflector.

## Software repositories

### HTM

HTM software can be checked in

https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/

Therefore for each integrator an own software branch will be created by the software coordinator containing the current anchor version or the version of the previous integrator:

e.g.  branches/0.7-mycompany

The branch of the last integrator will become the new release candidate tag.

e.g. tags/0.8rc1

This tag can be cross-check by the group for. If no problems occur the release candidate will become the new tag after 7-days:

e.g. tags/0.8

If reasonable intermediate release candidate tags can be created by the software coordinator.

### ATM

An official release of the 3DV-ATM software can be checked out by mpeg3dv SVN users from the following location:

http://mpeg3dv.research.nokia.com/svn/mpeg3dv/trunk/

Following every MPEG 3DV meeting, software coordinator creates a new branch for the integration of adopted proposals:

http://mpeg3dv.research.nokia.com/svn/mpeg3dv/branches/

(Note: Would be desirable to rename “mpeg3dv” to “jct3v”)

Software integrator checks-out the software from integration branch at its turn of integration plan and integrates proposal as it is specified in Section 11.3.

Software integrator communicates a new software version over the email list to the cross-checker and to the software coordinator.

The software coordinator checks-in to the integration branch every new software integration with confirmed cross-check, as it is specified in Section 11.3.

Once the integration plan is completed, the software coordinator cross-check version available in the integration branch and checks-in a new official release of the 3DV-ATM to the http://mpeg3dv.research.nokia.com/svn/mpeg3dv/trunk/.

## List of CEs (update)

(chaired by A. Vetro)

The following CEs were planned, and initial versions of the descriptions were presented Thu PM.

- CE1 on Residual Prediction [H. Liu, T. Ikai]

  - Sub-block ARP (H0130)

  - Memory bandwidth reduction (H0063, H0132)

- CE2 on depth intra coding [Y. Chen, Y.W. Chen]

  - Adaptive boundary chain coding (H0121)

  - Single depth mode (H0087)

  - Residual DC quantization (H0102)

# Establishment of ad hoc groups (update)

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-3V reflector (jct-3v@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-3V project management (AHG1) (jct-3v@lists.rwth-aachen.de) - Coordinate overall JCT-3V interim efforts. - Report on project status to JCT-3V reflector. - Provide report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| MV-HEVC / 3D-HEVC Draft and Test Model editing (AHG2) (jct-3v@lists.rwth-aachen.de) - Produce and finalize JCT3V-H1001 3D-HEVC Draft Text 4. - Produce and finalize JCT3V-H1002 MV-HEVC Draft Text 8. - Produce and finalize JCT3V-H1003 3D-HEVC and MV-HEVC Test Model 8. - Gather and address comments for refinement of these documents. Coordinate with the 3D-HEVC Software Integration AhG to address issues relating to mismatches between software and text. | G. Tech, K. Wegner (co-chairs), J. Boyce, Y. Chen, M. Hannuksela, T. Suzuki, S. Yea, J.-R. Ohm, G. Sullivan (vice chairs) | N |
| 3D-AVC Software Integration (AHG3) (jct-3v@lists.rwth-aachen.de) - Coordinate the finalization of the 3DV-ATM software and its distribution to JCT-3V members - Identify issues (if any) where the software deviates from the standard or the test model JCT3V-G1003, and clean up the software if necessary - Produce documentation of software usage for distribution with the software - Prepare and deliver 3DV-ATM v12.0 software to address bug (expected by 30 April 2014). - Perform analysis and reconfirmation checks of the behaviour of technical changes adopted into the draft design, and report the results of such analysis. - Start an effort for software code clean up, particularly regarding the usage of macros. | D. Rusanovskyy (chair), F. C. Chen, J. Y. Lee, J.-L. Lin, O. Stankiewicz, T. Suzuki, D. Tian (vice chairs) | N |
| MV-HEVC / 3D-HEVC Software Integration (AHG4) (jct-3v@lists.rwth-aachen.de) - Coordinate development of the HTM software and its distribution to JCT-3V members - Produce documentation of software usage for distribution with the software - Prepare and deliver HTM-11.0 software version and the reference configuration encodings according to JCT3V-H1100 based on common conditions suitable for use in most core experiments (expected within four weeks after the meeting). - Prepare and deliver HTM-11.1 software that include additional items not integrated into the 11.0 version. - Perform analysis and reconfirmation checks of the behaviour of technical changes adopted into the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. Coordinate with MV-HEVC Draft and 3D-HEVC Test Model editing AhG to identify any mismatches between software and text. | G. Tech, H. Liu (co-chairs), Y. W. Chen, K. Wegner (vice chairs) | N |
| 3D Coding Tool Testing (AHG5) (jct-3v@lists.rwth-aachen.de) - Discuss and finalize CE work plans - Study the Common Test Conditions and suggest possible changes - Coordinate between core experiments when necessary. - Study coding tool performance of non-CTC configurations - Report on status of core experiments. - Prepare viewing logistics for 9th JCT-3V meeting. | K. Müller, A. Vetro, S. Yea (co-chairs) | N |
| 3D High level syntax (AHG6) (jct-3v@lists.rwth-aachen.de) - Identify opportunities for common approaches for multi-view and scalable high-level extensions of HEVC. - Study NAL unit header, video parameter set, sequence parameter set, picture parameter set, and slice header syntax designs. - Study SEI messages and VUI syntax designs needed for HEVC extensions. - Study mechanisms to indicate decoding capabilities/requirements for auxiliary picture layers - Investigate mechanisms for picture referencing across CRA pictures - Study and compare mechanisms for enabling temporal enhancement layers and diagonal inter-layer prediction - Study the operation of the Hypothetical Reference Decoder for multi-layer operation. - Assist in software development and text drafting for the high-level syntax in the HEVC extensions designs. Coordinate efforts with JCT-VC AHG9 on high-level syntax issues in relation to 3D extensions. | J. Boyce and M. M. Hannuksela (co-chairs), Y. Chen, S. Deshpande, J. Samuelsson, G. Tech, Y.-K. Wang, P. Wu (vice-chairs) | Y (Tuesday and Wednesday before the 9th JCT-3V meeting) + Tel |
| Complexity assessment (AHG7) (jct-3v@lists.rwth-aachen.de) - Study intrinsic complexity measures or metrics to assess hardware and software implementations of algorithms/computational models based on number of operations, potential for parallelism, data transfer rate, and data storage requirements. - Analyse existing designs and produce complexity perspective per tool, from the viewpoint of possible slowdown, bottlenecks or implementation issues. - Develop a recommendation to JCT-3V on formalized complexity assessment in a reporting document. | C. Lee (chair), G. Bang, T. Ikai, K. Rapaka (vice chairs) | N |
| 3D Test Material (AHG8) (jct-3v@lists.rwth-aachen.de) - Identify deficiencies and limitations of current 3D video test material. - Identify, collect, and make available a variety of additional 3D video sequences. - Study the characteristics of test materials and their impact on coding performance, synthesis quality etc. - Recommend appropriate test materials for use in 3D Video Coding Extension Development. - Assess the suitability of the test material that was offered to the 5th and 6th JCT-3V meetings,. - Assess additional stereo material for an upcoming MV-HEVC verification | S. Shimizu, S. Yea (co-chairs) | N |
| Conformance testing development (AHG9) (jct-3v@lists.rwth-aachen.de) - Further discuss and improve the conformance drafts related to 3D-AVC (JCT3V-H1004) - Collect the conformance test streams | T. Suzuki (chair), D. Rusanovskyy, D. Tian, Y. W. Chen (vice chairs) | N |
| Layered coding constraint specifications and capability indications (AHG 10) (jct-3v@lists.rwth-aachen.de) - Study how constraints on bitstream syntax affect specifications of bitstream conformance and decoder conformance for layered coding. - Study indications of bitstream properties and decoding process requirements for layered coding. - Identify potential side-effects of constraint specifications and capability indications and recommend alternative approaches. - Identify potential needs for modifications and additional indications of constraints and decoding process requirements for layered coding. - Coordinate with AHG 6 on high-level syntax for constraint specifications and capability indications for layered coding. | G. Sullivan (chair) | Tel. |

On Wednesday afternoon, it was discussed whether a new AHG on studying simplifications of tools in general would be beneficial. No interest was expressed in this regard.

# Output documents (update)

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate those responsible for document production.

(it was communicated by the chair that only the subsequent lists of adoptions is relevant, and in case of contradiction with any of meeting notes above the latter are considered invalid in first place)

JCT3V-H1000 Meeting Report of 8th JCT-3V Meeting [J.-R. Ohm, G. J. Sullivan]

(Note: Initial versions of the subsequent draft documents should be uploaded by the end of the meeting, continually updating to be performed until the final the version is released)

JCT3V-H1001 3D-HEVC Draft Text 4 [G. Tech, K. Wegner, Y. Chen, S. Yea] (Study Text of ISO/IEC 23008-2 PDAM4, WG 11 N14424) [2014-05-15]

Summary of Adoptions for 3D-HEVC

- JCT3V-H0070 Adaptive Disabling Inter-view Motion Vector Candidates (simplified version as per WD text in v2 of the contribution)

- JCT3V-H0062 Simplification of merge candidate construction

- JCT3V-H0205 Disallow bi-prediction in case of 4x8 and 8x4 sub PU sizes

- JCT3V-H00077/JCTVC-H0099/JCTVC-H0111/JCTVC-H0133 Sub-PU MPI (option 1)

- JCT3V-H0131 Delta DC coding for SDC and DMM modes (test 2)

- JCT3V-H0092 Simplified DMM mode coding

- JCT3V-H0119/JCT3V-H0135 Simplification of dim_not_present_flag coding

- JCT3V-H0095 SDC signaling (encoder restriction)

- JCT3V-H0091 DLT signaling (encoder restrictions)

- JCT3V-H0105 Remove DLT based residual coding

- JCT3V-H0204 Intra depth slice

- JCT3V-H0136 Constraints on DMM (bug fix)

- JCT3V-H0108 Clean-up on DMM and SDC DC prediction

- JCT3V-H0057 Sub-PU Restriction for DBBP

- JCT3V-H0072 Bug-fix of depth-based block partitioning (method 2)

- JCT3V-H0094 Improvement on the signaling of DBBP

- JCT3V-H0104 Partition boundary filtering in DBBP

- JCT3V-H0103 Vertical DV restriction after depth-based refinement

- JCT3V-H0137 Control of the availability of advanced inter-view coding predictions (second part)

- This list was presented during the closing plenary and no complaints were made.

JCT3V-H1002 MV-HEVC Draft Text 8 [G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce] (Preliminary Draft Text of ISO/IEC 23008-2 FDAM2, WG 11 N14423) [2014-05-31]

Finalization planned for 14-07

Summary of Adoptions for MV-HEVC

(table provided by Gerhard)

Item

Q0081 Decision (Ed.): Delegated to editors to clarify, as necessary, that the alpha channel content persists until cancelled or updated in output order.

Q0081 Decision: Remove the constraint that an alpha picture must be accompanied by a primary picture.

HLS for hybrid scalability (Provided with JCT-VC AhG 15 report) Decision: The proposed text was endorsed, with non-editorial open issues considered as follows: - Regarding whether, in F.7.3, to send sub_layers_vps_max_minus1[ i ] and max_tid_il_ref_pics_plus1[ i ][ j ] for the externally-provided base layer, it was agreed that these should be sent. M. M. Hannuksela agreed to provide the semantics. - Regarding the question of "Further study whether all the six specified types of IRAP NAL unit types should be defined instead of just three, and whether codec specific definitions, e.g. when the base layer is AVC, are needed, such as defined in P0184v2_attachment", no action appeared necessary. - Would the addition of the support of external-means-provided base layer affect the level definitions? This aspect to be resolved by review of H0043/Q0145. It was suggested that we may not necessarily define separate profiles for support of external-means-provided base layer, since we do not have separate profiles for other type of external means signalling. If this is agreed, then the addition of the hybrid profiles should be removed and the added restriction on vps_base_layer_internal_flag for the non-hybrid profiles should also be removed. Pending an identification of a need to do otherwise, we will assume there is no need to specify the hybrid case as a separate profile. Regarding whether a separate profile is needed for the case where the base layer is provided by external means, there was discussion about that. Further study was encouraged. There should be a constraint on the size of the pictures provided by external means and of their representation format. For MV-HEVC, don't remove the restriction to prohibit the external base layer (at least for now). Otherwise, the MV-HEVC draft will include the same specification for hybrid operation as for SHVC.

H0032/Q0101 .Decision (BF/Cleanup): Adopt (sub-proposals 1–11, refined as described).

H0082/Q0182 Sub-proposal 1: Allow the vps_vui_bsp_hrd_parameters( ) syntax structure and the bitstream partition HRD parameters SEI message to be present. Specify that when both structures are present, their contents shall be semantically identical. The following text is proposed: "When both the bitstream partition HRD parameters SEI message and the vps_vui_bsp_hrd_parameters( ) syntax structure in the active VPS are present, the contents of the SEI message shall be semantically identical to the contents of the vps_vui_bsp_hrd_parameters( ) syntax structure of the active VPS." Decision (BF/Cleanup/Ed): Adopted (such that we use the main proposal for sub-proposal 1.

H0082/Q0182 Sub-proposal 2 Alternative 1: Clarify that the VPS hrd_parameters( ) syntax structure that applies to the layer set which is associated with the bitstream partition initial arrival time SEI message is used to determine the lengths of the nal_initial_arrival_delay[ i ] and vcl_initial_arrival_delay[ i ] syntax elements. Decision (BF/Cleanup/Ed): Adopted alternative 1 for sub-proposal 2.

H0033/Q0102 Sub-proposal 1: A modification to the derivation of number of sub-DPBs and assignment of sub-DPBs to each layer considering separate_colour_plane_vps_flag is proposed. Decision (BF/Cleanup/Ed.): Adopt. (The need for sub-proposal 1 is conditioned on the specification of a shared sub-DPB.)

H0033/Q0102 Sub-proposal 2: A semantics bug fix is proposed for sps_max_dec_pic_buffering_minus1 as a bug-fix. In discussion, the first option was preferred. Decision (BF/Cleanup/Ed.): Adopt. .

H0033/Q0102 It was suggested that also the separate_colour_plane_flag should affect inference of NoOutputOfPriorPicsFlag. Decision (Ed.): Agreed (affects RExt text).

H0045/Q0154 On picture flushing and DPB parameters Decision: Adopted (some details to be discussed further in BoG).

H0021/Q0045 SEI message on selectable overlays Decision: Adopt (modified for variable-length strings).

H0036/Q0108 Proposal #1: Remove restriction on alignment of TSA and STSA with dependent layers

H0036/Q0108 Proposal #2: Allow STSA pics of non-base layers to have TemporalId equal to 0

H0036/Q0108 Proposal #2: Change range of values for HighestTid in Annex C to reduce number of encoder conformance tests

H0042/Q0146 Proposal 1.1: Change inference rule in semantics of poc_msb_val_present_flag

H0042/Q0146 Proposal 1.4, Add an inference value of 0 when poc_reset_info_present_flag is not present

H0024/Q0060 Proposal 2: Add a condition to the derivation of refLayerPicIdc of (TemporalId == 0)

H0053/Q0165 proposal 1.a), add a constraint to disallow an empty output layer set

H0053/Q0165 proposal 2. change condition for presence of num_add_output_layer_sets to avoid sending it when there is only one layer set.

H0168/Q0195 proposal 1: restructure syntax table for sending of rep_format_idx_present_flag and vps_num_ref_formats_minus1

H0168/Q0195 proposal 2: add a semantic constraint on the value of update_ref_format_flag

H0056/Q0177 proposal 1: modify inference rule for sps_temporal_id_nesting_flag when it is not present

H0082/Q0183, numerous small clean-ups on SEI messages

H0026/Q0078, Items 3 b,c and 4, clarifying which pictures in an output layer sets are applied the values of general_progressive_source_flag, general_interlaced_source_flag, general_non_packed_constraint_flag and general_frame_only_constraint_flag.

H0023/Q0055 Several editorial issues regarding HRD operation from Q0055. Review text modifications with contribution author.

H0028/Q0091Editorial improvements are needed for the association of NAL units within an access unit and access unit boundary detection. Language in 0091 may be useful to study.

H0042/Q0146, proposal 1.2, consider adding a NOTE and check if usage of HandleCraAsBlaFlag and EOS behavior is clearly described, and proposal 1.3 specification bug fix

H0022/Q0054 proposal #2, bug fix in the semantics of vps_profile_present_flag[ ]

Q0116, consider adding a note regarding how random accessibility is affected by the recovery point SEI message

H0033/Q0102 proposal #3, change the max loop index for signaling bit rate and pic rate info to MaxSubLayersInLayerSetMinus1

Add a profile constraint to the Scalable Main, Scalable Main 10, and Stereo Main profiles against allowing layers with duplicate values of DependencyId (or ViewOrderIdx) when AuxId equal to 0.

H0044/Q0142 v2: Add semantic constraints to poc_lsb_not_present_flag.

H0048/Q0160 v2: Add constraint to stereo main profile that it must contain exactly two texture views, and add a note to state that the constraint implies a restriction that alt_output_flag equal to 0.

H0138/Q0189 Proposal 2.2, clarification of semantics of slice temporal mvp enabled flag

H0138/Q0189 Proposal 2.3, SEI message for indicating constraints on TMVP

H0031/Q0100 v3, add constraint to RPS semantics

H0208/Q0253, modified semantics of layers not present SEI message to correct bug introduced during editing

H0045/Q0154 Proposal in C.5.2.1: Add in the decoding process that when a new VPS is activated, all pictures in the DPB are marked as unused for reference

H0026/Q0078: mechanism for signaling a profile/tier/level conformance point for auxiliary pictures, including the aspects listed below. Revisit in track the contribution’s requirement for including the additional VPS, in the VPS rewriting SEI message, in all cases where auxiliary pictures are used. Clarification needed of an earlier track decision: “if an aux picture is part of an output layer set but is not part of a target output layer, the buffer flow includes the aux picture but the decoding process requirements do not.” - signaling of additional layer sets - description of how profile_tier_level applies - rewriting process - VPS rewriting SEI message - output layer set nesting SEI message Regarding the H0026/Q0078 requirement to send an additional VPS in an SEI message (to be used by the rewritten bitstream). It was agreed that the output bitstream of the rewriting process would be allowed to produce a bitstream that is otherwise conforming but does not contain a VPS for the case of single-layer rewritten bitstreams. Further study to determine whether this solution is fully adequate was encouraged. To clarify the decision on not including requirement for decoding process for aux pictures, it is only an editorial matter whether something is described in the body and prohibited in all profiles, or prohibited in general – the latter is probably better (until a future version needs to allow it in some profile).

Clarify that an auxiliary picture can be associated with more than one primary picture. Consider if the language associating an alpha auxiliary picture with a primary picture in the semantics of dimension_id[ ][ ] near the AuxId derivation could be moved to the alpha SEI message.

Q0142, consider adding a note to clarify that if IDR alignment is indicated, the constraint on poc_lsb_not_present[ ] does not apply.

Remove sub-DPB sharing and processes that mark inter-layer reference pictures as "unused for reference"

Normative rewriting process to be defined, so that independent non-base layers can be rewritten to be decodable by non-layered profiles (i.e.,. Main, Main 10)

Auxiliary picture concepts: - Profile indication in the SPS of an aux pic is the profile that would apply after rewriting. - When an output layer set that doesn't include the base layer, the bitstream is rewritten and the same output layer set is required to be present for the rewritten bitstream, which contains a base layer - When an aux picture is part of an output layer set but is not part of a target output layer, the buffer flow includes the aux picture but the decoding process requirements do not. - An auxiliary picture can be associated with more than one primary picture. - More than one auxiliary picture of a particular type may be associated with the same primary picture

H0126 Phrasing used in specifying the Stereo Main profile.

This table was presented during the closing plenary and no complaints were made.

- See notes in section 6.2 for more details (TBD)

- JCTVC-Qxxxx/JCT3V-Hxxxx …

- …

- In the closing plenary, it is furthermore agreed that several SEI messages which are relevant for stereo seem to be missing from the MV-HEVC spec, e.g. view position ID, camera parameters. It is to the discretion of the editors to copy such SEI messages from the AVC spec.

JCT3V-H1003 Test Model 8 of 3D-HEVC and MV-HEVC [Y. Chen, G. Tech, K. Wegner, S. Yea] (WG 11 N14425) [2014-05-15]

It is planned to structure this document into 2 parts (Test models for MV-HEVC and 3D-HEVC) later.

Bug fixes:

- JCT3V-H0085 Complexity reduction of bi-prediction for advanced residual prediction

- JCT3V-H0084/JCT3V-H0100/JCT3V-H0113 Depth DC flag

Software:

- JCT3V-H0083 Bug-fix and clean up on merge list construction

- JCT3V-H0086 Low-latency illumination compensation (IC) encoding algorithm (non CTC)

- JCT3V-H0091 DLT signaling

JCT3V-H1004 Draft 5 of 3D-AVC Conformance [D. Rusanovskyy, T. Suzuki, D. Tian, Y. W. Chen] (ISO/IEC 14496-4:2004 DAM43, WG 11 N14383) [2014-04-18]

JCT3V-H1005 Draft 3 of 3D-AVC Reference Software [D. Rusanovskyy, D. Tian] (ISO/IEC 14496-5:2001 DAM35, WG 11 N14385) [2014-04-18]

JCT3V-H1006 Draft 4 of MVC+D Reference Software [D. Rusanovskyy, T. Suzuki, D. Tian] [2014-04-18]

JCT3V-G1100 Common Test Conditions of 3DV Core Experiments remains valid

JCT3V-H1101 Description of Core Experiment 1 (CE1) on Residual Prediction [H. Liu, T. Ikai]

JCT3V-H1102 Description of Core Experiment 2 (CE2) on Depth Intra Coding [Y. Chen, Y.-W. Chen]

# Future meeting plans, expressions of thanks, and closing of the meeting

During the closing plenary, it was discussed that the production of conformance streams for MV-HEVC (relating to Stereo Main Profile) is necessary, and whether it could be bundled with SHVC conformance. To be discussed with JCT-VC during the next meeting.

It is also discussed that verification testing for the performance of the Stereo Main Profile is necessary, which could be organized similarly as the previous 3D-AVC testing.

The document upload deadline for the 10th meeting of the JCT-3V will be Oct XX, 2014, 2359 MET (Geneva/Paris time zone). Documents that are intended to be discussed in the joint meeting of JCT-VC and JCT-3V AHG6 for HLS (starting from Oct XX) shall be registered as joint contributions, the deadline of the 19th JCT-VC meeting is to be applied, i.e. Oct. XX, 2014, 2359 MET (Geneva/Paris time zone).

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Wednesday or Thursday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting);

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Saturday prior to such meetings and closing it on the last day of the WG 11 meeting);

- In cases where JCT-3V meets during the first week of the SG16 meeting under ITU-T auspices, and co-located with an MPEG meeting at a nearby meeting place, the meeting dates could also be approximately aligned with the MPEG meeting.

Some specific future meeting plans were established as follows:

- 18-24 Oct. 2014 under WG 11 auspices in Strasbourg, FR

- 12-18 Feb. 2015 under ITU-T auspices in Geneva, CH

- 20-26 June 2015 under WG 11 auspices in Warsaw, PL

- XX-XX Oct. 2015 under ITU-T auspices in XXXX, XX

Note: Depending on expectations about future work load, shorter meeting durations may be considered starting from the 10th meeting.

XXX were thanked for the excellent hosting of the 9th meeting of the JCT-3V.

It was also reminded that final output documents (if also registered under a WG11 output doc number) have to be uploaded separately with a WG11 header.  To do this in a timely fashion is particularly important for standards submitted for ballot.

It was also reminded that slide presentation decks should be made available.

The JCT-3V meeting was closed at approximately XXXX hours on Wednesday 9 July 2014.

# Annex A to JCT-3V report:
List of documents

# Annex B to JCT-3V report:
List of meeting participants

The participants of the fourth meeting of the JCT-3V, according to a sign-in sheet passed around during the meeting (approximately XX in total), were as follows:

- …