| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 |  |  |  | Document: JCT3V-I_Notes_dA |
| --- | --- | --- | --- | --- |
| Title: | Meeting report of the ninth meeting of the Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V), Sapporo, JP, 3 – 9 July 2014 |  |  |  |
| Status: | Report document from chairs of JCT-3V |  |  |  |
| Purpose: | Report |  |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communications Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |  |
| Source: | Chairs |  |  |  |

_____________________________

- Summary

The Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its ninth meeting during 3 – 9 July 2014 at the Sapporo Convention Center, Sapporo, JP. The JCT-3V meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany) and Dr Gary Sullivan (Microsoft/USA). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.12 of this document.

The meeting was mainly held in a “single track” fashion, with few breakout activities (as documented in this report) held in parallel. All decisions were made in plenaries when one of the chairs was present.

The JCT-3V meeting sessions began at approximately 900 hours on Thursday 3 July 2014. Meeting sessions were held on all days until the meeting was closed at approximately 1320 hours on Wednesday 9 July 2014. Approximately 118 people attended the JCT-3V meeting, and approximately 200 input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JCT-3V. The subject matter of the JCT-3V meeting activities consisted of work on 3D extensions of the Advanced Video Coding (AVC) and the High Efficiency Video Coding (HEVC) standards.

The AHG on 3D High-level Syntax (AHG7) had met the 2 days before the JCT-3V meeting (1-2 July), to discuss HEVC HLS input contributions jointly with JCT-VC experts. The outcome of this joint meeting, which continued after the beginning of the JCT-3V meeting, is documented in the JCT-VC report JCTVC-R1000.

The primary goals of the meeting were to review the work that was performed in the interim period since the seventh JCT-3V meeting in producing

- Draft 3 of 3D-AVC reference software (ISO/IEC 14496-5:2001/DAM 35 – for ballot)

- Draft 4 of MVC+D reference software (bug fixing before progressing to FDAM – not for ballot)

- Draft Text 8 of MV-HEVC (Preliminary Draft of ISO/IEC 23008-2:200X/FDAM 2 – not for ballot)

- Test Model 8 of 3D-HEVC and  MV-HEVC, and associated software

- Draft 4 of 3D-HEVC text specification (ISO/IEC 23008-2:2013/PDAM4 Study – not for ballot)

- Draft 5 of 3D-AVC Conformance (ISO/IEC 14496-4:2004/DAM 43 – for ballot)

- Common Test Conditions of 3DV Core Experiments and CE Descriptions.

Furthermore, the JCT-3V reviewed the results from two interim Core Experiments (CE); reviewed technical input documents; produced updated versions of the draft texts, framework descriptions and software implementations of the items above.

The JCT-3V produced 10 particularly important output documents from the meeting:

- 3D-HEVC Draft Text 5 (Text of ISO/IEC 23008-2 DAM4 –for ballot)

- MV-HEVC Draft Text 9 (to be integrated to ISO/IEC 23008-2:201X – for ballot)

- Test Model 9 of 3D-HEVC and  MV-HEVC, and associated software

- Draft 4 of 3D-AVC Reference Software (Study of ISO/IEC 14496-5:2001 DAM35, not for ballot)

- Draft 5 of MVC+D Reference Software (ISO/IEC 14496-5:2001 FDAM33 – for ballot)

- Draft 1 of MFC plus Depth and Texture/Depth View Packing SEI Message (ISO/IEC 14496-10:201X PDAM1 – for ballot)

- MV-HEVC Conformance Draft 1

- MV-HEVC Software Draft 1

- 3D-HEVC Conformance Draft 1

- MV-HEVC Draft Verification Test Plan v1

Moreover, plans were established to conduct 1 future CE in the interim period until the next meeting.

For the organization and planning of its future work, the JCT-3V established 10 "Ad Hoc Groups" (AHGs) to progress the work on particular subject areas. The next four JCT-3V meetings are planned for 18 – 24 Oct. 2014 under WG 11 auspices in Strasbourg, FR, during 12 – 18 Feb. 2015 under ITU-T auspices in Geneva, CH, during 20 – 26 June 2015 under WG 11 auspices in Warsaw, PL, and during 15 – 21 Oct. 2015 under ITU-T auspices in XXXX, XX.

The document distribution site http://phenix.it-sudparis.eu/jct3v/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-3V and all of its AHGs is jct-3v@lists.rwth-aachen.de.

- Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-3V are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

## Meeting logistics

The JCT-3V meeting sessions began at approximately 900 hours on Thursday 3 July 2014. Meeting sessions were held on all days until the meeting was closed at approximately 1320 hours on Wednesday 9 July 2014. Approximately 118 people attended the JCT-3V meeting, and approximately 200 input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JCT-3V. The subject matter of the JCT-3V meeting activities consisted of work on 3D extensions of the Advanced Video Coding (AVC) and the High Efficiency Video Coding (HEVC) standards.

The AHG on 3D High-level Syntax (AHG6) and Layered Coding Constraint Specifications (AHG10) had met the 2 days before the JCT-3V meeting (1-2 July), to discuss HEVC HLS input contributions jointly with JCT-VC experts.

Information regarding preparation and logistics arrangements for the meeting had been provided via the email reflector jct-3v@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jct3v-site/2014_07_I_Sapporo/.

## Documents and document handling considerations

### General

The documents of the JCT-3V meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct3v/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

Document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (rather than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a bug in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding … simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of contexts, number of context-coded bins, etc.) … "Decision (Compl.):"

This meeting report is based primarily on notes taken by the chairs and projected (if possible) for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp (http://wftp3.itu.int/av-arch/jct3v-site/) during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much discussion of the contributions and discussions as is feasible in the interest of aiding study, although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Thursday, 26 June 2014. For documents on HEVC high level syntax, if submitted as joint contributions to JCT-VC and JCT-3V, the deadline of Friday, 20 June 2014 had been announced.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Friday 27 June 2014 were considered "officially late". For HLS documents, a similar consideration applies with the date of 21 June, 2359 hours.

Most documents in the "late" category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All regular contribution documents with registration numbers JCT3V-I0150 and higher, as well as common SHVC/MV-HEVC HLS documents with registration numbers JCT3V-I0059 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). Some documents in the "I0150+" range include break-out activity reports that were generated during the meeting and are therefore considered report documents rather than late contributions.

Generally, the late document submission situation had improved relative to previous history.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with this issue was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical proposal contributions were classified as late either due to late upload or late registration:

- JCT3V-I0033 (a technical proposal on hybrid codec layers) [uploaded 06-22]

- JCT3V-I0034 (a technical proposal on layer sets) [uploaded 06-23]

- JCT3V-I0047 (a technical proposal on bitstream partitions) [uploaded 06-22]

- JCT3V-I0059 (a technical proposal on SEI message persistence) [uploaded 06-23]

- JCT3V-I0060 (a technical proposal on overlay SEI message) [uploaded 06-23]

- JCT3V-I0062 (a technical proposal on layered conformance cleanups) [uploaded 06-25]

- JCT3V-I0064 (a technical proposal on active parameter sets SEI message) [uploaded 06-25]

- JCT3V-I0067 (a technical proposal on sub bitstream extraction) [uploaded 06-25]

- JCT3V-I0091 (a technical proposal on bitstream conformance restrictions) [uploaded 06-26]

- JCT3V-I0092 (a technical proposal on overlay SEI message) [uploaded 06-26]

- JCT3V-I0175 (a technical proposal on simplifying DMM) [uploaded 07-01]

- JCT3V-I0176 (a technical proposal on quantization of residual depth) [uploaded 07-01]

- JCT3V-I0191 (a technical proposal on 2Nx2N restriction) [uploaded 07-03]

- JCT3V-I0192 (a technical proposal on DBBP signaling improvement) [uploaded 07-03]

- JCT3V-I0200 (a technical proposal on HLS syntax cleanups) [uploaded 07-08]

- JCT3V-I0203 (a technical proposal on VUI semantics cleanups) [uploaded 07-09]

- JCT3V-I0204 (a technical proposal on repetition of parameter sets) [uploaded 07-09]

The following other documents not proposing normative technical content were classified as late (this did not apply in the current meeting):

- JCT3V-I0XXX (an input on …) [uploaded XX-XX]

The following cross-verification reports were classified as late either due to late upload or late registration: JCT3V-I0071, JCT3V-I0133, JCT3V-I0135, JCT3V-I0137, JCT3V-I0138, JCT3V-I0139, JCT3V-I0140, JCT3V-I0141, JCT3V-I0142, JCT3V-I0143, JCT3V-I0144, JCT3V-I0147, JCT3V-I0148, JCT3V-I0149, JCT3V-I0150, JCT3V-I0151, JCT3V-I0152, JCT3V-I0153, JCT3V-I0154, JCT3V-I0155, JCT3V-I0156, JCT3V-I0157, JCT3V-I0158, JCT3V-I0159, JCT3V-I0160, JCT3V-I0161, JCT3V-I0163, JCT3V-I0164, JCT3V-I0165, JCT3V-I0166, JCT3V-I0167, JCT3V-I0168, JCT3V-I0169, JCT3V-I0170, JCT3V-I0171, JCT3V-I0172, JCT3V-I0173, JCT3V-I0174, JCT3V-I0178, JCT3V-I0179, JCT3V-I0180, JCT3V-I0181, JCT3V-I0182, JCT3V-I0183, JCT3V-I0184, JCT3V-I0185, JCT3V-I0186, JCT3V-I0187, JCT3V-I0188, JCT3V-I0189, JCT3V-I0193, JCT3V-I0197, JCT3V-I0198.

The following document registrations were later cancelled or otherwise never provided or never discussed due to lack of availability or registration errors: JCT3V-I0121, JCT3V-I0128, JCT3V-I0162, JCT3V-I0177, JCT3V-I0202.

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were rejected in the document management system, as has been agreed since the third meeting.

(the following cases did not happen in the current meeting)

The initial uploads of the following contribution documents were rejected as "placeholders" without any significant content and were not corrected until after the upload deadline:

- JCT3V-I0XXX (a contribution of ... , corrected ...)

- ...

A few documents had some problems relating to IPR declarations (missing or excess declarations of contributing companies), inconsistent filenames in the header etc. in the initial uploaded versions. These issues were corrected by later uploaded versions in all cases (to the extent of the awareness of the chairs). In case of JCTVC-I0XXX and ..., the first complete versions appeared only on 06-XX, such that this contribution was also flagged as “late”.

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Measures to facilitate the consideration of contributions

For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by the cross-check contributor.

### Outputs of the preceding meeting

The report documents of the previous meeting, particularly the meeting report (JCT3V-H1000), the 3D-HEVC draft text 4 (JCT3V-H1001), the multi-view MV-HEVC draft text 8 (JCT3V-H1002), the MV-/3D-HEVC test model 8 (JCT3V-H1003), Draft 5 of 3D-AVC Conformance (JCT3V-H1004), Draft 3 of 3D-AVC Reference Software (JCT3V-H1005), and Draft 4 of MVC+D Reference Software (JCT3V-H1006), which had been produced in the interim period, were approved. The ATM and HTM reference software packages produced by AHG3 and AHG4 on software development, and the software technical evaluations were also approved.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

## Attendance

The list of participants in the JCT-3V meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/ SC 29/ WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on 3D video coding projects guidance

- Consideration of 3D video coding technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-3V and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-3V as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jct3v-site (JCT-3V contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-3v/index.html (JCT-3V general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/ SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that it is our understanding according to the practices of the parent bodies to make reference software available under copyright license header language which is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as e.g. recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/ SC 29/ WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software header is currently used in the process of designing the new HEVC standard and for evaluating proposals for technology to be included in this design. Additionally, after development of the coding technology, the software will be published by ITU-T and ISO/IEC as an example implementation of the 3D video standard(s) and for use as the basis of products to promote adoption of the technology. This is likely to require further communication with and between the parent organizations.

The ATM, HTM and MFC software packages that are used in JCT-3V follow these principles. The view synthesis software used for non-normative post processing is included in the HTM package and also has the BSD header.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct3v/. Furthermore, the site http://ftp3.itu.int/av-arch/jct3v-site was used for distribution of the contribution document template and circulation of drafts of this meeting report.

Communication of JCT-3V is performed via the list jct-3v@lists.rwth-aachen.de (to subscribe or unsubscribe, go to http://mailman.rwth-aachen.de/mailman/listinfo/jct-3v).

It was emphasized that reflector subscriptions and email sent to the reflector must use their real names when subscribing and sending messages and must respond to inquiries regarding their type of interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Note: Acronyms should be used consistently. For example, “IV” is sometimes used for “inter-view” and sometimes for “intra-view”.

Some terminology used in this report is explained below:

- AHG: Ad hoc group.

- AMVP: Advanced motion vector prediction.

- ARP: Advanced residual prediction.

- ATM: AVC based 3D test model

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BoG: Break-out group.

- BR: Bit rate.

- B-VSP: Backward view synthesis prediction.

- CABAC: Context-adaptive binary arithmetic coding.

- CD: Committee draft – the first formal ballot stage of the approval process in ISO/IEC.

- CE: Core experiment – a coordinated experiment conducted between two subsequent JCT-3V meetings and approved to be considered a CE by the group.

- Consent: A step taken in ITU-T to formally consider a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CPB: Coded picture buffer.

- CTC: Common test conditions.

- DBBP: Depth based block partitioning.

- DC: Disparity compensation

- DDD: Disparity derived depth (which uses the motion disparity vector to reconstruct a certain block (PU) of the depth map)

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC.

- DF: Deblocking filter.

- DLT: Depth lookup table.

- DMC: Depth based motion competition.

- DMM: Depth modeling mode.

- DPB: Decoded picture buffer.

- DRPS: Depth-range parameter set.

- DRWP: Depth-range based weighted prediction.

- DT: Decoding time.

- DV: Disparity vector

- ET: Encoding time.

- HEVC: High Efficiency Video Coding – the video coding standardization initiative under way in the JCT-VC.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- HRD: Hypothetical reference decoder.

- HTM: HEVC based 3D test model

- IC: Illumination compensation

- IDV: Implicit disparity vector

- IVMP: Inside-view motion prediction (which means motion for depth component is inherited from texture component motion)

- IVRC: Inter-view residual prediction.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/ SC 29, one of the two parent bodies of the JCT-3V).

- MPI: Motion parameter inheritance.

- MV: Motion vector.

- NAL: Network abstraction layer (HEVC/AVC).

- NBDV: Neighbored block disparity vector (used to derive unavailable depth data from reference view’s depth map) and DoNBDV = depth oriented NBDV

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NUT: NAL unit type (HEVC/AVC).

- PDM: Predicted Depth Map

- POC: Picture order count.

- PPS: Picture parameter set (HEVC/AVC).

- QP: Quantization parameter (as in AVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD). Often loosely associated with HE.

- RAP: Random access picture.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- REXT: Range extensions (of HEVC).

- RPS: Reference picture set.

- RQT: Residual quadtree.

- SAO: Sample-adaptive offset.

- SEI: Supplemental enhancement information (as in AVC).

- SD: Slice data; alternatively, standard-definition.

- SDC: Segment-wise DC coding.

- SH: Slice header.

- SHVC: Scalable HEVC.

- SPS: Sequence parameter set (HEVC/AVC).

- TSA: Temporal sublayer access.

- Unit types: Needs change

  - CTB: code tree block (synonymous with LCU or TB).

  - CU: coding unit.

  - LCU: (formerly LCTU) largest coding unit (synonymous with CTB or TB).

  - PU: prediction unit, with four shape possibilities.

    - 2Nx2N: having the full width and height of the CU.

    - 2NxN: having two areas that each have the full width and half the height of the CU.

    - Nx2N: having two areas that each have half the width and the full height of the CU.

    - NxN: having four areas that each have half the width and half the height of the CU.

  - TB: tree block (synonymous with LCU – LCU seems preferred).

  - TU: transform unit.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-3V).

- VPS: Video parameter set.

- VS: View synthesis.

- VSO: View synthesis optimization (RDO tool for depth maps).

- VSP: View synthesis prediction.

- WD: Working draft – the draft HEVC standard corresponding to the HM.

- WG: Working group (usually used in reference to WG 11, a.k.a. MPEG).

## Liaison activity

The JCT-3V did not send or receive formal liaison communications at this meeting.

## Opening remarks

Software and conformance for MVC+D, 3D-AVC and MFC should be progressed to a new edition in the ITU version of the spec.

Karsten Sühring coordinates the software package, Teruhiko Suzuki the conformance.

Documents to be approved by SG16 need to be delivered 07-09 evening at latest

New edition HEVC including MV-HEVC

It was also suggested to plan amending conformance and reference software for MV-HEVC

plan verification tests for MV-HEVC, and possibly conformance for 3D-HEVC.

PDAM ballot for 3D-HEVC closed, whereas DAM is planned for Oct. – need to consider ballot comments – Gerhard will report on Friday

Software development for 3D AVC: DAM delivered, but software still needs bug fixing

3D-AVC Conformance not yet delivered (w14383, JCT3V-H1004, ISO/IEC 14496-4:2004/DAM43)

New amendment for AVC suggested by parent bodies: MFC plus depth, SEI message

MFC conformance (ISO/IEC 14496-4:2004/DAM 42) Summary of Voting in m33645 – no negative votes – only comment was from U.S. with a Yes vote – encouraging unspecified general improvement if possible. We can proceed directly to publication (issuing a DoCR to reply to the comments, saying that no needs for action were identified). (P. Yin prepares DoC)

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows.

- AHG reports (section ) (10)

- Project development and status (section 3) (7)

- CE1 and related : Residual prediction (section 4.1) (29)

- CE2 and related: Depth intra coding (section Error: Reference source not found) (17)

- 3DV standards development (incl. software, conformance) (section 5) (6)

- High-Level Syntax (section 6) (42)

- Non-CE technical contributions (section 7) (60)

- Non-normative Contributions (section 8) (0)

- Withdrawn, unclear allocation (section 9) (3)

NOTE – The number of contributions noted in each category, as shown in parenthesis above, may not be 100% precise.

## Scheduling planning

Scheduling: Generally meeting time was scheduled during 0900 – 2000, with coffee and lunch breaks as convenient. Ongoing refinements were announced on the group email reflector as needed.

Some particular scheduling notes are shown below, although not necessarily 100% accurate:

- Thursday, first day

  - 0900-1100: Opening and AHG report review

  - 1130-13:00 and 1400-XX CE review

  - Joint BoG on HLS in parallel

- Friday, second day

  - 900-1030 New AVC amendment, DoCs (MV-HEVC, MFC SW&Conf),

  - 1030-1300 Reviewing technical inputs 3D-HEVC (7.2, 7.3)

  - 1400-1430 Software 3D-AVC

  - 1430- Reviewing technical inputs 3D-HEVC (7.2, 7.3, 7.4, 7.5, 7.7)

  - + 3D HEVC performance (3.4)

- Saturday, third day

  - 930-1300 BoG on DBBP (–Sehoon Yea)

  - 1400-1600 Decide technical inputs DBBP (7.6) (Gary)

  - 1600-1800 BoG merge list (Shimizu)

  - 1800- Joint meeting on HLS for MV-HEVC/SHVC

- Sunday, fourth day

  - Remaining documents

  - 3D HEVC HLS

  - Conformance +

- Monday, fifth day

  - Morning: MPEG plenary, Breakouts etc.

  - 15:00 Merge BoG review

  - 16:00-18:15 Joint Meeting with parent bodies

  - More revisits, planning, breakout review

- Tuesday, sixth day

  - Revisits (AVC SEI message) & other

  - verification test MV-HEVC

  - Planning CEs, AHGs, output docs

  - 11:00-13:00 Joint Meeting with parent bodies (Middle Hall B)

  - 1400-1800 FTV workshop

  - 1600-1800 BoG 3D HEVC conformance

  - 18:00 Software workplan (Gerhard), viewing of new test sequences (Shimizu) in room 203

- Wednesday, seventh day

  - 930-1400 (or shorter) final plenary, any remaining business

# AHG reports (10)

The activities of ad hoc groups that had been established at the prior meeting are discussed in this section.

JCT3V-I0001 JCT-3V AHG Report: JCT-3V project management (AHG1) [J.-R. Ohm, G. J. Sullivan]

The work of the JCT-3V overall has proceeded well in the interim period. A large amount of discussion was carried out on the group email reflector. All documents from the preceding meeting had been made available at the document site (http://phenix.it-sudparis.eu/jct3v/) or the ITU-based JCT-3V site (http://wftp3.itu.int/av-arch/jct3v-site/2014_07_I_Sapporo/), particularly including the following:

- The meeting report (JCT3V-H1000)

- Draft 4 of 3D-HEVC text specification (JCT3V-H1001) (submitted for ISO/IEC PDAM ballot)

- Draft 8 of MV-HEVC text specification (JCT3V-H1002)

- Test Model 8 of 3D-HEVC and  MV-HEVC (JCT3V-H1003), and associated software

- Draft 5 of 3D-AVC Conformance (JCT3V-H1004) (was planned to be submitted for ISO/IEC DAM ballot, but has not been delivered yet)

- Draft 3 of 3D-AVC reference software (JCT3V-H1005) (submitted as ISO/IEC DAM)

- Draft 4 of MVC+D reference software (JCT3V-H1006)

- Finalized core experiment descriptions (JCT3V-H1101 and JCT3V-H1102).

The 10 ad hoc groups and 2 core experiments had made progress, and various reports from those activities had been submitted.

The software for ATM version 12, and HTM version 11 had been prepared and released approximately as scheduled.

A "bug tracking" systems for software and text specifications had been installed. The sites are https://hevc.hhi.fraunhofer.de/trac/3d-hevc and https://hevc.hhi.fraunhofer.de/trac/3d-avc/. The bug tracker reports were automatically forwarded to the group email reflector, where the issues could be further discussed.

More than 180 input contributions to the current meeting had been registered. Late-registered and late-uploaded contributions were mostly cross-checks.

The meeting announcement had been made available from the aforementioned document site and http://wftp3.itu.int/av-arch/jct3v-site/2014_07_I_Sapporo/JCT3V-I_Logistics.doc. A joint meeting of AHG7 (high level syntax) with JCT-VC on topics of HEVC high-level syntax, two days prior to the current JCT-3V meeting, was also organized.

A preliminary basis for the document subject allocation and meeting notes had been circulated to the participants as http://wftp3.itu.int/av-arch/jct3v-site/2014_07_I_Sapporo/JCT3V-I_Notes_d0.doc.

JCT3V-I0002 JCT-3V AHG Report: MV-HEVC / 3D-HEVC Draft and Test Model editing (AHG2) [G. Tech, K. Wegner, J. Boyce, Y. Chen, T. Suzuki, S. Yea, J.-R. Ohm, G. Sullivan]

The fourth 3D-HEVC draft, the eighth Test Model of 3D-HEVC and MV-HEVC and the eighth MV-HEVC draft were developed from the third 3D-HEVC draft, the seventh 3D-HEVC Test Model and the seventh MV-HEVC draft, respectively, following the decisions taken at the 8th JCT-3V meeting in Valencia.

Three editorial teams were formed to work on the three documents that were to be produced:

- H1001 3D-HEVC Draft Text 4

- Gerhard Tech, Ying Chen, Krzysztof Wegner, Sehoon Yea

- H1002 MV-HEVC Draft Text 8

- Gerhard Tech, Miska Hannuksela, Ying Chen, Jill Boyce, Krzysztof Wegner

- Ye-Kui Wang, Adarsh Ramasubramonian

- H1003 3D-HEVC Test Model 8

- Ying Chen, Gerhard Tech, Krzysztof Wegner, Sehoon Yea

Two versions of H1001 have been published by the editing AHG following the 8th JCT-3V meeting in Valencia, the last version has been submitted as study text to the MPEG secretariat. The 3D-HEVC draft text has been updated to be aligned with MV-HEVC draft 8.

Open issues:

- A list of other minor issues is listed in the bug tracking system.

- Subsequent to the Valencia meeting, proponents of H0094 and H0205 provided revised specification text. However, additional changes have neither mentioned in the original input proposals nor in the meetings notes such that they have not been incorporated into the draft. Further clarification might be required since the changes have been integrated to the software.

One version of H1003 has been published by the editing AHG following the 8th JCT-3V meeting in Valencia. The last version corresponds to the text submitted to MPEG secretariat.

Five versions of H1002 were published by the AHG following the 8th JCT-3V meeting in Valencia. The last version corresponds to the text submitted to MPEG secretariat.

Open issues in H1002:

- Some text bugs reported in the bug tracking system or indicated by editor's comments.

The recommendations of the MV-HEVC / 3D-HEVC Draft and Test Model editing AHG are to:

- Approve the edited documents H1001, H1002 and H1003 as JCT-3V outputs.

- Continue to edit documents to ensure that all agreed elements of 3D- and MV-HEVC are fully described.

- Compare the documents with the HTM-software and resolve any discrepancies that may exist, in collaboration with the Software AHG.

- Continue to improve the overall editorial quality of the documents.

- Ensure that properly drafted candidate text is available prior to making any decision to change the specifications.

- Discuss reported open issues.

The three documents were approved as requested.

JCT3V-I0003 JCT-3V AHG Report: 3D-AVC Software Integration (AHG3) [D. Rusanovskyy, F. C. Chen, J. Y. Lee, J.-L. Lin, O. Stankiewicz, T. Suzuki, D. Tian]

No report available during opening plenary. Presented by JRO Fri afternoon

With respect to the Mandates 1 and 4, JCT-3V AhG conducted development of the 3D-AVC software and distributions to the JCT-3V community.

Software enabling IBP inter-view prediction functionality and addressing all publicly reported bugs has been made available from the following SVN location.

http://mpeg3dv.nokiaresearch.com/svn/mpeg3dv/tags/3DV-ATMv12.0/

The provided solution has limited impact on normative part of decoder, introducing change in 1 line of code.  However, the encoding algorithm for enable IBP coding was significantly modified.

Unfortunately, the software was not released on the specified deadlines, due to significant amount of work which was required to address all issues and limited resources available to AhG.

Mandate 5

With respect to mandate 5, produced software was evaluated against the previous version. Coding results produced with modified software, compare to the previous version are reported in Table 1, 2 and 3 given below.

Note: In version 11.0, the encoder was able to produce streams for IPP or IBP configurations, but the decoder would crash. This is fixed now.

Table 1. Coding results of v12.0 for EHP_IBP coding, compared to version 11.0

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0.000% | -0.552% | 0.121% | -0.099% | -0.093% |
| Kendo | 0.000% | -0.132% | -0.174% | -0.029% | -0.016% |
| Newspaper_CC | 0.000% | -0.525% | -0.168% | -0.196% | -0.190% |
| GT_Fly | 0.000% | 0.296% | 0.161% | 0.057% | -0.925% |
| Poznan_Hall2 | 0.000% | -0.313% | 0.586% | 0.119% | 0.115% |
| Poznan_Street | 0.000% | 0.447% | 0.443% | 0.098% | 0.082% |
| Undo_Dancer | 0.000% | -0.392% | -0.341% | -0.077% | -0.072% |
| 1024x768 | 0.000% | -0.403% | -0.074% | -0.108% | -0.100% |
| 1920x1088 | 0.000% | 0.009% | 0.212% | 0.049% | -0.200% |
| average | 0.000% | -0.167% | 0.090% | -0.018% | -0.157% |

Table 2. Coding results of v12.0 for EHP_IPP coding, compared to version 11.0

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0.000% | -0.230% | -0.552% | -0.167% | -0.072% |
| Kendo | 0.000% | -0.239% | -0.132% | -0.050% | 0.037% |
| Newspaper_CC | 0.000% | -0.375% | -0.525% | -0.186% | -0.042% |
| GT_Fly | 0.000% | 0.366% | 0.039% | 0.052% | 0.031% |
| Poznan_Hall2 | 0.000% | 0.128% | -0.151% | -0.006% | 0.014% |
| Poznan_Street | 0.000% | 0.318% | 0.447% | 0.096% | -0.006% |
| Undo_Dancer | 0.000% | -0.197% | -0.392% | -0.077% | 0.005% |
| 1024x768 | 0.000% | -0.281% | -0.403% | -0.135% | -0.025% |
| 1920x1088 | 0.000% | 0.154% | -0.014% | 0.016% | 0.011% |
| Average | 0.000% | -0.033% | -0.181% | -0.048% | -0.004% |

Table 3. Coding results of v12.0 for EHP_PIP coding, compared to version 11.0

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0.000% | -0.366% | -0.578% | -0.199% | -0.195% |
| Kendo | 0.000% | -0.484% | -0.792% | -0.246% | -0.221% |
| Newspaper_CC | 0.000% | -0.349% | -0.138% | -0.086% | -0.085% |
| GT_Fly | 0.000% | 0.069% | 0.799% | 0.093% | -0.909% |
| Poznan_Hall2 | 0.000% | -0.238% | -0.232% | -0.044% | -0.041% |
| Poznan_Street | 0.000% | 0.177% | 0.783% | 0.106% | 0.090% |
| Undo_Dancer | 0.000% | -0.157% | -0.255% | -0.048% | -0.047% |
| 1024x768 | 0.000% | -0.400% | -0.503% | -0.177% | -0.167% |
| 1920x1088 | 0.000% | -0.037% | 0.274% | 0.027% | -0.227% |
| average | 0.000% | -0.193% | -0.059% | -0.061% | -0.201% |

Mandates 2, 3 and 6

Due to limited resources, AhG was not able to conduct work with respect to mandate 2, 3 and 6,  however, significant efforts was conducted outside of the AhG3 activity and result of that work are reported as input contribution JCT3V-I0025.

It is expected that the remaining bugs (particularly in SHP) will be fixed by the time when the final standard for reference software needs to be delivered.

JCT3V-I0004 JCT-3V AHG Report: MV-HEVC and 3D-HEVC Software Integration (AHG4) [G. Tech, H. Liu, Y.W. Chen, K. Wegner]

Development of the software was coordinated with the parties needing to integrate changes.

The distribution of the software was announced on the JCT-3V e-mail reflector and the software was made available through the SVN server:

https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/

Anchor bitstreams have been created and uploaded to:

ftp.hhi.fraunhofer.de; login: mpeg3dv_guest; path: /MPEG-3DV/HTM-Anchors/

One version of the HTM software were produced and announced on the JCT-3V email reflector. The following sections give a brief summary of the integrated tools and achieved coding gains.

Starting point for development of HTM-11.0 was HTM-10.2. Development of HTM-11.0 was conducted in three parallel tracks each performing sequential integration. Development of each branch has been supervised by one software coordinator. Software of all three tracks was merged by the software coordinators.

MV-HEVC: The coding results for MV-HEVC are identical to results obtained from version HTM-10.2.

3D-HEVC: HTM-11.0 vs. HTM-10.2 (CTC, three view configuration)

|  | video video rate | video total rate | synth total rate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0,1% | -0,1% | -0,1% | 104,7% | 130,5% | 113,9% |
| Kendo | -0,1% | 0,1% | 0,1% | 103,4% | 129,9% | 113,7% |
| Newspaper_CC | 0,0% | 0,1% | 0,1% | 104,0% | 108,4% | 113,8% |
| GT_Fly | 0,0% | -0,1% | 0,0% | 100,2% | 105,1% | 100,7% |
| Poznan_Hall2 | -0,2% | -0,2% | -0,3% | 102,4% | 106,5% | 101,5% |
| Poznan_Street | -0,1% | -0,1% | -0,1% | 99,6% | 100,3% | 103,9% |
| Undo_Dancer | 0,0% | -0,1% | 0,0% | 100,2% | 115,0% | 100,6% |
| Shark | 0,0% | -0,1% | -0,1% | 100,2% | 104,0% | 101,2% |
| 1024x768 | -0,1% | 0,0% | 0,1% | 104,1% | 122,9% | 113,8% |
| 1920x1088 | -0,1% | -0,1% | -0,1% | 100,5% | 106,2% | 101,6% |
| average | -0,1% | -0,1% | 0,0% | 101,9% | 112,5% | 106,1% |

Note: Computation time numbers are likely unreliable, since no changes were applied to the renderer.

HTM-11.1 has not been released yet. A preliminary version that will be cross checked and issued during the meeting can be found at:

https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/branches/HTM-11.0-dev0

In this version MV-HEVC 8 HLS has been integrated in a single track. Moreover the software basis was updated to HM-14.0.

Open issues:

- Some minor mismatches related to 3D-HEVC HLS.

- Most of MV-and 3D-HEVC SEI messages, auxiliary pictures, and hybrid scalability are not supported yet.

- Some items related to MV-HEVC HLS (POC alignment, DBP management) are not integrated yet.

- Other minor issues are listed in the bug tracking system.

The recommendations of the 3D-HEVC Software integration group are:

- Develop reference software HTM version 12 based on HTM-11 and improve its quality.

- When a proposal is adopted to discuss how to enable it in the HTM software (e.g. encoder parameter / parameter set flag, or always on).

- Continue to identify bugs and discrepancies with text, and address them.

- Fix open issues.

- Discuss on how to address open issues.

Comments: The bug tracking system reports an encoder crash. This may however be due to compilation with 32 bit compiler version, where the memory allocation is insufficient.

JCT3V-I0005 JCT-3V AHG Report: 3D Coding Tool Testing (AHG5) [K. Müller (Fraunhofer HHI), A. Vetro (MERL), S. Yea (LG Electronics)]

Mandate 1: 	Discuss and finalize CE work plans.

The CE work plans were finalized and made available via the JCT-3V web site by April 4, 2014.

Mandate 2: 	Study the Common Test Conditions and suggest possible changes.

Possible changes to the Common Test Conditions from the CEs should be discussed further during the meeting.

Mandate 3: 	Coordinate between core experiments when necessary.

For the CEs, the 3DV-HTM software version 11.0 was used, as announced over the reflector by April 25, 2014.

Other than this, there were no other needs during this AHG period to coordinate between any of the core experiments.

Mandate 4: 	Study coding tool performance of non-CTC configurations.

The two CEs used the CTC configuration. 
In addition, CE2 used the all-intra configuration.

Furthermore, two AHG5-related input contributions, JCT3V-I0115 on Performance evaluation for 3D-HEVC and its coding tools and JCT3V-I0116 on Coding results for flexible coding order (FCO) have been received.

Mandate 5: 	Report on status of core experiments.

There were 2 CEs established at the previous meeting to further study various aspects of the HEVC-based coding designs.

To get a sense of where the main areas of development are focused, the approximate number of CE reports and CE related contributions are noted below.

CE2 on Depth Intra Coding is most active with 17 contributions. CE1 on Residual Prediction is less active with 9 contributions.

Mandate 6: 	Prepare viewing logistics for 9th JCT-3V meeting.

The 3D viewing equipment has been prepared and the test room 203 has been setup. Viewing will be performed during the week.

JCT3V-I0006 JCT-3V AHG report: 3D High level syntax (AHG6) [J. Boyce, M. M. Hannuksela (co-chairs), Y. Chen, S. Deshpande, J. Samuelsson, G. Tech, Y.-K. Wang, P. Wu (vice-chairs)]

Not reported in opening plenary

done joint meeting Saturday 1800

JCT3V-I0007 JCT-3V AHG Report: Complexity Assessment (AHG7) [G. G. Lee, G. Bang, T. Ikai, K. Rapaka]

Mandate 1: Study intrinsic complexity measures or metrics to assess hardware and software implementations of algorithms/computational models based on number of operations, potential for parallelism, data transfer rate, and data storage requirements. An informative guideline which introduces complexity assessment without biasing towards either software or hardware has been presented in JCT3V-D0301.

- Discussed, identified and tabulated coding tools or cases to be studied at different suggested data granularities in a spreadsheet which provides a high level overview of the scope of the work in this ad hoc group. Furthermore, several examples performed by co-chairs and proponents in previous JCT3V meetings were also included in this spreadsheet which was uploaded together with AHG 7 report.

Mandate 2: Analyze existing designs and produce complexity perspective per tool, from the viewpoint of possible slowdown, bottlenecks or implementation issues.

- According to Mandate 1, complexity of several coding tools are assessed and they also are discussed the bottleneck per tools, such as depth intra coding tools, inter-prediction tools, etc. The analyzed results also included the attached spreadsheet which is illustrated in Mandate 1.

JCT3V-I0069, AHG7: Complexity Assessment on Depth Intra Modes with Simplified Data Flows.

This document provides an informative case study for the coding tools in 3D-HEVC based on number of operations, data transfer rate, and data storage requirements. With respective to JCT3V-H0127, this proposal provides simplified data flow to express the process of depth intra modes from the perspectives of segment-wise coding. The results show that the efficient data flow could clear designers’ doubts with reasonable expressions.

JCT3V-I0111, AHG7: Complexity assessment of inter-prediction tools.

This contribution reports a complexity assessment of inter prediction tools in terms of number of operation and memory bandwidth. This report has changes from the former report of JCT3V-G0208 in the following: (1) include memory bandwidth analysis instead of data transfer rate analysis, (2) include depth access for memory bandwidth calculation, (3) consider memory pattern for memory bandwidth calculation, (4) add DBBP to the target tools.

Assessed tools:

3D-HEVC

- Inter-prediction tools, 9th meeting, JCT3V-I0111, Sharp

- Depth intra modes and its simplified data flows, 8th meeting, JCT3V-H0127, NCKU and Sharp Error: Reference source not found, 9th meeting, JCT3V-I0069, NCKU

- Illumination compensation (IC), 7th meeting, JCT3V-G0114, NCKU Error: Reference source not found; 7th meeting, JCT3V-G0208, Sharp Error: Reference source not found

- Advanced residual prediction (ARP), 7th meeting, JCT3V-G0208, Sharp Error: Reference source not found

- Simplified depth coding, 5th meeting, JCT3V-E0132, NCKU Error: Reference source not found (It was integrated into the segment-wise DC coding in depth intra modes)

- View synthesis prediction (VSP), 4th meeting, JCT3V-D0089, NCKU Error: Reference source not found; 7th meeting, JCT3V-G0208, Sharp Error: Reference source not found

3D-AVC

- Adaptive luminance compensation (ALP), 4th meeting, JCT3V-D0090, NCKU Error: Reference source not found

- Plane Segmentation based Intra Prediction (PSIP), 4th meeting, JCT3V-D0090, NCKU Error: Reference source not found (It was removed from 3D-AVC)

new contributions (I0069, I0078, I0111)

This AHG on Complexity Assessment recommends to:

- review the input contribution during the meeting

- continue with mandates 1 and 2 and move on to mandate 3

JCT3V-I0008 JCT-3V AHG Report: 3D Test Material (AHG8) [S. Shimizu, S. Yea]

There was no e-mail exchange over the reflector.

There were some offline activities to prepare the 3D test materials that were offered at the 5th and 6th JCT-3V meetings.

Deutsche Telekom has kindly prepared six stereo clips, 1 Kaunatal and 5 MissInternet, for the evaluation on whether they are useful for our activities. These sequences can be used for MV-HEVC verification and conformance. However, it is necessary to estimate depth maps if we want to use them for 3D-HEVC/3D-AVC developments.

Regarding the KBS sequences, some investigations including de-interlacing and camera parameter estimations are ongoing by LGE.

Action item: View the new stereo sequences during the meeting.

The AHG on 3D Test Material recommends:

- To discuss the current status and potential problems on making the test sequence available.

- To discuss and possibly set up a plan to resolve the issues.

JCT3V-I0009 JCT-3V AHG Report: Conformance testing development (AHG9) [T. Suzuki, D. Rusanovskyy, D. Tian, Y. W. Chen]

In Valencia, problems in the reference software and MVC+D conformance bitstreams were reported. The problems are,

- 3D-AVC HLS were included in MVC+D conformance bitstreams

- Bugs related to IBP configuration

In Valencia, it was decided to include the revised bitstreams of MVC+D conformance bitstreams rather than producing COR to fix the problem.

But after the meeting, it took a time to fix the problems in reference software, and the revision of bitstreams was delayed. Therefore, the output document from Valencia meeting, JCT3V-H1004 and w14383, are not approved yet.

Before Sapporo, all 3D-AVC conformance bitstreams were generated and are available at the following site.

http://wftp3.itu.int/av-arch/jct3v-site/bitstream_exchange/under_test/3D-AVC/revised/

3D-AVC are available,  7 bitstreams which require correction in MVC+D are still missing.

Discussion:

Target was to include the corrected and new MVC+D streams into the 3D-AVC DAM. Some of the corrected MVC+D bitstreams are not delivered yet. It was decided to proceed with issuing the DAM (and expect NB comments in case that further corrections are necessary); for the ITU version, bitstreams need to be delivered Wednesday, and if further updates are anticipated during editing period, this should be described.

JCT3V-I0010 JCT-3V AHG report: Layered coding constraint specifications and capability indications (AHG10) [G. J. Sullivan (coordinator)]

Not reported in opening plenary

done in joint meeting Saturday 1800

# Project development, status, and guidance (7)

## Communication by parent bodies (0)

## 3D video application areas (0)

## MV-HEVC / 3D-HEVC Conformance

Generate bitstreams for different configurations (All intra, simulcast, only inter-frame, only inter-view, all, different SPS e.g. 2 views with different size, only base view extracted etc., bit stream with aux depth picture where the depth does not necessarily need to be decoded, but the decoder should not crash)

Contributing companies: Qualcomm, Nokia, NTT, … more needed.

Breakout group (Y. Chen) to discuss the set of streams and identify more volunteers

Target to issue a first “draft” (just listing the streams and the contributors)

Until next meeting start working to generate the streams (can only be done after availability of software)

Note: Planning 3D-HEVC conformance should be started sooner, since new tools require significantly more action. A BoG (Y. Chen) will be held 07-08 to make an initial planning about elements for which conformance testing is needed. An AHG will be set up to further prepare this until next meeting.

JCT3V-I0196 BoG report on MV-HEVC conformance test [Y. Chen, S. Shimizu, T. Suzuki]

The aspects that are requested by JCT-3V to be considered to generate multiple bitstreams are listed as follows:

- Picture/slice type

  - A picture may be random access picture, e.g., IDR or CRA, or just non-IRAP.

  - A slice can be I, P or B.

- Prediction type

  - Inter-view prediction, Intra prediction, Inter prediction and different combinations.

- Spatial resolution

  - Symmetric or asymmetric resolutions

- Presence of auxiliary depth pictures

  - Even though the MV-HEVC bitstream includes depth pictures, as auxiliary pictures, the decoder shall not crash by decoding such a bitstream. The decoder as conforming to the current profile is not required to output the depth views.

In addition, the BoG discussed the following two topics, w.r.t. single-view compatibility and hybrid codec, a.k.a. multi-standard scalability.

The case which has an AVC base view and HEVC enhancement view was discussed and it was concluded that such a case might not be relevant for MV-HEVC conformance, mainly because the AVC base view cannot be decoded by MV-HEVC.

Inter-operability between MV-HEVC coded bitstream and HM software: even no specific bitstreams are created for this specific purpose, it is generally requested that the conformance bitstreams should be tested by the HM decoder and the base view shall be correctly decoded, i.e., by outputting the same pictures as the output pictures (with layerId equal to 0) from the MV-HEVC reference software. In addition, it is suggested to go through the bitstream extraction process to generate a sub-bitstream without NAL units that are not recognizable by HEVC version 1.

Volunteers for conformance bitstreams: Qualcomm, LGE, Nokia, NTT, Sony, Sharp.

10 bitstreams are defined for testing the basic functionalities of the 2-view and 3-view case

Auxiliary pictures / depth maps are also tested

Additional topics that were discussed in the second joint meeting Tue 11-13:

- Is hybrid scalability to be tested e.g. with a dummy or zero picture as base layer input? It was confirmed that this should be tested. The content to be filled into the reference picture buffer should either be provided directly, or some means to generate it from a compressed file should be provided.

- Substream extraction – shall this be tested? It was confirmed that this should be tested for both SHVC and MV-HEVC separately. Moreover, the version 1 conformance test set has some “dummy” streams that already test the substream extraction functionality for version 1 decoders, but would fail with layered decoders since they do not actually include a dependent layer. The dummy streams should be removed by the time when actual layered streams.

WD for MV-HEVC conformance.

JCT3V-I0201 BoG report on 3D-HEVC conformance test [Y. Chen, S. Shimizu, T. Ikai]

Was presented 07-09. It was decided to issue a first working draft of 3D-HEVC conformance, which in the further process should become PDAM (in ISO) already in the next meeting. First sets of bitstreams will also be produced until the next meeting.

## Profile/level definitions (1)

JCT3V-I0134 On the Stereo Main profile [M. M. Hannuksela (Nokia)]

The following items are proposed to change the specification of the Stereo Main profile:

- It is proposed to allow the use of external base layers with the Stereo Main profile.

- It is proposed to allow indicating scaled reference layer offsets between layers that do not have an inter-layer prediction relationship.

- It is also remarked that since the AHG10 output text makes the level specifications independent of the number of layers, it could be considered to generalize the Stereo Main profile to allow more than two views. Such a generalized profile could e.g. suit autostereoscopic multiview displays.

1. is agreed provided that conformance to that profile only requires the HEVC part, i.e. there is a certain DPB size, and one of the reference pictures is filled by an external device.

2. is not agreed. It is not used in the MV-HEVC decoding process. It is undesirable to add overhead to the PPS just to carry metadata. For the intended use cases, this would better be implemented as an SEI message (which could for example be defined in the context of 3D-HEVC). It is asserted that an appropriate constraint would be to set num_resampling_position_info to zero in the PPS multiview profile. It is remarked that in the future, when combined scalable/multiview profiles would be defined, such a constraint would need to be removed for enhancement layers.

3. should be considered at the level of parent bodies. There would also be the option of having two profiles: stereo and multiview. It was later adopted.

The proponent expresses his disagreement about the resolution of item 2.

## 3D-HEVC / MV-HEVC performance assessment (2)

(chaired by JRO, 07-04 evening)

JCT3V-I0175 3D-HEVC: Simplification of DMM method [H. Sabirin, K. Kawamura, S. Naito (KDDI)] [late]

This contribution presents the effect of using only one mode of DMM in depth coding. Since the first proposal time, the modes of DMM have been simplified into two modes: DMM1 and DMM4 by the last meeting. It is reported from the experiments that only one DMM mode is possible to maintain the coding performance instead of two modes.

It is proposed to remove DMM4. Results are presented that indicate negligible differences (average 0% but sequence dependent) for CTC, and 0.2% average BR increase in AI

JCT3V-I0115 AHG5: Performance evaluation for 3D-HEVC and its coding tools [S. Yoo, S. Yea (LGE)]

In this document, the overall coding performance of the current 3D-HEVC is shown. Firstly, 3D-HEVC is compared with simulcast HEVC (HEVC version 1) and MV-HEVC. After that, the coding performances of each coding tools integrated in 3D-HEVC are assessed. For these evaluations, HTM-10.0r1 is used. In order to test the simulcast HEVC and MV-HEVC, all 3D-HEVC coding tools are disabled. For the coding tools evaluation, each coding tool is disabled and compared with the anchor. As a result, 3D-HEVC achieved 54.84% of coding performance from synthesis view, while the coding performance of MV-HEVC is 45.44% from synthesis view, compared to the simulcast HEVC.

Presentation deck not provided.

Comparison of 3D-HEVC vs. MV-HEVC both using VSO

In case of DoNBDV disabling, NBDV is invoked instead for other tools such as VSP

With reference to I0069, it is mentioned that by disabling all depth specific intra coding tools, the total loss is higher than the sum of all losses is higher than reported in I0115 (around 10%). This may be due to the fact that a) percentages are not additive b) interdependencies of tools are not considered when turning them off selectively.

Opinions from the discussion:

- The benefit of coding tools is different, but the benefit should be weighted in terms of compression versus complexity (there are examples in the presented numbers where both bit rate and decoding time increase when switching a tool off, which indicates that it is low complex and at the same time gives good compression)

- 3D-HEVC should give sufficient benefit to be adopted by the market, where the results which unveil 17% BR reduction compared to MV-HEVC for synthesized views may not show everything in terms of subjective quality and for more generic test material it may be different.

- Some of the coding tools may not show the gain any more that they showed when they were adopted

- It is suggested to further continue efforts on complexity analysis of dedicated tools

What is of concern in terms of complexity?

- Memory bandwidth

- number of operations per sample

- regularity of these operations, parallelization, in particular for pixel operation

- memory/buffer sizes (cache & external)

- (CABAC throughput)

Is memory bandwidth of concern for any of these tools (compared to the current in HEVC, which is the 8x8 bidirectional PU sub-sample motion comp)? (ARP, DBBP, DoNBDV each in combination with MC)

(the ARP memory BW issue seems to be resolved after this meeting)

Is number of operations per sample critical (compared to the current worst case, which is bidirectional pred. with non-full-sample phase and WP)? (DBBP, IC)

Is regularity/parallelization for pixel operations an issue? (likely not)

Is memory/buffer size an issue? (still 40 kbytes after DMM1 modification)

Note: It was mentioned that an implementation could further use lossless compression of these quite regular binary patterns and decompress them when needed on-the-fly.

One expert mentions that the current HRD definitions for SHVC/MV-HEVC do not allow sharing DPB between different layers across access units. This might imply (unless it is changed for 3D-HEVC) memory resources to be allocated for ARP

(note: also the concept discussed for TMVP sharing would not be possible)

Conclusions:

- consider these issues in the study of the complexity AHG

- raise issue about buffer sharing across AU/layer in joint meeting

JCT3V-I0116 AHG5: Coding results for flexible coding order (FCO) [J. Nam, S. Yea (LGE)]

In the current 3D-HEVC, flexible coding order (FCO) scheme is only available in a software which has several bugs and makes compile errors when the macro for FCO is activated. The draft text is implicitly designed for a texture-first coding order. However, a depth-first coding might be potentially increased to a coding gain by exploiting collocated depth information like the texture-first coding is doing using texture information in the same view. In this proposal, it is proposed to support FCO functionality in 3D- HEVC. The experimental results show a little coding gain on video only and 1% coding loss on synthesized video under the common test conditions.

Presentation deck not provided.

The report refers to video only rate, which is not realistic, as the decoded depth maps are required to decode the texture views. Coded views total rate gives a bit rate increase of approx. 1.9%, which indicates that the total rate becomes higher. One of the reasons could be that MPI is not applied, which is particularly effective.

It is also asked which application would require depth first coding? Not fully clear; in case that both are needed, the configuration giving more benefit in terms of compression would be preferable, which from the results shown is still the texture-first approach.

Applications which only need the texture would also become more complex, which is not desirable.

It is also reported that various bug fixes were applied to the current FCO implementation (which does not even compile, FCO was running in the older HTM implementations but was never fixed since the migration to HM software basis).

Decision(SW/BF): Include the bug fixes in the HTM software.

JCT3V-I0182 AHG5: Crosscheck on Coding results for flexible coding order (JCT3V-I0116) [S. Shimizu (NTT)] [late]

## Complexity assessment (4)

(chaired by JRO, 07-04 evening)

JCT3V-I0069 AHG7: Complexity Assessment on Depth Intra Modes with Simplified Data Flows [G. G. Lee, C.-F. Chen, C.-H. Huang (NCKU)]

This proposal provides a case study of complexity assessment on depth intra modes in 3D-HEVC. The complexity metrics, including number of operations, data storage requirement, and data transfer rate are assessed for evaluating depth intra modes. With respect to the previous proposal, this proposal use another data flow which simplified segment-wise depth coding (SDC) through the segment-based processing; moreover, the complexities of de-quantization and inverse transform are also included when SDC is disabled. Therefore, the analysis results might be considered as anchor complexity to assist JCT3V in evaluating the proposals coming from several proponents in a fair manner. This proposal also presents the experimental results with various configurations, such as enabling/disabling depth modeling modes (DMM), SDC, etc. When SDC, DMM, and DLT are all disabled, the results reveal that the coding loss is 0.0%, 1.8%, and 10.3% for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate, respectively in all intra coding condition; on the other hand, in CTC coding condition, the results reveal that the coding loss is 0.1%, 0.7%, and 3.6% for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate, respectively. Consequently, SDC not only reduces the complexity in depth coding but also increases the coding performance of 3D-HEVC since it provides a reasonable approach to handle depth data.

Presentation deck not provided.

Indicates that SDC with or without DLT is dramatically less complex than transform coding.

Some editorial improvements of the draft text are also provided which should be considered by the editors. A missing part in SDC is also pointed out.

Action: Draft text to be provided in new version of contribution

Decision (Ed): Integrate suggested editorial changes

JCT3V-I0078 Memory Complexity for DBBP and VSP [M. W. Park, J. Y. Lee, Y. Cho, C. Kim (Samsung)]

(include summary)

Analysis shows that the worst case memory bandwidth of DBBP increases the current HEVC worst case to approx. 128%.

The contribution also includes a proposal how to resolve that – to be discussed in DBBP BoG.

JCT3V-I0160 Cross-check of Memory Complexity for DBBP and VSP (JCT3V-I0078) [T. Ikai (Sharp)] [late]

JCT3V-I0111 AHG7: Complexity assessment of inter-prediction tools [T. Ikai (Sharp)]

This contribution reports a complexity assessment of inter prediction tools in terms of number of operation and memory bandwidth. This report has changes from the former report of JCT3V-G0208 in the following:

- Include memory bandwidth analysis instead of data transfer rate analysis

- Include depth access for memory bandwidth calculation

- Consider memory pattern for memory bandwidth calculation

- Add DBBP to the target tools

In the contribution, it is recommended to keep complexity under control and to improve the balance between performance and complexity until 3D-HEVC finalization.

Some findings of this analysis are summarized as follows:

In terms of number of operations, the followings are observed.

- MC, ARP and VSP doesn’t increase number of operations

- IC shows 105 % number of operations

- DBBP shows 138 % number of operations

In terms of number of operations, the followings are observed.

- MC and IC shows 101% to 106% (110 % to 114 %) memory bandwidth

- ARP shows 122 % to 164 % (132 % to 172 %) memory bandwidth

- DBBP shows 111 % to 119 % (120 % to 128 %) memory bandwidth

- VSP doesn’t increase memory bandwidth

(Note: ARP is still the old version, not investigating modifications per CE1)

Table 1. Summary of complexity

|  |  | Number of operation [%] |  | Data transfer rate [%] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3D-HEVC |  | Mult [%] | Add [%] | Partial depth access in DoNBDV |  |  |  | Full depth access in DoNBDV |  |  |  |
|  |  |  |  | 1x1 | 4x2 | 4x4 | 8x2 | 1x1 | 4x2 | 4x4 | 8x2 |
|  | MC | 100% | 100% | 101% | 103% | 105% | 106% | 110% | 113% | 111% | 114% |
|  | ARP | 46% | 38% | 122% | 138% | 133% | 164% | 132% | 148% | 139% | 172% |
|  | IC | 105% | 103% | 101% | 103% | 105% | 106% | 110% | 113% | 111% | 114% |
|  | VSP | 18% | 17% | 41% | 60% | 60% | 78% | 50% | 70% | 65% | 94% |
|  | DBBP | 100% | 140% | 111% | 116% | 116% | 119% | 120% | 125% | 123% | 128% |

Table 2. Summary of complexity (Partial depth access in DoNBDV, 1x1 memory pattern)

| 3D-HEVC |  |  | CU | PU | Mult [%] | Add [%] | TotalBandwidth [%] |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MC | 64x64 | 64x64 | 74% | 75% | 36% |
|  |  |  | 32x32 | 32x32 | 78% | 78% | 43% |
|  |  |  | 16x16 | 16x16 | 85% | 86% | 60% |
|  |  |  | 8x8 | 8x8 | 100% | 100% | 101% |
|  |  | Sub-block (8x8) | 64x64 | 64x64 | 100% | 100% | 100% |
|  |  |  | 32x32 | 32x32 | 100% | 100% | 100% |
|  |  |  | 16x16 | 16x16 | 100% | 100% | 100% |
|  |  |  | 8x8 | 8x8 | 100% | 100% | 101% |
|  |  | ARP | 64x64 | 64x64 | 43% | 36% | 93% |
|  |  |  | 32x32 | 32x32 | 43% | 37% | 97% |
|  |  |  | 16x16 | 16x16 | 44% | 37% | 105% |
|  |  |  | 8x8 | 8x8 | 46% | 38% | 122% |
|  |  | IC | 64x64 | 64x64 | 79% | 78% | 36% |
|  |  |  | 32x32 | 32x32 | 83% | 82% | 43% |
|  |  |  | 16x16 | 16x16 | 91% | 90% | 60% |
|  |  |  | 8x8 | 8x8 | 105% | 103% | 101% |
|  |  | VSP | 64x64 | 64x64 | 18% | 17% | 41% |
|  |  |  | 32x32 | 32x32 | 18% | 17% | 41% |
|  |  |  | 16x16 | 16x16 | 18% | 17% | 41% |
|  |  |  | 8x8 | 8x8 | 18% | 17% | 41% |
|  | DBBP |  | 64x64 | 64x64 | 74% | 115% | 46% |
|  |  |  | 32x32 | 32x32 | 78% | 118% | 53% |
|  |  |  | 16x16 | 16x16 | 85% | 126% | 70% |
|  |  |  | 8x8 | 8x8 | 100% | 140% | 111% |

Table 3. Summary of complexity (Full depth access in DoNBDV, 8x2 memory pattern)

| 3D-HEVC |  | CU | PU | Mult [%] | Add [%] | TotalBandwidth [%] |
| --- | --- | --- | --- | --- | --- | --- |
|  | MC | 64x64 | 64x64 | 74% | 75% | 30% |
|  |  | 32x32 | 32x32 | 78% | 78% | 39% |
|  |  | 16x16 | 16x16 | 85% | 86% | 59% |
|  |  | 8x8 | 8x8 | 100% | 100% | 114% |
|  | Sub-block (8x8) | 64x64 | 64x64 | 100% | 100% | 106% |
|  |  | 32x32 | 32x32 | 100% | 100% | 107% |
|  |  | 16x16 | 16x16 | 100% | 100% | 109% |
|  |  | 8x8 | 8x8 | 100% | 100% | 114% |
|  | ARP | 64x64 | 64x64 | 43% | 36% | 67% |
|  |  | 32x32 | 32x32 | 43% | 37% | 80% |
|  |  | 16x16 | 16x16 | 44% | 37% | 107% |
|  |  | 8x8 | 8x8 | 46% | 38% | 172% |
|  | IC | 64x64 | 64x64 | 79% | 78% | 30% |
|  |  | 32x32 | 32x32 | 83% | 82% | 39% |
|  |  | 16x16 | 16x16 | 91% | 90% | 59% |
|  |  | 8x8 | 8x8 | 105% | 103% | 114% |
|  | VSP | 64x64 | 64x64 | 18% | 17% | 73% |
|  |  | 32x32 | 32x32 | 18% | 17% | 74% |
|  |  | 16x16 | 16x16 | 18% | 17% | 76% |
|  |  | 8x8 | 8x8 | 18% | 17% | 81% |
|  | DBBP | 64x64 | 64x64 | 74% | 115% | 36% |
|  |  | 32x32 | 32x32 | 78% | 118% | 46% |
|  |  | 16x16 | 16x16 | 85% | 126% | 69% |
|  |  | 8x8 | 8x8 | 100% | 140% | 128% |

## Quality assessment (0)

## Source video test material (0)

# Core experiments

## CE1: Residual prediction (17)

(chaired by JRO, 07-03 morning)

### Summary (1)

JCT3V-I0011 CE1 Summary Report: Residual Prediction [H. Liu, T. Ikai]

Summary of coding performance and complexity

The following table shows the simulation results and complexity comparisons for CE proposals. In this table, memory bandwidth of depth (in DoNBDV) is not considered since both proposals only focus on residual prediction.

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Memory bandwidth compared to HEVC (1x1) | Memory bandwidth compared to HEVC (4x2) | Memory bandwidth compared to HEVC (4x4) | Memory bandwidth compared to HEVC (8x2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ARP in 3D-HEVC |  | CTC |  | 122% | 135% | 128% | 158% |
| JCT3V-I0051 #1 | 0.00% | -0.04% | -0.01% | 101% | 113% | 106% | 132% |
| JCT3V-I0151 #1 + #2 | 0.30% | 0.31% | 0.08% | 94% | 103% | 96% | 119% |
| JCT3V-I0104 | 0.41% | 0.41% | 0.11% | 105% | 95% | 88% | 108% |

It is noted that for both JCT3V-I0051 #2 and JCT3V-I0104, coding loss of chroma components is much higher than luma component. For example, the coding loss of Cb (Cr) PSNR/video bitrate on video 1 is 0.46% (0.42%) and 0.51% (0.68%) for JCT3V-I0051 #2 and JCT3V-I0104 respectively. This however not very indicative, since the rate used for chroma BD rate computation is the total rate, and values like this can even occur with a quite small PSNR drop.

Combination of I0051#1 and I0104 would bring the memory bandwidth of ARP down to 94% (compared to the worst case of current HEVC, which is 8x8 bi-pred.). This would give a drop in luma performance of approx. 0.1% BR increase.

Decision: Adopt JCT3V-I0051#1 and JCT3V-I0104.

Related proposals

Summary of coding performance and complexity

The following table shows the simulation results and complexity comparisons for CE1 related subset A proposals. In this table, memory bandwidth of depth (in DoNBDV) is considered since both proposals try to remove depth access when ARP applies. Here, it is assumed that the whole depth block is fetched in DoNBDV.

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Memory bandwidth compared to HEVC (1x1) | Memory bandwidth compared to HEVC (4x2) | Memory bandwidth compared to HEVC (4x4) | Memory bandwidth compared to HEVC (8x2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ARP + DoNBDV in 3D-HEVC |  | CTC |  | 132% | 148% | 139% | 172% |
| JCT3V-I0052 | -0.08% | -0.09% | -0.02% | 122% | 135% | 128% | 158% |
| JCT3V-I0081 | -0.08% | -0.09% | -0.02% | 122% | 135% | 128% | 158% |

DoNBDV increases the memory bandwidth of worst case in HEVC by approx. 14%. This means that the worst case memory bandwidth is 114% anyway, and disallowing DoNBDV just in case of ARP (which is not the worst case mode any more) does not help.

No action on the proposals I0052 and I0081.

### CE contributions (4)

JCT3V-I0051 CE1: Simplification of advanced residual prediction [H. Liu, Y. Chen (Qualcomm)]

Two aspects are proposed in this contribution:

- For bi-predictively coded PUs, the number of accessed blocks used in ARP is reduced from 4 to 3:

  - When current PU is predicted from one temporal reference picture and one inter-view reference picture, the temporal reference block of base block in inter-view ARP is identified with the coded motion vector.

  - When current PU is predicted from two inter-view reference pictures, the temporal reference block in the ARP target reference picture associated with reference picture list 1 is set to the one associated with reference picture list 0.

- It is proposed to disable bi-predictive 4x4 chroma blocks with ARP mode by applying uni-prediction from reference picture list 0.

This proposal was crosschecked by Sharp, and detail information can be referred to JCT3V-I0155.

A comment was made on 1b that it might not allow independent processing of list 0 and list 1 any more. It is however argued that similar interdependencies exist also in other places.

JCT3V-I0155 CE1: Cross-check of Simplification of advanced residual prediction (JCT3V-I0051) [T. Ikai (Sharp)] [late]

JCT3V-I0104 CE1: Results of ARP simplification [T. Ikai (Sharp)]

It is proposed to disable ARP for 4x4 chroma blocks to reduce ARP’s bandwidth.

This proposal was crosschecked by Qualcomm, and detail information can be referred to JCT3V-I0166.

JCT3V-I0166 CE1: Crosscheck of results of ARP simplification (JCT3V-I0104) [H. Liu (Qualcomm)] [late]

### Related contributions (12)

JCT3V-I0052 CE1 related: Alignment of disparity vector in inter-view motion and residual predictions for ARP [H. Liu, Y. Chen (Qualcomm)]

This contribution proposes to align the disparity vector used in inter-view motion prediction and residual prediction. In this way, DoNBDV is not performed when ARP applies and memory bandwidth can be saved for ARP.

This proposal was crosschecked by Hisilicon and Sharp, and detail information can be referred to JCT3V-I0071 and I0159.

Disposition: See above

JCT3V-I0071 Cross check of alignment of disparity vector in inter-view motion and residual predictions for ARP (JCT3V-I0052) [Y. Zhang, Y.-B. Lin (HiSilicon)] [late]

JCT3V-I0159 CE1 related: Cross-check of Alignment of disparity vector in inter-view [T. Ikai (Sharp)] [late]

JCT3V-I0081 3D-CE1 related: Reduction of Worst Case Memory Bandwidth for ARP [M. W. Park, J. Y. Lee, Y. Cho, C. Kim (Samsung)]

This contribution proposes to use NBDV instead of DoNBDV for the inter-view merge candidates when the ARP is enabled so that depth memory fetch is not needed anymore for ARP in the worst case.

This proposal was crosschecked by Sharp, and detail information can be referred to JCT3V-I0157.

Based on the idea description, memory bandwidth analysis and coding performance results, JCT3V-I0052 and JCT3V-I0081 are identical.

Disposition: See above

JCT3V-I0157 3D-CE1 related: Cross-check of Reduction of Worst Case Memory [T. Ikai (Sharp)] [late]

## CE2: Depth intra coding (23)

(chaired by JRO, 07-03 afternoon)

### Summary (1)

JCT3V-I0012 CE2 Summary Report on Depth Intra Coding [Y. Chen, Y. W. Chen]

Results of the proposals

Although there might be interactions between the three proposals especially on the overlaps of the coding gains. The results of these proposals are separately presented, although each of them has already been investigated in a way that the more closely related tools have been tested.

Results for JCT3V-I0118: CE2: Advanced boundary chain coding for depth

|  | Video 1 | Video 2 | Video PSNR | synth PSNR / total bitrate | Enc. time | Dec. time |
| --- | --- | --- | --- | --- | --- | --- |
| CTC |  |  |  |  |  |  |
| JCT3V-I0118 #1 | -0.11% | -0.05% | 0.14% | -0.28% | 100.8% | 98.4% |
| JCT3V-I0118 #2 (w/o CABAC) | -0.11% | -0.09% | 0.14% | -0.21% | 100.9% | 99.7% |
| AI |  |  |  |  |  |  |
| JCT3V-I0118 #1 | 0.00% | 0.00% | 0.02% | -0.29% | 101.4% | 100.4% |
| JCT3V-I0118 #2 (w/o CABAC) | 0.00% | 0.00% | 0.04% | -0.22% | 101.8% | 97.3% |

Note: The proposal shows some loss in case of coded views (0.14% on average). This indicates that the new coding mode effects a higher bit rate for depth, which might give a better tradeoff w.r.t. synthesized PSNR. Another version of boundary chain coding was removed during the 6th JCT-3V meeting, where the results by that time were as follows:

Results on removal of RBC (CTC)

Results for JCT3V-I0095: 3D-CE2: Single depth intra mode for 3D-HEVC

|  | Video 1 | Video 2 | Video PSNR | synth PSNR / total bitrate | Enc. time | Dec. time |
| --- | --- | --- | --- | --- | --- | --- |
| CTC |  |  |  |  |  |  |
| I0095 Test 1 | 0.01% | 0.07% | -0.07% | -0.29% | 101.7% | 98.2% |
| I0095 Test 2 | 0.02% | 0.01% | 0.00% | 0.00% | 104.7% | 98.9% |
| I0095 Test 3 | 0.01% | 0.07% | -0.06% | -0.26% | 100.3% | 97.1% |
| I0095 Test 4 | -0.01% | 0.01% | -0.06% | -0.29% | 105.0% | 98.4% |
| AI |  |  |  |  |  |  |
| I0095 Test 1 | 0.00% | 0.00% | -0.2% | -0.34% | 99.2% | 96.0% |
| I0095 Test 2 | 0.00% | 0.00% |  | -0.01% | 123.8% | 101.7% |
| I0095 Test 3 | 0.00% | 0.00% |  | -0.34% | 98.9% | 96.6% |
| I0095 Test 4 | 0.00% | 0.00% |  | -0.35% | 106.3% | 99.0% |

Results for JCT3V-I0082: 3D-CE2: Residual DC quantization in intra SDC

|  | Video PSNR / video bitrate | Video PSNR / total bitrate | Video PSNR | Synth PSNR / total bitrate | Enc. time | Dec. time |
| --- | --- | --- | --- | --- | --- | --- |
| CTC |  |  |  |  |  |  |
| I0082 #1 | 0.00% | 0.01% | 0.01% | 0.04% | 100.0% | 99.9% |
| I0082 #2 | 0.02% | -0.01% | -0.01% | 0.07% | 100.3% | 99.8% |
| I0082 #3 | -0.01% | -0.07% | -0.07% | -0.07% | 100.2% | 99.6% |
| I0082 #4 | 0.01% | -0.07% | -0.07% | -0.06% | 100.2% | 100.0% |
| AI |  |  |  |  |  |  |
| I0082 #1 | 0.00% | 0.01% | 0.01% | 0.13% | 99.6% | 99.7% |
| I0082 #2 | 0.00% | -0.05% | -0.05% | 0.19% | 100.3% | 99.8% |
| I0082 #3 | 0.00% | -0.07% | -0.07% | -0.07% | 100.1% | 99.6% |
| I0082 #4 | 0.00% | -0.11% | -0.11% | -0.08% | 99.7% | 99.5% |

### CE contributions (8)

JCT3V-I0082 3D-CE2: Residual DC quantization in intra SDC [J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung)]

This contribution presents the results of CE2 in which residual DC quantization is proposed to reduce the magnitude of the residual DC for the general modes in intra SDC, instead of DLT. DLT signals an index instead of the DC value in intra SDC. Hence, to reconstruct the residual DC in a decoder side, DC of the predicted pixels should be first calculated and then re-mapping from the DLT index to the value is performed. In order to simplify these operation, the proposed method quantizes the residual DC with one shift operation to reduce the magnitude of the residual DC, instead of using DLT. The results shows that the proposed method achieves the coding gain of -0.1% and removes the DC calculation and re-mapping processes.

Test#1: DLT is applied to all the sequences by removing a threshold with 128

Test#2: The residual quantization is implemented without a decoder change by using the existing DLT function. When DLT is not constructed by the threshold, it stores every even depth pixel value as an index increases from 0 to 127.

Test#3: The residual DC quantization with default QP equal to 0

Test#4: The residual DC quantization with default QP equal to 1

This proposal is cross checked by JCT3V-I0122 Test#2/#3 (RWTH) and JCT3V-I0135 Test#4 (Sharp).

Test#3 is discussed as a most promising candidate from the results

In the discussion, it is pointed out the change would not be a simplification, since it changes the way of residual quantization for some modes, whereas DMM1 and DMM4 still would use the other method, and both methods need to be described and implemented.

There was also some discussion whether the proposed method restricts the possible accuracy of residual quantization.

Reduction in terms of rate rather low, no consensus that this is beneficial.

JCT3V-I0122 Cross Check of Method 2 and 3 of Residual DC quantization in intra SDC (JCT3V-I0082) [F. Jäger (RWTH Aachen University)]

JCT3V-I0135 3D-CE2: Cross-check of Test 4 of Residual DC quantization in intra SDC (JCT3V-I0082) [T. Tsukuba (Sharp)] [late]

JCT3V-I0095 3D-CE2: Single depth intra mode for 3D-HEVC [Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek)]

This contribution presents the results of CE2 in which the single depth mode is proposed to code the smooth area within a depth map. A CU-level flag is signaled to indicate whether a CU is coded as single depth mode. When a CU is coded as single depth mode, it is reconstructed by filling this CU with one single depth value. Additionally, an index is transmitted to select one out of several available depth sample candidates derived from the spatial neighboring pixels. Compared to the anchor HTM-11.0, the proposed method reportedly shows overall 0.3% BD-rate savings under common test conditions.

This proposal is cross checked by JCT3V-I0168 (Qualcomm).

The tests related to this category are listed as below.

Test 1: Test the coding performance of the proposed single depth mode. The proposed scheme in H0087 has been further simplified by reducing the size of the candidate list from 5 to 2 and replacing the corner spatial candidates with the candidates located at the center of the PU boundary.

Several additional experiments have also been tested to address several mandates of CE2, including the following.

Test 2: Test the coding performance of the current intra SDC scheme with modified encoder design. The encoder is modified by additionally testing the RDO performance for those DC values equal to the sample candidates of single depth mode. For each block coded as intra SDC mode, as shown in figure 1, the values of the neighboring pixels An-1, Bn-1, B0, A0, B-1 are additionally tested as the DC value at the encoder to see whether the coding performance could be further improved.

Figure 1. The spatial sample candidates.

Test 3: Test the coding performance of inserting DDD value as a new candidate for the single depth mode.

Test 4: Test the coding performance of applying residual coding to single depth mode.

Generally, the method is asserted to give a reasonable benefit in compression while being of relative low complexity (less complex than many other modes used in 3D-HEVC)

Decision: Adopt I0095 Test 1 (method with 2 candidates)

It was also pointed out that the benefit of other compression tools (e.g. DDD) might become smaller after this adoption. Generally, contributions showing such cases are welcome.

JCT3V-I0168 CE2: Crosscheck of single depth intra mode for 3D-HEVC (JCT3V-I0095) [H. Liu (Qualcomm)] [late]

JCT3V-I0118 CE2: Advanced boundary chain coding for depth intra mode [J. Seo, M. Koo, J. Nam, S. Yea (LGE)]

This proposal provides the complexity assessment of the advanced boundary chain coding (ABC). The ABC is an improved coding tool based on the region boundary coding which separates a block into two partitions following a contour described by a chain code. The method of complexity assessment is based on JCT3V-H0127 which provides a complexity analysis for depth intra coding modes. This proposal also presents the experimental results of ABC mode with and without CABAC (coded bins). The proposed method with CABAC reportedly shows 0.3% and 0.3% BDR gains in synthesized views under CTC and AI configurations, respectively. The coding gains of ABC without CABAC are 0.2% and 0.2% under CTC and AI configurations, respectively.

This proposal is cross checked by JCT3V-I0140 (MediaTek).

The proposal shows better benefit now than the previous chain coding mode that had been removed during the 6th meeting (where no benefit in terms of synthesis PSNR had been observed).

It is however increasing the complexity, in particular the possible maximum number of context-coded bins per sample might be of concern. If it would increase the worst case CABAC throughput that HEVC currently allows, this would be highly undesirable when comparing against the relative small benefit in terms of compression quality.

Action item: investigate highest possible number of context coded bins for an 8x8 CU.

It was reported that the worst case number of context coded bins/pixel (occurring in case of 4x4 PU sizes) due to the chain code is 1.455.

In the discussion, it was also pointed out that due to the fact that variable length elements of bins are used, sequential decoding may be necessary.

This amount of CABAC throughput would be undesirable, regarding the fact that CABAC coding only gives 0.1% compared to coding without CABAC.

Without CABAC, the bit rate reduction is 0.2%.

No support by other experts for adopting this proposal.

Further study recommended, could become interesting if it provides higher gain without unacceptable large number of context coded bins.

JCT3V-I0140 3D-CE2: Crosscheck on advanced boundary chain coding for depth intra mode (JCT3V-I0118) [Y.-W. Chen (MediaTek)] [late]

JCT3V-I0150 3D-CE2: Cross check of advanced boundary chain coding for depth intra mode (JCT3V-I0118) [J. Y. Lee, C. Kim (Samsung)] [late]

### Related contributions (14)

JCT3V-I0066 CE2 related: Simplification of DMM4 [P. Merkle, D. Marpe (HHI)]

This contribution proposes two modification of the DMM4 depth intra coding mode: simplification of threshold calculation as well as encoder optimization and simplification.

Currently the threshold value for contour segmentation derivation is calculated as the mean value of all depth samples, requiring N²-1 additions and one shift operation for an NxN block. Similar as in proposal JCT3V-H0058, the threshold calculation is simplified by using the mean of the four corner values as the threshold value for contour segmentation, requiring 3 additions and one shift, independent of the block size.

Currently, intra modes in inter slices are only tested under certain conditions in order to limit the encoder complexity. It is proposed that this encoder restriction is removed for DMM4 mode, i.e. always testing it in inter slices. There is also another encoder optimization aspect proposed that avoids a double check.

A similar approach is proposed for DBBP in JCT3V-I0076 (original proposal was JCT3V-H0068).

Decision(SW): Adopt encoder modification proposed in I0066

For threshold derivation simplification, consider a unified solution with DBBP (was revisited in context of I0076)

Decision: Adopt

JCT3V-I0068 Cross Check of CE2 related: Simplification of DMM4 (JCT3V-I0066) [F. Jäger (RWTH Aachen University)]

JCT3V-I0110 Lookup table size reduction in DMM1 [T. Tsukuba, T. Ikai (Sharp)]

This contribution proposes a method to reuse wedgelet pattern of 16x16 block for 32x32 block instead of specifying 32x32 wedgelet pattern in order to reduce the storage size for wedgelet pattern in DMM1. Actually, a pattern for a 16x16 block is scaled to form a 32x32 block. Experimental result reportedly shows that 0.06% / 0.03% coding loss for synth PSNR vs. total bitrate in the CTC / all-intra configuration, respectively. It is asserted that this proposed method can achieve storage size reduction by 79.5%.

General consensus that this is a very useful simplification, and the systematic structure of the 32x32 is also well known.

The cross-checking party and another expert confirmed that the WD text matches with the software implementation.

Decision: Adopt I0110

JCT3V-I0142 Crosscheck on lookup table size reduction in DMM1 (JCT3V-I0110) [Y.-W. Chen (MediaTek)] [late]

JCT3V-I0083 3D-CE2 related: Allowing zero DC candidate in inter SDC [J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung)]

At the 7th meeting, multiple DC candidates were adopted to improve the performance of the inter SDC (G0101). However, the zero DC candidate was not adopted due to the additional branch in merge mode in WD (G0058). To figure out the problem and for the design generality (that covers the zero DC candidate as well), the proposed method allows the zero candidate. It can be represented by replacing or deleting one condition in WD. Results demonstrate that the proposed method does not affect the coding performance a lot.

This proposal suggests two solutions to change the condition of depth_dc_flag, indicating whether the DC values are present or not. One solution leads to slight coding loss and the other doesn’t introduce coding loss.

Presentation deck not uploaded.

Option 1 would remove one condition check at the CU level, but would incur a small loss.

Option 2 changes one condition check by a different one, with no loss (or negligible gain)

No action

JCT3V-I0169 CE2 related: Crosscheck of allowing zero DC candidate in inter SDC (JCT3V-I0083) [H. Liu (Qualcomm)] [late]

JCT3V-I0084 3D-CE2 related: Fast SDC DC offset decision [J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung)]

Multiple DC candidates were adopted to improve the performance of inter and intra SDC. At an encoder, SDC checks five different candidates, which consist of DC, DC-1, DC+1, DC-2, and DC+2, and decides the best one through exhaustive comparison. In order to reduce the encoding complexity, the proposed method introduces a fast SDC DC offset decision method. Results demonstrate that the proposed method save the encoding time without coding loss.

Presentation deck not uploaded.

Decision(SW): Adopt I0084

JCT3V-I0143 3D-CE2 related: Crosscheck on fast SDC dC offset decision (JCT3V-I0084) [Y.-W. Chen (MediaTek)] [late]

JCT3V-I0117 3D-CE2 related: Reduction of the number of intra prediction modes for depth coding [J. Heo, J. Nam, S. Yea (LGE)]

This contribution presents an intra prediction mode for depth map coding. The proposed method reduces the number of intra prediction modes for depth map because the depth map has different characteristics compared to the texture video: 1) little variation within a block 2) simple or smooth region. It means that 35 intra prediction modes used in the conventional intra coding is unnecessarily expensive for depth intra coding. Therefore, this proposal reduced the number of conventional intra prediction modes for depth map coding. The proposed method has minor coding gains under the common test condition (CTC) and all-intra configuration (AI) with negligible encoding/decoding complexity.

Presentation deck not uploaded.

The proposal is to use only 11 intra coding modes instead of 35 (i.e. 3 MPM and 8 remaining instead of 32)

This would be a substantial modification of existing directional intra prediction (including the parsing process) for the purpose of depth map coding, which is not justified by the relative small compression benefit it gives.

JCT3V-I0179 3D-CE2 related: Crosscheck on reduction of the number of intra prediction modes for depth coding (JCT3V-I0117) [S. Shimizu (NTT)] [late]

JCT3V-I0120 CE2 related: Remove “depth_dc_flag” signalling in DMM cases [X. Chen, X. Zheng, Y. Lin, J. Zheng (HiSilicon)]

In the current 3D-HEVC, SDC DMM and DMM are available in depth intra coding and 4 DMM cases(depth_dc_flag=0 or 1, DMM1 DMM4) are used in DMM process. In this contribution, depth_dc_flag is removed in DMM process. Experimental results reportedly show the proposed simplifications bring no coding efficiency impact.

It is proposed that the depth_dc_flag (indicating whether residual value of segments to be 0 in case of sdc_flag equal to 1 and whether dc offset exists in case of when sdc_flag is equal to 0) is conditionally signalled when sdc_flag (indicating whether SDC is used) is unequal to 0. Experimental results reportedly show the proposed simplifications bring no coding efficiency impact.

Several experts expressed that this is a useful simplification of the syntax

Decision: Adopt I0120

JCT3V-I0171 CE2 related: Crosscheck of remove “depth_dc_flag” signalling in DMM cases (JCT3V-I0120) [H. Liu (Qualcomm)] [late]

JCT3V-I0085 3D-CE2 related: Separate enabling flag for intra coding tools [J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung)]

Intra coding tools such as intra SDC and DMM (DMM1 and DMM4) are enabled and disabled by a single flag ‘vps_depth_modes_flag’, while inter coding tools are individually selectable by their corresponding flags. When intra SDC is on and DMM is off, a DMM flag need to be signalled in each CU, which may be redundant. This contribution proposes to define a separate flag for DMM and intra SDC. In addition, it is suggested to change the names of DMM1 and DMM4 into DMM and IVP (Intra-view prediction), respectively.

Presentation deck not uploaded.

Consensus that separate DMM4 flag is needed

Separate DMM1/intra SDC not needed, as these are always used in combination

Decision:

    - Retain inter_sdc flag as is

    - Introduce new dmm4 flag

    - Change semantics of depth_modes flag such that it controls dmm1 and intra sdc

(naming is to discretion of editors)

JCT3V-I0119 Cross check of separate enabling flag for intra coding tools (JCT3V-I0085) [P. Merkle (HHI)]

JCT3V-I0175 3D-HEVC: Simplification of DMM method [H. Sabirin, K. Kawamura, S. Naito (KDDI)] [late]

This contribution presents the effect of using only one mode of DMM in depth coding. Since the first proposal time, the modes of DMM have been simplified into two modes: DMM1 and DMM4 by the last meeting. It is reported from the experiments that only one DMM mode is possible to maintain the coding performance instead of two modes.

It is proposed to remove DMM4. Results are presented that indicate negligible differences (average 0% but sequence dependent) for CTC, and 0.2% average BR increase in AI

# 3DV standards development (incl. software, conformance) (6)

## AVC Extensions (5)

JCT3V-I0025 AhG4: Software Improvements on 3DVATM [D. Tian, A. Vetro (MERL), S. Shimizu (NTT)]

New release of 3D-AVC software

- IBP configuration working

- still some issues e.g. slice header prediction

- software manual also included in the contribution

Shimizu will deliver the appropriate version of the software to Karsten, who will turn in the entire software package for ITU consent procedures.

JCT3V-I0102 3D-AVC defect correction: texture view dependencies missing from SPS [M. M. Hannuksela (Nokia), X. Huang (USTC)]

It is asserted that the signalling of inter-view dependencies between texture views is missing in 3D-AVC (i.e. SPSs for the Enhanced Multiview Depth High profile). In the proposed fix, the seq_parameter_set_3davc_extension( ) is appended to include the inter-view dependencies between texture views, when the texture and depth views are unpaired (i.e. for some views a texture view is present while a depth view is absent, or vice versa).

The intent in the SPS design for 3D-AVC has been to share the same SPS between 3D-AVC-coded depth and texture views.

In a 3D-AVC bitstream with three texture and depth views that are all 3D-AVC-coded except the texture base view, the SPS activation could take place as follows:

- Texture base view: SPS with ID 0, High profile

- Texture non-base views and all depth views: SPS with ID 1, Enhanced Multiview Depth High profile (profile_idc 139)

In another example, the bitstream contains three texture and depth views, out of which the texture base view conforms to the High profile, two texture views conform to MVC and two depth views conform to MVC+D, while the remaining views conform to 3D-AVC. The SPS activation could take place as follows:

- Texture base view: SPS with ID 0, High profile

- Texture view 1: SPS with ID 1, Stereo High profile

- Texture view 2: SPS with ID 2, Enhanced Multiview Depth High profile (profile_idc 139)

- Depth base view and depth view 1, SPS with ID 3, Multiview Depth High profile (profile_idc 138)

- Depth view 2: SPS with ID 2, Enhanced Multiview Depth High profile (profile_idc 139)

The suggested solution is to add the same structure that is currently defined in the MVC+D sps extension to the 3D-AVC sps extension.

Plan is to include this in the new amendment. Proponent is asked to get in contact with the editors and include the text.

Decision: Adopt

JCT3V-I0126 Proposed Draft Text of New AVC Amendment: MFC plus Depth, and Texture and Depth View Packing SEI message [P. Yin, T. Lu, H. Ganapathy, T. Chen, W. Husak (Dolby), M. Tanimoto (NISRI), T. Senoh (NICT), Y. Chen (Qualcomm)]

This contribution proposes a draft text of the new amendment to ITU-T Rec. H.264 | ISO/IEC 14496-10, including MFC plus depth, and texture and depth view packing SEI message. It is also suggested several errata items for 3D-AVC may be included in the new amendment.

MFC+D would combine MFC for texture coding with the mechanisms from MVC+D for depth coding. New profile is defined for that, no new tools.

SEI message: Need to clarify code point assignment – re-use the old one; or introduce a new one and remove the old one.

Note: The SEI message code point is already in the spec (probably was forgotten to remove).

Various changes were suggested to the SEI message – further review and editing was performed 07-08 morning, the result in r2 is assessed to be mature enough for progressing it into the draft standard.

Further, I0102 should be included in the new extension (see above).

It is further confirmed that some minor editorial corrections can still be integrated before publication of the new edition in ISO, and before ITU consent.

JCT3V-I0127 On MFC plus Depth: Reference Software Update [L. Grimaldi, K. Schueuer, P. Yin, T. Lu, H. Ganapathy, T. Chen, W. Husak (Dolby)]

Bitstreams generated with the package can be decoded by 3D-AVC software

Full interlaced support not implemented yet – to be done by the next meeting.

JCT3V-I0063 Software Improvement for Texture and Depth View Packing SEI Message [T. Senoh, K. Wakunami, Y. Ichihashi, H. Sasaki, R. Oi, K. Yamamoto (NICT)]

New version of the software, speedup of encoder and view synthesis – coding results unchanged.

## MV-HEVC (0)

## 3D-HEVC (1)

(reviewed 07-07 evening, chaired by JRO)

JCT3V-I0093 Depth coding compatible with arbitrary bit-depth in 3D-HEVC [K. Zhang, J. An, X. Zhang, H. Huang, J.-L. Lin, S. Lei (MediaTek)]

In the current 3D-HEVC, the disparity vector between two views used in inter-view motion prediction (IVMP) for depth coding is converted from a fixed depth value number 128. When the depth component is coded with a bit-depth value other than 8, 128 is no longer the average depth value. In this contribution, it is proposed to make depth coding compatible with arbitrary bit-depth by replacing the number 128 with 1<<(BitDepthY –1 ).

Decision(Ed): Adopt

It is also pointed out that editorial alignment with the second edition is necessary in various places, e.g. regarding the color format signalling via ChromaArrayType, SubWidthC, SubHeightC etc. It is to the discretion of the editors to make such changes.

Open issue from AHG2 report:

- Subsequent to the Valencia meeting, proponents of H0094 and H0205 provided revised specification text. However, additional changes have neither mentioned in the original input proposals nor in the meetings notes such that they have not been incorporated into the draft. Further clarification might be required since the changes have been integrated to the software.

The proponents were requested to submit late input documents about the intended changes.

JCT3V-I0191 Removal of the 2Nx2N restriction for sub-PU IVMP [J. An, J.-L. Lin, K. Zhang (MediaTek)] [late]

According to the last meeting notes (under discussion CE1, above header H0205), the intended action was to replace the 2Nx2N restriction by a restriction to disallow bi-prediction nly for 4x8 and 8x4 sub-PUs. This was a result of a discussion in the meeting, and the document providing a text as requested action was H0205. However, the text from H0205 only implemented the new 8x4 and 4x8 restriction, without removing the 2Nx2N restriction. This was found during software implementation. The new text implements the change as originally intended. It is also confirmed by other experts that now the checking of dbbp flag is necessary at this position. However, one more change appears necessary in 8.5.3.2.1 (to be clarified offline and be corrected in a new version)

Decision: Adopt (new version 2 with additional modification to be delivered)

JCT3V-I0192 Draft text for the improvement on the signaling of DBBP [J.-L. Lin, Y.-W. Chen, T.-D. Chuang, X. Zhang, Y.-W. Huang, S. Lei (MediaTek)] [late]

Apparently, the WD text provided by the last meeting was incomplete in not documenting the CABAC context change that had been included in the software. Therefore, it was probably correct that the editor rejected the integration of the proposal when receiving a text which differed from the one received in the meeting. As the adoption was mainly seen as a simplification of syntax (as from last meeting notes), including only the syntax change was the correct action of the editor.

No need to take any more action, due to new adoption in the current meeting the change of contexts would anyway be obsolete.

# High-level syntax (42)

## MV-HEVC/3D-HEVC specific (6)

JCT3V-I0090 3D HEVC HLS Cleanup [Y. Cho, M. W. Park, J. Y. Lee, C. Kim (Samsung)]

This contribution proposes some high level clean-ups in the current 3D-HEVC draft as the following:

- Modification of vps_extension2_flag semantics such that the flag shall be zero when the number of views is equal to 1

- Modification of two for-loops in vps_extension2 syntax such that they make more sense than those in the current draft.

Since number of views equal to one can still include texture and depth, the vps extension is needed. No action on the first bullet.

Second bullet: case a) Since i=0 in the loop always refers to the base layer which implicitly has layer_id=0, the if statement is not necessary if the loop starts from i=1. Case b) Also camera parameters are not needed if there is only one view-

Decision(ed.): Adopt the second aspect case a)

Decision: Adopt the second aspect case b)

Note: The editors are asked to look for other cases in the spec. where such simplifications could be applied.

JCT3V-I0099 On the video parameter set extension 2 in 3D-HEVC [H. Huang, J. An, J.-L. Lin, K. Zhang, X. Zhang, S. Lei (MediaTek)]

In the video parameter set extension 2 of current 3D-HEVC, there are misalignments between the working draft and the reference software. Also, there is redundancy in signaling of sub-PU size. In this contribution, it is proposed to align the working draft to the reference software, and it is also proposed to unify the signaling of sub-PU size to remove the redundancy.

In current 3D-HEVC [1], the syntax for video parameter set (VPS) extension 2 is shown in Table 1. There are several misalignments as described below:

- In the working draft, iv_mv_pred_flag and log2_sub_pb_size_minus3 are sent for each layer with layer ID not equal to zero. However, in the reference software HTM-11.0 [2], when coding depth picture, iv_mv_pred_flag and log2_sub_pb_size_minus3 are only sent if layer ID is not equal to 1.

- In the working draft, lim_qt_pred_flag is signaled, but it’s removed in the HTM-11.0.

- In the working draft, the flag iv_mv_scaling_flag is sent before log2_mpi_sub_pb_size_minus3, however, the flag iv_mv_scaling_flag is sent after log2_mpi_sub_pb_size_minus3 in the HTM-11.0.

For depth coding, another syntax element log2_mpi_sub_pb_size_minus3 is sent to indicate the sub-PU size for motion parameter inheritance (MPI). Therefore, signaling of log2_sub_pb_size_minus3 in depth coding is redundant.

In this contribution, it is proposed to align the working draft to the reference software, and it is also proposed to unify the two syntax elements log2_sub_pb_size_minus3 and log2_mpi_sub_pb_size_minus3 to remove the redundancy.

Agreement:

1. The text provides the intended solution, since layer_id=1 does not necessarily associate with depth. Software needs to be aligned.

2. The lim_qt_present_flag is intended and has to be implemented in software.

3. the order of the working draft shall be implemented in the software.

Proponents volunteer to implement the necessary software changes.

Decision (SW/BF): Align the software to the WD text.

The contribution further points out that the syntax element log2_mpi_sub_pb_size_minus3 is redundant, since the WD text also signals the sub pb size per layer, where in case of depth it only applies to MPI. This would allow signalling sub pb size individually per layer also in case of MPI

Agreement:

- Syntax element log2_sub_pb_size_minus3[ layerId ] into the if statement of texture

- Syntax element log2_mpi_sub_pb_size_minus3[ layerId ] into the if statement of depth

- Remove the current layer independent log2_mpi_sub_pb_size_minus3

Draft text for this solution was provided in v4 (I0099WDr1). The alignment of semantics for log2_mpi_sub_pb_size_minus3 (making it layer dependent) is straightforward and shall be done by editors.

Decision: Adopt the version I0099WDr1

JCT3V-I0147 Cross check of On the video parameter set extension 2 in 3D-HEVC (JCT3V-I0099) [M. W. Park, C. Kim (Samsung)] [late]

JCT3V-I0100 3D-HEVC HLS: General comments on 3D-HEVC high level syntax [B. Choi, J.Y. Lee, C. Kim (Samsung)]

The following changes of 3D-HEVC high level syntax are proposed.

- Illumination compensation syntax elements are not present when NumDirectRefLyaers[ i ] is equal to 0.

- The syntax element iv_mv_pred_flag[ layerId ] in vps_extension2 is present only when NumDirectRefLayers[ nuh_layer_id ] is greater than 0.

- MvHevcCompatibilityFlag is replaced by mv_hevc_compatibility_flag[i] in VPS_extension2.

- iv_mv_scaling_flag is present when iv_mv_pred_flag is equal to 1.

First aspect: Agreed. Not necessary when views are coded independent.

Second aspect: Not agreed. Several experts expressed opinion saving one flag conditionally in VPS is not a big deal, and only complicates the syntax.

Third aspect: Not agreed. Putting this flag layer specific into vps extension 2 would not be  useful, as an HEVC decoder cannot recognize it. This rather could be handled via output layer sets and profile idc of extension 1.

Fourth aspect: Agreed. Making iv mv scaling layer dependent could make sense for certain camera configurations. However, not necessary to put this flag conditionally on iv_mv_pred flag

Decision: Adopt the first and the fourth aspect (the latter without condition)

JCT3V-I0101 3D-HEVC HLS: Dependency between texture and depthmap [B. Choi, J.Y. Lee, C. Kim (Samsung)]

In order to explicitly indicate the dependencies between the texture pictures and the depth map pictures referenced by depth-oriented texture coding tools, it is asserted that the direct_dependency_flags[i][j] value of the j-th layer with depth map pictures which is referenced by the i-th layer with texture pictures shall be set equal to 1 and vice versa. Moreover, a condition checking of the VpsDepthFlag values is proposed in the reference picture list derivation, to prevent inserting the depth map picture into the reference picture list of the texture picture.

Had been proposed before (JCT3V-H0040). No new information brought here.

From JCT3V-H1000: “It was confirmed that the current design is fine in that it would not put reference pictures of texture into the depth inter-layer RPS or vice-versa.”

However, it is likely necessary to further investigate aspects of HRD of 3D-HEVC, e.g. POC alignment across layers, access to other layers DPB in different AU etc. (Put some of this under mandates of HLS AHG)

JCT3V-I0103 MV-HEVC HLS: Clean-up of MV-HEVC SEI messages [M. M. Hannuksela (Nokia)]

The contribution proposes some modifications to the SEI messages specified in clause G.14. The detailed proposal is indicated with change marks with user identity "(I0103)" in the accompanying specification text document, which also contains the changes proposed in JCTVC-R0255/JCT3V-I0059 indicated with user identity "Miska Hannuksela". The proposed changes are summarized as follows:

- A few reportedly editorial changes have been made and a few editor's notes have been added in the semantics of 3D reference displays SEI message.

- The depth representation information SEI message has been generalized to apply to any bit-depth and a few reportedly editorial changes have also been made.

- A few changes are proposed for the multiview scene information SEI message to repond to an editor's note. The related editor's note is copied below:

- It is suggested to include both maximum positive disparity (as specified below, for objects perceived behind the screen level) and minimum negative disparity (for objects perceived in front of the screen level).

- MV-HEVC allows different spatial resolution between views provided that inter-view/inter-layer prediction is not used. In such a case, it should be specified which view/layer is used as reference for the units of luma samples.

- It should be considered whether it would be desirable to modify this SEI message to concern explicitly specified pairs of layers/views and enable nesting of this SEI message, where the scalable nesting SEI message would imply which layers or views the multiview scene information concerns.

As suggested by an editor's note, the following changes are  proposed for the multiview scene information SEI message:

- Minimum negative disparity (for objects perceived in front of the screen level) has been added into the SEI message.

- It has been clarified that the right-side view serves as the reference for units of luma samples, when views have different horizontal spatial resolutions.

- The view to which the SEI message applies has been specified as follows: If the SEI message is not nested within a scalable nesting SEI message, it applies to the views with nuh_layer_id greater than or equal to the nuh_layer_id value of the SEI NAL unit that contains the SEI message. Otherwise (the SEI message is nested within a scalable nesting SEI message), the SEI message applies to the view with nuh_layer_id values identified in the scalable nesting SEI message. To views to which the SEI message applies are referred to as applicable views below.

Most items are editorial and agreed

Technical changes that were

-  Min/max disparity in multiview_scene_info: In the discussion, it is pointed out that the minimum disparity value could become positive or negative. A possible solution would be to define the minimum as a signed value and the maximum as an offset (range) relative to that, both with 10 bit

-  Multiview acquisition information SEI message: For indicating separate or shared parameters of texture and depth, re-use the scalable nesting SEI of MVC+D

Text changes were provided and reviewed 07-09 morning. About the min/max disparity (which is now expressed via min and range) it was further remarked during the discussion that it might also be beneficial to express this with relation to picture size (to make it independent of picture width). It was however noted that the current specification would likely be sufficient for 4K and 8K formats.

The suggested changes regarding scalable nesting SEI were also agreed.

Decision: Adopt I0103 (text version in spectect_r1.doc)

## Common MV-HEVC/SHVC (36)

### Summary

JCT3V-I0190 BoG report on HLS [J. Boyce]

A joint meeting with JCT-VC was held 07-05 18:30-20:30. All recommendations of the joint BoG were approved.

JCT3V-I0199 BoG report on editors' notes in SHVC/MV-HEVC specs [Y.-K. Wang]

(=JCTVC-R0359)

The issue on specifying a minimum required value of view_id_length depending on the number of views was agreed to be unnecessary. The sentence specifying the minimum value shall be removed.

Otherwise, approved as is.

JCT3V-I0200 MV-HEVC/SHVC HLS: Miscellaneous cleanups re editors' notes [?? (??)] [late]

This document does not contain open issues related to MV-HEVC. Approved as is.

### Contributions (36)

JCT3V-I0021 AHG10: Profile indication for additional layer sets [M. M. Hannuksela (Nokia)]

(=JCTVC-R0042)

JCT3V-I0022 SHVC/MV-HEVC level definitions and related decoder capability requirements [A. K. Ramasubramonian, Y.-K. Wang, K. Rapaka, Hendry (Qualcomm)]

(=JCTVC-R0043)

JCT3V-I0023 Use of general_profile_space for auxiliary picture profile indication [J. Boyce (Vidyo)]

(=JCTVC-R0041)

JCT3V-I0024 AHG10: On decoder capability indication [M. M. Hannuksela (Nokia)]

(=JCTVC-R0044)

JCT3V-I0026 AHG10: Comments on JCTVC-R0043/JCT3V-I0022 [M. M. Hannuksela (Nokia)]

(=JCTVC-R0046)

JCT3V-I0027 MV-HEVC/SHVC HLS: VPSs with nuh_layer_id greater than 0 [M. M. Hannuksela (Nokia)]

(=JCTVC-R0049)

JCT3V-I0028 Picture size related constraints and level limits [S. Deshpande (??), Y.-K. Wang, Hendry, A. K. Ramasubramonian (??), M. M. Hannuksela (??)]

(=JCTVC-R0052)

JCT3V-I0029 AHG10 and MV-HEVC/SHVC HLS: On invocation of a correct decoding process for the current picture [M. M. Hannuksela (Nokia)]

(=JCTVC-R0053)

JCT3V-I0030 AHG10: Comments on JCTVC-R0041/JCT3V-I0023 [M. M. Hannuksela (Nokia)]

(=JCTVC-R0054)

JCT3V-I0031 On monochrome coding (concerning multi-layer profiles and auxiliary pictures) [M. M. Hannuksela (Nokia)]

(=JCTVC-R0062)

JCT3V-I0032 MV-HEVC/SHVC HLS: On cross-layer impacts of IRAP and EOS [M. M. Hannuksela (Nokia)]

(=JCTVC-R0071)

JCT3V-I0033 MV-HEVC/SHVC HLS: On hybrid codec scalability [B. Choi, C. Kim (Samsung)] [late]

(=JCTVC-R0124)

JCT3V-I0034 MV-HEVC/SHVC HLS: On layer set and bitstream extraction [B. Choi, C. Kim (Samsung)] [late]

(=JCTVC-R0125)

JCT3V-I0035 MV-HEVC/SHVC HLS: On External Base Layer [S. Deshpande (Sharp)]

(=JCTVC-R0153)

JCT3V-I0036 MV-HEVC/SHVC HLS: On Alternative Output Layer and Default Output Layer Indication [S. Deshpande (Sharp)]

(=JCTVC-R0154)

JCT3V-I0037 MV-HEVC/SHVC HLS: Comments on SHVC and MV-HEVC [S. Deshpande (Sharp)]

(=JCTVC-R0155)

JCT3V-I0038 MV-HEVC/SHVC HLS: Move conformance cropping window parameters to rep_format( ) syntax structure of VPS extension [J. Boyce, D. Hong (Vidyo)]

(=JCTVC-R0156)

JCT3V-I0039 Modifications to Scalable Main profile and general and profile-specific level limits [J. Boyce (Vidyo)]

(=JCTVC-R0158)

JCT3V-I0040 HEVC/MV-HEVC/SHVC HLS: SEI message recommended by MMT for improvement of HEVC packet loss resilience [M. Sychev, S. Ikonin (Huawei)]

(=JCTVC-R0159)

JCT3V-I0041 HEVCv1/MV-HEVC/SHVC HLS: On extensibility of nesting SEI messages [A. K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)]

(=JCTVC-R0221)

JCT3V-I0042 MV-HEVC/SHVC HLS: On POC [Hendry, A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)]

(=JCTVC-R0223)

JCT3V-I0043 MV-HEVC/SHVC HLS: Clarifications to the specification of end of sequence (EOS) NAL unit [A. K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)]

(=JCTVC-R0224)

JCT3V-I0044 MV-HEVC/SHVC HLS: On indication of the use of temporal motion vector prediction [Hendry, A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)]

(=JCTVC-R0226)

JCT3V-I0045 MV-HEVC/SHVC HLS: Miscellaneous cleanups [Y.-K. Wang, Hendry, A. K. Ramasubramonian (Qualcomm)]

(=JCTVC-R0227)

JCT3V-I0046 MV-HEVC/SHVC HLS: Unavailable and useless stuff [Y.-K. Wang, A. K. Ramasubramonian, Hendry (Qualcomm)]

(=JCTVC-R0230)

JCT3V-I0047 MV-HEVC/SHVC: Signalling and use of HRD parameters for bitstream partitions [Y.-K. Wang, Hendry, A. K. Ramasubramonian (Qualcomm), S. Deshpande (Sharp), M. M. Hannuksela (Nokia)] [late]

(=JCTVC-R0231)

JCT3V-I0048 AHG10: Processing of bitstreams without an available base layer [M. M. Hannuksela (Nokia)]

(=JCTVC-R0235)

JCT3V-I0049 MV-HEVC/SHVC HLS: On access unit boundary detection [M. M. Hannuksela (Nokia)]

(=JCTVC-R0236)

JCT3V-I0050 MV-HEVC/SHVC HLS: Editorial cleanups [A. K. Ramasubramonian, Y.-K. Wang, Hendry (Qualcomm)]

(=JCTVC-R0238)

JCT3V-I0059 MV-HEVC/SHVC HLS: On persistence of SEI messages [M. M. Hannuksela (Nokia)] [late]

(=JCTVC-R0255)

JCT3V-I0060 Language indication for Overlay information SEI message [K. Suehring (Fraunhofer HHI)] [late]

(=JCTVC-R0257)

JCT3V-I0062 HEVCv1/MV-HEVC/SHVC HLS: Conformance cleanups [Y.-K. Wang, A. K. Ramasubramonian, Hendry (Qualcomm)] [late]

(=JCTVC-R0272)

JCT3V-I0064 MV-HEVC/SHVC HLS: On active parameter sets SEI message [M. M. Hannuksela (Nokia)] [late]

(=JCTVC-R0274)

JCT3V-I0067 MV-HEVC/SHVC HLS: On sub-bitstream extraction [M. M. Hannuksela (Nokia)] [late]

(=JCTVC-R0276)

JCT3V-I0091 MV-HEVC/SHVC HLS: More miscellaneous cleanups [Hendry, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)] [late]

(=JCTVC-R0279)

JCT3V-I0092 MV-HEVC/SHVC HLS: Bitstream conformance restrictions in subclause C.4 [A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)] [late]

(=JCTVC-R0280)

JCT3V-I0203 MV-HEVC/SHVC HLS: Cleanup of VUI semantics [A. K. Ramasubramonian (Qualcomm), S. Deshpande (Sharp Labs)] [late]

(=JCTVC-R0361)

JCT3V-I0204 MV-HEVC/SHVC HLS: Repetition of parameter sets [A. K. Ramasubramonian (Qualcomm), M. M. Hannuksela (Nokia)] [late]

(=JCTVC-R0362)

# Non-CE technical contributions (60)

## ARP (11)

(chaired by JRO, 07-03 afternoon/evening)

JCT3V-I0089 3D-CE1 related: Diagonal prediction using ARP [J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung)]

Note: This is not CE1 related.

Advanced residual prediction (ARP) is used to efficiently code a residual between original and predicted pixels. Both temporal and inter-view ARP are supported and three weighting factors such as 0, 1, and 0.5 are considered. However, it is not clear why the 0.5 weighting factor is used. This contribution proposes to support diagonal prediction instead of the 0.5 weighting factor. The result illustrates that, even though the proposed method achieves the minor coding gain (about -0.1% in a dependent view), it reduces the memory bandwidth (from 172% to 67% in the 8x2 memory pattern) by removing the additional block accesses when the weighting factor is equal to 0.5.

Presentation deck not uploaded.

Reduction of memory bandwidth refers to average (not worst case).

Worst case memory bandwidth has been resolved per adoptions from CE1

The removal of 0.5 weighting does not give a critical complexity advantage and would give up flexibility that may be desirable in ARP.

No action.

JCT3V-I0178 3D-CE1 related: Crosscheck on Diagonal prediction using ARP (JCT3V-I0089) [S. Shimizu (NTT)] [late]

JCT3V-I0096 On complexity reduction of bi-prediction for advanced residual prediction [Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek)]

In HTM-11.0, when motion information of list0 is the same as that of list1, i.e., two motion vectors point to a same reference block, the encoder and the decoder can non-normatively change the inter prediction direction from bi-prediction to uni-prediction to avoid the unnecessary computation caused by the redundant motion compensation process. Experimental results show that the decoder could save around 5% decoding time by this non-normative fast bi-prediction. However, this non-normative fast bi-prediction is currently disabled for advanced residual prediction (ARP) in reference software, HTM-11.0 due to the unequal operations between uni-prediction and bi-prediction in ARP. To resolve this issue, it is proposed to align the operations between uni-prediction and bi-prediction for ARP. Experimental results reportedly show that the proposed method causes no coding efficiency loss while the unnecessary computation of bi-prediction can be removed for ARP coded PUs.

Presented results are somewhat confusing (e.g. the reported 5% reduction of decoding time includes the benefit of normal bi-prediction for which HEVC already includes such mechanism)

In the last meeting (JCT3V-H0097), it was analysed that the benefit would only be effective in 1% of all MC blocks with current test set (i.e. where ARP with identical motion realistically appears). There is not doubt that this is realistic.

The new proposal (which give a slight increase in compression) would make the normal process more complicated due to additional clipping operation and block-level check about the reference to use.

Benefit not obvious.

No action.

JCT3V-I0167 Crosscheck of On complexity reduction of bi-prediction for advanced residual prediction (JCT3V-I0096) [H. Liu (Qualcomm)] [late]

JCT3V-I0173 Cross-check of On complexity reduction of bi-prediction for advanced residual prediction (JCT3V-I0096) [T. Ikai (Sharp)] [late]

JCT3V-I0185 Crosscheck on complexity reduction of bi-prediction for advanced residual prediction (JCT3V-I0096) [S. Shimizu (NTT)] [late]

JCT3V-I0072 Problem fix for MV scaling in inter-view ARP [J. An, K. Zhang, X. Zhang, H. Huang, J.-L. Lin, S. Lei (MediaTek)]

In current inter-view ARP, a problem exists in the scaling process when the MV in the reference view is in inter-view direction (IBP configuration) which results in the software crash. This contribution proposes to fix this problem by setting the derived MV directly as (0, 0) for that situation to avoid the problematic scaling.

Decision(BF/ED/SW): Adopt

Note (to editor): Terms like ‘temporal reference picture’, ‘inter view reference picture’ should be avoided and/or clearly specified.

JCT3V-I0129 AHG4: Editorial improvement for ARP specification text in 3D-HEVC [L. Zhang, H. Liu, Y. Chen (Qualcomm)]

In current 3D-HEVC, the advanced residual prediction (ARP) working draft text is implemented in a way that reuses the sub-clauses of weighted sample prediction process in HEVC version 1 which invokes right shifting operations due to the increased bit-depth in interpolation process. However, the bilinear interpolation process is defined in a way that the internal bit-depth of the prediction signal in interpolation process is kept unchanged during the bilinear filtering process used by ARP process. Therefore, the reuse of the weighted sample prediction process results in wrong prediction signal. It is proposed to fix these unintended bugs in this contribution by modifying the bilinear filtering process.

There is no doubt that the current text description of bilinear interpolation and the subsequent processing for generating the prediction deviates from the software. It is intended:

- describe the interpolation process in a unified style with other HEVC interpolation processes

- have the same number of clipping operation (asserted to be one in the very end) as in the usual HEVC prediction process

- clarify the bit depth of the residual

Offline activity (Y. Chen, J.-L. Lin, Shimizu) to identify the issues and suggest actions both in text and software.

Was reported back 07-08. Even though there would be other solutions to the problem (e.g. applying clipping before averaging in bi-prediction), the proposed solution is also appropriate and should be adopted.

Decision(Ed/SW): Adopt I0129

JCT3V-I0180 AHG4: Crosscheck on editorial improvement for ARP specification text in 3D-HEVC (JCT3V-I0129) [S. Shimizu (NTT)] [late]

JCT3V-I0055 AHG4: Semantics clean-up for advanced residual prediction weighting factor [Y. Chen, L. Zhang (Qualcomm)]

In current 3D-HEVC, a weighting factor index is signaled to indicate whether advanced residual prediction (ARP) is enabled for one coding unit (CU), and if yes, what is the weight applied to the predicted residue. The weighting factor index iv_res_pred_weight_idx however has an issue of having unclear semantics. Even when it is not equal to 0, it might happen that ARP doesn’t apply to the current CU. Such a behavior is considered contradictive to the value of this syntax element of iv_res_pred_weight_idx. In this proposal, it is proposed that iv_res_pred_weight_idx has to be set to 0 (at the encoder side) if eventually no ARP applies to the current CU. From the decoder perspective, a simple constraint is added to clean-up the design and make the semantics of iv_res_pred_weight_idx more reasonable. In addition, the iv_res_pred_weight_idx is currently constrained to be 0 when NBDV doesn’t return a valid disparity vector. However, this constraint shouldn’t apply for inter-view ARP thus the constraint is proposed to be modified. The proposed fixes don’t introduce any coding performance change under common test conditions.

There is no doubt that the current spec. is clear enough such that a decoder when receiving a bitstream would not run into problems. In principle, the proposal would modify the spec such that additional bitstream constraints are imposed. On the other hand, some concern was raised that encoders might be forced to follow these constraints and implement additional checks which could increase encoder complexity. Decoders would not benefit by reduced complexity when these constraints are imposed.

Furthermore, the results show that the coding performance is not unchanged when these constraints are imposed. This seems to indicate that the combination of syntax elements and other states that is intended to be disallowed can happen even under CTC. Therefore, there might even be the possibility that specifically smart encoders utilize this to gain some more benefit.

No action at this point

Note: It was discussed that it might also be checked whether the existing constraint disallowing ARP when NBDV is unavailable is necessary. Y. Chen reported back on this issue and confirmed that this constraint should be retained.

JCT3V-I0181 AHG4: Crosscheck on Semantics clean-up for advanced residual prediction weighting factor (JCT3V-I0055) [S. Shimizu (NTT)] [late]

## IC (4)

(chaired by JRO, 07-04 morning)

JCT3V-I0080 Parameter Derivation for Illumination Compensation [M. W. Park, J. Y. Lee, Y. Cho, C. Kim (Samsung)]

In the current IC process, if the reference position is equal to or less than zero then neighboring samples of the reference block are not used to derive the IC parameter, and if the reference position is greater than zero then neighboring samples are always used and the proper description of this is omitted in the draft text. Therefore, this contribution proposes to clip the position of neighboring samples and to remove the useless condition from the IC parameter derivation process. This proposed method reportedly provides no coding loss. Additionally, even though the position of the top-left pixel of the reference block is in fractional sample position, the nearest integer samples are used to derive the IC parameter. To obtain more accurate IC parameter, it is proposed to use the fraction samples derived by the bi-linear interpolation. This proposed method reportedly provides 0.1% bit-saving for coded views.

First aspect of the proposal: Access to samples outside of reference picture – this requires specification in the draft text. The software uses padding of samples from the picture boundary, which is asserted to be identical with the suggested method of clipping the coordinate of the sample to max size.

Decision: Adopt this aspect (clipping of coordinate)

Usage of sub-pixel values for illumination compensation increases worst case complexity (number of mult/sample) while giving relatively low gain.

No action on this second aspect.

JCT3V-I0158 Cross-check of Parameter Derivation for Illumination Compensation [T. Ikai (Sharp)] [late]

JCT3V-I0132 Analysis for redundant clippings in illumination compensation [H. Liu, Y. Chen (Qualcomm)]

In HEVC, when generating inter prediction sample for a pixel, only one clipping is required during the weighted sample prediction process as in sub-clause 8.5.3.3.4. In current 3D-HEVC, however, when Illumination Compensation (IC) is enabled, 2 additional clippings per pixel are required for each prediction direction. In total, four additional clippings are required for each pixel in bi-prediction case when IC applies. The four clippings are for the original motion compensated signal and the IC compensated signal of each prediction direction. This contribution proposes to remove such additional clippings in IC, while it should be noted that removal of such redundant clippings can still support conversion from bi-prediction to uni-prediction in IC in a non-normative manner (bi-prediction can be performed as uni-prediction if two motion vectors and their associated reference pictures are the same). Simulation results show that proposed method has no coding performance loss. The proposed methods provide complexity reduction in the case of IC by removing redundant clipping.

Aspects that were discussed:

- Usage of multiple clippings allows to implement the last part of IC with 8 bit logic, which would be an advantage mostly for hardware

- Software implementations would rather benefit from saving clipping operation

- The specification text currently suggested would increase worst case complexity, since it would allow using IC and WP in the same PU

It is requested to provide spec. text which conditionally disallows WP when IC is used in a PU

The updated text was provided. I0132textr2 (Section 2 Version 1) requires an additional comparison at a later stage.

On the aspect of benefit clipping vs. 8 bit word length, no consensus was reached, and no support by non-proponents was given.

No action

JCT3V-I0172 Crosscheck of analysis for redundant clippings in illumination compensation (JCT3V-I0132) [S. Yoo, S. Yea (LGE)] [late]

JCT3V-I0193 Crosscheck of conversion verification for JCT3V-I0132 [X.Chen, X.Zheng (HiSilicon)] [late]

## MV/DV inheritance / coding (21)

(chaired by JRO, 07-04 morning/afternoon)

JCT3V-I0053 Simplifications for 3D-HEVC merge candidate list generation [H. Liu, Y. Chen (Qualcomm)]

As discussed in previous JCT-3V meetings, in derivation process of 3D-HEVC merge list, there are several aspects that may not be optimal. First, the insertion of HEVC merge candidates into 3D-HEVC merge candidate list depends on spatial position and availability of spatial merge candidates. Second, the VSP (View Synthesis Prediction) candidate may be generated in a way that requiring a status to be stored (a VSP variable/flag) for spatial neighboring so that the merging process takes this variable into consideration. Third, when deriving shifted disparity merging candidate, up to 5 spatial merge candidates may be checked to find the first available DMV (Disparity Motion Vector).

This contribution provides solutions that can be considered by 3D-HEVC. First, when inserting the additional merge candidates, the dependency on spatial positions of the HEVC merge candidates is removed. Second, the VSP flag is not used to generate VSP candidates therefore storage and check of spatial neighboring blocks are not needed. Third, only allow check up to 2 HEVC merge candidates when searching the first available DMV for the first two HEVC merge candidates. It is reported that proposed method has negligible coding loss compared to HTM under the common test conditions and the condition when depth dependent texture coding is not enabled.

The method is claimed to reduce the number of additions from 6 to 3, and the number of comparisons from 42 to 10, concerning computation of shifted inter-view merge candidate.One expert expresses that the proposed solution has the disadvantage of introducing dependencies between original HEVC and new 3D-HEVC merge candidates.

Another expert expressed support, since the coding loss is small.

See further notes in context of BoG report I0195

JCT3V-I0139 Crosscheck for JCT3V-I0053 Simplifications for 3D-HEVC merge candidate list generation [J. An (Mediatek)] [late]

JCT3V-I0154 Cross check of simplifications for 3D-HEVC merge candidate list generation (JCT3V-I0053) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-I0198 Cross-check of additional test for Simplifications for 3D-HEVC merge candidate list generation (JCT3V-I0053) [T. Ikai (Sharp)] [late]

JCT3V-I0087 Simplification of 3D Merge list construction [J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung), Y. Zhang, L. Yu (Zhejiang University)]

At Geneva meeting, the simple Merge candidate list construction was adopted (F0093). It introduced a simple and low-complexity way to implement the Merge list construction module for 3D-HEVC by re-using the existing HEVC Merge list construction. As follow-up of H0097, this contribution proposes to remove pruning processes, VSP inheritance, availability checking processes of HEVC candidates such as candidate A0, A1, B0, B1, and B2 in the 3D Merge list construction. As a result, it can minimize the interaction between the HEVC candidates and 3D candidates. The result demonstrates that the proposed method has small coding loss with about 0.1%. In terms of simplification, the availability of the candidate A0, A1, B0, B1, and B2 is not needed to be checked in the Merge list construction, the number of pruning processes is reduced from 8 to 0, and the VSP flag storage is not required. In addition, this contribution proposes several alternative options for simplification.

Some common aspects with I0053. Indicates that higher losses (0.1% bit rate increase) occur when interaction between HEVC/new merge candidates is given up.

(Interaction means that knowledge is required what the source of the different candidates in the HEVC candidate list was, e.g. if they came from A1, B1 etc. Preference is expressed by several experts for such a solution.)

“Option 1” of the proposal also comes with practically no loss, but still requires interaction in pruning for the first list positions.

To sort those aspects out and possibly come to a unified solution, establish BoG (Shimizu), to further investigate aspects of merge list construction from I0053, I0086, I0087, I0098, I0106, I0112, I0130, I0131

See further notes in context of BoG report I0195

JCT3V-I0156 Cross-check of Simplification of 3D Merge list construction (JCT3V-I0087) [T. Ikai (Sharp)] [late]

JCT3V-I0073 MV sharing for depth coding in 3D-HEVC [K. Zhang, J. An, X. Zhang, H. Huang, J.-L. Lin, S. Lei (MediaTek)]

In this contribution, a MV sharing method between the depth and texture components is proposed. MVs needed by temporal motion vector prediction for depth coding are fetched from the corresponding texture picture instead of a depth picture. MV storage can be halved in the proposed approach. In addition, the times of loading the MV buffer from external memory to on-chip memory can also be halved. The experimental results reportedly show that the proposed simplification brings no coding loss.

H0089 was a previous similar proposal (where however the shared memory had also been suggested for AMVP in current AU). This was recommended for further study, but some aspects that had been brought up in the context of the previous proposal are not answered in the new contribution.

Firstly, the claimed advantage of saving memory bandwidth is not true, since it cannot be assumed that texture and depth are processed simultaneously.

Second, there are without doubt cases where sharing of stored motion vectors is undesirable, a) decoding of MV-HEVC streams b) cases where texture and depth are unregistered. Therefore, sharing the memory could only be useful as a switchable option (not unconditionally as an architectural constraint). Then, however the question arises whether every decoder implementation should be enforced to implement both (shared and unshared memory).

This would be an advantage for encoders, because they could use the available DPB resources for a given profile/level compliant bitstream more flexibly. At this time, it is not fully clear whether it would be an unreasonable burden for decoders to implement it.

The proponent is requested to prepare a specification text which would enable this as a switchable option.

Text was provided in I0073v3. Several experts expressed reservations against the proposal because

- It changes the TMVP process if made switchable (condition check)

- Real benefit is not obvious

- With current high level syntax, it would not be applicable (however, the same problem exists for ARP, and changes will be necessary here anyway) since it requires accessing DPB from another layer in another access unit.

No support by non-proposing experts – no action.

JCT3V-I0161 Cross-check of MV sharing for depth coding in 3D-HEVC (JCT3V-I0073) [T. Ikai (Sharp)] [late]

JCT3V-I0074 Depth compensation calibration in 3D-HEVC [K. Zhang, J. An, X. Zhang, H. Huang, J.-L. Lin, S. Lei (MediaTek)]

In this contribution, depth compensation calibration (DCC) is proposed for depth coding. Given a motion compensation block in depth coding, the prediction block is calibrated based on the difference between two texture pictures in different views. DCC is applied as a new merging candidate for depth coding on dependent views without introducing new syntax elements. The experimental results reportedly show that proposed method can achieve 0.3% BD-rate saving for the synthesized views in the common test condition.

SAD between current view and base view texture is computed to determine the DCC candidate.

Per 4x4 block, 16 SAD computations in worst case, 8-9 on average. For each SAD, 16 absval and 15 additions are required. Additional memory accesses also required.

Too complex to justify the compression gain. Would require significant decrease in complexity to be seriously considered.

JCT3V-I0151 Cross check of depth compensation calibration in 3D-HEVC (JCT3V-I0074) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-I0086 Simplification of shift DV candidate [J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung)]

From BoG I0195:

In 3D-HEVC, shift DV is used in merge mode. In order to find the shift DV, all the neighboring blocks are searched whether they have DV or not. After the DV searching process, it is generated by adding four to a horizontal component of the existing DV. However, during several meetings it was reported that the shift DV provides the small coding gain. Therefore, this contribution proposes to remove and simplify the shift DV as the first and second options, respectively. The third option removes a pruning process of DV on top of the second option. Based on WD, both the options reduce 30 condition checks in the worst case by removing a subclause H.8.5.3.2.15 (DV searching process). The experiment shows the negligible coding loss in the first option and no coding loss in the second option.

The benefits of shifted DV candidates is small (0.04% BR saving for synthesized views under CTC and less than 0.1% with VSP and DoNBDV off)

There are three aspects in this contribution:

- Make DV candidate available only when horizontal component of DoNBDV is not equal to zero

- Generate shifted DV candidates by adding 4 to the DV candidate instead of searching neighboring blocks

- Remove the pruning process on DV candidate

No action on the first aspect since this aspect considered as coding efficiency improvement proposal, but no notable gain was observed.

Removing search (the second aspect) is preferable; however no separate result was available.

One expert requests separate results (both CTC and VSP/DoNBDV/DBBP off) and corresponding working draft for the second aspect.

Third aspect will also be discussed after the separate results are provided.

Additional results with just removing DMV search were presented (in I0086r2).

CTC: 0.01%/0.01% BR saving for coded/synthesized views

Non-CTC (DoNBDV/VSP/DBBP off): 0.02%/0.01% BR increases for coded/synthesized views

Small losses are observed for video1/video2 (0.08% on average, 0.33% in the maximum).

Several experts express some support for the proposal.

Decision: Adopt removal of DMV search for shifted DV candidate derivation (in I0086_WD_r1)

JCT3V-I0113 Crosscheck on simplification of shift DV candidate (JCT3V-I0086) [Y. Zhang (Zhejiang University)]

JCT3V-I0197 Cross-check of additional test for Simplification of shift DV candidate (JCT3V-I0086) [T. Ikai (Sharp)] [late]

JCT3V-I0098 Unification of Merging Candidate List Construction in 3D-HEVC [J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)

Was reviewed in BoG I0195

JCT3V-I0152 Cross check of unification of the merge candidate sets in 3D-HEVC (JCT3V-I0098) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-I0106 Further VSP inheritance simplification [T. Ikai, T. Tsukuba (Sharp)]

Was reviewed in BoG I0195

JCT3V-I0188 Crosscheck on Further VSP inheritance simplification (JCT3V-I0106) [S. Shimizu (NTT)] [late]

JCT3V-I0112 Simplification for VSP-related Merge Candidate List Construction [Y. Zhang, Q. Wang, L. Yu (Zhejiang Univ.)]

Was reviewed in BoG I0195

JCT3V-I0136 Crosscheck results on simplification for VSP-related merge candidate list construction (JCT3V-I0112) [M. Li, P. Wu (ZTE)]

JCT3V-I0114 Improvement on merge list construction [S. Yoo, J. Nam, S. Yea (LGE)]

In the current 3D-HEVC, an IC flag can be parsed for merge blocks according to the merge index. In this method in D0060Error: Reference source not found, the IC flag is only parsed when the merge index is larger than zero. On the other side, MLC(merge list construction) for dependent views is performed according to the IC flag in H0070Error: Reference source not found. The MLC process can skip IvMC candidate if the IC flag is equal to 1. Coincidence of these two methods can make the design undesirable, not being chosen the first spatial candidate if the IC flag is set to 1. Therefore, it is proposed to solve this problem with two solutions. The first solution is to make D0060 disabled, and it is reportedly shown that the coding loss is 0.16% from synthesis view. The second solution is to remove H0070. It is found out that it losses 0.03% of coding gain from synthesis view.

Presentation deck not made available

The contribution proposes to remove the technology proposed in JCT3V-H0070, which was adopted by the last meeting. No new evidence is brought why the inclusion of this technology is disadvantageous (no technical flaw, no complexity advantage by removing it again). A more precise analysis and demonstration of disadvantages of this technology would be required to take such a step.

No action.

JCT3V-I0141 Crosscheck on Improvement on merge list construction (JCT3V-I0114) [Y.-W. Chen (MediaTek)] [late]

JCT3V-I0130 Simplification of VSP candidate derivation [G. Bang (ETRI), M. S. Lee, Y. S. Heo, G. H. Park (KHU), G. S. Lee, N. H. Hur (ETRI)]

Was reviewed in BoG I0195

JCT3V-I0174 Crosscheck on Simplification of VSP Candidate Derivation (JCT3V-I0130) [Y. Zhang (Zhejiang University)] [late]

JCT3V-I0131 Adaptive candidate selection [G. Bang (ETRI), Y. S. Heo, G. H. Park (KHU), G. S. Lee, N. H. Hur (ETRI)]

Was reviewed in BoG I0195

JCT3V-I0149 Cross check of Adaptive candidate selection (JCT3V-I0131) [M. W. Park, C. Kim (Samsung)] [late]

JCT3V-I0195 BoG report on merge list construction [S. Shimizu (NTT)]

The BoG met on 5th July 2014 (3:30 to 6 pm) in the room Small Hall. Approximately 20 experts attended the BoG meeting. The BoG met again on 7th July 2014 (1 to xxpm), and additional results were presented.

The BoG recommends the following:

- Encouraging further study on removal of the pruning process

- Adopting the removal of DMV search in I0086r3

BoG participants agreed that the current design of merge candidate list construction is mature.

Follow-up discussion in JCT-3V plenary:

Compression performance does not play a role in the decision

The overall construction of the merge list will be more complicated than with HEVC version1, there are no proposals which would give less number of operations

Two different opinions exist

1. It is important to keep the first part of the process identical with HEVC, and implementing the second part of the process (3D specific) in a way that it is not necessary to know where the candidates of the list resulting from the first part came from.

2. It is important to keep the overall design as simple as possible, even if the first part needs to transfer some knowledge to the second part; basically for the first part also changes of the merge list construction are regarded as acceptable.

For the first camp, 2 different methods: I0053 and I0087, both of equivalent complexity and both having the intended advantage.

The second is the current design.

What is the increased complexity of the first methods compared to the current method?

- The current design can better be parallelized

- The second approach is definitely better for software

- The first approach would allow to  re-use an existing building block in hardware. However, a new building block has to be designed anyway for the second part of the process. Therefore, this advantage seems to be minor.

Agreement: Keep the current design, no action on I0053 and I0087.

## VSP (3)

(chaired by JRO, 07-04 afternoon)

JCT3V-I0079 DV Derivation for VSP [M. W. Park, J. Y. Lee, I. K. Choi, Y. Cho, C. Kim (Samsung)]

In the current 3D-HEVC, there can have multiple VSP candidates in the merge list. However, every VSP candidate has the same DV and the same DV derivation method for Sub-PU. That is, the VSP candidates in the same merge list are identical each other. To utilize multiple candidates efficiently, it is proposed to use the different DV derivation method for the different VSP candidate. Three methods are proposed in this contribution. And all proposed methods reportedly provide 0.1% bit-saving for coded and synthesized views.

Gain comes mainly in two CG sequences (shark, undodancer)

Additional complexity by computing the mean value, and identifying the identical candidates in the merge list.

JCT3V-I0184 Crosscheck on DV Derivation for VSP (JCT3V-I0079) [S. Shimizu (NTT)] [late]

JCT3V-I0105 Adaptive depth access for VSP enhancement [T. Ikai, T. Tsukuba (Sharp)]

This contribution proposes to change the reference points in VSP from the current always four corner points to four or one point to improve coding efficiency. It is reported that the gain of proposed method is 0.08, 0.09 and 0.05 in texture, video and synthesis respectively.

The contribution of JCT-3V G0035 has proposed one point access per VSP subblock instead of the four corner access in order to simplify the depth access. Then, it was found that one point access was better than four corner access in terms of overall coding efficiency. However in the meeting, the coordination of the point (x+2, y+2) was questioned and the loss of one sequence in GT fly was concerned.  The proposal itself was not adopted because the simplification impact was considered not so large.

In this contribution, we focus on coding improvement aspect and found an improved depth access method.  This proposal addresses (x+2, y+2) coordination issue and GT fly’s coding efficiency concern related to the former contribution.

In the proposed method, the depth access per sub-block is selected depending on the PU as follows.

- Four corner access (If PU is 2Nx2N)

- One center access (Otherwise, PU is not 2Nx2N)

In the discussion, it was pointed out that this method would not give an advantage in terms of complexity reduction. Moreover, different processing depending on block size/shape is rather undesirable, unless justified by sufficient benefit. Since the advantage in terms of compression is relatively low, it was concluded to take no action.

JCT3V-I0187 Crosscheck on Adaptive depth access for VSP enhancement (JCT3V-I0105) [S. Shimizu (NTT)] [late]

## SDC (6)

(chaired by JRO, 07-04 afternoon)

JCT3V-I0075 Segmental prediction for Inter-SDC in 3D-HEVC [K. Zhang, J. An, X. Zhang, H. Huang, J.-L. Lin, S. Lei (MediaTek)]

In this contribution, a segmental prediction method for Inter-SDC is proposed in depth coding. Besides the current Inter-SDC mode, three segmental Inter-SDC modes with 1, 2 and 3 segments are added. A single depth value is assigned to all reconstructed samples in a segment. The experimental results reportedly show that proposed method can achieve 0.4% BD-rate saving for the synthesized views in the common test condition.

Several experts expressed the opinion that this idea is interesting. It was therefore recommended for investigation in a CE. The following aspects should be further studied in this context:

- Usage in combination with intra SDC (where it would target similar depth map characteristics as DMMx)

- Restriction of number of segments to two (instead of three)

(Note: There is a similar proposal on segmental prediction for intra block copy in the screen content coding activity of JCT-VC, see JCTVC-R0117

JCT3V-I0133 Cross check of MediaTek's proposal on Segmental prediction for Inter-SDC in 3D-HEVC(JCT3V-I0075) [J. Zheng (HiSilicon)] [late]

## DBBP (23)

(initially investigated in BoG chaired by S. Yea Sat. morning, review of BoG recommendations chaired by G. Sullivan 07-05 p.m.)

JCT3V-I0194 BoG report on DBBP (Depth Based Block Partitioning) [S. Yea (LGE)]

BoG I0194 met on 07-05 a.m. and reviewed relevant contributions.

They were categorized into 5 technical categories as tabulated below.

DBBP involves applying masking, rather than averaging, for biprediction, using a mask derived from collocated depth map, followed by a spatial filtering of the result. It is only applied to Nx2N and 2NxN prediction types. It was tested (per the CTC) in the context of texture-first operation, so the depth is from the base view rather than the current view.

There were seven relevant contributions.

| Category | Samsung | Qualcomm | MediaTek | RWTH | Sharp | Hisilicon |
| --- | --- | --- | --- | --- | --- | --- |
| Partition Derivation | I0077, I0076 |  | I0097, I0094 | I0056 | I0107 | I0070, I0125 |
| Filter Simplification | I0088 |  | I0094 |  | I0109 | I0124 |
| Memory Requirement | I0078 |  |  |  | I0108 |  |
| Motion/disparity Derivation |  | I0054 |  | I0058 |  |  |

- Partition derivation

  - BoG recommended, and JCT-3V confirmed, adoption of a simplification by the removal of decoder-side partition derivation (e.g. Test1 in I0097 with the proposed WD change) and adoption of syntax condition of the partition mode as per Test2 (the corresponding WD text is also provided in I0097). The simplification was a common element of four proposals. The software and text provided in a revision (-v3) of I0077, which includes harmonization with I0125, will be used.

  - The BoG recommended, and JCT-3V confirmed, adoption of the simplification proposal to use four corner samples rather than all samples to form an average, per JCT3V-I0076.

- Boundary filter simplification

  - The BoG recommended, and JCT-3V confirmed, the filter simplification in option 2 of I0109 (which is a harmonization of three proposals including I0088, I0094, and a prior version of I0109), reducing of the number of filter conditions to be tested and applying a 1D rather than 2D filtering.

- Memory requirements

  - The BoG recommended, and JCT-3V confirmed, adoption of the memory reduction method 2 (disallow DBBP for 8x8 CU) of I0078

- Motion/disparity vector derivation

  - No action. (Further study encouraged.)

JCT3V-I0054 Improved DBBP in 3D-HEVC [Y. Chen, Q. Yu, H. Liu (Qualcomm), S. Ma (PKU)]

JCT3V-I0061 Cross Check of Improved DBBP in 3D-HEVC (JCT3V-I0054) [F. Jäger (RWTH Aachen University)]

JCT3V-I0056 Cleanup of Depth-based Block Partitioning (DBBP) [F. Jäger (RWTH Aachen University)]

JCT3V-I0065 Cross check of cleanup of DBBP (JCT3V-I0056) [P. Merkle (HHI)]

JCT3V-I0058 Segment-wise Prediction (SP) Merge Candidate [F. Jäger (RWTH Aachen University), J. Konieczny, G. Cordara (Huawei Technologies)]

JCT3V-I0170 Crosscheck of segment-wise Prediction (SP) Merge Candidate (JCT3V-I0058) [H. Liu (Qualcomm)] [late]

JCT3V-I0070 Simplification of partition derivation in DBBP [C. Zhu, Y.-H. Zhang (UESTC), Y.-B. Lin, J.-H. Zheng (HiSilicon), J.-J. Song (UESTC)]

JCT3V-I0148 Cross check of Simplification of partition derivation in DBBP (JCT3V-I0070) [M. W. Park, C. Kim (Samsung)] [late]

JCT3V-I0076 Simplification of Threshold Derivation for DBBP and DMM4 [M. W. Park, J. Y. Lee, Y. Cho, C. Kim (Samsung)]

JCT3V-I0138 Crosscheck for JCT3V-I0076 Simplification of Threshold Derivation for DBBP and DMM4 [H. Huang (MediaTek)] [late]

JCT3V-I0077 Partition Derivation for DBBP [M. W. Park, J. Y. Lee, Y. Cho, C. Kim (Samsung)]

JCT3V-I0183 Crosscheck on Partition Derivation for DBBP (JCT3V-I0077) [S. Shimizu (NTT)] [late]

JCT3V-I0088 Simplification of filtering in DBBP and its extension to PU boundary [J. Y. Lee, M. Mishurovskiy, M. W. Park, Y. Cho, C. Kim (Samsung)]

JCT3V-I0137 Crosscheck for JCT3V-I0088, Samsung's Simplification of filtering in DBBP and its extension to PU boundary [X. Zhang (MediaTek)] [late]

JCT3V-I0094 DBBP simplification based on sub-block level processing [X. Zhang, K. Zhang, J. An, H. Huang, J.-L. Lin, S. Lei (MediaTek)]

JCT3V-I0153 Cross check of DBBP simplification based on sub-block level processing (JCT3V-I0094) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-I0097 Removal of the decoder-side DBBP partition derivation process [J.-L. Lin, Y.-W. Chen, X. Zhang, K. Zhang, Y.-W. Huang, S. Lei (MediaTek)]

JCT3V-I0186 Crosscheck on removal of the decoder-side DBBP partition derivation process (JCT3V-I0097) [S. Shimizu (NTT)] [late]

JCT3V-I0107 Partition decision unification for DBBP and VSP [T. Ikai, T. Tsukuba (Sharp)]

JCT3V-I0145 Cross check of Partition decision unification for DBBP and VSP (JCT3V-I0107) [M. W. Park, C. Kim (Samsung)]

JCT3V-I0108 Disparity vector unification for DBBP and VSP [T. Ikai, T. Tsukuba (Sharp)]

JCT3V-I0146 Cross check of Disparity vector unification of DBBP and VSP (JCT3V-I0108) [M. W. Park, C. Kim (Samsung)]

JCT3V-I0109 One dimensional DBBP boundary filtering [T. Ikai, T. Tsukuba (Sharp)]

JCT3V-I0144 Cross check of One dimensional DBBP boundary filtering (JCT3V-I0109) [M. W. Park, C. Kim (Samsung)] [late]

JCT3V-I0189 Cross check for option2 of one dimensional DBBP boundary filtering (JCT3V-I0109) [H. Huang (MediaTek)] [late]

JCT3V-I0124 On DBBP Filtering Simplification [Z. Gu (SCU), J. Zheng (HiSilicon), N. Ling (SCU), P. Zhang (HiSilicon)]

JCT3V-I0165 Crosscheck for JCT3V-I0124, On DBBP Filtering Simplification [X. Zhang, K. Zhang (MediaTek)] [late]

JCT3V-I0125 Bug-fixing of depth-based block partitioning signaling [Y. Lin, J. Zheng (HiSilicon), X. Wang, C. Zhu (UESTC)]

JCT3V-I0163 Cross-check of Bug-fixing of depth-based block partitioning signaling (JCT3V-I0125) [T. Ikai, T. Tsukuba (Sharp)] [late]

## Other (3)

(chaired by JRO, 07-04 afternoon)

JCT3V-I0057 Clipping Operation for DLT Indexes [F. Jäger (RWTH Aachen University)]

This contribution proposes to add a clipping operation when accessing the Depth Lookup Table (DLT) for coding of DC offsets. In B0036 the DLT was proposed as an efficient coding approach for sparse depth maps. In case of transmission errors, predicted DC values and signaled DC offsets may not be aligned anymore and resulting DLT indexes may exceed the defined range of the lookup table. In such cases, the decoder behavior is undefined in the current working draft text. The proposed modification does not have an impact on the performance under CTC.

It is not necessary to take care about transmission errors here, since decoder operations in such context are not normatively specified and better approaches of error concealment may exist.

However, the same case could also appear with bitstreams that are compliant according to current spec, and decoder behaviour would be unspecified in such cases. Therefore, it is necessary to either impose bitstream restrictions, or perform clipping to valid range, as proposed in the contribution. The latter method is supported by several experts (it is also pointed out that similar cases and solutions are applied in MV coding).

It is however pointed out

- That it is desirable to define a fixed value of clipping (as proposed in the contribution)

- That even after clipping, the index value could address an unassigned entry of the DLT, which according to the spec may includes less values than the bit depth of the index would allow.

The preferred solution is padding the unassigned values of the DLT with zero values, which needs to be specified. An alternative approach that was also suggested would be to allow the encoder specifying the value by which the unassigned values are padded with some default (e.g. mean depth value would be  reasonable).

The proponent is requested to provide a text specification including the zero padding.

Decision: Adopt the text of I0057r1, with additional editorial improvement that Idx2DepthValue[ i ] shall be set to zero.

JCT3V-I0176 3D-HEVC: Quantization of Residual Depth Data [H. Sabirin, K. Kawamura, S. Naito (KDDI)] [late]

Residual depth signals by a depth intra prediction have been a target of quantization since the complexity of residual depth signals is not as much as that of texture signals. The depth look-up table is a quantization method with a high complexity. To overcome the complexity issue, in this contribution, the method of quantizing residual depth signals based on an angular prediction modes is introduced. The BD-rate gains under the common test conditions are 0.00%, 0.03%, and 0.01% for texture, video, and synthesis, respectively.

Presentation deck not available.

The proposal does not provide benefit in terms of implementation complexity, since it makes dequantization dependent on the prediction mode, which usually is not directly available at the dequant unit and would need to be stored or that purpose.

No action.

# Non-normative contributions (2)

## Encoder optimization (2)

JCT3V-I0123 Fast Intra SDC coding for 3D-HEVC Intra Coding [Z. Gu (SCU), J. Zheng (HiSilicon), N. Ling (SCU), P.Zhang (HiSilicon)]

In current High Efficiency Video Coding (HEVC)-based depth intra coding, a full RD search list is created and both non-SDC coding and SDC coding are tested for full-RD cost calculation. To speedup the depth intra coding, we propose to test SDC coding for selected candidates modes based on the non-SDC RD cost of each mode. Two methods are proposed. Experiment results show that the proposed methd1 achieves more than 15% All Intra encoding time saving with negligible BD-rate change and method2 achieves more than 10% All Intra encoding time saving with small BD-rate gain under CTC.

Presentation deck not available.

Several experts express support for method 2.

Proponents are requested to provide a flow diagram of method 2 (upload with updated contribution) which could be used along with textual description for the  3D-HEVC TM document.

Decision (SW): Adopt (default in CTC)

JCT3V-I0164 Crosscheck for JCT3V-I0123, Fast Intra SDC coding for 3D-HEVC Intra Coding [X. Zhang, K. Zhang (MediaTek)] [late]

## Rate control (0)

# Plenary Discussions and BoG Reports

## Project development

From joint discussion with parent bodies 07-07 1600-1815:

- decoding aux pictures should be conformant

- Monochrome to be implemented with 4:2:0 and flag that indicates that both chroma components are zero (as version 1)

- Define more generic multiview profile (Multiview Main). Max number of views equal to max layer_id (which is 63), but max 5 levels of dependency (base layer + 4 dependent)

- vertical disparity constraint? Was discussed to have this only for view order index 1 (2-view case)

## BoGs

# Project planning

## General issues for CEs

A preliminary CE description is to be approved at the meeting at which the CE plan is established.

It is possible to define sub-experiments within particular CEs. As a general rule, it was agreed that each CE should be run under the same testing conditions using same software codebase, which should be based on either the ATM or HTM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for experiments were described in the output document JCT3V-F1100.

A deadline of two weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-3V reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the relevant software basis. Exceptions must be justified, discussed on the general JCT-3V reflector, and recorded in the abstract of the summary report.

Final CEs shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

Experiment descriptions should be written in a way such that it is understood as a JCT-3V output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-3V document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

Non-final CE plan documents were reviewed and given tentative approval during the meeting (in some cases with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE is described in an associated output document JCT3V-E110x for CE n, where “n" is the CE number (n = 1, 2, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-3V is not obliged to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-3V members can participate in a CE

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

### Cross check

For cross checking the same steps as described in the section above should be carried out as soon as possible, but should not take more than five working days. Moreover it should be checked if integrated tools correspond to the adopted proposal. Results of cross check should be announced to the reflector. The result-sheet should be made available to the group.

### Procedure if cross check fails or planned delivery data cannot be held

If a planned delivery date cannot be held this should be announced to the reflector.

If the crosscheck fails or the previous integrator has not delivered the software within 3 days or result sheet within 5 days after planned delivery date

- the current integrator should integrate in the last cross-checked version

- the previous integrator falls back to the end of the integration plan

## Common Conditions for 3D Video Coding Experiments

Preferred Common Conditions for experiment testing that are intended to be appropriate for both CEs and other experiments were selected by the group and described in output document JCT3V-G1100.

## Software development

ATM software:

Due to the absence of key developers of this software from the meeting, the remaining bug fixing has to be done and planned within the mandates of the AHG and software coordinator / software specification editing team. It has already been expressed by email that this will be done.

HTM software:

The Integration plan had been developed in a BoG meeting chaired by G. Tech (07-08 18:00)

The plan was confirmed by the plenary 07-09

HTM-12.0 Integration plan (Starting date: July, 10th)

Track 1: Merge, DBBP, others

Coordinator: Hongbin Liu

| Item | Integrator | Days |
| --- | --- | --- |
| JCT3V-I0086 Simplification of shift DV candidate | Samsung | 1 |
| JCT3V-I0077 Partition Derivation for DBBP | Samsung | 1 |
| JCT3V-I0078 Memory Complexity for DBBP and VSP | Samsung | 2 |
| JCT3V-I0109 One dimensional DBBP boundary filtering | Sharp | 2 |
| ?? JCT3V-I0093 Depth coding compatible with arbitrary bit-depth in 3D-HEVC | Mediatek | 1 |

Track 2: ARP, HLS, IC, others

Coordinator: Gerhard Tech

| Item | Integrator | Days |
| --- | --- | --- |
| JCT3V-I0051 CE1: Simplification of advanced residual prediction | Qualcomm | 2 |
| JCT3V-I0129 AHG4: Editorial improvement for ARP specification text in 3D-HEVC | Qualcomm | 1 |
| JCT3V-I0104 CE1: Results of ARP simplification | Sharp | 2 |
| JCT3V-I0099 On the video parameter set extension 2 in 3D-HEVC | Mediatek | 2 |
| JCT3V-I0072 Problem fix for MV scaling in inter-view ARP | Mediatek | 2 |
| JCT3V-I0090 3D HEVC HLS Cleanup | Samsung | 1 |
| JCT3V-I0100 3D-HEVC HLS: General comments on 3D-HEVC high level syntax | Samsung | 1 |
| JCT3V-I0080 Parameter Derivation for Illumination Compensation | Samsung | 2 |

Track 3: DMM, SDC, others

Coordinator: Yi-Wen Chen

| Item | Integrator | Days |
| --- | --- | --- |
| JCT3V-I0095 CE2: Single depth intra mode for 3D-HEVC | Mediatek | 3 |
| JCT3V-I0123 Fast Intra SDC coding for 3D-HEVC Intra Coding | HiSilicon | 2 |
| JCT3V-I0120 CE2: Remove “depth_dc_flag” signalling in DMM cases | HiSilicon | 2 |
| JCT3V-I0110 Lookup table size reduction in DMM1 | Sharp | 2 |
| JCT3V-I0057 Clipping Operation for DLT Indexes | RWTH | 1 |
| JCT3V-I0076 Simplification of Threshold Derivation for DBBP and DMM4 | Samsung | 1 |
| JCT3V-I0084 CE2: Fast SDC DC offset decision | Samsung | 2 |
| JCT3V-I0085 CE2: Separate enabling flag for intra coding tools | Samsung | 2 |
| JCT3V-I0066 CE2: Simplification of DMM4 | HHI | 1 |

HTM-12.1 Integration plan

Coordinator: Gerhard Tech

| Item | Integrator | Days |
| --- | --- | --- |
| JCT3V-I0116 AHG5: Coding results for flexible coding order (FCO) | LGE | 3 |
| MV-HEVC 9 HLS |  |  |

It is to the discretion of the software coordinators to set up a time line for the integration and request proponents to finish integration by a given date.

Integration Procedure & Guidelines

Integration is done in a serial way. Each integrator cross-checks the version provided by his predecessor.  The cross check for the last version is carried out by the software coordinators.

Integration Guidelines

When integrating

- software changes should be enclosed by macros switchable by defines including company and proposal number e.g.

#define MYCOMPANYS_DEPTHFILTER_JCT3V_B0555  1

#if MYCOMPANYS_DEPTHFILTER_JCT3V_B0555

// do stuff

#endif

- new tools should be made switchable in the cfg-file if reasonable

- cfg-files should be updated

Delivery of software

Before delivering the software to the next integrator it should be checked if

- the software compiles under windows and linux

- software compiles and delivers same results as previous version when integrated tools are disabled by macro or cfg-settings

- there are encoder-decoder mismatches

- there are memory leaks by measuring maximum memory consumption (or specific tools e.g. valgrind) is

- visual quality is not disturbed

Additional to the software cfg-files that reflect proposed settings and an excel sheet with coding results should be provided. Software and cfg-files should be delivered by checking it in to the corresponding (HTM or ATM) software repositories.

When software is delivered this should be announced to the reflector. Moreover, every further change on the software should be announced. If there is a delay in integration this should be communicated to the reflector.

## Software repositories

### HTM

HTM software can be checked in

https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/

Therefore for each integrator an own software branch will be created by the software coordinator containing the current anchor version or the version of the previous integrator:

e.g.  branches/0.7-mycompany

The branch of the last integrator will become the new release candidate tag.

e.g. tags/0.8rc1

This tag can be cross-check by the group for. If no problems occur the release candidate will become the new tag after 7-days:

e.g. tags/0.8

If reasonable intermediate release candidate tags can be created by the software coordinator.

### ATM

An official release of the 3DV-ATM software can be checked out by mpeg3dv SVN users from the following location:

http://mpeg3dv.research.nokia.com/svn/mpeg3dv/trunk/

Following every MPEG 3DV meeting, software coordinator creates a new branch for the integration of adopted proposals:

http://mpeg3dv.research.nokia.com/svn/mpeg3dv/branches/

(Note: Would be desirable to rename “mpeg3dv” to “jct3v”)

Software integrator checks-out the software from integration branch at its turn of integration plan and integrates proposal as it is specified in Section 11.3.

Software integrator communicates a new software version over the email list to the cross-checker and to the software coordinator.

The software coordinator checks-in to the integration branch every new software integration with confirmed cross-check, as it is specified in Section 11.3.

Once the integration plan is completed, the software coordinator cross-check version available in the integration branch and checks-in a new official release of the 3DV-ATM to the http://mpeg3dv.research.nokia.com/svn/mpeg3dv/trunk/.

## List of CEs

(chaired by JRO, 07-08 morning)

The following CE was planned:

- CE1 on Segmental Prediction [J. L. Lin]

  - Segmental prediction in Inter SDC (I0075)

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-3V reflector (jct-3v@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-3V project management (AHG1) (jct-3v@lists.rwth-aachen.de) - Coordinate overall JCT-3V interim efforts. - Report on project status to JCT-3V reflector. - Provide report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| MV-HEVC / 3D-HEVC Draft and Test Model editing (AHG2) (jct-3v@lists.rwth-aachen.de) - Produce and finalize JCT3V-I1001 3D-HEVC Draft Text 5. - Produce and finalize JCT3V-I1002 MV-HEVC Draft Text 9. - Produce and finalize JCT3V-I1003 3D-HEVC and MV-HEVC Test Model 9. - Gather and address comments for refinement of these documents. - Coordinate with the 3D-HEVC Software Integration AhG to address issues relating to mismatches between software and text. - Coordinate with AHG6 w.r.t. text improvements in 3D-HEVC specific HLS. | G. Tech, K. Wegner (co-chairs), J. Boyce, Y. Chen, M. Hannuksela, T. Suzuki, S. Yea, J.-R. Ohm, G. Sullivan (vice chairs) | N |
| 3D-AVC Software Integration (AHG3) (jct-3v@lists.rwth-aachen.de) - Coordinate the finalization of the 3DV-ATM software and its distribution to JCT-3V members. - Identify issues (if any) where the software JCT3V-I1005 deviates from the standard or the test model JCT3V-G1003, and clean up the software if necessary. - Produce documentation of software usage for distribution with the software. - Prepare and deliver 3DV-ATM v13.0 software to address all known bugs (expected by 18 July 2014). Prepare and deliver v13.1 before the next meeting in case tht further bugs become known. | D. Rusanovskyy (chair), F. C. Chen, J. Y. Lee, J.-L. Lin, O. Stankiewicz, T. Suzuki, D. Tian (vice chairs) | N |
| MV-HEVC / 3D-HEVC Software Integration (AHG4) (jct-3v@lists.rwth-aachen.de) - Coordinate development of the HTM software and its distribution to JCT-3V members. - Produce documentation of software usage for distribution with the software. - Prepare and deliver HTM-12.0 software version and the reference configuration encodings according to JCT3V-G1100 based on common conditions suitable for in the core experiment (expected within four weeks after the meeting). - Prepare and deliver HTM-12.1 software that include additional items not integrated into the 12.0 version. - Prepare and deliver the WD text of MV-HEVC software JCT3V-I1009. - Perform analysis and reconfirmation checks of the behaviour of technical changes adopted into the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with MV-HEVC Draft and 3D-HEVC Test Model editing AhG to identify any mismatches between software and text. | G. Tech, H. Liu (co-chairs), Y. W. Chen, K. Wegner (vice chairs) | N |
| 3D Coding Verification Testing (AHG5) (jct-3v@lists.rwth-aachen.de) - Prepare a set of test sequences and encodings to be used in upcoming MV-HEVC verification testing. - Identify test sites that could conduct such tests. - Suggest Improvements to the draft test plan JCT-3V-I1011. - Prepare viewing logistics for 10th JCT-3V meeting. | V. Baroncini, K. Müller, S. Shimizu, A. Vetro, S. Yea (co-chairs) | N |
| 3D High level syntax (AHG6) (jct-3v@lists.rwth-aachen.de) - Study video parameter set, sequence parameter set, picture parameter set, and slice header syntax designs for the 3D-HEVC extension. - Study SEI messages and VUI syntax designs needed for the 3D-HEVC extension. - Study and compare mechanisms for enabling diagonal inter-layer prediction. - Study the operation of the Hypothetical Reference Decoder the 3D-HEVC extension. - Assist in software development and text drafting for the high-level syntax in the HEVC extensions designs. | Y. Chen, T. Ikai (co-chairs) | N |
| Complexity assessment (AHG7) (jct-3v@lists.rwth-aachen.de) - Study intrinsic complexity measures or metrics to assess hardware and software implementations of algorithms/computational models based on number of operations, potential for parallelism, data transfer rate, and data storage requirements. - Analyse existing designs and produce complexity perspective per tool, from the viewpoint of possible slowdown, bottlenecks or implementation issues. - Develop a recommendation to JCT-3V on formalized complexity assessment in a reporting document. | C. Lee (chair), G. Bang, T. Ikai, H. Liu (vice chairs) | N |
| 3D Test Material (AHG8) (jct-3v@lists.rwth-aachen.de) - Identify deficiencies and limitations of current 3D video test material. - Identify, collect, and make available a variety of additional 3D video sequences. - Study the characteristics of test materials and their impact on coding performance, synthesis quality etc. - Recommend appropriate test materials for use in 3D Video Coding Extension Development. - Assess the suitability of the test material that was offered to the 5th and 6th JCT-3V meetings. - Assess additional stereo material for an upcoming MV-HEVC verification | S. Shimizu, S. Yea (co-chairs) | N |
| AVC Conformance testing development (AHG9) (jct-3v@lists.rwth-aachen.de) - Further discuss and improve the conformance drafts related to 3D-AVC (JCT3V-I1004). - Collect the conformance test streams. | T. Suzuki (chair), D. Rusanovskyy, D. Tian, Y. W. Chen (vice chairs) | N |
| HEVC Conformance testing development (AHG 10) (jct-3v@lists.rwth-aachen.de) - Further discuss and improve the conformance drafts related to MV-HEVC (JCT3V-I1008) and 3D-HEVC (JCT3V-I1010). - Collect the conformance test streams. | Y. Chen, T. Ikai, S. Shimizu, T. Suzuki (co-chairs) | N |

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate those responsible for document production.

(it was communicated by the chair that only the subsequent lists of adoptions is relevant, and in case of contradiction with any of meeting notes above the latter are considered invalid in first place)

JCT3V-I1000 Meeting Report of 9th JCT-3V Meeting [J.-R. Ohm, G. J. Sullivan] (before next meeting)

(Note: Initial versions of the subsequent draft documents should be uploaded by the end of the meeting, continually updating to be performed until the final the version is released)

JCT3V-I1001 3D-HEVC Draft Text 5 [G. Tech, K. Wegner, Y. Chen, S.Yea] (Text of ISO/IEC 23008-2 DAM4, WG 11 N14701) [2014-08-20]

Summary of Adoptions for 3D-HEVC

- JCT3V-I0051 CE1: Simplification of advanced residual prediction: Method #1

- JCT3V-I0104 CE1: Results of ARP simplification

- JCT3V-I0095 3D-CE2: Single depth intra mode for 3D-HEVC: Test 1 (method with 2 candidates)

- JCT3V-I0110 Lookup table size reduction in DMM1

- JCT3V-I0084 3D-CE2 related: Fast SDC DC offset decision

- JCT3V-I0120 CE2 related: Remove “depth_dc_flag” signalling in DMM case

- JCT3V-I0085 3D-CE2 related: Separate enabling flag for intra coding tools: Retain inter_sdc flag as is; Introduce new dmm4 flag; Change semantics of depth_modes flag such that it controls dmm1 and intra sdc

- JCT3V-I0090 3D HEVC HLS Cleanup: second aspect cases a) and b)

- JCT3V-I0100 3D-HEVC HLS: General comments on 3D-HEVC high level syntax: first and the fourth aspect (the latter without condition)

- JCT3V-I0072 Problem fix for MV scaling in inter-view ARP (editorial)

- Note (to editor): Terms like ‘temporal reference picture’, ‘inter view reference picture’ should be avoided and/or clearly specified.

- JCT3V-I0080 Parameter Derivation for Illumination Compensation: clipping of coordinate

- JCT3V-I0086 Simplification of shift DV candidate:removal of DMV search for shifted DV candidate derivation (in I0086_WD_r1)

- JCT3V-I0057 Clipping Operation for DLT Indexes: text of I0057r1, with additional editorial improvement that Idx2DepthValue[ i ] shall be set to zero.

- JCT3V-I0077 Partition Derivation for DBBP: simplification by the removal of decoder-side partition derivation (e.g. Test1 in I0097 with the proposed WD change) and adoption of syntax condition of the partition mode as per Test2 (the corresponding WD text is also provided in I0097). The simplification was a common element of four proposals. The software and text provided in a revision (-v3) of I0077, which includes harmonization with I0125, will be used.

- JCT3V-I0076 Simplification of Threshold Derivation for DBBP and DMM4: simplification proposal to use four corner samples rather than all samples to form an average, per JCT3V-I0076.

- JCT3V-I0109 One dimensional DBBP boundary filtering: filter simplification in option 2 of I0109 (which is a harmonization of three proposals including I0088, I0094, and a prior version of I0109), reducing of the number of filter conditions to be tested and applying a 1D rather than 2D filtering.

- JCT3V-I0078 Memory Complexity for DBBP and VSP: memory reduction method 2 (disallow DBBP for 8x8 CU)

- JCT3V-I0069 AHG7: Complexity Assessment on Depth Intra Modes with Simplified Data Flows: Integrate suggested editorial changes

- JCT3V-I0093 Depth coding compatible with arbitrary bit-depth in 3D-HEVC: Editorial changes

- JCT3V-I0191 Removal of the 2Nx2N restriction for sub-PU IVMP  (new version 2 with additional modification to be delivered)

- JCT3V-I0129: Editorial improvement for ARP specification text in 3D-HEVC

The list above which had been circulated via the email was confirmed in the plenary 07-09.

JCT3V-I1002 MV-HEVC Draft Text 9 [G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce] (Integrated to ISO/IEC 23008-2:201X, WG 11 N14700) [2014-08-15]

(this text will appear as integrated version in the new edition of HEVC)

(delivery date to be aligned with SHVC)

Summary of Adoptions for MV-HEVC approved by JCT-3V

- JCT3V-I0103 revision 1

- JCT3V-I0134 aspects 1 and 3

- View_id_length (see under JCT3V-I0199)

The following list of adoptions was collected by the editors following the recordings from the joint BoG and the joint meetings, as found in the JCT-VC notes. In case that it does not fully reflect the decisions recommended by the BoG and approved by the joint meeting, the recording of the joint BoG reports and JCT-VC notes are to be regarded as reference. This practice was agreed during the JCT-3V plenary 07-09.

Item

Conformance point specifications, multiview 5-deep (total number of direct and indirect reference layers and the current layer) – agreed

Conformance point specifications, Monochrome (RExt profile ID) – details consistent with RExt – agreed

Conformance point specifications, Requiring decoding of auxiliary pictures if indicated in OLS – confirmed

Definition in terms of enhancement capability (and external base layer), Q6 supporting per-layer PTL spec. – confirmed.

Stereo Main profile Multiview Main profile – confirmed, Remove constraint on vertical displacement? If the view order index = 1 the constraint applies; the remaining ones are not. – agreed

HEVC version 1 compatibility, Presence of layer 0 – agreed to remove requirement.

HEVC version 1 compatibility, Independent non-base layer decoding – agree to define.

HEVC version 1 compatibility, Access unit boundary (definition of "picture"), active parameter sets SEI compatibility (R0274), RawMinCuBits – agree to corrective actions

I0064 On active parameter sets SEI message It was pointed out that the "if payload_extension_present()" condition test, and an associated presence constraint were not necessary and agreed that they should be removed (only an editorial change). Decision: Adopt (as modified).

I0060 Language indication for Overlay information SEI message Editor action item: Connect "UTF-8" to its meaning. Decision: Null-terminated st(v) string of characters encoded according to RFC 5646. Decision: Adopted, RFC 5646, variant 2, byte aligned, 0..255 range. Editors to double-check reference for UTF-8 and improve the reference (or reference a better definition) as necessary.

SEI maturity in current draft/ Overlay information SEI message Decision: The persistence should end within each identified layer when the CLVS of that layer ends. Regarding language identification, see I0060. Parsing depends on the SPS of each associated layer ID Decision: Outside the outer-most loop, just prior to num_overlays_minus1, signal the number of bits minus 8 that is used within the loop for signalling overlay_element_label_min(max). Editor action item: In the definition of st(v), a parenthetical reference to the specification in which this is defined should be added, and further editorial improvement of that description seems desirable.

I0041 Extensibility of nesting SEI messages Editor action item: Editors are suggested to consider adding the proposed note. Decision (BF): For bitstream partition nesting (BSP) SEI message, add an explicit counter to indicate the number of SEI messages in the BSP SEI message, with up to 64 allowed to be nested.

I0047 Signalling and use of HRD parameters for bitstream partitions Decision (BF/Cleanup): Adopt.

I0048 AHG10: Processing of bitstreams without an available base layer Decision: Adopt (conditioned on the relevance of the basis text).

I0035 External Base Proposal 1: When the base layer is externally specified, signalling modifications and constraints for the max_vps_dec_pic_buffering_minus1[ i ][ 0 ][ j ] syntax elements are proposed. Two options for handling this were described. Decision: Adopt "option 1" (not signalling syntax element and inferring its value as 0).

I0035 External Base Layer Proposal 1: When the base layer is externally specified, signalling modifications and constraints for the max_vps_dec_pic_buffering_minus1[ i ][ 0 ][ j ] syntax elements are proposed. Two options for handling this were described. Decision: Adopt "option 1" (not signalling syntax element and inferring its value as 0).

I0034 Non-base layer subtree and auxiliary pictures 1. In order to keep the compatibility of slice headers with nuh_layer_id equal to 0 after non-base layer subtree extraction, poc_lsb_not_present_flag[ i ] shall be equal to 1, when layer_id_in_nuh[ i ] is equal to AssignedBaseLayerId[ j ]. It was remarked that this constraint is effectively already present by the requirement that a rewritten bitstream conforms. Editor action item: The issue is purely editorial and the editors can consider whether some editorial action is desirable to make the constraint impact more clear or explicit.

I0034 Non-base layer subtree and auxiliary pictures 3. An asserted bug fix in highest_layer_idx_plus1 derivation is reported. Decision: Bug confirmed – adopt fix.

I0033 On hybrid codec scalability 3a. Making it a requirement of bitstream conformance that, when vps_base_layer_internal_flag is equal to 0, vps_max_layers_minus1 shall be greater than 0. Decision: Adopt.

I0033 On hybrid codec scalability 3d. Making it a requirement of bitstream conformance that, when vps_base_layer_internal_flag is equal to 0, single_layer_for_non_irap_flag and higher_layer_irap_skip_flag shall be equal to 0. It was agreed that the current text does not handle this issue, but a different solution was desired. Decision (Ed.): Define semantics of these flags in an equivalent way as they would be used with an internal base layer. Editor action item: Check other places in the specification to ensure that they are also well defined to work for an external base layer.

I0042 On POC handle missing pictures and discardable pictures in the same manner as different picture types across layers in requiring POC resetting, Decision: consider adding informative text to address item 1.

I0042 On POC 2.) allow mixing of CRA and BLA pictures without requiring POC resetting (changing language about "different NAL unit type" to IRAP versus non-IRAP); and Decision: Adopt 2

I0042 On POC explicitly spell out a restriction on the value of full_poc_reset_flag based on the value of poc_reset_idc of pictures in the first access unit in the same POC resetting period. Decision: Adopt 3 .

I0092 Bitstream conformance restrictions in subclause C.4 [A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)] Decision (Ed.): Adopted (as modified).

I0029 On invocation of a correct decoding process for the current picture It is proposed that the decoding process of the current picture (subclause 8.1, subclause F.8.1, subclause G.8.1 or subclause H.8.1) is selected on the basis of nuh_layer_id of the current picture and the profile to which the bitstream partition containing the current picture conforms. Decision (Ed.): Adopt items 1 and 3a.

I0029 On invocation of a correct decoding process for the current picture It is proposed that the general decoding process in subclause 8.1 selects TargetPartitioningScheme based on external means or HRD, and if no other means is available TargetPartitioningScheme is set to 0. The BitstreamToDecode derived in subclause 8.1 is proposed to consist of the partitions containing target output layers, the partitions containing the direct and indirect reference layers of the layers in the partitions containing the target output layers and the partition containing the base layer. Decision (Ed.): Regarding Item 2, the intent is for the decoding process to be invoked only for output layers and layer on which they depend.

I0029 AHG10 and On invocation of a correct decoding process for the current picture 3a It is clearly indicated that the processing for the external base-layer pictures is invoked at the beginning of an access unit, and the processing is separated into its own subclause (F.8.1.6).Decision (Ed.): Adopt items 1 and 3a.

I0029 AHG10 and On invocation of a correct decoding process for the current picture It was discussed whether any well-specified constraints are established for a layer that is not indicated to be decoded by any OLS and does not have an associated profile_idc. Decision (BF/Ed.): Prohibit such layers from being present.

I0067 On sub-bitstream extraction Item 4 of the proposal had been withdrawn. Decision: Adopt (as modified).

I0050 Editorial cleanups Decision (BF/Ed.): Adopt.

I0032 On cross-layer impacts of IRAP and EOS It is proposed that if the current picture is the first picture that follows an end of sequence NAL unit (with nuh_layer_id equal to 0) in decoding order, NoClrasOutputFlag is set equal to 1. (This causes marking of all pictures in the DPB as "unused for reference" as well as setting LayerInitializedFlag and FirstPicInLayerDecodedFlag equal to 0 for all layers, which initializes the layer-wise start-up process.) Decision: Adopt this aspect.

I0032 On cross-layer impacts of IRAP and EOS It is proposed that when nuh_layer_id is greater than 0, BLA pictures, CRA pictures with HandleCraAsBlaFlag equal to 1 and IDR pictures with cross_layer_bla_flag equal to 1 cause the following: LayerInitializedFlag and FirstPicInLayerDecodedFlag is set equal to 0 for the current layer and all layers directly or indirectly predicted from the current layer. (Consequently, layer-wise start-up is enforced for these layers.) Each picture in the DPB for the current layer and all layers directly or indirectly predicted from the current layer are marked as "unused for reference". Decision: Adopt this aspect.

I0032 On cross-layer impacts of IRAP and EOS It is proposed to allow EOS NAL unit with nuh_layer_id greater than 0. It is proposed that the decoding of the EOS NAL unit with nuh_layer_id greater than 0 causes the same impacts as those above (2a and 2b). Furthermore, for the semantics of SEI messages applying for nuh_layer_id greater than 0, it is proposed that EOS NAL unit with nuh_layer_id greater than 0 is treated similarly to an end of sequence for the persistence of the SEI message. When present, an EOS NAL unit with a particular nuh_layer_id value shall be the last NAL unit with that particular nuh_layer_id value within an access unit, except an end of bitstream NAL unit with that nuh_layer_id value (when present). Refined text was reviewed, as provided in a revision of the document. Item 3 was clarified. Decision: Adopt these aspects (items 3 as modified).

I0032 On cross-layer impacts of IRAP and EOS A new item 4 was added, regarding the derivation of NoOutputOfPriorPicsFlag for a INBL and its dependent layers. Decision: Adopt these aspects (item 4).

I0062 HEVCv1/ Conformance cleanups The proposal would be to add PTL information for the base layer sub-bitstream. The PTL seen by a version 1 decoder contains the entire bitstream. The PTL for the 0-th OLS is placed at the beginning of the VPS extension (when the base layer is not provided by external means and there is some enhancement layer). Decision: Adopted (item 1).

I0062 HEVCv1/ Conformance cleanups Clarify that directly nested BP, PT, and DUI SEI messages only apply to the operation points that correspond to base-layer-output-only OLSs. Decision: Adopted (items 3 and 4), with key elements of the text provided in -v2.

I0062 HEVCv1/ Conformance cleanups Clarify the bitstream conformance test process to include three parts as described in subclause 1.7 of the contribution document. Decision: Adopted (items 3 and 4), with key elements of the text provided in -v2.

I0038 Move conformance cropping window parameters to rep_format( ) syntax structure of VPS extension [J. Boyce, D. Hong (Vidyo)] It is proposed to move the signaling of the conformance cropping window parameters of enhancement layers from the SPS to the VPS extension, as part of the rep_format( ) syntax structure. It is asserted that this movement will enable more frequent parameter set sharing.Decision: Adopt.

I0046 Unavailable and useless stuff Decision (Ed.): However, it was agreed, as an editorial matter, to only discuss the conformance requirements of OLSs that contain at least one VCL NAL unit..

I0046 Unavailable and useless stuff Editor action item: Also add a note, to advise encoders to be careful about fixed frame rate and CBR indications in enhancement layers.

I0046 Unavailable and useless stuff Decision (Ed.): Only require conformance of extracted sub-bitstreams corresponding to operation points for which all layers of the operation point (both the "necessary" and "unnecessary" layers for the decoding of the output layers) have VCL data in the bitstream and for which the highest TID of the operation point is a TD in that VCL data.

I0091 More miscellaneous cleanups Clarification of the semantics of access unit delimiter RBSP to be applicable in multi-layer context (text in an attachment) The proposal is to specify that the pic_type syntax element, which identifies the slice types allowed in the current picture, applies to all pictures in the AU.Decision (Ed.): Adopt.

I0091 More miscellaneous cleanups Clarification of the specifications of order of NAL units and coded pictures and their association to access units in both subclauses 7.4.2.4.4 and F.7.4.2.4.4 (text in an attachment) Decision (Ed.): Append "with layer_id equal to 0" to ordering constraints for AUD NAL unit in clause 7. Editor action item: Some other elements are clearly editorial and are delegated to the editors for consideration.+

I0091 More miscellaneous cleanups To specify that for any independent non-base layer the used representation format is the one that is signalled in the active SPS for the layer. Decision (Ed.): Adopt. Editor action item: It was recommended for the editors to also add some note or expression of a requirement that the representation format must be signalled in the active SPS.

I0091 More miscellaneous cleanups To clarify the use of parameters in parameter sets for interpretation of semantics of the frame-field information SEI message, including the following two parts, and it is also suggested to make a systematic check for other SEI messages for the need of similar clarifications. Decision (Ed.): Adopt (both identified aspects, and further similar clarifications if identified by the editors)

I0091 More miscellaneous cleanups To clarify the semantics of the temporal sub-layer zero index SEI message for its use in multi-layer contexts, and to add a syntax element to indicate the number of previous consecutive temporal sub-layer zero pictures that have discardable_flag equal to 1 (text in an attachment). The contribution proposed extending the syntax of the temporal sub-layer zero index SEI message. It was remarked that the proposal seems to change the spirit of what the prior interpretation of the SEI message would be. Editor action item: Add a note to advise encoders that, when using this SEI message, not to set discardable flag = 1 for TL0 pictures that are not RASL, RADL or sub-layer non-reference pictures. Decision (Ed.): Change semantics to clarify that it applies to the pictures of the associated layers.

I0091 More miscellaneous cleanups 6. To change the recovery point SEI message to correct the POC derivation when random access is performed and to clarify/add the POC derivation when layer-switching is performed. Decision: Adopt constraint on presence of the SEI message as described in "Option 1" of the -v2 version of the contribution.

I0049 On access unit boundary detection [Ed. Add clarification – note that the AVC definition of "picture" which is what we were assuming when we wrote version 1, was define a picture as containing all VCL NAL units of the AU. We later changed the definition of "picture" for scalability extension, and this caused the problem.]

I0049 On access unit boundary detection Decision: Adopt the proposed clarification and issue a defect report document that highlights the issue.

I0049 On access unit boundary detection The phrasing in in subclause 7.4.2.4.4 is not explicit whether nuh_layer_id equal to 0 or any nuh_layer_id value is considered in determining the first NAL unit of an access unit. This aspect is a clarification of the prior basis text. Decision (Ed.): Adopt this aspect.

I0044 Modification to semantics of slice_temporal_mvp_enabled_flag.

I0044 Modification to semantics in temporal motion vector prediction constraints SEI message.

I0045Signalling of bit rate and picture rate information for additional layer sets.

I0045 dding a restriction on update of representation format of the base layer.

I0045 Bug fix and simplification for value ranges of num_negative_pics, num_positive_pics, and num_long_term_pics.

I0045 Add semantic constraint of vps_vui_bsp_hrd_present_flag.

I0045 Remove restriction on update of separate_color_plane_flag in SPS.

I0045 It was suggested that when the base layer is internal, we should send the VST also for the base layer along with the other layers (e.g., for session negotiation purposes). Decision: Agreed (send VST for base layer when internal).

I0067 Generalize scalable nesting SEI message so that it can be applied to additional layer sets.

I0067 Editorial cleanups to derivation of BitstreamToDecode in the HRD and semantics of scalable nesting SEI message and the sub-bitstream property SEI message to properly reference the subclauses of F.10.x. Decision: The BoG recommended actions are agreed (see also additional notes for R0276).

I0036 Modify semantics of default_output_layer_idc and output_layer_flag[ i ][ j ] to consistently handle auxiliary pictures.

I0036 Add inference value to alt_output_layer_flag[ i ].

I0037 Modify the semantics of slice_type for I slice_type with respect to IRAP pictures and sps_max_dec_pic_buffering_minus1[ TemporalId ] value to enable inter-layer prediction could be used for pictures with nuh_layer_id > 0.

I0037 Constrain the allowed range for values of output_layer_set_idx_to_vps[ i ].

I0010 JCT-VC AHG report: Layered coding constraint specifications and capability indications Scalable Main and Stereo Main decoders should be capable of decoding Main profile bitstreams (at least when the base layer is not provided by external means). Decision (Ed.): Confirmed (and Scalable Main 10 should decode Main 10).

I0010 JCT-VC AHG report: Layered coding constraint specifications and capability indications When the target output layers of an output layer set do not include the auxiliary pictures (or any such non-target "unnecessary" layers), the bits of the auxiliary/non-target pictures in the extracted bitstream subset would be counted in the CPB buffer flow, but the decoding of the auxiliary pictures would not be part of the associated decoding process requirements. Decision (Ed.): Confirmed.

I0010 JCT-VC AHG report: Layered coding constraint specifications and capability indications When the target output layers of an output layer set do include the auxiliary pictures, the profile specification for (partitions of) the output layer set would need to include decoding process requirements for decoding those pictures. Unless some way is found to specify the necessary decoding process capabilities (e.g., by a profile definition that includes such capabilities), the specification would need to disallow having target output layers of an output layer set that are auxiliary pictures Decision (Ed.): Confirmed.

I0010 JCT-VC AHG report: Layered coding constraint specifications and capability indications It was suggested that the profile/tier/level information for such an output layer set should include all the layers in the output layer set in the CPB operation but not include the "unnecessary" layers in its decoding process requirements. In principle, it was suggested to be desirable to support this possibility (esp. for the future extensibility purpose), and suggested that we should conduct further study to ensure that the specification is consistent with this design intent. Decision (Ed.): Confirmed that this should be supported.

I0010 Layered coding constraint specifications and capability indications Regarding DPB specification (incl. Q0103 / JCT3V-H0034) level limit constraints related to maximum DPB size, the VPS-level DPB needs to have constraints as well as the SPS level having such constraints (each layer in the TargetDecLayerIdList shall obey profile-specific level limit in A.4.1 d, which applies to sps_max_dec_pic_buffering_minus1[ HighestTid ]). Decision (Ed.): Confirmed.

I0010 Layered coding constraint specifications and capability indications The text should clearly specify the following: Values of general_profile_idc for indicating SHVC and MV-HEVC profiles. Decoding requirements specification needs to require decoding (an editorial oversight) If the capabilities of one profile are "nested" within the capabilities of another profile, decoders should be required to recognize that and decode the bitstream. Decision (Ed.): Confirmed. Editors are requested to share general_profile_idc for naturally-nested profiles (e.g., Scalable Main 10 and Scalable Main) as was done in RExt. Monochrome 8 should use the RExt general_profile_idc. Some details of Monochrome 8 are yet to be defined. Define these in a manner consistent with other RExt profiles unless some contribution indicates otherwise. Nesting relationships are to be specified for Main, and Main Still Picture for all three new profiles and for Main 10 for the Scalable Main 10 profile.

I0010 Layered coding constraint specifications and capability indications Make the PPS syntax HEVCv1 compatible, and enable the syntax of SPS with nuh_layer_id > 0 to be either HEVCv1 compatible or not. Decision (Cleanup): Confirmed.

I0010 Layered coding constraint specifications and capability indications Simplify the bitstream rewriting process for independent non-base layers (part of R0042). Decision (Cleanup): Confirmed.

I0010 Layered coding constraint specifications and capability indications Specify independent non-base layer decoding for profiles specified in Annex A. For example, the following phrasing could be used "When expressing the capabilities of a decoder for a profile specified in Annex A, the capability of decoding an independent non-base layer should be expressed." Decision (Cleanup): Confirmed.

I0010 Allow an SHVC/MV-HEVC bitstream to be considered conforming without containing a base layer (i.e., without containing pictures of layer 0), and indicate such a bitstream using the vps_reserved_one_bit (with the syntax element name and semantics changed) Decision (Cleanup): Confirmed.

I0010 Remove the bitstream partition HRD parameters SEI message. Decision (Cleanup): Confirmed.

I0010 Wording improvement is needed for item b of the profile specific level limits, to correctly implement the intent to consider output times of pictures in different AUs. Editor action item: Editors to consider.

I0010 It is noted that the drafted Scalable Main profile does not support monochrome decoding. Decision (Cleanup): Remove constraint prohibiting 4:2:0 for alpha and depth and specify the the semantics of associated SEI messages, if not already specified, to refer to the luma component only. True monochrome will not be required to be supported (parent).

I0010 R0041 is on this topic. Text was requested for using the last of the "44 reserved bits" for this indicator. The text was provided in a revision of R0041 and reviewed Minor comments included harmonization with RExt, including the flag for sub-layers, requiring the flag to be equal to zero if the associated layer ID is zero and otherwise equal to 1, clarifying the scope of the flag to CLVS scope, having the rewriting process set the flag to 0, and making the value of the flag an exception to the requirement for the PTL in the SPS to match that of the VPS for an INBL. (The question of whether the rewriting process is removed or not is an editorial matter, and it seems that we may retain it for convenience while providing additional informative text.) Decision: Confirmed (modified as suggested above for 07-06 review).

I0010 Should some additional adjustment be made of the syntax of independent non-base layers in regard, e.g., to VPS repetition and handling of end of sequence and end of bitstream NAL units? Consider allowing VPS presence in independent non-base layer (text preparation requested). Check whether other PSs can be repeated in other layers. Decision (Cleanup): It was agreed to allow this for all PSs (observing that SPSs in layer 0 must use compatible syntax). Revisit for text review. Decision (Ed.): Regarding ordering of pictures within one AU, it is intended to require the VCL NAL units of the picture to be in the order of increasing layer ID value.

I0199 BoG report on editor’s notes Editor action item: Most aspects were suggested to be clearly editorial and were delegated to the editors for consideration. Item 2 relates to ordering of pictures in an AU, which was found in Annex F. This seemed adequate, so the suggested change may be unnecessary.

I0022 SHVC/MV-HEVC level definitions and related decoder capability requirements Decision: The intent is agreed as stated above; the editors are delegated the task of expressing that in the language of the text.

This table was presented during the closing plenary and no complaints were made.

JCT3V-I1003 Test Model 9 of 3D-HEVC and MV-HEVC [Y. Chen, G. Tech, K. Wegner, S. Yea] (WG 11 N14704) [2014-08-20]

Bug fixes:

- …

Software:

- JCT3V-I0066 CE2 related: Simplification of DMM4: Adopt encoder modification proposed in I0066. For threshold derivation simplification, consider a unified solution with DBBP (see also I0076)

- JCT3V-I0099 On the video parameter set extension 2 in 3D-HEVC: Align the software to the WD text (version I0099WDr1)

- JCT3V-I0072 Problem fix for MV scaling in inter-view ARP (bugfix/software)

- JCT3V-I0123 Fast Intra SDC coding for 3D-HEVC Intra Coding:  (default in CTC)

- JCT3V-I0116 AHG5: Coding results for flexible coding order (FCO): Include the bug fixes in the HTM software.

JCT3V-H1004 Draft 4 of 3D-AVC Conformance

Remains valid

JCT3V-I1005 Draft 4 of 3D-AVC Reference Software [D. Rusanovskyy, D. Tian] (Study of ISO/IEC 14496-5:2001 DAM35, WG 11 N14673) [2014-08-31]

JCT3V-I1006 Draft 5 of MVC+D Reference Software [D. Rusanovskyy, T. Suzuki, D. Tian] (ISO/IEC 14496-5:2001 FDAM33, WG 11 N14671) [2014-08-31]

JCT3V-I1007 Draft 1 of MFC plus Depth and Texture/Depth View Packing SEI Message [P. Yin, T. Senoh, M. Hannuksela, Y. Chen, J.-R. Ohm, G. J. Sullivan] (ISO/IEC 14496-10:201X PDAM1, WG11 N14677) [2014-08-01]

JCT3V-I1008 MV-HEVC Conformance Draft 1 [Y. Chen, S. Shimizu, T. Suzuki] (WG11 N14714) [14-07-25]

(not containing streams yet)

JCT3V-I1009 MV-HEVC Software Draft 1 [G. Tech] (WG11 N14710) [14-10-10]

JCT3V-I1010 3D-HEVC Conformance Draft 1 [Y. Chen, T. Ikai, T. Suzuki] (WG11 N14715) [14-07-25]

(not containing streams yet)

JCT3V-I1011 MV-HEVC Draft Verification Test Plan v1 [S. Shimizu] (WG11 N14707) [14-07-10]

- Coded results with new Telekom and the sequences that were used in MVC+D testing prepared for next meeting

- Investigation of suitability, selection of test cases by next meeting

- Identify test labs and time schedule for performing the tests by next meeting

- Availability of results: 15-02 or 15-06

JCT3V-F1012 Draft 2 of MFC Conformance [P. Yin, D. Tian]

(remains valid, text unchanged, delivered for ISO Amd. and ITU consent)

JCT3V-F1013 Draft 2 of MFC Reference Software [P. Yin, D. Tian]

(remains valid, text unchanged, delivered for ISO Amd. and ITU consent)

JCT3V-G1100 Common Test Conditions of 3DV Core Experiments remains valid

JCT3V-I1101 Description of Core Experiment 1 (CE1) on Segmental Prediction [J.-L. Lin]

# Future meeting plans, expressions of thanks, and closing of the meeting

The document upload deadline for the 10th meeting of the JCT-3V will be Oct 10, 2014, 2359 MET (Geneva/Paris time zone). Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Wednesday or Thursday or Friday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting);

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Saturday prior to such meetings and closing it on the last day of the WG 11 meeting);

- In cases where JCT-3V meets during the first week of the SG16 meeting under ITU-T auspices, and co-located with an MPEG meeting at a nearby meeting place, the meeting dates could also be approximately aligned with the MPEG meeting.

Some specific future meeting plans were established as follows:

- 18-24 Oct. 2014 under WG 11 auspices in Strasbourg, FR

- 12-18 Feb. 2015 under ITU-T auspices in Geneva, CH

- 20-26 June 2015 under WG 11 auspices in Warsaw, PL

- 15-21 Oct. 2015 under ITU-T auspices in XXXX, XX

Note: Depending on expectations about future work load, shorter meeting durations may be considered starting from the 11th meeting.

The Ministry of International Affairs and Communications of Japan was thanked for the excellent hosting of the 9th meeting of the JCT-3V under the auspices of ITU-T SG 16.

NICT was thanked for providing equipment used in viewing 3D video material.

It was also reminded that final output documents (if also registered under a WG11 output doc number) have to be uploaded separately with a WG11 header.  To do this in a timely fashion is particularly important for standards submitted for ballot.

It was also reminded that slide presentation decks should be made available.

The JCT-3V meeting was closed at approximately 1320 hours on Wednesday 9 July 2014.

# Annex A to JCT-3V report:
List of documents

| JCT3V-I0001 | m34440 | 2014-07-02 16:35:50 | 2014-07-02 16:53:08 | 2014-07-03 15:24:43 | JCT-3V AHG Report: JCT-3V project management (AHG1) | J.-R. Ohm, G. J. Sullivan |
| --- | --- | --- | --- | --- | --- | --- |
| JCT3V-I0002 | m34407 | 2014-07-02 03:43:43 | 2014-07-02 18:21:22 | 2014-07-02 18:21:22 | JCT-3V AHG Report: MV-HEVC / 3D-HEVC Draft and Test Model editing (AHG2) | G. Tech, K. Wegner, J. Boyce, Y. Chen, T. Suzuki, S. Yea, J.-R. Ohm, G. Sullivan |
| JCT3V-I0003 | m34405 | 2014-07-02 02:43:56 | 2014-07-03 05:02:31 | 2014-07-03 05:02:31 | JCT-3V AHG Report: 3D-AVC Software Integration (AHG3) | D. Rusanovskyy, F. C. Chen, J. Y. Lee, J.-L. Lin, O. Stankiewicz, T. Suzuki, D. Tian |
| JCT3V-I0004 | m34406 | 2014-07-02 02:51:24 | 2014-07-03 01:22:16 | 2014-07-03 01:22:16 | JCT-3V AHG Report: MV-HEVC and 3D-HEVC Software Integration (AHG4) | G. Tech, H. Liu, Y.W. Chen, K. Wegner |
| JCT3V-I0005 | m33907 | 2014-06-23 09:49:37 | 2014-06-30 10:49:16 | 2014-06-30 10:49:16 | JCT-3V AHG Report: 3D Coding Tool Testing (AHG5) | K. Müller (Fraunhofer HHI), A. Vetro (MERL), S. Yea (LG Electronics) |
| JCT3V-I0006 | m34435 | 2014-07-02 15:56:50 | 2014-07-02 15:57:52 | 2014-07-02 15:57:52 | JCT-3V AHG report: 3D High level syntax (AHG6) | J. Boyce, M. M. Hannuksela (co-chairs), Y. Chen, S. Deshpande, J. Samuelsson, G. Tech, Y.-K. Wang, P. Wu (vice-chairs) |
| JCT3V-I0007 | m34441 | 2014-07-02 17:38:16 | 2014-07-02 17:39:37 | 2014-07-03 05:48:11 | JCT-3V AHG Report: Complexity Assessment (AHG7) | G. G. Lee (NCKU), G. Bang (ETRI), T. Ikai (Sharp), K. Rapaka (Qualcomm) |
| JCT3V-I0008 | m34452 | 2014-07-03 03:15:56 | 2014-07-03 03:16:32 | 2014-07-03 03:16:32 | JCT-3V AHG Report: 3D Test Material (AHG8) | S. Shimizu, S. Yea |
| JCT3V-I0009 | m34412 | 2014-07-02 07:55:59 | 2014-07-02 14:35:14 | 2014-07-02 14:35:14 | JCT-3V AHG Report: Conformance testing development (AHG9) | T. Suzuki, D. Rusanovskyy, D. Tian, Y. W. Chen |
| JCT3V-I0010 | m33667 | 2014-06-14 05:59:58 | 2014-06-14 06:01:22 | 2014-06-21 01:57:05 | JCT-3V AHG report: Layered coding constraint specifications and capability indications (AHG10) | G. J. Sullivan (coordinator) |
| JCT3V-I0011 | m34448 | 2014-07-03 00:14:09 | 2014-07-03 00:15:14 | 2014-07-03 00:15:14 | CE1 Summary Report: Residual Prediction | H. Liu, T. Ikai |
| JCT3V-I0012 | m34418 | 2014-07-02 10:30:39 | 2014-07-03 02:05:07 | 2014-07-03 07:03:19 | CE2 Summary Report on Depth Intra Coding | Y. Chen, Y. W. Chen |
| JCT3V-I0021 | m33587 | 2014-04-30 18:14:26 | 2014-04-30 18:27:38 | 2014-05-23 10:44:40 | AHG10: Profile indication for additional layer sets | M. M. Hannuksela (Nokia) |
| JCT3V-I0022 | m33589 | 2014-05-01 07:51:42 | 2014-05-01 21:54:00 | 2014-07-08 10:45:55 | SHVC/MV-HEVC level definitions and related decoder capability requirements | A. K. Ramasubramonian, Y.-K. Wang, K. Rapaka, Hendry (Qualcomm), M. M. Hannuksela (Nokia) |
| JCT3V-I0023 | m33590 | 2014-05-01 16:41:20 | 2014-05-01 16:43:45 | 2014-06-06 04:03:03 | Use of general_profile_space for auxiliary picture profile indication | J. Boyce (Vidyo) |
| JCT3V-I0024 | m33592 | 2014-05-15 15:12:02 | 2014-05-15 15:16:40 | 2014-05-15 15:16:40 | AHG10: On decoder capability indication | M. M. Hannuksela (Nokia) |
| JCT3V-I0025 | m33593 | 2014-05-19 21:11:58 | 2014-05-19 21:35:40 | 2014-06-24 17:19:41 | AhG4: Software Improvements on 3DVATM | Dong Tian, Anthony Vetro (MERL), Shinya Shimizu (NTT) |
| JCT3V-I0026 | m33596 | 2014-05-23 15:21:50 | 2014-05-23 15:25:28 | 2014-05-23 15:25:28 | AHG10: Comments on JCTVC-R0043/JCT3V-I0022 | M. M. Hannuksela (Nokia) |
| JCT3V-I0027 | m33663 | 2014-06-11 10:26:57 | 2014-06-11 10:43:22 | 2014-06-11 10:43:22 | MV-HEVC/SHVC HLS: VPSs with nuh_layer_id greater than 0 | M. M. Hannuksela (Nokia) |
| JCT3V-I0028 | m33674 | 2014-06-17 08:37:18 | 2014-06-17 08:42:50 | 2014-06-17 08:42:50 | Picture size related constraints and level limits | S. Deshpande, Y.-K. Wang, Hendry, A. K. Ramasubramonian, M. M. Hannuksela |
| JCT3V-I0029 | m33678 | 2014-06-17 16:11:11 | 2014-06-17 16:59:34 | 2014-06-21 10:13:03 | AHG10 and MV-HEVC/SHVC HLS: On invocation of a correct decoding process for the current picture | M. M. Hannuksela (Nokia) |
| JCT3V-I0030 | m33679 | 2014-06-17 16:54:42 | 2014-06-17 17:00:55 | 2014-06-17 17:00:55 | AHG10: Comments on JCTVC-R0041/JCT3V-I0023 | M. M. Hannuksela (Nokia) |
| JCT3V-I0031 | m33689 | 2014-06-19 19:08:19 | 2014-06-19 19:12:24 | 2014-06-19 19:12:24 | On monochrome coding (concerning multi-layer profiles and auxiliary pictures) | M. M. Hannuksela (Nokia) |
| JCT3V-I0032 | m33699 | 2014-06-19 21:01:00 | 2014-06-21 10:26:14 | 2014-07-06 08:02:56 | MV-HEVC/SHVC HLS: On cross-layer impacts of IRAP and EOS | M. M. Hannuksela (Nokia) |
| JCT3V-I0033 | m33759 | 2014-06-20 15:00:24 | 2014-06-22 10:36:36 | 2014-06-23 13:12:37 | MV-HEVC/SHVC HLS: Improvements and clean-ups on external base layer | B. Choi, C. Kim (Samsung) |
| JCT3V-I0034 | m33760 | 2014-06-20 15:01:36 | 2014-06-23 13:12:55 | 2014-06-23 13:12:55 | MV-HEVC/SHVC HLS: Non-base layer subtree and auxiliary pictures | B. Choi, C. Kim (Samsung) |
| JCT3V-I0035 | m33790 | 2014-06-20 20:48:43 | 2014-06-21 04:30:28 | 2014-06-21 04:30:28 | MV-HEVC/SHVC HLS: On External Base Layer | S. Deshpande (Sharp) |
| JCT3V-I0036 | m33791 | 2014-06-20 20:49:15 | 2014-06-21 04:30:47 | 2014-07-02 15:51:06 | MV-HEVC/SHVC HLS: On Alternative Output Layer and Default Output Layer Indication | S. Deshpande (Sharp) |
| JCT3V-I0037 | m33792 | 2014-06-20 20:49:46 | 2014-06-21 04:31:15 | 2014-06-21 04:31:15 | MV-HEVC/SHVC HLS: Comments on SHVC and MV-HEVC | S. Deshpande (Sharp) |
| JCT3V-I0038 | m33796 | 2014-06-20 22:04:20 | 2014-06-20 22:08:30 | 2014-06-20 22:08:30 | MV-HEVC/SHVC HLS: Move conformance cropping window parameters to rep_format( ) syntax structure of VPS extension | J. Boyce, D. Hong (Vidyo) |
| JCT3V-I0039 | m33797 | 2014-06-20 22:04:54 | 2014-06-20 23:29:15 | 2014-06-20 23:29:15 | Modifications to Scalable Main profile and general and profile-specific level limits | J. Boyce (Vidyo) |
| JCT3V-I0040 | m33799 | 2014-06-20 22:18:40 | 2014-06-20 22:34:34 | 2014-07-03 11:21:07 | HEVC/MV-HEVC/SHVC HLS: SEI message recommended by MMT for improvement of HEVC packet loss resilience | M. Sychev, S. Ikonin (Huawei) |
| JCT3V-I0041 | m33863 | 2014-06-21 05:21:35 | 2014-06-21 10:20:18 | 2014-06-21 10:20:18 | HEVCv1/MV-HEVC/SHVC HLS: On extensibility of nesting SEI messages | A. K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm) |
| JCT3V-I0042 | m33866 | 2014-06-21 05:35:43 | 2014-06-21 09:25:34 | 2014-06-21 09:25:34 | MV-HEVC/SHVC HLS: On POC | Hendry, A. K. Ramasubramonian, Y.-K. Wang (Qualcomm) |
| JCT3V-I0043 | m33867 | 2014-06-21 05:36:20 | 2014-06-21 10:06:22 | 2014-06-21 10:06:22 | MV-HEVC/SHVC HLS: Clarifications to the specification of end of sequence (EOS) NAL unit | A. K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm) |
| JCT3V-I0044 | m33871 | 2014-06-21 05:43:34 | 2014-06-21 09:28:14 | 2014-06-21 09:28:14 | MV-HEVC/SHVC HLS: On indication of the use of temporal motion vector prediction | Hendry, A. K. Ramasubramonian, Y.-K. Wang (Qualcomm) |
| JCT3V-I0045 | m33873 | 2014-06-21 06:27:21 | 2014-06-21 09:03:20 | 2014-07-10 06:18:50 | MV-HEVC/SHVC HLS: Miscellaneous cleanups | Y.-K. Wang, Hendry, A. K. Ramasubramonian (Qualcomm) |
| JCT3V-I0046 | m33877 | 2014-06-21 08:17:28 | 2014-06-21 10:07:21 | 2014-06-21 10:07:21 | MV-HEVC/SHVC HLS: Unavailable and useless stuff | Y.-K. Wang, A. K. Ramasubramonian, Hendry (Qualcomm) |
| JCT3V-I0047 | m33878 | 2014-06-21 08:36:55 | 2014-06-22 08:04:12 | 2014-06-22 08:04:12 | MV-HEVC/SHVC: Signalling and use of HRD parameters for bitstream partitions | Y.-K. Wang, Hendry, A. K. Ramasubramonian (Qualcomm), S. Deshpande (Sharp), M. M. Hannuksela (Nokia) |
| JCT3V-I0048 | m33884 | 2014-06-21 09:29:54 | 2014-06-21 10:10:19 | 2014-06-21 10:10:19 | AHG10: Processing of bitstreams without an available base layer | M. M. Hannuksela (Nokia) |
| JCT3V-I0049 | m33886 | 2014-06-21 09:35:14 | 2014-06-21 10:02:58 | 2014-06-21 10:02:58 | MV-HEVC/SHVC HLS: On access unit boundary detection | M. M. Hannuksela (Nokia) |
| JCT3V-I0050 | m33888 | 2014-06-21 10:21:31 | 2014-06-21 11:14:53 | 2014-06-21 11:14:53 | MV-HEVC/SHVC HLS: Editorial cleanups | A. K. Ramasubramonian, Y.-K. Wang, Hendry (Qualcomm) |
| JCT3V-I0051 | m33891 | 2014-06-21 16:03:05 | 2014-06-26 09:18:55 | 2014-06-26 09:18:55 | CE1: Simplification of advanced residual prediction | H. Liu, Y. Chen (Qualcomm) |
| JCT3V-I0052 | m33892 | 2014-06-21 16:03:59 | 2014-06-26 16:32:41 | 2014-06-26 16:32:41 | CE1 related: Alignment of disparity vector in inter-view motion and residual predictions for ARP | H. Liu, Y. Chen (Qualcomm) |
| JCT3V-I0053 | m33893 | 2014-06-21 16:06:50 | 2014-06-26 16:35:53 | 2014-06-26 16:35:53 | Simplifications for 3D-HEVC merge candidate list generation | H. Liu, Y. Chen (Qualcomm) |
| JCT3V-I0054 | m33894 | 2014-06-21 16:09:06 | 2014-06-26 09:23:34 | 2014-06-26 09:23:34 | Improved DBBP in 3D-HEVC | Y. Chen, Q. Yu, H. Liu (Qualcomm), S. Ma (PKU) |
| JCT3V-I0055 | m33895 | 2014-06-21 16:11:32 | 2014-06-26 09:16:37 | 2014-07-04 07:33:59 | AHG4: Semantics clean-up for advanced residual prediction weighting factor | Y. Chen, L. Zhang (Qualcomm) |
| JCT3V-I0056 | m33911 | 2014-06-23 17:04:29 | 2014-06-24 09:33:02 | 2014-06-24 09:33:02 | Cleanup of Depth-based Block Partitioning (DBBP) | F. Jäger (RWTH Aachen University) |
| JCT3V-I0057 | m33912 | 2014-06-23 17:05:21 | 2014-06-24 09:28:02 | 2014-07-04 09:51:09 | Clipping Operation for DLT Indexes | F. Jäger (RWTH Aachen University) |
| JCT3V-I0058 | m33913 | 2014-06-23 17:06:26 | 2014-06-26 22:56:05 | 2014-07-04 11:11:22 | Segment-wise Prediction (SP) Merge Candidate | F. Jäger (RWTH Aachen University), J. Konieczny, G. Cordara (Huawei Technologies) |
| JCT3V-I0059 | m33923 | 2014-06-23 19:15:59 | 2014-06-23 19:41:15 | 2014-07-06 08:29:54 | MV-HEVC/SHVC HLS: On persistence of SEI messages | M. M. Hannuksela (Nokia) |
| JCT3V-I0060 | m33926 | 2014-06-23 20:42:06 | 2014-06-23 20:45:21 | 2014-07-06 09:08:24 | Language indication for Overlay information SEI message | K. Suehring (Fraunhofer HHI) |
| JCT3V-I0061 | m33935 | 2014-06-24 09:39:49 | 2014-06-26 09:42:28 | 2014-06-26 09:42:28 | Cross Check of Improved DBBP in 3D-HEVC (JCT3V-I0054) | F. Jäger (RWTH Aachen University) |
| JCT3V-I0062 | m33947 | 2014-06-25 01:27:36 | 2014-06-25 01:32:07 | 2014-07-07 05:32:58 | HEVCv1/MV-HEVC/SHVC HLS: Conformance cleanups | Y.-K. Wang, A. K. Ramasubramonian, Hendry (Qualcomm) |
| JCT3V-I0063 | m33949 | 2014-06-25 03:27:17 | 2014-06-25 03:34:24 | 2014-06-25 03:34:24 | Software Improvement for Texture and Depth View Packing SEI Message | Takanori Senoh, Koki Wakunami, Yasuyuki Ichihashi, Hisayuki Sasaki, Ryutaro Oi, Kenji Yamamoto |
| JCT3V-I0064 | m33952 | 2014-06-25 10:37:36 | 2014-06-25 13:24:39 | 2014-07-07 13:08:31 | MV-HEVC/SHVC HLS: On active parameter sets SEI message | M. M. Hannuksela (Nokia) |
| JCT3V-I0065 | m33956 | 2014-06-25 18:39:38 | 2014-06-25 18:42:06 | 2014-06-25 18:42:06 | Cross check of cleanup of DBBP (JCT3V-I0056) | P. Merkle (HHI) |
| JCT3V-I0066 | m33957 | 2014-06-25 18:43:44 | 2014-06-26 11:57:34 | 2014-06-26 11:57:34 | CE2 related: Simplification of DMM4 | P. Merkle, D. Marpe (HHI) |
| JCT3V-I0067 | m33959 | 2014-06-25 19:34:58 | 2014-06-25 19:39:08 | 2014-07-06 09:16:24 | MV-HEVC/SHVC HLS: On sub-bitstream extraction | M. M. Hannuksela (Nokia) |
| JCT3V-I0068 | m33960 | 2014-06-25 21:58:30 | 2014-06-27 09:59:55 | 2014-06-27 09:59:55 | Cross Check of CE2 related: Simplification of DMM4 (JCT3V-I0066) | F. Jäger (RWTH Aachen University) |
| JCT3V-I0069 | m33966 | 2014-06-26 04:13:32 | 2014-06-26 10:52:36 | 2014-07-04 13:12:41 | AHG7: Complexity Assessment on Depth Intra Modes with Simplified Data Flows | C.-F. Chen, C.-H. Huang, G. G. Lee (NCKU/ITRI) |
| JCT3V-I0070 | m33967 | 2014-06-26 05:27:44 | 2014-06-26 13:57:08 | 2014-07-02 03:07:38 | Simplification of partition derivation in DBBP | C. Zhu, Y. Zhang (UESTC), Y. Lin, J. Zheng (HiSilicon), J. Song (UESTC) |
| JCT3V-I0071 | m33968 | 2014-06-26 05:28:56 | 2014-07-02 03:02:59 | 2014-07-02 03:02:59 | Cross check of alignment of disparity vector in inter-view motion and residual predictions for ARP (JCT3V-I0052) | Y. Zhang, Y. Lin (HiSilicon) |
| JCT3V-I0072 | m33969 | 2014-06-26 05:37:06 | 2014-06-26 11:04:45 | 2014-06-26 11:04:45 | Problem fix for MV scaling in inter-view ARP | J. An, K. Zhang, X. Zhang, H. Huang, J.-L. Lin, S. Lei (MediaTek) |
| JCT3V-I0073 | m33970 | 2014-06-26 05:39:00 | 2014-06-26 15:35:52 | 2014-07-05 02:37:15 | MV sharing for depth coding in 3D-HEVC | K. Zhang, J. An, X. Zhang, H. Huang, J.-L. Lin, S. Lei (MediaTek) |
| JCT3V-I0074 | m33971 | 2014-06-26 05:39:15 | 2014-06-26 15:36:45 | 2014-06-26 15:36:45 | Depth compensation calibration in 3D-HEVC | K. Zhang, J. An, X. Zhang, H. Huang, J.-L. Lin, S. Lei (MediaTek) |
| JCT3V-I0075 | m33972 | 2014-06-26 05:40:14 | 2014-06-26 15:37:55 | 2014-06-26 15:37:55 | Segmental prediction for Inter-SDC in 3D-HEVC | K. Zhang, J. An, X. Zhang, H. Huang, J.-L. Lin, S. Lei (MediaTek) |
| JCT3V-I0076 | m33973 | 2014-06-26 06:28:41 | 2014-06-26 17:14:07 | 2014-07-03 04:36:16 | Simplification of Threshold Derivation for DBBP and DMM4 | M. W. Park, J. Y. Lee, Y. Cho, C. Kim (Samsung) |
| JCT3V-I0077 | m33974 | 2014-06-26 06:28:58 | 2014-06-26 17:14:54 | 2014-07-05 05:38:45 | Partition Derivation for DBBP | M. W. Park, J. Y. Lee, Y. Cho, C. Kim (Samsung) |
| JCT3V-I0078 | m33975 | 2014-06-26 06:29:07 | 2014-06-26 17:36:20 | 2014-07-04 03:18:50 | Memory Complexity for DBBP and VSP | M. W. Park, J. Y. Lee, Y. Cho, C. Kim (Samsung) |
| JCT3V-I0079 | m33976 | 2014-06-26 06:29:16 | 2014-06-26 17:37:05 | 2014-07-04 03:19:24 | DV Derivation for VSP | M. W. Park, J. Y. Lee, I. K. Choi, Y. Cho, C. Kim (Samsung) |
| JCT3V-I0080 | m33977 | 2014-06-26 06:29:30 | 2014-06-26 17:37:35 | 2014-07-03 04:43:01 | Parameter Derivation for Illumination Compensation | M. W. Park, J. Y. Lee, Y. Cho, C. Kim (Samsung) |
| JCT3V-I0081 | m33978 | 2014-06-26 06:29:42 | 2014-06-26 17:15:24 | 2014-07-03 04:35:22 | 3D-CE1 related: Reduction of Worst Case Memory Bandwidth for ARP | M. W. Park, J. Y. Lee, Y. Cho, C. Kim (Samsung) |
| JCT3V-I0082 | m33979 | 2014-06-26 06:30:29 | 2014-06-26 17:34:09 | 2014-06-26 17:34:09 | 3D-CE2: Residual DC quantization in intra SDC | J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung) |
| JCT3V-I0083 | m33980 | 2014-06-26 06:31:16 | 2014-06-26 17:34:39 | 2014-06-26 17:34:39 | 3D-CE2 related: Allowing zero DC candidate in inter SDC | J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung) |
| JCT3V-I0084 | m33981 | 2014-06-26 06:31:31 | 2014-06-26 17:35:02 | 2014-06-26 17:35:02 | 3D-CE2 related: Fast SDC DC offset decision | J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung) |
| JCT3V-I0085 | m33982 | 2014-06-26 06:31:44 | 2014-06-26 17:35:27 | 2014-06-26 17:35:27 | 3D-CE2 related: Separate enabling flag for intra coding tools | J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung) |
| JCT3V-I0086 | m33983 | 2014-06-26 06:31:58 | 2014-06-26 17:35:53 | 2014-07-07 07:03:20 | Simplification of shift DV candidate | J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung) |
| JCT3V-I0087 | m33984 | 2014-06-26 06:32:16 | 2014-06-26 17:36:30 | 2014-06-26 17:36:30 | Simplification of 3D Merge list construction | J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung), Y. Zhang, L. Yu (Zhejiang University) |
| JCT3V-I0088 | m33985 | 2014-06-26 06:32:29 | 2014-06-26 17:37:02 | 2014-06-26 17:37:02 | Simplification of filtering in DBBP and its extension to PU boundary | J. Y. Lee, M. Mishurovskiy, M. W. Park, Y. Cho, C. Kim (Samsung) |
| JCT3V-I0089 | m33986 | 2014-06-26 06:32:31 | 2014-06-26 17:37:29 | 2014-06-26 17:37:29 | 3D-CE1 related: Diagonal prediction using ARP | J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung) |
| JCT3V-I0090 | m33987 | 2014-06-26 06:32:43 | 2014-06-26 17:37:52 | 2014-06-26 17:37:52 | 3D HEVC HLS Cleanup | Y. Cho, M. W. Park, J. Y. Lee, C. Kim (Samsung) |
| JCT3V-I0091 | m33990 | 2014-06-26 07:29:51 | 2014-06-26 08:49:03 | 2014-07-07 13:17:34 | MV-HEVC/SHVC HLS: More miscellaneous cleanups | Hendry, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm) |
| JCT3V-I0092 | m33991 | 2014-06-26 07:30:33 | 2014-06-26 13:12:18 | 2014-07-06 11:09:54 | MV-HEVC/SHVC HLS: Bitstream conformance restrictions in subclause C.4 | A. K. Ramasubramonian, Y.-K. Wang (Qualcomm) |
| JCT3V-I0093 | m33992 | 2014-06-26 07:42:45 | 2014-06-26 15:39:00 | 2014-06-26 15:39:00 | Depth coding compatible with arbitrary bit-depth in 3D-HEVC | K. Zhang, J. An, X. Zhang, H. Huang, J.-L. Lin, S. Lei (MediaTek) |
| JCT3V-I0094 | m33993 | 2014-06-26 07:51:07 | 2014-06-26 11:08:24 | 2014-06-30 04:19:48 | DBBP simplification based on sub-block level processing | X. Zhang, K. Zhang, J. An, H. Huang, J.-L. Lin, S. Lei (MediaTek) |
| JCT3V-I0095 | m33994 | 2014-06-26 07:51:30 | 2014-06-26 11:12:28 | 2014-07-01 10:09:38 | 3D-CE2: Single depth intra mode for 3D-HEVC | Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek) |
| JCT3V-I0096 | m33995 | 2014-06-26 07:51:32 | 2014-06-26 11:13:31 | 2014-07-03 06:22:24 | On complexity reduction of bi-prediction for advanced residual prediction | Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek) |
| JCT3V-I0097 | m33996 | 2014-06-26 07:51:46 | 2014-06-26 17:27:43 | 2014-06-26 17:27:43 | Removal of the decoder-side DBBP partition derivation process | J.-L. Lin, Y.-W. Chen, X. Zhang, K. Zhang, Y.-W. Huang, S. Lei (MediaTek) |
| JCT3V-I0098 | m33997 | 2014-06-26 07:57:50 | 2014-06-26 17:28:06 | 2014-07-03 06:25:05 | Unification of the merge candidate sets in 3D-HEVC | J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek) |
| JCT3V-I0099 | m33998 | 2014-06-26 07:57:56 | 2014-06-26 11:09:51 | 2014-07-06 05:18:07 | On the video parameter set extension 2 in 3D-HEVC | H. Huang, J. An, J.-L. Lin, K. Zhang, X. Zhang, S. Lei (MediaTek) |
| JCT3V-I0100 | m33999 | 2014-06-26 08:04:32 | 2014-06-27 03:31:39 | 2014-07-03 03:39:27 | 3D-HEVC HLS: General comments on 3D-HEVC high level syntax | B. Choi, M.W. Park, C. Kim (Samsung) |
| JCT3V-I0101 | m34000 | 2014-06-26 08:05:34 | 2014-06-27 03:32:09 | 2014-06-27 03:32:09 | 3D-HEVC HLS: Dependency between texture and depthmap | B. Choi, J.Y. Lee, C. Kim (Samsung) |
| JCT3V-I0102 | m34002 | 2014-06-26 08:52:10 | 2014-06-26 19:17:46 | 2014-07-07 02:09:12 | 3D-AVC defect correction: texture view dependencies missing from SPS | M. M. Hannuksela (Nokia), X. Huang (USTC) |
| JCT3V-I0103 | m34003 | 2014-06-26 09:31:33 | 2014-06-26 12:57:00 | 2014-07-07 10:49:51 | MV-HEVC HLS: On MV-HEVC SEI messages | M. M. Hannuksela (Nokia) |
| JCT3V-I0104 | m34005 | 2014-06-26 10:14:37 | 2014-06-27 06:45:57 | 2014-06-27 06:45:57 | CE1: Results of ARP simplification | T. Ikai (Sharp) |
| JCT3V-I0105 | m34006 | 2014-06-26 10:14:54 | 2014-06-27 06:46:22 | 2014-06-27 06:46:22 | Adaptive depth access for VSP enhancement | T. Ikai, T. Tsukuba (Sharp) |
| JCT3V-I0106 | m34007 | 2014-06-26 10:15:07 | 2014-06-27 06:46:51 | 2014-06-27 06:46:51 | Further VSP inheritance simplification | T. Ikai, T. Tsukuba (Sharp) |
| JCT3V-I0107 | m34008 | 2014-06-26 10:15:27 | 2014-06-27 06:48:05 | 2014-06-27 06:48:05 | Partition decision unification for DBBP and VSP | T. Ikai, T. Tsukuba (Sharp) |
| JCT3V-I0108 | m34009 | 2014-06-26 10:15:53 | 2014-06-27 06:48:37 | 2014-06-27 06:48:37 | Disparity vector unification for DBBP and VSP | T. Ikai, T. Tsukuba (Sharp) |
| JCT3V-I0109 | m34010 | 2014-06-26 10:16:09 | 2014-06-27 06:49:05 | 2014-07-05 06:36:50 | One dimensional DBBP boundary filtering | T. Ikai, T. Tsukuba (Sharp) |
| JCT3V-I0110 | m34011 | 2014-06-26 10:16:26 | 2014-06-27 06:18:02 | 2014-06-27 06:18:02 | Lookup table size reduction in DMM1 | T. Tsukuba, T. Ikai (Sharp) |
| JCT3V-I0111 | m34012 | 2014-06-26 10:16:38 | 2014-06-27 06:49:34 | 2014-07-03 01:38:07 | AHG7: Complexity assessment of inter-prediction tools | T. Ikai (Sharp) |
| JCT3V-I0112 | m34013 | 2014-06-26 10:24:28 | 2014-06-26 14:43:56 | 2014-07-04 08:30:07 | Simplification for VSP-related Merge Candidate List Construction | Y. Zhang, Q. Wang, L. Yu (Zhejiang Univ.) |
| JCT3V-I0113 | m34014 | 2014-06-26 10:25:31 | 2014-06-26 14:44:23 | 2014-06-30 07:08:14 | Crosscheck on simplification of shift DV candidate (JCT3V-I0086) | Y. Zhang (Zhejiang University) |
| JCT3V-I0114 | m34015 | 2014-06-26 10:37:24 | 2014-06-26 10:42:14 | 2014-07-04 08:21:40 | Improvement on merge list construction | S. Yoo, J. Nam, S. Yea (LGE) |
| JCT3V-I0115 | m34016 | 2014-06-26 10:39:01 | 2014-06-26 13:05:04 | 2014-07-05 03:50:16 | AHG5 : Performance evaluation for 3D-HEVC and its coding tools | S. Yoo, S. Yea (LGE) |
| JCT3V-I0116 | m34017 | 2014-06-26 10:40:57 | 2014-06-26 14:04:41 | 2014-07-05 03:47:25 | AHG5: Coding results for flexible coding order (FCO) | J. Nam, S. Yea (LGE) |
| JCT3V-I0117 | m34018 | 2014-06-26 10:42:14 | 2014-06-26 12:54:23 | 2014-07-03 09:31:22 | 3D-CE2 related: Reduction of the number of intra prediction modes for depth coding | J. Heo, J. Nam, S. Yea (LGE) |
| JCT3V-I0118 | m34019 | 2014-06-26 10:43:29 | 2014-06-26 14:00:20 | 2014-07-07 06:37:48 | CE2: Advanced boundary chain coding for depth intra mode | J. Seo, M. Koo, J. Nam, S. Yea (LGE) |
| JCT3V-I0119 | m34020 | 2014-06-26 11:30:07 | 2014-06-26 11:31:54 | 2014-06-26 11:31:54 | Cross check of separate enabling flag for intra coding tools (JCT3V-I0085) | P. Merkle (HHI) |
| JCT3V-I0120 | m34022 | 2014-06-26 11:53:17 | 2014-06-26 15:51:09 | 2014-07-02 12:26:33 | CE2 related: Remove depth_dc_flag signalling in DMM cases | X.Chen, X.Zheng, Y.Lin, J.Zheng (HiSilicon) |
|  |  |  |  |  | WD |  |
| JCT3V-I0122 | m34025 | 2014-06-26 13:56:59 | 2014-06-26 14:00:13 | 2014-06-26 14:00:13 | Cross Check of Method 2 and 3 of Residual DC quantization in intra SDC (JCT3V-I0082) | F. Jäger (RWTH Aachen University) |
| JCT3V-I0123 | m34026 | 2014-06-26 14:26:24 | 2014-06-26 23:52:55 | 2014-07-05 13:46:01 | Fast Intra SDC coding for 3D-HEVC Intra Coding | Z. Gu(SCU), J. Zheng(HiSilicon), N. Ling(SCU), P.Zhang(HiSilicon) |
| JCT3V-I0124 | m34027 | 2014-06-26 14:36:23 | 2014-06-26 23:28:20 | 2014-06-26 23:28:20 | On DBBP Filtering Simplification | Z. Gu (SCU), J. Zheng (HiSilicon), N. Ling (SCU), P. Zhang (HiSilicon) |
| JCT3V-I0125 | m34030 | 2014-06-26 15:16:45 | 2014-06-26 17:17:32 | 2014-07-05 01:09:57 | Bug-fixing of depth-based block partitioning signaling | Y. Lin, J. Zheng (HiSilicon), X. Wang, C. Zhu (UESTC) |
| JCT3V-I0126 | m34031 | 2014-06-26 15:41:39 | 2014-06-26 21:38:39 | 2014-07-08 04:03:08 | Proposed Draft Text of New AVC Amendment: MFC plus Depth, and Texture and Depth View Packing SEI message | P. Yin, T. Lu, H. Ganapathy, T. Chen, W. Husak (Dolby), M. Tanimoto (NISRI), T. Senoh (NICT), Y. Chen (Qualcomm) |
| JCT3V-I0127 | m34032 | 2014-06-26 15:46:38 | 2014-06-26 15:54:33 | 2014-06-26 15:54:33 | On MFC plus Depth: Reference Software Update | L. Grimaldi, K. Schueuer, P. Yin, T. Lu, H. Ganapathy, T. Chen, W. Husak (Dolby) |
|  |  |  |  |  | WD |  |
| JCT3V-I0129 | m34035 | 2014-06-26 16:17:28 | 2014-06-26 22:13:03 | 2014-06-26 22:13:03 | AHG4: Editorial improvement for ARP specification text in 3D-HEVC | L. Zhang, H. Liu, Y. Chen (Qualcomm) |
| JCT3V-I0130 | m34036 | 2014-06-26 16:52:01 | 2014-06-27 00:18:09 | 2014-07-04 06:52:34 | Simplification of VSP candidate derivation | G. Bang(ETRI), M.S. Lee, Y.S. Heo, G.H.Park(KHU), G.S. Lee, N.H.Hur(ETRI) |
| JCT3V-I0131 | m34037 | 2014-06-26 16:57:54 | 2014-06-27 00:19:05 | 2014-07-04 04:41:07 | Adaptive candidate selection | G. Bang(ETRI), Y.S. Heo, G.H.Park(KHU), G.S. Lee, N.H.Hur(ETRI) |
| JCT3V-I0132 | m34039 | 2014-06-26 17:59:14 | 2014-06-26 22:34:37 | 2014-07-06 04:08:33 | Analysis for redundant clippings in illumination compensation | H. Liu, Y. Chen (Qualcomm) |
| JCT3V-I0133 | m34041 | 2014-06-26 18:10:54 | 2014-07-03 00:58:37 | 2014-07-03 00:58:37 | Cross check of MediaTek' proposal on Segmental prediction for Inter-SDC in 3D-HEVC(JCT3V-I0075) | J. Zheng (HiSilicon) |
| JCT3V-I0134 | m34057 | 2014-06-27 08:46:34 | 2014-06-27 09:14:00 | 2014-06-27 09:14:00 | On the Stereo Main profile | M. M. Hannuksela (Nokia) |
| JCT3V-I0135 | m34064 | 2014-06-27 10:07:04 | 2014-07-01 04:04:55 | 2014-07-01 04:04:55 | 3D-CE2: Cross-check of Test 4 of Residual DC quantization in intra SDC (JCT3V-I0082) | T. Tsukuba (Sharp) |
| JCT3V-I0136 | m34067 | 2014-06-27 11:07:36 | 2014-06-27 11:10:47 | 2014-06-27 11:10:47 | Crosscheck results on simplification for VSP-related merge candidate list construction (JCT3V-I0112) | M. LI, P. WU(ZTE) |
| JCT3V-I0137 | m34069 | 2014-06-27 11:24:36 | 2014-07-01 12:52:46 | 2014-07-01 12:52:46 | Crosscheck for JCT3V-I0088, Samsung's Simplification of filtering in DBBP and its extension to PU boundary | X. Zhang (MediaTek) |
| JCT3V-I0138 | m34070 | 2014-06-27 11:24:58 | 2014-07-02 04:16:04 | 2014-07-06 05:17:16 | Crosscheck for JCT3V-I0076 Simplification of Threshold Derivation for DBBP and DMM4 | H. Huang (MediaTek) |
| JCT3V-I0139 | m34071 | 2014-06-27 11:25:15 | 2014-07-01 04:10:19 | 2014-07-01 04:10:19 | Crosscheck for JCT3V-I0053 Simplifications for 3D-HEVC merge candidate list generation | J. An (Mediatek) |
| JCT3V-I0140 | m34072 | 2014-06-27 11:25:29 | 2014-06-30 04:22:39 | 2014-06-30 04:22:39 | 3D-CE2: Crosscheck on advanced boundary chain coding for depth intra mode (JCT3V-I0118) | Y.-W. Chen (MediaTek) |
| JCT3V-I0141 | m34073 | 2014-06-27 11:25:41 | 2014-06-30 04:23:02 | 2014-06-30 04:23:02 | Crosscheck on Improvement on merge list construction (JCT3V-I0114) | Y.-W. Chen (MediaTek) |
| JCT3V-I0142 | m34074 | 2014-06-27 11:25:53 | 2014-07-01 03:41:59 | 2014-07-03 09:55:39 | Crosscheck on lookup table size reduction in DMM1 (JCT3V-I0110) | Y.-W. Chen (MediaTek) |
| JCT3V-I0143 | m34075 | 2014-06-27 11:26:05 | 2014-07-01 03:42:27 | 2014-07-01 03:42:27 | 3D-CE2 related: Crosscheck on fast SDC DC offset decision (JCT3V-I0084) | Y.-W. Chen (MediaTek) |
| JCT3V-I0144 | m34080 | 2014-06-27 14:53:07 | 2014-06-30 09:21:35 | 2014-06-30 09:21:35 | Cross check of One dimensional DBBP boundary filtering (JCT3V-I0109) | M. W. Park, C. Kim (Samsung) |
| JCT3V-I0145 | m34081 | 2014-06-27 14:53:20 | 2014-06-27 15:43:36 | 2014-06-27 15:43:36 | Cross check of Partition decision unification for DBBP and VSP (JCT3V-I0107) | M. W. Park, C. Kim (Samsung) |
| JCT3V-I0146 | m34082 | 2014-06-27 14:53:29 | 2014-06-27 15:44:07 | 2014-06-27 15:44:07 | Cross check of Disparity vector unification of DBBP and VSP (JCT3V-I0108) | M. W. Park, C. Kim (Samsung) |
| JCT3V-I0147 | m34083 | 2014-06-27 14:53:42 | 2014-06-30 03:28:16 | 2014-06-30 03:28:16 | Cross check of On the video parameter set extension 2 in 3D-HEVC (JCT3V-I0099) | M. W. Park, C. Kim (Samsung) |
| JCT3V-I0148 | m34084 | 2014-06-27 14:53:53 | 2014-07-01 05:48:48 | 2014-07-01 05:48:48 | Cross check of Simplification of partition derivation in DBBP (JCT3V-I0070) | M. W. Park, C. Kim (Samsung) |
| JCT3V-I0149 | m34085 | 2014-06-27 14:56:44 | 2014-07-01 05:49:17 | 2014-07-01 05:49:17 | Cross check of Adaptive candidate selection (JCT3V-I0131) | M. W. Park, C. Kim (Samsung) |
| JCT3V-I0150 | m34118 | 2014-06-29 02:24:33 | 2014-06-29 02:33:43 | 2014-06-29 02:33:43 | 3D-CE2: Cross check of advanced boundary chain coding for depth intra mode (JCT3V-I0118) | J. Y. Lee, C. Kim (Samsung) |
| JCT3V-I0151 | m34119 | 2014-06-29 02:24:48 | 2014-06-29 02:34:13 | 2014-06-29 02:34:13 | Cross check of depth compensation calibration in 3D-HEVC (JCT3V-I0074) | J. Y. Lee, C. Kim (Samsung) |
| JCT3V-I0152 | m34120 | 2014-06-29 02:25:02 | 2014-06-29 02:34:45 | 2014-06-29 02:34:45 | Cross check of unification of the merge candidate sets in 3D-HEVC (JCT3V-I0098) | J. Y. Lee, C. Kim (Samsung) |
| JCT3V-I0153 | m34121 | 2014-06-29 02:25:22 | 2014-06-29 02:35:18 | 2014-06-29 02:35:18 | Cross check of DBBP simplification based on sub-block level processing (JCT3V-I0094) | J. Y. Lee, C. Kim (Samsung) |
| JCT3V-I0154 | m34122 | 2014-06-29 02:25:38 | 2014-06-29 02:36:48 | 2014-06-29 02:36:48 | Cross check of simplifications for 3D-HEVC merge candidate list generation (JCT3V-I0053) | J. Y. Lee, C. Kim (Samsung) |
| JCT3V-I0155 | m34127 | 2014-06-29 07:56:35 | 2014-06-29 21:12:26 | 2014-06-29 21:12:26 | CE1: Cross-check of Simplification of advanced residual prediction (JCT3V-I0051) | T. Ikai (Sharp) |
| JCT3V-I0156 | m34128 | 2014-06-29 07:57:02 | 2014-06-29 21:12:54 | 2014-06-29 21:12:54 | Cross-check of Simplification of 3D Merge list construction (JCT3V-I0087) | T. Ikai (Sharp) |
| JCT3V-I0157 | m34129 | 2014-06-29 07:57:20 | 2014-06-29 21:13:38 | 2014-06-29 21:13:38 | 3D-CE1 related: Cross-check of Reduction of Worst Case Memory | T. Ikai (Sharp) |
| JCT3V-I0158 | m34130 | 2014-06-29 07:57:41 | 2014-06-29 21:14:11 | 2014-06-29 21:14:11 | Cross-check of Parameter Derivation for Illumination Compensation | T. Ikai (Sharp) |
| JCT3V-I0159 | m34131 | 2014-06-29 07:58:01 | 2014-06-29 21:15:10 | 2014-06-29 21:15:10 | CE1 related: Cross-check of Alignment of disparity vector in inter-view | T. Ikai (Sharp) |
| JCT3V-I0160 | m34132 | 2014-06-29 07:58:19 | 2014-06-29 21:15:44 | 2014-06-29 21:15:44 | Cross-check of Memory Complexity for DBBP and VSP (JCT3V-I0078) | T. Ikai (Sharp) |
| JCT3V-I0161 | m34133 | 2014-06-29 07:58:35 | 2014-06-29 21:16:11 | 2014-06-29 21:16:11 | Cross-check of MV sharing for depth coding in 3D-HEVC (JCT3V-I0073) | T. Ikai (Sharp) |
|  |  |  |  |  | WD |  |
| JCT3V-I0163 | m34135 | 2014-06-29 07:59:48 | 2014-07-03 04:30:46 | 2014-07-03 04:30:46 | Cross-check of Bug-fixing of depth-based block partitioning signaling (JCT3V-I0125) | T. Ikai, T. Tsukuba (Sharp) |
| JCT3V-I0164 | m34176 | 2014-06-30 04:05:20 | 2014-07-03 11:24:30 | 2014-07-03 16:18:05 | Crosscheck for JCT3V-I0123, Fast Intra SDC coding for 3D-HEVC Intra Coding | X. Zhang, K. Zhang (MediaTek) |
| JCT3V-I0165 | m34178 | 2014-06-30 04:05:38 | 2014-07-01 12:54:00 | 2014-07-01 12:54:00 | Crosscheck for JCT3V-I0124, On DBBP Filtering Simplification | X. Zhang, K. Zhang (MediaTek) |
| JCT3V-I0166 | m34186 | 2014-06-30 07:26:51 | 2014-07-01 05:27:53 | 2014-07-01 05:27:53 | CE1: Crosscheck of results of ARP simplification (JCT3V-I0104) | H. Liu (Qualcomm) |
| JCT3V-I0167 | m34187 | 2014-06-30 07:28:34 | 2014-07-01 05:30:39 | 2014-07-01 05:30:39 | Crosscheck of On complexity reduction of bi-prediction for advanced residual prediction (JCT3V-I0096) | H. Liu (Qualcomm) |
| JCT3V-I0168 | m34188 | 2014-06-30 07:29:33 | 2014-07-01 05:56:10 | 2014-07-01 05:56:10 | CE2: Crosscheck of single depth intra mode for 3D-HEVC (JCT3V-I0095) | H. Liu (Qualcomm) |
| JCT3V-I0169 | m34189 | 2014-06-30 07:31:00 | 2014-07-01 05:56:49 | 2014-07-01 05:57:01 | CE2 related: Crosscheck of allowing zero DC candidate in inter SDC (JCT3V-I0083) | H. Liu (Qualcomm) |
| JCT3V-I0170 | m34191 | 2014-06-30 07:33:58 | 2014-07-01 06:15:41 | 2014-07-01 06:15:41 | Crosscheck of segment-wise Prediction (SP) Merge Candidate (JCT3V-I0058) | H. Liu (Qualcomm) |
| JCT3V-I0171 | m34192 | 2014-06-30 07:34:55 | 2014-07-01 06:16:29 | 2014-07-01 06:16:29 | CE2 related: Crosscheck of remove depth_dc_flag signalling in DMM cases (JCT3V-I0120) | H. Liu (Qualcomm) |
| JCT3V-I0172 | m34367 | 2014-07-01 03:00:38 | 2014-07-02 12:38:22 | 2014-07-02 12:38:22 | Crosscheck of analysis for redundant clippings in illumination compensation (JCT3V-I0132) | S. Yoo, S. Yea (LGE) |
| JCT3V-I0173 | m34378 | 2014-07-01 07:05:18 | 2014-07-01 07:08:38 | 2014-07-04 01:14:04 | Cross-check of On complexity reduction of bi-prediction for advanced residual prediction (JCT3V-I0096) | T. Ikai (Sharp) |
| JCT3V-I0174 | m34381 | 2014-07-01 09:55:20 | 2014-07-02 11:51:49 | 2014-07-02 11:51:49 | Crosscheck on Simplification of VSP Candidate Derivation (JCT3V-I0130) | Y. Zhang (Zhejiang University) |
| JCT3V-I0175 | m34383 | 2014-07-01 10:28:24 | 2014-07-01 10:30:23 | 2014-07-03 05:19:43 | 3D-HEVC: Simplification of DMM method | H. Sabirin, K. Kawamura, S. Naito (KDDI) |
| JCT3V-I0176 | m34384 | 2014-07-01 10:31:23 | 2014-07-01 10:32:19 | 2014-07-01 16:04:14 | 3D-HEVC: Quantization of Residual Depth Data | H. Sabirin, K. Kawamura, S. Naito (KDDI) |
|  |  |  |  |  | WD |  |
| JCT3V-I0178 | m34419 | 2014-07-02 11:52:43 | 2014-07-04 09:21:24 | 2014-07-04 09:21:24 | 3D-CE1 related: Crosscheck on Diagonal prediction using ARP (JCT3V-I0089) | S. Shimizu (NTT) |
| JCT3V-I0179 | m34420 | 2014-07-02 11:53:01 | 2014-07-04 09:21:44 | 2014-07-04 09:21:44 | 3D-CE2 related: Crosscheck on reduction of the number of intra prediction modes for depth coding (JCT3V-I0117) | S. Shimizu (NTT) |
| JCT3V-I0180 | m34421 | 2014-07-02 11:53:13 | 2014-07-04 09:22:04 | 2014-07-04 09:22:04 | AHG4: Crosscheck on editorial improvement for ARP specification text in 3D-HEVC (JCT3V-I0129) | S. Shimizu (NTT) |
| JCT3V-I0181 | m34422 | 2014-07-02 11:53:28 | 2014-07-04 09:22:23 | 2014-07-04 09:22:23 | AHG4: Crosscheck on Semantics clean-up for advanced residual prediction weighting factor (JCT3V-I0055) | S. Shimizu (NTT) |
| JCT3V-I0182 | m34423 | 2014-07-02 11:53:58 | 2014-07-04 09:22:44 | 2014-07-04 09:22:44 | AHG5: Crosscheck on Coding results for flexible coding order (JCT3V-I0116) | S. Shimizu (NTT) |
| JCT3V-I0183 | m34424 | 2014-07-02 11:54:15 | 2014-07-04 09:23:03 | 2014-07-04 09:23:03 | Crosscheck on Partition Derivation for DBBP (JCT3V-I0077) | S. Shimizu (NTT) |
| JCT3V-I0184 | m34425 | 2014-07-02 11:54:26 | 2014-07-04 09:23:24 | 2014-07-04 09:23:24 | Crosscheck on DV Derivation for VSP (JCT3V-I0079) | S. Shimizu (NTT) |
| JCT3V-I0185 | m34426 | 2014-07-02 11:54:39 | 2014-07-04 09:23:45 | 2014-07-04 09:23:45 | Crosscheck on complexity reduction of bi-prediction for advanced residual prediction (JCT3V-I0096) | S. Shimizu (NTT) |
| JCT3V-I0186 | m34427 | 2014-07-02 11:54:52 | 2014-07-04 09:38:25 | 2014-07-04 09:38:25 | Crosscheck on removal of the decoder-side DBBP partition derivation process (JCT3V-I0097) | S. Shimizu (NTT) |
| JCT3V-I0187 | m34428 | 2014-07-02 11:55:10 | 2014-07-04 09:38:45 | 2014-07-04 09:38:45 | Crosscheck on Adaptive depth access for VSP enhancement (JCT3V-I0105) | S. Shimizu (NTT) |
| JCT3V-I0188 | m34429 | 2014-07-02 11:55:23 | 2014-07-04 09:39:08 | 2014-07-04 09:39:08 | Crosscheck on Further VSP inheritance simplification (JCT3V-I0106) | S. Shimizu (NTT) |
| JCT3V-I0189 | m34458 | 2014-07-03 06:19:29 | 2014-07-03 06:27:15 | 2014-07-03 06:27:15 | Cross check for option2 of one dimensional DBBP boundary filtering (JCT3V-I0109) | H. Huang (MediaTek) |
| JCT3V-I0190 | m34465 | 2014-07-03 10:41:44 | 2014-07-03 11:56:25 | 2014-07-06 09:18:09 | BoG report on HLS | J. Boyce |
| JCT3V-I0191 | m34468 | 2014-07-03 15:55:51 | 2014-07-03 16:14:43 | 2014-07-08 02:15:37 | Removal of the 2Nx2N restriction for sub-PU IVMP | J. An, J.-L. Lin, K. Zhang (MediaTek) |
| JCT3V-I0192 | m34469 | 2014-07-03 15:56:55 | 2014-07-03 16:15:03 | 2014-07-03 16:15:03 | Draft text for the improvement on the signaling of DBBP | J.-L. Lin, Y.-W. Chen, T.-D. Chuang, X. Zhang, Y.-W. Huang, S. Lei (MediaTek) |
| JCT3V-I0193 | m34471 | 2014-07-04 03:07:43 | 2014-07-04 12:35:27 | 2014-07-04 12:35:27 | Crosscheck of conversion verification for JCT3V-I0132 | X.Chen, X.Zheng (HiSilicon), |
| JCT3V-I0194 | m34505 | 2014-07-05 11:00:59 | 2014-07-05 11:07:28 | 2014-07-05 11:07:28 | BoG report on DBBP (Depth Based Block Partitioning) | S. Yea (LGE) |
| JCT3V-I0195 | m34506 | 2014-07-05 12:50:51 | 2014-07-05 12:52:24 | 2014-07-07 07:15:41 | BoG report on merge list construction | S. Shimizu (NTT) |
| JCT3V-I0196 | m34522 | 2014-07-06 16:44:18 | 2014-07-06 16:49:41 | 2014-07-22 20:55:46 | BoG report on MV-HEVC conformance test | Y. Chen, S. Shimizu, T. Suzuki |
| JCT3V-I0197 | m34528 | 2014-07-07 04:53:18 | 2014-07-07 05:07:11 | 2014-07-07 05:07:11 | Cross-check of additional test for Simplification of shift DV candidate (JCT3V-I0086) | T. Ikai (Sharp) |
| JCT3V-I0198 | m34529 | 2014-07-07 04:53:51 | 2014-07-07 05:07:39 | 2014-07-07 05:07:39 | Cross-check of additional test for Simplifications for 3D-HEVC merge candidate list generation (JCT3V-I0053) | T. Ikai (Sharp) |
| JCT3V-I0199 | m34541 | 2014-07-08 06:21:16 | 2014-07-08 06:55:00 | 2014-07-08 06:55:00 | BoG report on editors' notes in SHVC/MV-HEVC specs | Y.-K. Wang |
| JCT3V-I0200 | m34543 | 2014-07-08 06:58:03 | 2014-07-08 08:35:14 | 2014-07-08 08:35:14 | MV-HEVC/SHVC HLS: Miscellaneous cleanups re editors' notes | Y.-K. Wang |
| JCT3V-I0201 | m34545 | 2014-07-08 11:33:10 | 2014-07-08 12:00:58 | 2014-07-08 12:00:58 | BoG report on 3D-HEVC conformance test | Y. Chen, S. Shimizu, T. Ikai |
|  |  |  |  |  | WD |  |
| JCT3V-I0203 | m34553 | 2014-07-08 19:48:49 | 2014-07-09 03:54:32 | 2014-07-09 03:54:32 | MV-HEVC/SHVC HLS: Cleanup of VUI semantics | A. K. Ramasubramonian (Qualcomm), S. Deshpande (Sharp Labs) |
| JCT3V-I0204 | m34558 | 2014-07-09 01:55:07 | 2014-07-09 02:37:02 | 2014-07-09 02:37:02 | MV-HEVC/SHVC HLS: Repetition of parameter sets | A. K. Ramasubramonian (Qualcomm), M. M. Hannuksela (Nokia) |
| JCT3V-I1000 | m34586 | 2014-07-11 10:36:05 |  |  | Meeting Report of 9th JCT-3V Meeting | J.-R. Ohm, G. J. Sullivan |
| JCT3V-I1001 | m34570 | 2014-07-09 15:01:20 | 2014-08-06 12:30:17 | 2014-08-13 18:32:53 | 3D-HEVC Draft Text 5 | G. Tech, K. Wegner, Y. Chen, S.Yea |
| JCT3V-I1002 | m34571 | 2014-07-09 15:03:42 | 2014-07-10 06:14:11 | 2014-10-01 19:15:40 | MV-HEVC Draft Text 9 | G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce |
| JCT3V-I1003 | m34583 | 2014-07-11 07:59:24 | 2014-08-26 06:00:39 | 2014-08-26 06:00:39 | Test Model 9 of 3D-HEVC and MV-HEVC | Y. Chen, G. Tech, K. Wegner, S. Yea |
| JCT3V-I1005 | m34587 | 2014-07-11 10:43:48 | 2014-08-31 10:43:42 | 2014-09-18 06:29:38 | Draft 4 of 3D-AVC Reference Software | D. Rusanovskyy, D. Tian |
| JCT3V-I1006 | m34588 | 2014-07-11 10:46:49 | 2014-08-31 10:44:14 | 2014-09-18 06:30:18 | Draft 5 of MVC+D Reference Software | D. Rusanovskyy, T. Suzuki, D. Tian |
| JCT3V-I1007 | m34589 | 2014-07-11 10:49:22 | 2014-08-07 08:21:47 | 2014-08-12 20:33:13 | Draft 1 of MFC plus Depth and Texture/Depth View Packing SEI Message | P. Yin, T. Senoh, M. Hannuksela, Y. Chen, J.-R. Ohm, G. J. Sullivan |
| JCT3V-I1008 | m34584 | 2014-07-11 08:00:52 | 2014-07-24 18:42:46 | 2014-08-11 19:35:14 | MV-HEVC Conformance Draft 1 | Y. Chen, S. Shimizu, T. Suzuki |
| JCT3V-I1009 | m34590 | 2014-07-11 10:53:17 |  |  | MV-HEVC Software Draft 1 | G. Tech |
| JCT3V-I1010 | m34585 | 2014-07-11 08:02:15 | 2014-07-25 20:18:48 | 2014-08-12 00:03:56 | 3D-HEVC Conformance Draft 1 | Y. Chen, T. Ikai, T. Suzuki |
| JCT3V-I1011 | m34567 | 2014-07-09 07:10:47 | 2014-07-09 07:11:18 | 2014-07-09 07:11:18 | MV-HEVC Draft Verification Test Plan v1 | S. Shimizu |
| JCT3V-I1101 | m34592 | 2014-07-11 15:32:20 | 2014-07-11 15:36:05 | 2014-07-11 15:36:05 | Description of Core Experiment 1 (CE1) on Segmental Prediction | J.-L. Lin |

# Annex B to JCT-3V report:
List of meeting participants

The participants of the ninth meeting of the JCT-3V, according to the online registration (approximately 118 in total), were as follows:

- Simao Campos (ITU)

- Rosa Angeles-Leon de (ITU/TSB)

- Uemura Akira (Nippon Television Network Corporation)

- Elena Alshina (Samsung Electronics)

- Ichiro Ando (Nikon Corporation)

- Jun Arai (NHK)

- Kohtaro Asai (Mitsubishi Electric Corporation)

- Gun Bang (ETRI)

- Guillaume Barroux (Fujitsu Laboratories Ltd.)

- Lazar Bivolarski (LZ Associates)

- Philippe Bordes (Technicolor)

- Stephen Botzko (Polycom)

- Jill Boyce (Vidyo)

- Benjamin Bross (Fraunhofer HHI)

- Döne Bugdayci Sansli (Tampere University of Technology)

- Chun-Fu Chen (NCKU/ITRI)

- Weizhong Chen (Huawei Technologies)

- Chun-Chi Chen (NCTU/ITRI International)

- Xu Chen (Huawei Technologies)

- Ying Chen (Qualcomm)

- Yi-Jen Chiu (Intel Corp.)

- Yongjin Cho (Samsung Electronics)

- Byeongdoo Choi (Samsung Electronics)

- Chris Rosewarne (Canon Inc.)

- Tzu-Der Chuang (MediaTek Inc.)

- Takeshi Chujoh (Toshiba Corporation)

- David Daniels (BSkyB)

- Sachin Deshpande (Sharp)

- Edouard Francois (Technicolor)

- Zhouye Gu (Santa Clara University)

- Miska Hannuksel (Nokia Corporation)

- Arianne Hinds (Cable Television Laboratories)

- Yo-Sung Ho (GIST)

- Sabirin Houari (KDDI R&D Laboratories, Inc.)

- Yu-Wen Huang (MediaTek Inc.)

- Chao Hsiung Hung (ITRI)

- Atsuro Ichigaya (NHK)

- Takayuki Ito (Toshiba)

- Fabian Jäger (RWTH Aachen University)

- Byeungwoo Jeon (Sungkyunkwan University)

- Dong Jiang (Lenovo)

- Ye Jiangli (Fujitsu R&D Center Co. Ltd)

- Rajan Joshi (Qualcomm)

- Joel Jung (Orange Labs)

- Jung Won  (ETRI)

- Toshiyuki Kaya (Renesas Electronics Corporation)

- Kimihiko Kazui (Fujitsu Labs. Ltd.)

- Kei Kawamura (KDDI Corporation)

- Hyun Gyu Kim (Chips Media)

- Dae Yeon Kim (Chips Media)

- Krasimir Kolarov (Apple, Inc.)

- Guillaume Laroche (Canon Research Centre France)

- Thorsten Laude (Leibniz Universität Hannover)

- Fabrice le Leannec (Technicolor)

- Bae-Keun Lee (KT)

- Hahyun Lee (ETRI)

- Jin Young Lee (Samsung)

- Yung Lyul Lee (Sejong University)

- Ming Li (ZTE Corporation)

- Shuai Li (University of Electronic Science and Technology of China)

- Woong Lim (Kwangwoon University)

- Chong Soon Lim (Panasonic Corporation)

- Jian-Liang Lin (MediaTek Inc.)

- Ching-Chieh Lin (ITRI)

- Hongbin Liu (Qualcomm)

- Ying Luo (AMD)

- Jonghyun Ma (Kwangwoon University)

- Ken McCann (Samsung / Zetacast)

- Philipp Merkle (Fraunhofer HHI)

- Akira Minezawa (Mitsubishi Electric)

- Kazuyuki Miyazawa (Mitsubishi Electric)

- Ohji Nakagami (Sony Corporation)

- Junghak Nam (LG Electronics)

- Takahiro Nishi (Panasonic)

- Byungtae Oh (Korea Aerospace University)

- Jens-Rainer Ohm (RWTH Aachen University)

- Patrice Onno (Canon Research Centre France S.A.S)

- Jörn Ostermann (Leibniz Universität Hannover)

- Min Woo Park (Samsung Electronics Co., Ltd.)

- Wen-Hsiao Peng (ITRI International/NCTU)

- Martin Pettersson (Ericsson AB)

- Mohamad Raad (RaadTech Consulting)

- Adarsh Krishnan Ramasubramonian                                            (Qualcomm)

- Krishnakanth Rapaka (Qualcomm)

- Justin Ridge (Nokia)

- Hashimoto Ryoji (Renesas Electronics)

- Jesus Sampedro (POLYCOM)

- Jonatan Samuelsson (Ericsson)

- Takanori Senoh (NICT)

- Shinya Shimizu (NTT Corporation)

- Olgierd Stankiewicz (Poznan University of Technology)

- Yeping Su (Apple Inc)

- Karsten Sühring (Fraunhofer HHI)

- Yasuko Sugito (NHK)

- Gary Sullivan (Microsoft Corporation)

- Teruhiko Suzuki (Sony Corporation)

- Maxim Sychev (Huawei Technologies Russia)

- Watanabe Takashi (Toshiba Corporation)

- Thiow Keng Tan (NTT DOCOMO, Inc.)

- Gerhard Tech (Fraunhofer HHI)

- Ikai Tomohiro (Sharp Corporation)

- Yi-Shin Tung (MStar Semiconductor Inc./ ITRI)

- Kemal Ugur (Nokia)

- Ye-Kui Wang (Qualcomm Inc.)

- Krzysztof Wegner (Poznan University of Technology)

- Stephan Wenger (Vidyo, Inc.)

- Zhang Wenhao (Intel Corporation)

- Ping Wu (ZTE (UK) Ltd)

- Xiaoyu Xiu (InterDigital Communications Inc.)

- Yoshihisa Yamada (XIU)

- Tomoyuki Yamamoto (Sharp Corporation)

- Yan Ye (Interdigital Communications)

- Sehoon Yea (LG Electronics)

- Peng Yin (Dolby Laboratories, Inc.)

- Kai Zhang (Mediatek Inc.)

- Li Zhang (Qualcomm)

- Jianhua Zheng (Huawei Technologies)

- Jian Qing Zhu (Fujitsu R&D Center Co., Ltd.)