| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0086 |
| --- | --- |

| Title: | Simplification of shift DV candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, Yongjin Cho, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

Proposed Text (Option3)

- I.8.5.3.2.1 Derivation process for luma motion vectors for merge mode

…

if( availableFlagIvDC  
				&&  ( i < ( 5 + NumExtraMergeCand ) ) )
				extMergeCandList[ i++ ] = IvDC

...

7. The variable availableFlagIvDCShift is set equal to 0, and when availableFlagIvMCShift is equal to 0, DepthFlag is equal to 0, availableFlagIvDC is equal to 1, and i is less than ( 5 + NumExtraMergeCand ), availableFlagIvDCShift is set to 1 and the following applies for X being 0 to 1, inclusive:

predFlagLXIvDCShift is set equal to predFlagLXIvDC,

refIdxLXIvDCShift is set equal to refIdxLXIvDC,

mvLXIvDCShift[ 0 ] is set equal to mvL0IvDC[ 0 ] + 4

mvLXIvDCShift[ 1 ] is set equal to mvL0IvDC[ 1 ]

…

- I.8.5.3.2.12 Derivation process for a disparity inter-view motion vector candidate

…

For each i from 0 to num_ref_idx_lX_active_minus1, inclusive, the following applies:

When PicOrderCnt( RefPicListX[ i ] ) is equal to the PicOrderCntVal, ViewIdx( RefPicListX[ i ] ) is equal to refViewIdx and availableFlagLXInterView is equal to 0 the following applies:

-)

mvLXInterView[ 0 ] = mvDisp[ 0 ]		(I-147)

mvLXInterView[ 1 ] = 0		(I-148)

availableFlagLXInterView = mvDisp[ 0 ] != 0 ? 1 : 0 		(I-146

refIdxLX = i		(I-149)

-