| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V- I0123 |
| --- | --- |

| Title: | Fast Intra SDC coding for 3D-HEVC Intra Coding |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Zhouye Gu1 Jianhua Zheng2 Nam Ling1 Philipp Zhang2 | Tel: +1-206-816-2367 Email: zgu@scu.edu zhengjianhua@hisilicon.com |  |  |
| Source: Santa Clara University1 and HiSilicon Technologies2 |  |  |  |  |

_____________________________

# Abstract

In current High Efficiency Video Coding (HEVC)-based depth intra coding, a full RD search list is created and both non-SDC coding and SDC coding are tested for full-RD cost calculation. To speedup the depth intra coding, we propose to test SDC coding for selected candidates modes based on the non-SDC RD cost of each mode. Experiment results show that the proposed algorithm achieves 15%-20% encoding time saving with negligible BD-rate change under All Intra test configuration.

# Introduction

In current 3D-HEVC depth intra coding, the Segment-wise DC Coding (SDC) approach provides an alternative residual coding method. With SDC, the residual data (one or two constant residual values within one PU) is coded without transform and quantization processes. Whether SDC is used is signalled in the coding unit parameters structure at PU level. The partition size of CU containing a SDC coded PU is always 2Nx2N. SDC can be applied to all depth intra prediction modes including HEVC intra prediction modes and depth modelling modes. For HEVC intra prediction modes, the entire PU is considered as one segment, while for DMM modes, there are two segments. [1]

However, not all depth intra prediction modes (including HEVC intra prediction modes and depth modelling modes) need to test SDC for full-RD cost calculation. Since SDC is a supplementary mode based on intra prediction modes, it makes sense to just test SDC for intra prediction modes with small RD cost rather than all the intra prediction modes.

# Proposed solution

In this proposal, we selectively skip intra SDC full RD cost calculation based on the non-SDC RD cost of each candidate mode. Basically, we notice the modes with small full-RD cost when SDC is disabled are also very likely to have small full-RD cost when SDC is enabled. Therefore, we use the mode with smallest full-RD cost when SDC is disabled, Non_SDC_Mode1, as one of the indicators for SDC full RD cost calculation skip decision.

Our proposed algorithm is summarized in Figure 1. Here, Non_SDC_Mode1 is mode with smallest full-RD cost when SDC is disabled; Non_SDC_Mode2 is mode with second smallest full-RD cost when SDC is disabled; Non_SDC_Mode3 is mode with third smallest full-RD cost when SDC is disabled. SDC_Mode1 is mode with smallest full-RD cost when we decide to enabled SDC. The variance threshold, varThreshold, in Figure 1 is set to 1.

Figure 1. Flow chart of proposed fast SDC mode selection algorithm for intra depth map coding

# Experimental results

The proposed method is implemented on top of HTM-11.0, and simulations were performed under All Intra configuration [2].

The results are summarized in Table 1. Under All Intra test configuration, about 15% encoding time saving is achieved with negligible BD-bitrate change.

Table 1: BD rate results for 3-view case under All Intra Condition

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea “Test Model 8 of 3D-HEVC and MV-HEVC” in JCT3V-H1003, Valencia Spain, 29 Mar. – 4 Apr. 2014

- K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-G1100, San Jose, USA, January 2014.

# Patent rights declaration(s)

HiSilicon Technologies, Santa Clara University and Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).