| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18 – 24 Oct. 2014 |  |  |  | Document: JCT3V-J_Notes_d1 |
| --- | --- | --- | --- | --- |
| Title: | Meeting report of the tenth meeting of the Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V), Strasbourg, FR, 18 – 24 Oct. 2014 |  |  |  |
| Status: | Report document from chairs of JCT-3V |  |  |  |
| Purpose: | Report |  |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |  |
| Source: | Chairs |  |  |  |

_____________________________

- Summary

The Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its tenth meeting during 18 – 24 Oct. 2014 at the Palais de la Musique et des Congres, Strasbourg, FR. The JCT-3V meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany) and Dr Gary Sullivan (Microsoft/USA). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.12 of this document.

The meeting was mainly held in a “single track” fashion, with few breakout activities (as documented in this report) held in parallel. All decisions were made in plenaries when one of the chairs was present.

The JCT-3V meeting sessions began at approximately 900 hours on Saturday 18 Oct. 2014. Meeting sessions were held on all days until the meeting was closed at approximately XXXX hours on Friday 24 Oct. 2014. Approximately XXX people attended the JCT-3V meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JCT-3V. The subject matter of the JCT-3V meeting activities consisted of work on 3D extensions of the Advanced Video Coding (AVC) and the High Efficiency Video Coding (HEVC) standards.

The primary goals of the meeting were to review the work that was performed in the interim period since the ninth JCT-3V meeting in producing

- 3D-HEVC Draft Text 5 (Text of ISO/IEC 23008-2 DAM4 –for ballot)

- MV-HEVC Draft Text 9 (to be integrated to ISO/IEC 23008-2:201X – for ballot)

- Test Model 9 of 3D-HEVC and  MV-HEVC, and associated software

- Draft 4 of 3D-AVC Reference Software (Study of ISO/IEC 14496-5:2001 DAM35, not for ballot)

- Draft 5 of MVC+D Reference Software (ISO/IEC 14496-5:2001 FDAM33 – for ballot)

- Draft 1 of MFC plus Depth and Texture/Depth View Packing SEI Message (ISO/IEC 14496-10:201X PDAM1 – for ballot)

- MV-HEVC Conformance Draft 1

- MV-HEVC Software Draft 1

- 3D-HEVC Conformance Draft 1

- MV-HEVC Draft Verification Test Plan v1

Furthermore, the JCT-3V reviewed the results from one interim Core Experiment (CE); reviewed technical input documents; produced updated versions of the draft texts, framework descriptions and software implementations of the items above.

The JCT-3V produced XX particularly important output documents from the meeting (update):

- 3D-HEVC Draft Text 5 (Text of ISO/IEC 23008-2 DAM4 –for ballot)

- MV-HEVC Draft Text 9 (to be integrated to ISO/IEC 23008-2:201X – for ballot)

- Test Model 9 of 3D-HEVC and  MV-HEVC, and associated software

- Draft 4 of 3D-AVC Reference Software (Study of ISO/IEC 14496-5:2001 DAM35, not for ballot)

- Draft 5 of MVC+D Reference Software (ISO/IEC 14496-5:2001 FDAM33 – for ballot)

- Draft 1 of MFC plus Depth and Texture/Depth View Packing SEI Message (ISO/IEC 14496-10:201X PDAM1 – for ballot)

- MV-HEVC Conformance Draft 1

- MV-HEVC Software Draft 1

- 3D-HEVC Conformance Draft 1

- MV-HEVC Draft Verification Test Plan v1

Moreover, plans were established to conduct 1 future CE in the interim period until the next meeting.

For the organization and planning of its future work, the JCT-3V established XX "Ad Hoc Groups" (AHGs) to progress the work on particular subject areas. The next four JCT-3V meetings are planned for 12 – 18 Feb. 2015 under ITU-T auspices in Geneva, CH, during 20 – 26 June 2015 under WG 11 auspices in Warsaw, PL, during 15 – 21 Oct. 2015 under ITU-T auspices in XXXX, XX, and … .

The document distribution site http://phenix.it-sudparis.eu/jct3v/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-3V and all of its AHGs is jct-3v@lists.rwth-aachen.de.

- Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-3V are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

## Meeting logistics

The JCT-3V meeting sessions began at approximately 900 hours on Saturday 18 Oct. 2014. Meeting sessions were held on all days until the meeting was closed at approximately XXXX hours on Friday 24 Oct. 2014. Approximately XXX people attended the JCT-3V meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JCT-3V. The subject matter of the JCT-3V meeting activities consisted of work on 3D extensions of the Advanced Video Coding (AVC) and the High Efficiency Video Coding (HEVC) standards.

Information regarding preparation and logistics arrangements for the meeting had been provided via the email reflector jct-3v@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jct3v-site/2014_10_J_Strasbourg/.

## Documents and document handling considerations

### General

The documents of the JCT-3V meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct3v/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

Document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (rather than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a bug in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding … simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of contexts, number of context-coded bins, etc.) … "Decision (Compl.):"

This meeting report is based primarily on notes taken by the chairs and projected (if possible) for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp (http://wftp3.itu.int/av-arch/jct3v-site/) during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much discussion of the contributions and discussions as is feasible in the interest of aiding study, although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Friday, 10 Oct. 2014. Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Saturday 11 Oct. 2014 were considered "officially late".

Most documents in the "late" category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All contribution documents with registration numbers JCT3V-J0067 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). Some documents in the "J0067+" range include break-out activity reports that were generated during the meeting and are therefore considered report documents rather than late contributions.

Generally, the late document submission situation had improved relative to previous history.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with this issue was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical proposal contributions were classified as late either due to late upload or late registration:

- JCT3V-J0XXX (a technical proposal on …) [uploaded 10-XX]

The following other documents not proposing normative technical content were classified as late (this did not apply in the current meeting):

- JCT3V-J0XXX (an input on …) [uploaded XX-XX]

The following cross-verification reports were classified as late either due to late upload or late registration: JCT3V-J0XXX, … .

The following document registrations were later cancelled or otherwise never provided or never discussed due to lack of availability or registration errors: JCT3V-J0XXX, … .

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were rejected in the document management system, as has been agreed since the third meeting.

(the following cases did not happen in the current meeting)

The initial uploads of the following contribution documents were rejected as "placeholders" without any significant content and were not corrected until after the upload deadline:

- JCT3V-J0XXX (a contribution of ... , corrected ...)

- ...

A few documents had some problems relating to IPR declarations (missing or excess declarations of contributing companies), inconsistent filenames in the header etc. in the initial uploaded versions. These issues were corrected by later uploaded versions in all cases (to the extent of the awareness of the chairs). In case of JCTVC-J0XXX and ..., the first complete versions appeared only on 06-XX, such that this contribution was also flagged as “late”.

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Measures to facilitate the consideration of contributions

For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by the cross-check contributor.

### Outputs of the preceding meeting

The report documents of the previous meeting, particularly the meeting report (JCT3V-I1000), the 3D-HEVC draft text 5 (JCT3V-I1001), the multi-view MV-HEVC draft text 9 (JCT3V-I1002), the MV-/3D-HEVC test model 9 (JCT3V-I1003), Draft 4 of 3D-AVC Reference Software (JCT3V-I1005), Draft 5 of MVC+D Reference Software (JCT3V-I1006), Draft 1 of MFC plus Depth and Texture/Depth View Packing SEI Message (JCT3V-I1007), MV-HEVC Conformance Draft 1 (JCT3V-I1008), MV-HEVC Software Draft 1 (JCT3V-I1009), 3D-HEVC Conformance Draft 1 (JCT3V-I1010), and MV-HEVC Draft Verification Test Plan v1 (JCT3V-I1011), which had been produced in the interim period, were approved. The ATM and HTM reference software packages produced by AHG3 and AHG4 on software development, and the software technical evaluations were also approved.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

## Attendance

The list of participants in the JCT-3V meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/ SC 29/ WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Opening remarks

- Contribution document allocation

- Reports of ad hoc group activities

- Report of Core Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on 3D video coding projects guidance

- Consideration of 3D video coding technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-3V and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-3V as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jct3v-site (JCT-3V contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-3v/index.html (JCT-3V general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/ SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that it is our understanding according to the practices of the parent bodies to make reference software available under copyright license header language which is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as e.g. recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/ SC 29/ WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software header is currently used in the process of designing the new HEVC standard and for evaluating proposals for technology to be included in this design. Additionally, after development of the coding technology, the software will be published by ITU-T and ISO/IEC as an example implementation of the 3D video standard(s) and for use as the basis of products to promote adoption of the technology. This is likely to require further communication with and between the parent organizations.

The ATM, HTM and MFC software packages that are used in JCT-3V follow these principles. The view synthesis software used for non-normative post processing is included in the HTM package and also has the BSD header.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct3v/. Furthermore, the site http://ftp3.itu.int/av-arch/jct3v-site was used for distribution of the contribution document template and circulation of drafts of this meeting report.

Communication of JCT-3V is performed via the list jct-3v@lists.rwth-aachen.de (to subscribe or unsubscribe, go to http://mailman.rwth-aachen.de/mailman/listinfo/jct-3v).

It was emphasized that reflector subscriptions and email sent to the reflector must use their real names when subscribing and sending messages and must respond to inquiries regarding their type of interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Note: Acronyms should be used consistently. For example, “IV” is sometimes used for “inter-view” and sometimes for “intra-view”.

Some terminology used in this report is explained below:

- AHG: Ad hoc group.

- AMVP: Advanced motion vector prediction.

- ARP: Advanced residual prediction.

- ATM: AVC based 3D test model

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BoG: Break-out group.

- BR: Bit rate.

- B-VSP: Backward view synthesis prediction.

- CABAC: Context-adaptive binary arithmetic coding.

- CD: Committee draft – the first formal ballot stage of the approval process in ISO/IEC.

- CE: Core experiment – a coordinated experiment conducted between two subsequent JCT-3V meetings and approved to be considered a CE by the group.

- Consent: A step taken in ITU-T to formally consider a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CPB: Coded picture buffer.

- CTC: Common test conditions.

- DBBP: Depth based block partitioning.

- DC: Disparity compensation

- DDD: Disparity derived depth (which uses the motion disparity vector to reconstruct a certain block (PU) of the depth map)

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC.

- DF: Deblocking filter.

- DLT: Depth lookup table.

- DMC: Depth based motion competition.

- DMM: Depth modeling mode.

- DPB: Decoded picture buffer.

- DRPS: Depth-range parameter set.

- DRWP: Depth-range based weighted prediction.

- DT: Decoding time.

- DV: Disparity vector

- ET: Encoding time.

- HEVC: High Efficiency Video Coding – the video coding standardization initiative under way in the JCT-VC.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- HRD: Hypothetical reference decoder.

- HTM: HEVC based 3D test model

- IC: Illumination compensation

- IDV: Implicit disparity vector

- IVMP: Inside-view motion prediction (which means motion for depth component is inherited from texture component motion)

- IVRC: Inter-view residual prediction.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/ SC 29, one of the two parent bodies of the JCT-3V).

- MPI: Motion parameter inheritance.

- MV: Motion vector.

- NAL: Network abstraction layer (HEVC/AVC).

- NBDV: Neighbored block disparity vector (used to derive unavailable depth data from reference view’s depth map) and DoNBDV = depth oriented NBDV

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NUT: NAL unit type (HEVC/AVC).

- PDM: Predicted Depth Map

- POC: Picture order count.

- PPS: Picture parameter set (HEVC/AVC).

- QP: Quantization parameter (as in AVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD). Often loosely associated with HE.

- RAP: Random access picture.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- REXT: Range extensions (of HEVC).

- RPS: Reference picture set.

- RQT: Residual quadtree.

- SAO: Sample-adaptive offset.

- SEI: Supplemental enhancement information (as in AVC).

- SD: Slice data; alternatively, standard-definition.

- SDC: Segment-wise DC coding.

- SH: Slice header.

- SHVC: Scalable HEVC.

- SPS: Sequence parameter set (HEVC/AVC).

- TSA: Temporal sublayer access.

- Unit types: Needs change

  - CTB: code tree block (synonymous with LCU or TB).

  - CU: coding unit.

  - LCU: (formerly LCTU) largest coding unit (synonymous with CTB or TB).

  - PU: prediction unit, with four shape possibilities.

    - 2Nx2N: having the full width and height of the CU.

    - 2NxN: having two areas that each have the full width and half the height of the CU.

    - Nx2N: having two areas that each have half the width and the full height of the CU.

    - NxN: having four areas that each have half the width and half the height of the CU.

  - TB: tree block (synonymous with LCU – LCU seems preferred).

  - TU: transform unit.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-3V).

- VPS: Video parameter set.

- VS: View synthesis.

- VSO: View synthesis optimization (RDO tool for depth maps).

- VSP: View synthesis prediction.

- WD: Working draft – the draft HEVC standard corresponding to the HM.

- WG: Working group (usually used in reference to WG 11, a.k.a. MPEG).

## Liaison activity

The JCT-3V did not send or receive formal liaison communications at this meeting.

## Opening remarks

- New AVC edition (including MVC+depth, 3D-AVC, MFC) – published by ISO/IEC 2014-09-01; publication in ITU-T expected soon.

- There are still some problems in the text related to 3D-AVC view synthesis prediction which may require a corrigendum action. By the time of the opening plenary, no input contribution had been submitted on these. Establish AHG on this. Issue Defect report?

- Upcoming publication of HEVC version 2 – including RExt, SHVC and MV-HEVC (Last Call in ITU-T closing 2014-10-28, FDIS: SC 29 N 14494 submitted to ITTF 2014-10-07)

- Status of conformance and reference software in the different amending activities:

  - Reference software MVC+D: FDAM ballot to be started

  - Reference software MFC, 3D-AVC to become FDAM in ISO; all three have been submitted for H.264.2, last call closing 2014-10-28

  - Conformance MVC+D: Published by ISO/IEC 2014-08-01

  - Conformance MFC: TBP, 3D-AVC under DAM ballot

  - Conformance MFC+D, MFC submitted to ITU-T, last call end Oct.

  - MV-HEVC & 3D-HEVC software: Alignment with HSVC, HM16

  - MV-HEVC & 3D-HEVC conformance

  - Joint meeting with JCT-VC on software & conformance

- Output docs:

  - FDAM MFC & 3D-AVC software

  - 3D-HEVC draft

  - MFC+D & SEI message draft (DAM)

  - Defect report on 3D-AVC VSP

  - MV-HEVC & 3D-HEVC conformance

  - MV-HEVC & 3D-HEVC software

  - MV-HEVC & 3D-HEVC test models

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows. (update counts)

- AHG reports (section 2) (10)

- Project development and status (section 3) (2)

- CE1 and related: Segmental prediction (section 4.1) (4)

- 3DV standards development (incl. software, conformance) (section 5) (3)

- High-Level Syntax (section 6) (4)

- Non-CE technical contributions (section 7) (62)

- Non-normative Contributions (section 8) (0)

- Withdrawn, unclear allocation (section 9) (1)

NOTE – The number of contributions noted in each category, as shown in parenthesis above, may not be 100% precise.

## Scheduling planning

Scheduling: Generally meeting time was scheduled during 0900 – 2000, with coffee and lunch breaks as convenient. Ongoing refinements were announced on the group email reflector as needed.

Some particular scheduling notes are shown below, although not necessarily 100% accurate:

- Saturday, first day

  - 0900-1100: Opening and AHG report review

  - 1130-13:00 CE review, 3DV standards development

  - Non-CE technical contributions review

- Sunday, second day

  - Continue non-CE review…

  - No meeting during 14-18 (to allow attending FTV AHG)

- Monday, third day

  - 900-1300 WG11 plenary

  - …

- Tuesday, fourth day

  - …

  - Afternoon: Workshop future video coding

- Wednesday, fifth day

  - Morning: MPEG plenary

  - …

- Thursday, sixth day

  - …

- Friday, seventh day

  - 900-1200 (or shorter) final plenary, any remaining business

# AHG reports (10)

The activities of ad hoc groups that had been established at the prior meeting are discussed in this section.

JCT3V-J0001 JCT-3V AHG Report: JCT-3V project management (AHG1) [J.-R. Ohm, G. J. Sullivan]

The work of the JCT-3V overall has proceeded well in the interim period. A large amount of discussion was carried out on the group email reflector. All documents from the preceding meeting had been made available at the document site (http://phenix.it-sudparis.eu/jct3v/) or the ITU-based JCT-3V site (http://wftp3.itu.int/av-arch/jct3v-site/2014_07_I_Sapporo/), particularly including the following:

- The meeting report (JCT3V-I1000)

- Draft 5 of 3D-HEVC text specification (JCT3V-I1001) (submitted for ISO/IEC DAM ballot)

- Draft 9 of MV-HEVC text specification (JCT3V-I1002) (also integrated into edition 2 of HEVC, doc. JCTVC-R1011)

- Test Model 8 of 3D-HEVC and  MV-HEVC (JCT3V-I1003), and associated software

- Draft 4 of 3D-AVC reference software (JCT3V-I1005)

- Draft 5 of MVC+D reference software (JCT3V-I1006)

- Draft 1 of MFC plus Depth and Texture/Depth View Packing SEI Message (JCT3V-I1007)

- MV-HEVC Conformance Draft 1 (JCT3V-I1008)

- MV-HEVC Software Draft 1 (JCT3V-I1009)

- 3D-HEVC Conformance Draft 1 (JCT3V-I1010)

- MV-HEVC Draft Verification Test Plan v1 (JCT3V-I1011)

- Finalized core experiment description (JCT3V-I1101).

The 10 ad hoc groups and one core experiment had made progress, and various reports from those activities had been submitted.

The software for ATM version 13 was issued including remaining bug fixing, and HTM version 12 had been prepared and released approximately as scheduled.

A "bug tracking" systems for software and text specifications had been installed. The sites are https://hevc.hhi.fraunhofer.de/trac/3d-hevc and https://hevc.hhi.fraunhofer.de/trac/3d-avc/. The bug tracker reports were automatically forwarded to the group email reflector, where the issues could be further discussed.

Approximately 100 input contributions to the current meeting had been registered. Late-registered and late-uploaded contributions were mostly cross-checks.

The meeting announcement had been made available from the aforementioned document site and http://wftp3.itu.int/av-arch/jct3v-site/2014_10_J_Strasbourg/JCT3V-J_Logistics.doc.

A preliminary basis for the document subject allocation and meeting notes had been circulated to the participants as http://wftp3.itu.int/av-arch/jct3v-site/2014_10_J_Strasbourg/JCT3V-J_Notes_d0.doc.

JCT3V-J0002 JCT-3V AHG Report: MV-HEVC / 3D-HEVC Draft and Test Model editing (AHG2) [G. Tech, K. Wegner, J. Boyce, Y. Chen, T. Suzuki, S. Yea, J.-R. Ohm, G. Sullivan]

The fifth 3D-HEVC draft, the seventh Test Model of 3D-HEVC and MV-HEVC and the ninth MV-HEVC draft were developed from the fourth 3D-HEVC draft, the sixth 3D-HEVC Test Model and the eighth MV-HEVC draft, respectively, following the decisions taken at the 9th JCT-3V meeting in Sapporo.

Three editorial teams were formed to work on the three documents that were to be produced:

- I1001 3D-HEVC Draft Text 4

- G. Tech, Y. Chen

- I1002 MV-HEVC Draft Text 8

- G. Tech, M. Hannuksela, Y. Chen, J. Boyce

- Y.-K. Wang, A. Ramasubramonian, J. Chen, G. J. Sullivan, Y. Ye

- I1003 Test Model 9 of 3D-HEVC and MV-HEVC

- Y. Chen, G. Tech

- 3D-HEVC draft text:

- Three versions of JCT3V-I1001 have been published by the AHG following the 9th JCT-3V meeting in Sapporo. The last version has been submitted as DAM text to the MPEG secretariat.

All, but one, adoptions of the last meeting have been incorporated (forgotten accidentally). Most of the changes have been minor modifications. Existing text has been revised and improved. One major change was the alignment with range extension related specifications of the 2nd HEVC edition.

Changes (26) of I1001 relative to H1001 are:

Editorial updates (1)

- (Update HEVC V2 ) Range extension parts.

Editorial adjustments (4)

- (3DE−05) Restructuring of DBBP output.

- (3DE−04) Adjusted page breaks

- (3DE−04) Adjusted heading indentation and size to match ISO basic template version 3.

- (3DN−10/JCT3V−I0069) Complexity Assessment on Depth Intra Modes with Simplified Data Flows. Decision (Ed): Integrate suggested editorial changes and further SDC fixes.

Fixes (2)

- (3DC−03) Fix ticket 72

- (3DC−02) Fix additional scalability dimensions.

Single Depth intra mode (1)

- (3DN−20/JCT3V−I0095) Single depth intra mode for 3D−HEVC Decision: Adopt I0095 Test 1 (method with 2 candidates)

Residual Prediction (3)

- (3DN−19/JCT3V−I0051) Simplification of advanced residual prediction Decision: Adopt JCT3V−I0051#1

- (3DN−14/JCT3V−I0072) Problem fix for MV scaling in inter−view ARP Decision(BF/ED/SW): Adopt

- (3DN−12/JCT3V−I0104) CE1: Results of ARP simplification Decision: Adopt JCT3V−I0104.

DBBP (4)

- (3DN−18/JCT3V−I0077) Partition Derivation for DBBP adoption of a simplification by the removal of decoder−side partition derivation (e.g. Test1 in I0097 with the proposed WD change) and adoption of syntax condition of the partition mode as per Test2 (the corresponding WD text is also provided in I0097).The software and text provided in a revision (−v3) of I0077, which includes harmonization with I0125, will be used.

- (3DN−17/JCT3V−I0109) One dimensional DBBP boundary filtering the filter simplification in option 2 of I0109 (which is a harmonization of three proposals including I0088, I0094, and a prior version of I0109), reducing of the number of filter conditions to be tested and applying a 1D rather than 2D filtering.

- (3DN−16/JCT3V−I0078) Memory Complexity for DBBP and VSP, adoption of the memory reduction method 2 (disallow DBBP for 8x8 CU)

- (3DN−15/JCT3V−I0076) Simplification of Threshold Derivation for DBBP and DMM4 JCT−3V confirmed, adoption of the simplification proposal to use four corner samples rather than all samples to form an average.

DMM/SDC (2)

- (3DN−02/JCT3V−I0120) CE2 : Remove “depth_dc_flag” signalling in DMM cases

- (3DN−01/JCT3V−I0110) Lookup table size reduction in DMM1

HLS (4)

- (3DN−06/JCT3V−I0100) HLS: General comments. Decision: Adopt the first and the fourth aspect (the latter without condition)

- (3DN−05/JCT3V−I0099) On the video parameter set extension 2 in 3D−HEVC. Decision: Adopt the version I0099WDr1

- (3DN−04/JCT3V−I0090) HLS Cleanup. Decision(ed.): Adopt the second aspect case a) Decision: Adopt the second aspect case b)

- (3DN−03/JCT3V−I0085) CE2: Separate enabling flag for intra coding tools. Decision: Retain inter_sdc flag as is; Introduce new dmm4 flag; Change semantics of depth_modes flag such that it controls dmm1 and intra sdc

Others (5)

- (3DN−13/JCT3V−I0191) Removal of the 2Nx2N restriction for sub−PU IVMP Decision: Adopt (new version 2 with additional modification to be delivered)

- (3DN−11/JCT3V−I0093) Depth coding compatible with arbitrary bit−depth Decision(Ed): Adopt

- (3DN−08/JCT3V−I0086) Simplification of shift DV candidate, Decision: Adopt removal of DMV search for shifted DV candidate derivation (in I0086_WD_r1)

- (3DN−09/JCT3V−I0057) Clipping Operation for DLT Indexes Decision: Adopt the text of I0057r1, with additional editorial improvement that Idx2DepthValue[ i ] shall be set to zero.

- (3DN−07/JCT3V−I0080) Parameter Derivation for Illumination Compensation Decision: Adopt this aspect (clipping of coordinate)

- Open issues:

- A list of other minor issues is listed in the bug tracking system.

- One adoption, (JCT3V-I0129 (ARP)), which has been indicated as an adopted editorial change in the meeting notes has accidently not been incorporated.

- Further editorial alignments with the 2nd edition of HEVC are required.

- The specification of profiles, tiers, and levels needs to be discussed.

- Test model 9:

One version of JCT3V-I1003 has been published by the editing AHG following the 9th JCT-3V meeting in Sapporo. The last version corresponds to the text submitted to MPEG secretariat.

All adoptions of the last meeting have been incorporated. Moreover existing text has been revised and improved and missing text from previous meeting has been added.

Changes (7) of I1003 relative to H1003 are:

- (3DN-01/JCT3V-I0086) Simplification of shift DV candidate, Decision: Adopt removal of DMV search for shifted DV candidate derivation.

- (3DN-02/JCT3V-I0078) Memory Complexity for DBBP and VSP, adoption of the memory reduction method 2 (disallow DBBP for 8x8 CU).

- (3DN-04/JCT3V-I0051) Simplification of advanced residual prediction Decision: Adopt JCT3V-I0051#1.

- (3DN-05/JCT3V-I0104) CE1: Results of ARP simplification Decision: Adopt JCT3V-I0104.

- (3DN-03/JCT3V-I0109) One dimensional DBBP boundary filtering the filter simplification in option 2 of I0109 (which is a harmonization of three proposals including I0088, I0094, and a prior version of I0109), reducing of the number of filter conditions to be tested and applying a 1D rather than 2D filtering.

- (3DN-06/JCT3V-I0095) Single depth intra mode for 3D-HEVC Decision: Adopt I0095 Test 1 (method with 2 candidates).

- (3DN-07/JCT3V-I0123) Fast Intra SDC coding for 3D-HEVC Intra Coding.

- MV-HEVC draft text

Seven versions of I1002 were published by the AHG following the 9th JCT-3V meeting in Sapporo. Editing has been taken place in an editing meeting following the Sapporo meeting. The text was developed together with the editors of the 2nd edition of HEVC. The last version corresponds to the text integrated to 2nd edition of the HEVC specification.

All adoptions of the last meeting have been incorporated. Moreover existing text has been revised and improved.

Changes (~100) of I1002 relative to JCT3V-H1002 are: (the list might be incomplete)

- Conformance point specifications, multiview and scalability 5-deep (total number of direct and indirect reference layers and the current layer) – agreed

- Conformance point specifications, Requiring decoding of auxiliary pictures if indicated in OLS – confirmed

- Definition in terms of enhancement capability (and external base layer), Q6 supporting per-layer PTL spec. – confirmed.

- Stereo Main profile Multiview Main profile – confirmed, Remove constraint on vertical displacement? If the view order index = 1 the constraint applies; the remaining ones are not. – agreed

- HEVC version 1 compatibility, Presence of layer 0 – agreed to remove requirement.

- HEVC version 1 compatibility, Independent non-base layer decoding – agree to define.

- HEVC version 1 compatibility, Access unit boundary (definition of "picture"), active parameter sets SEI compatibility (R0274), RawMinCuBits – agree to corrective actions

- R0274 On active parameter sets SEI message Bug fix of active parameter sets SEI message  It was pointed out that the "if payload_extension_present()" condition test, and an associated presence constraint were not necessary and agreed that they should be removed (only an editorial change). Decision: Adopt (as modified).

- R0257 Language indication for Overlay information SEI message Editor action item: Connect "UTF-8" to its meaning. Decision: Null-terminated st(v) string of characters encoded according to RFC 5646. Decision: Adopted, RFC 5646, variant 2, byte aligned, 0..255 range. Editors to double-check reference for UTF-8 and improve the reference (or reference a better definition) as necessary.

- SEI maturity in current draft/ Overlay information SEI message Decision: The persistence should end within each identified layer when the CLVS of that layer ends. Regarding language identification, see R0257. Parsing depends on the SPS of each associated layer ID Decision: Outside the outer-most loop, just prior to num_overlays_minus1, signal the number of bits minus 8 that is used within the loop for signalling overlay_element_label_min(max). Editor action item: In the definition of st(v), a parenthetical reference to the specification in which this is defined should be added, and further editorial improvement of that description seems desirable.

- R0355 TMCTS-SEI Decision: Adopt as modified. (Further editorial improvement may still be desirable during finalization.)

- R0255 Persistence of SEI messages Decision: Adopt (clarified as noted, notes regarding overlay information).

- R0221 Extensibility of nesting SEI messages Editor action item: Editors are suggested to consider adding the proposed note. Decision (BF): For bitstream partition nesting (BSP) SEI message, add an explicit counter to indicate the number of SEI messages in the BSP SEI message, with up to 64 allowed to be nested.

- R0231 Signalling and use of HRD parameters for bitstream partitions Decision (BF/Cleanup): Adopt.

- R0235 AHG10: Processing of bitstreams without an available base layer  Decision: Adopt (conditioned on the relevance of the basis text).

- R0153 External Base Proposal 1: When the base layer is externally specified, signalling modifications and constraints for the max_vps_dec_pic_buffering_minus1[ i ][ 0 ][ j ] syntax elements are proposed. Two options for handling this were described. Decision: Adopt "option 1" (not signalling syntax element and inferring its value as 0).

- R0153 Proposal 2: Modifications are proposed to the semantics of poc_reset_idc syntax element for handling the case of an externally specified base layer. Decision: Adopt.

- R0125 Non-base layer subtree and auxiliary pictures 1. In order to keep the compatibility of slice headers with nuh_layer_id equal to 0 after non-base layer subtree extraction, poc_lsb_not_present_flag[ i ] shall be equal to 1, when layer_id_in_nuh[ i ] is equal to AssignedBaseLayerId[ j ]. It was remarked that this constraint is effectively already present by the requirement that a rewritten bitstream conforms. Editor action item: The issue is purely editorial and the editors can consider whether some editorial action is desirable to make the constraint impact more clear or explicit.

- R0125 Non-base layer subtree and auxiliary pictures 3. An asserted bug fix in highest_layer_idx_plus1 derivation is reported. Decision: Bug confirmed – adopt fix.

- R0124 On hybrid codec scalability 3a.	Making it a requirement of bitstream conformance that, when vps_base_layer_internal_flag is equal to 0, vps_max_layers_minus1 shall be greater than 0. Decision: Adopt.

- R0124 On hybrid codec scalability 3d.	Making it a requirement of bitstream conformance that, when vps_base_layer_internal_flag is equal to 0, single_layer_for_non_irap_flag and higher_layer_irap_skip_flag shall be equal to 0. It was agreed that the current text does not handle this issue, but a different solution was desired. Decision (Ed.): Define semantics of these flags in an equivalent way as they would be used with an internal base layer. Editor action item: Check other places in the specification to ensure that they are also well defined to work for an external base layer.

- R0223 On POC handle missing pictures and discardable pictures in the same manner as different picture types across layers in requiring POC resetting, Decision: consider adding informative text to address item 1.

- R0223 On POC 2.) allow mixing of CRA and BLA pictures without requiring POC resetting (changing language about "different NAL unit type" to IRAP versus non-IRAP); and Decision: Adopt 2

- R0223 On POC explicitly spell out a restriction on the value of full_poc_reset_flag based on the value of poc_reset_idc of pictures in the first access unit in the same POC resetting period. Decision: Adopt 3 .

- R0280 Bitstream conformance restrictions in subclause C.4Decision (Ed.): Adopted (as modified).

- R0053 On invocation of a correct decoding process for the current picture  It is proposed that the decoding process of the current picture (subclause 8.1, subclause F.8.1, subclause G.8.1 or subclause H.8.1) is selected on the basis of nuh_layer_id of the current picture and the profile to which the bitstream partition containing the current picture conforms. Decision (Ed.): Adopt items 1 and 3a.

- R0053 On invocation of a correct decoding process for the current picture  It is proposed that the general decoding process in subclause 8.1 selects TargetPartitioningScheme based on external means or HRD, and if no other means is available TargetPartitioningScheme is set to 0. The BitstreamToDecode derived in subclause 8.1 is proposed to consist of the partitions containing target output layers, the partitions containing the direct and indirect reference layers of the layers in the partitions containing the target output layers and the partition containing the base layer. Decision (Ed.): Regarding Item 2, the intent is for the decoding process to be invoked only for output layers and layer on which they depend.

- R0053 AHG10 and On invocation of a correct decoding process for the current picture  3a	It is clearly indicated that the processing for the external base-layer pictures is invoked at the beginning of an access unit, and the processing is separated into its own subclause (F.8.1.6).Decision (Ed.): Adopt items 1 and 3a.

- R0053 AHG10 and On invocation of a correct decoding process for the current picture  It was discussed whether any well-specified constraints are established for a layer that is not indicated to be decoded by any OLS and does not have an associated profile_idc. Decision (BF/Ed.): Prohibit such layers from being present.

- R0276 On sub-bitstream extraction  Item 4 of the proposal had been withdrawn. Decision: Adopt (as modified).

- R0238 Editorial cleanups  Decision (BF/Ed.): Adopt.

- R0071 On cross-layer impacts of IRAP and EOS  It is proposed that if the current picture is the first picture that follows an end of sequence NAL unit (with nuh_layer_id equal to 0) in decoding order, NoClrasOutputFlag is set equal to 1. (This causes marking of all pictures in the DPB as "unused for reference" as well as setting LayerInitializedFlag and FirstPicInLayerDecodedFlag equal to 0 for all layers, which initializes the layer-wise start-up process.) Decision: Adopt this aspect.

- R0071 On cross-layer impacts of IRAP and EOS  It is proposed that when nuh_layer_id is greater than 0, BLA pictures, CRA pictures with HandleCraAsBlaFlag equal to 1 and IDR pictures with cross_layer_bla_flag equal to 1 cause the following: LayerInitializedFlag and FirstPicInLayerDecodedFlag is set equal to 0 for the current layer and all layers directly or indirectly predicted from the current layer. (Consequently, layer-wise start-up is enforced for these layers.) Each picture in the DPB for the current layer and all layers directly or indirectly predicted from the current layer are marked as "unused for reference". Decision: Adopt this aspect.

- R0071 On cross-layer impacts of IRAP and EOS  It is proposed to allow EOS NAL unit with nuh_layer_id greater than 0. It is proposed that the decoding of the EOS NAL unit with nuh_layer_id greater than 0 causes the same impacts as those above (2a and 2b). Furthermore, for the semantics of SEI messages applying for nuh_layer_id greater than 0, it is proposed that EOS NAL unit with nuh_layer_id greater than 0 is treated similarly to an end of sequence for the persistence of the SEI message. When present, an EOS NAL unit with a particular nuh_layer_id value shall be the last NAL unit with that particular nuh_layer_id value within an access unit, except an end of bitstream NAL unit with that nuh_layer_id value (when present). Refined text was reviewed, as provided in a revision of the document. Item 3 was clarified.  Decision: Adopt these aspects (items 3 as modified).

- R0071 On cross-layer impacts of IRAP and EOS  A new item 4 was added, regarding the derivation of NoOutputOfPriorPicsFlag for a INBL and its dependent layers. Decision: Adopt these aspects (item 4).

- R0272 HEVCv1/ Conformance cleanups  The proposal would be to add PTL information for the base layer sub-bitstream. The PTL seen by a version 1 decoder contains the entire bitstream. The PTL for the 0-th OLS is placed at the beginning of the VPS extension (when the base layer is not provided by external means and there is some enhancement layer). Decision: Adopted (item 1).

- R0272 HEVCv1/ Conformance cleanups  Clarify that directly nested BP, PT, and DUI SEI messages only apply to the operation points that correspond to base-layer-output-only OLSs. Decision: Adopted (items 3 and 4), with key elements of the text provided in -v2.

- R0156 Move conformance cropping window parameters to rep_format( ) syntax structure of VPS extension It is proposed to move the signaling of the conformance cropping window parameters of enhancement layers from the SPS to the VPS extension, as part of the rep_format( ) syntax structure. It is asserted that this movement will enable more frequent parameter set sharing.Decision: Adopt.

- R0230 Unavailable and useless stuff  Decision (Ed.): However, it was agreed, as an editorial matter, to only discuss the conformance requirements of OLSs that contain at least one VCL NAL unit.

- R0230 Unavailable and useless stuff  Editor action item: Also add a note, to advise encoders to be careful about fixed frame rate and CBR indications in enhancement layers.

- R0230 Unavailable and useless stuff  Decision (Ed.): Only require conformance of extracted sub-bitstreams corresponding to operation points for which all layers of the operation point (both the "necessary" and "unnecessary" layers for the decoding of the output layers) have VCL data in the bitstream and for which the highest TID of the operation point is a TD in that VCL data.

- R0279 More miscellaneous cleanups  Clarification of the semantics of access unit delimiter RBSP to be applicable in multi-layer context (text in an attachment) The proposal is to specify that the pic_type syntax element, which identifies the slice types allowed in the current picture, applies to all pictures in the AU.Decision (Ed.): Adopt.

- R0279 More miscellaneous cleanups  Clarification of the specifications of order of NAL units and coded pictures and their association to access units in both subclauses 7.4.2.4.4 and F.7.4.2.4.4 (text in an attachment) Decision (Ed.): Append "with layer_id equal to 0" to ordering constraints for AUD NAL unit in clause 7. Editor action item: Some other elements are clearly editorial and are delegated to the editors for consideration.+

- R0279 More miscellaneous cleanups  To specify that for any independent non-base layer the used representation format is the one that is signalled in the active SPS for the layer. Decision (Ed.): Adopt. Editor action item: It was recommended for the editors to also add some note or expression of a requirement that the representation format must be signalled in the active SPS.

- R0279 More miscellaneous cleanups  To clarify the use of parameters in parameter sets for interpretation of semantics of the frame-field information SEI message, including the following two parts, and it is also suggested to make a systematic check for other SEI messages for the need of similar clarifications. Decision (Ed.): Adopt (both identified aspects, and further similar clarifications if identified by the editors)

- R0279 More miscellaneous cleanups  To clarify the semantics of the temporal sub-layer zero index SEI message for its use in multi-layer contexts, and to add a syntax element to indicate the number of previous consecutive temporal sub-layer zero pictures that have discardable_flag equal to 1 (text in an attachment). The contribution proposed extending the syntax of the temporal sub-layer zero index SEI message. It was remarked that the proposal seems to change the spirit of what the prior interpretation of the SEI message would be. Editor action item: Add a note to advise encoders that, when using this SEI message, not to set discardable flag = 1 for TL0 pictures that are not RASL, RADL or sub-layer non-reference pictures. Decision (Ed.): Change semantics to clarify that it applies to the pictures of the associated layers.

- R0279 More miscellaneous cleanups  6. To change the recovery point SEI message to correct the POC derivation when random access is performed and to clarify/add the POC derivation when layer-switching is performed. Decision: Adopt constraint on presence of the SEI message as described in "Option 1" of the -v2 version of the contribution.

- R0236 On access unit boundary detection  [Ed. Add clarification – note that the AVC definition of "picture" which is what we were assuming when we wrote version 1, was define a picture as containing all VCL NAL units of the AU. We later changed the definition of "picture" for scalability extension, and this caused the problem.]

- R0236 On access unit boundary detection  Decision: Adopt the proposed clarification and issue a defect report document that highlights the issue.

- R0236 On access unit boundary detection  The phrasing in in subclause 7.4.2.4.4 is not explicit whether nuh_layer_id equal to 0 or any nuh_layer_id value is considered in determining the first NAL unit of an access unit. This aspect is a clarification of the prior basis text. Decision (Ed.): Adopt this aspect.

- R0226 Modification to semantics of slice_temporal_mvp_enabled_flag.

- R0226 Modification to semantics in temporal motion vector prediction constraints SEI message.

- R0227 Signalling of bit rate and picture rate information for additional layer sets.

- R0227 Adding a restriction on update of representation format of the base layer.

- R0227 Bug fix and simplification for value ranges of num_negative_pics, num_positive_pics, and num_long_term_pics.

- R0227 Add semantic constraint of vps_vui_bsp_hrd_present_flag.

- R0227 Remove restriction on update of separate_color_plane_flag in SPS.

- R0227 It was suggested that when the base layer is internal, we should send the VST also for the base layer along with the other layers (e.g., for session negotiation purposes). Decision: Agreed (send VST for base layer when internal).

- R0276 Generalize scalable nesting SEI message so that it can be applied to additional layer sets.

- R0276 Editorial cleanups to derivation of BitstreamToDecode in the HRD and semantics of scalable nesting SEI message and the sub-bitstream property SEI message to properly reference the subclauses of F.10.x. Decision: The BoG recommended actions are agreed (see also additional notes for R0276).

- R0154 Modify semantics of default_output_layer_idc and output_layer_flag[ i ][ j ] to consistently handle auxiliary pictures.

- R0154 Add inference value to alt_output_layer_flag[ i ].

- R0155 Modify the semantics of slice_type for I slice_type with respect to IRAP pictures and sps_max_dec_pic_buffering_minus1[ TemporalId ] value to enable inter-layer prediction could be used for pictures with nuh_layer_id > 0.

- R0155 Constrain the allowed range for values of output_layer_set_idx_to_vps[ i ].

- R0157 Constrain DependencyId and ViewOrderIdx to increase with increasing values of nuh_layer_id (text in v2 version). Decision: The BoG recommended actions are agreed (see also additional notes for R0276).

- R0010 JCT-VC AHG report: Layered coding constraint specifications and capability indications  Scalable Main and Stereo Main decoders should be capable of decoding Main profile bitstreams (at least when the base layer is not provided by external means). Decision (Ed.): Confirmed (and Scalable Main 10 should decode Main 10).

- R0010 JCT-VC AHG report: Layered coding constraint specifications and capability indications  When the target output layers of an output layer set do not include the auxiliary pictures (or any such non-target "unnecessary" layers), the bits of the auxiliary/non-target pictures in the extracted bitstream subset would be counted in the CPB buffer flow, but the decoding of the auxiliary pictures would not be part of the associated decoding process requirements. Decision (Ed.): Confirmed.

- R0010 JCT-VC AHG report: Layered coding constraint specifications and capability indications  When the target output layers of an output layer set do include the auxiliary pictures, the profile specification for (partitions of) the output layer set would need to include decoding process requirements for decoding those pictures. Unless some way is found to specify the necessary decoding process capabilities (e.g., by a profile definition that includes such capabilities), the specification would need to disallow having target output layers of an output layer set that are auxiliary pictures Decision (Ed.): Confirmed.

- R0010 JCT-VC AHG report: Layered coding constraint specifications and capability indications  It was suggested that the profile/tier/level information for such an output layer set should include all the layers in the output layer set in the CPB operation but not include the "unnecessary" layers in its decoding process requirements. In principle, it was suggested to be desirable to support this possibility (esp. for the future extensibility purpose), and suggested that we should conduct further study to ensure that the specification is consistent with this design intent. Decision (Ed.): Confirmed that this should be supported.

- R0010 Layered coding constraint specifications and capability indications  Regarding DPB specification (incl. Q0103 / JCT3V-H0034) level limit constraints related to maximum DPB size, the VPS-level DPB needs to have constraints as well as the SPS level having such constraints (each layer in the TargetDecLayerIdList shall obey profile-specific level limit in A.4.1 d, which applies to sps_max_dec_pic_buffering_minus1[ HighestTid ]). Decision (Ed.): Confirmed.

- R0010 Layered coding constraint specifications and capability indications  The text should clearly specify the following: Values of general_profile_idc for indicating SHVC and MV-HEVC profiles. Decoding requirements specification needs to require decoding (an editorial oversight) If the capabilities of one profile are "nested" within the capabilities of another profile, decoders should be required to recognize that and decode the bitstream. Decision (Ed.): Confirmed. Editors are requested to share general_profile_idc for naturally-nested profiles (e.g., Scalable Main 10 and Scalable Main) as was done in RExt. Monochrome 8 should use the RExt general_profile_idc. Some details of Monochrome 8 are yet to be defined. Define these in a manner consistent with other RExt profiles unless some contribution indicates otherwise. Nesting relationships are to be specified for Main, and Main Still Picture for all three new profiles and for Main 10 for the Scalable Main 10 profile.

- R0010 Layered coding constraint specifications and capability indications  Make the PPS syntax HEVCv1 compatible, and enable the syntax of SPS with nuh_layer_id > 0 to be either HEVCv1 compatible or not.  Decision (Cleanup): Confirmed.

- R0010 Layered coding constraint specifications and capability indications  Simplify the bitstream rewriting process for independent non-base layers (part of R0042). Decision (Cleanup): Confirmed.

- R0010 Layered coding constraint specifications and capability indications  Specify independent non-base layer decoding for profiles specified in Annex A. For example, the following phrasing could be used "When expressing the capabilities of a decoder for a profile specified in Annex A, the capability of decoding an independent non-base layer should be expressed." Decision (Cleanup): Confirmed.

- R0010 Allow an SHVC/MV-HEVC bitstream to be considered conforming without containing a base layer (i.e., without containing pictures of layer 0), and indicate such a bitstream using the vps_reserved_one_bit (with the syntax element name and semantics changed) Decision (Cleanup): Confirmed.

- R0010 Remove the bitstream partition HRD parameters SEI message. Decision (Cleanup): Confirmed.

- R0010 Wording improvement is needed for item b of the profile specific level limits, to correctly implement the intent to consider output times of pictures in different AUs. Editor action item: Editors to consider.

- R0010 It is noted that the drafted Scalable Main profile does not support monochrome decoding. Decision (Cleanup): Remove constraint prohibiting 4:2:0 for alpha and depth and specify the the semantics of associated SEI messages, if not already specified, to refer to the luma component only. True monochrome will not be required to be supported (parent).

- R0010 R0041 is on this topic. Text was requested for using the last of the "44 reserved bits" for this indicator. The text was provided in a revision of R0041 and reviewed Minor comments included harmonization with RExt, including the flag for sub-layers, requiring the flag to be equal to zero if the associated layer ID is zero and otherwise equal to 1, clarifying the scope of the flag to CLVS scope, having the rewriting process set the flag to 0, and making the value of the flag an exception to the requirement for the PTL in the SPS to match that of the VPS for an INBL. (The question of whether the rewriting process is removed or not is an editorial matter, and it seems that we may retain it for convenience while providing additional informative text.) Decision: Confirmed (modified as suggested above for 07-06 review).

- R0362/Parameter set repetition: R0010 Should some additional adjustment be made of the syntax of independent non-base layers in regard, e.g., to VPS repetition and handling of end of sequence and end of bitstream NAL units? Consider allowing VPS presence in independent non-base layer (text preparation requested). Check whether other PSs can be repeated in other layers.  Decision (Cleanup): It was agreed to allow this for all PSs (observing that SPSs in layer 0 must use compatible syntax). Revisit for text review.  Decision (Ed.): Regarding ordering of pictures within one AU, it is intended to require the VCL NAL units of the picture to be in the order of increasing layer ID value.  Text in R0362 parameter sets repetition

- R0360 YKW Cleanup Editor action item: Most aspects were suggested to be clearly editorial and were delegated to the editors for consideration. Item 2 relates to ordering of pictures in an AU, which was found in Annex F. This seemed adequate, so the suggested change may be unnecessary.

- R0043 SHVC/MV-HEVC level definitions and related decoder capability requirements Decision: The intent is agreed as stated above; the editors are delegated the task of expressing that in the language of the text.

- R0361/VUI cleanups: R0361 Adarsh VUI cleanups

- JCT3V-I0103 MV-HEVC HLS: Clean-up of MV-HEVC SEI messages I0103 rev1

- JCT3V-I0134 aspects 1 and 3

- JCT3V-I0199 (BoG editors' notes), in addition to other items, view_id_len: remove the value range of the syntax element.

- Remaining items identified through BoG on editors' notes in SHVC/MV-HEVC specs in R0359/I0199

- R0357 Updates to chroma resampler hint SEI message

- Updates to knee function info SEI message Decision: Agreed to include the message with the modifications agreed above. Further editorial improvement during the editing period for finalization also seems desirable.

- Add constraint on SHVC specific syntax to be zero in MV-HEVC profile definition, including, single_layer_for_non_irap_flag equal and colour_mapping_enabled_flag.

The recommendations of the MV-HEVC / 3D-HEVC Draft and Test Model editing AHG are to:

- Approve the edited documents I1001, I1002 and I1003 as JCT-3V outputs.

- Continue to edit documents I1001 and I1003 to ensure that all agreed elements of 3D-HEVC are fully described, in particular integrate I0129 (ARP).

- Compare the documents I1001, I1002 and I1003 with the HTM-software and resolve any discrepancies that may exist, in collaboration with the Software AHG.

- Continue to improve the overall editorial quality of the documents I1001, and I1003.

- Ensure that properly drafted candidate text is available prior to making any decision to change the specifications.

- Reviewing specification text of each adopted proposal, since 3D-HEVC is close to finalization.

- Discuss reported open issues.

JCT3V-J0003 JCT-3V AHG Report: 3D-AVC Software Integration (AHG3)  [D. Rusanovskyy, F. C. Chen, J. Y. Lee, J.-L. Lin, O. Stankiewicz, T. Suzuki, D. Tian]

JCT-3V AhG3 conducted development of the 3D-AVC software and its distributions to the JCT-3V community. The most recent version of the reference software was released on 22.Aug.2014 and is currently available from the following location:

http://mpeg3dv.nokiaresearch.com/svn/mpeg3dv/tags/3DV-ATMv13.1/

Reference 3DV-ATMv13.1 software was also released as JCT3V/MPEG output documents, and are available online as I1005 and w14673 respectively.

With respect to the second mandate, the software provided in the JCT3V-I0025 and approved by JCT-3V group was utilized as a codebase and it was updated to resolve a known High Level Syntax bug, related to a Slice Header Prediction.

With respect to the third mandate, a document describing software usage was created and distributed along with the software. Software manual document can be found at the SW repository and JCT3V/MPEG documents in the /docs folder.

http://mpeg3dv.nokiaresearch.com/svn/mpeg3dv/tags/3DV-ATMv13.1/docs/

The software 3DV-ATMv13.1 was utilized in work of JCT3V-AhG9 to produce MVC+D and 3D-AVC conformance bitstreams.

During editing of 3D-AVC specification text, several issues related to the text and the software implementation has been brought to attention.

A revision to the text in J.8.2 was proposed during the editing. Amount of proposed to the text changes suggested that editors and proponents of related tools to review them and consider for including in the next amendment (MFC+D).

In addition to this, it was found that signaling of syntax element bvsp_flag_lx in specification text and reference software is not aligned.  The text specifies that signaling of syntax elements ref_idx_l0 and bvsp_flag_l0 are interleaved, whereas software implements signaling of these two elements in independent loops. It is expected that identified missaligment does not affect the coding performance.

Possible solution to the latter issue may include either introducing changes to the reference software with reproducing 3D-AVC conformance bitstreams, or introducing changes to the specification text (introducing independent loop for signaling of bvsp_flagl0),  along with changes proposed to J.8.2 text.

The report further points out another mismatch between text and software in the context of VSP.

JCT3V-J0004 JCT-3V AHG Report: MV-HEVC and 3D-HEVC Software Integration (AHG4) [G. Tech, H. Liu, Y. Chen, K. Wegner]

Development of the software was coordinated with the parties needing to integrate changes.

The distribution of the software was announced on the JCT-3V e-mail reflector and the software was made available through the SVN server:

https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/

Anchor bitstreams have been created and uploaded to:

ftp.hhi.fraunhofer.de; login: mpeg3dv_guest; path: /MPEG-3DV/HTM-Anchors/

One version of the HTM software were produced and announced on the JCT-3V email reflector. The following sections give a brief summary of the integrated tools and achieved coding gains.

Starting point for development of HTM-12.0 was HTM-11.2. Development of HTM-12.0 was conducted in three parallel tracks each performing sequential integration. Development of each branch has been supervised by one software coordinator. Software of all three tracks was merged by the software coordinators.

MV-HEVC: The coding results for MV-HEVC are identical to results obtained from version HTM-11.2.

3D-HEVC: HTM-12.0 vs. HTM-11.2 (CTC, three view configuration)

|  | video video rate | video total rate | synth total rate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,3% | 0,3% | 0,2% | 98,7% | 100,9% | 99,4% |
| Kendo | 0,2% | 0,1% | -0,2% | 96,2% | 103,9% | 98,0% |
| Newspaper_CC | 0,1% | 0,1% | -0,1% | 98,5% | 103,0% | 101,0% |
| GT_Fly | 0,0% | -0,1% | -0,6% | 100,6% | 95,6% | 99,7% |
| Poznan_Hall2 | 0,0% | 0,0% | -0,2% | 101,2% | 100,3% | 98,4% |
| Poznan_Street | 0,0% | 0,0% | -0,1% | 99,7% | 101,6% | 99,5% |
| Undo_Dancer | 0,0% | 0,1% | -0,1% | 100,4% | 96,0% | 98,7% |
| Shark | 0,0% | -0,1% | -0,5% | 100,3% | 102,0% | 98,9% |
| 1024x768 | 0,2% | 0,2% | 0,0% | 97,8% | 102,6% | 99,5% |
| 1920x1088 | 0,0% | 0,0% | -0,3% | 100,4% | 99,1% | 99,0% |
| average | 0,1% | 0,0% | -0,2% | 99,4% | 100,4% | 99,2% |

The MV-HEVC Software draft 1 (JCT3V-I1009) has been released. The software has been generated by removing 3D-HEVC related source code and configuration files from HTM-12.1. The software can also be accessed using the svn:

https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/branches/HTM-12.1-MV-draft-1

For MV-HEVC non-CTC configuration parameters files have been included e.g. for generation of bitstreams using INBL decoding, auxiliary pictures, layer-wise startup, and IBP prediction.

Open issues:

- Some minor mismatches related to 3D-HEVC HLS.

- Most of MV-and 3D-HEVC SEI messages are not supported yet.

- Some items related to MV-HEVC decoding processes (e.g. hybrid scalability, correct bumping, POC reset) have not been integrated yet.

- Especially MV-HEVC related decoding processes and syntax need to be reviewed and tested.

- Other minor issues are listed in the bug tracking system.

The recommendations of the MV-HEVC and 3D-HEVC Software Integration AHG are:

- Develop HTM-13 based on HTM-12 and improve its quality.

- When a proposal is adopted to discuss how to enable it in the HTM software (e.g. encoder parameter / parameter set flag, or always on).

- Continue to identify bugs and discrepancies with text, and address them.

- Fix open issues.

- Discuss alignment of HTM with HM versions including range extension.

- Discuss on how to address open issues.

It is mentioned that the alignment of 3D-HEVC software (HTM) with HM16 would cause a significant amount of work.

Should MV-HEVC software be aligned with SHVC software? The encoder implementation may be a bigger issue than the decoder.

Revisit on these issues, also discuss in joint meeting with JCT-VC.

JCT3V-J0005 JCT-3V AHG Report: 3D Coding Verification Testing (AHG5) [V. Baroncini, K. Muller, S. Shimizu, A. Vetro, S. Yea]

Mandate 1: 	Prepare a set of test sequences and encodings to be used in upcoming MV-HEVC verification testing.

During the last meeting, it was suggested to compare four configurations: MVC, simulcast HEVC, frame-compatible HEVC, and MV-HEVC. And 8 test sequences are identified for the verification test.

Many offline activities were done to prepare a set of encoded bitstreams. Unfortunately, no material is available now, but some of them can be prepared during the meeting.

Mandate 2: 	Identify test sites that could conduct such tests.

No test site is identified at this moment at this time since a more concrete test plan is necessary.

Mandate 3: 	Suggest Improvements to the draft test plan JCT-3V-I1011.

The AHG does not have any recommendations at this time on changes to the evaluation methodology, but would certainly consider this based on the experiments.

Mandate 4: 	Prepare viewing logistics for 10th JCT-3V meeting.

The 3D viewing equipment has been prepared and the test room XXX has been setup. Viewing will be performed during the week.

Revisit: Further discuss with Vittorio.

JCT3V-J0006 JCT-3V AHG report: 3D High level syntax (AHG6) [Y. Chen, T. Ikai]

There are three input documents which are relevant to this AhG.

The documents are listed as follows:

| Doc No. | Title |
| --- | --- |
| JCT3V-J0044 | On camera parameter transmission for 3D-HEVC |
| JCT3V-J0060 | 3D-HEVC HLS: Single depth flag signaling |
| JCT3V-J0063 | Improvement of Alternative depth info SEI message in 3D-HEVC |

Another late contribution: J0107. This also includes the necessity to bring the enabling flags from VPS to SPS or PPS.

JCT3V-J0007 JCT-3V AHG Report: Complexity Assessment (AHG7) [G. G. (Chris) Lee (NCKU), G. Bang (ETRI), T. Ikai (Sharp), H. Liu (Qualcomm)]

Mandate 1: Study intrinsic complexity measures or metrics to assess hardware and software implementations of algorithms/computational models based on number of operations, potential for parallelism, data transfer rate, and data storage requirements. An informative guideline which introduces complexity assessment without biasing towards either software or hardware has been presented in JCT3V-D0301 Error: Reference source not found.

- Discussed, identified and tabulated coding tools or cases to be studied at different suggested data granularities in a spreadsheet which provides a high level overview of the scope of the work in this ad hoc group. Furthermore, several examples performed by co-chairs and proponents in previous JCT3V meetings were also included in this spreadsheet which was uploaded together with AHG 7 report.

Mandate 2: Analyze existing designs and produce complexity perspective per tool, from the viewpoint of possible slowdown, bottlenecks or implementation issues.

- According to Mandate 1, complexity of several coding tools are assessed and they also are discussed the bottleneck per tools, such as depth intra coding tools, inter-prediction tools, etc. The analyzed results also included the attached spreadsheet which is illustrated in Mandate 1.

Mandate 3: Develop a recommendation to JCT-3V on formalized complexity assessment in a reporting document.

- A draft report summarizes the guidelines of this AHG in assessing complexity and states the definition of complexity metrics for evaluating coding tools in a fair perspective.

JCT3V-J0061, AHG7: Draft Report of Complexity Assessment

A summarized report of complexity assessment AHG in 3D-HEVC. This draft report re-states the metrics used to evaluate the coding tools in previous meetings and provides an example to guide the complexity assessment.

This AHG on Complexity Assessment recommends to:

- review the draft report during the meeting and finalize mandate 3 at next meeting

JCT3V-J0008 JCT-3V AHG Report: 3D Test Material (AHG8) [S. Shimizu, S. Yea]

There was no e-mail exchange over the reflector.

At the last meeting, we listed some stereo materials for the MV-HEVC verification testing. Some of them are already available on the servers, but test sequences provided by Deutsche Telekom are not currently accessible by JCT-3V members. Copyright statement is also missing for these sequences at this time. It is necessary to discuss the content holder about the possibility and the way on the distribution. One distribution way is to use the MPEG content repository.

To enable access by ITU members, also an alternative download facility should be installed.

AVC Conformance testing development (AHG9) T. Suzuki (chair), D. Rusanovskyy, D. Tian, Y. W. Chen (vice chairs)

JCT3V-J0010 JCT-3V AHG report: HEVC Conformance testing development (AHG 10) [Y. Chen, T. Ikai, S. Shimizu, T. Suzuki]

During the last meeting in Sapporo, this AhG has been established and the activities on collecting the MV-HEVC and 3D-HEVC bitstreams started.

The latest draft for MV-HEVC conformance test is available as JCT3V-I1008 and the latest draft for 3D-HEVC conformance is available as JCT3V-I1010.

Several companies have started generating bitstreams for 3D-HEVC as well as MV-HEVC.

A final software for MV-HEVC was recently released and can be used to generate the MV-HEVC bitstreams.

One volunteer, KDDI has offered to double-check the MV-HEVC bitstreams with its own non-HTM based decoder implementation.

3D-HEVC

| Categories | Sub category | Bitstream | File name | Profile and Level | Provided |
| --- | --- | --- | --- | --- | --- |
| Texture tool | ARP | 3DHC_T_A | 3DHC_T_A_Qualcomm_5 | 5.1 and higher | X |
|  | Sub-PU inter-view motion prediction | 3DHC_T_B | 3DHC_T_B_MediaTek_5 | 5.1 and higher | X |
|  | Illumination compensation | 3DHC_T_C | 3DHC_T_C_Sharp_5 | 5.1 and higher | X |
|  | Combined | 3DHC_T_D | 3DHC_T_D_Sharp_5 | 5.1 and higher | X |
|  | Combined texture only bitstream | 3DHC_T_E | 3DHC_T_E_HHI_5 | 5.1 and higher |  |
| Depth tool | Depth Intra (DMM1) | 3DHC_D1_A | 3DHC_D1_A_HHI_5 | 5.1 and higher |  |
|  | Depth Intra (DMM1) | 3DHC_D1_B | 3DHC_D1_B_HHI_5 | 5.1 and higher |  |
|  | Depth Intra (SDC) | 3DHC_D1_C | 3DHC_D1_C_RWTH_5 | 5.1 and higher |  |
|  | Depth Intra (SDC) | 3DHC_D1_D | 3DHC_D1_D_RWTH_5 | 5.1 and higher |  |
|  | Depth Intra (single depth) | 3DHC_D1_E | 3DHC_D1_E_MediaTek_5 | 5.1 and higher | X |
|  | Depth Intra (single depth) | 3DHC_D1_F | 3DHC_D1_F_MediaTek_5 | 5.1 and higher | X |
|  | Depth Intra (combined) | 3DHC_D1_G | 3DHC_D1_G_RWTH_5 | 5.1 and higher |  |
|  | Depth Intra (combined) | 3DHC_D1_H | 3DHC_D1_H_RWTH_5 | 5.1 and higher |  |
| Depth tool | Depth Inter (inter-view motion) | 3DHC_D2_A | 3DHC_D2_A_Samsung_5 | 5.1 and higher |  |
|  | Depth Inter (inter-SDC) | 3DHC_D2_B | 3DHC_D2_B_LGE_5 | 5.1 and higher |  |
| Depth dependent texture tool | DoNBDV | 3DHC_DT_A | 3DHC_DT_A_MediaTek_5 | 5.1 and higher |  |
|  | VSP | 3DHC_DT_B | 3DHC_DT_B_NTT_5 | 5.1 and higher |  |
|  | DBBP | 3DHC_DT_C | 3DHC_DT_C_Hisilicon_5 | 5.1 and higher |  |
|  | Combined | 3DHC_DT_D | 3DHC_DT_D_NTT_5 | 5.1 and higher |  |
| Texture dependent depth tool | Sub-PU MPI | 3DHC_TD_A | 3DHC_TD_A_Qualcomm_5 | 5.1 and higher | X |
|  | MPI | 3DHC_TD_B | 3DHC_TD_B_MediaTek_5 | 5.1 and higher | X |
|  | DDD | 3DHC_TD_C | 3DHC_TD_C_MediaTek_5 | 5.1 and higher |  |
|  | QTL | 3DHC_TD_D | 3DHC_TD_D_Orange_5 | 5.1 and higher |  |
| Others | 3-view random access | 3DHC_C_A | 3DHC_C_A_HHI_5 | 5.1 and higher |  |
|  | 3-view all Intra | 3DHC_C_B | 3DHC_C_B_HHI_5 | 5.1 and higher |  |
|  | 2-view random access | 3DHC_C_C | 3DHC_C_C_Sharp_5 | 5.1 and higher | X |

MV-HEVC

| Categories | Sub category | Bitstream | File name | Provided |
| --- | --- | --- | --- | --- |
| Prediction Structure (2-view) | Inter-view prediction | MVHEVCS-A | MVHEVCS_A_Qualcomm_5 |  |
|  | All Intra | MVHEVCS-B | MVHEVCS_B_Sharp_5 |  |
|  | Simulcast | MVHEVCS-C | MVHEVCS_C_Sony_5 |  |
|  | Simulcast with asymmetric resolutions | MVHEVCS-D | MVHEVCS_D_NTT_5 | X |
|  | Inter-view prediction and hierarchical B | MVHEVCS-E | MVHEVCS_E_Qualcomm_5 |  |
|  | Inter-view prediction for IRAP AUs only | MVHEVCS-F | MVHEVCS_F_Qualcomm_5 |  |
| Prediction Structure (3-view) | Inter-view prediction | MVHEVCS-G | MVHEVCS_G_NTT_5 |  |
|  | Inter-view prediction with PIP view structure | MVHEVCS-H | MVHEVCS_H_LGE_5 |  |
|  | Inter-view prediction with IBP view structure | MVHEVCS-I | MVHEVCS_I_Nokia_5 |  |
| Hybrid scalability |  | MVHEVCS-J | MVHEVCS_J_XXX_5 |  |

The AHG recommends

- to improve and complete the draft text for MV-HEVC conformance test and 3D-HEVC conformance test

- to collect the missing conformance bitstreams for 3D-HEVC

- to collect the missing conformance bitstreams for MV-HEVC

- to discuss procedures on how to cross verify the 3D bitstreams

Further discussion & BoG during week

Revisit: Consider issuing PDAM from this meeting?

# Project development, status, and guidance (1)

## Communication by parent bodies (0)

## 3D video application areas (0)

## MV-HEVC / 3D-HEVC Conformance (0)

## Profile/level definitions (1)

JCT3V-J0106 Initial text for the specification of profiles, tiers, and levels in 3D-HEVC [G. Tech, K. Müller (HHI)] [late]

On 3D-HEVC

## (chaired by …)

## Complexity assessment (1)

(chaired by …)

JCT3V-J0061 AHG7: Draft Report of Complexity Assessment [G. G. Lee (NCKU), T. Ikai (Sharp), H. Liu (Qualcomm), G. Bang (ETRI)]

## Quality assessment (0)

## Source video test material (0)

# Core experiments

## CE1: Segmental prediction (4)

(chaired by JRO, Sat. 11:40-12:10)

### Summary (1)

JCT3V-J0011 CE1: Summary Report on Segmental Prediction [J.-L. Lin]

In JCT3V-J0032, a segmental prediction method for Inter-SDC is proposed in depth coding. Besides the current Inter-SDC mode, three segmental Inter-SDC modes with 1, 2 and 3 segments are added. A single depth value is assigned to all reconstructed samples in a segment.

For a CU which is coded with Inter-SDC mode, a flag is signaled to indicate whether to apply the normal Inter-SDC or the segInter-SDC. When the segInter-SDC is enabled, two steps are applied to obtain the reconstructed block from the prediction block.

- In the first step, samples in the current block are classified into one, two or three segments.

- In the second step, a single value is derived for each segment in the reconstructed block.

Fig. 1 An example of segInter-SDC with 2 segments for an 8x8 block.

The following aspects should be further studied in this context:

-	Usage in combination with Intra SDC (where it would target similar depth map characteristics as DMM)

-	Restriction of number of segments to two (instead of three)

Table 1.  Configuration for tests

|  | 1 segment | 2 segments | 3 segments | For Inter-SDC | For Intra-SDC |
| --- | --- | --- | --- | --- | --- |
| Test 1 | Yes | Yes | Yes | Yes | No |
| Test 2 | No | Yes | Yes | Yes | No |
| Test 3 | Yes | Yes | No | Yes | No |
| Test 4 | No | Yes | No | Yes | No |
| Test 5 | No | Yes | Yes | Yes | Yes |
| Test 6 | No | Yes | No | Yes | Yes |

#### Coding results

|  | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1 | 0.0% | 0.0% | 0.0% | 0.1% | -0.4% | 114.1% | 101.5% |
| Test 2 | 0.0% | -0.1% | 0.0% | 0.1% | -0.5% | 107.3% | 101.7% |
| Test 3 | 0.0% | 0.0% | 0.0% | 0.1% | -0.3% | 109.8% | 101.2% |
| Test 4 | 0.0% | 0.0% | 0.0% | 0.1% | -0.3% | 105.5% | 100.7% |
| Test 5 | 0.0% | 0.0% | 0.0% | 0.1% | -0.6% | 108.4% | 99.2% |
| Test 6 | 0.0% | 0.0% | 0.0% | 0.0% | -0.4% | 106.3% | 101.1% |

The case of 1 segment is similar to current SDC

Basically, the segmentation could be similar as with current tools (like DMM)

The approach requires computing the mean value in order to determine a threshold for segmentation (necessary at the decoder), which likely increases the complexity.

Furthermore, it is pointed out that the worst-case complexity of intra mode is increased, since the method allows to combine with angular prediction (which DMM does not allow)

One expert expresses the opinion that for the case of 3 segments (which requires two thresholds) the complexity would be too large.

Revisit: Provide results that use only the four corner samples to derive the threshold. Prepare corresponding draft text with exclusively two segments; provide a detailed analysis about the complexity impact in inter and intra (showing whether and by how much the worst case complexity increases).

    - CE contributions (3)

JCT3V-J0032 3D-CE1: Segmental prediction in 3D-HEVC [K. Zhang, J. An, X. Zhang, H. Huang, J.-L. Lin, S. Lei (MediaTek)]

JCT3V-J0031 CE1: Cross-check on Test 2 and Test 4 [T. Tsukuba (Sharp)] [late]

JCT3V-J0074 CE1: Cross check of segmental prediction, test3 (JCT3V-J0032) [P. Merkle (HHI)] [late]

JCT3V-J0098 CE1: Crosscheck on CE1 test 6 (JCT3V-J0032) [X. Zheng, J. Zheng (Hisilicon)] [late] [miss]

JCT3V-J0102 3D-CE1: Crosscheck on Segmental prediction in 3D-HEVC (JCT3V-J0032) [S. Shimizu (NTT)] [late]

### Related contributions (0)

# 3DV standards development (incl. software, conformance) (3)

## AVC Extensions (3)

JCT3V-J0056 On MFC plus Depth: Reference Software Update [L. Grimaldi, K. Schueuer, P. Yin, T. Lu, H. Ganapathy, T. Chen, W. Husak (Dolby)]

TBP

JCT3V-J0057 On MFC plus Depth: Conformance Testing [P. Yin, T. Lu, H. Ganapathy, T. Chen, W. Husak (Dolby), D. Tian (MERL)]

TBP

JCT3V-J0062 Software for Texture and Depth View Packing SEI message in ATM13.1 [T. Senoh, K. Wakunami, Y. Ichihashi, H. Sasaki, R. Oi, K. Yamamoto (NICT)] [late]

This contribution reports the integration of texture and depth view packing SEI message in ATM13.1. It is proposed to include this software in the reference software for 14496-10:201x/AMD1.

Was presented

JCT3V-J0063 Improvement of Alternative depth info SEI message in 3D-HEVC [T. Senoh, K. Wakunami, Y. Ichihashi, H. Sasaki, R. Oi, K. Yamamoto, M. Tanimoto (NICT)] [late]

An updated version of the text should be provided which takes into account the ballot comments on the AVC amendment, and also improves the quality of the description w.r.t. avoiding encoder perspective (register this as a new document “update of SEI message in … AVC”, and after further discussion apply similar changes to the HEVC SEI message.

Revisit

## MV-HEVC (0)

## 3D-HEVC (0)

# 3D HEVC High-level syntax (4)

JCT3V-J0044 On camera parameter transmission for 3D-HEVC [S. Yoo, J. Nam, S. Yea (LGE)]

JCT3V-J0060 3D-HEVC HLS: Single depth flag signaling [Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek)]

JCT3V-J0063 Improvement of Alternative depth info SEI message in 3D-HEVC [T. Senoh, K. Wakunami, Y. Ichihashi, H. Sasaki, R. Oi, K. Yamamoto, M. Tanimoto (NICT)] [late]

JCT3V-J0064 CShift in HTM [T. Senoh, K. Wakunami, Y. Ichihashi, H. Sasaki, R. Oi (NICT)] [late]

JCT3V-J0107 On 3D-HEVC HLS and its alignment with MV-HEVC HLS [G. Tech, K. Müller (HHI)]

Initial te

## DMM (9)

(chaired by JRO, Sat afternoon)

JCT3V-J0021 Structured lookup table definition in DMM [T. Tsukuba, T. Ikai (Sharp)]

This contribution proposes a structured LUT definition in DMM, which generate 16x16 to 32x32 pattern by scaling and position-shifting wedgelet pattern of 8x8 in order to reduce the storage size. Experimental result reportedly shows that 0.10% and 0.06% bdrate changes for synthesis in the CTC and all-intra configuration, respectively. It is asserted that this proposed method can achieve storage size to 5Kbytes without significant coding loss.

One expert points out that in the last meeting it had been discussed that lossless compression and on-the-fly reconstruction could also reduce the necessary storage.

JCT3V-J0095 Cross-check on Structured lookup table definition in DMM (JCT3V-J0021) [J. Zheng(HiSilicon)] [late] [miss]

JCT3V-J0035 On Lookup Table Size Reduction for DMM1 [X. Zhang, K. Zhang, J. An, H. Huang, J.-L. Lin, S. Lei (MediaTek)]

For the DMM1 intra prediction, it requires 400k-bit storage to store the lookup table in the current design. In this contribution, it is proposed to downsample the wedgelet patterns for DMM1 to reduce the lookup table size. Experimental results reportedly show that the proposed simplification could achieve more than 50% storage saving of the wedgelet patterns with negligible BD-rate increase.

It is proposed to reduce the number of 16x16 patterns by keeping only those that start at even positions, (not downsampled). The storage would be reduced from approx. 400,000 to about 196,000.

The results indicate neglible loss.

JCT3V-J0100 Crosscheck on Lookup Table Size Reduction for DMM1 (JCT3V-J0035) [X. Zheng, Y. Lin (Hisilicon)] [late]

JCT3V-J0025 Simplification of DMM table derivation [T. Ikai, T. Tsukuba (Sharp)]

This contribution proposes to simplify the DMM table derivation process by rotating.  Experimental result reportedly shows 0.01 % and 0.00 % BD-rate changes in synthesis in CTC and all-intra configuration respectively.

The proposal is to harmonize the tables by retaining the generation of table entries only for two directional orientations, and generate remaining table entries by rotation. This way, also the pruning process in the current specification of the tables is removed. In terms of specification, the proposal removes about 1 page of the description, but adds another ¾ pages. It is claimed that at the same time this could be used for compression, if the rotation would be implemented on the fly. In the uncompressed format, the table size would slightly increase. The proposal likely also introduces some (small) additional redundancy due to the removal of the pruning, which seems to be negligible since the bit rate change is marginal.

Basically, this method could also be combined with J0021 and J0035.

Summary on all three proposals: Further information would be desirable, in particular

- How the benefit in compression compares to some other simple compression methods for the original patterns

- What the actual implementation complexity of the decompression is (except for 35 which only reduces the number of 16x16 patterns)

- Combination of the different methods.

Revisit if more information can be provided during the meeting, otherwise start a CE.

JCT3V-J0073 Crosscheck of Simplification of DMM table derivation (JCT3V-J0025) [G. Bang(ETRI), Y.S. Heo, G.H.Park(KHU), G.S. Lee, N.H.Hur(ETRI)] [late]

JCT3V-J0023 Simplification of depth prediction in DMM [T. Tsukuba, T. Ikai (Sharp)]

This contribution proposes a simplified derivation process of predicted DC offsets for DMM by utilizing only partition pattern conditions. Experimental result reportedly shows -0.01 % and -0.01 % BD-rate changes in synthesis in CTC and all-intra configuration respectively.

Presentation deck missing.

The proposal is a modest simplification in the sense that it reduces checking 5 different cases to three cases in the derivation of dcValBR. It consists of two modifications which in combination give no loss on average, however this is not homogeneous over all sequences. Modification 2 is not a simplification but rather accessing data from another CU which slightly improves the compression.

Separate results are given for the two modifications in CTC, and it is requested to provide separate results also for AI condition. These show that in fact modification 1 provides a small loss over all sequences, whereas modification 2 has a small gain which compensates the loss.

Benefit not as obvious to take any action.

JCT3V-J0067 Cross check of simplification of depth prediction in DMM (JCT3V-J0023) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-J0033 A cleanup of DMM index coding in 3D-HEVC [K. Zhang, J. An, X. Zhang, H. Huang, J.-L. Lin, S. Lei (MediaTek)]

In the current 3D-HEVC, the wedgelet table index in DMM1 is binarized as a fixed-length code, which may represent a number exceeding the wedgelet table size. To address this problem, it is proposed to clip the wedge_full_tab_idx into a valid range.

The current spec does not specify a limitation into the valid range. Instead of defining a clipping operation at the decoder (which would likely not lead to a correct value anyway), the specification of the syntax element should include a definition of the valid range.

Decision: Specify the valid range in the semantics of wedge_full_tab_idx

JCT3V-J0080 Cross-check on A cleanup of DMM index coding in 3D-HEVC (JCT3V-J0033) [T.Ikai (Sharp)] [late]

JCT3V-J0049 Restriction of large-sized DMM [J. Nam, S. Yea (LGE)]

In current 3D-HEVC, DMM is disallowed when current CU block size is greater than maximum transform block size and SDC scheme is not applied. However, DMM could be allowed up to 32x32 block size regardless of maximum transform size when SDC is applied. In this contribution, restriction of large-sized DMM is proposed. The proposed method is that DMM is disallowed when current CU block size is greater than maximum transform block size. Experimental results incur no impact in terms of synthesized PSNR.

The software correction bug fix should be confirmed (Y. Chen) – revisit.

Some concerns were expressed about the suggested draft text modification (disallowing DMM globally to be applied for CU sizes larger than max TU size). This does not seem to have any advantage in terms of complexity, but might be desirable in terms of harmonization (not treating DMM with SDC different from other prediction modes).

No action on the normative aspect.

JCT3V-J0104 Crosscheck on Restriction of large-sized DMM (JCT3V-J0049) [S. Shimizu (NTT)] [late]

## Single depth mode (11)

(chaired by JRO, Sat. afternoon)

JCT3V-J0040 Simplification for single depth mode pruning process [S. Yoo, S. Yea (LGE)]

Single depth mode for depth intra coding is adopted at the last meeting. In the single depth mode process, up to five samples on the fixed position from the neighbor samples are compared with each other to remove the duplicated candidates. Once the candidates are pruned, the first two available samples are used and one of them is signaled with an index. However, for most cases, the first two samples are available and they are chosen if they are not same from each other. Therefore, it is proposed to compare only first two samples and make them as the candidate samples. The experimental result reportedly shows that there is no impact on the coding efficiency.

Coding is <0.1% for all sequences, on average 0.01.

JCT3V-J0096 Cross-check on Simplification for single depth mode pruning process(JCT3V-J0040) [J. Zheng(HiSilicon)] [late] [miss]

JCT3V-J0058 Simplification on candidate list construction for single depth mode [X.Chen, X.Zheng, Y.Lin, J.Zheng (HiSilicon)]

At the 9th JCT-3V meeting, method of single depth intra coding was adopted in JCT3V-I0095 that is reconstructed by filling this CU with one single depth value when a CU is coded as single depth mode. The candidate list derivation process in JCT3V-I0095 is relatively high complex, as it needs to insert into the candidate list by analyzing up to five reconstructed spatial neighboring samples around the current CU in the order with pruning process. In this contribution, a low complex approach for candidate list construction is proposed. The impact on the coding efficiency of single depth intra coding is negligible.

Part 1: Check two candidates only, and add default candidate if they are identical, likely identical with J0040; almost no loss

Part 2: Remove pruning process (i.e. don’t add a default candidate if they are identical), 0.01% loss on average

Part 3: Add clipping to valid range (implemented in SW, not in WD)

Part 3 is also similarly proposed in JCT3V-J0052

Side activity of proponents JCT3V-J0040, 0054, 0058p1&2 to identify commonalities and differences in removing the pruning, and suggest a unified solution. Revisit.

JCT3V-J0087 Crosscheck on simplification on candidate list construction for single depth mode (JCT3V-J0058) [X. Zhang (MediaTek)] [late] [miss]

JCT3V-J0052 Clipping for candidate index of single depth mode [J. Seo, J. Nam, S. Yoo, S. Yea (LGE)]

(include abstract)

Identical with JCT3V-J0058 part3

Decision(BF): Align the specification with the software, clipping id necessary.

JCT3V-J0038 Depth intra skip coding (DISC) mode [J. Y. Lee, M. W. Park, C. Kim (Samsung)]

At the last meeting, the single depth mode was adopted (I0095). When a CU is coded as the single depth mode, the reconstruction is performed by filling the CU with a single depth value, which is derived from the spatial neighboring pixels. This contribution proposes to use the pixels generated by one of the conventional horizontal and vertical intra prediction modes for the reconstruction, instead of the single depth value. Since the proposed method does not send the residual after the prediction such as the inter skip mode, it is called a depth intra skip coding (DISC) mode in this contribution. The results shows that the DISC mode has better performance than the single depth mode. Especially, the bit saving of about -1.1% is achieved in Poznan_Hall2. In addition, the single depth value derivation part (I.8.4.4.2.9) can be removed.

Average rate saving: 0.25% in CTC

Signalling is at the CU level (similar as in single depth mode, i.e. enabling flag and then a flag that signals the direction). Both bins are context coded.

Gain is probably due to avoiding signalling the zero residual in the transform tree, and improving the prediction relative to the single depth mode.

Gain could also come due to the fact that not as many intra modes are necessary for depth

Establish CE for further study to identify whether there would be alternative ways to achieve the gain, e.g. by enabling CBF at the RQT root in intra, or by reducing the number of intra modes for depth maps.

JCT3V-J0075 Cross-check on Depth intra skip coding mode (JCT3V-J0038) [T.Tsukuba (Sharp)] [late]

JCT3V-J0053 Simplification on CABAC context models for single depth mode [J. Seo, J. Nam, S. Yoo, S. Yea (LGE)]

In the Sapporo meeting, single depth mode is adopted for depth intra coding. The mode reconstructs the current coding unit as a smooth area with a single depth sample value. For the depth sample, two neighboring pixels are selected as candidates and candidate index is signaled to indicate the selected sample value with CABAC. This contribution presents a simplified method to encode the candidate index of single depth mode without CABAC contexts. The proposed method reportedly shows 0.03% and 0.06% BD-rate loss under the CTC and AI configuration, respectively.

It is reported that the draft specification is missing the context model definition of the two syntax elements (which are both signalled context coded bins)

It is further reported that the search order is different in SW and WD

The proposal suggests to code the second syntax element (candidate) in bypass mode

The contribution reports that in CTC the loss is 0.03% on average, but >0.1% for two sequences.

From the discussion:

The presence of two context coded syntax elements is not critical in terms of complexity (there are worse case of modes), and at least for two sequences losses are observed.

No action on changing the syntax element single_sample_idx to bypass coded

Decision(BF/ed.): Various issue pointed out in this contributions need to be fixed, missing context tables, misalignment between text and software w.r.t. sequence of candidates processing, specification that single_sample_idx is a flag is missing.

JCT3V-J0088 Crosscheck on simplification on CABAC context models for single depth mode (JCT3V-J0053) [Y.-W. Chen (MediaTek)] [late]

JCT3V-J0054 Simplification of single depth intra mode [G. Bang (ETRI), Y.S. Heo, W.W. Gwun, G.H.Park (KHU), G.S. Lee, N.H.Hur (ETRI)]

This contribution proposes removing of derivation process for pruning and deriving virtual candidates during single depth intra prediction mode.

Coding performance: CTC 0% on average (0.05 to -0.08 depending on sequence)

Basically similar to J0040, J0058.

JCT3V-J0082 Cross-check on Simplification of single depth intra mode (JCT3V-J0054) [T.Ikai (Sharp)] [late]

JCT3V-J0065 On Single Depth Mode Simplification [Z. Gu (SCU), J. Zheng (HiSilicon), N. Ling (SCU), P. Zhang (HiSilicon)] [late]

This contribution proposes a simplified candidate pixel selection for Single Depth Mode. It not only saves the candidate list construction process but also reduces the worst case of candidate pixel check from 5 to 2.  Experiment result show that this proposal simplified candidate pixel selection simplifies Single Depth Mode with negligible BD-rate change under both All Intra and CTC configuration. Moreover, about 7% decoding time saving is observed under All Intra configuration.

It appears that other methods removing the pruning process (e.g. 0040, 0054, 0058) are even simpler with comparable or better results.

JCT3V-J0093 Cross-check on Single Depth Mode Simplification (JCT3V-J0065) [T. Tsukuba (Sharp)] [late]

## Other depth intra coding (7)

(chaired by JRO, Sat. afternoon)

JCT3V-J0022 Syntax cleanup of depth dc offset [T. Tsukuba, T. Ikai (Sharp)]

This contribution proposes to clean up dc offset related syntax element by removing depth_dc_flag with a context model adjustment. Experimental result reportedly shows -0.02% and -0.04% bdrate changes for synthesis in the CTC and all-intra configuration, respectively. It is asserted that this proposed method can remove a syntax element and simplify dc offset decoding process with no coding loss.

First part identical to JCT3V-J0047. Small loss (0.04% on average) is observed when the flag is removed from the syntax and instead the value of 1 is added to DC offset. Loss is >0.1% for two sequences in CTC, whereas only two sequences have small rate reduction.

Second part proposes context coding instead of bypass for depth_dc_abs, which compensates for the loss.

The coding loss probably comes due to cases where zero values need to be signalled for depth_dc_abs.

Complexity advantage of removing the flag would be minor, and does not justify the loss.

JCT3V-J0091 Crosscheck on Syntax cleanup of depth dc offset(JCT3V-J0022) [K. Zhang, X. Zhang (MediaTek)] [late]

JCT3V-J0047 Removal of depth_dc_flag syntax [J. Nam, S. Yea (LGE)]

As per decision of the last meeting, depth_dc_flag syntax is no longer used for DMM mode. Therefore, current depth_dc_flag syntax specifies that depth residual is present only when SDC is applied. In the contribution, removal of depth_dc_flag syntax is proposed. depth_dc_flag syntax can be removed by modifying calculation of DC offset. The experimental results show no impact under the common test conditions.

Note: The equation for computing the DC offset in the draft text may be incorrect, the value of 1 should be added (not subtracted) in inter mode

JCT3V-J0072 Cross check of Removal of depth_dc_flag syntax (JCT3V-J0047) [M. W. Park, C. Kim (Samsung)] [late]

JCT3V-J0051 Advanced boundary chain coding for depth intra coding [J. Seo, J. Nam, S. Yoo, S. Yea (LGE)]

This proposal provides results of the advanced boundary chain coding (ABC) based on HTM-12.0. The ABC is an improved coding tool based on the region boundary coding which separates a block into two partitions following an arbitrary contour described by a chain code. This proposal presents the experimental results of ABC without CABAC. The proposed method shows 0.2% and 0.2% BDR gains in synthesized views under CTC and AI configurations, respectively.

Two different methods are proposed: Only first bin of chain code context coded, and entirely bypass coded. The first method would result in 8 context coded bins per 4x4 block.

Average BR reduction in CTC for first method is 0.18%, for the second method 0.16%

Additional text around 5 pages for specifying the mode.

Some concern is raised about the additional implementation complexity of this new mode, compared to the relatively low compression benefit.

No support by other experts – no action.

JCT3V-J0094 Cross-check results of advanced boundary chain coding (JCT3V-J0051) [K. Kawamura, H. Sabirin, S. Naito (KDDI)] [late]

JCT3V-J0105 Crosscheck on Advanced boundary chain coding for depth intra coding (JCT3V-J0051) [S. Shimizu (NTT)] [late]

## IC (3)

(chaired by JRO, Sat evening)

JCT3V-J0034 Complexity reduction on illumination compensation for 3D-HEVC [X. Zhang, K. Zhang, J. An, H. Huang, J.-L. Lin, S. Lei (MediaTek)]

In the current 3D-HEVC, for each block coded with illumination compensation (IC), the LLS(linear least square) method is utilized to calculate α and β parameters for each color component. To reduce the usage of LLS which requires numbers of additional multiplication operations and memory accessing, this contribution proposes to disable bi-prediction for IC and set α as 1 for chroma components. Experimental results reportedly show the proposed simplifications only introduces 0.01% BD-rate increase whereas saves the operation number and bandwidth in worst case.

Method 1: Disallow bi-prediction with IC

Method 2: Simplify IC for chroma by setting gain =1

JCT3V-J0081 Cross-check on Complexity reduction on illumination compensation for 3D-HEVC (JCT3V-J0034) [T.Ikai (Sharp)] [late]

JCT3V-J0099 Crosscheck on complexity reduction on illumination compensation for 3D-HEVC (JCT3V-J0034) [X. Zheng, Y. Lin (Hisilicon)] [late]

JCT3V-J0050 Simplification of chroma IC [J. Nam, S. Yea (LGE)]

In the current 3D-HEVC, to compensate illumination change between inter-view texture pictures, IC which designed by based on linear model including scaling factor a close to 1 and an offset b applies both luma and chroma components. Basically, chroma is not an illumination component but a color component. Therefore, it is required to use IC process for chroma different from that for luma. In this contribution, simplification of chroma IC is proposed. The first option is to remove chroma IC completely. IC process only applies luma component. The other option is to use offset model instead of linear model. Experimental results show 0.1% loss for removing chroma IC and no impact for using offset model.

Two methods are proposed:

Method 1: Completely disable IC for chroma – 0.1% rate increase

Method 2: Simplify IC for chroma by setting gain =1 – identical with method 2 of J0034, negligible loss

Both J0034 and J0050 give an interesting tradeoff between complexity reduction and only small change in compression performance. From the discussion, it is desirable to get better understanding about various implications, e.g. the impact on non-CTC condition (IBP coding) for disabling the bi-prediction with illumination compensation. For the chroma simplification part, it is also suggested to investigate whether the saving in complexity could be re-invested to enable IC for 4x4 chroma blocks (which is currently prohibited)

It is also not clear whether the illumination compensation is critical in terms of worst case complexity.

Further information on these aspects would be necessary. If not possible during the meeting, establish CE.

Several experts expressed support for the part of modifying chroma IC (offset only, i.e. method 2 of both proposals). However, as the results indicate that this has at least a little compression loss, it would be desirable to confirm that IC establishes a worst case in terms of complexity

- Cross-checkers and proponents should investigate the worst case issue and report back

- Proponents of J0034 and J0050 should present a unified version of the draft text concerning their methods 2

Revisit on the bullets above.

Further consideration appears necessary about disabling bi-prediction

- The consequences of this restriction may require more careful investigation beyond CTC

- The restriction may not be necessary if it is not the worst case

JCT3V-J0097 Cross-check on Simplification of chroma IC(JCT3V-J0050) [J. Zheng(HiSilicon)] [late] [miss]

## Merge list construction (7)

(chaired by …)

JCT3V-J0024 Simplification of merge candidates list construction [T. Ikai, T. Tsukuba (Sharp)]

JCT3V-J0084 Crosscheck on simplification of merge candidates list construction (JCT3V-J0024) [X. Zhang, J.-L. Lin (MediaTek)] [late]

JCT3V-J0039 Removal of redundant VSP candidates in Merge mode [J. Y. Lee, M. W. Park, C. Kim (Samsung)]

JCT3V-J0092 Cross-check on Removal of redundant VSP candidates in Merge mode (JCT3V-J0039) [T. Ikai (Sharp)] [late]

JCT3V-J0042 Simplification of depth merge candidate list [S. Yoo, S. Yea (LGE)]

JCT3V-J0068 Cross check of simplification of depth merge candidate list (JCT3V-J0042) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-J0048 Restriction of bi-prediction for IvDC and IvDCShift candidates [J. Nam, S. Yea (LGE)]

JCT3V-J0085 Crosscheck on restriction of bi-prediction for IvDC and IvDCShift candidates (JCT3V-J0048) [X. Zhang (MediaTek)] [late]

JCT3V-J0059 A cleanup of the size derivation for 3D-HEVC merge candidate list [J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

## MV/DV derivation / coding (9)

(chaired by …)

JCT3V-J0027 Cleanup1: Derived DV reference derivation [T. Ikai, T. Tsukuba (Sharp)]

JCT3V-J0069 Crosscheck of Cleanup1: Derived DV reference derivation (JCT3V-J0027) from Sharp [S. Yoo, S. Yea (LGE)] [late]

JCT3V-J0028 Cleanup2: Default DV derivation [T. Ikai, T. Tsukuba (Sharp)]

JCT3V-J0071 Cross check of Cleanup2: Default DV simplification (JCT3V-J0028) [M. W. Park, C. Kim (Samsung)] [late]

JCT3V-J0041 Simplification of an NBDV availability check [S. Yoo, S. Yea (LGE)]

JCT3V-J0076 Cross-check on Simplification of an NBDV availability check (JCT3V-J0041) [T.Ikai (Sharp)] [late]

JCT3V-J0045 Alignment of motion derivation from other components [J. Nam, S. Yea (LGE)]

JCT3V-J0046 Modification of reference index for depth disparity derivation [J. Nam, S. Yea (LGE)]

JCT3V-J0086 Crosscheck on modification of reference index for depth disparity derivation (JCT3V-J0046) [X. Zhang, J. An (MediaTek)] [late]

## Other (VSP, SDC, DBBP, DLT, ARP etc.) (16)

(chaired by …)

JCT3V-J0026 VSP access improvement [T. Ikai, T. Tsukuba (Sharp)]

JCT3V-J0101 Crosscheck on VSP access improvement (JCT3V-J0026) [S. Shimizu (NTT)] [late]

JCT3V-J0029 Cleanup3: DLT table derivation [T. Ikai, T. Tsukuba (Sharp)]

JCT3V-J0070 Cross check of Cleanup3: DLT table derivation (JCT3V-J0029) [M. W. Park, C. Kim (Samsung)] [late]

JCT3V-J0030 Cleanup4: Remove DDD [T. Ikai, T. Tsukuba (Sharp)]

JCT3V-J0090 Crosscheck on cleanup4: remove DDD (JCT3V-J0030) [K. Zhang (MediaTek)] [late]

JCT3V-J0036 Reduction of Worst Case Memory Bandwidth in 3D-HEVC [M. W. Park, J. Y. Lee, C. Kim (Samsung)]

JCT3V-J0078 Cross-check on Reduction of Worst Case Memory Bandwidth in 3D-HEVC (JCT3V-J0036) [T.Ikai (Sharp)] [late]

JCT3V-J0037 ARP, IC and DBBP Flags Signaling for 3D-HEVC [M. W. Park, J. Y. Lee, B. Choi, C. Kim (Samsung)]

JCT3V-J0077 Cross-check on ARP, IC and DBBP Flags Signaling for 3D-HEVC (JCT3V-J0037) [T.Ikai (Sharp)] [late]

JCT3V-J0043 PU boundary deblocking restriction for DBBP blocks [S. Yoo, J. Nam, S. Yea (LGE)]

JCT3V-J0103 Crosscheck on PU boundary deblocking restriction for DBBP blocks (JCT3V-J0043) [S. Shimizu (NTT)] [late]

JCT3V-J0055 Motion buffer reduction for depth [G. Bang (ETRI), Y.S. Heo, W.W. Gwun, G.H.Park (KHU), G.S. Lee, N.H.Hur (ETRI)] [late]

JCT3V-J0083 Cross-check on Motion buffer reduction for depth (JCT3V-J0055) [T.Ikai (Sharp)] [late]

JCT3V-J0066 Simplification and improvement of sub-PU [X. Zheng, Y. Lin, X. Xu, J. Zheng (HiSilicon)]

JCT3V-J0089 Crosscheck on simplification and improvement of sub-PU (JCT3V-J0066) [J. An (MediaTek)] [late]

# Non-normative contributions (0)

## Encoder optimization (0)

(chaired by …)

## View synthesis (1)

JCT3V-J0064 CShift in HTM [T. Senoh, K. Wakunami, Y. Ichihashi, H. Sasaki, R. Oi (NICT)] [late]

# Withdrawn, unclear allocation (1)

JCT3V-J0079 Withdrawn

# Plenary Discussions and BoG Reports

## Project development

## BoGs

# Project planning

## General issues for CEs

A preliminary CE description is to be approved at the meeting at which the CE plan is established.

It is possible to define sub-experiments within particular CEs. As a general rule, it was agreed that each CE should be run under the same testing conditions using same software codebase, which should be based on either the ATM or HTM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for experiments were described in the output document JCT3V-F1100.

A deadline of two weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-3V reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the relevant software basis. Exceptions must be justified, discussed on the general JCT-3V reflector, and recorded in the abstract of the summary report.

Final CEs shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

Experiment descriptions should be written in a way such that it is understood as a JCT-3V output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-3V document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

Non-final CE plan documents were reviewed and given tentative approval during the meeting (in some cases with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE is described in an associated output document JCT3V-E110x for CE n, where “n" is the CE number (n = 1, 2, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-3V is not obliged to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-3V members can participate in a CE

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

### Cross check

For cross checking the same steps as described in the section above should be carried out as soon as possible, but should not take more than five working days. Moreover it should be checked if integrated tools correspond to the adopted proposal. Results of cross check should be announced to the reflector. The result-sheet should be made available to the group.

### Procedure if cross check fails or planned delivery data cannot be held

If a planned delivery date cannot be held this should be announced to the reflector.

If the crosscheck fails or the previous integrator has not delivered the software within 3 days or result sheet within 5 days after planned delivery date

- the current integrator should integrate in the last cross-checked version

- the previous integrator falls back to the end of the integration plan

## Common Conditions for 3D Video Coding Experiments

Preferred Common Conditions for experiment testing that are intended to be appropriate for both CEs and other experiments were selected by the group and described in output document JCT3V-G1100.

## Software development (update)

ATM software:

Due to the absence of key developers of this software from the meeting, the remaining bug fixing has to be done and planned within the mandates of the AHG and software coordinator / software specification editing team. It has already been expressed by email that this will be done.

HTM software:

The Integration plan had been developed in a BoG meeting chaired by G. Tech (07-08 18:00)

The plan was confirmed by the plenary 07-09

HTM-12.0 Integration plan (Starting date: July, 10th)

Track 1: Merge, DBBP, others

Coordinator: Hongbin Liu

| Item | Integrator | Days |
| --- | --- | --- |
| JCT3V-I0086 Simplification of shift DV candidate | Samsung | 1 |
| JCT3V-I0077 Partition Derivation for DBBP | Samsung | 1 |
| JCT3V-I0078 Memory Complexity for DBBP and VSP | Samsung | 2 |
| JCT3V-I0109 One dimensional DBBP boundary filtering | Sharp | 2 |
| ?? JCT3V-I0093 Depth coding compatible with arbitrary bit-depth in 3D-HEVC | Mediatek | 1 |

Track 2: ARP, HLS, IC, others

Coordinator: Gerhard Tech

| Item | Integrator | Days |
| --- | --- | --- |
| JCT3V-I0051 CE1: Simplification of advanced residual prediction | Qualcomm | 2 |
| JCT3V-I0129 AHG4: Editorial improvement for ARP specification text in 3D-HEVC | Qualcomm | 1 |
| JCT3V-I0104 CE1: Results of ARP simplification | Sharp | 2 |
| JCT3V-I0099 On the video parameter set extension 2 in 3D-HEVC | Mediatek | 2 |
| JCT3V-I0072 Problem fix for MV scaling in inter-view ARP | Mediatek | 2 |
| JCT3V-I0090 3D HEVC HLS Cleanup | Samsung | 1 |
| JCT3V-I0100 3D-HEVC HLS: General comments on 3D-HEVC high level syntax | Samsung | 1 |
| JCT3V-I0080 Parameter Derivation for Illumination Compensation | Samsung | 2 |

Track 3: DMM, SDC, others

Coordinator: Yi-Wen Chen

| Item | Integrator | Days |
| --- | --- | --- |
| JCT3V-I0095 CE2: Single depth intra mode for 3D-HEVC | Mediatek | 3 |
| JCT3V-I0123 Fast Intra SDC coding for 3D-HEVC Intra Coding | HiSilicon | 2 |
| JCT3V-I0120 CE2: Remove “depth_dc_flag” signalling in DMM cases | HiSilicon | 2 |
| JCT3V-I0110 Lookup table size reduction in DMM1 | Sharp | 2 |
| JCT3V-I0057 Clipping Operation for DLT Indexes | RWTH | 1 |
| JCT3V-I0076 Simplification of Threshold Derivation for DBBP and DMM4 | Samsung | 1 |
| JCT3V-I0084 CE2: Fast SDC DC offset decision | Samsung | 2 |
| JCT3V-I0085 CE2: Separate enabling flag for intra coding tools | Samsung | 2 |
| JCT3V-I0066 CE2: Simplification of DMM4 | HHI | 1 |

HTM-12.1 Integration plan

Coordinator: Gerhard Tech

| Item | Integrator | Days |
| --- | --- | --- |
| JCT3V-I0116 AHG5: Coding results for flexible coding order (FCO) | LGE | 3 |
| MV-HEVC 9 HLS |  |  |

It is to the discretion of the software coordinators to set up a time line for the integration and request proponents to finish integration by a given date.

Integration Procedure & Guidelines

Integration is done in a serial way. Each integrator cross-checks the version provided by his predecessor.  The cross check for the last version is carried out by the software coordinators.

Integration Guidelines

When integrating

- software changes should be enclosed by macros switchable by defines including company and proposal number e.g.

#define MYCOMPANYS_DEPTHFILTER_JCT3V_B0555  1

#if MYCOMPANYS_DEPTHFILTER_JCT3V_B0555

// do stuff

#endif

- new tools should be made switchable in the cfg-file if reasonable

- cfg-files should be updated

Delivery of software

Before delivering the software to the next integrator it should be checked if

- the software compiles under windows and linux

- software compiles and delivers same results as previous version when integrated tools are disabled by macro or cfg-settings

- there are encoder-decoder mismatches

- there are memory leaks by measuring maximum memory consumption (or specific tools e.g. valgrind) is

- visual quality is not disturbed

Additional to the software cfg-files that reflect proposed settings and an excel sheet with coding results should be provided. Software and cfg-files should be delivered by checking it in to the corresponding (HTM or ATM) software repositories.

When software is delivered this should be announced to the reflector. Moreover, every further change on the software should be announced. If there is a delay in integration this should be communicated to the reflector.

## Software repositories

### HTM

HTM software can be checked in

https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/

Therefore for each integrator an own software branch will be created by the software coordinator containing the current anchor version or the version of the previous integrator:

e.g.  branches/0.7-mycompany

The branch of the last integrator will become the new release candidate tag.

e.g. tags/0.8rc1

This tag can be cross-check by the group for. If no problems occur the release candidate will become the new tag after 7-days:

e.g. tags/0.8

If reasonable intermediate release candidate tags can be created by the software coordinator.

### ATM

An official release of the 3DV-ATM software can be checked out by mpeg3dv SVN users from the following location:

http://mpeg3dv.research.nokia.com/svn/mpeg3dv/trunk/

Following every MPEG 3DV meeting, software coordinator creates a new branch for the integration of adopted proposals:

http://mpeg3dv.research.nokia.com/svn/mpeg3dv/branches/

(Note: Would be desirable to rename “mpeg3dv” to “jct3v”)

Software integrator checks-out the software from integration branch at its turn of integration plan and integrates proposal as it is specified in Section 11.3.

Software integrator communicates a new software version over the email list to the cross-checker and to the software coordinator.

The software coordinator checks-in to the integration branch every new software integration with confirmed cross-check, as it is specified in Section 11.3.

Once the integration plan is completed, the software coordinator cross-check version available in the integration branch and checks-in a new official release of the 3DV-ATM to the http://mpeg3dv.research.nokia.com/svn/mpeg3dv/trunk/.

## List of CEs

(chaired by JRO, 07-08 morning)

The following CE was planned:

- CE1 on Segmental Prediction [J. L. Lin]

  - Segmental prediction in Inter SDC (I0075)

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-3V reflector (jct-3v@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-3V project management (AHG1) (jct-3v@lists.rwth-aachen.de) - Coordinate overall JCT-3V interim efforts. - Report on project status to JCT-3V reflector. - Provide report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| MV-HEVC / 3D-HEVC Draft and Test Model editing (AHG2) (jct-3v@lists.rwth-aachen.de) - Produce and finalize JCT3V-I1001 3D-HEVC Draft Text 5. - Produce and finalize JCT3V-I1002 MV-HEVC Draft Text 9. - Produce and finalize JCT3V-I1003 3D-HEVC and MV-HEVC Test Model 9. - Gather and address comments for refinement of these documents. - Coordinate with the 3D-HEVC Software Integration AhG to address issues relating to mismatches between software and text. - Coordinate with AHG6 w.r.t. text improvements in 3D-HEVC specific HLS. | G. Tech, K. Wegner (co-chairs), J. Boyce, Y. Chen, M. Hannuksela, T. Suzuki, S. Yea, J.-R. Ohm, G. Sullivan (vice chairs) | N |
| 3D-AVC Software Integration (AHG3) (jct-3v@lists.rwth-aachen.de) - Coordinate the finalization of the 3DV-ATM software and its distribution to JCT-3V members. - Identify issues (if any) where the software JCT3V-I1005 deviates from the standard or the test model JCT3V-G1003, and clean up the software if necessary. - Produce documentation of software usage for distribution with the software. - Prepare and deliver 3DV-ATM v13.0 software to address all known bugs (expected by 18 July 2014). Prepare and deliver v13.1 before the next meeting in case tht further bugs become known. | D. Rusanovskyy (chair), F. C. Chen, J. Y. Lee, J.-L. Lin, O. Stankiewicz, T. Suzuki, D. Tian (vice chairs) | N |
| MV-HEVC / 3D-HEVC Software Integration (AHG4) (jct-3v@lists.rwth-aachen.de) - Coordinate development of the HTM software and its distribution to JCT-3V members. - Produce documentation of software usage for distribution with the software. - Prepare and deliver HTM-12.0 software version and the reference configuration encodings according to JCT3V-G1100 based on common conditions suitable for in the core experiment (expected within four weeks after the meeting). - Prepare and deliver HTM-12.1 software that include additional items not integrated into the 12.0 version. - Prepare and deliver the WD text of MV-HEVC software JCT3V-I1009. - Perform analysis and reconfirmation checks of the behaviour of technical changes adopted into the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with MV-HEVC Draft and 3D-HEVC Test Model editing AhG to identify any mismatches between software and text. | G. Tech, H. Liu (co-chairs), Y. W. Chen, K. Wegner (vice chairs) | N |
| 3D Coding Verification Testing (AHG5) (jct-3v@lists.rwth-aachen.de) - Prepare a set of test sequences and encodings to be used in upcoming MV-HEVC verification testing. - Identify test sites that could conduct such tests. - Suggest Improvements to the draft test plan JCT-3V-I1011. - Prepare viewing logistics for 10th JCT-3V meeting. | V. Baroncini, K. Müller, S. Shimizu, A. Vetro, S. Yea (co-chairs) | N |
| 3D High level syntax (AHG6) (jct-3v@lists.rwth-aachen.de) - Study video parameter set, sequence parameter set, picture parameter set, and slice header syntax designs for the 3D-HEVC extension. - Study SEI messages and VUI syntax designs needed for the 3D-HEVC extension. - Study and compare mechanisms for enabling diagonal inter-layer prediction. - Study the operation of the Hypothetical Reference Decoder the 3D-HEVC extension. - Assist in software development and text drafting for the high-level syntax in the HEVC extensions designs. | Y. Chen, T. Ikai (co-chairs) | N |
| Complexity assessment (AHG7) (jct-3v@lists.rwth-aachen.de) - Study intrinsic complexity measures or metrics to assess hardware and software implementations of algorithms/computational models based on number of operations, potential for parallelism, data transfer rate, and data storage requirements. - Analyse existing designs and produce complexity perspective per tool, from the viewpoint of possible slowdown, bottlenecks or implementation issues. - Develop a recommendation to JCT-3V on formalized complexity assessment in a reporting document. | C. Lee (chair), G. Bang, T. Ikai, H. Liu (vice chairs) | N |
| 3D Test Material (AHG8) (jct-3v@lists.rwth-aachen.de) - Identify deficiencies and limitations of current 3D video test material. - Identify, collect, and make available a variety of additional 3D video sequences. - Study the characteristics of test materials and their impact on coding performance, synthesis quality etc. - Recommend appropriate test materials for use in 3D Video Coding Extension Development. - Assess the suitability of the test material that was offered to the 5th and 6th JCT-3V meetings. - Assess additional stereo material for an upcoming MV-HEVC verification | S. Shimizu, S. Yea (co-chairs) | N |
| AVC Conformance testing development (AHG9) (jct-3v@lists.rwth-aachen.de) - Further discuss and improve the conformance drafts related to 3D-AVC (JCT3V-I1004). - Collect the conformance test streams. | T. Suzuki (chair), D. Rusanovskyy, D. Tian, Y. W. Chen (vice chairs) | N |
| HEVC Conformance testing development (AHG 10) (jct-3v@lists.rwth-aachen.de) - Further discuss and improve the conformance drafts related to MV-HEVC (JCT3V-I1008) and 3D-HEVC (JCT3V-I1010). - Collect the conformance test streams. | Y. Chen, T. Ikai, S. Shimizu, T. Suzuki (co-chairs) | N |

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate those responsible for document production.

(it was communicated by the chair that only the subsequent lists of adoptions is relevant, and in case of contradiction with any of meeting notes above the latter are considered invalid in first place)

JCT3V-I1000 Meeting Report of 9th JCT-3V Meeting [J.-R. Ohm, G. J. Sullivan] (before next meeting)

(Note: Initial versions of the subsequent draft documents should be uploaded by the end of the meeting, continually updating to be performed until the final the version is released)

JCT3V-I1001 3D-HEVC Draft Text 5 [G. Tech, K. Wegner, Y. Chen, S.Yea] (Text of ISO/IEC 23008-2 DAM4, WG 11 N14701) [2014-08-20]

Summary of Adoptions for 3D-HEVC

- …

The list above which had been circulated via the email was confirmed in the plenary 07-09.

JCT3V-I1002 MV-HEVC Draft Text 9 [G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce] (Integrated to ISO/IEC 23008-2:201X, WG 11 N14700) [2014-08-15]

(void / corrigenda?)

JCT3V-I1003 Test Model 9 of 3D-HEVC and MV-HEVC [Y. Chen, G. Tech, K. Wegner, S. Yea] (WG 11 N14704) [2014-08-20]

Bug fixes:

- …

Software:

- …

JCT3V-H1004 Draft 4 of 3D-AVC Conformance

Remains valid

JCT3V-I1005 Draft 4 of 3D-AVC Reference Software [D. Rusanovskyy, D. Tian] (Study of ISO/IEC 14496-5:2001 DAM35, WG 11 N14673) [2014-08-31]

JCT3V-I1006 Draft 5 of MVC+D Reference Software [D. Rusanovskyy, T. Suzuki, D. Tian] (ISO/IEC 14496-5:2001 FDAM33, WG 11 N14671) [2014-08-31]

JCT3V-I1007 Draft 1 of MFC plus Depth and Texture/Depth View Packing SEI Message [P. Yin, T. Senoh, M. Hannuksela, Y. Chen, J.-R. Ohm, G. J. Sullivan] (ISO/IEC 14496-10:201X PDAM1, WG11 N14677) [2014-08-01]

JCT3V-I1008 MV-HEVC Conformance Draft 1 [Y. Chen, S. Shimizu, T. Suzuki] (WG11 N14714) [14-07-25]

(not containing streams yet)

JCT3V-I1009 MV-HEVC Software Draft 1 [G. Tech] (WG11 N14710) [14-10-10]

JCT3V-I1010 3D-HEVC Conformance Draft 1 [Y. Chen, T. Ikai, T. Suzuki] (WG11 N14715) [14-07-25]

(not containing streams yet)

JCT3V-I1011 MV-HEVC Draft Verification Test Plan v1 [S. Shimizu] (WG11 N14707) [14-07-10]

- Coded results with new Telekom and the sequences that were used in MVC+D testing prepared for next meeting

- Investigation of suitability, selection of test cases by next meeting

- Identify test labs and time schedule for performing the tests by next meeting

- Availability of results: 15-02 or 15-06

JCT3V-G1100 Common Test Conditions of 3DV Core Experiments remains valid

JCT3V-I1101 Description of Core Experiment 1 (CE1) on Segmental Prediction [J.-L. Lin]

# Future meeting plans, expressions of thanks, and closing of the meeting

The document upload deadline for the 10th meeting of the JCT-3V will be Oct 10, 2014, 2359 MET (Geneva/Paris time zone). Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Wednesday or Thursday or Friday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting);

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Saturday prior to such meetings and closing it on the last day of the WG 11 meeting);

- In cases where JCT-3V meets during the first week of the SG16 meeting under ITU-T auspices, and co-located with an MPEG meeting at a nearby meeting place, the meeting dates could also be approximately aligned with the MPEG meeting.

Some specific future meeting plans were established as follows:

- 12-18 Feb. 2015 under ITU-T auspices in Geneva, CH

- 20-26 June 2015 under WG 11 auspices in Warsaw, PL

- 15-21 Oct. 2015 under ITU-T auspices in Geneva, CH

- …

Note: Depending on expectations about future work load, shorter meeting durations may be considered starting from the 11th meeting.

The Ministry of International Affairs and Communications of Japan was thanked for the excellent hosting of the 9th meeting of the JCT-3V under the auspices of ITU-T SG 16.

NICT was thanked for providing equipment used in viewing 3D video material.

It was also reminded that final output documents (if also registered under a WG11 output doc number) have to be uploaded separately with a WG11 header.  To do this in a timely fashion is particularly important for standards submitted for ballot.

It was also reminded that slide presentation decks should be made available.

The JCT-3V meeting was closed at approximately XXXX hours on Friday 24 Oct. 2014.

# Annex A to JCT-3V report:
List of documents

# Annex B to JCT-3V report:
List of meeting participants

The participants of the ninth meeting of the JCT-3V, according to the online registration (approximately XXX in total), were as follows:

- …