| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0106 |
| --- | --- |

| Title: | Initial text for the specification of profiles, tiers, and levels in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gerhard Tech Karsten Müller | Email: | gerhard.tech@hhi.fraunhofer.de karsten.mueller@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

Initial text for the specification of profiles, tiers and levels in 3D-HEVC is provided, which can be further discussed and refined at the meeting.

# Introduction

The current 3D-HEVC Draft 9 does not include text defining profiles, tiers and levels. To fill this gap an initial specification is provided in this document, which can be the basis for further discussion. The profile specification is based on the Multiview Main profile and has been modified to cover 3D-HEVC related aspects.

# Proposed specification of profiles, tiers and levels

Differences to the Multiview Main profile are indicated in red. Comments on the changed items are highlighted yellow.

  - Profiles, tiers, and levels

    - Profiles

      - Multiview 3D Main profile

For a layer in an output operation point associated with an OLS in a bitstream, the layer being conforming to the Multiview 3D Main profile, the following applies:

–	Let olsIdx be the OLS index of the OLS.

–	The sub-bitstream subBitstream is derived as follows:

–	If OlsIdxToLsIdx[ olsIdx ] is less than or equal to vps_num_layer_sets_minus1, subBitstream is derived by invoking the sub-bitstream extraction process as specified in clause F.10.1 with the following inputs: the bitstream, tIdTarget equal to OpTid of the output operation point, and layerIdListTarget containing the nuh_layer_id value layerId of the layer and all the reference layers of the layer.

–	Otherwise, subBitstream is derived by invoking the sub-bitstream extraction process as specified in clause F.10.3 with tIdTarget equal to OpTid of the output operation point and with layerIdListTarget containing the nuh_layer_id value of the layer and all the reference layers of the layer.

–	The base layer sub-bitstream baseBitstream is derived as follows:

–	If VCL NAL units with nuh_layer_id equal to 0 are included in subBitstream, baseBitstream is derived by invoking the sub-bitstream extraction process as specified in clause F.10.1 with the subBitstream, tIdTarget equal to OpTid of the output operation point, and layerIdListTarget containing only one nuh_layer_id value that is equal to 0 as inputs.

–	Otherwise, baseBitstream is derived by invoking the independent non-base layer rewriting process as specified in clause F.10.2 with subBitstream, tIdTarget equal to OpTid of the output operation point, and layerIdListTarget containing only the smallest nuh_layer_id value of the VCL NAL units of subBitstream as inputs.

The base layer sub-bitstream baseBitstream shall obey the following constraints:

- The base layer sub-bitstream baseBitstream shall be indicated to conform to the Main profile.

The sub-bitstream subBitstream shall obey the following constraints:

- All active VPSs shall have vps_num_rep_formats_minus1 in the range of 0 to 15, inclusive.

- All active SPSs for layers in subBitstream shall have chroma_format_idc equal to 1 only.

- All active SPSs for a layer with nuh_layer_id equal to i and VpsDepthFlag[ i ] equal to 0 in subBitstream shall have chroma_format_idc equal to 1 only.

- All active SPSs for a layer with nuh_layer_id equal to i and VpsDepthFlag[ i ] equal to 1 in subBitstream shall have chroma_format_idc equal to 0 or 1 only.

[Remark: Additionally to above changes following needs to be added to semantics of VpsDepthFlag (corresponding to specification for depth auxiliary pictures):

It is a requirement of bitstream conformance that when VpsDepthFlag[ lId ] is equal to 1, either of the following applies:

–	chroma_format_idc is equal to 0 in the active SPS for the layer with nuh_layer_id equal to lId.

–	The value of all decoded chroma samples is equal to 1  <<  ( BitDepthC − 1 ) in all pictures that have nuh_layer_id equal to lId and for which this VPS RBSP is the active VPS RBSP.

]

[Remark: Alternatively, only one of the two options could be chosen.]

- All active SPSs for layers in subBitstream shall have transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, implicit_rdpcm_enabled_flag, explicit_rdpcm_enabled_flag, extended_precision_processing_flag, intra_smoothing_disabled_flag, high_precision_offsets_enabled_flag, persistent_rice_adaptation_enabled_flag, and cabac_bypass_alignment_enabled_flag, when present, equal to 0 only.

- CtbLog2SizeY derived from all active SPSs for layers in subBitstream shall be in the range of 4 to 6, inclusive.

- All active PPSs for layers in subBitstream shall have log2_max_transform_skip_block_size_minus2 and chroma_qp_offset_list_enabled_flag, when present, equal to 0 only.

- ScalabilityId[ j ][ smIdx ] derived according to any active VPS shall be equal to 0 for any smIdx value not equal to 1 or 3  0 or 1 and for any value of j such that layer_id_in_nuh[ j ] is among layerIdListTarget that was used to derive subBitstream.

[Remark: Above change allows depth maps to be present using ScalabilityId[ j ][ 0 ] greater than 0. Furthermore, auxiliary pictures are not allowed to be in sub-bitstreams confirming to 3D Main profile. (However, they could be included in other sub streams (or respectively output layer sets) not confirming to the 3D Main profile). Another option would be to allow auxiliary pictures in the sub-bitstream, e.g. alpha data. This would raise further question related to e.g. coding order, allowed 3D-HEVC coding tools, handling of depth auxiliary pictures.]

- When NumLayersInIdList[ OlsIdxToLsIdx[ olsIdx ] ] is equal to 2, output_layer_flag[ olsIdx ][ j ] derived according to any active VPS shall be equal to 1 for j in the range of 0 to 1, inclusive, for subBitstream.

[Remark: Above constraint was introduced to always output both layers for the stereo case. Since applications for 3D-HEVC might require output of only one view, even when two views are provided in an OLS this constraint can be removed.]

- All active VPSs shall have alt_output_layer_flag[ olsIdx ] equal to 0 only.

- When ViewOrderIdx[ i ] derived according to any active VPS is equal to 1 for the layer with nuh_layer_id equal to i in subBitstream, inter_view_mv_vert_constraint_flag shall be equal to 1 in the sps_multilayer_extension( ) syntax structure in each active SPS for that layer.

[Remark: Above constraint was introduced to restrict vertical components of motion vectors in the stereo case, for 3D-HEVC it might be removed.]

- When ViewOrderIdx[ i ] or VpsDepthFlag[ i ] derived according to any active VPS is greater than to 0 for the layer with nuh_layer_id equal to i in subBitstream, num_ref_loc_offsets shall be equal to 0 in each active PPS for that layer.

- When ViewOrderIdx[ i ] or VpsDepthFlag[ i ] derived according to any active VPS is greater than 0 for the layer with nuh_layer_id equal to i in subBitstream, the values of pic_width_in_luma_samples and pic_height_in_luma_samples in each active SPS for that layer shall be equal to the values of pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, in each active SPS for all reference layers of that layer.

[Remark: For 3D-HEVC the two constraints above are extended to depth maps.]

- For a layer with nuh_layer_id iNuhLId equal to any value included in layerIdListTarget that was used to derive subBitstream, the value of NumRefLayers[ iNuhLId ], which specifies the total number of direct and indirect dependent layers and is derived as specified in F.7.4.3.1, shall be less than or equal to 4.

[Remark: We might consider increasing the above number of allowed direct and indirect reference layers for 3D-HEVC.]

- All active SPSs for layers in subBitstream shall have sps_range_extension_flag equal to 0 only and sps_extension_6bits equal to 0 only.

- All active PPSs for layers in subBitstream shall have pps_range_extension_flag equal to 0 only and pps_extension_6bits equal to 0 only.

- All active SPSs for layers in subBitstream shall have bit_depth_luma_minus8 equal to 0 only.

- All active SPSs for layers in subBitstream shall have bit_depth_chroma_minus8 equal to 0 only.

- All active PPSs for layers in subBitstream shall have colour_mapping_enabled_flag equal to 0 only.

- When an active PPS for any layer in subBitstream has tiles_enabled_flag equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

- When an active PPS for any layer in subBitstream has tiles_enabled_flag equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

- The number of times read_bits( 1 ) is called in clauses 9.3.4.3.3 and 9.3.4.3.4 when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 5 * RawCtuBits / 3.

- general_level_idc and sub_layer_level_idc[ i ] for all values of i in active SPSs for any layer in subBitstream shall not be equal to 255 (which indicates level 8.5).

- The level constraints specified for the Multiview 3D Main profile in clause G.11.2I.11.2 shall be fulfilled.

- For any active VPS, ViewOrderIdx[ i ] shall be greater than ViewOrderIdx[ j ] for any values of i and j among layerIdListTarget that was used to derive subBitstream such that AuxId[ i ] is equal to AuxId[ j ] and i is greater than j.

[Remark: Above requirement can be removed, when auxiliary pictures are disallowed in the 3D Main profile.]

- For any active VPS, ViewOrderIdx[ i ] shall be greater than ViewOrderIdx[ j ] for any values of i and j among layerIdListTarget that was used to derive subBitstream such that VpsDepthFlag[ i ] is equal to VpsDepthFlag[ j ] and i is greater than j.

- For any active VPS, VpsDepthFlag[ i ] shall be greater than VpsDepthFlag[ j ] for any values of i and j among layerIdListTarget that was used to derive subBitstream such that ViewOrderIdx[ i ] is equal to ViewOrderIdx[ j ] and i is greater than j.

[Remark: Above two requirements specify the coding order we adopted for 3D-HEVC.]

In the remainder of this clause and clause I.11.2.1G.11.2.1, all syntax elements in the profile_tier_level( ) syntax structure refer to those in the profile_tier_level( ) syntax structure associated with the layer.

Conformance of a layer in an output operation point associated with an OLS in a bitstream to the Multiview 3D Main profile is indicated as follows:

- If OpTid of the output operation point is equal to vps_max_sub_layer_minus1, the conformance is indicated by general_profile_idc being equal to 6 8 or general_profile_compatibility_flag[ 6 8 ] being equal to 1, and general_max_12bit_constraint_flag being equal to 1, general_max_10bit_constraint_flag being equal to 1, general_max_8bit_constraint_flag being equal to 1, general_max_422chroma_constraint_flag being equal to 1, general_max_420chroma_constraint_flag being equal to 1, general_max_monochrome_constraint_flag being equal to 0, general_intra_constraint_flag being equal to 0, and general_one_picture_only_constraint_flag being equal to 0, and general_lower_bit_rate_constraint_flag being equal to 1.

- Otherwise (OpTid of the output operation point is less than vps_max_sub_layer_minus1), the conformance is indicated by sub_layer_profile_idc[ OpTid ] being equal to 6 8 or sub_layer_profile_compatibility_flag[ OpTid ][ 6 8 ] being equal to 1, and sub_layer_max_12bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_10bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_8bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_422chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_420chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_monochrome_constraint_flag[ OpTid ] being equal to 0, sub_layer_intra_constraint_flag[ OpTid ] being equal to 0, and sub_layer_one_picture_only_constraint_flag[ OpTid ] being equal to 0, and sub_layer_lower_bit_rate_constraint_flag[ OpTid ] being equal to 1.

[Remark.: Here *_max_monochrome_constraint_flag might be changed to 1 for depth layers, when chroma_format_idc is required to be equal to 0.]

    - Tiers and levels

The specification in sub-clause G.11.2 and its sub-clauses apply, with the following modifications:

- "Multiview Main profile" is replaced by "3D Main profile"

    - Decoder capabilities

When a decoder conforms to any profile specified in Annex GI, it shall also have the INBLD capability specified in clause F.11.1.

Clause F.11.2 specifies requirements for a decoder conforming to any profile specified in Annex GI.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).