- I.7.3.8.5.2 Coding unit extension syntax

| cu_extension( x0 , y0 , log2CbSize ) { | Descriptor |
| --- | --- |
| … |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( k = 0; k < nCbS; k = k + pbOffset ) |  |
| if( DmmFlag[ x0 + k ][ y0 + j ] ¦ ¦ sdc_flag[ x0 ][ y0 ] ) { |  |
| if(sdc_flag[ x0 ][ y0 ] && !DmmFlag[ x0 ][ y0 ]){ |  |
| seg_ pred_flag[ x0 ][ y0 ] | ae(v) |
| dcNumSeg = nSegNum[ x0 ][ y0 ] |  |
| } |  |
| if(( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && sdc_flag[ x0 ][ y0 ] ) ¦¦ seg_pred_flag[ x0 ][ y0 ]) |  |
| depth_dc_flag[ x0 + k ][ y0 + j ] | ae(v) |
| if(!seg_pred_flag[ x0 ][ y0 ]) |  |
| dcNumSeg = DmmFlag[ x0 + k ][ y0 + j ] ? 2 : 1 |  |
| … |  |
| } |  |

- I.7.4.9.5.2 Coding unit extension semantics

…

seg_pred_flag[ x0 ][ y0 ] equal to 1 specifies that multiple segmental prediction is applied. seg_pred_flag[ x0 ][ y0 ] equal to 0 specifies that multiple segmental prediction is not applied. When not present, seg_pred_flag[ x0 ][ y0 ] is inferred to be equal to 0.

A variable nSegNum[ x0 ][ y0 ] is set equal to seg_pred_flag [ x0 ][ y0 ] + 1.

…

depth_dc_abs[ x0][ y0 ][ i ], depth_dc_sign_flag[ x0 ][ y0 ][ i ] are used to derive DcOffset[ x0 ][ y0 ][ i ]. When not present, the values of depth_dc_abs[ x0][ y0 ][ i ] and depth_dc_sign_flag[ x0 ][ y0 ][ i ] are inferred to be equal to 0.

If seg_pred_flag[ x0 ][ y0 ] is equal to 0, the variable dcOffsetMin is set equal to 2 – dcNumSeg;

- Otherwise (seg_pred_flag[ x0 ][ y0 ] is equal to 1), dcOffsetMin is derived as follows

If i < dcNumSeg  - 1, DcOffsetMin = 0;

Otherwise ( i == dcNumSeg - 1 ), dcOffsetMin = (DcOffset[ x0 ][ y0 ][ 0 ]== 0 ? 1 : 0 ).

The variable DcOffset[ x0 ][ y0 ][ i ] is derived as specified in the following:

- DcOffset[ x0 ][ y0 ][ i ]  = 
( 1 − 2 *depth_dc_sign_flag[ x0 ][ y0 ][ i ] ) * ( depth_dc_abs[ x0 ][ y0 ][ i ] + dcOffsetMin − dcNumSeg +2 )	(I-37)

- I.8.4.4.1 General decoding process for intra blocks

- …

- 5. The general intra sample prediction process as specified in subclause I.8.4.4.2.1 is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the intra prediction mode predModeIntra, the transform block size nTbS, and the variable cIdx as inputs, and the output is an (nTbS)x(nTbS) array predSamples.

- 6. When sdc_flag is equal to 1 and seg_pred_flag[ xCb ][ yCb ] is equal to 1, the derivation process for segmental prediction as specified in subclause I.8.8.2 is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the transform block size nTbS, and predSamples as inputs, and the output is the modified predSamples.

- …

- I.8.5.3.3.5 Full sample interpolation process

- …

The prediction luma sample value predSampleLXL[ xL ][ yL ] is derived as specified in the following:

- predSampleLXL[ xL ][ yL ] = refPicLXL[ xIntL ][ yIntL ]	                                                                 (I-190)

When sdc_flag is equal to 1 and seg_pred_flag[ xCb ][ yCb ] is equal to 1, the derivation process for segmental prediction as specified in subclause I.8.8.2 is invoked with the block location ( xCb, yCb ), the block size nPbW, and predSampleLXL as inputs, and the output is the modified predSampleLXL.

…

- I.8.5.4.1 General

…

Otherwise (sdc_flag is equal to 1), if seg_pred_flag [ xCb ][ yCb ] is equal to 0, for x in the range of 0 to nCbSL − 1 and y in the range of 0 to nCbSL − 1, resSamplesL[ x ][ y ] is set equal to DcOffset[ xCb ][ yCb ][ 0 ]. Otherwise (seg_pred_flag [ xCb ][ yCb ] is equal to 1 ), for x in the range of 0 to nCbSL − 1 and y in the range of 0 to nCbSL − 1, resSamplesL[ x ][ y ] is set equal to 0.

…

- I.8.8.2	Derivation process for segmental prediction

This process is only invoked when seg_pred_flag [ xCb ][ yCb ] is equal to 1.

Inputs to this process are:

a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

a variable nTbS specifying the block size,

a reference sample array refSamples[ x ][ y ], with x, y =0..nTbS − 1.

Output of this process is:

the modified reference sample array refSamples[ x ][ y ], with x, y =0..nTbS − 1.

- A variable predAvg is derived as

- predAvg = refSamples[ 0 ][ 0 ]+ refSamples[ 0 ][ nTbS-1 ] + refSamples[ nTbS-1 ][ 0 ]+ refSamples[ nTbS-1 ][ nTbS-1 ];

predAvg = ( predAvg + 2) >>2.

- A variable segIdx[ x ][ y ] is set equal to 0 for  x = 0 .. nTbS-1, y = 0 .. nTbS -1.

- The following applies for x = 0.. nTbS -1, y = 0 .. nTbS-1:

If refSamples[ x ][ y ] < predAvg, segIdx[ x ][ y ] is set equal to 0;

Otherwise ( refSamples[ x ][ y ] >= predAvg), segIdx[ x ][ y ] is set equal to 1.

- A variable sampleCount[ j ][ k ] is set equal to 1  <<  ( BitDepthY – 1 ) for k = 0 .. (1  <<  BitDepthY ) -1, and j = 0 .. nSegNum[ xCb ][ yCb ] - 1.

- A variable mostCount[ j ] is set equal to 0 for j from 0 to nSegNum[ xCb ][ yCb ] – 1.

- A variable segPred[ j ] is set equal to 0 for j from 0 to nSegNum[ xCb ][ yCb ] – 1.

- For y in the range of 0 to nTbS-1, inclusive, the following applies:

For x in the range of 0 to nTbS-1, inclusive, the following applies:

j = segIdx[ x ][ y ]; sampleCount[ j ][ refSamples [ x ][ y ] ]++.

mostCount[ j ] = Max(mostCount[ j ], sampleCount[ j ][ refSamples [ x ][ y ]]).

- For a variable k in the range 0 to nSegNum[ xCb ][ yCb ] – 1, the following applies,

- A variable recSamp[ k ] is derived as recSamp[ k ] = DltFlag[ nuh_layer_id ] ? Idx2DepthValue[Clip1Y( DepthValue2Idx( segPred[ k ]) + DcOffset[ xCb ][ yCb ][ k ])] : Clip1Y( segPred[ k ] + DcOffset[ xCb ][ yCb ][ k ]).

- For y in the range of 0 to nTbS-1, inclusive, the following applies:

For x in the range of 0 to nTbS-1, inclusive, the following applies:

refSamples[ x ][ y ] is set equal to recSamp[ segIdx[ x ][ y ]].

Table I-12 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
|  | ... |  |  |  |  |
|  | dim_not_present_flag | Table I-20 | 0 | 1 | 2 |
|  | seg_pred_flag | Table I-21’ | 0 | 1 | 2 |
|  | … |  |  |  |  |

Table I-21’ – Values of initValue for seg_pred_flag ctxIdx

| Initialization variable | ctxIdx of seg_pred_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table I-21 – Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| cu_extension( ) | … | … | … |
|  | sdc_flag | FL | cMax = 1 |
|  | seg_pred_flag | FL | cMax = 1 |
|  | … |  |  |

Table I-23 –Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| ... | .. | .. | … | … | … | … |
| sdc_flag | 0 | na | na | na | na | na |
| seg_pred_flag | 0 | na | na | na | na | na |
| … |  |  |  |  |  |  |