| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1016r2_attachement |
| --- | --- |

| Title: | APS syntax and semantics, and parameter set extension syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Attachment to APS BoG report |  |  |
| Author(s) or Contact(s): | Stephan Wenger Ye-Kui Wang Teruhiko Suzuki | Email: | stewe@stewe.org yekuiw@qualcomm.com teruhikos@jp.sony.com |
| Source: | APS BoG |  |  |

_____________________________

# Abstract

This document is an attachment to the APS BoG report, and includes the following changes agreed related to APS during this JCT-VC meeting:

- Syntax and semantics for APS, including the APS extensibility changes

- SPS and PPS extensibility changes

Note that syntax and semantics for quantization matrices at SPS level are also included in this document. However, the purpose of this document is for documentation of the agreed changes listed above.

# Syntax

## Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| [Ed. (BB): x y padding syntax missing here, present in HM software ] |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| [Ed. (BB): chroma bit depth present in HM software but not used further ] |  |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| log2_max_frame_num_minus4 | ue(v) |
| pic_order_cnt_type | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |
| delta_pic_order_always_zero_flag | u(1) |
| offset_for_non_ref_pic | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |
| offset_for_ref_frame[ i ] | se(v) |
| } |  |
| max_num_ref_frames | ue(v) |
| gaps_in_frame_num_value_allowed_flag | u(1) |
| [Ed. (BB): not present in HM software, depends on reference list construction decision ] |  |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| chroma_pred_from_luma_enabled_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| scaling_list_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| pcm_loop_filter_disable_flag | u(1) |
| temporal_id_nesting_flag | u(1) |
| inter_4x4_enabled_flag | u(1) |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| [Ed. (BB): not present in HM software, depends on reference list construction decision ] |  |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| [Ed. (BB): not present in HM software, signaled in slice header as absolute value slice_qp, should be implemented to be used for slice_qp_delta] |  |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| max_cu_qp_delta_depth | ue(v) |
| [Ed. (BB): not present in HM software, should be implemented ] |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Adaptation parameter set RBSP syntax

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_scaling_list_data_present_flag | u(1) |
| aps_sample_adaptive_offset_flag | u(1) |
| aps_adaptive_loop_filter_flag | u(1) |
| if( aps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| if( aps_sample_adaptive_offset_flag ) |  |
| sao_param( ) |  |
| if( aps_adaptive_loop_filter_flag ) |  |
| alf_param( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

### Scaling list data syntax

| scaling_list_param( ) { | Descriptor |
| --- | --- |
| use_default_scaling_list_flag | u(1) |
| if( !use_default_scaling_list_flag ) |  |
| for( SizeID = 0; SizeID < 4; SizeID++ ) |  |
| for( MatrixID = 0; MatrixID < (SizeID = = 3) ? 2:6; MatrixID++ ) { |  |
| pred_mode_flag | u(1) |
| if( !pred_mode_flag ) |  |
| pred_matrix_id_delta | ue(v) |
| else |  |
| scaling_list( QuantMatrix[ SizeID ][ MatrixID ], ( 1 << ( 4 + ( SizeID << 1) ) ) ) |  |
| } |  |
| } |  |

### Scaling list syntax

| scaling_list( ScalingList, coefNum ) { | Descriptor |
| --- | --- |
| nextcoef = 8 |  |
| for( i=0; i<coefNum, i++ ) { |  |
| delta_coef | se(v) |
| nextcoef = ( nextcoef + delta_coef + 256 ) % 256 |  |
| ScalingList[ i ] = nextcoef |  |
| } |  |
| } |  |

# Semantics

## Sequence parameter set RBSP semantics

…

scaling_list_enable_flag equal to 1 specifies that scaling list is used for scaling process for transform coefficients in 8.5.3. scaling_list_enable_flag equal to 0 specifies that scaling list is not used for scaling process for transform coefficients in 8.5.3.

…

sps_extension_flag equal to 0 specifies that no sps_extension_data_flag syntax elements are present in the sequence parameter set RBSP syntax structure. sps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for sps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for sps_extension_flag in a sequence parameter set NAL unit.

sps_extension_data_flag may have any value. It shall not affect the conformance to profiles specified in this Recommendation | International Standard.

## Picture parameter set RBSP semantics

…

pps_extension_flag equal to 0 specifies that no pps_extension_data_flag syntax elements are present in the picture parameter set RBSP syntax structure. pps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for pps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for pps_extension_flag in a picture parameter set NAL unit.

pps_extension_data_flag may have any value. It shall not affect the conformance to profiles specified in this Recommendation | International Standard.

## Adaptation parameter set RBSP semantics

aps_scaling_list_data_present_flag equal to 1 specifies that the scaling list parameters exist in this APS, equal to 0 specifies that scaling list parameters do not exist in this APS.

aps_extension_flag equal to 0 specifies that no aps_extension_data_flag syntax elements are present in the picture parameter set RBSP syntax structure. aps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for aps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for aps_extension_flag in a picture parameter set NAL unit.

aps_extension_data_flag may have any value. It shall not affect the conformance to profiles specified in this Recommendation | International Standard.

### Scaling list parameter semantics

use_default_scaling_list_flag equql to 0 specifies that the value of scaling list is present and the values. use_default_scaling_list_flag equal to 1 specifies that the value of scaling list is not present and the values of all scaling lists are set equal to the default scaling list. The default value of the scaling list is specified in the following tables.

# Table XX – Specification of default scaling lists Default_4x4_Intra and Default_4x4_Inter

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Default_4x4_Intra[ idx ] | 6 | 13 | 13 | 20 | 20 | 20 | 28 | 28 | 28 | 28 | 32 | 32 | 32 | 37 | 37 | 42 |
| Default_4x4_Inter[ idx ] | 10 | 14 | 14 | 20 | 20 | 20 | 24 | 24 | 24 | 24 | 27 | 27 | 27 | 30 | 30 | 34 |

# Table XX – Specification of default scaling lists Default_8x8_Intra and Default_8x8_Inter

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Default_8x8_Intra[ idx ] | 6 | 10 | 10 | 13 | 11 | 13 | 16 | 16 | 16 | 16 | 18 | 18 | 18 | 18 | 18 | 23 |
| Default_8x8_Inter[ idx ] | 9 | 13 | 13 | 15 | 13 | 15 | 17 | 17 | 17 | 17 | 19 | 19 | 19 | 19 | 19 | 21 |

# Table XX (continued) – Specification of default scaling lists Default_8x8_Intra and Default_8x8_Inter

| idx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Default_8x8_Intra[ idx ] | 23 | 23 | 23 | 23 | 23 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 27 | 27 | 27 | 27 |
| Default_8x8_Inter[ idx ] | 21 | 21 | 21 | 21 | 21 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 24 | 24 | 24 | 24 |

# Table XX (continued) – Specification of default scaling lists Default_8x8_Intra and Default_8x8_Inter

| idx | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Default_8x8_Intra[ idx ] | 27 | 27 | 27 | 27 | 29 | 29 | 29 | 29 | 29 | 29 | 29 | 31 | 31 | 31 | 31 | 31 |
| Default_8x8_Inter[ idx ] | 24 | 24 | 24 | 24 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 27 | 27 | 27 | 27 | 27 |

# Table XX (concluded) – Specification of default scaling lists Default_8x8_Intra and Default_8x8_Inter

| idx | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Default_8x8_Intra[ idx ] | 31 | 33 | 33 | 33 | 33 | 33 | 36 | 36 | 36 | 36 | 38 | 38 | 38 | 40 | 40 | 42 |
| Default_8x8_Inter[ idx ] | 27 | 28 | 28 | 28 | 28 | 28 | 30 | 30 | 30 | 30 | 32 | 32 | 32 | 33 | 33 | 35 |

# Table XX – Specification of default scaling lists Default_16x16_Intra

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Default_16x16_Intra[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| idx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| Default_16x16_Intra[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| idx | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| Default_16x16_Intra[ idx ] | 16 | 16 | 16 | 16 | 17 | 16 | 16 | 16 | 17 | 16 | 16 | 16 | 17 | 17 | 17 | 17 |
| idx | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| Default_16x16_Intra[ idx ] | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 18 | 18 | 17 | 17 | 18 | 18 | 18 | 17 | 17 |
| idx | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| Default_16x16_Intra[ idx ] | 18 | 18 | 19 | 18 | 18 | 18 | 19 | 19 | 19 | 19 | 18 | 18 | 18 | 19 | 21 | 20 |
| idx | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 |
| Default_16x16_Intra[ idx ] | 19 | 19 | 20 | 20 | 21 | 20 | 20 | 19 | 19 | 20 | 21 | 22 | 21 | 20 | 20 | 21 |
| idx | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 |
| Default_16x16_Intra[ idx ] | 22 | 23 | 23 | 22 | 21 | 20 | 20 | 21 | 22 | 24 | 23 | 22 | 21 | 22 | 23 | 24 |
| idx | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 |
| Default_16x16_Intra[ idx ] | 25 | 24 | 23 | 22 | 21 | 22 | 23 | 24 | 27 | 25 | 23 | 23 | 23 | 24 | 25 | 27 |
| idx | 128 | 129 | 130 | 131 | 132 | 133 | 134 | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 |
| Default_16x16_Intra[ idx ] | 27 | 25 | 24 | 23 | 23 | 23 | 25 | 27 | 27 | 25 | 25 | 25 | 26 | 27 | 29 | 30 |
| idx | 144 | 145 | 146 | 147 | 148 | 149 | 150 | 151 | 152 | 153 | 154 | 155 | 156 | 157 | 158 | 159 |
| Default_16x16_Intra[ idx ] | 29 | 27 | 26 | 25 | 25 | 25 | 27 | 28 | 27 | 27 | 27 | 29 | 31 | 32 | 32 | 31 |
| idx | 160 | 161 | 162 | 163 | 164 | 165 | 166 | 167 | 168 | 169 | 170 | 171 | 172 | 173 | 174 | 175 |
| Default_16x16_Intra[ idx ] | 29 | 27 | 27 | 27 | 28 | 29 | 29 | 29 | 31 | 33 | 35 | 36 | 35 | 33 | 31 | 29 |
| idx | 176 | 177 | 178 | 179 | 180 | 181 | 182 | 183 | 184 | 185 | 186 | 187 | 188 | 189 | 190 | 191 |
| Default_16x16_Intra[ idx ] | 29 | 29 | 31 | 32 | 33 | 35 | 38 | 40 | 40 | 38 | 35 | 33 | 32 | 31 | 34 | 36 |
| idx | 192 | 193 | 194 | 195 | 196 | 197 | 198 | 199 | 200 | 201 | 202 | 203 | 204 | 205 | 206 | 207 |
| Default_16x16_Intra[ idx ] | 38 | 41 | 43 | 44 | 43 | 41 | 38 | 36 | 34 | 39 | 41 | 44 | 47 | 49 | 49 | 47 |
| idx | 208 | 209 | 210 | 211 | 212 | 213 | 214 | 215 | 216 | 217 | 218 | 219 | 220 | 221 | 222 | 223 |
| Default_16x16_Intra[ idx ] | 44 | 41 | 39 | 44 | 47 | 51 | 54 | 56 | 54 | 51 | 47 | 44 | 51 | 55 | 59 | 62 |
| idx | 224 | 225 | 226 | 227 | 228 | 229 | 230 | 231 | 232 | 233 | 234 | 235 | 236 | 237 | 238 | 239 |
| Default_16x16_Intra[ idx ] | 62 | 59 | 55 | 51 | 60 | 65 | 69 | 70 | 69 | 65 | 60 | 70 | 75 | 79 | 79 | 75 |
| idx | 240 | 241 | 242 | 243 | 244 | 245 | 246 | 247 | 248 | 249 | 250 | 251 | 252 | 253 | 254 | 255 |
| Default_16x16_Intra[ idx ] | 70 | 82 | 88 | 90 | 88 | 82 | 97 | 101 | 101 | 97 | 113 | 115 | 113 | 130 | 130 | 149 |

# Table XX – Specification of default scaling lists Default_16x16_Inter

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Default_16x16_Inter[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| idx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| Default_16x16_Inter[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| idx | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| Default_16x16_Inter[ idx ] | 16 | 16 | 16 | 16 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 |
| idx | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| Default_16x16_Inter[ idx ] | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 |
| idx | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| Default_16x16_Inter[ idx ] | 18 | 18 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 20 | 20 |
| idx | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 |
| Default_16x16_Inter[ idx ] | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 21 | 21 | 21 | 21 | 21 |
| idx | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 |
| Default_16x16_Inter[ idx ] | 21 | 21 | 21 | 21 | 21 | 21 | 21 | 21 | 21 | 22 | 22 | 22 | 22 | 22 | 22 | 22 |
| idx | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 |
| Default_16x16_Inter[ idx ] | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 |
| idx | 128 | 129 | 130 | 131 | 132 | 133 | 134 | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 |
| Default_16x16_Inter[ idx ] | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 |
| idx | 144 | 145 | 146 | 147 | 148 | 149 | 150 | 151 | 152 | 153 | 154 | 155 | 156 | 157 | 158 | 159 |
| Default_16x16_Inter[ idx ] | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 |
| idx | 160 | 161 | 162 | 163 | 164 | 165 | 166 | 167 | 168 | 169 | 170 | 171 | 172 | 173 | 174 | 175 |
| Default_16x16_Inter[ idx ] | 28 | 28 | 28 | 28 | 28 | 29 | 29 | 29 | 29 | 29 | 29 | 29 | 29 | 29 | 29 | 29 |
| idx | 176 | 177 | 178 | 179 | 180 | 181 | 182 | 183 | 184 | 185 | 186 | 187 | 188 | 189 | 190 | 191 |
| Default_16x16_Inter[ idx ] | 29 | 29 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 32 | 32 |
| idx | 192 | 193 | 194 | 195 | 196 | 197 | 198 | 199 | 200 | 201 | 202 | 203 | 204 | 205 | 206 | 207 |
| Default_16x16_Inter[ idx ] | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 36 | 36 | 36 | 36 | 36 | 36 | 36 |
| idx | 208 | 209 | 210 | 211 | 212 | 213 | 214 | 215 | 216 | 217 | 218 | 219 | 220 | 221 | 222 | 223 |
| Default_16x16_Inter[ idx ] | 36 | 36 | 36 | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 45 | 45 | 45 | 45 |
| idx | 224 | 225 | 226 | 227 | 228 | 229 | 230 | 231 | 232 | 233 | 234 | 235 | 236 | 237 | 238 | 239 |
| Default_16x16_Inter[ idx ] | 45 | 45 | 45 | 45 | 52 | 52 | 52 | 52 | 52 | 52 | 52 | 59 | 59 | 59 | 59 | 59 |
| idx | 240 | 241 | 242 | 243 | 244 | 245 | 246 | 247 | 248 | 249 | 250 | 251 | 252 | 253 | 254 | 255 |
| Default_16x16_Inter[ idx ] | 59 | 68 | 68 | 68 | 68 | 68 | 79 | 79 | 79 | 79 | 91 | 91 | 91 | 103 | 103 | 117 |

Table XX – Specification of default scaling lists Default_32x32_Intra

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Default_32x32_Intra[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| idx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| Default_32x32_Intra[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| idx | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| Default_32x32_Intra[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| idx | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| Default_32x32_Intra[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| idx | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| Default_32x32_Intra[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| idx | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 |
| Default_32x32_Intra[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| idx | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 |
| Default_32x32_Intra[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| idx | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 |
| Default_32x32_Intra[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| idx | 128 | 129 | 130 | 131 | 132 | 133 | 134 | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 |
| Default_32x32_Intra[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 17 | 16 | 16 | 16 | 16 | 16 | 16 | 17 |
| idx | 144 | 145 | 146 | 147 | 148 | 149 | 150 | 151 | 152 | 153 | 154 | 155 | 156 | 157 | 158 | 159 |
| Default_32x32_Intra[ idx ] | 17 | 17 | 16 | 16 | 16 | 16 | 16 | 16 | 17 | 17 | 17 | 17 | 16 | 16 | 16 | 17 |
| idx | 160 | 161 | 162 | 163 | 164 | 165 | 166 | 167 | 168 | 169 | 170 | 171 | 172 | 173 | 174 | 175 |
| Default_32x32_Intra[ idx ] | 17 | 17 | 17 | 17 | 17 | 16 | 16 | 16 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 |
| idx | 176 | 177 | 178 | 179 | 180 | 181 | 182 | 183 | 184 | 185 | 186 | 187 | 188 | 189 | 190 | 191 |
| Default_32x32_Intra[ idx ] | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 18 | 17 |
| idx | 192 | 193 | 194 | 195 | 196 | 197 | 198 | 199 | 200 | 201 | 202 | 203 | 204 | 205 | 206 | 207 |
| Default_32x32_Intra[ idx ] | 17 | 17 | 17 | 17 | 17 | 17 | 18 | 18 | 18 | 18 | 17 | 17 | 17 | 17 | 17 | 17 |
| idx | 208 | 209 | 210 | 211 | 212 | 213 | 214 | 215 | 216 | 217 | 218 | 219 | 220 | 221 | 222 | 223 |
| Default_32x32_Intra[ idx ] | 17 | 18 | 18 | 18 | 18 | 17 | 17 | 17 | 17 | 18 | 18 | 18 | 18 | 18 | 18 | 18 |
| idx | 224 | 225 | 226 | 227 | 228 | 229 | 230 | 231 | 232 | 233 | 234 | 235 | 236 | 237 | 238 | 239 |
| Default_32x32_Intra[ idx ] | 17 | 17 | 17 | 17 | 18 | 18 | 18 | 19 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 |
| idx | 240 | 241 | 242 | 243 | 244 | 245 | 246 | 247 | 248 | 249 | 250 | 251 | 252 | 253 | 254 | 255 |
| Default_32x32_Intra[ idx ] | 19 | 19 | 19 | 19 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 19 | 19 | 19 | 18 |

Table XX (continued) – Specification of default scaling lists Default_32x32_Intra

| idx | 256 | 257 | 258 | 259 | 260 | 261 | 262 | 263 | 264 | 265 | 266 | 267 | 268 | 269 | 270 | 271 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Default_32x32_Intra[ idx ] | 18 | 18 | 18 | 18 | 18 | 19 | 19 | 19 | 20 | 19 | 19 | 19 | 18 | 18 | 18 | 18 |
| idx | 272 | 273 | 274 | 275 | 276 | 277 | 278 | 279 | 280 | 281 | 282 | 283 | 284 | 285 | 286 | 287 |
| Default_32x32_Intra[ idx ] | 18 | 18 | 19 | 19 | 20 | 19 | 19 | 19 | 19 | 18 | 19 | 19 | 19 | 20 | 20 | 20 |
| idx | 288 | 289 | 290 | 291 | 292 | 293 | 294 | 295 | 296 | 297 | 298 | 299 | 300 | 301 | 302 | 303 |
| Default_32x32_Intra[ idx ] | 20 | 20 | 20 | 19 | 19 | 19 | 18 | 19 | 19 | 19 | 19 | 20 | 21 | 20 | 20 | 19 |
| idx | 304 | 305 | 306 | 307 | 308 | 309 | 310 | 311 | 312 | 313 | 314 | 315 | 316 | 317 | 318 | 319 |
| Default_32x32_Intra[ idx ] | 19 | 19 | 19 | 19 | 20 | 20 | 20 | 21 | 21 | 21 | 20 | 20 | 20 | 19 | 19 | 19 |
| idx | 320 | 321 | 322 | 323 | 324 | 325 | 326 | 327 | 328 | 329 | 330 | 331 | 332 | 333 | 334 | 335 |
| Default_32x32_Intra[ idx ] | 19 | 19 | 20 | 20 | 21 | 22 | 21 | 20 | 20 | 20 | 19 | 20 | 20 | 20 | 20 | 21 |
| idx | 336 | 337 | 338 | 339 | 340 | 341 | 342 | 343 | 344 | 345 | 346 | 347 | 348 | 349 | 350 | 351 |
| Default_32x32_Intra[ idx ] | 21 | 22 | 22 | 21 | 21 | 20 | 20 | 20 | 20 | 19 | 20 | 20 | 20 | 21 | 22 | 22 |
| idx | 352 | 353 | 354 | 355 | 356 | 357 | 358 | 359 | 360 | 361 | 362 | 363 | 364 | 365 | 366 | 367 |
| Default_32x32_Intra[ idx ] | 22 | 21 | 21 | 20 | 20 | 20 | 20 | 21 | 21 | 22 | 22 | 23 | 23 | 23 | 22 | 22 |
| idx | 368 | 369 | 370 | 371 | 372 | 373 | 374 | 375 | 376 | 377 | 378 | 379 | 380 | 381 | 382 | 383 |
| Default_32x32_Intra[ idx ] | 21 | 21 | 20 | 20 | 20 | 20 | 21 | 21 | 22 | 22 | 23 | 23 | 22 | 21 | 21 | 21 |
| idx | 384 | 385 | 386 | 387 | 388 | 389 | 390 | 391 | 392 | 393 | 394 | 395 | 396 | 397 | 398 | 399 |
| Default_32x32_Intra[ idx ] | 21 | 21 | 21 | 22 | 22 | 23 | 23 | 24 | 24 | 23 | 23 | 22 | 22 | 21 | 21 | 21 |
| idx | 400 | 401 | 402 | 403 | 404 | 405 | 406 | 407 | 408 | 409 | 410 | 411 | 412 | 413 | 414 | 415 |
| Default_32x32_Intra[ idx ] | 21 | 21 | 21 | 22 | 23 | 23 | 24 | 23 | 23 | 22 | 22 | 21 | 21 | 21 | 22 | 22 |
| idx | 416 | 417 | 418 | 419 | 420 | 421 | 422 | 423 | 424 | 425 | 426 | 427 | 428 | 429 | 430 | 431 |
| Default_32x32_Intra[ idx ] | 23 | 23 | 24 | 24 | 25 | 24 | 24 | 23 | 23 | 22 | 22 | 21 | 21 | 21 | 22 | 22 |
| idx | 432 | 433 | 434 | 435 | 436 | 437 | 438 | 439 | 440 | 441 | 442 | 443 | 444 | 445 | 446 | 447 |
| Default_32x32_Intra[ idx ] | 23 | 23 | 24 | 25 | 24 | 24 | 23 | 23 | 22 | 22 | 22 | 22 | 23 | 23 | 24 | 25 |
| idx | 448 | 449 | 450 | 451 | 452 | 453 | 454 | 455 | 456 | 457 | 458 | 459 | 460 | 461 | 462 | 463 |
| Default_32x32_Intra[ idx ] | 25 | 26 | 26 | 25 | 25 | 24 | 23 | 23 | 22 | 22 | 22 | 22 | 23 | 23 | 24 | 24 |
| idx | 464 | 465 | 466 | 467 | 468 | 469 | 470 | 471 | 472 | 473 | 474 | 475 | 476 | 477 | 478 | 479 |
| Default_32x32_Intra[ idx ] | 25 | 27 | 25 | 25 | 24 | 23 | 23 | 23 | 23 | 23 | 23 | 24 | 25 | 25 | 26 | 27 |
| idx | 480 | 481 | 482 | 483 | 484 | 485 | 486 | 487 | 488 | 489 | 490 | 491 | 492 | 493 | 494 | 495 |
| Default_32x32_Intra[ idx ] | 27 | 27 | 26 | 25 | 25 | 24 | 23 | 23 | 23 | 23 | 23 | 23 | 24 | 25 | 25 | 27 |
| idx | 496 | 497 | 498 | 499 | 500 | 501 | 502 | 503 | 504 | 505 | 506 | 507 | 508 | 509 | 510 | 511 |
| Default_32x32_Intra[ idx ] | 28 | 27 | 26 | 25 | 24 | 24 | 24 | 24 | 24 | 24 | 25 | 25 | 26 | 27 | 28 | 28 |

Table XX (continued) – Specification of default scaling lists Default_32x32_Intra

| idx | 512 | 513 | 514 | 515 | 516 | 517 | 518 | 519 | 520 | 521 | 522 | 523 | 524 | 525 | 526 | 527 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Default_32x32_Intra[ idx ] | 28 | 28 | 27 | 26 | 25 | 25 | 24 | 24 | 24 | 24 | 24 | 24 | 25 | 26 | 27 | 28 |
| idx | 528 | 529 | 530 | 531 | 532 | 533 | 534 | 535 | 536 | 537 | 538 | 539 | 540 | 541 | 542 | 543 |
| Default_32x32_Intra[ idx ] | 28 | 27 | 26 | 25 | 25 | 25 | 25 | 25 | 25 | 26 | 26 | 27 | 28 | 29 | 29 | 30 |
| idx | 544 | 545 | 546 | 547 | 548 | 549 | 550 | 551 | 552 | 553 | 554 | 555 | 556 | 557 | 558 | 559 |
| Default_32x32_Intra[ idx ] | 29 | 29 | 28 | 27 | 26 | 26 | 25 | 25 | 25 | 25 | 25 | 25 | 26 | 27 | 28 | 28 |
| idx | 560 | 561 | 562 | 563 | 564 | 565 | 566 | 567 | 568 | 569 | 570 | 571 | 572 | 573 | 574 | 575 |
| Default_32x32_Intra[ idx ] | 27 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 27 | 28 | 29 | 30 | 31 | 31 | 31 | 31 |
| idx | 576 | 577 | 578 | 579 | 580 | 581 | 582 | 583 | 584 | 585 | 586 | 587 | 588 | 589 | 590 | 591 |
| Default_32x32_Intra[ idx ] | 30 | 29 | 28 | 27 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 27 | 28 | 28 | 28 | 27 |
| idx | 592 | 593 | 594 | 595 | 596 | 597 | 598 | 599 | 600 | 601 | 602 | 603 | 604 | 605 | 606 | 607 |
| Default_32x32_Intra[ idx ] | 27 | 27 | 27 | 27 | 27 | 28 | 29 | 30 | 31 | 32 | 32 | 33 | 32 | 32 | 31 | 30 |
| idx | 608 | 609 | 610 | 611 | 612 | 613 | 614 | 615 | 616 | 617 | 618 | 619 | 620 | 621 | 622 | 623 |
| Default_32x32_Intra[ idx ] | 29 | 28 | 27 | 27 | 27 | 27 | 27 | 27 | 28 | 28 | 29 | 28 | 28 | 28 | 28 | 28 |
| idx | 624 | 625 | 626 | 627 | 628 | 629 | 630 | 631 | 632 | 633 | 634 | 635 | 636 | 637 | 638 | 639 |
| Default_32x32_Intra[ idx ] | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 34 | 34 | 34 | 33 | 32 | 31 | 30 | 29 | 28 |
| idx | 640 | 641 | 642 | 643 | 644 | 645 | 646 | 647 | 648 | 649 | 650 | 651 | 652 | 653 | 654 | 655 |
| Default_32x32_Intra[ idx ] | 28 | 28 | 28 | 28 | 28 | 29 | 30 | 29 | 29 | 29 | 29 | 29 | 30 | 31 | 32 | 33 |
| idx | 656 | 657 | 658 | 659 | 660 | 661 | 662 | 663 | 664 | 665 | 666 | 667 | 668 | 669 | 670 | 671 |
| Default_32x32_Intra[ idx ] | 34 | 35 | 36 | 36 | 36 | 35 | 34 | 33 | 32 | 31 | 30 | 29 | 29 | 29 | 29 | 29 |
| idx | 672 | 673 | 674 | 675 | 676 | 677 | 678 | 679 | 680 | 681 | 682 | 683 | 684 | 685 | 686 | 687 |
| Default_32x32_Intra[ idx ] | 30 | 30 | 30 | 30 | 30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 38 | 37 |
| idx | 688 | 689 | 690 | 691 | 692 | 693 | 694 | 695 | 696 | 697 | 698 | 699 | 700 | 701 | 702 | 703 |
| Default_32x32_Intra[ idx ] | 36 | 35 | 34 | 33 | 32 | 31 | 30 | 30 | 30 | 30 | 30 | 31 | 31 | 31 | 32 | 32 |
| idx | 704 | 705 | 706 | 707 | 708 | 709 | 710 | 711 | 712 | 713 | 714 | 715 | 716 | 717 | 718 | 719 |
| Default_32x32_Intra[ idx ] | 33 | 34 | 35 | 37 | 38 | 39 | 40 | 40 | 40 | 39 | 38 | 37 | 35 | 34 | 33 | 32 |
| idx | 720 | 721 | 722 | 723 | 724 | 725 | 726 | 727 | 728 | 729 | 730 | 731 | 732 | 733 | 734 | 735 |
| Default_32x32_Intra[ idx ] | 32 | 31 | 31 | 31 | 33 | 33 | 33 | 33 | 34 | 35 | 36 | 38 | 39 | 41 | 42 | 42 |
| idx | 736 | 737 | 738 | 739 | 740 | 741 | 742 | 743 | 744 | 745 | 746 | 747 | 748 | 749 | 750 | 751 |
| Default_32x32_Intra[ idx ] | 42 | 42 | 41 | 39 | 38 | 36 | 35 | 34 | 33 | 33 | 33 | 33 | 34 | 34 | 35 | 36 |
| idx | 752 | 753 | 754 | 755 | 756 | 757 | 758 | 759 | 760 | 761 | 762 | 763 | 764 | 765 | 766 | 767 |
| Default_32x32_Intra[ idx ] | 37 | 38 | 39 | 41 | 42 | 43 | 44 | 44 | 44 | 43 | 42 | 41 | 39 | 38 | 37 | 36 |

Table XX (concluded) – Specification of default scaling lists Default_32x32_Intra

| idx | 768 | 769 | 770 | 771 | 772 | 773 | 774 | 775 | 776 | 777 | 778 | 779 | 780 | 781 | 782 | 783 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Default_32x32_Intra[ idx ] | 35 | 34 | 34 | 36 | 36 | 37 | 38 | 39 | 41 | 42 | 44 | 45 | 46 | 47 | 47 | 46 |
| idx | 784 | 785 | 786 | 787 | 788 | 789 | 790 | 791 | 792 | 793 | 794 | 795 | 796 | 797 | 798 | 799 |
| Default_32x32_Intra[ idx ] | 45 | 44 | 42 | 41 | 39 | 38 | 37 | 36 | 36 | 38 | 39 | 39 | 41 | 42 | 44 | 45 |
| idx | 800 | 801 | 802 | 803 | 804 | 805 | 806 | 807 | 808 | 809 | 810 | 811 | 812 | 813 | 814 | 815 |
| Default_32x32_Intra[ idx ] | 47 | 48 | 49 | 50 | 49 | 48 | 47 | 45 | 44 | 42 | 41 | 39 | 39 | 38 | 40 | 41 |
| idx | 816 | 817 | 818 | 819 | 820 | 821 | 822 | 823 | 824 | 825 | 826 | 827 | 828 | 829 | 830 | 831 |
| Default_32x32_Intra[ idx ] | 42 | 44 | 45 | 47 | 49 | 51 | 52 | 52 | 52 | 52 | 51 | 49 | 47 | 45 | 44 | 42 |
| idx | 832 | 833 | 834 | 835 | 836 | 837 | 838 | 839 | 840 | 841 | 842 | 843 | 844 | 845 | 846 | 847 |
| Default_32x32_Intra[ idx ] | 41 | 40 | 43 | 44 | 45 | 47 | 49 | 51 | 53 | 54 | 55 | 56 | 55 | 54 | 53 | 51 |
| idx | 848 | 849 | 850 | 851 | 852 | 853 | 854 | 855 | 856 | 857 | 858 | 859 | 860 | 861 | 862 | 863 |
| Default_32x32_Intra[ idx ] | 49 | 47 | 45 | 44 | 43 | 46 | 47 | 49 | 51 | 53 | 55 | 57 | 58 | 59 | 59 | 58 |
| idx | 864 | 865 | 866 | 867 | 868 | 869 | 870 | 871 | 872 | 873 | 874 | 875 | 876 | 877 | 878 | 879 |
| Default_32x32_Intra[ idx ] | 57 | 55 | 53 | 51 | 49 | 47 | 46 | 49 | 51 | 53 | 55 | 57 | 59 | 61 | 62 | 62 |
| idx | 880 | 881 | 882 | 883 | 884 | 885 | 886 | 887 | 888 | 889 | 890 | 891 | 892 | 893 | 894 | 895 |
| Default_32x32_Intra[ idx ] | 62 | 61 | 59 | 57 | 55 | 53 | 51 | 49 | 53 | 55 | 57 | 60 | 62 | 64 | 65 | 66 |
| idx | 896 | 897 | 898 | 899 | 900 | 901 | 902 | 903 | 904 | 905 | 906 | 907 | 908 | 909 | 910 | 911 |
| Default_32x32_Intra[ idx ] | 66 | 65 | 64 | 62 | 60 | 57 | 55 | 53 | 57 | 60 | 62 | 65 | 67 | 69 | 70 | 70 |
| idx | 912 | 913 | 914 | 915 | 916 | 917 | 918 | 919 | 920 | 921 | 922 | 923 | 924 | 925 | 926 | 927 |
| Default_32x32_Intra[ idx ] | 70 | 69 | 67 | 65 | 62 | 60 | 57 | 62 | 65 | 67 | 70 | 72 | 74 | 74 | 74 | 74 |
| idx | 928 | 929 | 930 | 931 | 932 | 933 | 934 | 935 | 936 | 937 | 938 | 939 | 940 | 941 | 942 | 943 |
| Default_32x32_Intra[ idx ] | 72 | 70 | 67 | 65 | 62 | 67 | 70 | 73 | 75 | 77 | 79 | 79 | 79 | 77 | 75 | 73 |
| idx | 944 | 945 | 946 | 947 | 948 | 949 | 950 | 951 | 952 | 953 | 954 | 955 | 956 | 957 | 958 | 959 |
| Default_32x32_Intra[ idx ] | 70 | 67 | 73 | 76 | 79 | 81 | 83 | 84 | 84 | 83 | 81 | 79 | 76 | 73 | 79 | 82 |
| idx | 960 | 961 | 962 | 963 | 964 | 965 | 966 | 967 | 968 | 969 | 970 | 971 | 972 | 973 | 974 | 975 |
| Default_32x32_Intra[ idx ] | 85 | 88 | 89 | 90 | 89 | 88 | 85 | 82 | 79 | 86 | 89 | 92 | 94 | 95 | 95 | 94 |
| idx | 976 | 977 | 978 | 979 | 980 | 981 | 982 | 983 | 984 | 985 | 986 | 987 | 988 | 989 | 990 | 991 |
| Default_32x32_Intra[ idx ] | 92 | 89 | 86 | 93 | 97 | 99 | 101 | 101 | 101 | 99 | 97 | 93 | 101 | 104 | 107 | 108 |
| idx | 992 | 993 | 994 | 995 | 996 | 997 | 998 | 999 | 1000 | 1001 | 1002 | 1003 | 1004 | 1005 | 1006 | 1007 |
| Default_32x32_Intra[ idx ] | 108 | 107 | 104 | 101 | 110 | 113 | 114 | 115 | 114 | 113 | 110 | 119 | 121 | 122 | 122 | 121 |
| idx | 1008 | 1009 | 1010 | 1011 | 1012 | 1013 | 1014 | 1015 | 1016 | 1017 | 1018 | 1019 | 1020 | 1021 | 1022 | 1023 |
| Default_32x32_Intra[ idx ] | 119 | 128 | 130 | 131 | 130 | 128 | 138 | 139 | 139 | 138 | 148 | 149 | 148 | 159 | 159 | 170 |

# Table XX – Specification of default scaling lists Default_32x32_Inter

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Default_32x32_Inter[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| idx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| Default_32x32_Inter[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| idx | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| Default_32x32_Inter[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| idx | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| Default_32x32_Inter[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| idx | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| Default_32x32_Inter[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| idx | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 |
| Default_32x32_Inter[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| idx | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 |
| Default_32x32_Inter[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| idx | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 |
| Default_32x32_Inter[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| idx | 128 | 129 | 130 | 131 | 132 | 133 | 134 | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 |
| Default_32x32_Inter[ idx ] | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 |
| idx | 144 | 145 | 146 | 147 | 148 | 149 | 150 | 151 | 152 | 153 | 154 | 155 | 156 | 157 | 158 | 159 |
| Default_32x32_Inter[ idx ] | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 |
| idx | 160 | 161 | 162 | 163 | 164 | 165 | 166 | 167 | 168 | 169 | 170 | 171 | 172 | 173 | 174 | 175 |
| Default_32x32_Inter[ idx ] | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 |
| idx | 176 | 177 | 178 | 179 | 180 | 181 | 182 | 183 | 184 | 185 | 186 | 187 | 188 | 189 | 190 | 191 |
| Default_32x32_Inter[ idx ] | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 18 | 18 |
| idx | 192 | 193 | 194 | 195 | 196 | 197 | 198 | 199 | 200 | 201 | 202 | 203 | 204 | 205 | 206 | 207 |
| Default_32x32_Inter[ idx ] | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 |
| idx | 208 | 209 | 210 | 211 | 212 | 213 | 214 | 215 | 216 | 217 | 218 | 219 | 220 | 221 | 222 | 223 |
| Default_32x32_Inter[ idx ] | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 |
| idx | 224 | 225 | 226 | 227 | 228 | 229 | 230 | 231 | 232 | 233 | 234 | 235 | 236 | 237 | 238 | 239 |
| Default_32x32_Inter[ idx ] | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 |
| idx | 240 | 241 | 242 | 243 | 244 | 245 | 246 | 247 | 248 | 249 | 250 | 251 | 252 | 253 | 254 | 255 |
| Default_32x32_Inter[ idx ] | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 |

# Table XX (continued) – Specification of default scaling lists Default_32x32_Inter

| idx | 256 | 257 | 258 | 259 | 260 | 261 | 262 | 263 | 264 | 265 | 266 | 267 | 268 | 269 | 270 | 271 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Default_32x32_Inter[ idx ] | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 |
| idx | 272 | 273 | 274 | 275 | 276 | 277 | 278 | 279 | 280 | 281 | 282 | 283 | 284 | 285 | 286 | 287 |
| Default_32x32_Inter[ idx ] | 19 | 19 | 19 | 19 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| idx | 288 | 289 | 290 | 291 | 292 | 293 | 294 | 295 | 296 | 297 | 298 | 299 | 300 | 301 | 302 | 303 |
| Default_32x32_Inter[ idx ] | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 21 | 21 | 21 | 21 |
| idx | 304 | 305 | 306 | 307 | 308 | 309 | 310 | 311 | 312 | 313 | 314 | 315 | 316 | 317 | 318 | 319 |
| Default_32x32_Inter[ idx ] | 21 | 21 | 21 | 21 | 21 | 21 | 21 | 21 | 21 | 21 | 21 | 21 | 21 | 21 | 21 | 21 |
| idx | 320 | 321 | 322 | 323 | 324 | 325 | 326 | 327 | 328 | 329 | 330 | 331 | 332 | 333 | 334 | 335 |
| Default_32x32_Inter[ idx ] | 21 | 21 | 21 | 21 | 21 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 |
| idx | 336 | 337 | 338 | 339 | 340 | 341 | 342 | 343 | 344 | 345 | 346 | 347 | 348 | 349 | 350 | 351 |
| Default_32x32_Inter[ idx ] | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 |
| idx | 352 | 353 | 354 | 355 | 356 | 357 | 358 | 359 | 360 | 361 | 362 | 363 | 364 | 365 | 366 | 367 |
| Default_32x32_Inter[ idx ] | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 |
| idx | 368 | 369 | 370 | 371 | 372 | 373 | 374 | 375 | 376 | 377 | 378 | 379 | 380 | 381 | 382 | 383 |
| Default_32x32_Inter[ idx ] | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 22 | 23 | 23 | 23 | 23 | 23 | 23 |
| idx | 384 | 385 | 386 | 387 | 388 | 389 | 390 | 391 | 392 | 393 | 394 | 395 | 396 | 397 | 398 | 399 |
| Default_32x32_Inter[ idx ] | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| idx | 400 | 401 | 402 | 403 | 404 | 405 | 406 | 407 | 408 | 409 | 410 | 411 | 412 | 413 | 414 | 415 |
| Default_32x32_Inter[ idx ] | 23 | 23 | 23 | 23 | 23 | 23 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| idx | 416 | 417 | 418 | 419 | 420 | 421 | 422 | 423 | 424 | 425 | 426 | 427 | 428 | 429 | 430 | 431 |
| Default_32x32_Inter[ idx ] | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| idx | 432 | 433 | 434 | 435 | 436 | 437 | 438 | 439 | 440 | 441 | 442 | 443 | 444 | 445 | 446 | 447 |
| Default_32x32_Inter[ idx ] | 24 | 24 | 24 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| idx | 448 | 449 | 450 | 451 | 452 | 453 | 454 | 455 | 456 | 457 | 458 | 459 | 460 | 461 | 462 | 463 |
| Default_32x32_Inter[ idx ] | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| idx | 464 | 465 | 466 | 467 | 468 | 469 | 470 | 471 | 472 | 473 | 474 | 475 | 476 | 477 | 478 | 479 |
| Default_32x32_Inter[ idx ] | 25 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 |
| idx | 480 | 481 | 482 | 483 | 484 | 485 | 486 | 487 | 488 | 489 | 490 | 491 | 492 | 493 | 494 | 495 |
| Default_32x32_Inter[ idx ] | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 |
| idx | 496 | 497 | 498 | 499 | 500 | 501 | 502 | 503 | 504 | 505 | 506 | 507 | 508 | 509 | 510 | 511 |
| Default_32x32_Inter[ idx ] | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 |

# Table XX (continued) – Specification of default scaling lists Default_32x32_Inter

| idx | 512 | 513 | 514 | 515 | 516 | 517 | 518 | 519 | 520 | 521 | 522 | 523 | 524 | 525 | 526 | 527 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Default_32x32_Inter[ idx ] | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 |
| idx | 528 | 529 | 530 | 531 | 532 | 533 | 534 | 535 | 536 | 537 | 538 | 539 | 540 | 541 | 542 | 543 |
| Default_32x32_Inter[ idx ] | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 |
| idx | 544 | 545 | 546 | 547 | 548 | 549 | 550 | 551 | 552 | 553 | 554 | 555 | 556 | 557 | 558 | 559 |
| Default_32x32_Inter[ idx ] | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 |
| idx | 560 | 561 | 562 | 563 | 564 | 565 | 566 | 567 | 568 | 569 | 570 | 571 | 572 | 573 | 574 | 575 |
| Default_32x32_Inter[ idx ] | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 |
| idx | 576 | 577 | 578 | 579 | 580 | 581 | 582 | 583 | 584 | 585 | 586 | 587 | 588 | 589 | 590 | 591 |
| Default_32x32_Inter[ idx ] | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 |
| idx | 592 | 593 | 594 | 595 | 596 | 597 | 598 | 599 | 600 | 601 | 602 | 603 | 604 | 605 | 606 | 607 |
| Default_32x32_Inter[ idx ] | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 |
| idx | 608 | 609 | 610 | 611 | 612 | 613 | 614 | 615 | 616 | 617 | 618 | 619 | 620 | 621 | 622 | 623 |
| Default_32x32_Inter[ idx ] | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 29 | 29 | 29 | 29 | 29 | 29 |
| idx | 624 | 625 | 626 | 627 | 628 | 629 | 630 | 631 | 632 | 633 | 634 | 635 | 636 | 637 | 638 | 639 |
| Default_32x32_Inter[ idx ] | 29 | 29 | 29 | 29 | 29 | 29 | 29 | 29 | 29 | 29 | 29 | 29 | 29 | 29 | 29 | 29 |
| idx | 640 | 641 | 642 | 643 | 644 | 645 | 646 | 647 | 648 | 649 | 650 | 651 | 652 | 653 | 654 | 655 |
| Default_32x32_Inter[ idx ] | 29 | 29 | 29 | 29 | 29 | 29 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
| idx | 656 | 657 | 658 | 659 | 660 | 661 | 662 | 663 | 664 | 665 | 666 | 667 | 668 | 669 | 670 | 671 |
| Default_32x32_Inter[ idx ] | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
| idx | 672 | 673 | 674 | 675 | 676 | 677 | 678 | 679 | 680 | 681 | 682 | 683 | 684 | 685 | 686 | 687 |
| Default_32x32_Inter[ idx ] | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
| idx | 688 | 689 | 690 | 691 | 692 | 693 | 694 | 695 | 696 | 697 | 698 | 699 | 700 | 701 | 702 | 703 |
| Default_32x32_Inter[ idx ] | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 31 | 31 | 31 | 31 | 31 |
| idx | 704 | 705 | 706 | 707 | 708 | 709 | 710 | 711 | 712 | 713 | 714 | 715 | 716 | 717 | 718 | 719 |
| Default_32x32_Inter[ idx ] | 31 | 31 | 31 | 31 | 31 | 31 | 31 | 31 | 31 | 31 | 31 | 31 | 31 | 31 | 31 | 31 |
| idx | 720 | 721 | 722 | 723 | 724 | 725 | 726 | 727 | 728 | 729 | 730 | 731 | 732 | 733 | 734 | 735 |
| Default_32x32_Inter[ idx ] | 31 | 31 | 31 | 31 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| idx | 736 | 737 | 738 | 739 | 740 | 741 | 742 | 743 | 744 | 745 | 746 | 747 | 748 | 749 | 750 | 751 |
| Default_32x32_Inter[ idx ] | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 33 | 33 | 33 | 33 |
| idx | 752 | 753 | 754 | 755 | 756 | 757 | 758 | 759 | 760 | 761 | 762 | 763 | 764 | 765 | 766 | 767 |
| Default_32x32_Inter[ idx ] | 33 | 33 | 33 | 33 | 33 | 33 | 33 | 33 | 33 | 33 | 33 | 33 | 33 | 33 | 33 | 33 |

# Table XX (concluded) – Specification of default scaling lists Default_32x32_Inter

| idx | 768 | 769 | 770 | 771 | 772 | 773 | 774 | 775 | 776 | 777 | 778 | 779 | 780 | 781 | 782 | 783 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Default_32x32_Inter[ idx ] | 33 | 33 | 33 | 34 | 34 | 34 | 34 | 34 | 34 | 34 | 34 | 34 | 34 | 34 | 34 | 34 |
| idx | 784 | 785 | 786 | 787 | 788 | 789 | 790 | 791 | 792 | 793 | 794 | 795 | 796 | 797 | 798 | 799 |
| Default_32x32_Inter[ idx ] | 34 | 34 | 34 | 34 | 34 | 34 | 34 | 34 | 34 | 35 | 35 | 35 | 35 | 35 | 35 | 35 |
| idx | 800 | 801 | 802 | 803 | 804 | 805 | 806 | 807 | 808 | 809 | 810 | 811 | 812 | 813 | 814 | 815 |
| Default_32x32_Inter[ idx ] | 35 | 35 | 35 | 35 | 35 | 35 | 35 | 35 | 35 | 35 | 35 | 35 | 35 | 35 | 36 | 36 |
| idx | 816 | 817 | 818 | 819 | 820 | 821 | 822 | 823 | 824 | 825 | 826 | 827 | 828 | 829 | 830 | 831 |
| Default_32x32_Inter[ idx ] | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 |
| idx | 832 | 833 | 834 | 835 | 836 | 837 | 838 | 839 | 840 | 841 | 842 | 843 | 844 | 845 | 846 | 847 |
| Default_32x32_Inter[ idx ] | 36 | 36 | 38 | 38 | 38 | 38 | 38 | 38 | 38 | 38 | 38 | 38 | 38 | 38 | 38 | 38 |
| idx | 848 | 849 | 850 | 851 | 852 | 853 | 854 | 855 | 856 | 857 | 858 | 859 | 860 | 861 | 862 | 863 |
| Default_32x32_Inter[ idx ] | 38 | 38 | 38 | 38 | 38 | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 |
| idx | 864 | 865 | 866 | 867 | 868 | 869 | 870 | 871 | 872 | 873 | 874 | 875 | 876 | 877 | 878 | 879 |
| Default_32x32_Inter[ idx ] | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 42 |
| idx | 880 | 881 | 882 | 883 | 884 | 885 | 886 | 887 | 888 | 889 | 890 | 891 | 892 | 893 | 894 | 895 |
| Default_32x32_Inter[ idx ] | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 45 | 45 | 45 | 45 | 45 | 45 | 45 | 45 |
| idx | 896 | 897 | 898 | 899 | 900 | 901 | 902 | 903 | 904 | 905 | 906 | 907 | 908 | 909 | 910 | 911 |
| Default_32x32_Inter[ idx ] | 45 | 45 | 45 | 45 | 45 | 45 | 45 | 45 | 48 | 48 | 48 | 48 | 48 | 48 | 48 | 48 |
| idx | 912 | 913 | 914 | 915 | 916 | 917 | 918 | 919 | 920 | 921 | 922 | 923 | 924 | 925 | 926 | 927 |
| Default_32x32_Inter[ idx ] | 48 | 48 | 48 | 48 | 48 | 48 | 48 | 52 | 52 | 52 | 52 | 52 | 52 | 52 | 52 | 52 |
| idx | 928 | 929 | 930 | 931 | 932 | 933 | 934 | 935 | 936 | 937 | 938 | 939 | 940 | 941 | 942 | 943 |
| Default_32x32_Inter[ idx ] | 52 | 52 | 52 | 52 | 52 | 56 | 56 | 56 | 56 | 56 | 56 | 56 | 56 | 56 | 56 | 56 |
| idx | 944 | 945 | 946 | 947 | 948 | 949 | 950 | 951 | 952 | 953 | 954 | 955 | 956 | 957 | 958 | 959 |
| Default_32x32_Inter[ idx ] | 56 | 56 | 60 | 60 | 60 | 60 | 60 | 60 | 60 | 60 | 60 | 60 | 60 | 60 | 64 | 64 |
| idx | 960 | 961 | 962 | 963 | 964 | 965 | 966 | 967 | 968 | 969 | 970 | 971 | 972 | 973 | 974 | 975 |
| Default_32x32_Inter[ idx ] | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 69 | 69 | 69 | 69 | 69 | 69 | 69 |
| idx | 976 | 977 | 978 | 979 | 980 | 981 | 982 | 983 | 984 | 985 | 986 | 987 | 988 | 989 | 990 | 991 |
| Default_32x32_Inter[ idx ] | 69 | 69 | 69 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 80 | 80 | 80 | 80 |
| idx | 992 | 993 | 994 | 995 | 996 | 997 | 998 | 999 | 1000 | 1001 | 1002 | 1003 | 1004 | 1005 | 1006 | 1007 |
| Default_32x32_Inter[ idx ] | 80 | 80 | 80 | 80 | 87 | 87 | 87 | 87 | 87 | 87 | 87 | 94 | 94 | 94 | 94 | 94 |
| idx | 1008 | 1009 | 1010 | 1011 | 1012 | 1013 | 1014 | 1015 | 1016 | 1017 | 1018 | 1019 | 1020 | 1021 | 1022 | 1023 |
| Default_32x32_Inter[ idx ] | 94 | 101 | 101 | 101 | 101 | 101 | 108 | 108 | 108 | 108 | 115 | 115 | 115 | 123 | 123 | 131 |

pred_mode_flag equal to 0 specifies that the value of the scaling list is the same as the value of the scaling list already sent. The scaling list to copy the value is specified by pred_matrix_id_delta. pred_mode_flag equal to 1 speficies that the value of the scaling list is encoded by DPCM and exp-Golomb code.

pred_matrix_id_delta specifies the target reference matrix to copy the value of scaling list. When pred_mode_flag is equal to 0, pred_matrix_id_delta specifies which matrix should be used in the current matrix by the following formula.

Table xx: Semantics of SizeID

| Value of SizeID | Size of quantization matrix |
| --- | --- |
| 0 | 4x4 |
| 1 | 8x8 (16x4, 4x16) |
| 2 | 16x16 (32x8, 8x32) |
| 3 | 32x32 |

Table xx: Semantics of MatrixID (SizeID < 3)

| Value of MatrixID | MB prediction type | Component |
| --- | --- | --- |
| 0 | Intra | Y |
| 1 | Intra | Cb |
| 2 | Intra | Cr |
| 3 | Inter | Y |
| 4 | Inter | Cb |
| 5 | Inter | Cr |

Table 5: Semantics of MatrixID (SizeID = = 3)

| Value of MatrixID | MB prediction type | Component |
| --- | --- | --- |
| 0 | Intra | Y |
| 1 | Inter | Y |

RefMatrixID = MatrixID – ( 1 + pred_matrix_id_delta )

Note: For example, when MatrixID (current) is equal to 5 (InterCr) and pred_matrix_id_delta is equal to 1, then the matrix is copied from RefMatrixID = MatrixID – ( 1 + pred_matrix_id_delta ) = 3

### Scaling_list semantics

delta_coef specifies the difference of the matrix coefficient from the previous matrix, when pred_mode_flag is equal to 1.

For 4x4,32x32 : ScalingList[SizeId][MatrixID] specifies the value of scaling list.

For 8x8,16x16 : ScalingList[SizeId][MatrixID][Direction] specifies the value of scaling list.

ScalingList[SizeId][MatrixID][Direction] specifies the value of scaling list.

where Direction is 0 : Square / 1 : Horizontal / 2 : Vertical (1 and 2 are used for NSQT)

Derivation of ScalingList for NSQT:

The ScalingList for NSQT is derived as follows.

16x4 ScalingList derivation:

The ScalingList is derived exctracting 4 horizontal lines from 16x16 ScalingList.

for(MatrixId = 0; MatrixId < 6; MatrixId++)

for(j=0;j=4;j++)

for(i=0;i=16;i++)

ScalingList[1][MatrixId][1][j*16+i] = ScalingList[2][MatrixId][0][(j*4)*16+i]

4x16 ScalingList derivation:

The ScalingList is derived exctracting 4 vertical lines from 16x16 ScalingList.

for(MatrixId = 0; MatrixId < 6; MatrixId++)

for(j=0;j=16;j++)

for(i=0;i=4;i++)

ScalingList[1][MatrixId][2][j*4+i] = ScalingList[2][MatrixId][0][j*16+i*4];

32x8 ScalingList derivation:

The ScalingList is derived exctracting 4 horizontal lines from 32x32 ScalingList.

for(MatrixId = 0; MatrixId < 2; MatrixId++)

for(j=0;j=8;j++)

for(i=0;i=32;i++)

ScalingList[2][MatrixId*3][1][j*32+i] = ScalingList[3][MatrixId][(j*4)*32+i];

8x32 ScalingList derivation:

The ScalingList is derived exctracting 4 vertical lines from 32x32 ScalingList.

for(MatrixId = 0; MatrixId < 2; MatrixId++)

for(j=0;j=32;j++)

for(i=0;i=8;i++)

ScalingList[2][MatrixId*3][2][j*8+i] = ScalingList[3][MatrixId][j*32+i*4];