| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0137 |
| --- | --- |

| Title: | AHG21: Reference picture lists combination syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yan Ye Yong He 9710 Scranton Rd, Suite 250 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4803 yan.ye@interdigital.com Yong.he@interdigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

In this contribution, the reference picture list combination process is slightly modified to remove redundant signalling. Then, a reordering method for the combined list based on POC distance is implemented in the HM5.0 software.

# Reference picture list combination

Currently in WD5 [1], the reference picture list combination syntax shown in Table 1 is used. Some signalling in Table 1 is redundant, and the revised syntax shown in Table 2 removes such redundancy. Changes are highlighted in yellow. As shown in Table 2, the syntax ref_idx_list_curr only needs to be signaled when L0 (if pic_from_list_0_flag is 1) or L1 (if pic_from_list_0_flag is 0) contains more than 1 entry. Additionally, instead of using ue(v), te(v) is a more efficient way to signal ref_idx_list_curr.

- Current reference picture list combination syntax in WD5

| ref_pic_list_combination( ) { | Descriptor |
| --- | --- |
| if( slice_type % 5 = = 1 ) { // b slice |  |
| ref_pic_list_combination_flag | u(1) |
| if( ref_pic_list_combination_flag ) { |  |
| num_ref_idx_lc_active_minus1 | ue(v) |
| ref_pic_list_modification_flag_lc | u(1) |
| if( ref_pic_list_modification_flag_lc ) |  |
| for ( i =0; i <= num_ref_idx_lc_active_minus1; i++ ) { |  |
| pic_from_list_0_flag | u(1) |
| ref_idx_list_curr | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

- Proposed reference picture lists combination syntax

| ref_pic_list_combination( ) { | Descriptor |
| --- | --- |
| if( slice_type % 5 = = 1 ) { // b slice |  |
| ref_pic_list_combination_flag | u(1) |
| if( ref_pic_list_combination_flag ) { |  |
| num_ref_idx_lc_active_minus1 | ue(v) |
| ref_pic_list_modification_flag_lc | u(1) |
| if( ref_pic_list_modification_flag_lc ) |  |
| for ( i =0; i <= num_ref_idx_lc_active_minus1; i++ ) { |  |
| pic_from_list_0_flag | u(1) |
| if ( ( pic_from_list_0_flag == 1 && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( pic_from_list_0_flag == 0 && num_ref_idx_l1_activie_minus1 > 0 ) |  |
| ref_idx_list_curr | te(v) |
| } |  |
| } |  |
| } |  |
| } |  |

ref_idx_list_curr indicates the reference index of the picture in the reference picture list specified by pic_from_list_0_flag to be appended at the end of the reference picture lists combination. If ref_idx_list_curr is not present, it is set to 0.

# POC distance and QP based reordering of combined list

Based on the syntax proposed in Table 2, a POC-distance and QP based LC reordering method is implemented in HM5.0. The proposed LC reordering method uses the following two steps to reorder LC:

- Sort LC by POC distance between the current picture and the reference picture in increasing order; that is, reference pictures closer to the current picture are given smaller index in LC

- For entries with the same POC distance, break the tie by sorting the entries based on QP in increasing order; that is, reference pictures coded with smaller QP are given smaller index in LC

Table 3 shows the difference between the default LC in HM5.0 and the reordered LC based on the proposed method for the Random Access GOP structure used in HM5.0 common testing conditions (without temporal scalability) in Figure 1.

- Proposed reference picture lists combination syntax

| POC in coding order | Default LC in HM5.0 | Reordered LC |
| --- | --- | --- |
| 16 | {8, 6, 4, 0} | {8, 6, 4, 0} |
| 12 | {8, 16, 6} | {8, 16, 6} |
| 10 | {8, 12, 6, 16} | {8, 12, 6, 16} |
| 9 | {8, 10, 12} | {8, 10, 12} |
| 11 | {10, 12, 8, 16} | {12, 10, 8, 16} |
| 14 | {12, 16, 10} | {16, 12, 10} |
| 13 | {12, 14, 8, 16} | {12, 14, 16, 8} |
| 15 | {14, 16, 12} | {16, 14, 12} |

Figure 1. GOP structure for Random Access

# Simulation results

Simulations are underway.

# References

B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand. WD5: Working Draft 5 of High-Efficiency Video Coding. Document no JCTVC-G1103, November 2011.

F. Bossen, Common HM test conditions and software reference configurations. Document no JCTVC-G1200, November 2011.

Y. Suzuki, et al, Extension of uni-prediction simplification in B slices. Document no JCTVC-D421, January 2011.

# Patent rights declaration(s)

InterDigital Communications, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).