| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0491 |
| --- | --- |

| Title: | Non-CE6a: Remove the large multiplier for LM mode calculation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lingzhi Liu (Hisilicon Technologies), Guichun Li (Santa Clara University) Yongbing Lin (Hisilicon Technologies) | Tel: Email: | +1-408-330-4996 lingzhi.liu@huawei.com g1li@scu.edu linyongbing@huawei.com |
| Source: | Hisilicon Technologies & Santa Clara University |  |  |

_____________________________

# Abstract

This proposal provides a solution to simplify the Chroma from Luma intra prediction (LM mode) in HEVC standard. The BD-Rate for this optimization is: AI-HE: Y: 0.0%, U: 0.0%, V: 0.1%. The complexity can be further reduced by reducing the bit width of other multipliers. These modifications will contribute to complexity reduction both for HW and SW implementation.

# Introduction

For chroma blocks, the current design of High Efficiency Video Coding (HEVC) has a chroma from luma intra prediction mode, which is also referred as LM mode, in addition to the directional modes and the DC mode.

According to the HEVC working draft [1] and the HM-5.0 software, the current design of LM mode uses reconstructed luma samples to generate chroma samples. The values of the prediction are derived as the following ordered steps [1]:

- Variable k3 and the sample array pY’ are derived as:

k3 = Max( 0, BitDepthC + log2( nS ) – 14 )		(1)

pY’[ x, y ] = ( recSamplesL[ 2x-1, 2y+1 ] + 
2*recSamplesL[ 2x, 2y+1 ] + recSamplesL[ 2x+1, 2y+1 ] + 2 ) >> 2, with x=0..nS-1, y = -1	(2)

pY’[ x, y ] = ( recSamplesL[ 2x, 2y ] + recSamplesL[ 2x, 2y+1 ] ) >> 1, with x=-1..nS-1, y = 0..nS-1	(3)

- Variables L, C, LL, LC and k2 are derived as follows:

L = 		(4)

C = 		(5)

LL = 	(6)

LC = 	(7)

k2 = log2( (2*nS) >> k3 )		(8)

- Variables a, b and k are derived as:

a1 	= ( LC << k2 ) – L*C		(9)
a2 	= ( LL << k2 ) – L*L		(10)
k1 	= Max( 0, log2( abs( a2 ) ) – 5 ) – Max( 0, log2( abs( a1 ) ) – 14 ) + 2	(11)
a1s	= a1 >> Max(0, log2( abs( a1 ) ) – 14 )		(12)
a2s	= abs( a2 >> Max(0, log2( abs( a2 ) ) – 5 ) )		(13)
a3	= a2s < 1 ? 0 : Clip3( -215, 215-1, a1s*lmDiv + ( 1 << ( k1 – 1 ) ) >> k1 )	(14)

alpha	= a3 >> Max( 0, log2( abs( a3 ) ) – 6 )		(15)
k	= 13 – Max( 0, log2( abs( alpha ) ) – 6 )		(16)

beta	= ( L – ( ( alpha * C ) >> k1 ) + ( 1 << ( k2 – 1 ) ) ) >> k2	(17)

where lmDiv is specified in a look-up table [1] with the input a2s.

- The values of the prediction samples predSamples[ x, y ] are derived as:

predSamples[ x, y ] = Clip1C( ( ( pY’[ x, y ] * alpha ) >> k ) + beta ), with x, y = 0..nS-1	(18)

The lmDiv calculation of (14) in current LM mode is base a 63-entry look-up table, where each entry is defined as:

UInt   lmDiv [ 64 ];

for( Int i = 1; i < 64; i++ )

lmDiv [i-1] = ( (1 << 15) + i/2 ) / i;                       (19)

After a2s is calculated from (10) and (13), it is used as the index of (19) to get the value of lmDiv.

# Simulation results

Table 1. Testing results of proposed algorithm Case 1 vs. HM 5.0

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | -0.01% | -0.13% | 0.00% |
| Class B | -0.04% | -0.22% | 0.15% |
| Class C | -0.02% | -0.04% | 0.08% |
| Class D | -0.01% | 0.16% | 0.09% |
| Class E | -0.01% | 0.10% | 0.39% |
| Overall | -0.02% | -0.03% | 0.14% |
|  | -0.02% | -0.03% | 0.15% |
| Class F | -0.05% | -0.38% | -0.24% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

Table 2. Testing results of proposed algorithm Case 2 vs. HM 5.0

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | -0.01% | 0.09% | -0.06% |
| Class B | 0.01% | 0.25% | 0.05% |
| Class C | 0.00% | 0.03% | 0.13% |
| Class D | 0.00% | 0.16% | 0.02% |
| Class E | 0.01% | 0.21% | 0.37% |
| Overall | 0.00% | 0.16% | 0.10% |
|  | 0.00% | 0.16% | 0.10% |
| Class F | -0.02% | -0.29% | -0.32% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 99% |  |  |

Table 3. Testing results of proposed algorithm Case 3 vs. HM 5.0

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | 0.08% | 0.59% | -0.08% |
| Class B | 0.11% | 1.18% | 0.41% |
| Class C | 0.08% | 0.61% | 0.67% |
| Class D | 0.07% | 0.43% | 0.28% |
| Class E | 0.02% | 0.88% | 0.68% |
| Overall | 0.07% | 0.77% | 0.43% |
|  | 0.07% | 0.78% | 0.43% |
| Class F | 0.06% | 0.21% | 0.20% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 96% |  |  |

# Conclusion

It is proposed to consider adopting the optimization for LM mode calculation.

# Reference

[1] Benjamin Bross, et. Al, “WD5: Working Draft 5 of High-Efficiency Video Coding”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1103

[2] Jianle Chen, et. Al, “Complexity reduction of chroma intra LM prediction mode”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F494

[3] Frank Bossen, “Common HM test conditions and software reference configurations”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1200

# Patent rights declaration(s)

Hisilicon Technologies and Santa Clara University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).