Changes to WD (JCTVC-G1103-d8) for H0530 with the simple “transform and quantization and in-loop filtering by-pass” lossless solution.

7.4.9         Transform coefficient Semantics

The transform coefficient levels are parsed into the arrays transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ]. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. The array indices xC, yC specify the transform coefficient location ( xC, yC ) within the current transform block.

When PredMode is equal to MODE_INTRA, different scanning orders are used. The array ScanType[ log2TrafoSize − 2 ][ IntraPredMode ], specifying the scanning order for various luma transform block sizes and intra prediction modes, is derived as specified in Table 7-16.

Table Error! No text of specified style in document.-1 – Specification of ScanType[ log2TrafoSize − 2 ][ IntraPredMode ]

| IntraPredMode | log2TrafoSize – 2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 0 | 1 | 1 | 0 | 0 |
| 1 | 2 | 2 | 0 | 0 |
| 2-3 | 0 | 0 | 0 | 0 |
| 4-5 | 1 | 1 | 0 | 0 |
| 6 | 0 | 0 | 0 | 0 |
| 7-8 | 2 | 2 | 0 | 0 |
| 9-10 | 0 | 0 | 0 | 0 |
| 11-12 | 1 | 1 | 0 | 0 |
| 13-14 | 0 | 0 | 0 | 0 |
| 15-16 | 2 | 2 | 0 | 0 |
| 17-19 | 0 | 0 | 0 | 0 |
| 20-23 | 1 | 1 | 0 | 0 |
| 24-27 | 0 | 0 | 0 | 0 |
| 28-31 | 2 | 2 | 0 | 0 |
| 32-33 | 0 | 0 | 0 | 0 |
| 34 | 0 | 0 | 0 | 0 |
| 35 | 0 | 0 | 0 | 0 |

cu_qp_delta can change the value of QPY for a quantization group of coding units where the quantization group of coding units is specified as follows:

- If a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 0 and the log2CUSize is greater than or equal to log2MinCUDQPSize, the quantization group includes this coding unit only.

- Otherwise, if a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 1 and the log2CUSize is equal to log2MinCUDQPSize, the quantization group includes all coding units split from this coding unit.

The decoded value of cu_qp_delta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. cu_qp_delta shall be inferred to be equal to 0 when it is not present for any quantization group of coding units.

The value of QPY is derived as

QPY = ( ( ( QPY,PREV + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(7-21)

where QPY,PREV is the luma quantization parameter, QPY, of the left neighbor quantization group of coding units in the current slice. If the left neighbor quantization group in the current slice is not available, QPY,PREV is the luma quantization parameter, of the previous quantization group in decoding order that is available in the current slice. For the first quantization group of coding units in the slice QPY,PREV is initially set equal to SliceQPY at the start of each slice. In addition, for the first set of quantization group of coding units in a tile QPY,PREV is initially set equal to SliceQPY . If  QPY, PREV is equal to 0 and cu_qp_delta is not equal to 0, QPY,PREV is set equal to SliceQPY.

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(7-22)

## Scaling, transformation and array construction process prior to deblocking filter process

### Scaling and transformation process

Inputs to this process are:

–	a luma location ( xT, yT ) specifying the top-left luma sample of the current transform unit relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable nW specifying the width of the current transform block.

–	a variable nH specifying the height of the current transform block.

Output of this process is:

–	a modified version of the (nW)x(nH) array of residual samples r with elements rij.

The quantization parameter qP is derived as follows.

–	If cIdx is equal to 0,

qP = QP’Y			(8-150)

–	Otherwise,

qP = QP’C			(8-151)

The (nW)x(nH) array of residual samples r are derived as specified in the following ordered steps.

- The scaling process for transform coefficients as specified in subclause 8.5.3 is invoked with the width of the transform unit nW, the height of the transform unit nH, the (nW)x(nH) array of transform coefficients transCoeffLevel[ xT ][ yT ][ cIdx ], the chroma component variable cIdx and the quantization parameter qP as the inputs and the output is a scaled transform coefficient (nW)x(nH) array d.

- The transformation process for scaled transform coefficients as specified in subclause 8.5.4 is invoked with the width of the transform unit nW, the height of the transform unit nH, the scaled transform coefficient (nW)x(nH) array d and the chroma component variable cIdx as the inputs and the output is a residual samples (nW)x(nH) array r.

### Scaling process for transform coefficients

Inputs of this process are:

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit,

–	a (nW)x(nH) array c of transform coefficients with elements cij,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable qP specifying the quantization parameter.

Output of this process is scaled transform coefficients as a (nW)x(nH) array of d with elements dij.

If QP’Y	 is equal to 0,

dij = cij, 	
	          with i = 0..nW − 1, j = 0..nH − 1

Otherwise (if QP’Y	 is not equal to 0),

the variable log2TrSize is derived as follows:

log2TrSize = ( log2( NW ) + log2( NH ) ) >> 1		(8-158)

The variable shift is derived as follows:

–	If cIdx is equal to 0,

shift = BitDepthY + log2TrSize – 9		(8-150)

–	Otherwise,

shift = BitDepthC + log2TrSize – 9		(8-150)

The scaling array levelScale[·] is specified as levelScale[k] = { 40, 45, 51, 57, 64, 72 } with k=0..5.

The variable shiftScale is set equal to shift + 4 – QP/6.

The elements of array M[ i ][ j ] with i = 0..nW – 1, j = 0..nH – 1 are set equal to ScalingList[ SizeID ][ RefMatrixID ][ trafoType ][ i*nW+j ], where SizeID and RefMatrixID are specified in Error! Reference source not found. and Error! Reference source not found., respectively, and trafoType is derived by

trafoType = ( ( nW = = nH ) ? 0 : ( ( nW > nH ) ? 1 : 2 ) )	(8-158)

The scaled transform coefficient dij with i = 0..nW − 1, j = 0..nH − 1 is derived as follows.

–	If seq_scaling_list_present_flag is equal to 1 or pic_scaling_list_present_flag is equal to 1, [Ed. (WJ): should be replaced by better conditioning]

dij = Clip3( -32768, 32767,( ( cij * levelScale[ qP%6 ] << ( qP/6 ) ) + ( 1 << ( shift – 1 ) ) ) >> shift )	(8-158)

–	Otherwise, if shiftScale is greater than 0,

dij = Clip3( -32768, 32767, ( cij * M[ i ][ j ]*levelScale[ qP%6 ] + 					( 1 << ( shiftScale – 1 ) ) ) >> shiftScale )	(8-158)

–	Otherwise,

dij = Clip3( -32768, 32767, ( cij * M[ i ][ j ]*levelScale[ qP%6 ] ) << ( -shiftScale ) )	(8-158)

[Ed. (WJ): do we need to clip cij to 16b before computing dij? – related to G719 – maybe not]

### Transformation process for scaled transform coefficients

Inputs of this process are:

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit,

–	a (nW)x(nH) array d of scaled transform coefficients with elements dij.

–	a variable cIdx specifying the chroma component of the current block,

Output of this process is residual samples as a (nW)x(nH) array r with elements rij.

If QP’Y	 is equal to 0,

rij = dij, with i=0..(nW)−1, j=0..(nH)−1

Otherwise (if QP’Y is not equal to 0),

depending on PredMode and IntraPredMode, the following applies:

–	If PredMode is equal to MODE_INTRA, log2(nW*nH) is equal to 4, and cIdx is equal to 0, the variables horizTrType and vertTrType are specified as Table 8-11 with IntraPredMode as input. [Ed. (WJ): DST is applied only for luma 4x4 block]

–	Otherwise, the variables horizTrType and vertTrType are set equal to 0.

Table Error! No text of specified style in document.-1 – Specification of horizTrType and vertTrType

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| horizTrType | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 |
| vertTrType | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 |

| IntraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| horizTrType | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| vertTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |

The constructed residual samples are derived as specified in the following ordered steps.

- Each (horizontal) row of scaled transform coefficients dij (i=0..nW−1, j=0..nH−1) is transformed to eij (i=0..nW−1, j=0..nH−1) by invoking the one-dimensional transformation process as specified in subclause 8.5.4.1 according to the width of the transform unit nW, with the (nW)x(nH) array d and the transform type variable horizTrType as the inputs and the output is the (nW)x(nH) array e.

- The intermediate sample values gij (i=0..nW-1, j=0..nH-1) are derived by

gij = Clip3( -32768, 32767, ( eij + 64 ) >> 7 )		(8-159)

- Each (vertical) column of the resulting matrix gij (i=0..nW−1, j=0..nH−1) is transformed to fij (i=0..nW−1, j=0..nH−1) by invoking the one-dimensional transformation process as specified in subclause 8.5.4.1 according to the height of the transform unit nH, with the (nW)x(nH) array e and the transform type variable vertTrType as the inputs and the output is the (nW)x(nH) array f.

- The variable shift is derived as follows:

–	If cIdx is equal to 0,

shift = 20 – BitDepthY		(8-150)

–	Otherwise,

shift = 20 – BitDepthC		(8-150)

- The residual sample value rij with i=0..(nW)−1, j=0..(nH)−1 is derived as follows.

rij = ( fij + (1 << ( shift – 1) ) ) >> shift		(8-160)

## In-loop filter process

[Ed: (WJ) overall process description need to be inserted]

### Deblocking filter process

A conditional filtering process shall be performed on a treeblock basis after the completion of the picture construction process prior to deblocking filter process for the entire decoded picture (as specified in subclauses XXX and YYY) [Ed.: (WJ) those subclauses seem not defined yet], with all treeblocks in a picture processed in order of increasing treeblock addresses.

Each treeblock is processed on a coding unit basis with the same order as decoding process. For each coding unit, vertical edges are filtered first, starting with the edge on the left-hand side of the coding unit proceeding through the edges towards the right-hand side of the coding unit in their geometrical order. Then, the horizontal edges are filtered starting with the edge on the top of the coding unit proceeding through the edges towards the bottom of the coding unit in their geometrical order.

Sample values above of the current coding unit that may have already been modified by the filtering of horizontal edges of deblocking filter process operation on previous coding unit shall be used as inputs to the deblocking filter process on the current coding unit and may be further modified during the filtering of the current coding unit. Sample values to the left of the current coding unit shall be used as inputs to the deblocking filter process on the current coding unit and may be further modified during the filtering of the current coding unit. Sample values to the left of the current coding unit may be modified by the filtering of vertical edge and may be further modified by the filtering of horizontal edges.

Sample values modified during filtering of vertical edges are used as input for the filtering of the horizontal edges. For sample values modified by both filtering of horizontal edges and filtering of vertical edges, filtering of horizontal edges is applied after filtering of vertical edges.

The deblocking filter process shall be applied to all prediction unit edges and transform unit edges of a picture, except edges at the boundary of the picture, any edges for which the deblocking filter process is disabled by disable_deblocking_filter_flag, any edges coinside with tile boundaries when loop_filter_across_tile_flag is equal to 0, and any edges coinside with slice boundaries when loop_filter_across_slice_flag is equal to 0. For the transform units and prediction units with edges smaller than 8 samples in either vertical or horizontal direction, only the edges lying on the 8x8 sample grid are filtered.

When disable_deblocking_filter_flag is not equal to 1, the deblocking filter process is invoked as the following ordered steps for each coding unit with the same order as decoding process.

- The coding unit size nS is set equal to 1 << log2CUSize.

- The variables FilterInternalEdgesFlag, FilterLeftCuEdgeFlag and FilterTopCuEdgeFlag are derived as follows.

- The variable FilterInternalEdges is set equal to 1.

- If the left boundary of current coding unit is the left boundary of the picture, or if the left boundary of current coding unit is the left boundary of the tile and loop_filter_across_tile_flag is equal to 0, or if the left boundary of current coding unit is the left boundary of the slice and loop_filter_across_slice_flag is equal to 0, the variable FilterLeftCuEdgeFlag is set equal to 0, otherwise set equal to 1.

- If the top boundary of current coding unit is the top boundary of the picture, or if the top boundary of current coding unit is the top boundary of the tile and loop_filter_across_tile_flag is equal to 0, or if the top boundary of current coding unit is the top boundary of the slice and loop_filter_across_slice_flag is equal to 0, the variable FilterTopCuEdgeFlag is set equal to 0, otherwise set equal to 1.

- All elements of two-dimensional array of size (nS)x(nS), horEdgeFlags and verEdgeFlags are initialized to zero.

- The derivation process of transform unit boundary specified in subclause 8.6.1.1 are invoked with the luma location ( xB, yB ) set equal to ( 0, 0 ), the transform unit width log2TrafoWidth set equal to log2CUSize, the transform unit height log2TrafoHeight set equal to log2CUSize and the variable trafoDepth set equal to 0 as the inputs and the modified horEdgeFlags and verEdgeFlags as outputs.

- The derivation process of prediction unit boundary specified in subclause 8.6.1.2 are invoked with the coding unit size log2CUSize and the prediction partition mode PartMode as inputs, and the modified horEdgeFlags and verEdgeFlags as outputs.

- The derivation process of the boundary filtering strength specified in subclause 8.6.1.3 is invoked with the luma location ( xC, yC ), the coding unit size log2CUSize, horEdgeFlags and verEdgeFlags as inputs and an array of size (2)x(nS)x(nS), bS as output.

- The filtering process for coding unit specified in subclause 8.6.1.4 are invoked with the luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture, the coding unit size log2CUSize and the array bS as inputs and the modified reconstructued picture as output.

#### Derivation process of transform unit boundary

Inputs of this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current coding unit,

–	a variable log2TrafoWidth specifying the width of the current block,

–	a variable log2TrafoHeight specifying the height of the current block,

–	a variable trafoDepth.

Outputs of this process are:

–	two-dimensional arrays of (nS)x(nS), horEdgeFlags and verEdgeFlags.

Depending on split_transform_flag[ xB ][ yB ][ trafoDepth ], the following applies:

–	If split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 1, the following ordered steps apply:

- The variables xB1, yB1, xB2, yB2, xB3 and yB3 are derived as follows.

  - If InterTUSplitDirection is equal to 2, the following applies.

  - The variable xB1 is set equal to xB + ( ( 1 << log2TrafoWidth ) >> 1 ).

  - The variable yB1 is set equal to yB.

  - The variable xB2 is set equal to xB.

  - The variable yB1 is set equal to yB + ( ( 1 << log2TrafoHeight ) >> 1 ).

  - The variable xB3 is set equal to xB1.

  - The variable yB3 is set equal to yB2.

  - The variable log2TrafoWidth1 is set equal to log2TrafoWidth − 1.

  - The variable log2TrafoHeight1 is set equal to log2TrafoHeight − 1.

  - Otherwise (InterTUSplitDirection is equal to 0 or 1), the following applies.

  - The variable xB1 is set equal to xB + ((1 << (log2TrafoWidth)) >> 2) * InterTUSplitDirection.

  - The variable yB1 is set equal to yB + ((1 << (log2TrafoHeight)) >> 2) * (1 − InterTUSplitDirection).

  - The variable xB2 is set equal to xB1 + ((1 << (log2TrafoWidth)) >> 2) * InterTUSplitDirection.

  - The variable yB2 is set equal to yB1 + ((1 << (log2TrafoHeight)) >> 2) * (1 − InterTUSplitDirection).

  - The variable xB3 is set equal to xB2 + ((1 << (log2TrafoWidth)) >> 2) * InterTUSplitDirection.

  - The variable yB3 is set equal to yB2 + ((1 << (log2TrafoHeight)) >> 2) * (1 − InterTUSplitDirection).

  - The variable log2TrafoWidth1 is set equal to (log2TrafoWidth − 2) * InterTUSplitDirection.

  - The variable log2TrafoHeight1 is set equal to (log2TrafoHeight − 2) * (1 − InterTUSplitDirection).

- The deriviation process of transform unit boundary as specified in this subclause is invoked with the luma location ( xB, yB ), the variable log2TrafoWidth set equal to log2TrafoWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1 and the variable trafoDepth1 set equal to trafoDepth + 1 as inputs and the outputs are the modified versions of two arrays, horEdgeFlags and verEdgeFlags.

- The deriviation process of transform unit boundary as specified in this subclause is invoked with the luma location ( xB1, yB1 ), the variable log2TrafoWidth set equal to log2TrafoSizeWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1 and the variable trafoDepth1 set equal to trafoDepth + 1 as inputs and the outputs are the modified versions of two arrays, horEdgeFlags and verEdgeFlags.

- The deriviation process of transform unit boundary as specified in this subclause is invoked with the luma location ( xB2, yB2 ), the variable log2TrafoWidth set equal to log2TrafoSizeWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1 and the variable trafoDepth1 set equal to trafoDepth + 1 as inputs and the outputs are the modified versions of two arrays, horEdgeFlags and verEdgeFlags.

- The deriviation process of transform unit boundary as specified in this subclause is invoked with the luma location ( xB3, yB3 ), the variable log2TrafoWidth1 set equal to log2TrafoSizeWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1 and the variable trafoDepth1 set equal to trafoDepth + 1 as inputs and the outputs are the modified versions of two arrays, horEdgeFlags and verEdgeFlags.

–	Otherwise (split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 0), the following applies:

- If yB is equal to zero, horEdgeFlags[ xB + k ][ yB ] is set equal to FilterTopCuEdgeFlag, otherwise horEdgeFlags[ xB + k ][ yB ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2TrafoWidth ) – 1.

- If xB is equal to zero, verEdgeFlags[ xB ][ yB + k ] is set equal to FilterLeftCuEdgeFlag, otherwise verEdgeFlags[ xB ][ yB + k ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2TrafoHeight ) – 1.

#### Derivation process of prediction unit boundary

Inputs of this process are:

–	a variable log2CUSize specifying the coding unit size,

–	a prediction partition mode PartMode.

Outputs of this process are:

–	two-dimensional arrays of (nS)x(nS), horEdgeFlags and verEdgeFlags.

Depending on PartMode, the following applies:

–	If PartMode is equal to PART_2NxN or PART_NxN, horEdgeFlags[ k ][ 1 << ( log2CUSize – 1 ) ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CUSize ) – 1.

–	If PartMode is equal to PART_Nx2N or PART_NxN, verEdgeFlags[ 1 << ( log2CUSize – 1 ) ][ k ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CUSize ) – 1.

–	If PartMode is equal to PART_2NxnU, horEdgeFlags[ k ][ (1 << ( log2CUSize – 1 )) – (1 << ( log2CUSize – 2 )) ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CUSize ) – 1.

–	If PartMode is equal to PART_2NxnD, horEdgeFlags[ k ][ (1 << ( log2CUSize – 1 )) + (1 << ( log2CUSize – 2 )) ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CUSize ) – 1.

–	If PartMode is equal to PART_nLx2N, verEdgeFlags[ (1 << ( log2CUSize – 1 )) – (1 << ( log2CUSize – 2 )) ][ k ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CUSize ) – 1.

–	If PartMode is equal to PART_nRx2N, verEdgeFlags[ (1 << ( log2CUSize – 1 )) + (1 << ( log2CUSize – 2 )) ][ k ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CUSize ) – 1.

#### Derivation process of boundary filtering strength

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a two-dimensional arrays of size (nS)x(nS), horEdgeFlags and verEdgeFlags.

Output of this process is an array of size (2)x(nS)x(nS), bS specifying the boundary filtering strength.

Let ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1 specify a set of edge sample locations where nE is set equal to ( ( 1 << log2CUSize ) >> 3 ), xE0 = 0, yE0 = 0, xEk+1 = xEk + 8 and yEj+1 = yEj + 8.

For ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1, the following applies.

- If horEdgeFlags[ xEk ][ yEj ] is equal to 1, for ( xEk+j, yEj ) with j=0, 1, the following applies.

- Set sample p0 = recPicture[ xC + xEk ][ yC + yEj – 1 ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 1.

- Otherwise, if verEdgeFlags[ xEk ][ yEj ] is equal to 1, for ( xEk, yEj +j) with j=0, 1, the following applies.

- Set sample p0 = recPicture[ xC + xEk – 1 ][ yC + yEj ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 0.

- Depending on the value of filterDir, the variable bS[ filterDir ][ xEk ][ yEj ] is derived as follows.

- If the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 2.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the block edge is also a transform unit edge and the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 1.

- The sample p0 or q0 is in a transform unit which contains non-zero transform coefficient level.

- Otherwise, if any of the following conditions are true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 1.

- The prediction unit containing sample p0 has different reference pictures or a different number of motion vectors with the prediction unit containing the sample q0.

NOTE – The determination of whether the reference pictures used for the two prediction are the same or different is based on which pictures are referenced, without regard to whether a prediction is formed using an index into list 0 or an index into list 1, and also without regard to whether or not the index position within a reference picture list is different or not.

- If horEdgeFlags[ xEk ][ yEj ] is equal to 1 and yC + yEj – 1 is smaller than YLCU, the following applies.

    - One motion vector is used to predict the prediction unit containing sample recPicture[ xL ][ yC + yEj – 1 ], one motion vector is used to predict the prediction unit containing sample q0, and the absolute difference between the horizontal or vertical component of the motion vector used is greater than or equal to 4 in units of quarter luma samples, where xL is set equal to ( ( ( xC + xEk ) >> Log2MinCUSize ) << Log2MinCUSize ) + ( ( ( xC + xEk ) >> Log2MinCUSize ) & 1 ) * ( MinPuSize * 2 – 1) and MinPuSize is set equal to 1<<(Log2MinCUSize – 1).

  - Otherwise, the following applies.

    - One motion vector is used to predict the prediction unit containing sample p0, one motion vector is used to predict the prediction unit containing sample q0, and the absolute difference between the horizontal or vertical component of the motion vector used is greater than or equal to 4 in units of quarter luma samples.

- [Ed.: (WJ) needs to be checked again whether this condition covers all 2-motion cases] Two motion vectors are used to predict the prediction unit containing sample p0, two motion vectors are used to predict the prediction unit containing sample q0, and at least one of the motion vector pairs corresponding the same reference pictures and the different boundary samples p0 and q0 satisfies the following condition:

    - The absolute difference between the horizontal or vertical component of a motion vector used in the prediction of the two prediction units is greater than or equal to 4 in units of quarter luma samples.

- Otherwise, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 0.

#### Filtering process for coding unit

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the coding unit size,

–	an array bS specifying the boundary filtering strength.

Output of this process is:

–	modified reconstruction of the picture.

The filtering process for luma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( log2CUSize – 3 ).

- All elements of the four-dimensional array of size (2)x(nD)x(nD), dEdge are initialized to zero.

- All elements of the three-dimensional array of size (2)x(nD)x(nD), bStrength ae initialized to zero.

- For xDk set equal to xC+( k << 3 ), k=0..nD – 1, the following applies:

- For yDm set equal to yC+( m << 3 ), m=0..nD – 1, the following ordered steps apply:

- Boundary filtering strength bSVer is derived as follows:

bSVer = Max( bS[ 0 ][ xDk ][ yDm + i ] ) for i = 0..7 	(8-428)

- bStrength[1][k][m] is set equal to bSVer.

- When bSVer is greater than 0, decision process for luma block edge in subclause 8.6.1.4.1 is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, and the boundary filtering strength bSVer as inputs, the decisions dEdge[0][1][k][m] and dEdge[1][1][k][m] as outputs.

- When bSVer is greater than 0, filtering process for luma block edge in subclause 8.6.1.4.2 is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, the boundary filtering strength bStrength[1][k][m], the decisions dEdge[0][1][k][m] and dEdge[1][1][k][m] as inputs and the modified luma picture buffer as outputs.

- For xDk set equal to xC + ( k << 3 ), k = 0..nD - 1, the following applies:

- For yDm set equal to yC+( m << 3 ), m=0..nD – 1, the following ordered steps apply:

- Boundary filtering strength bSHor is derived as follows:

bSHor = Max( bS[ 1 ][ xDk + i ][ yDm ] ) for i = 0..7 	(8-428)

- bStrength[0][k][m] is set equal to bSHor.

- When bSHor is greater than 0, decision process for luma block edge in subclause 8.6.1.4.1 is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, and the boundary filtering strength bSHor as inputs and the decisions dEdge[0][0][k][m] and dEdge[1][0][k][m] as outputs. [Ed. (WJ): comment from proponent – the decisions for the vertical filtering of the horizontal boundaries at the right hand side of a CU are being calculated based on the reconstructed signal and not based on the vertically filtered signal]

- For yDm set equal to yC+( m << 3 ), m=0..nD - 1, the following applies:

- For xDk set equal to xC+( k << 3 ), k=0..nD – 1, the following ordered steps apply:

    - If xDk is equal to 0, the parameter xPOS is set equal to 1. If xDk is equal to xC+( ( nD – 1) << 3 ) xPOS is set equal to 2. Otherwise xPOS is set to 0. [Ed. (WJ): comment from proponent – are horizontal edges at the right hand side of the picture really filtered vertically?]

    - dS[i] is set equal to dSample[0][m][ (k << 3) + i ] for i = 0..7.

    - When bSHor is greater than 0, filtering process for luma block edge in subclause 8.6.1.4.2 is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, the boundary filtering strength bStrength[0][k][m], the decisions dEdge[0][0][k][m] and dEdge[1][0][k][m], xPOS, dSL[m][], dEL[m], bSL[m], and tCL[m], as inputs and the modified luma picture buffer as output.

- The elements of the array of size (nD), dEL are set as follows. dEL[m] is set equal to dEdge[1][0][k][m].

- The elements of the array of size (nD), bSL are set as follows. bSL[m] is set equal to bStrength[0][k][m].

- The elements of the array of size (nD), tC are set as follows. tCL[m] is set equal to tc. [Ed. (WJ) – comment from proponent – dEp and dEq should be buffered / the stored signal with subscript “L” seems to be used at a wrong position due to CU scan order (z)]

The filtering process for chroma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( Max( log2CUSize, 4 ) – 4 ).

- For xDk set equal to ( xC / 2 )+( k << 3 ), k=0..nD – 1, the following applies:

- For yDm set equal to ( yC / 2)+( m << 2 ), m=0..nD*2 – 1, the following ordered steps apply:

- Boundary filtering strength bSVer is derived as follows:

bSVer = bS[ 0 ][ xDk*2 ][ yDm*2 ] 		(8-430)

- When bSVer is greater than 0, filtering process for chroma block edge in subclause 8.6.1.4.3 is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 1 and the boundary filtering strength bSVer as inputs and the modified chroma picture buffer as output.

- When bSVer is greater than 0, filtering process for chroma block edge in subclause 8.6.1.4.3 is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 2 and the boundary filtering strength bSVer as inputs and the modified chroma picture buffer as output.

- For yDm set equal to ( yC / 2 )+( m << 3 ), m=0..nD – 1, the following applies:

- For xDk set equal to ( xC / 2 )+( k << 2 ), k=0..nD*2 – 1, the following ordered steps apply:

- If xDk is equal to 0, the parameter xPOS is set equal to 1. If xDk is equal to xB+( ( nD*2 – 1) << 2 ) xPOS is set equal to 2. Otherwise xPOS is set to 0.

- Boundary filtering strength bSHor is derived as follows:

bSHor = bS[ 1 ][ xDk*2 ][ yDm*2 ] 		(8-431)

- When bSHor is greater than 0, filtering process for chroma block edge in subclause 8.6.1.4.3 is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 1 and the boundary filtering strength bSHor, xPOS, bSL[m] and tCL[m] as inputs and the modified chroma picture buffer as output.

- When bSHor is greater than 0, filtering process for chroma block edge in subclause 8.6.1.4.3 is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 2 and the boundary filtering strength bSHor, xPOS, bSL[m] and tCL[m] as inputs and the modified chroma picture buffer as output.

##### Decision process for luma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

Output of this process is:

–	variables dE[0], dE[1], dEp[0], dEp[1], dEq[0], and dEq[1] containing decisions,

This process is invoked only when bS is not equal to 0.

Let s’ represent the luma sample array recPictureL of the current picture.

A variable qPL is derived as follows:

qPL = ( ( QPY + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-447)

A variables β is specified as Table 8-13 with luma quantization parameter Clip3( 0, 51, qPL + ( beta_offset_div2 << 1 ) ) as input.

A variable tC is specified as Table 8-13 with luma quantization parameter Clip3( 0, 55, qPL + 2*( bS – 1) + ( tc_offset_div2 << 1) ) as input.

All elements of a one-dimensional array of size (4), dL are initialized to dL[0] = 0, dL[1] = 3, dL[2] = 4 and dL[3] = 7.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0, 3, 4, 7 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-447)

pi,k = s’[ xC + xB – i – 1, yC + yB + k ]		(8-448)

- The elements of one dimensional arrays of size (),dp, dq and dpq are derived as follows with i=0..3:

dp[i] = | p2,dL[i] – 2*p1,dL[i] + p0,dL[i] |		(8-449)

dq[i] = | q2,dL[i] – 2*q1,dL[i] + q0,dL[i] |		(8-449)

dpq[i] = dp[i] + dq[i]		(8-449)

- The elements of the one dimensional array d are derived as follows with i=0..1:

d[i] = dpq[ 2*i ] + dpq[ 2*i + 1 ]		(8-449)

- All elements of the one dimensional arrays dE, dEp and dEq are set equal to 0.

- For each j=0..1, if d[j] is less than β, the following ordered steps apply:

- For each sample location ( xC + xB, yC + yB + dL[k] ), k = 0..1, the decision process for a luma sample specified in subclause 8.6.1.4.4 is invoked with sample values pi,dL[k], qi,dL[k] with i = 0..3, the variables 2*dpq[ 2*j + k ], β and tC as inputs and a decision dSam[k] as output.

- The variable dE[j] is set equal to 1.

- If dSam[0] is equal to 1 and dSam[1] is equal to 1, the variable dE[j] is set equal to 2.

- If dp[ 2*j ] + dp[ 2*j + 1 ] is less than ( β + ( β >> 1 ) ) >> 3, the variable dEp[j] is set equal to 1.

- If dq[ 2*j ] + dq[ 2*j + 1 ] is less than ( β + ( β >> 1 ) ) >> 3, the variable dEq[j] is set equal to 1.

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0, 3, 4, 7 are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-435)

pi,k = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-436)

- The elements of one dimensional array of size (4), dp, dq and dpq are derived as follows with i=0..3:

dp[i] = | p2,dL[i] – 2*p1,dL[i] + p0,dL[i] |		(8-437)

dq[i] = | q2,dL[i] – 2*q1,dL[i] + q0,dL[i] |		(8-437)

dpq[i] = dp[i] + dq[i]		(8-437)

- The elements of the one dimensional array d are derived as follows with i=0..1:

d[i] = dpq[ 2*i ] + dpq[ 2*i + 1 ]		(8-449)

- All elements of the one dimensional arrays dE, dEp and dEq are set equal to 0.

- For each j=0..1, if d[j] is less than β, the following ordered steps apply:

- For each sample location ( xC + xB + dL[k] + 4 * j, yC + yB ), k = 0..1, the decision process for a luma sample specified in subclause 8.6.1.4.4 is invoked with sample values pi,dL[k], qi,dL[k] with i = 0..3, the variables 2*dpq[ 2*j + k ], β and tC as inputs and a decision dSam[k] as output.

- The variable dE[j] is set equal to 1.

- If dSam[0] is equal to 1 and dSam[1] is equal to 1, the variable dE[j] is set equal to 2.

- If dp[ 2*j ] + dp[ 2*j + 1 ] is less than ( β + ( β >> 1 ) ) >> 3, the variable dEp[j] is set equal to 1.

- If dq[ 2*j ] + dq[ 2*j + 1 ] is less than ( β + ( β >> 1 ) ) >> 3, the variable dEq[j] is set equal to 1.

##### Filtering process for luma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

–	variables dE[0], dE[1], dEp[0], dEp[1], dEq[0] and dEq[1] containing decisions,

–	a variable bSL,

–	a variable tCL,

Output of this process is:

–	modified reconstruction of the picture.

This process is invoked only when bS is not equal to 0.

Let s’ represent the luma sample array recPictureL of the current picture.

Depending on pcm_flag, a variable β is specified as follows:

–	If pcm_flag is equal to 1, the variables β is specified as Table 8-11 with luma quantization parameter 0 as input.

–	Otherwise, the variables β is specified as Table 8-11 with luma quantization parameter qPL as input.

A variable tC is specified as Table 8-11 with luma quantization parameter Clip3( 0, 55, qPL + 2*(bS – 1) + ( tc_offset_div2 << 1 ) ) as input.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0..7 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-432)

pi,k = s’[ xC + xB – i – 1, yC + yB + k ]		(8-433)

- For each m=0..1, if dE[m] is not equal to 0, for each sample location ( xC + xB, yC + yB + k + 4*m ), k = 0..3, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause 8.6.1.4.5 is invoked with sample values pi,k, qi,k with i = 0..3, the decision dE[m], variables dEp[m] and dEq[m], the variable tC as inputs and the number of filtered samples nDp and nDq from each side of the block boundary, and the filtered sample values pi’ and qj’ as outputs.

- The filtered sample values pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +j, yC + yB + k + 4*m ] = qj’		(8-435)

s’[ xC + xB – i – 1, yC + yB + k + 4*m ] = pi’	(8-436)

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- If xPOS is equal to 1, the parameters ks and ke are set to -3 and 4 respectively. If xPOS is equal to 2, the parameters ks and ke are set to 0 and 4 respectively. Otherwise ks and ke are set to 0 and 7 respectively.

- The sample values pi,k and qi,k with i = 0..3 and k = ks..ke are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-437)

pi,k = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-438)

- If xPOS is less than 0 [Ed. (WJ): maybe wrong condition] and dEL is not equal to 0, for each sample location ( xC + xB + k, yC + yB ), k = -3..-1, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause 8.6.1.4.5 is invoked with sample values pi,k, qi,k with i = 0..3, decision arrays dEL, dEpL and dEqL [Ed. (WJ): dEpL and dEqL do not exist. Needs to be stored], the variable tCL as inputs and the number of filtered samples nDp and nDq from each side of the block boundary and the filtered sample values pi’ and qj’ as outputs.

- The filtered sample values pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB + j ] = qj’		(8-459)

s’[ xC + xB +k, yC + yB – i – 1 ] = pi’		(8-460)

- For each m=0..1, if dE[m] is not equal to 0, for each sample location ( xC + xB + k + 4*m, yC + yB ), k = 0.. ke-4 , the following ordered steps apply:

- The filtering process for a luma sample specified in subclause 8.6.1.4.5 is invoked with sample values pi,k, qi,k with i = 0..3, decision dE[m], variables dEp[m] and dEq[m], the variable tC as inputs and the number of filtered samples nDp and nDq from each side of the block boundary and the filtered sample values pi’ and qj’ as outputs.

- The filtered sample values pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB + k + 4*m, yC + yB + j ] = qj’		(8-461)

s’[ xC + xB + k + 4*m, yC + yB – i – 1 ] = pi’	(8-462)

##### Filtering process for chroma block edge

[Ed.: (WJ) cIdx cannot be 0 here]

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left chroma sample of the current block relative to the top left chroma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

–	a variable cIdx specifying the chroma component index.

–	a variable xPOS,

–	a variable bSL,

–	a variable tCL

Output of this process is:

–	modified reconstruction of the picture.

This process is invoked only when bS is not equal to 0.

Let s’ be a variable specifying chroma sample array which is derived as follows.

–	If cIdx is equal to 1, s’ represents the chroma sample array recPictureCb of the current picture.

–	Otherwise (cIdx is equal to 2), s’ represents the chroma sample array recPictureCr of the current picture.

A variable qPL is derived as follows:

qPL = ( ( QPY + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-447)

A variable tC is specified as Table 8-13 with luma quantization parameter Clip3( 0, 55, qPL + 2*(bS – 1) + (tc_offset_div2 << 1 ) ) as input.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, for each sample location ( xC + xB, yC + yB + k ), k = 0..3, the following ordered steps apply:

- The sample values pi and qi with i = 0..1 are derived as follows:

qi = s’[ xC + xB +i, yC + yB + k ]		(8-442)

pi = s’[ xC + xB – i – 1, yC + yB + k ]		(8-443)

- The following ordered steps apply:

- The filtering process for a sample specified in subclause 8.6.1.4.6 is invoked with sample values pi, qi, with i = 0..1, the boundary filtering strength bS and the variable tC as inputs and the filtered sample values p0’ and q0’ as outputs.

- The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB , yC + yB + k ] = q0’		(8-444)

s’[ xC + xB – 1, yC + yB + k ] = p0’		(8-445)

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- If xPOS is equal to 1, the parameters ks and ke are set to -1 and 2 respectively. If xD is equal to 2, the parameters ks and ke are set to 0 and 2 respectively. Otherwise ks and ke are set to 0 and 3 respectively.

- The sample values pi and qi with i = 0..1 and k = ks..ke are derived as follows:

qi = s’[ xC + xB +k, yC + yB + i ]		(8-446)

pi = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-447)

- If xPOS is less than 0, and if bSL, is greater than 1, for each sample location ( xC + xB - 1, yC + yB ), the following ordered steps apply:

- The filtering process for a sample specified in subclause 8.6.1.4.6 is invoked with sample values pi, qi, with i = 0..1, the boundary filtering strength bSL and the variable tCL as inputs and the filtered sample values p0’ and q0’ as outputs.

- The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB ] = q0’		(8-452)

s’[ xC + xB +k, yC + yB – i – 1 ] = pi’		(8-445)

- If bS is greater than 1, for each sample location ( xC + xB + k, yC + yB ), k = 0.. ke, the following ordered steps apply:

- The filtering process for a sample specified in subclause 8.6.1.4.6 is invoked with sample values pi, qi, with i = 0..1, the boundary filtering strength bS and the variable tC as inputs and the filtered sample values p0’ and q0’ as outputs.

- The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB ] = q0’		(8-448)

s’[ xC + xB +k, yC + yB – 1 ] = p0’		(8-449)

##### Decision process for a luma sample

[Ed: (WJ) no filtering when bS is equal to 0]

Inputs of this process are:

–	sample values, pi and qi with i = 0..2,

–	variables d, β and tC.

Output of this process is:

–	a variable dSam containing a decision

The variable dSam is specified as follows:

–	If d is less than ( β >> 2 ), | p3 – p0 | + | q0 – q3 | is less than ( β >> 3 ) and | p0 – q0 | is less than ( 5*tC + 1 ) >> 1, dSam is set equal to 1.

–	Otherwise, dSam is set equal to 0.

##### Filtering process for a luma sample

Inputs of this process are:

–	sample values, pi and qi with i = 0..3,

–	variables dEp1 and dEq1 containing decisions to filter pixels p1 and q1 respectively,

–	a variable tC.Output of this process is:

–	number of filtered samples nDp and nDq,

–	filtered sample values, pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1

Depending on dE, the following applies:

–	When the variable dE is equal to 2, the following strong filtering applies while nDp and nDq are set equal to 3:

p0’ = ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 		(8-450)

p1’ = ( p2 + p1 + p0 + q0 + 2 ) >> 2 		(8-451)

p2’ = ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 		(8-452)

q0’ = ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 		(8-453)

q1’ = ( p0 + q0 + q1 + q2 + 2 ) >> 2 		(8-454)

q2’ = ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 		(8-455)

–	Otherwise, the following weak filtering applies while nDp and nDq are set equal to 0:

= ( 9 * ( q0 –  p0 ) – 3 * ( q1 – p1 ) + 8 ) >> 4		(8-456)

  - When abs() is less than tc*10, the following ordered steps apply:

    - The filtered sample values p0’ and q0’ are specified as follows:

= Clip3( -tc, tc,  )		(8-456)

p0’ = Clip1Y( p0 +  )		(8-457)

q0’ = Clip1Y( q0 -  )		(8-458)

    - If dEp1 is equal to 1, the filtered sample value pi’ is specified as follows:

p = Clip3( -(tc >> 1), tc >> 1, ( ( ( p2 + p0 + 1 ) >> 1 ) – p1 +  ) >>1 )	(8-458)

pi’ = Clip1Y( p1 + p )		(8-458)

    - If dEq1 is equal to 1, the filtered sample value qi’ is specified as follows:

q = Clip3( -(tc >> 1), tc >> 1, ( ( ( q2 + q0 + 1 ) >> 1 ) – q1 –  ) >>1 )	(8-458)

qi’ = Clip1Y( q1 + q )		(8-458)

    - nDp is set equal to dEp1+1 and nDq is set equal to dEq1+1.

Each of the filtered sample values, pi’ with i = 0..nDp-1, is substituted by the corresponding input sample value pi if all of the following conditions are true.

–	pi is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

Similarly, Each of the filtered sample values, pi’ with i = 0..nDp-1, is substituted by the corresponding input sample value pi if QP’Y is equal to 0.

Similarly, each of the filtered sample values, qj’ with j = 0..nDq-1, is substituted by the corresponding input sample value qj if all of the following conditions are true.

–	qj is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

Similarly, each of the filtered sample values, qj’ with j = 0..nDq-1, is substituted by the corresponding input sample value qj if if QP’Y is equal to 0.

[Ed. (WJ): for PCM case, deblocking filter applies first and the filtered pixels are restored. Rather than this, it’s better to skip the filtering itself for PCM samples since first filtering is actually not needed.]

Table Error! No text of specified style in document.-2 – Derivation of threshold variables β and tC from input Q

| Q | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| β | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 | 8 |
| tC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| Q | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 |
| β | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 |
| tC | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 |
| Q | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 |  |
| β | 38 | 40 | 42 | 44 | 46 | 48 | 50 | 52 | 54 | 56 | 58 | 60 | 62 | 64 | 64 | 64 | 64 | 64 |  |
| tC | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 9 | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 | 14 | 14 |  |

##### Filtering process for a chroma sample

[Ed: (WJ) no filtering when bS is equal or less than 1]

Inputs of this process are:

–	sample values, pi and qi with i = 0..1,

–	a variable tC.Output of this process is:

–	The filtered sample values, p0’ and q0’.

The filtered sample values p0’ and q0’ are derived by

= Clip3( -tC, tC, ( ( ( ( q0 – p0 ) << 2 ) + p1 – q1 + 4 ) >> 3 ) )	(8-461)

p0’ = Clip1C( p0 +  )		(8-462)

q0’ = Clip1C( q0 -  )		(8-463)

The filtered sample value, p0’ is substituted by the corresponding input sample value p0 if all of the following conditions are true.

–	p0 is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

Similarly, the filtered sample value, p0’ is substituted by the corresponding input sample value p0 if QP’Y is equal to 0

Similarly, the filtered sample value, q0’ is substituted by the corresponding input sample value q0 if all of the following conditions are true.

–	q0 is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

Similarly, the filtered sample value, q0’ is substituted by the corresponding input sample value q0 if QP’Y is equal to 0.

[Ed. (WJ): for PCM case, deblocking filter applies first and the filtered pixels are restored. Rather than this, it’s better to skip the filtering itself for PCM samples since first filtering is actually not needed.]

### Sample adaptive offset process

A sample adaptive offset process shall be conditionally performed after the completion of the deblocking filter process for the decoded picture. This process is invoked when both sample_adaptive_offset_enabled_flag and aps_sample_adaptive_offset_flag are equal to 1.

This process is performed on a region basis which is aligned with the LCU boundaries after the completion of the picture construction process prior to sample adaptive offset process for the entire decoded process.

The sample adaptive offset process is invoked with the region equal to entire picture here.

Depending on aps_sample_adaptive_offset_flag, sao_flag_cb and sao_flag_cr, the following applies:

–	When aps_sample_adaptive_offset_flag is equal to 1, the following applies:

  - The sample adaptive offset process for luma region as specified in subclause 8.6.2.1 is invoked with the component index set equal to 0, the region index ( 0, 0 ), the largest CU location ( 0, 0 ), the variable saoDepth set equal to 0, the variable saoWidthInLCUs set equal to PicWidthInLCUs, and the variable saoHeightInLCUs set equal to PicHeightInLCUs, and the modified luma picture buffer of current region as output.

  - When sao_flag_cb is equal to 1, the sample adaptive offset process for color component Cb as specified in subclause 8.6.2.1 is invoked with the component index set equal to 1, the region index ( 0, 0 ), the largest CU location ( 0, 0 ), the variable saoDepth set equal to 0, the variable saoWidthInLCUs set equal to PicWidthInLCUs, and the variable saoHeightInLCUs set equal to PicHeightInLCUs, and the modified chroma picture buffer for Cb of current region as output.

  - If sao_flag_cr is equal to 1, the sample adaptive offset process for color component Cr as specified in subclause 8.6.2.1 is invoked with the component index set equal to 2, the region index ( 0, 0 ), the largest CU location ( 0, 0 ), the variable saoDepth set equal to 0, the variable saoWidthInLCUs set equal to PicWidthInLCUs, and the variable saoHeightInLCUs set equal to PicHeightInLCUs, and the modified chroma picture buffer for Cr of current region as output.

#### Modification process for sample adaptive offset

Inputs to this process are:

–	a variable cIdx specifying chroma component index,

–	a pair of variables ( rx, ry ) specifying the region index,

–	a sample location ( x0, y0 ) specifying the top-left sample of the current largest coding unit,

–	a variable saoDepth specifying the split depth of the region,

–	a variable saoWidthInLCUs specifying the number of largest CUs in width of the current region

–	a variable saoHeightInLCUs specifying the number of largest CUs in height of the current region.

Output of this process is a modified picture buffer for the chroma component cIdx.

Depending on sao_split_flag[ cIdx ][ saoDepth ][ rx ][ ry ], the following applies:

–	If sao_split_flag[cIdx][saoDepth][rx][ry] is equal to 1 and saoDepth is smaller than SaoMaxDepth, the following ordered stesps apply:

- Variable nS is set equal to ( 1 << Log2MaxCUSize ).

- Variables saoWidthInLCU1, saoWidthInLCU2, saoHeightInLCU1, saoHeightInLCU2, x1, y1, x2, and y2 are derived as follows:

saoWidthInLCU1 = saoWidthInLCUs >> 1		(8-463)
	saoWidthInLCU2 = saoWidthInLCUs – saoWidthInLCU1	(8-463)
	saoHeightInLCU1 = saoHeightInLCUs >> 1		(8-463)
	saoHeightInLCU2 = saoHeightInLCUs – saoHeightInLCU1	(8-463)

x1 = x0		(8-463)
	y1 = y0		(8-463)
	x2 = x1 + saoWidthInLCU1 * nS		(8-463)
	y2 = y1 + saoHeightInLCU1 * nS		(8-463)

- The sample adaptive offset process for region as specified in this subclause is invoked with the component index cIdx, the region index ( 2*rx, 2*ry ), the top-left sample position of largest CU ( x0, y0 ), the variable saoDepth set equal to ( saoDepth+1 ), the variable saoWidthInLCUs set equal to saoWidthInLCU1, and the variable saoHeightInLCUs set equal to saoHeightInLCU1.

- The sample adaptive offset process for region as specified in this subclause is invoked with the component index cIdx, the region index ( 2*rx+1, 2*ry ), the top-left sample position of largest CU ( x1, y0 ), the variable saoDepth set equal to ( saoDepth+1 ), the variable saoWidthInLCUs set equal to saoWidthInLCU2, and the variable saoHeightInLCUs set equal to saoHeightInLCU1.

- The sample adaptive offset process for region as specified in this subclause is invoked with the component index cIdx, the region index ( 2*rx, 2*ry+1 ), the top-left sample position of largest CU ( x0, y1 ), the variable saoDepth set equal to ( saoDepth+1 ), the variable saoWidthInLCUs set equal to saoWidthInLCU1, and the variable saoHeightInLCUs set equal to saoHeightInLCU2.

- The sample adaptive offset process for region as specified in this subclause is invoked with the component index cIdx, the region index ( 2*rx+1, 2*ry+1 ), the top-left sample position of largest CU ( x1, y1 ), the variable saoDepth set equal to ( saoDepth+1 ), the variable saoWidthInLCUs set equal to saoWidthInLCU2, and the variable saoHeightInLCUs set equal to saoHeightInLCU2.

- Otherwise, (sao_split_flag[ cIdx ][ saoDepth ][ rx ][ ry ] is equal to 0), the following ordered steps apply:

- Variable nS is set equal to ( 1 << Log2MaxCUSize ).

- Modification process for samples in a largest coding unit covering ( iC, jC ) with iC = x0 + k*nS for k = 0..saoWidthInLCU – 1 and jC = y0 + j*nS for j = 0..saoHeightInLCU – 1 specified in subclause 8.6.2.1.1 is invoked with the component index cIdx, the top-left sample of current largest coding unit ( iC, jC ), the region index ( rx, ry ), the split depth saoDepth, the offset array saoValueArray set equal to SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ] and the largest coding unit size nS as inputs and the output is the modified picture buffer of chroma component cIdx.

##### Modification process for luma and chroma samples

Inputs to this process are:

–	a variable cIdx specifying chroma component index,

–	a sample position ( xC, yC ),

–	a pair of variables ( rx, ry ) specifying the region index,

–	a variable saoDepth specifying the split depth of the region,

–	an array saoValueArray specifying offset values,

–	a block size nS.

Output of this process is a modified picture buffer for the chroma component cIdx.

Let recSaoPicture represents the processed sample array of the current picture of chroma component cIdx and saoTypeIdx is set equal to sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ].

Variable bitDepth is set equal to BitDepthY if cIdx is equal to 0, otherwise, set equal to BitDepthC.

Depending on the value of saoTypeIdx, the following applies:

–	If saoTypeIdx is equal to one of the values of 1, 2, 3 or 4, the following ordered steps apply:

- Arrays hPos[2] and vPos[2] are specified in Table 8-16.

- A variable edgeIdx is specified as

edgeIdx = 2 + ∑k( Sign( recPicture[ xC + i, yC + j ] –

recPicture[ xC + i + hPos[ k ], yC + j + vPos[ k ] ] ) ) with k = 0..1	(8-463)

- The reconstructed picture buffer is modified as

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] + saoValueArray[ edgeTable[ edgeIdx ] ]	(8-463)

with i = 0..nS-1 and j = 0..nS-1 where 	edgeTable[5] = { 1, 2, 0, 3, 4}.

–	Otherwise, if saoTypeIdx is equal to one of the values of 5 or 6, the following ordered steps applies:

- A variable bandShift is set equal to BitDepthY – 5 if cIdx is equal to 0, otherwise, set equal to BitDepthC – 5.

- The reconstructed picture buffer is modified as

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] +

saoValueArray[ bandTable[ saoTypeIdx – 5 ][ bandIdx ] ]	(8-463)

with i = 0..nS-1 and j = 0..nS-1 where bandIdx is set equal to ( recPicture[ xC + i, yC + j ] >> bandShift ) and bandTable is specified in Table 8-17.

–	Otherwise (sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] is equal to 0), the following applies:

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] with i = 0..nS-1 and j = 0..nS-1	(8-463)

[Ed. (WJ): copy operation is necessary to use recSaoPicture later.]

Table Error! No text of specified style in document.-3 – Specification of hPos[2] and vPos[2] according to the type of sample adaptive offset process

| sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| hPos[0] | -1 | 0 | -1 | 1 |
| hPos[1] | 1 | 0 | 1 | -1 |
| vPos[0] | 0 | -1 | -1 | 1 |
| vPos[1] | 0 | 1 | 1 | -1 |

Table Error! No text of specified style in document.-4 – Specification of array bandTable according to bandIdx

| bandIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandTable[0][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| bandTable[1][bandIdx] | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandIdx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| bandTable[0][bandIdx] | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[1][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |

### Adaptive loop filter process

An adaptive loop filtering process shall be conditionally performed on a treeblock basis after the completion of the sample adaptive offset process for the entire decoded picture, with all treeblocks in a picture processed in order of increasing treeblock addresses.

Each treeblock is processed on a coding unit basis with the same order as decoding process.

This process is invoked when both adaptive_loop_filter_enabled_flag and aps_adaptive_loop_filter_flag are equal to 1.

This process is performed on a coding unit basis after the completion of the slice construction process prior to adaptive loop filter process for the entire decoded slice, with all coding units in a slice processed in order of coding unit scan order.

Filter coefficients cL for luma samples and cC for chroma samples are derived by invoking the process specified in subclause 8.6.2.1.Filter index array fIdx for luma samples are derived by invoking the process specified in subclause 8.6.2.2 with the luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture and the coding unit size log2CUSize as inputs.

When interpreting the luma samples of the coding unit as to be filtered, depending on pcm_flag, pcm_loop_filter_disable_flag and alf_cu_control_flag, the following applies.

- If pcm_loop_filter_disable_flag is equal to 1 and pcm_flag is equal to 1, the luma samples are not filtered.

- Otherwise, if QP’Y is equal to 0, the luma smaples are not filtered,

- Otherwise, if alf_cu_control_flag is equal to 0, the luma samples are filtered,

- Otherwise (alf_cu_control_flag is equal to 1), if AlfCuFlag[ xC ][ yC ] is equal to 1 where xC and yC are the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture, the luma samples are filtered.

- Otherwise, the luma samples are not filtered.

When interpreting the chroma samples of the coding unit as to be filtered, depending on pcm_flag, pcm_loop_filter_disable_flag and alf_chroma_idc, the following applies.

- If pcm_loop_filter_disable_flag is equal to 1 and pcm_flag is equal to 1, both chroma samples are not filtered.

- Otherwise, if QP’Y is equal to 0, both chroma smaples are not filtered,

- Otherwise, if alf_chroma_idc is equal to 1, the Cr samples are filtered.

- Otherwise, if alf_chroma_idc is equal to 2, the Cb samples are filtered.

- Otherwise, if alf_chroma_idc is equal to 3, both chroma samples are filtered.

- Otherwise (alf_chroma_idc is equal to 0), both chroma samples are not filtered.

For the luma samples of the coding unit interpreted as to be filtered, the filtering process for luma samples specified in subclause 8.6.2.3 is invoked with the luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture, the coding unit size log2CUSize, and the filter index array fIdx as inputs and the output is the modified filtered picture, recFiltPictureL.

For the chroma samples of the coding unit interpreted as to be filtered, the filtering process for chroma samples specified in subclause 8.6.2.4 is invoked with the chroma location ( xC/2, yC/2 ) specifying the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture, the coding unit size log2CUSize-1 and the chroma component index cIdx equal to 1 as inputs and the output is the modified filtered picture, recFiltPictureCb.

For the chroma samples of the coding unit interpreted as to be filtered, the filtering process for chroma samples specified in subclause 8.6.2.4 is invoked with the chroma location ( xC/2, yC/2 ) specifying the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture, the coding unit size log2CUSize-1 and the chroma component index cIdx equal to 2 as inputs and the output is the modified filtered picture, recFiltPictureCr.

[Ed.: (WJ) recPicture: deblocked/output picture and recFiltPicture: ALFed picture]

[Ed.: (WJ) depending aps_adaptive_loop_filter_flag, recFiltPicture should be copied to recPicture in subclause 8.6]