| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, SE, 11-20 July 2012 | Document: JCTVC-J0119 |
| --- | --- |

| Title: | AHG13: On reference picture list modification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Ying Chen Ye-Kui Wang 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-658-5804 aramasub@qualcomm.com 1-858-845-6589 cheny@qualcomm.com 1-858-651-8345 yekuiw@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this proposal, shortcomings of the reference picture list modification (RPLM) design in the latest HEVC draft spec (WD7) are discussed. Based on the discussion, a modified RPLM design based on the one in HEVC WD5 is proposed. It is reported by the proponents that, for test cases 2.8 and 3.5 in the common test conditions for reference picture marking and list construction proposals in JCTVC-H0725, 24% bit reduction of RPLM bits was achieved for the low-delay configuration compared to the RPLM method in HEVC WD7, and the performance is the same for the random access configuration. It is further that the proposed RPLM method, when applied to HEVC-based 3DV, outperforms the RPLM method in HEVC WD7, when applied to 3DV, with 34% bit reduction on average for non-base views under the 3DV common test conditions.

# Shortcomings of the RPLM method in HEVC WD7

The current design in HEVC WD7 has the following problems compared with the original design in HEVC WD5.

The reference picture list modification (RPLM) design in HEVC WD5 follows the similar fashion as in AVC, so that the number of commands can be much less than the number of commands needed for the RPLM in the current draft, especially for the following examples:

- Example 1: Five pictures in the initial list are to be put in the final list, and only the first two entries in the initial list need to be swapped to get the final list. For the AVC design, only one command is needed while for the design in HEVC WD7, five commands are needed.

- Example 2: Five pictures are to be put in the final list. A picture at the end of the initial list or not included in the initial list needs to be added at the first entry of the final list, the last entry is removed and the position of the other entries have their positions shifted by one. This is a typical case, e.g., in MVC.

Note that if in the WD5 design, if list_modification_idc and ref_pic_set_idx are both coded as u(v) instead of ue(v), it is more efficient than WD7 in LD case (16% bit-saving).

# Proposal

We proposed two solutions for reference picture list construction, a simplified one and an advanced one.

For the simplified solution, it is similar to WD5 design, thus solves the problems mentioned above.

## Simple solution

The proposed syntax, semantics and decoding process changes are as follows.

### Reference picture list modification syntax

| ref_pic_list_modification( ) { | Descriptor |
| --- | --- |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| ref_pic_list_modification_flag_l0 | u(1) |
| if( ref_pic_list_modification_flag_l0 ) |  |
| do { |  |
| ref_pic_list_modification_idc | u(v) |
| if( ref_pic_list_modification_idc != 3 ) |  |
| ref_pic_set_idx | u(v) |
| } while( ref_pic_list_modification_idc != 3 ) |  |
| } |  |
| if( slice_type = = B ) { |  |
| ref_pic_list_modification_flag_l1 | u(1) |
| if( ref_pic_list_modification_flag_l1 ) |  |
| do { |  |
| ref_pic_list_modification_idc | u(v) |
| if( ref_pic_list_modification_idc != 3 ) |  |
| ref_pic_set_idx | u(v) |
| } while( ref_pic_list_modification_idc != 3 ) |  |
| } |  |
| } |  |

### Semantics

The syntax elements ref_pic_list_modification_idc and ref_pic_set_idx specify the change from the initial reference picture lists to the reference picture lists to be used for decoding the slice.

ref_pic_list_modification_flag_l0 equal to 1 specifies that the syntax element ref_pic_list_modification_idc is present for specifying reference picture list 0. ref_pic_list_modification_flag_l0 equal to 0 specifies that this syntax element is not present.

When ref_pic_list_modification_flag_l0 is equal to 1, the number of times that ref_pic_list_modification_idc is not equal to 3 following ref_pic_list_modification_flag_l0 shall not exceed num_ref_idx_l0_active_minus1 + 1.

ref_pic_list_modification_flag_l1 equal to 1 specifies that the syntax element ref_pic_list_modification_idc is present for specifying reference picture list 1. ref_pic_list_modification_flag_l1 equal to 0 specifies that this syntax element is not present.

When ref_pic_list_modification_flag_l1 is equal to 1, the number of times that ref_pic_list_modification_idc is not equal to 3 following ref_pic_list_modification_flag_l1 shall not exceed num_ref_idx_l1_active_minus1 + 1.

ref_pic_list_modification_idc together with ref_pic_set_idx specifies which of the reference pictures are re-mapped. The values of ref_pic_list_modification_idc are specified in Table 7-4. The value of the first ref_pic_list_modification_idc that follows immediately after ref_pic_list_modification_flag_l0 or ref_pic_list_modification_flag_l1 shall not be equal to 3. ref_pic_list_modification_idc is in the range of 0 to N + 1, inclusive, wherein N is the number of RPS subset allowed for a coded video sequence.

Table 7-6 – ref_pic_list_modification_idc operations for modification of reference picture lists

| ref_pic_list_modification_idc | modification specified |
| --- | --- |
| 0 | For list 0: ref_pic_set_idx is present and corresponds to an index to RefPicSetStCurrBefore; For list 1: ref_pic_set_idx is present and corresponds to an index to RefPicSetStCurrAfter |
| 1 | For list 0: ref_pic_set_idx is present and corresponds to an index to RefPicSetStCurrAfter; For list 1: ref_pic_set_idx is present and corresponds to an index to RefPicSetStCurrBefore |
| 2 | ref_pic_set_idx is present and corresponds to an index to RefPicSetLtCurr |
| 3 | End loop for modification of the initial reference picture list |

ref_pic_set_idx specifies the index, to RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr, of the reference picture being moved to the current index in the reference picture list. The value of ref_pic_set_idx shall be in the range of 0 to max_num_ref_frames - 1, inclusive.

### Decoding process for reference picture list modification

8.2.4.3	Modification process for reference picture lists

After the invocation of this process, there shall be no reference pictures with greater temporal_id than the current slice included in the output RefPicListX (with X being equal to 0 or 1).

For X equal to either 0 or 1, when ref_pic_list_modification_flag_lX is equal to 1, the following applies:

1.	Let refIdxLX be an index into the reference picture list RefPicListX. It is initially set equal to 0.

2.	The corresponding syntax elements ref_pic_list_modification_idc are processed in the order they occur in the bitstream. For each of these syntax elements, the following applies.

- If ref_pic_list_modification_idc is not equal to 3, the process specified in subclause 8.2.4.3.1 is invoked with refIdxLX as input, and the output is assigned to refIdxLX.

- Otherwise (ref_pic_list_modification_idc is equal to 3), the modification process for reference picture list RefPicListX is finished.

8.2.4.3.1	Modification process of reference picture lists for a reference picture

Input to this process is an index refIdxLX (with X being 0 or 1).

Output of this process is an incremented index refIdxLX.

RefPicSetStCurr[ 0 ] = (X  = =  0) ? RefPicSetStCurrBefore : RefPicSetStCurrAfter
	RefPicSetStCurr[ 1 ] = (X  = =  0) ? RefPicSetStCurrAfter : RefPicSetStCurrBefore
	rplmIdc = ref_pic_list_modification_idc
	curRefPicSet = rplmIdc > 1 ? RefPicSetLtCurr : RefPicSetStCurr[ rplmIdc ]

The variable pocLX is derived as follows.

pocLX = curRefPicSet[ ref_pic_set_idx ] 							(8 X)

The following procedure is conducted to place the picture picR with PicOrderCnt( picR ) equal to pocLX into the index position refIdxLX, shift the position of any other remaining pictures to later in the list, and increment the value of refIdxLX.

for( cIdx = num_ref_idx_lX_active_minus1 + 1; cIdx > refIdxLX; cIdx− − )
		RefPicListX[ cIdx ] = RefPicListX[ cIdx – 1 ]
	RefPicListX[ refIdxLX++ ] = pocLX
	nIdx = refIdxLX										(8 X+1)
	for( cIdx = refIdxLX; cIdx <= num_ref_idx_lX_active_minus1 + 1; cIdx++ ) 
		if( PicOrderCnt( RefPicListX[ cIdx ] ) != pocLX )
			RefPicListX[ nIdx++ ] = RefPicListX[ cIdx ]

## Advanced solution

This solution is technically identical to JCTVC-I0348.

### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( lists_modification_present_flag ) { |  |
| ref_pic_list_modification( ) |  |
| if( slice_type != 2 ) |  |
| ref_pic_list_modification( 0 ) |  |
| if( slice_type = = 1 ) |  |
| ref_pic_list_modification( 1 ) |  |
| } |  |
| … |  |
| } |  |

### Reference picture list modification syntax

| ref_pic_list_modification( X ) { |  |
| --- | --- |
| ref_pic_list_modification_flag_lX | u(1) |
| if( ref_pic_list_modification_flag_lX ) { |  |
| start_pos_lX | u(v) |
| if( start_pos_lX < num_ref_idx_lX_active_minus1 ) |  |
| num_inserted_pics_lX_minus1 | ue(v) |
| for( i =0 ; i <= num_inserted_pics_lX_minus1; i++ ) { |  |
| if( NumberSourceRPS >1 ) |  |
| source_list_idx_lX[ i ] | u(v) |
| if( NumPicsInSourceList[ source_list_lX[ i ] ] > 1 ) |  |
| ref_pic_set_idx_lX[ i ] | u(v) |
| } |  |
| } |  |
| } |  |

### Semantics

ref_pic_list_modification_flag_lX (X being 0 or 1) equal to 1 specifies that the syntax element start_pos_lX is present in the syntax structure ref_pic_list_modification( X ), and the initial RefPicListX are to be modified. ref_pic_list_modification_flag_lX equal to 0 specifies that the syntax element start_pos_lX is not present in the syntax structure ref_pic_list_modification( X ), and the initial RefPicListX are not to be modified hence the final RefPicListX is identical to the initial RefPicListX.

start_pos_lX specifies the starting position of the initial list for reference picture list modification. The entries with a position smaller than start_pos_lX will not be modified. It is in the range of 0 to num_ref_idx_lX_active_minus1, inclusive. When not present, it is inferred to be equal to 0. The length of start_pos_lX is Ceil( Log2( num_ref_idx_lX_active_minus1 + 1 ) ) bits.

num_inserted_pics_lX_minus1 plus 1 specifies the number of entries signalled in the loop in this syntax structure ref_pic_list_modification( X ). The value of num_inserted_picsX_minus1 is in the range of 0 to num_ref_idx_lX_active_minus1 – start_pos_lX, inclusive. When not present, the value of num_inserted_pics_lX_minus1 is inferred to be equal to 0.

The arrays RPSSubsetX[ ] and NumPicsInSourceList[ ], and the variable NumberSourceRPS are derived as follows:

RefPicSetStCurr0 = (X  = =  0) ? RefPicSetStCurrBefore: RefPicSetStCurrAfter
	RefPicSetStCurr1 = (X  = =  0) ? RefPicSetStCurrAfter: RefPicSetStCurrBefore
	NumPicsStCurr0 = (X  = =  0) ? NumPocStCurrBefore: NumPocStCurrAfter
	NumPicsStCurr1 = (X  = =  0) ? NumPocStCurrAfter: NumPocStCurrBefore

- i is initialized to 0

- if RefPicSetStCurr0 is non-empty, then RPSSubsetX[ i ] = RefPicSetStCurr0, NumPicsInSourceList[ i ] = NumPocStCurr0, and i = i + 1

- if RefPicSetStCurr1 is non-empty, then RPSSubsetX[ i ] = RefPicSetStCurr1, NumPicsInSourceList[ i ] = NumPocStCurr1, and i = i + 1

- if RefPicListLtCurr is non-empty, then RPSSubsetX[ i ] = RefPicListLtCurr, NumPicsInSourceList[ i ] = NumPocLtCurr, and i = i + 1

- NumberSourceRPS = i

source_list_idx_lX[ i ] specifies the index to the list of candidate RPS subsets of the i-th reference picture to be moved to the current index (at that moment) in RefPicListX. The value of source_list_idx_lX[ i ] is in the range of 0 to NumberSourceRPS - 1, inclusive. The length is Ceil( Log2( NumberSourceRPS ) ) bits. When not present, the value of source_list_idx_lX[ i ] is inferred to be equal to 0.

ref_pic_set_idx_lX[ i ] specifies the index, to RPSSubsetX[ source_list_idx_lX[ i ] ], of the i-th reference picture to be moved to the current index in RefPicListX. ref_pic_set_idx_lX[ i ] is in the range of 0 to RPSSubsetX[ source_list_idx_lX[ i ] ] – 1, inclusive. The number of bits needed to signal ref_pic_set_idx_lX[ i ] is Ceil( Log2( RPSSubsetX[ source_list_idx_lX[ i ] ] ) ). When not present, ref_pic_set_idx_lX[ i ] is inferred to be equal to 0.

### Decoding process for reference picture list construction

Initialisation process for reference picture lists

This initialisation process is invoked when decoding a P or B slice header.

When decoding a P or B slice, there shall be at least one reference picture in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr.

The following procedure is conducted once for a P slice with X equal to 0 and twice for a B slice with X equal to 0 and 1 respectively to construct the initial RefPicListX (with X equal to 0 or 1):

cIdx = 0
nEntries = num_ref_idx_lX_active_minus1 + 1
while( cIdx < nEntries )
	for ( RPSSetIdx = 0; RPSSetIdx < NumberSourceRPS; RPSSetIdx++ )
		for ( i = 0; i < NumPicsInSourceList[ RPSSetIdx ] && cIdx < nEntries; i++, cIdx++)
			RefPicListX[ cIdx ] = RPSSubsetX[ RPSSetIdx ][ i ]

Modification process for reference picture lists

Input for this process is a list RefPicListX (with X being 0 or 1).

The variable NumPicsInSourceList is set equal to RPSSubsetX[ source_list_idx_lX[ i ] ].The following procedure is conducted to derive the modified list

if( ref_pic_list_modification_lX_flag )

for( cIdx = start_pos_lX, i = 0; i <= num_inserted_pics_lX_minus1; i++ ) {

for( nIdx = num_ref_idx_lX_active_minus1 + 1; nIdx > cIdx; nIdx− − )
				RefPicListX[ nIdx ] = RefPicListX[ nIdx − 1]

RefPicListX[ cIdx ] = RPSSubsetX[ source_list_idx_lX[ i ] ][ ref_pic_set_idx_lX[ i ] ]

for( pIdx = cIdx+1, nIdx = cIdx+1; nIdx <= num_ref_idx_lX_active_minus1+1; nIdx++ )

if( PicOrderCnt( RefPicListX[ cIdx ] ) != PicOrderCnt( RefPicListX[ nIdx ] ) )

RefPicListX[ pIdx++ ] = RefPicListX[ nIdx ];

}

# Signaling efficiency

We present below a bit-count comparison for parameters present in the syntax structures ref_pic_list_modification( 0 ) and ref_pic_list_modification( 1 ) for the test cases 2.8 and 3.5 in the common test conditions for reference picture marking and list construction proposals in [1]. The test results show that the advanced solution proposed provides overall bit-savings in the signalling compared to the reference picture list modification design in HEVC WD7. The simple solution provides improvement when compared to HEVC WD7 in the low-delay condition.

|  | HEVC WD5 | HEVC WD7 | Simple solution | Advanced solution |
| --- | --- | --- | --- | --- |
| Test 1: Low-delay |  |  |  |  |
| ref_pic_list_modification related bit-count | 8736 | 7530 | 6350 | 5752 |
| % reduction w.r.t WD7 |  |  | 16% | 24% |
| Test 2: Random access |  |  |  |  |
| ref_pic_list_modification related bit-count | 2430 | 1172 | 1949 | 1172 |
| % reduction w.r.t WD7 |  |  | -66% | 0% |

It is reported in the MPEG document m24944 that the proposed reference picture list modification method, when applied to HEVC-based 3DV, outperforms the reference picture list modification method in HEVC WD7 as extended to 3DV. The proposed modification method provides a 34% bit reduction, on an average, compared to the modification method in HEVC WD7 for non-base views in the 3DV common test conditions.

# References

[1] Y-K. Wang, M. M. Hannuksela, T. K. Tan, R. Sjöberg, and Yan Ye, “Common conditions for reference picture marking and list construction proposals,” JCT-VC document JCTVC-H0725, San Jose, February 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).