| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, Sweden, July 11-20, 2012 | Document: JCTVC-J0266 |
| --- | --- |

| Title: | AHG6: Modification to loop filtering across slice boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Matthias Narroschke, Thomas Wedi | Tel: Email: | +49 6103 766 - 1306 semih.esenlik@eu.panasonic.com |
| Source: | Panasonic Corp. |  |  |

_____________________________

# Abstract

In the current HEVC design slice_loop_filter_across_slices_enabled_flag in the slice header is used to control the loop filtering operations at the slice boundaries. The syntax element enables use cases such as “Gradual Decoder Refresh” (GDR) where it is desirable to control the loop filters across the boundaries of each slice independently. However the interpretation of the syntax element is different for the Deblocking Filter (DF) and SAO&ALF. The syntax element is used to control the top and left slice boundaries in the case of DF whereas all 4 boundaries (top, left, bottom and right) are controlled by the same flag in the case of ALF and SAO.

The proposal advocates two improvements in the current HEVC design. 1) Aligning of the loop filtering operation at the slice boundaries such that all 3 loop filters are controlled the at top and left slice boundaries. 2) Using the padding operation always at the 4 LCU corner samples in order to simplify boundary padding processes for SAO and ALF.

# Introduction

The tables 1 and 2 below describe the syntax and semantics for slice_loop_filter_across_slices_enabled_flag in the slice header according to [2].

| slice_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( seq_loop_filter_across_slices_enabled_flag && ( slice_adaptive_loop_filter_flag ¦ ¦ slice_sample_adaptive_offset_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |

Table 1: Loop filtering control in slice header

slice_loop_filter_across_slices_enabled_flag equal to 1 specifies that in-loop filtering operations are performed across slice boundaries; otherwise, the in-loop operations are slice-independent and not applied across slice boundaries. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter. When slice_loop_filter_across_slices_enabled_flag is not present, it is inferred to be equal to seq_loop_filter_across_slices_enabled_flag.

Table 2: Semantics for slice_loop_filter_across_slices_enabled_flag

The syntax element was introduced in the slice header according to the contribution [1] during the 8th JCT-VC meeting is San Jose. The flag provides a mechanism to disable loop filters across the slice boundaries independently for each slice. One use case that is enabled by the slice_loop_filter_across_slices_enabled_flag in the slice header is “Gradual Decoder Refresh”. In this case, it is desirable to disable loop filters at the slice boundaries which separate the “refreshed” region from the “non-refreshed” region, hence refreshed region in the frame does not depend on non-refreshed data. It should also be noted that a similar functionality exists also in the AVC.

## Control of DF across slice boundaries

In the current HEVC the slice_loop_filter_across_slices_enabled_flag is used to control the deblocking filtering at the top and left slice boundaries. If the slice_loop_filter_across_slices_enabled_flag is equal to 0, then top and left slice boundaries are not deblocked. This is illustrated with the figure below.

Figure 1: Control of deblocking filtering at slice boundaries. If slice_loop_filter_across_slices_enabled_flag of slice 2 is equal to 0, deblocking is disabled at the shaded region.

Figure 1 presents an example for deblocking filter control at slice boundaries. In the example slice_loop_filter_across_slices_enabled_flag is assumed to be equal to 0 in slice 2 and 1 in the other slices. According to the current HEVC design that is described in [2] the deblocking filter is disabled at the shaded region (top and left slice boundaries, or edges 1 and 2 in the figure).

## Control of SAO and ALF across slice boundaries

According to the HEVC text specification draft 7[2] and the HM7.0 software [3]  the slice_loop_filter_across_slices_enabled_flag is used to control the ALF and SAO operations at the top, left, bottom and right slice boundaries.

Figure 2: Control of ALF and SAO at slice boundaries. If slice_loop_filter_across_slices_enabled_flag of slice 2 is equal to 0, SAO is disabled and ALF use padding at the shaded region.

The figure above presents an example for SAO and ALF control at slice boundaries. According to [2];

- If slice_loop_filter_across_slices_enabled_flag of slice 2 is equal to 0, then slice 2 is processed by SAO independently from slices 1 and 3. In other words, if a sample from slices 1 and 3 is required for the SAO processing of a sample in slice 2, then the edgeIdx is set to 0 for the sample in slice 2.

- If slice_loop_filter_across_slices_enabled_flag of slice 2 is equal to 0, then slice 2 is processed by ALF independently from slices 1 and 3. In other words, the boundaries of slice 2 (edges 1 to 5 in the above figure) are padded outwards. As a result samples from slices 1 and 3 are not required during the processing of slice 2 with ALF.

# Problem

# The below figure illustrates the inconsistency in the loop filtering control across the slice boundaries.

Figure 3: Inconsistency of loop filter application across the slice boundaries. slice_loop_filter_across_slices_enabled_flag is assumed to be equal to 1 for slices 1 and 3, and 0 for slice 2.

In the example above slice_loop_filter_across_slices_enabled_flag is assumed to be equal to 1 for slices 1 and 3, and 0 for slice 2. According to the example:

- At the top/left boundary of slice 2:

Deblocking filtering is disabled avoiding information dependency between slices 1 and 2. However SAO and ALF follow the normal operation at the upper side of the boundary since slice_loop_filter_across_slices_enabled_flag of slice 1 is equal to 1. As a result the ALF and SAO create information dependency between slices 1 and 2.

- At the bottom/right boundary of slice 2:

Deblocking filtering is enabled. However SAO is disabled and ALF uses padding at the upper side of the boundary since slice_loop_filter_across_slices_enabled_flag of slice 2 is equal to 0.

The example in Figure 3 shows that setting the slice_loop_filter_across_slices_enabled_flag equal to 0 in slice 2 does not prevent the dependency between slices. Therefore during the gradual decoder refresh operation it is not possible to disable the loop filters across the slice boundaries in order to prevent information dependency between the slices.

The problem is emphasized especially in the case of top-to-down gradual decoder refresh operation (the most common GDR implementation) where slices in a frame are refreshed starting from the top-left corner of a frame. In such a case if slice 1 (slice no according to the coding order) is a refreshed slice and is proceeded by slice 2 which is a non-refreshed slice, then it is desirable to set the slice_loop_filter_across_slices_enabled_flag of slice 2 to 0 and disable all three loop filters between the two slices. However according to the current HEVC design, during the processing of slice 1 SAO and ALF operations still use samples from slice 2 (meaning that the refreshed slice is contaminated with the non-refreshed data). Therefore the requirement of “independent refreshed and non-refreshed regions” cannot be satisfied.

Please note that the problem that is described above exists only for the SAO and ALF operations in the current HEVC. The deblocking operation of HEVC follows the boundary filtering control mechanism of AVC in spirit. In other words only SAO and ALF cause conflict with respect to the requirements of the GDR operation.

# Proposal

The proposal advocates aligning all three loop filters by modifying the SAO and ALF slice boundary operation. It is proposed that all three loop filters are controlled jointly at the top/left slice boundaries by the slice_loop_filter_across_slices_enabled_flag in the slice header. In other words If slice_loop_filter_across_slices_enabled_flag is equal to 0 in a slice, then the following are applied according to the proposal:

- Deblocking is disabled at both sides of the top/left slice boundary. (No change with respect to current HEVC [2])

- SAO is controlled at both sides of the top/left slice boundary.

- Padding is used for ALF at both sides of the top/left slice boundary.

Figure 5: Proposed loop filtering control at the slice boundaries. All three loop filters are controlled at the top/left slice boundaries.

According to the proposal the SAO and ALF are aligned with the operation of deblocking filter, which is controlled only at the top and left boundaries of a slice. No change is proposed for the deblocking operation. Moreover no change is proposed for the specific padding operations of the ALF and SAO. It should also be noted that the proposed modification follows the AVC in spirit, where deblocking filter is controlled only at the top and left slice boundaries.

The description of the proposed changes is presented in the document “CD_text_modifications_option1.doc”.

## Further simplification at LCU corners

During the SAO and ALF processes, the samples that lie at the LCU corners require samples from neighboring slices in the diagonal direction. The following figure depicts the SAO and ALF filter masks at the top-right LCU corner position as an example. In the examples one sample from slice 2 is required for the ALF and SAO operations in slice 2.

Figure 6: The sample that is required from the neighbor slice in the diagonal direction. The ALF filter mask and 45-degree SAO filter masks are depicted at the top-right corner of slice 2.

According to the current HEVC design, in order to process an LCU with SAO and ALF the availability of 8 different neighboring LCUs need to be checked (top, left, bottom, right and 4 diagonal neighbors). If an LCU neighbor is available for reference (e.g. the currently processed LCU and the neighbor LCU belong to the same slice) then the SAO and ALF are applied in a normal fashion at the LCU boundary that separates current LCU from the neighboring LCU. If the neighboring LCU is unavailable for reference (e.g. the currently processed LCU and the neighbor LCU belong to different tiles and loop filters are disabled at tile boundaries.), then padding is applied during the processing with SAO and ALF.

However the four diagonal neighboring LCUs are checked for the purpose of processing only the 4 corner samples in an LCU. For instance in the case of 45-degree SAO filter mask, if the top-right LCU neighbor is available for reference, then SAO process is applied at the top-right LCU corner pixel in a normal fashion. If the top-right LCU neighbor is not available, then padding is applied (edgeIdx is set to 0).

In order to simplify the tile and slice boundary checking operation, it is proposed to eliminate the slice and tile boundary checking for the 4 diagonal LCU neighbors (top-left, top-right, bottom-left and bottom-right) completely. Therefore it is proposed to set 4 diagonal LCU neighbors to “unavailable for reference” at all times.

The proposed modification for ALF is illustrated in the figure below.

Figure 7: Proposed modification for ALF

According proposal the samples that lie at the LCU corners (and outside of the LCU) that are shown in red circles are always padded in the case of processing with ALF.

The proposed modification for SAO is illustrated in the figure below.

Figure 8: Proposed modification for SAO

According proposal the samples that lie at the LCU corners (and inside the LCU) that are shown in red circles are skipped by the SAO operation (edgeIdx is set to 0) if either one of the 45-degree and 135-degree SAO filter masks are selected for the processing of the LCU.

According to the proposed simplification the amount of neighbor LCU availability checks are reduced by 50% (Only left, top, bottom and right neighboring LCUs are required to be checked instead of all 8 neighbors). Therefore the implementations of ALF and SAO are simplified considerably.

The description of the proposed changes for further simplification at LCU corners is presented in the document “CD_text_modifications_option2.doc”.

# Conclusions

The contribution advocates two improvements in the current HEVC design. According to the first improvement the ALF and SAO slice boundary control operation is modified to align with the deblocking filtering control operation at the slice boundaries in order to enable gradual decoder refresh operation. It is proposed that the slice_loop_filter_across_slices_enabled_flag in the slice header is used to control all three loop filters jointly and only at the top/left slice boundaries. According to the second improvement it is proposed to simplify the slice/tile boundary checking operations by eliminating the availability checks for 4 diagonal neighbor LCUs. The second improvement eliminates 50% of the neighbor LCU checks, hence the implementation of ALF and SAO is simplified considerably.

# References

- R. Srinivasan, C. Ghone, “AHG15: Slice-Level Control of In-Loop Filter”, JCT-VC Document, JCTVC-H0391, 8th Meeting: San Jose, USA, January, 2012.

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand  “High Efficiency Video Coding (HEVC) text specification draft 7,” JCT-VC Document, JCTVC-I1003, 9th Meeting: Geneva, Switzerland, 27 April – 07 May, 2012.

- HM7.0 Software available under the website: http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-7.0

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).