| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, Sweden, July 11-20, 2012 | Document: JCTVC-J0571 m26297 |
| --- | --- |

| Title: | Side activity report on slice header parsing overhead reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA Alexis Michael Tourapis Apple Inc., USA | Tel: Email: Tel Email: | +1-214-480-3816 zhou@ti.com +1-408-783-5450 atourapis@apple.com |
| Source: | Texas Instruments Inc and Apple Inc. |  |  |

_____________________________

# Abstract

JCTVC-J0083 expresses concerns about the slice header parsing overhead in the evil case. The weighted prediction table is the most parsing intensive part of the slice header. Based on discussion on JCTVC-J0083, it is proposed to reduce the worst case number of weighted prediction tables from 32 in the current design to 8, and impose a limit on the sum of signaled luma/chroma weight flags (namely, luma_weight_l0_flag, luma_weight_l1_flag, chroma_weight_l0_flag, and chroma_weight_l1_flag) in pred_weight_table( ).  Also, it is recommended to make the syntax of pred_weight_table( ) more parsing friendly by pulling luma/chroma weight flags out of the loop. The proposed solution does not change the slice header syntax.

# Introduction

Slice header parsing can be a significant overhead for real time decoding in the worst case. The weighted prediction table is the most parsing intensive part of the slice header. It is proposed to reduce the worst case number of weighted prediction tables from 32 in the current design to 8. In the proposed solution, the constraint is expressed by imposing a limit on the sum of signaled luma/chroma weight flags (namely, luma_weight_l0_flag, luma_weight_l1_flag, chroma_weight_l0_flag, and chroma_weight_l1_flag) in pred_weight_table( ).  Also, the syntax of pred_weight_table( ) is made more parsing friendly by pulling luma/chroma weight flags out of the loop.

# CD text changes

Replace section 7.3.3.3 Prediction weight table syntax with

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( chroma_format_idc != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| luma_weight_l0_flag[ i ] | u(1) |
| if( chroma_format_idc != 0 ) |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| chroma_weight_l0_flag[ i ] | u(1) |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) { |  |
| if( luma_weight_l0_flag[ i ] ) { |  |
| delta_luma_weight_l0[ i ] | se(v) |
| luma_offset_l0[ i ] | se(v) |
| } |  |
| if( chroma_weight_l0_flag[ i ] ) |  |
| for( j =0; j < 2; j++ ) { |  |
| delta_chroma_weight_l0[ i ][ j ] | se(v) |
| delta_chroma_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| If (slice_type = = B ) { |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) |  |
| luma_weight_l1_flag[ i ] | u(1) |
| if( chroma_format_idc != 0 ) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) |  |
| chroma_weight_l1_flag[ i ] | u(1) |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) { |  |
| if( luma_weight_l1_flag[i] ) { |  |
| delta_luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_weight_l1_flag[i]) |  |
| for( j =0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | se(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

#### Add the following constraint at the end of section 7.4.3.3 Weighted prediction parameters semantics (changes marked in yellow)

luma_log2_weight_denom is the base 2 logarithm of the denominator for all luma weighting factors. The value of luma_log2_weight_denom shall be in the range of 0 to 7, inclusive.

delta_chroma_log2_weight_denom is the difference of the base 2 logarithm of the denominator for all chroma weighting factors.

The variable ChromaLog2WeightDenom is specified by luma_log2_weight_denom + delta_chroma_log2_weight_denom and it shall be in the range of 0 to 7, inclusive.

luma_weight_l0_flag[ i ] equal to 1 specifies that weighting factors for the luma component of list 0 prediction using RefPicList0[ i ] are present. luma_weight_l0_flag[i] equal to 0 specifies that these weighting factors are not present.

chroma_weight_l0_flag[ i ] equal to 1 specifies that weighting factors for the chroma prediction values of list 0 prediction using RefPicList0[ i ] are present. If chroma_format_idc == 0 then chroma_weight_l0_flag[i] is set equal to 0. chroma_weight_l0_flag[i] equal to 0 specifies that these weighting factors are not present.

delta_luma_weight_l0[ i ] is the difference of the weighting factor applied to the luma prediction value for list 0 prediction using RefPicList0[ i ].

The variable LumaWeightL0[ i ] is specified by (1 << luma_log2_weight_denom ) + delta_luma_weight_l0[ i ]. When luma_weight_l0_flag is equal to 1, the value of LumaWeightL0[ i ] shall be in the range of −128 to 127, inclusive.  When luma_weight_l0_flag is equal to 0, LumaWeightL0[ i ] is inferred to be equal to 2luma_log2_weight_denom for RefPicList0[ i ].

luma_offset_l0[ i ] is the additive offset applied to the luma prediction value for list 0 prediction using RefPicList0[ i ]. The value of luma_offset_l0[ i ] shall be in the range of −128 to 127, inclusive.  When luma_weight_l0_flag is equal to 0, luma_offset_l0[ i ] is inferred as equal to 0 for RefPicList0[ i ].

delta_chroma_weight_l0[ i ][ j ] is the difference of the weighting factor applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr.

The variable ChromaWeightL0[ i ][ j ] is specified by ( 1 << ChromaLog2WeightDenom ) + delta_chroma_weight_l0[ i ][ j ]. When chroma_weight_l0_flag is equal to 1, the value of ChromaWeightL0[ i ][ j ] shall be in the range of −128 to 127, inclusive.  When chroma_weight_l0_flag is equal to 0, ChromaWeightL0[ i ][ j ] is inferred to be equal to 2ChromaLog2WeightDenom for RefPicList0[ i ].

delta_chroma_offset_l0[ i ][ j ] is the difference of the additive offset applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr.

The variable ChromaOffsetL0[ i ][ j ] is specified as follows:

shift = 1 << ( BitDepthC − 1 )

ChromaOffsetL0[ i ][ j ] = (delta_chroma_offset_l0[i][j] – 
				( (shift*ChromaWeightL0[ i ][ j ]) >> ChromaLog2WeightDenom ) − shift )	 (7-63)

The variable ChromaOffsetL0[ i ][ j ] shall be in the range of −127 to 128, inclusive. When chroma_weight_l0_flag is equal to 0, ChromaOffsetL0[ i ][ j ] is inferred to be equal to 0 for RefPicList0[ i ].

luma_weight_l1_flag, chroma_weight_l1_flag, delta_luma_weight_l1, luma_offset_l1, delta_chroma_weight_l1, delta_chroma_offset_l1 have the same semantics as luma_weight_l0_flag, delta_luma_weight_l0, luma_offset_l0, chroma_weight_l0_flag, delta_chroma_weight_l0, delta_chroma_offset_l0, respectively, with l0, list 0, and List0 replaced by l1, list 1, and List1, respectively.

Placeholder (TBR)

Variant 1

For P slices, luma_weight_l1_flag[i] and chroma_weight_l1_flag[i] are inferred to 0.

The sum of all signaled luma_weight_lX_flag[i], with  X being 0 and 1, within a single picture_weight_table shall not exceed 8.“

The sum of all signaled chroma_weight_lX_flag[i], with  X being 0 and 1, within a single picture_weight_table shall not exceed 8.

Variant  2

For P slices, the sum of luma_weight_l0_flag[i], for i = 0, 1, …, num_ref_idx_l0_active_minus1, shall be in the range from 0 to 8, inclusive.

For P slices, the sum of chroma_weight_l0_flag[i], for i = 0, 1, …, num_ref_idx_l0_active_minus1, shall be in the range from 0 to 8, inclusive.

For B slices, the sum of sum_luma_weight_l0_flags and sum_luma_weight_l1_flags shall be in the range from 0 to 8, inclusive. Where sum_luma_weight_l0_flags is the sum  of  luma_weight_l0_flag[i], for i = 0, 1, …, num_ref_idx_l0_active_minus1, and sum_luma_weight_l1_flags  is the sum of  luma_weight_l1_flag[i], for i = 0, 1, …, num_ref_idx_l1_active_minus1.

For B slices, the sum of sum_chroma_weight_l0_flags and sum_chroma_weight_l1_flags shall be in the range from 0 to 8, inclusive. Where sum_chroma_weight_l0_flags is the sum of  chroma_weight_l0_flag[i], for i = 0, 1, …, num_ref_idx_l0_active_minus1, and sum_chroma_weight_l1_flags  is the sum of  chroma_weight_l1_flag[i], for i = 0, 1, …, num_ref_idx_l1_active_minus1.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-I1100, 9th Meeting: Geneva, Switzerland, 27 April – 07 May, 2012

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “High Efficiency Video Coding (HEVC) text specification draft 7,” JCT-VC Document, JCTVC-I1003, 9th Meeting: Geneva, Switzerland, 27 April – 07 May, 2012.

[3] M. Zhou, “AHG9: on slice header parsing overhead reduction”, JCTVC Document, JCTVC-J0083, 10th meeting: Stockholm, Sweden, July 12 – 20, 2012

# Patent rights declaration(s)

Texas Instruments, Inc. and Apple Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).